3GPP TSG RAN WG1 #114bis			R1-231nnnn

Xiamen, China, October 9th – October 13th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.16.6

Agenda Item:		8.16.6

Document for: 	Endorsement

### 8.16.6	UE features for XR enhancements

Agreement

- FG 50-1 is updated and an additional FG 50-1a is introduced as follows

| 50. NR_XR_Enh | 50-1 | Multi-PUSCHs for Configured Grant | 1. Determination of time-domain resource allocation for CG-PUSCHs associated to a multi-PUSCHs CG 2. Maximum supported number of consecutive slots configured for CG-PUSCH TOs in one CG period, candidate value set: {16, 32} FFS whether to separate this FG for type-1 and type-2 CG FFS whether to separate this FG for multiple CG configurations FFS whether to separate this FG for shared spectrum FFS whether to report maximum supported number of configured CG-PUSCH TOs in one CG period | One or both of {5-19, 5-20} | Yes | N/A | UE is not able to support Multi-PUSCHs per one period in Configured grant in licensed band |  |  |  |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_XR_Enh | 50-1a | Multiple active multi-PUSCHs configured grant configurations for a BWP of a serving cell | 1. Supported maximum number of configured/active configured grant configurations in a BWP of a serving cell Candidate values for component 1: {[2, 4, 8, 12]} 2. Supported maximum number of configured/active configured grant configurations across all serving cells, and across MCG and SCG in case of NR-DC Candidate values for component 2: {2, …, 32} | 50-1, [11-9] | Yes | N/A |  |  |  |  |  | [when UE supports both FG 11-9 and 50-1a the maximum number configured for legacy CG and multi-PUSCH CG should not exceed a maximum number, FFS maximum number] [For all the reported bands in FR1, a same X1 value is reported for component 2. For all the reported bands in FR2, a same X2 value is reported for component 2.] [The total number of configured/active configured grant configurations across all serving cells in FR1 is no greater than X1.] [The total number of configured/active configured grant configurations across all serving cells in FR2 is no greater than X2.] [If there are some serving cell(s) in FR1 and some serving cell(s) in FR2, the total number of configured/active configured grant configurations across all serving cells is no greater than max(X1, X2).] [Regarding the interpretation of UE capabilities in case of cross-carrier operation, support of FG50-9 is based on the support of this capability for the band of the scheduled/triggered/indicated cell only] | Optional with capability signaling |

Agreement

- FG 50-2 is updated as follows

| 50. NR_XR_Enh | 50-2 | UCI indication of unused CG-PUSCH transmission occasions | 1. Multiplexing of the Unused transmission occasions UCI (UTO-UCI) on a CG-PUSCH FFS whether to merge this FG into FG 50-1 FFS whether to separate this FG for UTO-UCI multiplexing with HARQ-ACK | One or both of {5-19, 5-20} | Yes | N/A | UE is not able to indicate the unused resources in CG |  |  |  |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Introduce following FG

| 50. NR_XR_Enh | 50-3 | PDCCH monitoring resumption after UL NACK | Support PDCCH monitoring resumption after UL NACK | 29-3a | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2308851	On UE features for XR enhancements	Nokia, Nokia Shanghai Bell

R1-2308883	UE features for XR enhancements	Huawei, HiSilicon

R1-2309008	Discussion on UE features for XR Enhancements	Spreadtrum Communications

R1-2309101	Discussion on Rel-18 XR UE features	vivo

R1-2309183	Discussion on UE features for XR enhancements	ZTE, Sanechips

R1-2309309	Discussion on UE features for XR enhancements	LG Electronics

R1-2309406	UE features for XR	Samsung

R1-2309449	Discussion on UE features for XR-specific capacity enhancements	xiaomi

R1-2309516	Discussion on Rel-18 UE features for XR in RAN1	CATT

R1-2309628	Discussion on UE features for XR	OPPO

R1-2309861	UE features for XR enhancements	Apple

R1-2310004	Discussion on UE features for XR enhancements	MediaTek Inc.

R1-2310060	Discussion on UE features for Rel-18 XR enhancements	NTT DOCOMO, INC.

R1-2310173	UE features for XR enhancements	Qualcomm Incorporated

R1-2310256	On UE features for XR Enhancements	Ericsson