3GPP TSG RAN WG1 #114bis			R1-231nnnn

Xiamen, China, October 9th – October 13th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.16.8

Agenda Item:		8.16.8

Document for: 	Endorsement

### 8.16.8	UE features for coverage enhancements

Agreement

- Component of FG 54-1 is confirmed as: Support of multiple PRACH transmissions with the same Tx spatial filter. Support {2, 4, 8} for the number of multiple PRACH transmissions with the same Tx spatial filter.

- The column of “Consequence if the feature is not supported by the UE” in FG 54-1 is confirmed as: UE doesn’t support multiple PRACH transmissions with the same Tx spatial filter

Agreement

- The column of “Need for the gNB to know if the feature is supported” for FG54-1 is confirmed as “Yes”

Agreement

- Add a note in FG 54-3: If UE supporting this FG supports FG 11-1, the UE supports FG 54-3 with DCI format 0_2

- Delete “FFS whether to separate this FG for DCI 0_1/0_2[/0_3]” from FG 54-3

Agreement

- No prerequisite FG is defined for FG 54-3

Agreement

- FG 54-3a is updated as follows

| 54. NR_cov_enh2 | 54-3a | PHR enhancement for dynamic waveform switching | Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH FFS details | 54-3 | Yes | N/A |  |  |  |  |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2308853	On UE features for coverage enhancements	Nokia, Nokia Shanghai Bell

R1-2309010	Discussion on UE features for coverage enhancements	Spreadtrum Communications

R1-2309103	Discussions on Rel-18 NR coverage enhancements UE features	vivo

R1-2309138	Discussion on UE feature for coverage enhancements	ZTE

R1-2309190	Discussion on UE features for NR coverage enhancement	Intel Coporation

R1-2309408	UE features for coverage enhancements	Samsung

R1-2309451	Discussion on UE features for coverage enhancements	xiaomi

R1-2309518	UE features for coverage enhancements	CATT

R1-2309560	Discussion on UE features for coverage enhancement	China Telecom

R1-2309616	Rel-18 UE features for coverage enhancements	OPPO

R1-2309632	Discussion on UE features for further NR coverage enhancement in Rel.18	Panasonic

R1-2309694	Discussion on UE feature for coverage enhancement	CMCC

R1-2309748	UE features for Coverage Enhancement	Huawei, HiSilicon

R1-2309863	Views on UE features for Rel-18 NR coverage ehancmenents	Apple

R1-2309970	UE Features for Coverage Enhancements	Ericsson

R1-2310003	Discussion on UE features for coverage enhancements	MediaTek Inc.

R1-2310062	Discussion on UE features for NR further coverage enhancements	NTT DOCOMO, INC.

R1-2310110	UE features for Rel-18 Coverage Enhancements	Sharp

R1-2310175	UE features for coverage enhancements	Qualcomm Incorporated