3GPP TSG RAN WG1 #115                                                                                              R1-2311064

Chicago, USA, November 13th - November 17th, 2023

Source:	vivo

Title:	Discussions on RAN2 LS on RACH-less Handover

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss RAN1 impacts on following aspects based on RAN2 LS on RACH-less Handover [1], the discussions in RAN1#113 as summarized in [2] and the final LS in RAN1#114 [3]. According to the discussions, a draft TP for TS38.213 is proposed.

- Pre-allocated grant and dynamic grant initial PUSCH transmission

- Beam determination for initial PUSCH transmission

- Power control for initial PUSCH transmission.

- Discussions

For mobility enhancement in Rel-18 NR NTN, RAN2 has discussed NTN RACH-less HO and made the following agreements.

Agreements in RAN2#121: - Support RACH-less Handover in Rel-18. - RACH-less Handover in NR NTN is a L3 mobility procedure (FFS if this is combined with the unchanged PCI approach, if supported) and uses the LTE’s RACH-less Handover procedure as a baseline. FFS on TA acquisition - In NTN RACH-less handover, network indicates (implicitly or explicitly) whether NTA in the target cell is identical to the source cell or explicitly provided by the NW. - Support dynamic grant from the target cell for RACH-less PUSCH transmission to reduce random access congestion in the target cell. FFS whether to limit the solution to same feeder link/gateway scenario Agreements in RAN2#121bis-e: - In Rel-18 we don’t aim at RACH-less HO for NTN-TN mobility - For initial UL transmission in RACH-less HO, support pre-allocated grant in RACH-less HO command - NTN RACH-less HO is supported for Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB; - NTN RACH-less HO can be supported for intra-satellite handover with different feeder links, i.e., with gateway/gNB switch, inter-satellite handover with gateway/gNB switch, and inter-satellite handover with same gateway/gNB. - RAN2 confirms the general UE procedure for NTN RACH-less HO - receive a RACH-less HO command which can include pre-allocated grant optionally. FFS N_TA is optional. (RRC) - start timer T304 for the target cell (RRC) - perform DL and UL synchronization, and start timer T430. FFS how to perform RACH-less UL synchronization to NTN target cell. (RRC, MAC) - start time alignment timer (MAC) - monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY) - send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY) - consider RACH-less HO is completed upon receiving NW confirmation. FFS how to confirm RACH-less HO is successfully completed. (RRC, MAC) - stop timer T304 for the target cell. (RRC) FFS whether to release UL grant if pre-allocated after RACH-less HO completion FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported - The pre-allocated grant is provided as type-1 CG - At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field. FFS if anything else is needed for dynamic grant - Consider to support combining RACH-less HO with time-based CHO for NTN, taking into account the 1) validity of pre-allocated grant and potential waste of reserved resource; 2) when/how to provide dynamic grant in PDCCH. Agreements in RAN2#122: - In NTN RACH-less handover, NW either indicates NTA in the target cell is identical to the source cell, or the NTA explicitly provided by the NW is 0. RAN2 will not discuss the case where NTA does not equal to 0 2. From RAN2 perspective synchronization among source and target cells is not an issue in NTN RACH-less HO 3. Release pre-allocated UL grant after RACH-less HO completion 4. LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI 5. Remove “FFS how to perform RACH-less UL synchronization to NTN target cell”, RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact, if any) Agreements in RAN2#123: - Single beam can be indicated in HO command to monitor target cell PDCCH for dynamic grant for initial UL transmission - The pre-allocated grant is provided with association to SSBs - The mapping between type-1 CG and SSBs in CG-SDT can be the baseline of how to configure pre-allocated grant mapped to SSBs (can rediscuss in case of different input from RAN1) - UE selects an SSB associated to the pre-allocated grant with RSRP above a configured threshold, use the selected SSB and the corresponding UL grant occasions for the initial UL transmission - ta-Report can be included in ServingCellConfigCommon in the RACH-less HO command - RAN2 understands that if pre-allocated grant is not configured and dynamic grant is used for first UL transmission, if UL HARQ mode is configured, HARQ mode A is recommended for the HARQ process (this is anyway up to NW implementation and there is no Stage2 and Stage3 spec impact) - The MAC entity applies the N_TA (value 0 or same as source cell) configured in the RACH-less HO command for the PTAG. FFS on when timerAlignmentTimer associated with this TAG starts - If no SSB mapping to pre-allocated grant has RSRP above the threshold, fallback to RACH HO (with new SSB selection), while T304 is running

In RAN1 #114 and RAN1 #114bis meetings, the following agreements were achieved for RACH-less handover.

Agreement The following response to Question 2 in RAN2 LS (R1-2304322) is agreed: - To monitor target cell PDCCH for dynamic grant for initial UL transmission, RAN1 think that there is no case where multiple beams are indicated for RACH-less handover. In this case, UE doesn’t expect that multiple beams are indicated from NW. Agreement For pathloss measurement in case of dynamic scheduled initial PUSCH for RACH-less handover, the UE calculates PLb,f,c(qd) using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to monitor PDCCH scheduling dynamic UL grant for initial transmission. Agreement The following response to Question 3 in RAN2 LS (R1-2304322) is agreed: - For the initial UL transmission scheduled by dynamic grant in RACH-less handover, RAN1 thinks that it follows the principle for power control for Msg3 (or MsgA) PUSCH as described in clause 7.1.1 in TS 38.213 except for pathloss determination. For pathloss determination, the UE uses a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to monitor PDCCH scheduling dynamic UL grant for initial transmission. - RAN1 may continue further discussion on question 3. Observation There is potential RAN1 discussion for the following aspects to support the RAN2 work on RACH-less handover. - The pre-allocated grant is provided with association to SSBs - The mapping between type-1 CG and SSBs in CG-SDT can be the baseline of how to configure pre-allocated grant mapped to SSBs

From the above, remaining issues can be summarized as following on RACH-less handover from RAN1 perspective:

- For initial PUSCH transmission in RACH-less HO, support pre-allocated grant in RACH-less HO command.

  - provided as type-1 CG.

  - provided with association to SSBs.

  - mapping between type-1 CG and SSBs in CG-SDT can be the baseline of how to configure pre-allocated grant mapped to SSBs.

- For initial PUSCH transmission in RACH-less HO, support monitoring PDCCH in target cell for dynamic grant if pre-allocated grant is not configured in RACH-less HO command.

  - single beam can be indicated in HO command to monitor target cell PDCCH for dynamic grant for initial PUSCH transmission.

  - UE doesn’t expect that multiple beams are indicated from NW.

  - power control for initial PUSCH transmission follows the principle for power control for Msg3 (or MsgA) PUSCH as described in clause 7.1.1 in TS 38.213 except for pathloss determination. For pathloss determination, the UE uses a RS resource from a single beam, corresponding to a DL RS or an SS/PBCH block with same SS/PBCH block index as the one the UE uses to monitor PDCCH scheduling dynamic UL grant for initial transmission.

In current spec., the RS that can be quasi co-located with the DMRS port of a PDCCH reception is based on the following text copied from 3GPP TS 38.213 v18.0.0.

For a CORESET other than a CORESET with index 0, - if a UE has not been provided a configuration of TCI state(s) by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET, or has been provided initial configuration of more than one TCI states for the CORESET by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block the UE identified during the initial access procedure, or for a most recent configured grant PUSCH transmission as described in clause 19 for a same HARQ process; - if a UE has been provided a configuration of more than one TCI states by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET as part of Reconfiguration with sync procedure as described in [12, TS 38.331] but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331]. <<unchanged text omitted>> For a CORESET other than a CORESET with index 0, if a UE is provided a single TCI state for a CORESET, or if the UE receives a MAC CE activation command for one or two of the provided TCI states for a CORESET, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by the TCI states. For a CORESET with index 0, the UE expects that a CSI-RS configured with qcl-Type set to 'typeD' in a TCI state indicated by a MAC CE activation command for the CORESET is provided by a SS/PBCH block

According to current spec., uplink measurements or uplink transmissions are always needed for determining the relevant DL RS beam unless the uplink resources are mapped to SSBs. For the first PDCCH reception in target cell to schedule DG PUSCH, although there’s no uplink transmission in target cell, source cell may report the downlink RRM measurements of target cell to the target cell so that downlink beam preferred by UE for this PDCCH transmission can be known by target cell before triggering RACH less handover. Thus, for the first PDCCH transmission in target cell to schedule initial DG PUSCH, the corresponding TCI state can be indicated in handover command in the RACH less procedure.

Therefore, for monitoring PDCCH in target cell for dynamic grant, a UE is provided a single TCI state for a CORESET, and the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the SS/PBCH block indicated by the single TCI state associated with the initial PUSCH transmission with respect to average gain and quasi co-location 'typeA' or 'typeD' properties.

According to above, we provide TP#1 which is proposed to include section 22 in TS38.213 V18.0.0.

Proposal 1: Adopt TP#1 to include section 22 in TS38.213 for RACH-less handover.

| Reason for change: | Completion of supporting RACH-less handover for NTN in NR. |
| --- | --- |
| Summary of change: | Add descriptions for the initial PUSCH transmission of RACH-less handover. |
| Consequences if not approved: | Incomplete support of RACH-less handover for NTN in NR. |
| ------------------------------------------------Start of TP#1 of TS38.213 V18.0.0--------------------------------------------------- 21 L1/L2-triggered mobility procedures <<unchanged text omitted>> A UE can be provided by a MAC CE in a PDSCH reception on the serving cell [11, TS 38.321] a TCI-State and/or TCI-UL-State in LTM-dl-OrJointTCI-StateToAddModList and/or LTM-ul-TCI-ToAddModList indicating a unified TCI state [6, TS 38.214] for applicable receptions or transmissions on a candidate cell from the number of candidate cells. The UE applies the TCI-State and/or TCI-UL-State, if indicated by the MAC CE, from a first slot that is TBD after the last symbol of a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, and μ is the SCS configuration for the TBD. 22 Initial PUSCH transmission in RACH-less handover A UE can be provided one or more configurations by respective one or more ConfiguredGrantConfig for a configured grant Type 1 PUSCH transmission as an initial PUSCH transmission in RACH-less handover. In this case, UE is provided by [rachLess-SSB-Subset] a number of SS/PBCH block indexes NPUSCHSS/PBCH to map to a number of valid PUSCH occasions for the initial PUSCH transmissions over an association period. A PUSCH occasion for a PUSCH transmission is defined by a time resource and a frequency resource and is associated with a DM-RS provided by cg-DMRS-Configuration for the configuration of PUSCH transmissions. A UE can be provided a number of repetitions for a PUSCH transmission by repK or numberOfRepetitions. If the number of repetitions is provided and larger than 1, all the PUSCH occasions of the repetitions for the PUSCH transmission are mapped to the same SS/PBCH block index(es), and the UE encodes the transport block using redundancy version number 0 if the UE is not provided repK-RV. All the PUSCH occasions of the repetitions are not valid if any PUSCH occasion of the repetitions is not valid. An association period, starting from frame with SFN 0, for mapping NPUSCHSS/PBCH SS/PBCH block indexes, from the number of SS/PBCH block indexes, to valid PUSCH occasions and associated DM-RS resources is the smallest value in the set determined by the PUSCH configuration period provided by periodicity in ConfiguredGrantConfig according to Table 19.1-1 such that NPUSCHSS/PBCH SS/PBCH block indexes are mapped at least once to valid PUSCH occasions and associated DM-RS resources within the association period. A UE is provided a number of SS/PBCH block indexes associated with a PUSCH occasion and a DM-RS resource by [rachLess-SSB-PerCG-PUSCH]. If after an integer number of SS/PBCH block indexes to PUSCH occasions and associated DMRS resources mapping cycles within the association period there is a set of PUSCH occasions and associated DMRS resources that are not mapped to NPUSCHSS/PBCH SS/PBCH block indexes, no SS/PBCH block indexes are mapped to the set of PUSCH occasions and associated DMRS resources. An association pattern period includes one or more association periods and is determined so that a pattern between PUSCH occasions with associated DMRS resources and SS/PBCH block indexes repeats at most every 640 msec. PUSCH occasions and associated DMRS resources not associated with SS/PBCH block indexes after an integer number of association periods, if any, are not used for PUSCH transmissions. NPUSCHSS/PBCH SS/PBCH block indexes are mapped to valid PUSCH occasions and associated DMRS resources in the following order - first, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211] - second, in increasing order of PUSCH configuration period indexes A PUSCH occasion is valid if it does not overlap with a valid PRACH occasion as described in clause 8.1. For unpaired spectrum and for SS/PBCH blocks with indexes provided by ssb-PositionsInBurst in SIB1 - if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion - does not precede a SS/PBCH block in the PUSCH slot, and - starts at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2 - if a UE is provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion - is within UL symbols - starts at least Ngap symbols after a last downlink symbol, and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2 If pre-allocated grant for initial PUSCH transmission is not configured in RACH-less handover command, a UE can be provided a search space set by RACH-less handover command to monitor PDCCH for detection of an UL grant for initial PUSCH transmission in RACH-less handover. The UE is provided a SS/PBCH block indicated by a single TCI state for a CORESET, and the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the SS/PBCH block associated with the initial PUSCH transmission with respect to average gain and quasi co-location 'typeA' or 'typeD' properties. For the initial PUSCH transmission scheduled by dynamic grant in RACH-less handover, a UE determines the power of initial PUSCH transmission as described in clause 7.1.1, where the PLb,f,c(qd) is calculated using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to monitor PDCCH scheduling the initial PUSCH transmission. <<unchanged text omitted>> ------------------------------------------------End of TP#1 to TS38.213 V18.0.0---------------------------------------------------- |  |

- Conclusion

In this contribution, we further discuss the remaining issues on RAN2 LS on RACH-less Handover from RAN1 perspective. According to the discussions, we have following proposal.

Proposals 1: Adopt TP#1 to include section 22 in TS38.213 for RACH-less handover.

References

- R1-2304322, LS on RACH-less Handover, 3GPP TSG RAN WG1#113, May 22nd - 26th, 2023.

- R1-2306152, Moderator summary on reply LS on RACH-less handover, 3GPP TSG RAN WG1 Meeting #113, May 22nd - May 26th, 2023.

- R1-2308568, Reply LS on RACH-less Handover, 3GPP TSG RAN WG1#114, August 21st - August 25th, 2023.