3GPP TSG-RAN WG1 Meeting #115 	R1-2311295

# Chicago, USA, November 13th – November 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on determining power control adjustment state for PUSCH transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2023-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. In TS 38.213, it is specified that ‘If the UE is provided SRI-PUSCH-PowerControl, the UE obtains a mapping between a set of values for the SRI field in a DCI format scheduling the PUSCH transmission and the l value(s) provided by sri-PUSCH-ClosedLoopIndex and determines the l value that is mapped to the SRI field value’, and a different UE behavior on determing the l value is also provided for the case that the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook'/'nonCodebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission includes two SRI fields. That is, two different behaviors on determing the l value are specified for the case that the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook'/'nonCodebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission includes two SRI fields. The above behaviors conflict with each other. 2. In TS 38.213, it is specified that ‘If the PUSCH transmission is scheduled by a DCI format that does not include an SRI field, or if an SRI-PUSCH-PowerControl is not provided to the UE, l=0’, and a different UE behavior on determing the l value is also provided for the case that the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission does not include an SRI field. That is, two different behaviors on determing the l value are specified for the case that the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission does not include an SRI field. The above behaviors conflict with each other as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Change ‘If the UE is provided SRI-PUSCH-PowerControl, the UE obtains a mapping between a set of values for the SRI field in a DCI format scheduling the PUSCH transmission and the l value(s) provided by sri-PUSCH-ClosedLoopIndex and determines the l value that is mapped to the SRI field value’ to ‘If the UE is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission includes an SRI field, the UE obtains a mapping between a set of values for the SRI field in a DCI format scheduling the PUSCH transmission and the l value(s) provided by sri-PUSCH-ClosedLoopIndex and determines the l value that is mapped to the SRI field value’. 2. Change ‘If the PUSCH transmission is scheduled by a DCI format that does not include an SRI field, or if an SRI-PUSCH-PowerControl is not provided to the UE, l=0’ to ‘If the UE is provided one SRS resource set in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field, or if an SRI-PUSCH-PowerControl is not provided to the UE, l=0’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE’s behaviour on determining the l value(s) is unclear for the following two cases, respectively: 1. The UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook'/'nonCodebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission includes two SRI fields. 2. The UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission does not include an SRI field. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.1.1	UE behaviour

If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as

[dBm]

where,

-	PCMAX,f,c(i)is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2] and [8-3, TS 38.101-3] for carrier f of serving cell c in PUSCH transmission occasion i.

-	PO_PUSCH,b,f,c(j) is a parameter composed of the sum of a component PO_NOMINAL,PUSCH,f,c(j) and a component PO_UE_PUSCH,b,f,c(j) where j∈0,1,…,J-1.

<Unrelated part omitted>

-	For the PUSCH power control adjustment state fb,f,c(i,l) for active UL BWP b of carrier f of serving cell c in PUSCH transmission occasion i

-	δPUSCH,b,f,c(i,l) is a TPC command value included in a DCI format that schedules the PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c or jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, as described in clause 11.3

-	l∈0,1 if the UE is configured with twoPUSCH-PC-AdjustmentStates and l=0 if the UE is not configured with twoPUSCH-PC-AdjustmentStates or if the PUSCH transmission is scheduled by a RAR UL grant as described in clause 8.3

-	if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes a SRS resource set indicator field, and for active UL BWP b of carrier f of serving cell

-	if the SRS resource set indicator value is 00, l is equal to the value of powerControlLoopToUse in ConfiguredGrantConfig

-	if the SRS resource set indicator value is 01, l is equal to the value of powerControlLoopToUse2 in ConfiguredGrantConfig

-	if the SRS resource set indicator value is 10 or 11, a first l and a second l respectively associated with the first and second SRS resource set are respectively equal to powerControlLoopToUse and powerControlLoopToUse2 in ConfiguredGrantConfig

-	else if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and is provided p0-PUSCH-Alpha2, for a transmission of a configured grant Type 1 PUSCH and for active UL BWP b of carrier f of serving cell

-	a first l is equal to the value of powerControlLoopToUse in ConfiguredGrantConfig that is associated with the first srs-ResourceIndicator in rrc-ConfiguredUplinkGrant

-	a second l is equal to the value of powerControlLoopToUse2 in ConfiguredGrantConfig that is associated with the second srs-ResourceIndicator in rrc-ConfiguredUplinkGrant

-	else if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format 0_0 and for active UL BWP b of carrier f of serving cell

-	l is equal to the value of powerControlLoopToUse in ConfiguredGrantConfig

-	else, for a PUSCH (re)transmission configured by ConfiguredGrantConfig, the value of l∈0,1 is provided to the UE by powerControlLoopToUse in ConfiguredGrantConfig.

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission includes two SRI fields, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the first and second SRI fields and the l values provided by sri-PUSCH-ClosedLoopIndex, and determines the l values mapped to the values of the first and second SRI fields corresponding to each SRS resource set with usage set to 'codebook', respectively

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission includes two SRI fields,	the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for

-	the first SRI field value and the l values provided by sri-PUSCH-ClosedLoopIndex, and determines the l value that is mapped to the first SRI field value corresponding to the first SRS resource set with usage set to 'nonCodebook', and

-	the value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, and the l value(s) provided by sri-PUSCH-ClosedLoopIndex, and determines the l value that is mapped to the value corresponding to the second SRS resource set with usage set to 'nonCodebook'

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission does not include an SRI field

-	If the UE is provided twoPUSCH-PC-AdjustmentStates

-	the UE determines l=0 for the PUSCH transmission corresponding to the first SRS resource set with usage set to 'codebook' or 'nonCodebook', and l=1 for the PUSCH transmission corresponding to the second SRS resource set with usage set to 'codebook' or 'nonCodebook'

-	else

-	the UE determines l=0 for the PUSCH transmission

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and is provided twoPUSCH-PC-AdjustmentStates

-	If the DCI format includes two TPC command values and the PUSCH transmissions are associated with l=0 and l=1, the UE applies the first TPC command value for l=0 and applies the second TPC command value for l=1

-	If the DCI format includes two TPC command values and the PUSCH transmissions are associated with l=0, the UE applies the first TPC command value for l=0 and ignores the second TPC command value

-	If the DCI format includes two TPC command values and the PUSCH transmissions are associated with l=1, the UE applies the second TPC command value for l=1 and ignores the first TPC command value

-	If the DCI format includes one TPC command value, the UE applies the TPC command value for all l associated with the PUSCH transmission

-	If the UE is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission includes an SRI field, the UE obtains a mapping between a set of values for the SRI field in a DCI format scheduling the PUSCH transmission and the l value(s) provided by sri-PUSCH-ClosedLoopIndex and determines the l value that is mapped to the SRI field value

-	If the UE is provided one SRS resource set in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field, or if an SRI-PUSCH-PowerControl is not provided to the UE, l=0

-	If the UE obtains one TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUSCH-RNTI, the l value is provided by the closed loop indicator field in DCI format 2_2

<Unrelated part omitted>