3GPP TSG RAN WG1 #115			R1-2311545

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:	8.4.1

Source:	China Telecom

Title:	Remaining issues on UE complexity reduction for eRedCap UEs

Document for:	Discussion

Introduction

In RAN1#114bis meeting, the following agreements on UE complexity reduction were reached [1]:

Agreement - The following does not apply to FG 48-2 UEs for CFRA: - RAR PDSCH timeline relaxation Agreement Adopt the following TP for 38.213 clause 17.1A: When - a UE receives a PDSCH scheduled by a DCI format with CRC scrambled by a RA-RNTI or a MsgB-RNTI over a number of PRBs that is larger than 25 PRBs for 15 kHz SCS or larger than 12 PRBs for 30 kHz SCS, and - the UE does not correctly receive the transport block provided by the PDSCH, or if the higher layers at the UE do not identify a RAPID associated with a corresponding PRACH transmission from the UE if requested by higher layers, Tthe UE shall be ready to transmit a PRACH no later than NT,1+1.75 msec for 15 kHz SCS, or no later than NT,1+1.25 msec for 30 kHz SCS, after the last symbol of the PDSCH reception, or after the last symbol of the window as described in Clauses 8.2 and 8.2A.. Agreement - For a UE with BB bandwidth reduction, for multicast MBS specified in Rel-17, the number of PRBs scheduled in DCI is not larger than 25/15 PRBs for 15/30 kHz SCS (irrespective of whether HARQ feedback is enabled or disabled). Agreement Adopt the following TP for 38.213 clause 17.1A: A UE that has not indicated FG 48-2 is not required to process a PDSCH reception in slot n that is scheduled by a DCI format with CRC scrambled by a G-RNTI for broadcast or a MCCH-RNTI over a number of PRBs that is larger than 25 PRBs for 15 kHz SCS, or larger than 12 PRBs for 30 kHz SCS, when the PDSCH reception is with repetitions or when the UE receives another PDSCH in slot n+1.. For the editors: The above endorsed text proposals to 38.213 are also collected in R1-2310567. Please consider them in the next specification revision. Agreement - An eRedCap UE with bandwidth reduction, depending on indicated UE capability, the UE can decode a PDSCH for MBS broadcast and a PDSCH for unicast with the two PDSCH partially or fully overlapping in time in non-overlapping PRBs, if the total number of PRBs does not exceed the maximum number of PRBs that the UE can receive or process per slot. Agreement For which (if any) of the following 2-step RACH cases, continue to discuss if there is a need to update the specifications to reflect the RAN1 agreement that RAR PDSCH timeline relaxation does not apply to FG 48-2 UEs for CFRA: - Case 2a: Between reception of fallbackRAR and transmission of Msg3 - Case 2b: Between reception of successRAR and transmission of corresponding HARQ-ACK - Case 2c: Between reception of MsgB PDSCH scheduled by MSGB-RNTI in which UE does not correctly receive the transport block in the corresponding PDSCH within the window and transmission of only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure. - Case 2d: Between reception of MsgB PDSCH scheduled by MSGB-RNTI with RAPID which is not associated with the corresponding PRACH transmission from the UE and transmission of only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure. Agreement - An eRedCap UE with bandwidth reduction, depending on indicated UE capability, the UE can decode a PDSCH for MBS multicast and a PDSCH for unicast with the two PDSCH partially or fully overlapping in time in non-overlapping PRBs, if the total number of PRBs does not exceed the maximum number of PRBs that the UE can receive or process per slot. Agreement - Continue to discuss whether and how to update the specification regarding the following aspect: - simultaneous MBS broadcast/multicast and unicast when the total number of PRBs exceeds the maximum number of PRBs that the UE can receive or process per slot (if this is a valid case) Conclusion For an eRedCap UE (with or without UE BB bandwidth reduction), the following specification in 38.214 for simultaneous reception of MCCH PDSCH and PBCH still applies: - The UE is expected to decode PDSCH scheduled with MCCH-RNTI and PBCH in PCell that partially or fully overlaps in time in non-overlapping PRBs in PCell. Agreement - Continue to discuss potential clarification of “A UE that has not indicated FG 48-2” in the paragraphs in 38.213 clause 17.1A - Continue to discuss potential clarification of “A UE that indicated FG 48-2” in the paragraphs in 38.213 clause 17.1A

This contribution provides views on remaining issues on UE complexity reduction for eRedCap UEs.

UE complexity reduction

Random access timeline

In last RAN1#114bis meeting, we finally achieved the following agreement for CFRA.

Agreement

- The following does not apply to FG 48-2 UEs for CFRA:

  - RAR PDSCH timeline relaxation

However, there was still no consensus on CBRA. From our perspective, if gNB cannot distinguish which type of eRedCap it is, e.g., FG 48-1 or FG 48-2, then the random access procedure would be the same. However, even if the UE type can be early identified or gNB knows the UE capability during CBRA, there may be hard to identify the benefits and necessity of not applying timeline relaxation and bandwidth restriction for FG 48-2 UEs. In addition, it may be too late to have more open discussion on such aspects which may have no performance optimization but much spec work to do. Hence, we propose to have the following proposal.

Proposal 1: No further optimization on timeline relaxation and bandwidth restriction for FG 48-2 UEs.

In last RAN1#114bis meeting, the following agreement on 2-step RACH cases was agreed.

Agreement

For which (if any) of the following 2-step RACH cases, continue to discuss if there is a need to update the specifications to reflect the RAN1 agreement that RAR PDSCH timeline relaxation does not apply to FG 48-2 UEs for CFRA:

- Case 2a: Between reception of fallbackRAR and transmission of Msg3

- Case 2b: Between reception of successRAR and transmission of corresponding HARQ-ACK

- Case 2c: Between reception of MsgB PDSCH scheduled by MSGB-RNTI in which UE does not correctly receive the transport block in the corresponding PDSCH within the window and transmission of only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure.

- Case 2d: Between reception of MsgB PDSCH scheduled by MSGB-RNTI with RAPID which is not associated with the corresponding PRACH transmission from the UE and transmission of only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure.

As discussed in the last meeting [2], the original intention is to optimize the delay. For CFRA, NW already knows the UE type and there may be no need to apply timeline relaxation for FG 48-2 UEs. However, for 2-step RACH, the timeline cases would need further discussion. The listed four cases are enough to have the clarification on 2-step RACH. Hence, we have the following proposal.

Proposal 2: For 2-step RACH, the following cases on timeline relaxation does not apply to FG 48-2 UEs for CFRA:

- Between reception of fallbackRAR and transmission of Msg3

- Between reception of successRAR and transmission of corresponding HARQ-ACK

- Between reception of MsgB PDSCH scheduled by MSGB-RNTI in which UE does not correctly receive the transport block in the corresponding PDSCH within the window and transmission of only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure.

- Between reception of MsgB PDSCH scheduled by MSGB-RNTI with RAPID which is not associated with the corresponding PRACH transmission from the UE and transmission of only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure.

Conclusion

In this contribution, we provide the following proposals:

Proposal 1: No further optimization on timeline relaxation and bandwidth restriction for FG 48-2 UEs.

Proposal 2: For 2-step RACH, the following cases on timeline relaxation does not apply to FG 48-2 UEs for CFRA:

- Between reception of fallbackRAR and transmission of Msg3

- Between reception of successRAR and transmission of corresponding HARQ-ACK

- Between reception of MsgB PDSCH scheduled by MSGB-RNTI in which UE does not correctly receive the transport block in the corresponding PDSCH within the window and transmission of only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure.

- Between reception of MsgB PDSCH scheduled by MSGB-RNTI with RAPID which is not associated with the corresponding PRACH transmission from the UE and transmission of only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure.

- References

Draft_Minutes_report_RAN1#114b_v020, 3GPP TSG RAN1 Meeting#114bis, 9th –13th October 2023.

R1-2310568 FL summary #4 on Rel-18 RedCap UE complexity reduction, 3GPP TSG RAN1 Meeting#114bis, 9th – 13th October 2023.