3GPP TSG RAN WG1 #115			R1-2311614

Chicago, US, November 13th – November 17th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on CSI enhancement

Agenda Item:	8.1.2

Document for: 	Discussion and Decision

- Introduction

In RAN#94-e meeting, a new Rel-18 WID on MIMO [1] was agreed. From 7 objectives, there are two objectives for CSI enhancements. In this contribution, we discuss remaining issues on CSI enhancements for M-TRP CJT for Rel-18 MIMO.

- Discussion

Dropping rule for TDCP reporting

Last meeting, RAN1 reached the following agreement:

Agreement For the Rel-18 TRS-based TDCP reporting, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for each CSI-RS resource for KTRS CSI-RS resource sets configured for TDCP reporting no later than CSI reference resource, otherwise drops the report. - This includes the cases of CSI report (re)configuration, serving cell activation, BWP change - FFS (RAN1#115): Whether DRX configuration needs to be included as a case

The above agreement is reflected in the latest CR for TS 38.214 [2], as follows:

#### 5.2.2.5 CSI reference resource definition […] For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'tdcp', after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each CSI-RS resource in the KTRS CSI-RS Resource Sets of the CSI-RS Resource Setting for channel measurement no later than the CSI reference resource. […]

A remaining issue, as captured in FFS above, is whether/how to consider DRX configuration for TDCP report dropping rule. We believe it should be considered, similar to the other CSI reporting, otherwise reported TDCP information may sometimes be unreliable from NW perspective (e.g., when the report is constructed assuming “not actually measured” TRS occasions:

Proposal 1:

  - For Rel-18 TRS-based TDCP reporting, support to include DRX configuration as a case of dropping

  - Adopt the following TP, on top of R1-2310760 [2]

    - i.e., adding “and within the same DRX Active Time, when DRX is configured, and drops the report otherwise” to dropping rule for TDCP reporting in 38.214 – 5.2.2.5

#### 5.2.2.5 CSI reference resource definition […] For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'tdcp', after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each CSI-RS resource in the KTRS CSI-RS Resource Sets of the CSI-RS Resource Setting for channel measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise. […]

- Reason for change: UE DRX configuration is not considered for determining whether to drop TDCP reporting

- Summary of change: Add “and within the same DRX Active Time, when DRX is configured, and drops the report otherwise” to the text specifying Rel-18 TDCP reporting dropping behavior (on top of R1-2310760)

- Consequences if not approved: Correctness of TDCP reporting may be questionable

- Conclusion

In this contribution, we discussed remaining issues on CSI enhancements for M-TRP CJT for Rel-18 MIMO. Based on the discussion, we made following proposals.

Proposal 1:

  - For Rel-18 TRS-based TDCP reporting, support to include DRX configuration as a case of dropping

  - Adopt the following TP, on top of R1-2310760 [2]

    - i.e., adding “and within the same DRX Active Time, when DRX is configured, and drops the report otherwise” to dropping rule for TDCP reporting in 38.214 – 5.2.2.5

#### 5.2.2.5 CSI reference resource definition […] For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'tdcp', after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each CSI-RS resource in the KTRS CSI-RS Resource Sets of the CSI-RS Resource Setting for channel measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise. […]

- Reason for change: UE DRX configuration is not considered for determining whether to drop TDCP reporting

- Summary of change: Add “and within the same DRX Active Time, when DRX is configured, and drops the report otherwise” to the text specifying Rel-18 TDCP reporting dropping behavior (on top of R1-2310760)

- Consequences if not approved: Correctness of TDCP reporting may be questionable

References

- 3GPP RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94-e, Dec. 2021.

- R1-2310760, “Correction of specification support for MIMO enhancements on CSI”, Nokia, October 2023