3GPP TSG RAN WG1 #115			      R1-2311623

Chicago, USA, November 13th – November 17th, 2023

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on Low Power High Accuracy Positioning

Agenda Item:	8.3.3

Document for: 	Discussion and Decision

- Introduction

At the RAN#98-e meeting, new WID on Rel-18 NR positioning enhancement was agreed [1]. The work item includes objectives related to improved positioning power efficiency as follows:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between eDRX and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we present our views on solutions for Low Power High Accuracy Positioning (LPHAP) for Rel-18 NR positioning.

- Text proposals on larger PRS/SRS periodicities

Regarding the text proposals on larger PRS/SRS periodicities, following proposals were discussed at the last RAN1 meeting [2].

TPs for TS 38.211 Clause 6.4.1.4.4

<Unchanged parts are omitted> 6.4.1.4.4 Sounding reference signal slot configuration For an SRS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, a periodicity (in slots) and slot offset are configured according to the higher-layer parameter periodicityAndOffset-p or periodicityAndOffset-sp in the SRS-Resource IE, or periodicityAndOffset-p or periodicityAndOffset-sp in the SRS-PosResource IE. Candidate slots in which the configured SRS resource may be used for SRS transmission are the slots satisfying NframeHFNslotframe,μnHF+Nslotframe,μnf+ns,fμ-Toffsetmod TSRS=0 where NframeHF is the number of frames in a hyper frame, nHF is the hyper frame index, which are present if TSRS larger than 2μ10240 are configured. SRS is transmitted as described in clause 11.1 of [5, TS 38.213]. <Unchanged parts are omitted>

TPs for TS 38.211 Clause 7.4.1.7.4

| <Unchanged parts are omitted> 7.4.1.7.4 Mapping to slots in a downlink PRS resource set For a downlink PRS resource in a downlink PRS resource set, the UE shall assume the downlink PRS resource being transmitted when the slot and frame numbers fulfil NframeHFNslotframe,μnHF+Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod TperPRS∈iTgapPRSi=0TrepPRS-1 and one of the following conditions are fulfilled: - the higher-layer parameters dl-PRS-MutingOption1 and dl-PRS-MutingOption2 are not provided; - the higher-layer parameter dl-PRS-MutingOption1 is provided with bitmap b1 but dl-PRS-MutingOption2 with bitmap b2 is not provided, and bit bi1 is set; - the higher-layer parameter dl-PRS-MutingOption2 is provided with bitmap b2 but dl-PRS-MutingOption1 with bitmap b1 is not provided, and bit bi2 is set; - the higher-layer parameters dl-PRS-MutingOption1 with bitmap b1 and dl-PRS-MutingOption2 with b2 are both provided, and both bit bi1 and bi2 are set. where - bi1 is bit i=NframeHFNslotframe,μnHF+Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS TmutingPRSTperPRSmod L in the bitmap given by the higher-layer parameter dl-PRS-MutingOption1 where L∈2, 4, 6, 8, 16, 32 is the size of the bitmap; - bi2 is bit i=NframeHFNslotframe,μnHF+Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod TperPRS TgapPRS mod TrepPRS in the bitmap given by the higher-layer parameter dl-PRS-MutingOption2; - the periodicity TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 and the slot offset ToffsetPRS∈0,1,…,TperPRS-1 are given by the higher-layer parameter dl-PRS-Periodicity-and-ResourceSetSlotOffset; - the downlink PRS resource slot offset Toffset,resPRS is given by the higher-layer parameter dl-PRS-ResourceSlotOffset; - the repetition factor TrepPRS∈1,2,4,6,8,16,32 is given by the higher-layer parameter dl-PRS-ResourceRepetitionFactor; - the muting repetition factor TmutingPRS is given by the higher-layer parameter dl-PRS-MutingBitRepetitionFactor; - the time gap TgapPRS∈1,2,4,8,16,32 is given by the higher-layer parameter dl-PRS-ResourceTimeGap; - NframeHF is the number of frames in a hyper frame, nHF is the hyper frame index, which are present if TperPRS larger than 2μ10240 are configured by [dl-PRS-Periodicity-and-ResourceSetSlotOffset]. For a downlink PRS resource in a downlink PRS resource set configured for RTT-based propagation delay compensation, the UE shall assume the downlink PRS resource being transmitted as described in clause 9 of [6, TS 38.214]; otherwise, the UE shall assume the downlink PRS resource being transmitted as described in clause 5.1.6.5 of [6, TS 38.214]. <Unchanged parts are omitted> |  |
| --- | --- |

TP for TS 38.214 Clause 5.1.6.5

<Unchanged parts are omitted> A DL PRS resource set is configured by NR-DL-PRS-ResourceSet, consists of one or more DL PRS resources and it is defined by: - nr-DL-PRS-ResourceSetID defines the identity of the DL PRS resource set configuration. - dl-PRS-Periodicity-and-ResourceSetSlotOffset defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, 20480, xxx slots, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively and the slot offset for DL PRS resource set with respect to SFN0 slot 0 of hyper SFN 0. All the DL PRS resources within one DL PRS resource set are configured with the same DL PRS resource periodicity. The UE does not expect that the product of DL PRS resource periodicity TperPRS, the higher layer parameter dl-prs-MutingBitRepetitionFactor and the size of the bitmap of dl-PRS-MutingOption1 exceeds 2μ×10240, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively. <Unchanged parts are omitted>

RAN1 agreed to introduce the larger PRS/SRS periodicities than 10240 ms. However, at the last meeting RAN1 decided to wait for RAN2’s final decision before proceeding above proposals. The reply LS from RAN2 is as follows [3].

# 1 Overall description RAN2 would like to thank RAN1 for informing the agreements related to the extension of the value for the PRS/SRS periodicities. RAN2 do not have enough information to define the signalling at this point. It is RAN2’s understanding that the new values of periodicities for PRS and SRS will be provided to RAN2 by RAN1 in the L1 parameter list in the usual way. # 2 Actions To RAN1: ACTION: RAN2 respectfully asks RAN1 to take the above information in the future work.

In our view, we still believe that RAN2’s final decision is necessary. However, if RAN2 could not determine the additional values, RAN1 may need to discuss it. According to the use case #6 in TS 22.104, positioning interval/duty cycle is 15 s to 30 s. Since PRS/SRS periodicities should be within this requirement, 20480 ms cycle should be included.

Proposal 1:

- Following text proposals should be adopted when RAN2 made the final decision or RAN1 decided to determine the periodicity candidate values in RAN1.

TPs for TS 38.211 Clause 6.4.1.4.4

<Unchanged parts are omitted> 6.4.1.4.4 Sounding reference signal slot configuration For an SRS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, a periodicity (in slots) and slot offset are configured according to the higher-layer parameter periodicityAndOffset-p or periodicityAndOffset-sp in the SRS-Resource IE, or periodicityAndOffset-p or periodicityAndOffset-sp in the SRS-PosResource IE. Candidate slots in which the configured SRS resource may be used for SRS transmission are the slots satisfying NframeHFNslotframe,μnHF+Nslotframe,μnf+ns,fμ-Toffsetmod TSRS=0 where NframeHF is the number of frames in a hyper frame, nHF is the hyper frame index, which are present if TSRS larger than 2μ10240 are configured. SRS is transmitted as described in clause 11.1 of [5, TS 38.213]. <Unchanged parts are omitted>

TPs for TS 38.211 Clause 7.4.1.7.4

| <Unchanged parts are omitted> 7.4.1.7.4 Mapping to slots in a downlink PRS resource set For a downlink PRS resource in a downlink PRS resource set, the UE shall assume the downlink PRS resource being transmitted when the slot and frame numbers fulfil NframeHFNslotframe,μnHF+Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod TperPRS∈iTgapPRSi=0TrepPRS-1 and one of the following conditions are fulfilled: - the higher-layer parameters dl-PRS-MutingOption1 and dl-PRS-MutingOption2 are not provided; - the higher-layer parameter dl-PRS-MutingOption1 is provided with bitmap b1 but dl-PRS-MutingOption2 with bitmap b2 is not provided, and bit bi1 is set; - the higher-layer parameter dl-PRS-MutingOption2 is provided with bitmap b2 but dl-PRS-MutingOption1 with bitmap b1 is not provided, and bit bi2 is set; - the higher-layer parameters dl-PRS-MutingOption1 with bitmap b1 and dl-PRS-MutingOption2 with b2 are both provided, and both bit bi1 and bi2 are set. where - bi1 is bit i=NframeHFNslotframe,μnHF+Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS TmutingPRSTperPRSmod L in the bitmap given by the higher-layer parameter dl-PRS-MutingOption1 where L∈2, 4, 6, 8, 16, 32 is the size of the bitmap; - bi2 is bit i=NframeHFNslotframe,μnHF+Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod TperPRS TgapPRS mod TrepPRS in the bitmap given by the higher-layer parameter dl-PRS-MutingOption2; - the periodicity TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 and the slot offset ToffsetPRS∈0,1,…,TperPRS-1 are given by the higher-layer parameter dl-PRS-Periodicity-and-ResourceSetSlotOffset; - the downlink PRS resource slot offset Toffset,resPRS is given by the higher-layer parameter dl-PRS-ResourceSlotOffset; - the repetition factor TrepPRS∈1,2,4,6,8,16,32 is given by the higher-layer parameter dl-PRS-ResourceRepetitionFactor; - the muting repetition factor TmutingPRS is given by the higher-layer parameter dl-PRS-MutingBitRepetitionFactor; - the time gap TgapPRS∈1,2,4,8,16,32 is given by the higher-layer parameter dl-PRS-ResourceTimeGap; - NframeHF is the number of frames in a hyper frame, nHF is the hyper frame index, which are present if TperPRS larger than 2μ10240 are configured by [dl-PRS-Periodicity-and-ResourceSetSlotOffset]. For a downlink PRS resource in a downlink PRS resource set configured for RTT-based propagation delay compensation, the UE shall assume the downlink PRS resource being transmitted as described in clause 9 of [6, TS 38.214]; otherwise, the UE shall assume the downlink PRS resource being transmitted as described in clause 5.1.6.5 of [6, TS 38.214]. <Unchanged parts are omitted> |  |
| --- | --- |

TP for TS 38.214 Clause 5.1.6.5

<Unchanged parts are omitted> A DL PRS resource set is configured by NR-DL-PRS-ResourceSet, consists of one or more DL PRS resources and it is defined by: - nr-DL-PRS-ResourceSetID defines the identity of the DL PRS resource set configuration. - dl-PRS-Periodicity-and-ResourceSetSlotOffset defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, 20480, xxx slots, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively and the slot offset for DL PRS resource set with respect to SFN0 slot 0 of hyper SFN 0. All the DL PRS resources within one DL PRS resource set are configured with the same DL PRS resource periodicity. The UE does not expect that the product of DL PRS resource periodicity TperPRS, the higher layer parameter dl-prs-MutingBitRepetitionFactor and the size of the bitmap of dl-PRS-MutingOption1 exceeds 2μ×10240, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively. <Unchanged parts are omitted>

- Conclusion

In this contribution, we discussed remaining issues for LPHAP for Rel-18 NR positioning. Based on the discussion, we made following proposal.

Proposal 1:

- Following text proposals should be adopted when RAN2 made the final decision or RAN1 decided to determine the periodicity candidate values in RAN1.

TPs for TS 38.211 Clause 6.4.1.4.4

<Unchanged parts are omitted> 6.4.1.4.4 Sounding reference signal slot configuration For an SRS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, a periodicity (in slots) and slot offset are configured according to the higher-layer parameter periodicityAndOffset-p or periodicityAndOffset-sp in the SRS-Resource IE, or periodicityAndOffset-p or periodicityAndOffset-sp in the SRS-PosResource IE. Candidate slots in which the configured SRS resource may be used for SRS transmission are the slots satisfying NframeHFNslotframe,μnHF+Nslotframe,μnf+ns,fμ-Toffsetmod TSRS=0 where NframeHF is the number of frames in a hyper frame, nHF is the hyper frame index, which are present if TSRS larger than 2μ10240 are configured. SRS is transmitted as described in clause 11.1 of [5, TS 38.213]. <Unchanged parts are omitted>

TPs for TS 38.211 Clause 7.4.1.7.4

| <Unchanged parts are omitted> 7.4.1.7.4 Mapping to slots in a downlink PRS resource set For a downlink PRS resource in a downlink PRS resource set, the UE shall assume the downlink PRS resource being transmitted when the slot and frame numbers fulfil NframeHFNslotframe,μnHF+Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod TperPRS∈iTgapPRSi=0TrepPRS-1 and one of the following conditions are fulfilled: - the higher-layer parameters dl-PRS-MutingOption1 and dl-PRS-MutingOption2 are not provided; - the higher-layer parameter dl-PRS-MutingOption1 is provided with bitmap b1 but dl-PRS-MutingOption2 with bitmap b2 is not provided, and bit bi1 is set; - the higher-layer parameter dl-PRS-MutingOption2 is provided with bitmap b2 but dl-PRS-MutingOption1 with bitmap b1 is not provided, and bit bi2 is set; - the higher-layer parameters dl-PRS-MutingOption1 with bitmap b1 and dl-PRS-MutingOption2 with b2 are both provided, and both bit bi1 and bi2 are set. where - bi1 is bit i=NframeHFNslotframe,μnHF+Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS TmutingPRSTperPRSmod L in the bitmap given by the higher-layer parameter dl-PRS-MutingOption1 where L∈2, 4, 6, 8, 16, 32 is the size of the bitmap; - bi2 is bit i=NframeHFNslotframe,μnHF+Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod TperPRS TgapPRS mod TrepPRS in the bitmap given by the higher-layer parameter dl-PRS-MutingOption2; - the periodicity TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 and the slot offset ToffsetPRS∈0,1,…,TperPRS-1 are given by the higher-layer parameter dl-PRS-Periodicity-and-ResourceSetSlotOffset; - the downlink PRS resource slot offset Toffset,resPRS is given by the higher-layer parameter dl-PRS-ResourceSlotOffset; - the repetition factor TrepPRS∈1,2,4,6,8,16,32 is given by the higher-layer parameter dl-PRS-ResourceRepetitionFactor; - the muting repetition factor TmutingPRS is given by the higher-layer parameter dl-PRS-MutingBitRepetitionFactor; - the time gap TgapPRS∈1,2,4,8,16,32 is given by the higher-layer parameter dl-PRS-ResourceTimeGap; - NframeHF is the number of frames in a hyper frame, nHF is the hyper frame index, which are present if TperPRS larger than 2μ10240 are configured by [dl-PRS-Periodicity-and-ResourceSetSlotOffset]. For a downlink PRS resource in a downlink PRS resource set configured for RTT-based propagation delay compensation, the UE shall assume the downlink PRS resource being transmitted as described in clause 9 of [6, TS 38.214]; otherwise, the UE shall assume the downlink PRS resource being transmitted as described in clause 5.1.6.5 of [6, TS 38.214]. <Unchanged parts are omitted> |  |
| --- | --- |

TP for TS 38.214 Clause 5.1.6.5

<Unchanged parts are omitted> A DL PRS resource set is configured by NR-DL-PRS-ResourceSet, consists of one or more DL PRS resources and it is defined by: - nr-DL-PRS-ResourceSetID defines the identity of the DL PRS resource set configuration. - dl-PRS-Periodicity-and-ResourceSetSlotOffset defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, 20480, xxx slots, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively and the slot offset for DL PRS resource set with respect to SFN0 slot 0 of hyper SFN 0. All the DL PRS resources within one DL PRS resource set are configured with the same DL PRS resource periodicity. The UE does not expect that the product of DL PRS resource periodicity TperPRS, the higher layer parameter dl-prs-MutingBitRepetitionFactor and the size of the bitmap of dl-PRS-MutingOption1 exceeds 2μ×10240, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively. <Unchanged parts are omitted>

References

- RP-223549, 	3GPP RAN#98-e, “New WID on Expanded and Improved NR Positioning”, Electronic Meeting, Dec. 2022.

- R1-2310318, 	3GPP RAN1#114bis, Summary #1 for low power high accuracy positioning, Oct. 2023.

- R1-2310785,	LS on extended PRS and SRS periodicity, Huawei