3GPP TSG-RAN WG1 Meeting #115	R1- 2312310

# Chicago, USA, November 13th – November 17th, 2023

Agenda Item:	7.2

Source:	Moderator (Ericsson)

Title:	Summary #1 of discusssion on scaling of angles for CDL models

Document for:	Discussion, Decision

# Introduction

## 1.1 Issue 1: scaling of angles for CDL models

In section 7.7.5.1 of TR 38.901 the following equation is given for scaling of CDL path angles

ϕn,scaled=ASdesiredASmodel∙ϕn,model-μϕ,model+μϕ,desired

The intention is to scale the path angles around the angular mean to achieve the desired angular spread and then rotate the path angles to achieve the desired angular mean as illustrated in Figure 1.

Figure 1 Illustration of scaling and rotation of path angles to achieve the desired angular spread and mean.

Issues related to angular scaling of CDL models were discussed in RAN1#114bis meeting on following perspectives:

- Wrapping on Azimuth angle

- Desired angle mean

- Desired angle spread

Contributions [1]-[6] for RAN1#115 provide different views on how to resolve those issues with following observations and proposals:

| Company | Conclusions |
| --- | --- |
| Ericsson [1] | Adopt draft Rel-17 CR to 38.901 Section 7.7.5.1 in [1] The translated and scaled ray angles can be obtained according to the following equation: ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired ϕn,intermediate=ASdesiredASmodel∙WrapTo180ϕn,model-μϕ,model (7.7-5) (7.7-5) in which: is the tabulated CDL ray angle ϕn,intermediate is the intermediate ray angle calculated according to equation 7.7-5 is the rms angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A is the mean angle of the tabulated CDL, calculated using the definition in Annex A μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A (dependent on Phy interdemiate) is the desired mean angle is the desired rms angular spread is the resulting scaled ray angle. WrapTo180 is a function wrapping angles to the interval [-180, 180] |
| Nokia [2] | Proposal 1: Correct for potential non-linear distortions from angle domain boundaries created by phase wrapping using either the correction provided in [1], or indicating clearly that angular spread trannsformation must be done using unwrapped angles as input. Proposal 2:Correct for potential mean variance of angular spread transformation using the correction proposed in [1]. Proposal 3:Further consider the tradeoff between model precision and computation complexity before addressing the issue of mismatch between observed and desired CDL angular spread . |
| Intel [3] | The translated and scaled ray angles can be obtained according to the following equation: ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired, (7.7-5) where ϕn,intermediate=ScalingFactor(ASdesired)∙Angleϕn,model-μϕ,model, and Anglex=x,if x is a zenith angleWrapTo180x,if x is an azimuth angle. ScalingFactorASdesired=minx≥0{x: ASx=ASdesired}. ASx=-2lnn=1Nm=1Mexpjxϕn,mPn,mn=1Nm=1MPn,m ϕn,scaled=ASdesiredASmodel∙ϕn,model-μϕ,model+μϕ,desired (7.7-5) in which: ϕn,model is the tabulated CDL ray angle ASmodel is the rms angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A μϕ,model is the mean angle of the tabulated CDL, calculated using the definition in Annex A μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A μϕ,desired is the desired mean angle ASdesired is the desired rms angular spread ϕn,scaled is the resulting scaled ray angle. WrapTo180(⋅) is a function which wraps an azimuth angle to the interval [-180, 180]. The angular scaling is applied on the ray angles including offsets from the tabulated cluster angles. Typical angular spreads for different scenarios can be obtained from the system-level model. Example scaling values are: - AOD spread (ASD) for each CDL model: {5, 10, 15, 25} degrees. - AOA spread (ASA) for each CDL model: {30, 45, 60} degrees. - ZOA spread (ZSA) for each CDL model: {5, 10, 15} degrees. - ZOD spread (ZSD) for each CDL model: {1, 3, 5} degrees. The angular scaling and translation can be applied to some or all of the azimuth and zenith angles of departure and arrival. Note 1: The azimuth angles may need to be wrapped around to be within [0, 360] [-180, 180] degrees, while the zenith angles may need to be clipped to be within [0, 180] degrees. Note 2: For each of AOA, AOD, ZOA, and ZOD, not all desired AS values, ASdesired, larger than the model AS value, ASmodel, may be achievable by equation (7.7-5). |
| ZTE [4] | Observation 1: The issue caused by the azimuth angle discontinuity at 180° and -180° can be fixed by wrapping azimuth angles to [0, 360] degree. Observation 2: In the commonly used range of angular spread in 3GPP simulation work, the error of angular spread and mean angle caused by the linear scaling is small enough to ignore. Proposal 1: The modification to angle scaling formula in TR38.901 is unnecessary. Proposal 2: If accurate angular spread and mean angle are needed, adopt draft Rel-17 CR in appendix A to TR38.901 Section 7.7.5.1. The translated and scaled ray angles can be obtained according to the following equation: ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired, (7.7-5) where ϕn,intermediate=ScalingFactor(ASdesired)∙Angleϕn,model-μϕ,model, and Anglex=x,if x is a zenith angleWrapTo180x,if x is an azimuth angle. ScalingFactorASdesired=minx≥0{x: ASx=ASdesired}. ASx=-2lnn=1Nm=1Mexpjx(ϕn,m+θn,m)Pn,mn=1Nm=1MPn,m θn,m=WrapTo180ϕn,model-μϕ,model-ϕn,model-μϕ,model (7.7-5) in which: ϕn,model is the tabulated CDL ray angle ASmodel is the rms angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A μϕ,model is the mean angle of the tabulated CDL, calculated using the definition in Annex A μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A μϕ,desired is the desired mean angle ASdesired is the desired rms angular spread ϕn,scaled is the resulting scaled ray angle WrapTo180(⋅) is a function which wraps an azimuth angle to the interval [-180, 180]. |
| Huawei, HiSilicon [5] | Proposal 1: The angles for CDL channel is scaled as below ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired, (7.7-5) where ϕn,intermediate=ScalingFactor(ASdesired)∙Angleϕn,model-μϕ,model, and Anglex=x,if x is a zenith angleWrapTo180x,if x is an azimuth angle. ScalingFactorASdesired=minx≥0{x: ASx=ASdesired}. ASx=-2lnn=1Nm=1Mexpjx*Angle(ϕn,m-μϕ,model)Pn,mn=1Nm=1MPn,m. |
| Samsung [6] | Proposal 1. No need to introduce wrapping around [-180,180] for the term “ϕn,model-μϕ,model” for azimuth angle. Proposal 2. If proposal 1 is not enough, we may introduce a note as “Assuming that “ϕn,model-μϕ,model” is wrapped within [-180, 180], if (ϕn,model-μϕ,model)∈[180°, 360°], then (ϕn,model-μϕ,model) is set to {360 - (ϕn,model-μϕ,model)} else if, (ϕn,model-μϕ,model)∈[-180°, -360°], then (ϕn,model-μϕ,model) is set to {-360 - (ϕn,model-μϕ,model)}. Proposal 3. Since the current scaling operation is not designed to get exact expected scaling, keep the current scaling operation as is. Proposal 4. If an update for scaling part is needed, we may add a note prohibiting large value of scaling factor, or reopen this issue in Rel-19 channel model study phase. |

## 1.2 Issue 2: correction of an erroneous reference to a figure

## 7.5	Fast fading model

< Unchanged parts are omitted >

Step 1: Set environment, network layout, and antenna array parameters

a)	Choose one of the scenarios (e.g. UMa, UMi-Street Canyon, RMa, InH-Office or InF). Choose a global coordinate system and define zenith angle θ, azimuth angle ϕ, and spherical basis vectors ,  as shown in Figure 7.5-2. Note: Scenario RMa is for up to 7GHz while others are for up to 100GHz

b)	Give number of BS and UT

c)	Give 3D locations of BS and UT, and determine LOS AOD (ϕLOS,AOD), LOS ZOD (θLOS,ZOD), LOS AOA (ϕLOS,AOA), and LOS ZOA (θLOS,ZOA) of each BS and UT in the global coordinate system

d)	Give BS and UT antenna field patterns Frx and Ftx in the global coordinate system and array geometries

e)	Give BS and UT array orientations with respect to the global coordinate system. BS array orientation is defined by three angles ΩBS,α (BS bearing angle), ΩBS,β (BS downtilt angle) and ΩBS,γ (BS slant angle). UT array orientation is defined by three angles ΩUT,α (UT bearing angle), ΩUT,β (UT downtilt angle) and ΩUT,γ (UT slant angle).

f)	Give speed and direction of motion of UT in the global coordinate system

g)	Specify system centre frequency  and bandwidth

Note:	In case wrapping is used, each wrapping copy of a BS or site should be treated as a separate BS/site considering channel generation.

< Unchanged parts are omitted >

In draftCR [7], an erroneous reference to a figure in 38.901 Clause 7.5 was identified.

This document is provided to discuss whether/how to resolve these issues in RAN1#115.

# 2	Discussion Round 1

## 2.1 Issue 1: scaling of angles for CDL models

In first round of discussion I’d like to start asking questions to get a high level unstanding of company’s position instead of diving deeply into the problem and open all the issues.

My plan is to first discuss if clarification on the incontinuity caused by wrapping boundary of azimuth angles is needed. If Yes, which way of clarification is preferred. This is the minimum step one can do to align the understanding of this issue.

Second step of discussion includes a few high level questions in oder to align understanding on the accuracy expectation of scaling of angles in 38.901 to achieve desired angle mean and angle spread.

In [1][2] states the error of angle mean with current formulation is more critical than error of angle spread and hence need to be fixed. In [4],”Table 1 Maximum error of AOA spread and mean angle for CDL-A, CDL-B and CDL-C channel”, shows the simulation results that maximum error of angle mean is much larger than the maximum error of angle spread. If RAN1 reach consensus on only modify the angle mean to achieve, the modification proposed in [1] can be a good starting point.

Second round of discussion would be dependent on the outcome of Round 1. In order to further discuss how to update the angle scaling formulation to reach desired angle spread, we need Round 1 consensus that a precise angle spread after the scaling is required. Note that it is pointed out in [6] the current scaling option is not designed to get exact expected scaling, that RAN1 shall keep the scaling operation and if needed add a note to prohibit large value of scaling factor.

Here are the Round 1 questions for you to answer.

Q1: For the issue on angle scaling for the CDL models with angle discontinuity at angle wrapping boundary, do you acknowledge current description using following formulation in 38.901 can be missguided and gives error result?

ϕn,scaled=ASdesiredASmodelϕn,model-μϕ,model+μϕ,desired

| Company | Comments |
| --- | --- |
| Ericsson | Yes. This is issue raised by two of Ericsson colleagues who noticed strange behavior in different simulations and found problem in this part of specs when they implemented the scaling according to the formular. |
| Intel | Yes. |
| ZTE | It’s related to the implementation of the equation to whether consider the wrap around operation. |
| Nokia | Agree that the current formulation is ambiguous. |
| Samsung | We do not think that the current formula causes a problem. It is a common understanding that ϕn,model-μϕ,model is within the range of [-180, 180] degrees. This can be seen in the note added to the spec as below. Note: The azimuth angles may need to be wrapped around to be within [0, 360] degrees, while the zenith angles may need to be clipped to be within [0, 180] degrees. When calculating azimuth and zenith angles, the wrapping operation is already taken into consideration. Therefore, the current specification is already sufficient. |

Q2: Are you OK to fix the issue on angle scaling for CDL models with angle discontinueity at angle wrapping boundary?

If your answer is YES, which option is your preference?

- Option 1: Add wrapTo180 in the formula as is suggested in [1].

- Option 2: Clarify it with text as is suggested in [4] and [5].

If your answer is NO, please elaborate your view.

| Company | Comments |
| --- | --- |
| Ericsson | We prefer Option 1. Because we think it is simple and clear to modify the formula with adding wrapTo180 than to clarify the operation with text. With the modification in Option 1 one can make sure same angle scaling results can be achieved. We can be open to clarify it with text if the updated text is simple and clear. |
| Intel | Option 1. The issue with option 2 is that if we assume all individual angles are wrapped around or clipped depending on azimuth or zenith angles, then it may lead to different results. An example of the issue with option 2 is provided in R1-2311537. |
| ZTE | Option-2, We prefer to go with Option-2. |
| Nokia | Slight preference to option 2. In our view, the issue is that we are performing a linear operation (angular scaling) in conjunction with a non-linear operation (angle wrapping). In this case, it should be made clear that the angle scaling operation is applied on the unwrapped angles. As the results in [3] indicate, the angular scaling operation can not work for all scaling values as intended over the wrapped angular domain. |
| Samsung | As we mentioned above, we think the current spec is clear so that there is no issue to fix. |

Q3: To achieve desired angle mean after scaling, do you acknowledge current formulation can produce undesired result? Are you OK to fix the issue for angle mean for Rel-17 spec?

| Company | Comments |
| --- | --- |
| Ericsson | We prefer to fix the angle mean for Rel-17 first. Achieve desired angle mean is more important than achieve desired angle spread for each angle. |
| Intel | Yes. We should fix the angle mean issue. |
| ZTE | No. For the impacts after scaling, as shown in the results (figure 1 ) in our contribution R1-2311196, the mean value is quite close to the desired value. Then, no change is also acceptable. |
| Nokia | Ok to support. |
| Samsung | The current formula can produce a different value from the desired one. However, unless a large scaling factor is used – which is not a typical use case – the value obtained from the current formula is quite close to the desired value. Furthermore, the purpose of the scaling is not to calculate scaled ray angles accurately. This scaling is intended to facilitate the use of various PMI values across simulations. Thus, updating the formula for results with high precision is unnecessary so that we do not support this. |

Q4: To achieve desired angle spread after scaling, do you acknowledge current formula can lead to undesired result? Are you OK to fix the issue for angle spread for Rel-17 spec?

| Company | Comments |
| --- | --- |
| Ericsson | We are aware that the linear scaling is not optimum for angle spread. If RAN1 want to achieve precise and accurate scaling, we are open for discussion. To resolve this problem we may need further study on the solutions. |
| Intel | Yes. We should fix the angular spread issue. |
| ZTE | As shown in the results (figure 1 ) in our contribution R1-2311196, the final results is closed to the desired value according to to the typical spread value. |
| Nokia | If the text makes it clear that angle scaling is performed on unwrapped angles, then the results should be as intended. The wrapping operation ensures that there will be a limit to how far the angular spread can be scaled. A correction to clarify how the angular spread should be scaled is preferred. Otherwise, a simple operation to avoid the unintended results may be agreeable, provided the proposed solution does not significantly increase computation. |
| Samsung | Same as above. |

## 2.2 Issue 2: Correction of reference to a figure

Q1: Are you OK to adopt the draftCR in [7]? If not, please elaborate your view.

| Company | Comments |
| --- | --- |
| Ericsson | This CR is proposed by us. We are OK to adopt it. |
| Intel | Generally ok. It should be noted that this CR does not fix the angular spread issue. Additionally, the note in [11] R1-2311538 should be adopted as well. Our preference to adopt the draft CR R1-2311538 which should include all changes in [7]. |
| Nokia | Ok to support. |
| Samsung | We are okay to correct the reference to the figure. |

## 2.3 Moderator Summary of Round 1

For Issue 1, it seems adding a note to clarify the angle wrapping is more preferred than making modification on the formular. For the corrections for angle mean and angle spread, as there’s strong concern on making any change on the formular, we will not try to make any change for Rel-17.

For the Round 2 discussion of Issue 1 let’s focus on how to address the issue on angle scaling for CDL models with angle discontinueity at angle wrapping boundary.

For Issue 2, all companies are OK to adopt the draftCR in [7] to correct the reference.

- Adopt the draft CR to 38.901 in R1-2310946

## 7.5	Fast fading model

< Unchanged parts are omitted >

Step 1: Set environment, network layout, and antenna array parameters

a)	Choose one of the scenarios (e.g. UMa, UMi-Street Canyon, RMa, InH-Office or InF). Choose a global coordinate system and define zenith angle θ, azimuth angle ϕ, and spherical basis vectors ,  as shown in Figure 7.5-2. Note: Scenario RMa is for up to 7GHz while others are for up to 100GHz

b)	Give number of BS and UT

c)	Give 3D locations of BS and UT, and determine LOS AOD (ϕLOS,AOD), LOS ZOD (θLOS,ZOD), LOS AOA (ϕLOS,AOA), and LOS ZOA (θLOS,ZOA) of each BS and UT in the global coordinate system

d)	Give BS and UT antenna field patterns Frx and Ftx in the global coordinate system and array geometries

e)	Give BS and UT array orientations with respect to the global coordinate system. BS array orientation is defined by three angles ΩBS,α (BS bearing angle), ΩBS,β (BS downtilt angle) and ΩBS,γ (BS slant angle). UT array orientation is defined by three angles ΩUT,α (UT bearing angle), ΩUT,β (UT downtilt angle) and ΩUT,γ (UT slant angle).

f)	Give speed and direction of motion of UT in the global coordinate system

g)	Specify system centre frequency  and bandwidth

Note:	In case wrapping is used, each wrapping copy of a BS or site should be treated as a separate BS/site considering channel generation.

< Unchanged parts are omitted >

# 3	Discussion Round 2

# Conclusion

In this paper we propose the following:

# References

R1-2310910, “Discussion on scaling of angles for CDL models”, Ericsson, RAN1#115, November 2023.

R1-2312160, “Discussion on Issues with Scaling of Angles with TR 38.901 CDL Channel Model”, Nokia, Nokia Shanghai Bell, RAN1#115, November 2023

R1-2311537, “Discussion on angle scaling procedure for CDL channel in TR38.901”, Intel Corporation, RAN1#115, November 2023

R1-2311196, “Discussion on angle scaling for CDL model”, ZTE, RAN1#115, November 2023

R1-2312241, “Discussion on scaling of angles for CDL channel model in TR38.901”, Huawei, HiSilicon, RAN1#115, November 2023

R1-2311824, “Discussion on angle scaling for CDL channel in TR38.901”, Samsung, RAN1#115, November 2023

R1-2310946, “Draft CR correcting reference to a figure in 38.901”, Ericsson, RAN1#115, November 2023

R1-2310945, “Draft CR on scaling of angles for CDL models”, Ericsson, RAN1#115, November 2023.

R1-2310339, “Discussion on angle scaling procedure for CDL channel in TR38.901”, Intel Corporation, RAN1#114bis, October 2023

R1-2309213, “Discussion on scaling of angles for CDL models”, Ericsson, RAN1#114bis, October 2023.

R1-2311538, “Draft Correction of angle scaling procedure for CDL channel in TR38.901”,	Intel Corporation, RAN1#115, November 2023

# Appendix A – Properties of angular spread and mean

In appendix A1 and A2 in TR 38.901 the angular spread (AS) and the power weighted mean angle μϕ are defined as

AS=-2lnn=1Nm=1Mexpjϕn.mPn,mn=1Nm=1MPn,m

μϕ=argn=1Nm=1Mexpjϕn,mPn,m

where Pn,m is the power for the mth subpath of the nth path and ϕn.m is the subpaths angle (either AOA, AOD, ZOA, ZOD) given in radians.

One may note that a rotation of the subpath angles

ϕ'n.m=ϕn.m+Δϕ

leaves the angular spread unchanged

AS'=-2lnn=1Nm=1Mexpjϕ'n.mPn,mn=1Nm=1MPn,m=-2lnn=1Nm=1Mexpjϕn.m+jΔϕPn,mn=1Nm=1MPn,m=-2lnexpjΔϕn=1Nm=1Mexpjϕn.mPn,mn=1Nm=1MPn,m=-2lnn=1Nm=1Mexpjϕn.mPn,mn=1Nm=1MPn,m=AS

while the mean angle is rotated with the same angle Δϕ

μϕ'=argn=1Nm=1Mexpjϕ'n,mPn,m=argn=1Nm=1Mexpjϕn,m+jΔϕPn,m=argexpjΔϕn=1Nm=1Mexpjϕn,mPn,m=Δϕ+argn=1Nm=1Mexpjϕn,mPn,m=μϕ+Δϕ

Scaling of subpath angles around the mean angle, doesn’t however in general leave the angular mean unchanged. Consequently, scaling with a factor s around the mean angle, followed by a translation with a desired mean angle μdesired

ϕ'n.m=s∙ϕn.m-μϕ+μdesired

doesn’t in general give the desired mean angle. Consider the following counter example with two rays

ϕ1=5°

P1=0.7969

ϕ2=-20°

P2=1-P1=0.2031

With

μϕ=0

Scale the angles around the mean with a factor s=5 and translate with a desired mean angle μdesired

ϕ'n=5∙WrapTo180ϕn-μϕ+μdesired

This gives

μϕ'=μdesired+11.3°

In order to get the desired aungular spread and the desired angular mean one has to perform the scaling and the rotation as two separate steps.

ϕn,intermediate=ASdesiredASmodel∙WrapTo180ϕn,model-μϕ,model

ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired

# Appendix B – Draft CR to 38.901 Section 7.7.5.1 [1]

*** Unchanged text omitted ***

#### 7.7.5.1	CDL extension: Scaling of angles

The angle values of CDL models are fixed, which is not very suitable for MIMO simulations for several reasons; The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle can be changed to  and angular spread can be changed by scaling. The translated and scaled ray angles can be obtained according to the following equation:

ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired                   ϕn,intermediate=ASdesiredASmodel∙WrapTo180ϕn,model-μϕ,model                         (7.7-5)

(7.7-5)

in which:

is the tabulated CDL ray angle

ϕn,intermediate is the intermediate ray angle calculated according to equation 7.7-5

is the rms angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A

is the mean angle of the tabulated CDL, calculated using the definition in Annex A

μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A (dependent on Phy interdemiate)

is the desired mean angle

is the desired rms angular spread

is the resulting scaled ray angle.

WrapTo180 is a function wrapping angles to the interval [-180, 180]

The angular scaling is applied on the ray angles including offsets from the tabulated cluster angles. Typical angular spreads for different scenarios can be obtained from the system-level model.

Example scaling values are:

-	AOD spread (ASD) for each CDL model: {5, 10, 15, 25} degrees.

-	AOA spread (ASA) for each CDL model: {30, 45, 60} degrees.

-	ZOA spread (ZSA) for each CDL model: {5, 10, 15} degrees.

-	ZOD spread (ZSD) for each CDL model: {1, 3, 5} degrees.

The angular scaling and translation can be applied to some or all of the azimuth and zenith angles of departure and arrival.

Note: The azimuth angles may need to be wrapped around to be within [0, 360] degrees, while the zenith angles may need to be clipped to be within [0, 180] degrees.

*** Unchanged text omitted ***

# Appendix C - Draft CR to 38.901 Section 7.7.5.1[3]

*** Unchanged text omitted ***

#### 7.7.5.1	CDL extension: Scaling of angles

The angle values of CDL models are fixed, which is not very suitable for MIMO simulations for several reasons; The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle can be changed to μϕ,desired and angular spread can be changed by scaling. The translated and scaled ray angles can be obtained according to the following equation:

ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired,                  (7.7-5)

where

ϕn,intermediate=ScalingFactor(ASdesired)∙Angleϕn,model-μϕ,model,

and

Anglex=x,if x is a zenith angleWrapTo180x,if x is an azimuth angle.

ScalingFactorASdesired=minx≥0{x: ASx=ASdesired}.

ASx=-2lnn=1Nm=1Mexpjxϕn,mPn,mn=1Nm=1MPn,m

ϕn,scaled=ASdesiredASmodel∙ϕn,model-μϕ,model+μϕ,desired	(7.7-5)

in which:

ϕn,model is the tabulated CDL ray angle

ASmodel is the rms angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A

μϕ,model is the mean angle of the tabulated CDL, calculated using the definition in Annex A

μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A

μϕ,desired is the desired mean angle

ASdesired is the desired rms angular spread

ϕn,scaled is the resulting scaled ray angle.

WrapTo180(⋅) is a function which wraps an azimuth angle to the interval [-180, 180].

The angular scaling is applied on the ray angles including offsets from the tabulated cluster angles. Typical angular spreads for different scenarios can be obtained from the system-level model.

Example scaling values are:

-	AOD spread (ASD) for each CDL model: {5, 10, 15, 25} degrees.

-	AOA spread (ASA) for each CDL model: {30, 45, 60} degrees.

-	ZOA spread (ZSA) for each CDL model: {5, 10, 15} degrees.

-	ZOD spread (ZSD) for each CDL model: {1, 3, 5} degrees.

The angular scaling and translation can be applied to some or all of the azimuth and zenith angles of departure and arrival.

Note 1: The azimuth angles may need to be wrapped around to be within [0, 360] [-180, 180] degrees, while the zenith angles may need to be clipped to be within [0, 180] degrees.

Note 2: For each of AOA, AOD, ZOA, and ZOD, not all desired AS values, ASdesired, larger than the model AS value, ASmodel, may be achievable by equation (7.7-5).

*** Unchanged text omitted ***

# Appendix D - Draft CR to 38.901 Section 7.7.5.1[4]

*** Unchanged text omitted ***

#### 7.7.5.1	CDL extension: Scaling of angles

The angle values of CDL models are fixed, which is not very suitable for MIMO simulations for several reasons; The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle can be changed to  and angular spread can be changed by scaling. The translated and scaled ray angles can be obtained according to the following equation:

ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired,                  (7.7-5)

where

ϕn,intermediate=ScalingFactor(ASdesired)∙Angleϕn,model-μϕ,model,

and

Anglex=x,if x is a zenith angleWrapTo180x,if x is an azimuth angle.

ScalingFactorASdesired=minx≥0{x: ASx=ASdesired}.

ASx=-2lnn=1Nm=1Mexpjx(ϕn,m+θn,m)Pn,mn=1Nm=1MPn,m

θn,m=WrapTo180ϕn,model-μϕ,model-ϕn,model-μϕ,model

(7.7-5)

in which:

ϕn,model is the tabulated CDL ray angle

ASmodel is the rms angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A

μϕ,model is the mean angle of the tabulated CDL, calculated using the definition in Annex A

μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A

μϕ,desired is the desired mean angle

ASdesired is the desired rms angular spread

ϕn,scaled is the resulting scaled ray angle

WrapTo180(⋅) is a function which wraps an azimuth angle to the interval [-180, 180].

The angular scaling is applied on the ray angles including offsets from the tabulated cluster angles. Typical angular spreads for different scenarios can be obtained from the system-level model.

Example scaling values are:

-	AOD spread (ASD) for each CDL model: {5, 10, 15, 25} degrees.

-	AOA spread (ASA) for each CDL model: {30, 45, 60} degrees.

-	ZOA spread (ZSA) for each CDL model: {5, 10, 15} degrees.

-	ZOD spread (ZSD) for each CDL model: {1, 3, 5} degrees.

The angular scaling and translation can be applied to some or all of the azimuth and zenith angles of departure and arrival.

Note: The azimuth angles may need to be wrapped around to be within [0, 360] degrees, while the zenith angles may need to be clipped to be within [0, 180] degrees.

*** Unchanged text omitted ***