3GPP TSG RAN WG1 #115		R1-2312430

Chicago, USA, November 13th – November 17th, 2023

Source: 	moderator (vivo)

Title:	Summary of discussion on SRS transmission occasion

Agenda Item:	7.1

Document for:	Discussion and decision

Introduction

Discuss and clarify the meaning of “the SRS transmission occasion ”  and “transmit power split equally across the configured antenna ports for SRS” when multiple SRS resources are configured, and introduce a CR if needed.

In [1], the SRS transmission occasion is discussed when multiple SRS resources in a set is configured, also it is proposed to clarify “transmit power split equally across the configured antenna ports for SRS” when multiple SRS resources are configured on the same symbol(s).

Discussion

According to current spec there is no restriction on multiple SRS resources configured on same or different symbol(s) for codebook, and there is UE capability for non-codebook, for antenna switching they are on different symbol(s).

Please answer the following questions below.

Question: do you think there is ambiguity in the spec on “the SRS transmission occasion ”  and “transmit power split equally across the configured antenna ports for SRS” when multiple SRS resources are configured? Please provide your understanding of current spec if you answer No.

| Company | Yes or No | comments |
| --- | --- | --- |
| QC | Yes | We understand that a SRS resource is a transmission occasion. For non-codebook SRS, we can discuss how to ensure the power is equally split among the resources/ports that are to be transmitted simultaneously. |
| MTK | Yes | Similar view as QC. |
| Nokia, NSB |  | In general, our understanding is that an SRS transmission occasion is defined as comprising all the SRS ports transmitted in L consecutive symbols of a given slot, whether the ports are in the same or different resources. |
| Google |  | In our view, the SRS transmission occasion is defined based on SRS in consecutive symbols. If there are multiple SRS resources at the same symbol, such SRS resources belongs to one SRS transmission occasion. |
| Moderator |  | Based on available comments, SRS transmission occasion could be understood as per symbol or consecutive symbols of one SRS resource or multiple overlapping SRS resources however it is not clear if multiple SRS resources are partially overlapped in time domain. Whether partial overlapping in time domain of multiple SRS resources is typical case or not is also a discussion point. When multiple SRS resources are fully overlapped in time, “transmit power split equally across the configured antenna ports for SRS” in current spec is not clear |
| Ericsson | Yes | “across the configured antenna ports for SRS” could refer to the ports in one SRS resource, all the ports in an SRS resource set, all ports in all resources, etc. Most of these would not make sense, but we should actually specify the power control here rather than have a group-guessing effort on what a reasonable behavior would be. |

If your answer is “Yes” above please provide your views on how to clarify in the spec

| company | comments |
| --- | --- |
| QC | Clarify in sec 7.5 that a same scaling factor is applied to all simultaneous non-codebook SRS resources when needed. |
| MTK | Similar view as QC. |
| Nokia, NSB | Agree that the same scaling should be applied, and don’t object specification clarification if that is seen really necessary. However, if we all agree on what a transmission occasion is, we can maybe just conclude that: “SRS power equally distributed all ports per transmission occasion (or an OFDM symbol) for a UE.” And don’t need to mention resources or resource sets. |
| Moderator | How to clarify in the spec can be further discussed |
| Ericsson | We should address all SRS usages; clarifying only non-codebook may make the other usages more confusing. We note that beam management SRS resources can be transmitted simultaneously as well. Something along the lines of splitting power among ports in an OFDM symbol seems a good starting point, but would like to give this careful thought. |

Conclusions

Observations:

- Based on available comments, common understanding is that there exists ambiguity in current spec when multiple SRS resources are configured.

- How to clarify in the spec can be further discussed.

Reference

[1] R1-2311072	Discussion on SRS transmission occasion	vivo