3GPP TSG RAN WG1 #115			R1-231nnnn

Chicago, USA, November 13th – November 17th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.16.2

Agenda Item:		8.16.2

Document for: 	Endorsement

### 8.16.2	UE features for NR sidelink evolution

Agreement

- FG 47-m8 is updated and FG 47-m9 is added as follows

| 47. NR_SL_enh2 | 47-m8 | Transmitting S-SSB on additional S-SSB occasion(s) | 1. UE supports transmitting S-SSB on additional S-SSB occasion(s) FFS whether/how to introduce Rx capability | 15-4, 47-k1 | No | No | UE does not support transmitting S-SSB on additional S-SSB occasion(s) but supports to exclude those occasion(s) from SL resource pool(s) | Per band | N/A | N/A |  |  | [Optional without capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-m9 | Receiving S-SSB on additional S-SSB occasion(s) | 1. UE supports receiving S-SSB on additional S-SSB occasion(s) |  | No | No | UE does not support receiving S-SSB on additional S-SSB occasion(s) but supports to exclude those occasion(s) from SL resource pool(s) | Per band | N/A | N/A |  |  | Optional without capability signalling |

Agreement

- Introduce following FGs

| 47. NR_SL_enh2 | 47-k5 | Resource allocation for multi-consecutive slots transmission | UE supports resource (re-)selection for PSCCH/PSSCH transmission on multiple consecutive slots | TBD | [Yes] | [No] | UE does not support resource (re-)selection for multi-consecutive slots transmission | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-m10 | Contiguous RB-based PSCCH/PSSCH transmission | 1. UE supports contiguous RB-based PSCCH/PSSCH transmission 2. UE supports resource (re-)selection for contiguous RB-based PSCCH/PSSCH transmission FFS whether/how to define the capabilities for reception and/or PSFCH | TBD | [Yes] | [No] | UE does not support contiguous RB-based PSCCH/PSSCH transmission | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m11 | PSFCH transmissions in multiple contiguous RB sets | UE supports PSFCH transmissions in multiple contiguous RB sets | TBD | [No] | [No] | UE does not support PSFCH transmissions in multiple contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m11a | PSFCH transmissions in multiple non-contiguous RB sets | UE supports PSFCH transmissions in multiple non-contiguous RB sets | TBD | [No] | [No] | UE does not support PSFCH transmissions in multiple non-contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m12 | S-SSB transmissions in multiple contiguous RB sets | UE supports S-SSB transmissions in multiple contiguous RB sets | TBD | [No] | [No] | UE does not support S-SSB transmissions in multiple contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m12a | S-SSB transmissions in multiple non-contiguous RB sets | UE supports S-SSB transmissions in multiple non-contiguous RB sets | TBD | [No] | [No] | UE does not support S-SSB transmissions in multiple non-contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k8 | CW autonomous update for SL transmission without HARQ feedback | UE support autonomous update of the to the next higher allowed value when the same value is consecutively used for X times for generation of for PSCCH/PSSCH transmission without HARQ feedback. FFS whether to merge with 47-k1 | TBD | No | No | UE does not update for PSCCH/PSSCH transmission without HARQ feedback. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k9 | Sidelink mode 1 resource allocation in shared spectrum | 1. UE can monitor DCI format 3_0 on a licensed band for NR sidelink dynamic scheduling and configured grant type 2 for transmitting interlaced RB-based PSCCH/PSSCH on a shared band] 2. UE supports reporting NACK to gNB when transmitting PSCCH/PSSCH on scheduled resource(s) is failed due to LBT failure | TBD | Yes | No | UE does not perform interlaced RB-based PSCCH/PSSCH based on mode 1 resource allocation. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m13 | Transmissions/receptions of multiple dedicated PRBs in interlace-based PSFCH | 1. UE can transmit PSFCH(s) on up to a total of K dedicated PRBs in a slot. 2. UE can receive PSFCH(s) on up to a total of L dedicated PRBs in a slot | TBD | No | No | UE does not support multiple transmissions/receptions of common interlace-based PSFCH. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. Candidate values for K are FFS Candidate values for L are FFS | Optional with capability signalling |

Agreement

- FG 47-s1 is updated as follows

| 47. NR_SL_enh2 | 47-s1 | Transmission/Reception using dynamic resource pool sharing | 1) Avoidance of NR PSCCH/PSSCH/PSFCH overlapping with EUTRA SL resources in dynamic resource pool sharing using LTE sidelink resource reservation information in NR mode2 resource (re)selection 2) UE supports NR sidelink TXs and RXs in a resource pool in 15kHz and 30kHz SCSs and uses the SCS that is (pre)configured for a SL BWP. | [15-3, 15-6, 15-11] | Yes | No | UE does not support transmission/reception using dynamic resource pool sharing | [Per band] | N/A | N/A |  | [UE supports EUTRA mode 4 fSL in the same band] Component 2 does not imply that two different SCSs can be (pre)configured simultaneously in a SL BWP | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 47-v1 is updated as follows

| 47. NR_SL_enh2 | 47-v1 | NR SL communication with SL CA | 1) UE supports transmitting/receiving PSCCH/PSSCH/PSFCH simultaneously over multiple X SL carriers: FFS candidate values for X FFS whether to report following - Maximum number of simultaneous PSCCH/PSSCH TX/RX, equal to X and 1 per carrier - FFS Maximum number of simultaneous PSCCH decoding - FFS whether to report Maximum number of non-overlapping RBs per slot across all carriers the UE can attempt to decode 2) UE can adjust the transmission power of the PSCCH/PSSCH/PSFCH across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. FFS whether/how to merge FG for SL-CA | [15-3, 15-11] | Yes | No |  | [Per band] | N/A | N/A |  | Component 1: Candidate value of X = {2, 3, 4, 5, 6, 7, 8}, FFS on some BW restriction | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2310825	Sidelink unlicensed UE features	FUTUREWEI

R1-2310855	UE features for NR sidelink evolution	Huawei, HiSilicon

R1-2310890	On UE features for NR sidelink evolution	Nokia, Nokia Shanghai Bell

R1-2311118	Discussion on Rel-18 sidelink UE features	vivo

R1-2311184	Discussion on UE features for NR sidelink evolution	Spreadtrum Communications

R1-2311278	UE features list for Rel-18 NR sidelink evolution WI	OPPO, Huawei, HiSilicon, LG Electronics

R1-2311329	Further discussion on UE features for NR sidelink evolution	CATT, CICTCI

R1-2311394	Discussion on UE features for NR sidelink evolution	xiaomi

R1-2311519	Discussion on UE features for NR sidelink evolution	ZTE, Sanechips

R1-2311643	Discussion on UE features for NR SL evolution	NTT DOCOMO, INC.

R1-2311707	Views on UE Features for Rel-18 NR Sidelink Evolution	Apple

R1-2311868	UE features for NR sidelink evolution	Samsung

R1-2311977	Discussion on UE feature list for NR Sidelink Evolution	MediaTek Inc.

R1-2312062	UE feature for sidelink evolution	Qualcomm Incorporated