3GPP TSG-RAN WG1 Meeting #115	R1-2310885

Chicago, USA, November 13th–17th, 2023

Agenda Item:	8.16.8

Source:	Huawei, HiSilicon

Title:	UE features for Coverage Enhancement

Document for:	Discussion and Decision

# Introduction

In RAN#114bis [1], the agreements for UE features of coverage enhancement were achieved as follows.

Agreement - Component of FG 54-1 is confirmed as: Support of multiple PRACH transmissions with the same Tx spatial filter. Support {2, 4, 8} for the number of multiple PRACH transmissions with the same Tx spatial filter. - The column of “Consequence if the feature is not supported by the UE” in FG 54-1 is confirmed as: UE doesn’t support multiple PRACH transmissions with the same Tx spatial filter Agreement - The column of “Need for the gNB to know if the feature is supported” for FG54-1 is confirmed as “Yes” Agreement - Add a note in FG 54-3: If UE supporting this FG supports FG 11-1, the UE supports FG 54-3 with DCI format 0_2 - Delete “FFS whether to separate this FG for DCI 0_1/0_2[/0_3]” from FG 54-3 Agreement - No prerequisite FG is defined for FG 54-3 Agreement - FG 54-3a is updated as follows 54. NR_cov_enh2; 54-3a; PHR enhancement for dynamic waveform switching; Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH FFS details; 54-3; Yes; N/A; Optional with capability signaling.

In this contribution, UE features for coverage enhancements are discussed further, including dynamic waveform switching.

# Discussion

  - Waveform switching

### Dynamic waveform switching

In dynamic waveform switching discussion, in order to assist gNB decision on waveform switching, UE has to report whether support dynamic waveform switching in PUSCH transmission.

Regarding DWS in multiple-carrier, it can be divided into two categories, i.e. category A with 1 carrier per band like SUL and inter-band UL-CA and the category B with more than 1 carriers per band, i.e. intra-band UL-CA. In category A, because only single carrier can be transmitted at one band, the RF impact across UL carriers with concurrent transmission in different bands is little. In category B, the intra-band UL-CA is more complicated as the RF impact across UL carriers with concurrent transmission could be large in one band. In our view, it is unnecessary to separate FG 54-3 for SUL and inter-band UL-CA, and it is necessary to add a separate feature group for intra-band UL-CA.

Proposal 1: For FG 54-3,

- add a component as “Support of dynamic waveform switching for SUL and inter-band UL-CA”

- add a note as “Note: UE does not expect to be configured with more than 1 carrier per band if this feature is enabled.”

- indicate this capability with band granularity.

Add a new feature with index FG 54-3b for dynamic waveform switching, belonging to features 54. NR_cov_enh2. For FG 54-3b,

- add a component “Support of dynamic waveform switching for intra-band UL-CA”,

- the prerequisite feature group of this capability is FG 54-3

- it is necessary for the gNB to know if the feature is supported by UEs

- if the feature is not supported, dynamic waveform switching is not supported for intra-band UL-CA by the UE

- indicate this capability with band granularity

- it is N/A for columns “Need of FDD/TDD differentiation”, “Need of FR1/FR2 differentiation”, “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”

- it is optional with capability signaling.

Proposal 2: Adopt the changes in red in the following table,

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. NR_cov_enh2 | 54-1 | PRACH coverage enhancements | Support of multiple PRACH transmissions with the same Tx spatial filter. Support {2, 4, 8} for the number of multiple PRACH transmissions with the same Tx spatial filter. |  | Yes |  | UE doesn’t support multiple PRACH transmissions with the same Tx spatial filter |  | N/A | N/A | N/A |  | Optional with capability signalling. |
| 54. NR_cov_enh2 | 54-3 | Dynamic waveform switching | Support of dynamic waveform switching for DCI format 0_1/0_2. Support of dynamic waveform switching for SUL and inter-band UL-CA Note: If UE supporting this FG supports FG 11-1, the UE supports FG 54-3 with DCI format 0_2 | No | Yes |  | Dynamic waveform switching is not supported | per Band | N/A | N/A | N/A | Note: UE does not expect to be configured with more than 1 carrier per band if this feature is enabled. | Optional with capability signaling. |
| 54. NR_cov_enh2 | 54-3a | PHR enhancement for dynamic waveform switching | Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH | 54-3 | Yes | N/A |  |  |  |  |  |  | Optional with capability signaling |
| 54. NR_cov_enh2 | 54-3b | Dynamic waveform switching | Support of dynamic waveform switching for intra-band UL-CA. | 54-3 | Yes |  | Dynamic waveform switching is not supported for intra-band UL-CA. | per Band | N/A | N/A | N/A |  | Optional with capability signaling. |

# Conclusions

According to the above discussion, we have the following proposals:

Proposal 1: For FG 54-3,

- add a component as “Support of dynamic waveform switching for SUL and inter-band UL-CA”

- add a note as “Note: UE does not expect to be configured with more than 1 carrier per band if this feature is enabled.”

- indicate this capability with band granularity.

Add a new feature with index FG 54-3b for dynamic waveform switching, belonging to features 54. NR_cov_enh2. For FG 54-3b,

- add a component “Support of dynamic waveform switching for intra-band UL-CA”,

- the prerequisite feature group of this capability is FG 54-3

- it is necessary for the gNB to know if the feature is supported by UEs

- if the feature is not supported, dynamic waveform switching is not supported for intra-band UL-CA by the UE

- indicate this capability with band granularity

- it is N/A for columns “Need of FDD/TDD differentiation”, “Need of FR1/FR2 differentiation”, “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”

- it is optional with capability signaling.

Proposal 2: Adopt the changes in red in the following table,

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. NR_cov_enh2 | 54-1 | PRACH coverage enhancements | Support of multiple PRACH transmissions with the same Tx spatial filter. Support {2, 4, 8} for the number of multiple PRACH transmissions with the same Tx spatial filter. |  | Yes |  | UE doesn’t support multiple PRACH transmissions with the same Tx spatial filter |  | N/A | N/A | N/A |  | Optional with capability signalling. |
| 54. NR_cov_enh2 | 54-3 | Dynamic waveform switching | Support of dynamic waveform switching for DCI format 0_1/0_2. Support of dynamic waveform switching for SUL and inter-band UL-CA Note: If UE supporting this FG supports FG 11-1, the UE supports FG 54-3 with DCI format 0_2 | No | Yes |  | Dynamic waveform switching is not supported | per Band | N/A | N/A | N/A | Note: UE does not expect to be configured with more than 1 carrier per band if this feature is enabled. | Optional with capability signaling. |
| 54. NR_cov_enh2 | 54-3a | PHR enhancement for dynamic waveform switching | Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH | 54-3 | Yes | N/A |  |  |  |  |  |  | Optional with capability signaling |
| 54. NR_cov_enh2 | 54-3b | Dynamic waveform switching | Support of dynamic waveform switching for intra-band UL-CA. | 54-3 | Yes |  | Dynamic waveform switching is not supported for intra-band UL-CA. | per Band | N/A | N/A | N/A |  | Optional with capability signaling. |

# Reference

- R1-2310627, “Session Notes for 8.16.8”, Ad-Hoc Chair (NTT DOCOMO), 3GPP RAN1 #114bis, Xiamen, China, October 9th – October 13th, 2023.