3GPP TSG RAN WG1 Meeting #115	                     R1-2311041

Chicago, USA, November 13th – 17th, 2023

Source:	Fujitsu

Title:	Maintenance on Rel-18 CSI enhancements

Agenda item:	8.1.2

Document for:	Discussion and Decision

Introduction

Starting from the RAN1#114bis meeting, Rel-18 MIMO has entered the maintenance phase. In this contribution, we share some discussions on the remaining issues for Rel-18 CSI enhancements.

Discussion on dropping rules for TDCP reporting

In the last meeting, the following agreements on dropping rules for TDCP reporting have been supported.

Agreement in #114bis For the Rel-18 TRS-based TDCP reporting, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for each CSI-RS resource for KTRS CSI-RS resource sets configured for TDCP reporting no later than CSI reference resource, otherwise drops the report. - This includes the cases of CSI report (re)configuration, serving cell activation, BWP change - FFS (RAN1#115): Whether DRX configuration needs to be included as a case

The remaining issue for TDCP is whether DRX configuration needs to be included as a case for the condition on reporting/dropping a CSI report. For Rel-18 CJT and mobility enhancement, this condition has been supported. For TDCP, since UE needs joint calculate Time-domain correlation profile based multiple TRS resources and resource sets, the alignment of phase among different resource channels is more essential. Therefore, to avoid the misalignment of different resource channels caused by the phase deviation between uplink and downlink switching, the restriction on the same DRX Active Time across different TRS resources/resource sets should be also supported for Rel-18 TDCP.

Furthermore, for the condition on reporting/dropping a CSI report, UE behavior for not receiving at least one transmission occasion is unexpectedly lost in current TS38.214[1]. Therefore, our suggestion is to add “and drops the report otherwise”.

The text proposal can be considered as shown below.

--------------------------------------- Start of the TP for TS38.214-------------------------------------------

5.2.2.5	CSI reference resource definition

<Unrelated part omitted>

For a CSI-ReportConfig configured with codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement and one CSI-RS and/or CSI-IM resource transmission occasion for the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise.

For a CSI-ReportConfig configured with codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least one aperiodic or Kp periodic or semipersistent consecutive CSI-RS transmission occasions for each CSI-RS resource in the corresponding CSI-RS Resource Set for channel measurement and one CSI-RS and/or CSI-IM resource transmission occasion for the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise. The value of Kp∈{1,2,4} is indicated by UE capability, as defined in clause 5.2.1.6.

For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'tdcp', after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each CSI-RS resource in the KTRS CSI-RS Resource Sets of the CSI-RS Resource Setting for channel measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise.

<Unrelated part omitted>

--------------------------------------- End of the TP for TS38.214----------------------------------------

- For the TDCP reporting, regarding the condition on reporting/dropping a CSI report in TS 38.214 section 5.2.2.5, the condition is updated to also contain “and within the same DRX Active Time, when DRX is configured, and drops the report otherwise”.

Conclusions

In this contribution, we provide our views on the enhancements for TDCP reporting. We have the following proposal:

- For the TDCP reporting, regarding the condition on reporting/dropping a CSI report in TS 38.214 section 5.2.2.5, the condition is updated to also contain “and within the same DRX Active Time, when DRX is configured, and drops the report otherwise”.

References

- R1-2308760, TS38.214, RAN1#114bis, October 9-13, 2023.