3GPP TSG RAN WG1 #115	R1-2311045

Chicago, USA, November 13th – November 17th, 2023

Agenda item:	8.11.1

Source: 	Fujitsu

Title:	Remaining issues on side control information and NCR behaviour

Document for:		Discussion and decision

# Introduction

In this contribution, some remaining issues on NCR are discussed.

# Discussion

## Clarification on indication by DCI format 2_8

The order of priority for different indications is defined to determine which beam index should be used in the case that different indications indicate different beam indexes for a certain time position.

For indications with same priority or a single indication, it is straight forward that no conflict is expected. However, this is only partially reflected in [1], with the case of single DCI format 2_8 missing. Therefore, a TP is provided to capture the missing case.

Proposal 1: Adopt the TP below for TS 38.213.

The NCR does not expect overlapping time resources provided by either NCR-PeriodicFwdResourceSet or NCR-SemiPersistentFwdResourceSet to be associated with different beam indexes. The NCR does not expect overlapping time resources provided by a single DCI format 2_8 to be associated with different beam indexes.

## Simultaneous UL transmission on C-link and BH-link

According to the following agreement in RAN1#114bis meeting, if an NCR does not support simultaneous UL transmission on C-link and BH-link, C-link always has higher priority than BH-link in case of collision.

Agreement If an NCR does not support simultaneous transmission on C-link and backhaul link, NCR-Fwd does not transmit over the time resource if NCR-MT transmits on the same time resource.

Following the same principle, for an NCR capable of simultaneous UL transmission of C-link and BH-link, if dropping or power sharing between C-link and BH-link is supported, C-link should always be prioritized. Note that the UL of C-link is mainly to transmit messages in RA, reference signals and UCI etc. which are critical to maintain NCR’s proper operation.

Proposal 2: For an NCR capable of simultaneous UL transmission of C-link and BH-link, C-link should be prioritized in dropping or power sharing between C-link and BH-link.

# Conclusion

The proposals are summarized as follows.

Proposal 1: Adopt the TP below for TS 38.213.

The NCR does not expect overlapping time resources provided by either NCR-PeriodicFwdResourceSet or NCR-SemiPersistentFwdResourceSet to be associated with different beam indexes. The NCR does not expect overlapping time resources provided by a single DCI format 2_8 to be associated with different beam indexes.

Proposal 2: For an NCR capable of simultaneous UL transmission of C-link and BH-link, C-link should be prioritized in dropping or power sharing between C-link and BH-link.

# 4. References

[1] 3GPP TS 38.213 V18.0.0 (2023-09)