3GPP TSG RAN WG1 #115	R1-2311085

Chicago, USA, November 13th – November 17th, 2023

Source:	vivo

Title:	Maintenance on CSI enhancement

Agenda Item:	8.1.2

Document for:	Discussion and Decision

Introduction

Following two objectives related to CSI enhancement are listed in MIMO Evolution for Downlink and Uplink WID [1].

1. Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking 4. Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we discuss CSI enhancement for high-medium UE velocities within the above WID scope and provide our suggested text proposals.

Discussion and Text proposals

In this chapter, we focus on the test proposals and discussion for high-medium UE velocities.

Capture of the K configured CSI-RS resources sharing the same BW and RE locations

Reason for change:

According to the latest CR on TS 38214[2],  the second sub-bullet of the following agreement achieved in RAN1 #112bis hasn’t been captured, i.e., all the K configured CSI-RS resources comprising the CMR shall share the same BW and RE locations.

Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding CSI calculation and measurement, - The number of CSI-RS ports is the same for all the K configured CSI-RS resources comprising the CMR and the antenna ports for the same antenna port index across the K CSI-RS resources are the same. - All the K configured CSI-RS resources comprising the CMR share the same BW and RE locations - For interference measurement, legacy specification is fully reused, including the configuration for NZP CSI-RS for interference measurement or CSI-IM in relation to the configured CMR, i.e. only one NZP CSI-RS resource for interference measurement or only one CSI-IM resource can be configured irrespective of the value of K - On PDSCH EPRE assumption for CQI calculation, a same powerControlOffset value is assumed for all the K configured CSI-RS resources comprising the CMR - Alt 1: The configured powerControlOffset value is the same for all the K configured CSI-RS resources comprising the CMR - Alt 2: The assumed PDSCH EPRE of all the K CSI-RS resources follows the configured powerControlOffset value of one fixed CSI-RS resource, e.g. the first one Note: This may imply that existing section 5.2.2.2.75 of TS38.214 can apply to Rel-18 Type-II Doppler codebook in terms of Rel-18 CMR (burst of CSI-RS resources) and Rel-18 CSI reference resource

Summary of change:

For the Rel-18 Type-II codebook refinement for high/medium velocities, capture that all the CSI-RS resources in the CSI-RS Resource Set are configured with the same bandwidth and RE locations.

Consequences if not approved:

The agreement is captured incompletely.

Text proposal:

Adopt following text proposal in 5.2.1.4.1 of TS 38.214.

#### 5.2.1.4.1 Resource Setting configuration < Unchanged parts are omitted > A UE configured with a CSI-ReportConfig with the higher layer parameter N4 and reportQuantity set to 'cri-RI-PMI-CQI', is expected to be configured with K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource in the resource set for channel measurement. For an aperiodic CSI-RS resource set for channel measurement, the K CSI-RS resources, which are configured with the same bandwidth and RE locations, are triggered by the same triggering instance and the separation between two consecutive CSI-RS resources is m∈{1,2} slots, which is configured by higher layer parameter in the NZP-CSI-RS-ResourceSet. The UE shall assume that the antenna port with the same port index of the K aperiodic CSI-RS resources is the same. If interference measurement is performed on CSI-IM, only one resource is configured in the corresponding csi-IM-ResourceSet. If interference measurement is performed on NZP CSI-RS, only one resource is configured in the corresponding NZP-CSI-RS-ResourceSet for interference measurement. < Unchanged parts are omitted >

Discussion on the order of CSI-RS transmitting

For predicted CSI acquisition, an aperiodic CSI-RS resource set can be configured with K∈{4,8,12} aperiodic CSI-RS resources for channel measurement, where the separation between two consecutive CSI-RS resources is m∈{1,2} slots. The UE shall assume that the antenna port with the same port index of the K aperiodic CSI-RS resources is the same, and all the CSI-RS resources in the CSI-RS Resource Set are configured with the same bandwidth and subcarrier locations. However, this does not mean that all the CSI-RS resources in the CSI-RS Resource Set are exactly the same. Some aspects in different CSI-RS resources can still vary, e.g., scrambling ID. Therefore, the order in which gNB transmits CSI-RS may not be consistent with the order assumed by UE, which may cause UE to obtain wrong channels.

If all the CSI-RS resources in the CSI-RS Resource Set are not the same, the order in which gNB transmits CSI-RS may not be consistent with the order assumed by UE.

The following approaches can be considered to address this issue.

Approach 1: All the CSI-RS resources in the CSI-RS Resource Set are exactly the same, including scrambling ID.

Approach 2: Define the order of the triggered CSI-RS, e.g., following the order of the CSI-RS resource IDs configured in the CSI-RS Resource Set.

- For approach1, from the actual effect, it appears to be equivalent to Alt2 of the following agreement excluded in RAN1 #110bis. Besides, all the CSI-RS resources in the CSI-RS Resource Set are the same may be inflexible for gNB implementation. Therefore, approach 2 is preferred.

Agreement On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, support the following CSI-RS resource types/structures for CMR: - Time-domain behaviour for NZP CSI-RS resource: periodic (P), semi-persistent (SP), aperiodic (AP) - FFS: Whether to introduce constraints on allowed configuration - Down select from the following: - Alt1. Support K>1 NZP CSI-RS resources, received via a single triggering instance, for aperiodic (AP) -CSI-RS-based channel measurement in a same CSI-RS resource set where the separation between 2 consecutive AP-CSI-RS resources is m slot(s): - Alt2. Support one NZP CSI-RS resource in a CSI-RS resource set, where K>1 occasions are received via a single triggering instance, for aperiodic (AP)-CSI-RS-based channel measurement where the separation between 2 consecutive AP-CSI-RS resources is m slot(s). - For any of the alternatives: - No CRI is reported - FFS: Details, e.g., supported value(s) of K, m, other use cases for the AP-CSI-RS resources (e.g., for training filter coefficients, prediction or performance monitoring) - Support only one NZP CSI-RS resource for P or SP-CSI-RS-based channel measurement

For predicted CSI based on an aperiodic CSI-RS resource set, clarify that the CSI-RS resources are transmitted following the order of the CSI-RS resource IDs configured in the CSI-RS Resource Set.

Conclusions

To summarize, we have following proposal and text proposals.

- Adopt following text proposal in 5.2.1.4.1 of TS 38.214.

#### 5.2.1.4.1 Resource Setting configuration < Unchanged parts are omitted > A UE configured with a CSI-ReportConfig with the higher layer parameter N4 and reportQuantity set to 'cri-RI-PMI-CQI', is expected to be configured with K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource in the resource set for channel measurement. For an aperiodic CSI-RS resource set for channel measurement, the K CSI-RS resources, which are configured with the same bandwidth and RE locations, are triggered by the same triggering instance and the separation between two consecutive CSI-RS resources is m∈{1,2} slots, which is configured by higher layer parameter in the NZP-CSI-RS-ResourceSet. The UE shall assume that the antenna port with the same port index of the K aperiodic CSI-RS resources is the same. If interference measurement is performed on CSI-IM, only one resource is configured in the corresponding csi-IM-ResourceSet. If interference measurement is performed on NZP CSI-RS, only one resource is configured in the corresponding NZP-CSI-RS-ResourceSet for interference measurement. < Unchanged parts are omitted >

For predicted CSI based on an aperiodic CSI-RS resource set, clarify that the CSI-RS resources are transmitted following the order of the CSI-RS resource IDs configured in the CSI-RS Resource Set.

References

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94e.

R1-2310760, CHANGE REQUEST 38.214: Correction of specification support for MIMO enhancements on CSI, 3GPP TSG-RAN WG1 Meeting #114bis, Xiamen, China, October 9th – October 13th, 2023.