3GPP TSG RAN WG1 #115                                                   R1-2311333

Chicago, USA, 13th – 17th November 2023

Source:              CATT

Title:                   Discussion on Rel-18 UE feature for XR in RAN1

Agenda Item:     8.16.6

Document for:   Discussion and Decision

# Introduction

The UE feature for the XR enhancement was discussed in RAN1#114bis with the following agreements,

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_XR_Enh | 50-1 | Multi-PUSCHs for Configured Grant | 1. Determination of time-domain resource allocation for CG-PUSCHs associated to a multi-PUSCHs CG 2. Maximum supported number of consecutive slots configured for CG-PUSCH TOs in one CG period, candidate value set: {16, 32} FFS whether to separate this FG for type-1 and type-2 CG FFS whether to separate this FG for multiple CG configurations FFS whether to separate this FG for shared spectrum FFS whether to report maximum supported number of configured CG-PUSCH TOs in one CG period | One or both of {5-19, 5-20} | Yes | N/A | UE is not able to support Multi-PUSCHs per one period in Configured grant in licensed band |  |  |  |  |  | Optional with capability signaling |
| 50. NR_XR_Enh | 50-1a | Multiple active multi-PUSCHs configured grant configurations for a BWP of a serving cell | 1. Supported maximum number of configured/active configured grant configurations in a BWP of a serving cell Candidate values for component 1: {[2, 4, 8, 12]} 2. Supported maximum number of configured/active configured grant configurations across all serving cells, and across MCG and SCG in case of NR-DC Candidate values for component 2: {2, …, 32} | 50-1, [11-9] | Yes | N/A |  |  |  |  |  | [when UE supports both FG 11-9 and 50-1a the maximum number configured for legacy CG and multi-PUSCH CG should not exceed a maximum number, FFS maximum number] [For all the reported bands in FR1, a same X1 value is reported for component 2. For all the reported bands in FR2, a same X2 value is reported for component 2.] [The total number of configured/active configured grant configurations across all serving cells in FR1 is no greater than X1.] [The total number of configured/active configured grant configurations across all serving cells in FR2 is no greater than X2.] [If there are some serving cell(s) in FR1 and some serving cell(s) in FR2, the total number of configured/active configured grant configurations across all serving cells is no greater than max(X1, X2).] [Regarding the interpretation of UE capabilities in case of cross-carrier operation, support of FG50-9 is based on the support of this capability for the band of the scheduled/triggered/indicated cell only] | Optional with capability signaling |
| 50. NR_XR_Enh | 50-2 | UCI indication of unused CG-PUSCH transmission occasions | 1. Multiplexing of the Unused transmission occasions UCI (UTO-UCI) on a CG-PUSCH FFS whether to merge this FG into FG 50-1 FFS whether to separate this FG for UTO-UCI multiplexing with HARQ-ACK | One or both of {5-19, 5-20} | Yes | N/A | UE is not able to indicate the unused resources in CG |  |  |  |  |  | Optional with capability signaling |
| 50. NR_XR_Enh | 50-3 | PDCCH monitoring resumption after UL NACK | Support PDCCH monitoring resumption after UL NACK | 29-3a | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |

RAN1 work on XR enhancement would focus on the enhancement of NR system capacity for XR with multiple CG PUSCH occasions with dynamic indication of unused CG PUSCH on UCI.

This paper discusses the RAN1 aspect of UE features for XR enhancement for NR.

# UE Features for NR XR in RAN1

RAN1 discussion of XR enhancement for NR focuses on the UL NR capacity enhancement through multiple configured CG PUSCH occasions with UE feedback of unused CG PUSCH occasions on UCI.  The UE capability of multiple CG PUSCH should not have any differentiation in the support of Type 1 or Type 2 CG since the only difference between Type 1 and Type CG is the L1 signaling of activation of CG for Type 2 CG.  The multiple CG configurations should be part of multiple CG PUSCH configuration for XR service.

Proposal 1:  The UE capability of multiple CG PUSCH should not have any differentiation in the support of Type 1 or Type 2 CG.  The multiple CG configurations should be part of multiple CG PUSCH configuration for XR service.

Multiple CG occasions are configured by RRC and only valid CG occasions are used for UL XR traffic transmission.   The TDRA determination of multiple CG occasion would follow the NR-U framework for type 1 and type 2 CGs. Even though the multiple PUSCHs CG is based on the framework of NR-U, the UE capability of multiple CG PUSCH is quite different from that of NR-U as following.

- The repetition is not supported for multiple PUSCHs CG, while the NR-U supports.

- The determination of HARQ process ID is different. The HPID is reported by UE in unlicensed band, while the multiple CG PUSCH for XR is calculated based on the formula.

- The consecutive slots N is configured separately from the parameters in NR-U, such as cg-nrofSlots-r16, cg-nrofPUSCH-InSlot-r16 and cgRetransmissionTimer while there is no RRC parameters for multiple CG PUSCHs for XR.

Proposal 2:  The UE capability of supporting multiple CG PUSCH in FG 50-1 for licensed spectrum only but not for shared spectrum

It was agreed in RAN1#114bis that the maximum number of the configured/active CG occasions in a BWP of a serving cell and total number cross all serving cells were single out as a new FG-50-1a.   Whether FG11-9 as the prerequisite was discussed and as the FFS.  The FG11-9 supports the multiple CG PUSCH for URLLC with prerequisite either one of FG 5-19 Type 1 CG or FG 5-20 Type 2 CG.  The multiple CG PUSCH feature for XR targets for large data size and is different to that of URLLC.  Thus, the multiple CG PUSCH in FG 50-1a should not have prerequisite of FG 11-9

Proposal 3:  The UE capability of supporting multiple CG PUSCH in FG 50-1a for XR does not have the prerequisite of supporting FG 11-9.

If some of the multiple configured CG occasions are not used, the unused CG occasions are indicated by new UTO-UCI.    While minimum number of configured multiple CG PUSCH is 2, the UTO-UCI indication of unused CG occasions should be a separated feature group to indicate the unused CG occasion for multiple CG PUSCH and legacy Type 1 or Type 2 CGs.

Proposal 4: FG 50-2 UCI indication of unused CG-PUSCH transmission occasions should be a separated UE capability for the indication of unused CG occasions for multiple CG PUSCH and legacy Type 1/Type 2 CGs.

Thus, the UE features for XR enhancement in RAN1 is proposed as follows,

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_XR_Enh | 50-1 | Multi-PUSCHs for Configured Grant | 1. Determination of time-domain resource allocation for CG-PUSCHs associated to a multi-PUSCHs CG 2. Maximum supported number of consecutive slots configured for CG-PUSCH TOs in one CG period, candidate value set: {16, 32} FFS whether to separate this FG for type-1 and type-2 CG FFS whether to separate this FG for multiple CG configurations FFS whether to separate this FG for shared spectrum FFS whether to report maximum supported number of configured CG-PUSCH TOs in one CG period | One or both of {5-19, 5-20} | Yes | N/A | UE is not able to support Multi-PUSCHs per one period in Configured grant in licensed band | Per UE | N | N | N |  | Optional with capability signaling |
| 50. NR_XR_Enh | 50-1a | Multiple active multi-PUSCHs configured grant configurations for a BWP of a serving cell | 1. Supported maximum number of configured/active configured grant configurations in a BWP of a serving cell Candidate values for component 1: {[2, 3, 4, ….,12]} 2. Supported maximum number of configured/active configured grant configurations across all serving cells, and across MCG and SCG in case of NR-DC Candidate values for component 2: {2, …, 32} | 50-1, [11-9] | Yes | N/A |  | Per UE | N | N | N | [when UE supports both FG 11-9 and 50-1a the maximum number configured for legacy CG and multi-PUSCH CG should not exceed a maximum number, FFS maximum number] [For all the reported bands in FR1, a same X1 value is reported for component 2. For all the reported bands in FR2, a same X2 value is reported for component 2.] [The total number of configured/active configured grant configurations across all serving cells in FR1 is no greater than X1.] [The total number of configured/active configured grant configurations across all serving cells in FR2 is no greater than X2.] [If there are some serving cell(s) in FR1 and some serving cell(s) in FR2, the total number of configured/active configured grant configurations across all serving cells is no greater than max(X1, X2).] [Regarding the interpretation of UE capabilities in case of cross-carrier operation, support of FG50-9 is based on the support of this capability for the band of the scheduled/triggered/indicated cell only] | Optional with capability signaling |
| 50. NR_XR_Enh | 50-2 | UCI indication of unused CG-PUSCH transmission occasions | 1. Multiplexing of the Unused transmission occasions UCI (UTO-UCI) on a CG-PUSCH FFS whether to merge this FG into FG 50-1 FFS whether to separate this FG for UTO-UCI multiplexing with HARQ-ACK | One or both of {5-19, 5-20} | Yes | N/A | UE is not able to indicate the unused resources in CG | Per UE | N | N | N |  | Optional with capability signaling |
| 50. NR_XR_Enh | 50-3 | PDCCH monitoring resumption after UL NACK | Support PDCCH monitoring resumption after UL NACK | 29-3a | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |

# Conclusion

In this contribution, we have discussed the UE features for XR enhancements for NR.   We have the following proposals,

- Proposal 1:  The UE capability of multiple CG PUSCH should not have any differentiation in the support of Type 1 or Type 2 CG.  The multiple CG configurations should be part of multiple CG PUSCH configuration for XR service.

- Proposal 2:  The UE capability of supporting multiple CG PUSCH in FG 50-1 for licensed spectrum only but not for shared spectrum

- Proposal 3:  The UE capability of supporting multiple CG PUSCH in FG 50-1a for XR does not have the prerequisite of supporting FG 11-9.

- Proposal 4: FG 50-2 UCI indication of unused CG-PUSCH transmission occasions should be a separated UE capability for the indication of unused CG occasions for multiple CG PUSCH and legacy Type 1/Type 2 CGs.

We proposed to use Table 1for the UE feature for UE power saving enhancements for NR.

Table 1:  UE capability for XR enhancement for NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_XR_Enh | 50-1 | Multi-PUSCHs for Configured Grant | 1. Determination of time-domain resource allocation for CG-PUSCHs associated to a multi-PUSCHs CG 2. Maximum supported number of consecutive slots configured for CG-PUSCH TOs in one CG period, candidate value set: {16, 32} | One or both of {5-19, 5-20} | Yes | N/A | UE is not able to support Multi-PUSCHs per one period in Configured grant in licensed band | Per UE | N | N | N |  | Optional with capability signaling |
| 50. NR_XR_Enh | 50-1a | Multiple active multi-PUSCHs configured grant configurations for a BWP of a serving cell | 1. Supported maximum number of configured/active configured grant configurations in a BWP of a serving cell Candidate values for component 1: {[2, 3, 4, …., 12]} 2. Supported maximum number of configured/active configured grant configurations across all serving cells, and across MCG and SCG in case of NR-DC Candidate values for component 2: {2, …, 32} | 50-1, [11-9] | Yes | N/A |  | Per UE | N | N | N | [For all the reported bands in FR1, a same X1 value is reported for component 2. For all the reported bands in FR2, a same X2 value is reported for component 2.] [The total number of configured/active configured grant configurations across all serving cells in FR1 is no greater than X1.] [The total number of configured/active configured grant configurations across all serving cells in FR2 is no greater than X2.] [If there are some serving cell(s) in FR1 and some serving cell(s) in FR2, the total number of configured/active configured grant configurations across all serving cells is no greater than max(X1, X2).] [Regarding the interpretation of UE capabilities in case of cross-carrier operation, support of FG50-9 is based on the support of this capability for the band of the scheduled/triggered/indicated cell only] | Optional with capability signaling |
| 50. NR_XR_Enh | 50-2 | UCI indication of unused CG-PUSCH transmission occasions | 1. Multiplexing of the Unused transmission occasions UCI (UTO-UCI) on a CG-PUSCH FFS whether to merge this FG into FG 50-1 FFS whether to separate this FG for UTO-UCI multiplexing with HARQ-ACK | One or both of {5-19, 5-20} | Yes | N/A | UE is not able to indicate the unused resources in CG | Per UE | N | N | N |  | Optional with capability signaling |
| 50. NR_XR_Enh | 50-3 | PDCCH monitoring resumption after UL NACK | Support PDCCH monitoring resumption after UL NACK | 29-3a | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Features | Index | Feature group | Components | Prerequisite feature groups |
| 50. NR_XR_Enh | 50-1 | Multi-PUSCHs for Configured Grant | - Determination of time-domain resource allocation for CG-PUSCHs associated to a multi-PUSCHs CG - maximum supported number of configured CG-PUSCH TOs in one CG period is 16 or 32 | 11-9 | Yes | N/A | UE is not able to support Multi-PUSCHs per one period in Configured grant in licensed band | Per UE | N | N | N |  | Optional with capability signaling |
| 50. NR_XR_Enh | 50-2 | UCI indication of unused CG-PUSCH transmission occasions | 1. Multiplexing of the Unused transmission occasions UCI (UTO-UCI) on a CG-PUSCH |  | Yes | N/A | UE is not able to indicate the unused resources in CG | Per UE | N | N | N |  | Optional with capability signaling |

# References

- RP-230786, “XR Enhancements for NR”, Nokia, Qualcomm

- R1-2310626, “Session Notes for 8.16.6”,  Ad-Hoc Chair (NTT DOCOMO)