3GPP TSG RAN WG1 #115		R1-2311486

Chicago, USA, November 13th – November 17th, 2023

Source: 	CMCC

Title:	Maintenance on further complexity reduction for eRedCap

Agenda item:	8.4.1

Document for:	Discussion & Decision

- Introduction

In RAN1#114b meeting, the following agreements about UE complexity reduction were agreed[1].

Agreement - The following does not apply to FG 48-2 UEs for CFRA: - RAR PDSCH timeline relaxation Agreement - For a UE with BB bandwidth reduction, for multicast MBS specified in Rel-17, the number of PRBs scheduled in DCI is not larger than 25/15 PRBs for 15/30 kHz SCS (irrespective of whether HARQ feedback is enabled or disabled). Agreement - An eRedCap UE with bandwidth reduction, depending on indicated UE capability, the UE can decode a PDSCH for MBS broadcast and a PDSCH for unicast with the two PDSCH partially or fully overlapping in time in non-overlapping PRBs, if the total number of PRBs does not exceed the maximum number of PRBs that the UE can receive or process per slot. Agreement - An eRedCap UE with bandwidth reduction, depending on indicated UE capability, the UE can decode a PDSCH for MBS multicast and a PDSCH for unicast with the two PDSCH partially or fully overlapping in time in non-overlapping PRBs, if the total number of PRBs does not exceed the maximum number of PRBs that the UE can receive or process per slot. Agreement Agreement - Continue to discuss whether and how to update the specification regarding the following aspect: - simultaneous MBS broadcast/multicast and unicast when the total number of PRBs exceeds the maximum number of PRBs that the UE can receive or process per slot (if this is a valid case) Agreement - Continue to discuss potential clarification of “A UE that has not indicated FG 48-2” in the paragraphs in 38.213 clause 17.1A - Continue to discuss potential clarification of “A UE that indicated FG 48-2” in the paragraphs in 38.213 clause 17.1A

In this contribution, we give our considerations on some remaining issues and spec corrections.

- Discussion on UE complexity reduction

- In this section, we will provide our considerations on MBS PDSCH bandwidth, MBS simultaneous reception, PUSCH bandwidth and corresponding spec corrections.

2.1 MBS PDSCH bandwidth

RAN1#114 meeting has agreed that for R18 RedCap UEs supporting FG48-1, broadcast MBS PDSCH without PDSCH in consecutive slots and without PDSCH repetition can be larger than maximum RB number. There has been no agreement on broadcast MBS PDSCH bandwidth in following case.

  - Case 1: Broadcast MBS PDSCH with PDSCH in consecutive slots and/or with PDSCH repetition

- For case 1, if broadcast MBS PDSCH is not scheduled in every slot, R18 RedCap UEs supporting FG48-1 can buffer broadcast MBS PDSCH firstly and process them without timeline requirement. Besides, considering coexistence of R17 and R18 RedCap UEs, if bandwidth of case 1 is less than maximum RB number, scheduling of broadcast MBS PDSCH of R17 RedCap UEs is restricted to be less than maximum RB number or without PDSCH repetition. To avoid imposing too much scheduling restriction on broadcast MBS PDSCH of R17 RedCap UEs, bandwidth larger than maximum RB number for case 1 is preferred.

- According to existing CR 38.213, R18 RedCap UEs supporting FG48-1 is not required to process a PDSCH reception in slot n if there is repetitions or another PDSCH in slot n+1. In our understanding, gNB can still schedule case 1 with broadcast MBS PDSCH bandwidth larger than 5MHz, FG48-1 UE does not process broadcast MBS PDSCH in slot n but can process PDSCH in subsequent slot.

38.213 section 17.1A A UE that has not indicated FG 48-2 is not required to process a PDSCH reception in slot n that is scheduled by a DCI format with CRC scrambled by a G-RNTI for broadcast over a number of PRBs that is larger than 25 PRBs for 15 kHz SCS, or larger than 12 PRBs for 30 kHz SCS, when the PDSCH reception is with repetitions or when the UE receives another PDSCH in slot n+1.

- Observation 1: For UE with BB bandwidth reduction, the number of PRBs scheduled in DCI can be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS for the following case:

  - Case 1: Broadcast MBS PDSCH with PDSCH in consecutive slots and/or with PDSCH repetition

2.2 MBS simultaneous reception

Last meeting has agreed that simultaneous reception of broadcast/multicast PDSCH and unicast depends on UE capability when the total number of PRBs is smaller than the maximum number of PRBs. One remaining issue is simultaneous MBS broadcast/multicast and unicast when the total number of PRBs exceeds the maximum number of PRBs that the UE can receive or process per slot.

Since broadcast has no processing timeline requirement and no HARQ feedback, UE can buffer broadcast and process it later, thus simultaneous reception of broadcast and unicast can depend on UE capability.

In R18 MBS topic, FDMed CG multicast and DG unicast, FDMed DG multicast and CG unicast, FDMed CG multicast and CG unicast are not allowed, FDMed DG multicast and DG unicast is allowed. Since there is restriction on maximum number of PRBs of R18 RedCap UEs, if bandwidth of FDMed DG multicast and DG unicast is larger than maximum number of PRBs and multicast has HARQ feedback, decoding unicast firstly may impact HARQ of multicast. One solution is gNB avoids scheduling unicast to be FDMed with DG multicast if total PRB is larger than maximum number of PRBs. In this way, when R18 RedCap UEs and legacy UEs are in the same multicast group, the scheduling of multicast of legacy UEs will not be impacted.

Proposal 1: An eRedCap UE with bandwidth reduction, depending on indicated UE capability, the UE can decode a PDSCH for MBS broadcast and a PDSCH for unicast with the two PDSCH partially or fully overlapping in time in non-overlapping PRBs, if the total number of PRBs exceeds the maximum number of PRBs that the UE can receive or process per slot.

Proposal 2: An eRedCap UE with bandwidth reduction does not expect a PDSCH for unicast to be partially or fully overlapping in time in non-overlapping PRBs with a PDSCH for MBS multicast, if the total number of PRBs exceeds the maximum number of PRBs that the UE can receive or process per slot.

TP suggestion for TS 38.214 section 5.1 is as the following:

<Unchanged text is omitted>

The maximum number of PDSCHs scheduled per slot per component carrier with C-RNTI/CS-RNTI and G-RNTI/G-CS-RNTI/MCCH-RNTI that the UE shall be able to decode is the same as the indicated UE capability for the number of unicast PDSCHs per slot per component carrier. If the UE is capable of receiving FDMed unicast and multicast PDSCH per slot per carrier, the UE shall be able to decode a PDSCH scheduled by a DCI format with C-RNTI or a PDSCH scheduled for a retransmission of a TB by a DCI format with CS-RNTI and a PDSCH scheduled by a DCI format with G-RNTI for multicast or a PDSCH scheduled for a retransmission of a TB by a DCI format with G-CS-RNTI that partially or fully overlap in time in non-overlapping PRBs. If the UE is capable of receiving FDMed unicast and broadcast PDSCH per slot per carrier, the UE shall be able to decode a PDSCH scheduled by a DCI format with C-RNTI or a PDSCH scheduled for a retransmission of a TB by a DCI format with CS-RNTI and a PDSCH scheduled with G-RNTI for broadcast/MCCH-RNTI that partially or fully overlap in time in non-overlapping PRBs.

For a reduced capability UE that indicates supportOfRedCap-r18 but not indicating FG 48-2, if the UE is capable of receiving FDMed unicast and multicast/broadcast PDSCH per slot, the UE can decode the two PDSCHs, with the two PDSCHs partially or fully overlapping in time in non-overlapping PRBs, if the total number of PRBs allocated is no more than 25 PRBs when configured with SCS m = 0 or no more than 12 PRBs when configured with SCS m = 1.

For a reduced capability UE that indicates supportOfRedCap-r18 but not indicating FG 48-2, the UE does not expect to decode a PDSCH scheduled by a DCI format with C-RNTI or a PDSCH scheduled for a retransmission of a TB by a DCI format with CS-RNTI that partially or fully overlaps in time in non-overlapping PRBs with a PDSCH scheduled by a DCI format with G-RNTI for multicast or a PDSCH scheduled for a retransmission of a TB by a DCI format with G-CS-RNTI, if the total number of PRBs allocated is more than 25 PRBs when configured with SCS m = 0 or more than 12 PRBs when configured with SCS m = 1.

<Unchanged text is omitted>

2.3 PUSCH bandwidth

Since RAN#99 concludes that UE supporting FG 48-1 and UE supporting FG 48-2 reuse the same initial access procedure, and these two kinds of UEs are not distinguished during initial access, PUSCH bandwidth of UE supporting FG 48-2 is also restricted within RB number of 5MHz during initial access.

One remaining issue is whether PUSCH bandwidth of UE supporting FG 48-2 can be larger than RB number of 5MHz after initial access. For CBRA after initial access, PUSCH bandwidth of UE supporting FG 48-2 is still less than RB number of 5MHz since gNB can not distinguish these two kinds of UEs during CBRA. For CFRA after initial access, RACH resource is dedicated configured, thus PUSCH bandwidth of UE supporting FG 48-2 should follow UE capability and can be larger than RB number of 5MHz. In CFRA with 4-step random access, there is no Msg3 PUSCH. In CFRA with 2-step random access, MsgA PUSCH can be larger than RB number of 5MHz.

Proposal 3: For UE without BB bandwidth reduction, for MsgA PUSCH of CFRA, the number of PRBs that the UE can transmit per slot or per hop can be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

For UE supporting FG 48-2 in inactive or connected mode, following TP is suggested to restrict PUSCH bandwidth during CBRA.

TP suggestion for TS 38.213 section 17.1A is as the following:

<Unchanged text is omitted>

A UE that indicated FG 48-2 does not expect to transmit a PUSCH over a bandwidth that is larger than 25 PRBs for 15 kHz SCS, or larger than 12 PRBs for 30 kHz SCS, per hop in a slot, where the PUSCH is scheduled by RAR UL grant or by a DCI scrambled by a TC-RNTI, or is configured for a Type-2 random access procedure and not configured in CFRA-TwoStep-r16.

<Unchanged text is omitted>

2.4 random access timeline

For UE without BB bandwidth reduction, after initial access, random access timeline is not extended. Following TP is suggested for TS 38.213 section 17.1A to exclude random access timeline relaxation for UE has indicated FG48-2.

<Unchanged text is omitted>

When

-	a UE has not indicated FG48-2 receives a PDSCH scheduled by a DCI format with CRC scrambled by a RA-RNTI or a MsgB-RNTI over a number of PRBs that is larger than 25 PRBs for 15 kHz SCS or larger than 12 PRBs for 30 kHz SCS, and

-	the PDSCH includes a RAR message with an RAR UL grant scheduling a Msg3 PUSCH transmission from the UE, as described in Clauses 8.2 and 8.2A

the UE transmits the Msg3 PUSCH if a time between the last symbol of a PDSCH reception conveying the RAR message and the first symbol of the Msg3 PUSCH transmission is not smaller than NT,1+NT,2+1.5 msec for 15 kHz SCS or NT,1+NT,2+1.0 msec for 30 kHz SCS where NT,1 and NT,2 are defined in clause 8.3; otherwise, the UE behaviour is based on UE implementation.

When

-	a UE has not indicated FG48-2 receives a PDSCH scheduled by a DCI format with CRC scrambled by a RA-RNTI or a MsgB-RNTI over a number of PRBs that is larger than 25 PRBs for 15 kHz SCS or larger than 12 PRBs for 30 kHz SCS, and

-	the UE does not correctly receive the transport block provided by the PDSCH, or if the higher layers at the UE do not identify a RAPID associated with a corresponding PRACH transmission from the UE

if requested by higher layers, the UE shall be ready to transmit a PRACH no later than NT,1+1.75 msec for 15 kHz SCS, or no later than NT,1+1.25 msec for 30 kHz SCS, after the last symbol of the PDSCH reception, or after the last symbol of the window as described in Clauses 8.2 and 8.2A.

When

-	a UE has not indicated FG48-2 receives a PDSCH scheduled by a DCI format with CRC scrambled by MsgB-RNTI over a number of PRBs that is larger than 25 PRBs for 15 kHz SCS or larger than 12 PRBs for 30 kHz SCS, and

-	the PDSCH includes a RAR message that is for successRAR for the UE as described in Clause 8.2A

the UE transmits a PUCCH with HARQ-ACK information if a time between the last symbol of the PDSCH reception conveying the RAR message and the first symbol of the PUCCH transmission is not smaller than NT,1+1.5 msec for 15 kHz SCS or NT,1+1.0 msec for 30 kHz SCS; otherwise, the UE behaviour is based on UE implementation.

<Unchanged text is omitted>

2.5 clarification of description of UE capability

- Regarding draft CR 38.213 clause 17.1A, understanding of “A UE that has not indicated FG 48-2” is not aligned. In our understanding, the wording now includes UE supporting FG 48-1 in all RRC mode, and UE supporting FG 48-2 in idle mode. The wording “A UE that indicated FG 48-2” means UE supporting FG 48-2 in inactive or connected mode.

- We prefer to clarify the description of UE capability instead of changing existing wording to “UE supporting FG 48-1/48-2” to avoid checking whole clause 17.1A and adding many new paragraphs.

- For example, if existing wording is changed to “UE supporting FG 48-1” in following paragraph, another paragraph is needed to restrict broadcast MBS PDSCH processing with PDSCH in consecutive slots and/or with PDSCH repetition for FG 48-2 UE in idle mode.

38.213 section 17.1A A UE that has not indicated FG 48-2 is not required to process a PDSCH reception in slot n that is scheduled by a DCI format with CRC scrambled by a G-RNTI for broadcast over a number of PRBs that is larger than 25 PRBs for 15 kHz SCS, or larger than 12 PRBs for 30 kHz SCS, when the PDSCH reception is with repetitions or when the UE receives another PDSCH in slot n+1.

- If existing wording is changed to “UE supporting FG 48-1” in following paragraph, another paragraph is needed to restrict PUSCH bandwidth for FG 48-2 UE in idle mode.

38.213 clause 17.1A A UE that has not indicated FG 48-2 does not expect to transmit a PUSCH over a bandwidth that is larger than 25 PRBs for 15 kHz SCS, or larger than 12 PRBs for 30 kHz SCS, per hop in a slot.

- Above all, following clarification of description of UE capability is needed.

- Proposal 4: Clarify that “A UE that has not indicated FG 48-2” includes UE supporting FG 48-1 in all RRC mode and UE supporting FG 48-2 in idle mode. Clarify that “A UE that has indicated FG 48-2” includes UE supporting FG 48-2 in inactive or connected mode.

- Conclusion

- In this contribution, we give our consideration on MBS PDSCH bandwidth, MBS simultaneous reception, PUSCH bandwidth, and the following proposals and spec corrections are made.

- Observation 1: For UE with BB bandwidth reduction, the number of PRBs scheduled in DCI can be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS for the following case:

  - Case 1: Broadcast MBS PDSCH with PDSCH in consecutive slots and/or with PDSCH repetition

Proposal 1: An eRedCap UE with bandwidth reduction, depending on indicated UE capability, the UE can decode a PDSCH for MBS broadcast and a PDSCH for unicast with the two PDSCH partially or fully overlapping in time in non-overlapping PRBs, if the total number of PRBs exceeds the maximum number of PRBs that the UE can receive or process per slot.

Proposal 2: An eRedCap UE with bandwidth reduction does not expect a PDSCH for unicast to be partially or fully overlapping in time in non-overlapping PRBs with a PDSCH for MBS multicast, if the total number of PRBs exceeds the maximum number of PRBs that the UE can receive or process per slot.

TP suggestion for TS 38.214 section 5.1 is as the following:

<Unchanged text is omitted>

The maximum number of PDSCHs scheduled per slot per component carrier with C-RNTI/CS-RNTI and G-RNTI/G-CS-RNTI/MCCH-RNTI that the UE shall be able to decode is the same as the indicated UE capability for the number of unicast PDSCHs per slot per component carrier. If the UE is capable of receiving FDMed unicast and multicast PDSCH per slot per carrier, the UE shall be able to decode a PDSCH scheduled by a DCI format with C-RNTI or a PDSCH scheduled for a retransmission of a TB by a DCI format with CS-RNTI and a PDSCH scheduled by a DCI format with G-RNTI for multicast or a PDSCH scheduled for a retransmission of a TB by a DCI format with G-CS-RNTI that partially or fully overlap in time in non-overlapping PRBs. If the UE is capable of receiving FDMed unicast and broadcast PDSCH per slot per carrier, the UE shall be able to decode a PDSCH scheduled by a DCI format with C-RNTI or a PDSCH scheduled for a retransmission of a TB by a DCI format with CS-RNTI and a PDSCH scheduled with G-RNTI for broadcast/MCCH-RNTI that partially or fully overlap in time in non-overlapping PRBs.

For a reduced capability UE that indicates supportOfRedCap-r18 but not indicating FG 48-2, if the UE is capable of receiving FDMed unicast and multicast/broadcast PDSCH per slot, the UE can decode the two PDSCHs, with the two PDSCHs partially or fully overlapping in time in non-overlapping PRBs, if the total number of PRBs allocated is no more than 25 PRBs when configured with SCS m = 0 or no more than 12 PRBs when configured with SCS m = 1.

For a reduced capability UE that indicates supportOfRedCap-r18 but not indicating FG 48-2, the UE does not expect to decode a PDSCH scheduled by a DCI format with C-RNTI or a PDSCH scheduled for a retransmission of a TB by a DCI format with CS-RNTI that partially or fully overlaps in time in non-overlapping PRBs with a PDSCH scheduled by a DCI format with G-RNTI for multicast or a PDSCH scheduled for a retransmission of a TB by a DCI format with G-CS-RNTI, if the total number of PRBs allocated is more than 25 PRBs when configured with SCS m = 0 or more than 12 PRBs when configured with SCS m = 1.

<Unchanged text is omitted>

Proposal 3: For UE without BB bandwidth reduction, for MsgA PUSCH of CFRA, the number of PRBs that the UE can transmit per slot or per hop can be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

TP suggestion for TS 38.213 section 17.1A is as the following:

<Unchanged text is omitted>

A UE that indicated FG 48-2 does not expect to transmit a PUSCH over a bandwidth that is larger than 25 PRBs for 15 kHz SCS, or larger than 12 PRBs for 30 kHz SCS, per hop in a slot, where the PUSCH is scheduled by RAR UL grant or by a DCI scrambled by a TC-RNTI, or is configured for a Type-2 random access procedure and not configured in CFRA-TwoStep-r16.

<Unchanged text is omitted>

TP suggestion for TS 38.213 section 17.1A is as the following:

<Unchanged text is omitted>

When

-	a UE has not indicated FG48-2 receives a PDSCH scheduled by a DCI format with CRC scrambled by a RA-RNTI or a MsgB-RNTI over a number of PRBs that is larger than 25 PRBs for 15 kHz SCS or larger than 12 PRBs for 30 kHz SCS, and

-	the PDSCH includes a RAR message with an RAR UL grant scheduling a Msg3 PUSCH transmission from the UE, as described in Clauses 8.2 and 8.2A

the UE transmits the Msg3 PUSCH if a time between the last symbol of a PDSCH reception conveying the RAR message and the first symbol of the Msg3 PUSCH transmission is not smaller than NT,1+NT,2+1.5 msec for 15 kHz SCS or NT,1+NT,2+1.0 msec for 30 kHz SCS where NT,1 and NT,2 are defined in clause 8.3; otherwise, the UE behaviour is based on UE implementation.

When

-	a UE has not indicated FG48-2 receives a PDSCH scheduled by a DCI format with CRC scrambled by a RA-RNTI or a MsgB-RNTI over a number of PRBs that is larger than 25 PRBs for 15 kHz SCS or larger than 12 PRBs for 30 kHz SCS, and

-	the UE does not correctly receive the transport block provided by the PDSCH, or if the higher layers at the UE do not identify a RAPID associated with a corresponding PRACH transmission from the UE

if requested by higher layers, the UE shall be ready to transmit a PRACH no later than NT,1+1.75 msec for 15 kHz SCS, or no later than NT,1+1.25 msec for 30 kHz SCS, after the last symbol of the PDSCH reception, or after the last symbol of the window as described in Clauses 8.2 and 8.2A.

When

-	a UE has not indicated FG48-2 receives a PDSCH scheduled by a DCI format with CRC scrambled by MsgB-RNTI over a number of PRBs that is larger than 25 PRBs for 15 kHz SCS or larger than 12 PRBs for 30 kHz SCS, and

-	the PDSCH includes a RAR message that is for successRAR for the UE as described in Clause 8.2A

the UE transmits a PUCCH with HARQ-ACK information if a time between the last symbol of the PDSCH reception conveying the RAR message and the first symbol of the PUCCH transmission is not smaller than NT,1+1.5 msec for 15 kHz SCS or NT,1+1.0 msec for 30 kHz SCS; otherwise, the UE behaviour is based on UE implementation.

<Unchanged text is omitted>

- Proposal 4: Clarify that “A UE that has not indicated FG 48-2” includes UE supporting FG 48-1 in all RRC mode and UE supporting FG 48-2 in idle mode. Clarify that “A UE that has indicated FG 48-2” includes UE supporting FG 48-2 in inactive or connected mode.

- References

- RAN1 Chairman’s Notes, RAN1#114b eom3, October 9th–October 13th, 2023.