3GPP TSG RAN WG1 #115	                                    		                        R1-2311533

Chicago, USA Nov 13 – Nov 17, 2023

Agenda Item:	8.6.1

Source:	InterDigital, Inc.

Title:	Remaining issues on XR-specific capacity enhancements

Document for:	Discussion and Decision

# Introduction

In RAN1#114bis [1], several remaining issues for Rel-18 XR enhancements were addressed and the agreements/conclusions are provided in the Appendix. In this contribution, we discuss the following remaining issues:

- Repetitions in multi-PUSCH CG

- UTO-UCI indication and CG release/deactivation

# Discussion on remaining issues

### 2.1 Repetitions in multi-PUSCH CG

In last RAN1#114bis meeting, the following TP was discussed [2] on repetitions for multi-PUSCH CG. This TP corresponds to conclusion made during RAN1#114 [3] where for Type-1 and Type-2 multi-PUSCH CG configuration, Type-A repetition is NOT supported in Rel-18.

#### 6.1.2.3 Resource allocation for uplink transmission with configured grant ******************** unchnaged text omitted ***************************** For PUSCH transmissions with a Type 1 or Type 2 configured grant, the number of (nominal) repetitions K to be applied to the transmitted transport block is provided by the indexed row in the time domain resource allocation table if numberOfRepetitions is present in the table; otherwise K is provided by the higher layer configured parameters repK. For a configuredGrantConfig, If if a UE is configured with higher layer parameter [nrofSlots_InCGperiod], in a configuredGrantConfig, the UE assumes the repetition factor equal to one and does not support repetition, and the UE does not support neither repetition of the transmitted transport block nor the TB processing over multiple slots is supported for the configuredGrantConfig. ******************** unchnaged text omitted *****************************

The TP above is also indended to avoid the ambiguity at UE on whether to apply repetition for multi-PUSCH CG when the TDRA table may otherwise indicate a repetition factor greater than one.

In our view, indicating in the TP that “the UE assumes the repetition factor equal to one”, and then “UE does not support repetition” seems unnecessary. For clarity and to avoid any misinterpretation with the text leading up to the TP (e.g. on repetitions and repK), we prefer removing the text “the UE assumes the repetition factor equal to one” from the TP.

Proposal 1: Adopt the following TP under Clause 6.1.2.3 in TS 38.214

#### 6.1.2.3 Resource allocation for uplink transmission with configured grant ******************** unchnaged text omitted ***************************** For PUSCH transmissions with a Type 1 or Type 2 configured grant, the number of (nominal) repetitions K to be applied to the transmitted transport block is provided by the indexed row in the time domain resource allocation table if numberOfRepetitions is present in the table; otherwise K is provided by the higher layer configured parameters repK. For a configuredGrantConfig, If if a UE is configured with higher layer parameter [nrofSlots_InCGperiod], in a configuredGrantConfig, the UE does not support repetition, and the UE does not support neither repetition of the transmitted transport block nor the TB processing over multiple slots is supported for the configuredGrantConfig. ******************** unchnaged text omitted *****************************

### 2.2 UTO-UCI indication and CG release/deactivation

In the last RAN1#114bis meeting, the following TP were discussed [2]. This TP corresponds to the proposals made in [4].

### 9.3.1 UE procedure for reporting UTO-UCI If the UE is provided nrof_UTO_UCI with value equal to OUTO-UCI in configuredGrantConfig of a CG-PUSCH configuration, the UE multiplexes UTO-UCI represented by a bitmap of OUTO-UCI bits in each CG-PUSCH transmission for the CG-PUSCH configuration. The OUTO-UCI bits of UTO-UCI, o0UTO-UCI, o1UTO-UCI, …, oOUTO-UCI-1UTO-UCI, have a one-to-one mapping to OUTO-UCI subsequent CG-PUSCH TOs in ascending order of start time. For unpaired spectrum operation, the OUTO-UCI subsequent CG-PUSCH TOs exclude invalid ones where a UE does not transmit a PUSCH based on the procedures in Clause 11.1. A bit value of ‘0’ indicates that the UE may transmit CG-PUSCH, and a bit value of ‘1’ indicates that the UE will not transmit CG-PUSCH, in a corresponding CG-PUSCH TO. When the UE indicates by UTO-UCI a value of ‘1’ for a CG-PUSCH TO, the UE continues to indicate the value of ‘1’ for the CG-PUSCH TO by UTO-UCI multiplexed in subsequent CG-PUSCH transmissions, and the UE does not transmit CG-PUSCH in the CG-PUSCH TO. For a Type-2 CG-PUSCH configuration an indicated UTO-UCI bit in a CG-PUSCH transmission is applicable if the corresponding CG PUSCH TO occurs before the UE receives a DCI format that indicates a release for the Type-2 CG PUSCH.

Based on the discussion during RAN1#114bis [1], it is clear that the UTO-UCI indication is valid only for the activated CG configuration, and there is no need to assume any new consequences regarding a previously transmitted UTO-UCI when the CG configuration is re-activated (upon deactivation) or released. The UE can assume the TOs become invalid regardless of whether they were previously indicated in UTO-UCI as used or unused upon deactivation or release of the CG configuration. As such, the TP is not necessary.

Proposal 2: Do not adopt the proposed TP on UTO-UCI indication and CG release/deactivation under Clause 9.3.1 in TS 38.213.

# Conclusion

In this contribution, the following proposals are made:

Proposal 1: Adopt the following TP under Clause 6.1.2.3 in TS 38.214

Proposal 2: Do not adopt the proposed TP on UTO-UCI indication and CG release/deactivation under Clause 9.3.1 in TS 38.213.

# Appendix

The following show the conclusions and agreements made during RAN1#114bis [1]:

Agreement Adopt TP1-1 below for Clause 6.1 of 38.214: ## 6.1 UE procedure for transmitting the physical uplink shared channel ************** Unchanged parts omitted************** When the UE is configured dl-OrJointTCI-StateList or ul-TCI-StateList, the UE shall perform PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant or a dynamic grant according to the spatial relation, if applicable, with a reference to the RS for determining UL Tx spatial filter. The RS is determined based on an RS configured with qcl-Type set to 'typeD' of the indicated TCI-State or an RS in the indicated TCI-UL-State. The reference RS in the indicated TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. The reference RS in the indicated TCI-UL-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, an SRS resource in an SRS resource set with the higher layer parameter usage set to 'beamManagement', or SS/PBCH block associated with the same or different PCI from the PCI of the serving cell. When [nrofSlots_InCGperiod] is configured for Type 1 configured grant or Type 2 configured grant, HARQ process ID for the Kth (1 < K ≤ [nrofSlots_InCGperiod]) valid configured PUSCH grant is determined as in clause 5.4.1 of [10, TS 38.321], excluding invalid configured PUSCH grant(s) that are not transmitted due to collision with the DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol(s) of an SS/PBCH block with index provided by ssb-PositionsInBurst as described in clause 11.1 of [6, TS 38.213]. ************** Unchanged parts omitted**************. Reason for change:; For determination of HARQ process ID for a multi-PUSCHs CG, the current specifications refer to the procedures in clause 11.1 of 38.213 which includes cases corresponding to collision with dynamic as well as semi-static transmissions or symbol direction indications. It is important to determine whether a CG PUSCH TO is valid or invalid for HARQ process ID determination of a multi-PUSCHs CG. In the corresponding agreements, it was clarified by the following Note the cases which are relevant for determining valid/invalid CG PUSCH TOs for HARQ process ID determination: Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB. Otherwise, it is valid. Hence, it should be clarified which collision cases in clause 11.1 are relevant for this purpose.. Summary of change:; Add description in clause 6.1 that for the procedures in clause 11.1, the CG PUSCH TO collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB results in an invalid CG PUSCH TO.. Consequences if not approved:; The definition of an invalid CG PUSCH has not been clearly captured in the specifications and results in inconsistency for the associated HARQ process ID determination procedures.. Agreement Adopt TP1-2 below for Clause 9.3.1 of 38.213: ### 9.3.1 UE procedure for reporting UTO-UCI If the UE is provided nrof_UTO_UCI with value equal to OUTO-UCI in configuredGrantConfig of a CG-PUSCH configuration, the UE multiplexes UTO-UCI represented by a bitmap of OUTO-UCI bits in each CG-PUSCH transmission for the CG-PUSCH configuration. The OUTO-UCI bits of UTO-UCI, o0UTO-UCI, o1UTO-UCI, …, oOUTO-UCI-1UTO-UCI, have a one-to-one mapping to OUTO-UCI subsequent CG-PUSCH TOs in ascending order of start time. For unpaired spectrum operation, the OUTO-UCI subsequent CG-PUSCH TOs exclude invalid ones where a UE does not transmit a PUSCH due to collision of the PUSCH with the DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol(s) of an SS/PBCH block with index provided by ssb-PositionsInBurst based on the procedures in Clause 11.1. A bit value of ‘0’ indicates that the UE may transmit CG-PUSCH, and a bit value of ‘1’ indicates that the UE will not transmit CG-PUSCH, in a corresponding CG-PUSCH TO. When the UE indicates by UTO-UCI a value of ‘1’ for a CG-PUSCH TO, the UE continues to indicate the value of ‘1’ for the CG-PUSCH TO by UTO-UCI multiplexed in subsequent CG-PUSCH transmissions, and the UE does not transmit CG-PUSCH in the CG-PUSCH TO.. Reason for change:; For UTO-UCI indication for a configured grant, the current specifications refer to the procedures in clause 11.1 of 38.213 which includes cases corresponding to collision with dynamic as well as semi-static transmissions or symbol direction indications. It is important to determine whether a CG PUSCH TO is valid or invalid since the UTO-UCI indication is applicable only to valid CG PUSCH TOs. In the corresponding agreements, it was clarified by the following Note the cases which are relevant for determining valid/invalid CG PUSCH TOs for UTO-UCI indication: Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB. Otherwise, it is valid. Hence, it should be clarified which collision cases in clause 11.1 are relevant for this purpose.. Summary of change:; Add description in clause 9.3.1 that for the procedures in clause 11.1, the CG PUSCH TO collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB results in an invalid CG PUSCH TO.. Consequences if not approved:; The definition of an invalid CG PUSCH has not been clearly captured in the specifications and results in inconsistency for the associated UTO-UCI indication procedures.. Agreement Rel-18 multi-PUSCH CG is not supported for operation on shared spectrum. - Capture the above in description of RAN1 higher layer parameter list for nrofSlots_InCGperiod Agreement Adopt TP4-1 below for Clause 6.3.2.1.4 of 38.212: ##### 6.3.2.1.4 HARQ-ACK and CG-UCI/UTO-UCI If the higher layer parameter nrof_UTO_UCI is configured, the procedure in this clause 6.3.2.1.4 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, and replacing "When higher layer parameter cg-UCI-Multiplexing is configured" with "When UTO-UCI and HARQ-ACK have the same priority index and are jointly encoded and transmitted on a PUSCH". *************** unchanged omitted *********************. Reason for change:; The procedures in clause 6.3.2.1.4 for CG-UCI can be reused for UTO-UCI. However, the current specification does not clarify that the procedure is this clause is applicable when UTO-UCI and HARQ-ACK have the same priority and are jointly encoded.. Summary of change:; Clarify that the procedures in clause 6.3.2.1.4 are applicable when UTO-UCI and HARQ-ACK have the same priority and are jointly encoded.. Consequences if not approved:; Inconsistent and ambiguous UE behaviour. Agreement Adopt TP5-1 below for Clause 6.3.2.4.1 of 38.212: ##### 6.3.2.4.1 UCI encoded by Polar code If the higher layer parameter nrof_UTO_UCI is configured, the procedures in this clause and the clauses it refers to apply by replacing CG-UCI with UTO-UCI in all the notations and texts, when applicable. ###### 6.3.2.4.1.1 HARQ-ACK For HARQ-ACK transmission on PUSCH not using repetition type B with UL-SCH and if numberOfSlotsTBoMS is not present in the resource allocation table, or if numberOfSlotsTBoMS is present in the resource allocation table and the value of numberOfSlotsTBoMS in the row indicated by the Time domain resource assignment field in DCI is equal to 1, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as , is determined as follows: ****************** unchnaged omitted ***********************. Reason for change:; The maximum length of UTO-UCI bit sequence is 8, which is not larger than 11. Hence, polar code is not applicable to UTO-UCI when it is not jointly encoded with HARQ-ACK. However, when UTO-UCI is jointly encoded with HARQ-ACK, depending on the size of HARQ-ACK code book, the UTO-UCI and HARQ-ACK sequences together may result in a code book with a size larger than 11 bits. In this case Polar codes should be applied for encoding. Currently, joint encoding of UTO-UCI and HARQ-ACK with Polar code is missing from the specification.. Summary of change:; Include joint encoding of UTO-UCI and HARQ-ACK with Polar code when applicable.. Consequences if not approved:; Unspecified UE behaviour for jointly encoding UTO-UCI and HARQ-ACK with more than 11 bits.. Agreement Adopt TP3-1 below for Clause 6.3.2.1.3A of 38.212: Reason for change:; In TS 38.212, there are two “given by clause x.x of [5, TS 38.213]” in Clause 6.3.2.1.3A and Clause 6.3.2.1.5. As the corresponding clause has been updated in TS 38.213, the incomplete parts in TS 38.212 should be fixed.. Summary of change:; Fix the two incomplete clause references of TS 38.213 in TS 38.212.. Consequences if not approved:; The references in specifications are unclear. ##### 6.3.2.1.3A UTO-UCI For UTO-UCI bits transmitted on a CG PUSCH when the higher layer parameter nrof_UTO_UCI is configured, the UTO-UCI bit sequence a0, a1, a2, a3, …,aA-1 is determined as follows: - set ai=oiUTO-UCI for i=0,1, …, OUTO-UCI-1 and A=OUTO-UCI, where OUTO-UCI is provided by nrof_UTO_UCI, and the UTO-UCI bit sequence o0UTO-UCI, o1UTO-UCI, …, oOUTO-UCI-1UTO-UCI is given by clause x.x9.3.1 of [5, TS 38.213]. ******************** unchnaged text omitted ***************************** ##### 6.3.2.1.5 UCI with different priority indexes If the higher layer parameter nrof_UTO_UCI is configured, the procedure in this clause 6.3.2.1.5 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, and replacing "is given by Table 6.3.2.1.3-1 mapped in the order from upper part to lower part" with "is given by clause x.x9.3.1 of [5, TS 38.213]". ******************** unchnaged text omitted *****************************. R1-2310425 Moderator Summary#2 - Maintenance of XR Enhancements Moderator (Ericsson) Agreement Adopt TP4-2 below for Clause 6.3.2.7 of 38.212: Reason for change:; The procedures in clause 6.3.2.7 for CG-UCI can be reused for UTO-UCI. However, the following highlighted case described in this clause is not applicable to UTO-UCI since UTO-UCI has the same priority as the CG-PUSCH that is multiplexed in: If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1 and/or CG-UCI associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH, - if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 1, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 as HARQ-ACK, and taking HARQ-ACK with priority index 0 as CSI part 2; - otherwise, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 if any as HARQ-ACK, taking CG-UCI associated with priority index 1 if any as CG-UCI, taking HARQ-ACK with priority index 0 as CSI part 1, and taking CSI part 1 as CSI part 2 if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0.. The inconsistency can be resolved by considering applicable cases for UTO-UCI when the corresponding CG-UCI procedures can be reused.. Summary of change:; Add “when applicable” to the condition to resue the CG-UCI procedures for UTO-UCI.. Consequences if not approved:; Inconsistent and ambiguous UE behaviour. #### 6.3.2.7 Multiplexing of coded UCI bits with different priority indexes to PUSCH If the higher layer parameter nrof_UTO_UCI is configured, the procedure in this clause 6.3.2.7 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, when applicable. ******************** unchnaged text omitted *****************************.

# Reference

[1]	RAN1 chair notes, RAN1#114bis, Oct 2023

[2]	R1-2310425, Moderator Summary – Maintenance of XR Enhancements, Ericsson

[3]	RAN1 chair notes, RAN1#114, Aug 2023

[4]	R1-2309304, Remaining issues on XR-specific capacity enhancements, LG Electronics