3GPP TSG RAN WG1 #115		R1-2311571

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:	8.1.4.2

Source:	Google

Title:	On SRI/TPMI Indication for 8Tx Transmission

Document for:	Discussion/Decision

# Introduction

In RAN1 #114b, the following agreements on SRI/TPMI indication for 8Tx transmission have been achieved.

Agreement Adopt following editorial change for TS 38.211. <omitted unchanged part> Table 6.3.1.5-12: Precoding matrix for codebook1=ng1n4n1 and four-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; (ordered from left to right in increasing order of TPMI index). 0 – 3. <omitted unchanged part>. Agreement Adopt the following text proposal to TS 38.211. <omitted unchanged part> Table 6.3.1.5-45: Intermediate precoding matrix for codebook3 and seven-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix. 0 – 731. <omitted unchanged part>. Agreement Adopt the following text proposal to TS 38.211. 6.3.1.5 Precoding < omitted unchanged parts > Table 6.3.1.5-44: Intermediate precoding matrix for codebook3 and six-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix. 0 – 7. 8 – 15. 16 – 79. < omitted unchanged parts >. Agreement Adopt the following text proposal to TS 38.211. 6.3.1.5 Precoding <omitted unchanged part> - the intermediate precoding matrix is given by Tables 6.3.1.5-29 to 6.3.1.5-47 with representing the all-zero matrix with rows and columns; - the submatrices are given by Tables 6.3.1.5-25 to 6.3.1.5-28 and 6.3.1.5-37 to 6.3.1.5-38. The TPMI index used in the tables above is obtained from the DCI scheduling the uplink transmission or the higher layer parameters according to the procedure in [6, TS 38.214]. <omitted unchanged part>. Agreement Adopt the following text proposal to TS 38.212. 5.4.2.1 Bit selection <omitted unchanged part> For one TB for UL-SCH, or for one TB for DL-SCH/PCH except for DL-SCH with PDSCH scheduled by DCI format 4_0/4_1/4_2, - maximum number of layers for one TB for UL-SCH is given by the minimum of X and 4, where - if the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter - elseif the higher layer parameter maxRank of pusch-Config of the serving cell is configured, X is given by the maximum value of maxRank across all BWPs of the serving cell - otherwise, X is given by the maximum number of layers for PUSCH supported by the UE for the serving cell - maximum number of layers for one TB for DL-SCH/PCH is given by the minimum of X and 4, where - if the higher layer parameter maxMIMO-Layers of PDSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter - otherwise, X is given by the maximum number of layers for PDSCH supported by the UE for the serving cell <omitted unchanged part>. Agreement Adopt the following text proposal to TS 38.212. 7.3.1.1.2 Format 0_1 <omitted unchanged part> UL-SCH indicator – 0 or 1 bit as follows - 0 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1; - 1 bit otherwise. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. If a UE does not support triggering SRS only in DCI, except for DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0" and CSI request of all zero(s). If a UE supports triggering SRS only in DCI, except for DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0", CSI request of all zero(s) and SRS request of all zero(s). UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0" when indicated number of PUSCH transmission layers is larger than 4. <omitted unchanged part>. Agreement For an 8TX UE, with Ng=8, configured for full power transmission with ‘fullpowerMode1’, the following precoder is supported. Rank = 4. If additional precoders cannot be agreed in RAN1#114bis, no additional precoders will be introduced in Rel-18. Agreement Adopt the following text proposal to TS 38.212. 7.3.1.1.2 Format 0_1 <Unchanged parts omitted> Table 7.3.1.1.2-5E: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled or maxRank=1 or 2 or 3 if transform precoder is disabled, CodebookType=Codebook1, ULcodebookFC-N1N2 = (4,1) or (2,2) Bit field mapped to index; transform precoder is enabled disabled, or maxRank=1 if transform precoder is disabled enabled; Bit field mapped to index; transform precoder is disabled, and maxRank=2; Bit field mapped to index; transform precoder is disabled, and maxRank=3. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1; 1; 1 layer: TPMI=1; 1; 1 layer: TPMI=1. …; …; …; …; …; …. 15; 1 layer: TPMI=15; 15; 1 layer: TPMI=15; 15; 1 layer: TPMI=15. 16; 2 layer2: TPMI=0; 16; 2 layer2: TPMI=0. 17; 2 layer2: TPMI=1; 17; 2 layer2: TPMI=1. …; …; …; …. 47; 2 layers: TPMI=31; 47; 2 layers: TPMI=31. 48-63; reserved; 48; 3 layers: TPMI=0. 49; 3 layers: TPMI=1. …; …. 71; 3 layers: TPMI=23. 72-127; reserved. <Unchanged parts omitted>. Agreement Adopt following text proposals for TS 38.212. #### 7.3.1.1.2 Format 0_1 <omitted unchanged part> - Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214] - New data indicator – 1 bit - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the transport block 2 is configured for the indicated bandwidth part and the transport block 2 is not configured for the active bandwidth part, the UE assumes zeros are padded when interpreting the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 according to Clause 12 of [5, TS38.213], and the UE ignores the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 for the indicated bandwidth part. <omitted unchanged part>. Agreement Adopt the following TP to TS 38.214. <Unchanged parts omitted> 6.1.1.1 Codebook based UL transmission … A UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission set to 'fullpowerMode1' and codebookSubset or codebookSubsetDCI-0-2 set to 'fullAndPartialAndNonCoherent' simultaneously. A UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission set to 'fullpowerMode1' and CodebookType set to 'Codebook1' simultaneously. The UE shall transmit PUSCH using the same antenna port(s) as the SRS port(s) in the SRS resource(s) indicated by the DCI format 0_1 or 0_2 or by configuredGrantConfig according to clause 6.1.2.3. … <Unchanged parts omitted>. Agreement Adopt the following text proposal to TS 38.212. - Note: The content in this agreement may be further updated in RAN#115 according to RAN1#114b agreements. - Reason for change: The current specifications in 38.212 does not include the agreed precoders for fullpowerMode1. - Summary of change: Addition of new columns to the related existing tables, introducing new tables, and text corresponding to the agreed precoders for fullpowerMode1. - Consequences if not approved: Incomplete support of fullpowerMode1. ===========================Start of text proposal to TS 38.212===================== ##### 7.3.1.1.2 Format 0_1 <Unchanged part omitted> - Precoding information and number of layers – number of bits determined by the following: <Unchanged part omitted> - 8 bits according to Table 7.3.1.1.2-5F for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank; - 6 or 7 or 8 bits according to Table 7.3.1.1.2-5G for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=2, 3 or 4, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank; - 3 bits according to Table 7.3.1.1.2-5H for 8 antenna ports, if CodebookType=Codebook4, transform precoder is enabled or maxRank=1 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower. - 10 bits according to Table 7.3.1.1.2-5I for 8 antenna ports, if CodebookType=Codebook2, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank; - 5, 9 or 10 bits according to Table 7.3.1.1.2-5J for 8 antenna ports, if CodebookType=Codebook2, transform precoder is enabled or maxRank =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRank; - 10 bits according to Table 7.3.1.1.2-5K for 8 antenna ports, if CodebookType=Codebook3, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank; - 4, 7, 9 or 10 bits according to Table 7.3.1.1.2-5L for 8 antenna ports, if CodebookType=Codebook3, transform precoder is enabled or maxRank =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRank; - 6 or 7 or 8 bits according to Table 7.3.1.1.2-5M for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=2, or 3 or 4, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank; - 4 bits according to Table 7.3.1.1.2-5N for 8 antenna ports, if CodebookType=Codebook4, transform precoder is enabled or maxRank=1 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1. - 6, 9 or 10 bits according to Table 7.3.1.1.2-5O for 8 antenna ports, if CodebookType=Codebook2, transform precoder is enabled or maxRank =1, 2, 3, 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1; - 5, 7, 9 or 10 bits according to Table 7.3.1.1.2-5P for 8 antenna ports, if CodebookType=Codebook3, transform precoder is enabled or maxRank =1, 2, 3, or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRank; - 8 or 9 bits according to Table 7.3.1.1.2-5Q for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank; - 10 bits according to Table 7.3.1.1.2-5R for 8 antenna ports, if CodebookType=Codebook2, transform precoder is disabled, maxRank=5, 6, 7 or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank; - 10 bits according to Table 7.3.1.1.2-5S for 8 antenna ports, if CodebookType=Codebook3, transform precoder is disabled, maxRank =5, 6, 7, or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank; <Unchanged part omitted> Table 7.3.1.1.2-5M: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 2, 3 or 4, CodebookType=Codebook4, and ul-FullPowerTransmission configured to fullpowerMode1 Bit field mapped to index; maxRank = 2; Bit field mapped to index; maxRank = 3; Bit field mapped to index; maxRank = 4. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. …; …; …; …; …; …. 7; 1 layer: TPMI=7; 7; 1 layer: TPMI=7; 7; 1 layer: TPMI=7. 8; 2 layers: TPMI=8; 8; 2 layers: TPMI=8; 8; 2 layers: TPMI=8. …; …; …; …; …; …. 35; 2 layers: TPMI=35; 35; 2 layers: TPMI=35; 35; 2 layers: TPMI=35. 36; 1 layer: TPMI=255; 36; 3 layers: TPMI=36; 36; 3 layers: TPMI=36. 37; 2 layers: TPMI=256; …; …; …; …. 38-63; reserved; 91; 3 layers: TPMI=91; 91; 3 layers: TPMI=91. 92; 1 layer: TPMI= 255; 92; 4 layers: TPMI=92. 93; 2 layers: TPMI= 256; …; …. 94; 3 layers: 257; 161; 4 layers: TPMI=161. 95-127; reserved; 162; 1 layer: TPMI=255. 163; 2 layers: TPMI=256. 164; 3 layers: TPMI=257. 165-255; reserved. Table 7.3.1.1.2-5N: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank=1 if transform precoder is disabled, CodebookType=Codebook4, and ul-FullPowerTransmission configured to fullpowerMode1 Bit field mapped to index; Precoding information and number of layers. 0; 1 layer: TPMI=0. …; …. 7; 1 layer: TPMI=7. 8; 1 layer: TPMI= 255. 9-15; reserved. Table 7.3.1.1.2-5O: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank = 1, 2, 3, 4 if transform precoder is disabled, CodebookType=Codebook2, and ul-FullPowerTransmission configured to fullpowerMode1 Bit field mapped to index; Transform precoder is enabled, or maxRank = 1 if transform precoder is disabled; Bit field mapped to index; transform precoder is disabled and maxRank = 2; Bit field mapped to index; transform precoder is disabled and maxRank = 3; Bit field mapped to index; transform precoder is disabled and maxRank = 4. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. …; …; …; …; …; …; …; …. 31; 1 layer: TPMI=31; 31; 1 layer: TPMI=31; 31; 1 layer: TPMI=31; 31; 1 layer: TPMI=31. 32; 1 layer: TPMI= 32; 32; 2 layers: TPMI=0; 32; 2 layers: TPMI=0; 32; 2 layers: TPMI=0. 33-63; reserved; …; …; …; …; …; …. 303; 2 layers: TPMI=271; 303; 2 layers: TPMI=271; 303; 2 layers: TPMI=271. 304; 1 layer: TPMI=32; 304; 3 layers: TPMI=0; 304; 3 layers: TPMI=0. 305-511; reserved; …; …; …; …. 567; 3 layers: TPMI=263; 567; 3 layers: TPMI=263. 568; 1 layer: TPMI=32; 568; 4 layers: TPMI=0. 569-1023; reserved; …; …. 635; 4 layers: TPMI=67. 636; 1 layer: TPMI=32. 637-1023; reserved. Table 7.3.1.1.2-5P: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank = 1, 2, 3, 4 if transform precoder is disabled, CodebookType=Codebook3, and ul-FullPowerTransmission is configured to fullpowerMode1 Bit field mapped to index; Transform precoder is enabled, or maxRank = 1 if transform precoder is disabled; Bit field mapped to index; transform precoder is disabled and maxRank = 2; Bit field mapped to index; transform precoder is disabled and maxRank = 3; Bit field mapped to index; transform precoder is disabled and maxRank = 4. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. …; …; …; …; …; …; …; …. 15; 1 layer: TPMI=15; 15; 1 layer: TPMI=15; 15; 1 layer: TPMI=15; 15; 1 layer: TPMI=15. 16; 1 layer: TPMI= 16; 16; 2 layers: TPMI=0; 16; 2 layers: TPMI=0; 16; 2 layers: TPMI=0. 17-31; reserved; …; …; …; …; …; …. 119; 2 layers: TPMI=103; 119; 2 layers: TPMI=103; 119; 2 layers: TPMI=103. 120; 1 layer: TPMI=16; 120; 3 layers: TPMI=0; 120; 3 layers: TPMI=0. 121; 2 layers: TPMI=104; …; …; …; …. 122-127; reserved; 423; 3 layers: TPMI=303; 423; 3 layers: TPMI=303. 424; 1 layer: TPMI=16; 424; 4 layers: TPMI=0. 425; 2 layers: TPMI=104; …; …. 426; 3 layers: TPMI=304; 703; 4 layers: TPMI=279. 427-511; reserved; 704; 1 layer: TPMI=16. 705; 2 layers: TPMI=104. 706; 3 layers: TPMI=304. 707-1023; reserved. Table 7.3.1.1.2-5Q: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7, 8, CodebookType=Codebook4, and ul-FullPowerTransmission is configured to fullpowerMode1 Bit field mapped to index; maxRank = 5; Bit field mapped to index; maxRank = 6; Bit field mapped to index; maxRank = 7; Bit field mapped to index; maxRank = 8. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. …; …; …; …; …; …; …; …. 7; 1 layer: TPMI=7; 7; 1 layer: TPMI=7; 7; 1 layer: TPMI=7; 7; 1 layer: TPMI=7. 8; 2 layers: TPMI=8; 8; 2 layers: TPMI=8; 8; 2 layers: TPMI=8; 8; 2 layers: TPMI=8. …; …; …; …; …; …; …; …. 35; 2 layers: TPMI=35; 35; 2 layers: TPMI=35; 35; 2 layers: TPMI=35; 35; 2 layers: TPMI=35. 36; 3 layers: TPMI=36; 36; 3 layers: TPMI=36; 36; 3 layers: TPMI=36; 36; 3 layers: TPMI=36. …; …; …; …; …; …; …; …. 91; 3 layers: TPMI=91; 91; 3 layers: TPMI=91; 91; 3 layers: TPMI=91; 91; 3 layers: TPMI=91. 92; 4 layers: TPMI=92; 92; 4 layers: TPMI=92; 92; 4 layers: TPMI=92; 92; 4 layers: TPMI=92. …; …; …; …; …; …; …; …. 161; 4 layers: TPMI=161; 161; 4 layers: TPMI=161; 161; 4 layers: TPMI=161; 161; 4 layers: TPMI=161. 162; 5 layers: TPMI=162; 162; 5 layers: TPMI=162; 162; 5 layers: TPMI=162; 162; 5 layers: TPMI=162. …; …; …; …; …; …; …; …. 217; 5 layers: TPMI=217; 217; 5 layers: TPMI=217; 217; 5 layers: TPMI=217; 217; 5 layers: TPMI=217. 218; 1 layer: TPMI=255; 218; 6 layers: TPMI=218; 218; 6 layers: TPMI=218; 218; 6 layers: TPMI=218. 219; 2 layers: TPMI=256; …; …; …; …; …; …. 220; 3 layers: TPMI=257; 245; 6 layers: TPMI=245; 245; 6 layers: TPMI=245; 245; 6 layers: TPMI=245. 221-255; reserved; 246; 1 layer: TPMI=255; 246; 7 layers: TPMI=246; 246; 7 layers: TPMI=246. 247; 2 layers: TPMI=256; …; …; …; …. 248; 3 layers: TPMI=257; 253; 7 layers: TPMI=253; 253; 7 layers: TPMI=253. 249-255; reserved; 254; 1 layer: TPMI=255; 254; 8 layers: TPMI=254. 255; 2 layers: TPMI=256; 255; 1 layer: TPMI=255. 256; 3 layers: TPMI=257; 256; 2 layers: TPMI=256. 257-511; reserved; 257; 3 layers: TPMI=257. 258-511; reserved. Table 7.3.1.1.2-5R: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7, 8, CodebookType=Codebook2, and ul-FullPowerTransmission is configured to fullpowerMode1 Bit field mapped to index; maxRank = 5; Bit field mapped to index; maxRank = 6; Bit field mapped to index; maxRank = 7; Bit field mapped to index; maxRank = 8. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. …; …; …; …; …; …; …; …. 31; 1 layer: TPMI=31; 31; 1 layer: TPMI=31; 31; 1 layer: TPMI=31; 31; 1 layer: TPMI=31. 32; 2 layers: TPMI=0; 32; 2 layers: TPMI=0; 32; 2 layers: TPMI=0; 32; 2 layers: TPMI=0. …; …; …; …; …; …; …; …. 303; 2 layers: TPMI=271; 303; 2 layers: TPMI=271; 303; 2 layers: TPMI=271; 303; 2 layers: TPMI=271. 304; 3 layers: TPMI=0; 304; 3 layers: TPMI=0; 304; 3 layers: TPMI=0; 304; 3 layers: TPMI=0. …; …; …; …; …; …; …; …. 567; 3 layers: TPMI=263; 567; 3 layers: TPMI=263; 567; 3 layers: TPMI=263; 567; 3 layers: TPMI=263. 568; 4 layers: TPMI=0; 568; 4 layers: TPMI=0; 568; 4 layers: TPMI=0; 568; 4 layers: TPMI=0. …; …; …; …; …; …; …; …. 635; 4 layers: TPMI=67; 635; 4 layers: TPMI=67; 635; 4 layers: TPMI=67; 635; 4 layers: TPMI=67. 636; 5 layers: TPMI=0; 636; 5 layers: TPMI=0; 636; 5 layers: TPMI=0; 636; 5 layers: TPMI=0. …; …; …; …; …; …; …; …. 667; 5 layers: TPMI=31; 667; 5 layers: TPMI=31; 667; 5 layers: TPMI=31; 667; 5 layers: TPMI=31. 668; 1 layer: TPMI=32; 668; 6 layers: TPMI=0; 668; 6 layers: TPMI=0; 668; 6 layers: TPMI=0. 669-1023; reserved; …; …; …; …; …; …. 683; 6 layers: TPMI=15; 683; 6 layers: TPMI=15; 683; 6 layers: TPMI=15. 684; 1 layer: TPMI=32; 684; 7 layers: TPMI=0; 684; 7 layers: TPMI=0. 685-1023; reserved; …; …; …; …. 691; 7 layers: TPMI=7; 691; 7 layers: TPMI=7. 692; 1 layer: TPMI=32; 692; 8 layers: TPMI=0. 693-1023; reserved; …; …. 695; 8 layers: TPMI=3. 696; 1 layer: TPMI=32. 697-1023; reserved. Table 7.3.1.1.2-5S: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7, 8, CodebookType=Codebook3, and ul-FullPowerTransmission is configured to fullpowerMode1 Bit field mapped to index; maxRank = 5; Bit field mapped to index; maxRank = 6; Bit field mapped to index; maxRank = 7; Bit field mapped to index; maxRank = 8. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. …; …; …; …; …; …; …; …. 15; 1 layer: TPMI=15; 15; 1 layer: TPMI=15; 15; 1 layer: TPMI=15; 15; 1 layer: TPMI=15. 16; 2 layers: TPMI=0; 16; 2 layers: TPMI=0; 16; 2 layers: TPMI=0; 16; 2 layers: TPMI=0. …; …; …; …; …; …; …; …. 119; 2 layers: TPMI=103; 119; 2 layers: TPMI=103; 119; 2 layers: TPMI=103; 119; 2 layers: TPMI=103. 120; 3 layers: TPMI=0; 120; 3 layers: TPMI=0; 120; 3 layers: TPMI=0; 120; 3 layers: TPMI=0. …; …; …; …; …; …; …; …. 423; 3 layers: TPMI=303; 423; 3 layers: TPMI=303; 423; 3 layers: TPMI=303; 423; 3 layers: TPMI=303. 424; 4 layers: TPMI=0; 424; 4 layers: TPMI=0; 424; 4 layers: TPMI=0; 424; 4 layers: TPMI=0. …; …; …; …; …; …; …; …. 703; 4 layers: TPMI=279; 703; 4 layers: TPMI=279; 703; 4 layers: TPMI=279; 703; 4 layers: TPMI=279. 704; 5 layers: TPMI=0; 704; 5 layers: TPMI=0; 704; 5 layers: TPMI=0; 704; 5 layers: TPMI=0. …; …; …; …; …; …; …; …. 863; 5 layers: TPMI=159; 863; 5 layers: TPMI=159; 863; 5 layers: TPMI=159; 863; 5 layers: TPMI=159. 864; 1 layer: TPMI=16; 864; 6 layers: TPMI=0; 864; 6 layers: TPMI=0; 864; 6 layers: TPMI=0. 865; 2 layers: TPMI=104; …; …; …; …; …; …. 866; 3 layers: TPMI=304; 943; 6 layers: TPMI=79; 943; 6 layers: TPMI=79; 943; 6 layers: TPMI=79. 867-1023; reserved; 944; 1 layer: TPMI=16; 944; 7 layers: TPMI=0; 944; 7 layers: TPMI=0. 945; 2 layers: TPMI=104; …; …; …; …. 946; 3 layers: TPMI=304; 975; 7 layers: TPMI=31; 975; 7 layers: TPMI=31. 944-1023; reserved; 976; 1 layer: TPMI=16; 976; 8 layers: TPMI=0. 977; 2 layers: TPMI=104; …; …. 978; 3 layers: TPMI=304; 991; 8 layers: TPMI=15. 979-1023; reserved; 992; 1 layer: TPMI=16. 993; 2 layers: TPMI=104. 994; 3 layers: TPMI=304. 995-1023; reserved. ===========================End of text proposal to TS 38.212=====================. Agreement Adopt the following TP to TS 38.212. - Reason for change: The current specifications in 38.212 does not include details related to the SRS resource indicated for fullpowerMode2 with 8 antenna ports. - Summary of change: Addition of a text related to indicated SRS resource for fullpowerMode2 with 8 antenna ports. - Consequences if not approved: Not support of fullpowerMode2. 7.3.1.1.2 Format 0_1 <Unchanged parts omitted> For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports or 8 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field if present, otherwise in an SRS resource set with usage set to 'codebook', and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used. For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 4, and at least one SRS resource with 8 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field if present, otherwise in an SRS resource set with usage set to 'codebook', and an SRS resource with 4 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-2 is used. <Unchanged parts omitted>.

In this contribution, we provide some discussions on SRI/TPMI indication for 8Tx transmission.

# Discussion

## Spatial Domain Fallback operation

For an 8Tx UE, the gNB may configure it to transmit the uplink signal from 2-port or 4-port as a spatial-domain fallback operation. Currently, UE is only able to report the UE capability indicating one supported full power mode and codebook subset. However, for such fallback operation, the UE may support different operations. For example, the UE may support full power mode 1 or mode 2 for 8Tx, but the UE is able to support the full power mode 0 when it is configured for 4Tx operation. Similarly, the UE may support 4-port partial-coherent transmission for 8Tx, but it is able to support full coherent transmission when it is configured for 4Tx operation. Thus, it is necessary to introduce the UE capability for each number of configured ports for UL transmission.

Proposal 3: Support the 8Tx UE to report the UE capability with regard to spatial domain fallback operation, including:

- Supported UL full power mode when it is configured as 2Tx, 4Tx and 8Tx based UL transmission

- Supported codebook coherency subset when it is configured as 2Tx, 4Tx and 8Tx based UL transmission

## UE assistance information for 8Tx UE

Currently, for UE power saving, a UE can report the UE assistance information indicating the UE preferred maximum number of layers as shown below.

UEAssistanceInformation-v1610-IEs ::= SEQUENCE { idc-Assistance-r16 IDC-Assistance-r16 OPTIONAL, drx-Preference-r16 DRX-Preference-r16 OPTIONAL, maxBW-Preference-r16 MaxBW-Preference-r16 OPTIONAL, maxCC-Preference-r16 MaxCC-Preference-r16 OPTIONAL, maxMIMO-LayerPreference-r16 MaxMIMO-LayerPreference-r16 OPTIONAL, minSchedulingOffsetPreference-r16 MinSchedulingOffsetPreference-r16 OPTIONAL, releasePreference-r16 ReleasePreference-r16 OPTIONAL, sl-UE-AssistanceInformationNR-r16 SL-UE-AssistanceInformationNR-r16 OPTIONAL, referenceTimeInfoPreference-r16 BOOLEAN OPTIONAL, nonCriticalExtension UEAssistanceInformation-v1700-IEs OPTIONAL } MaxMIMO-LayerPreference-r16 ::= SEQUENCE { reducedMaxMIMO-LayersFR1-r16 SEQUENCE { reducedMIMO-LayersFR1-DL-r16 INTEGER (1..8), reducedMIMO-LayersFR1-UL-r16 INTEGER (1..4) } OPTIONAL, reducedMaxMIMO-LayersFR2-r16 SEQUENCE { reducedMIMO-LayersFR2-DL-r16 INTEGER (1..8), reducedMIMO-LayersFR2-UL-r16 INTEGER (1..4) } OPTIONAL }

For 8Tx UE, the maximum number of layers increases from 4 to 8 compared to 4Tx UE. Therefore, it is necessary to extend the UE assistance information by indicating maximum number of UL layers {5, 6, 7, 8}.

Proposal 2: Support to extend the candidate value of the reducedMIMO-LayersFR1-UL in OverheatingAssistance and MaxMIMO-LayerPreference from {1…4} to {1…8}

- Send an LS to RAN2 on the extended UE assistance information

# Conclusion

In this contribution, we provided discussion on 8Tx enhancement. Based on the discussion, the following proposals have been achieved.

Proposal 1: Support the 8Tx UE to report the UE capability with regard to spatial domain fallback operation, including:

- Supported UL full power mode when it is configured as 2Tx, 4Tx and 8Tx based UL transmission

- Supported codebook coherency subset when it is configured as 2Tx, 4Tx and 8Tx based UL transmission

Proposal 2: Support to extend the candidate value of the reducedMIMO-LayersFR1-UL in OverheatingAssistance and MaxMIMO-LayerPreference from {1…4} to {1…8}

- Send an LS to RAN2 on the extended UE assistance information