3GPP TSG RAN WG1 #115                                  		   	             	   R1-2311596

Chicago, USA, November 13th - 17th, 2023

Agenda Item:	8.3.1.2

Source:	InterDigital, Inc.

Title:	Remaining issues on measurement and reporting for SL positioning

Document for:	Discussion and Decision

# Introduction

In RAN #98 meeting, the WI for R18 NR Positioning is approved, revised in RANP#101 meeting in RP-232670 which has the following objectives for SL-PRS design and power control for SL-PRS [1]:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2].

In RAN1#114bis meeting, a number of agreements and text proposals were agreed upon [2]. In this contribution, we discuss some of the remaining issues on SL measurement and reporting.

# Discussion

## Synchronization error mitigation for SL-PRS measurements

For SL-TDOA-based SL positioning methods, the RSTD or TDOA measurement accuracy can be degraded if there is a synchronization error between Anchor UEs. To mitigate the impact of synchronization errors between anchor UEs, the following agreements were made in RAN1#114:

Agreement To mitigate the impact of synchronization errors between anchor UEs for SL-PRS based measurement, the exchanged synchronization information of anchor UEs between a UE and LMF or another UE includes the following: - [The synchronization source type (GNSS, gNB/eNB, and UE) of anchor UEs, - If the synchronization source of an anchor UE is SyncRef UE, the anchor UE can optionally indicate the coverage status and synchronization connection status (whether the SyncRef UE is directly or indirectly synchronized to GNSS/gNB, or other SyncRef UE) of the SyncRef UE - If the synchronization source of an anchor UE is gNB, the anchor UE can further provide cell identity information] - [Synchronization quality/accuracy information] - The RTD between anchor UEs Agreement Support to include the following in the exchanged synchronization information of anchor UEs between a UE and LMF or another UE: - The synchronization source type (GNSS, gNB/eNB, and UE) of anchor UEs,

We think the exchanged information can additionally include sync accuracy or synchronization quality information. The information can be very useful for location determination. Based upon this information, different weights may be applied to the measurements of different anchors or to adjust the measurement results in the location determination.

Proposal 1: Make the following change in TS 38.214 to allow anchor UEs exchange the quality of synchronization information. The reason for the change is to mitigate the synchronization error among the anchor UEs for SL-TDoA based methods. The summary of the change is to introduce a new parameter which (anchor) UEs can exchange to mitigate the synchronization error. The consequence if not approved is the poor performance of SL-TDoA due to synchronization error among the anchor UEs.

--------------------------<Start of text proposal for TS 38.214>-------------------------- ### 8.4.4 SL PRS reception procedure < Unchanged parts are omitted > The UE may report synchronization information synchronization source type and/or relative time difference with the associated quality metric and/or synchronization quality/accuracy information, via [higher layer parameter(s)]. For the SL RSTD measurement, the UE may report a reference UE information. --------------------------<End of text proposal for TS 38.214>--------------------------

## Uncertainty in anchor UE location

To determine an absolute position of a target UE in a positioning method, e.g., in TDOA-based positioning, the location information of the anchor UEs are required at the entity performing the positioning computation, e.g., LMF (in-coverage) or the target UE (out-of-coverage). In addition to SL-PRS measurement, an anchor UE can provide its own location information and associated uncertainty information, e.g., to assist determination of the absolute position of a target UE in the SL positioning group. In RAN#112Bis meeting, the following agreement was made [5].

Agreement For provision of assistance information for absolute SL positioning, the anchor UE location information can be provided to LMF or UE. FFS: which UEs can receive the anchor UE location information (note: which may be decided by other WGs) FFS on quality information of anchor UE location information.

The quality or uncertainty information can be provided in various forms. Uncertainty information can be a range of the absolute location, similar to the uncertainty of TRP or ARP locations specified in TS 37.355[6]  or TS 38.455 [7], respectively. Since UE location information can be obtained using the methods of GNSS, Uu-based positioning or SL-based positioning, it can be beneficial for an Anchor UE to provide uncertainty information associated with each used method. In another option, uncertainty information can be indicated with a level of uncertainty with each level corresponding to a degree of uncertainty.

The quality or uncertainty information can be reported to LMF or the target UE. For example, a Target UE can use uncertainty information to select anchor UEs prior to performing a positioning method. In this case, a Target UE can select anchor UEs with the least uncertainty associated with their location information. Thus, we propose to support Anchor UE provides the quality or uncertainty information associated with its reported location information.

We performed numerical evaluation to analyze effect of availability of the uncertainty information of anchor UE’s position at the target UE. Accuracy performance is evaluated in the IIoT small hall setting. We model uncertainty in anchor UE’s position as normal distribution with zero mean and standard deviation of L meters. Anchor UEs are divided into 2 groups as following:

- Anchor UE group having no error: 50 % of the total number of available anchor UEs are assigned to this group.

- Anchor UE group having (L) m of the standard deviation in their error distribution: 50 % of the total number of available anchor UEs are assigned to this group. Error value ε, modeled by zero-mean Normal distribution, per anchor UE is added to actual position of the anchor UE. To model the impact of different level of uncertainty in anchor UE’s position, we sweep different standard deviation values for the Gaussian distribution with L= {0.1 m, 0.25 m, 1 m, 2 m, 4 m, 8 m} for the UEs belonging to this group.

A summary of the evaluation assumptions is presented below. Detailed simulation parameter assumptions are listed in Table A1 in Appendix.

- Channel model: BS-2-UE channel model defined in TR 38.901 is revised (Option 1 agreed in RAN1#109e)

- The target UE selects anchor UEs based on either of the following two criteria:

  - Based on RSRP measurements only

  - Based on RSRP measurements and uncertainty of anchor UE’s position

- Perfect synchronization is assumed among anchor UEs for SL-TDOA

- Anchor UEs of the both group are uniformly distributed across deployment area

In table 1, horizontal accuracy of each anchor UE selection mechanism for different values of standard deviation L of anchor UE’s error distribution is presented.

Table 1 Horizontal accuracy of IIOT sidelink positioning (m), with uncertainty information of anchor UE position at target UE

| Anchor UE selection method | Standard Deviation of positioning error in anchor UE group (m) | 50%ile | 67%ile | 80%ile | 90%ile |
| --- | --- | --- | --- | --- | --- |
| RSRP based anchor UE selection: SL-TDOA, InF-SH, FR1 | 0 | 0.7651 | 1.1581 | 1.6461 | 2.4845 |
|  | 0.1 | 0.8691 | 1.3779 | 2.1311 | 3.8224 |
|  | 0.25 | 0.8927 | 1.4053 | 2.1392 | 3.6906 |
|  | 1 | 1.0070 | 1.4695 | 2.1695 | 3.3238 |
|  | 2 | 1.3526 | 1.8542 | 2.5735 | 3.6365 |
|  | 4 | 2.1737 | 3.1034 | 4.1987 | 5.4756 |
|  | 8 | 3.6773 | 4.9952 | 6.7853 | 9.8044 |
| Anchor UEs with the least uncertainty in its location are prioritized: SL-TDOA, InF-SH, FR1 | 0.1 | 1.0986 | 1.8003 | 2.6570 | 4.2337 |
|  | 0.25 | 1.0986 | 1.7973 | 2.6411 | 4.1681 |
|  | 1 | 1.0844 | 1.7613 | 2.5636 | 4.0273 |
|  | 2 | 1.1063 | 1.8009 | 2.6558 | 3.9928 |
|  | 4 | 1.2315 | 2.0111 | 3.0000 | 4.5842 |
|  | 8 | 1.4787 | 2.5114 | 4.1152 | 7.8873 |
| RSRP based anchor UE selection: SL-AoA, InF-SH, FR2 | 0 | 4.2506 | 5.7197 | 7.8179 | 11.0314 |
|  | 4 | 5.1815 | 6.6108 | 9.6795 | 11.2079 |
|  | 8 | 6.2230 | 9.1968 | 10.8698 | 13.4300 |
| Anchor UEs with the least uncertainty in its location are prioritized: SL-AoA, InF-SH, FR2 | 8 | 4.3655 | 5.8399 | 9.3130 | 11.4471 |

For SL-AoA, only designated UEs’ uncertainty are increased. Thus, for the method that prioritizes to select anchor UEs with the least amount of uncertainty, its performance is not affected by the change in the uncertainty. Thus, the case for uncertainty with 8 meters is shown in the above table.

Based on the results presented in Table 1, we make following observations.

Observation 1: For SL-TDOA, simulation cases with standard deviation of positioning error in anchor UE group greater than 2 m, prioritization of selection of anchor UEs with the least uncertainty yields better horizontal positioning accuracy (90%) compared to RSRP based anchor UE selection.

Observation 2: For SL-AoA, simulation cases (FR2) with standard deviation of positioning error in anchor UE group greater than 4 m, prioritization of selection of anchor UEs with the least uncertainty yields better horizontal positioning accuracy (90%) compared to RSRP based anchor UE selection.

Observation 3: If anchor UE’s uncertainty information is known to the target UE, selection of anchor UEs based on uncertainty information yields improved positioning accuracy.

Based on the above observations, we make the following text proposal.

Proposal 2: Make the following changes in TS 38.214 to enable anchor UE providing the quality or uncertainty information associated with its location information.

| Reason for change: | Quality information associated to the location can improve accuracy. |
| --- | --- |
| Summary of change: | In clause 8.4.4, add modification on providing the location quality information. |
| Consequences if not approved: | Sidelink positioning accuracy may not be sufficient. |

*** Unchanged parts are omitted *** ### 8.4.4 SL PRS reception procedure The UE may be provided with the location information of other UEs via [higher layer parameter], along with the associated quality or uncertainty information. The UE may report the location information of the UE to the network. *** Unchanged parts are omitted ***

## Content of SL-PRS measurement reporting

The following agreement was made in RAN1#114 meeting:

Agreement

For SL Positioning measurement report content, the following can be included:

- [SL PRS resource ID]

- ARP ID used for reception

- Measurement results

  - Rx-Tx timing difference and quality

  - RSTD measurement and quality

  - RTOA measurement and quality

  - AoA measurement and quality

- RSRP, RSRPP measurement Time stamp

  - Rx timestamp

  - Tx timestamp

- LoS/NLOS indicator

- [UE identity information or information related UE identity information]

Note1: unified or separate report for different SL positioning methods is up to other WGs (e.g., RAN2)

Note2: whether to include UE identity information or information related UE identity information is up to RAN2, including whether this is optional in the report.

The agreement does not include (anchor) UE identity information in the report leaving it to RAN2. During a positioning session, Target UE or Anchor UE(s) will perform measurements associated with the selected SL positioning method. The results of the measurements will be reported to LMF or a (pre)selected Server UE for processing and computation of location information of the Target UE. The processing can also require the location information of a SL PRS transmitting UE, e.g., an Anchor UE in DL-like TDOA-based SL positioning. It is thus important that the LMF or (pre)selected Server UE is able to associate a measurement reporting of a SL PRS transmission with the identity of the UE who performs the SL PRS transmission. For example, a UE Source ID can be used to indicate this UE identity. From our understanding, RAN1 may indicate that this information is needed, the details can be left for RAN2 to decide.

Proposal 3: From RAN1 perspective, identity of UE performing a SL PRS transmission is included in the measurement reporting of the SL PRS transmission.

## SL PRS measurement reporting entity

For UE-based SL positioning, a Target UE can perform processing and compute its own location information based on measurement reporting received from anchor UEs. For example, in an out-of-coverage scenario, when a Target UE is also a Server UE, the Target UE will receive the measurement reporting and will perform the location determination.

Proposal 4: When a target UE is performing its location determination (i.e., target UE is a server UE), support the target UE to receive SL-PRS measurement reports from anchor UEs.

The use case for target UE to receive measurement reports from anchor UEs can be motivated in another complementary setting where the target UE may not be performing the location calculations itself but may receive measurement reporting from anchor UEs and forward those to LMF (in-coverage scenario) or a Server UE (out-of-coverage scenario).

# Conclusions

In this contribution, the following observations and proposals are made for measurement and reporting in SL positioning:

Observation 1: For SL-TDOA, simulation cases with standard deviation of positioning error in anchor UE group greater than 2 m, prioritization of selection of anchor UEs with the least uncertainty yields better horizontal positioning accuracy (90%) compared to RSRP based anchor UE selection.

Observation 2: For SL-AoA, simulation cases (FR2) with standard deviation of positioning error in anchor UE group greater than 4 m, prioritization of selection of anchor UEs with the least uncertainty yields better horizontal positioning accuracy (90%) compared to RSRP based anchor UE selection.

Observation 3: If anchor UE’s uncertainty information is known to the target UE, selection of anchor UEs based on uncertainty information yields improved positioning accuracy.

Proposal 1: Make the following change in TS 38.214 to allow anchor UEs exchange the quality of synchronization information. The reason for the change is to mitigate the synchronization error among the anchor UEs for SL-TDoA based methods. The summary of the change is to introduce a new parameter which (anchor) UEs can exchange to mitigate the synchronization error. The consequence if not approved is the poor performance of SL-TDoA due to synchronization error among the anchor UEs.

--------------------------<Start of text proposal for TS 38.214>-------------------------- ### 8.4.4 SL PRS reception procedure < Unchanged parts are omitted > The UE may report synchronization information synchronization source type and/or relative time difference with the associated quality metric and/or synchronization quality/accuracy information, via [higher layer parameter(s)]. For the SL RSTD measurement, the UE may report a reference UE information. --------------------------<End of text proposal for TS 38.214>--------------------------

Proposal 2: Make the following changes in TS 38.214 to enable anchor UE providing the quality or uncertainty information associated with its location information.

| Reason for change: | Quality information associated to the location can improve accuracy. |
| --- | --- |
| Summary of change: | In clause 8.4.4, add modification on providing the location quality information. |
| Consequences if not approved: | Sidelink positioning accuracy may not be sufficient. |

*** Unchanged parts are omitted *** ### 8.4.4 SL PRS reception procedure The UE may be provided with the location information of other UEs via [higher layer parameter], along with the associated quality or uncertainty information. The UE may report the location information of the UE to the network. *** Unchanged parts are omitted ***

Proposal 3: From RAN1 perspective, identity of UE performing a SL PRS transmission is included in the measurement reporting of the SL PRS transmission.

Proposal 4: When a target UE is performing its location determination (i.e., target UE is a server UE), support the target UE to receive SL-PRS measurement reports from anchor UEs.

# References

- RP-232670, “Revised WID on Expanded and Improved NR Positioning”, RANP#101, September. 2023.

- RAN1 Chairman’s note, RAN1 #114Bis, October 2023.

- RAN1 Chairman’s note, RAN1 #114, August 2023.

- RAN1 Chairman’s note, RAN1#113, May 2023.

- RAN1 Chairman’s note, RAN1#112-Bis, April 2023.

- 3GPP TS 37.355, “LTE Positioning Protocol (LPP) (Release 17), v17.0.0, Mar 2022.

- 3GPP TS 38.455, “NR Positioning Protocol A (NRPPa), ” v17.1.1, June 2022.

# Appendix

Table A1: IIoT baseline scenario parameters

| Parameter | FR1 specific Values | FR2 specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28 GHz |
| Bandwidth, MHz | 100MHz | 400 MHz |
| Subcarrier spacing, kHz | 30kHz | 120 kHz |
| Channel model | InF-SH |  |
| Hall size | small hall (L=120m x W=60m): D=20m |  |
| Number of anchor UEs dropped | 18 |  |
| Room height | 10 m |  |
| Number of floors | 1 |  |
| Clutter parameters: {density , height ,size } | InF-SH - {20%, 2m, 10m} |  |
| Synchronization error between anchor UEs | 0 ns |  |
| Target UE model parameters |  |  |
| UE noise figure, dB | 9dB | 13dB |
| UE antenna configuration | Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (2, 4, 2, 1, 1) |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| UE horizontal drop procedure | Uniformly distributed covering entire factory floor |  |
| UE antenna height | 1.5 m |  |
| Anchor UE model parameters |  |  |
| Anchor UE antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH=dV=0.5λ | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 1), dH=dV=0.5λ |
| Anchor UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| Anchor UE antenna height | 1.5 m |  |
| Inaccuracy of anchor UE position | Error value ε per anchor UE is added to actual position of the anchor UE. - ε =0 ε is modelled as a normal distribution with mean=0 and standard deviation - std= [0.1m 0.25m 1m 2m 4m 8m]. |  |