3GPP TSG RAN WG1 #115                                     		   	               R1-2311597

Chicago, USA, November 13th - 17th, 2023

Agenda Item:	8.3.1.3

Source:	InterDigital, Inc.

Title:	Remaining issues on SL PRS resource allocation

Document for:	Discussion and Decision

# Introduction

In RAN1 #114bis meeting, a set of new agreements and text proposals capturing existing SL-PRS resource allocation agreements were reached [1]. In this contribution, we discuss further remaining issues and text proposals for specification update related to SL-PRS resource allocation.

# Discussion

## SCI-based triggering of SL-PRS transmissions

In RAN1 #114bis meeting, an explicit 1-bit SL-PRS request indication is agreed in either SCI-1B or SCI-2D in Scheme 2 [1].

Agreement In Scheme 2, with regards to the SCI-based triggering of SL-PRS, the following WA is confirmed: Working assumption In Scheme 2, with regards to the triggering of SL-PRS, for the SCI-based triggering, the SL-PRS request, in either SCI-1B or SCI-2D, is an explicit field - If (pre-)configured per resource pool, then 1 bit is used, otherwise, it is 0 bits.

Additional aspects of the triggered SL-PRS transmission were discussed. For the aspect about properties of the triggered SL-PRS, most companies prefer Alt. 2 described below [2] and conclude there is no need to specify in RAN1 any constraints for the triggered SL-PRS.

- Aspect B: regarding properties of the triggered SL-PRS - Alt. 1: The triggered SL-PRS should share the same configuration with the received SL PRS, including periodicity, bandwidth, and priority. - Alt. 2: No need to specify in RAN1 any constraints for the triggered SL-PRS

In our view, the properties of the triggered SL-PRS transmission(s) should be further discussed in RAN2 and can include at least priority and comb-pattern of the triggered SL-PRS. To commence this discussion in RAN2, RAN1 can respond to question below listed in the RAN2 LS R1-2308834/R2-2309343 [3]. We think RAN1 should indicate in the response that higher layer provides the SL-PRS configuration including at least priority and comb pattern when SL-PRS is triggered by peer UE’s lower layer signaling, i.e. a SL-PRS request in SCI.

ACTION: RAN2 would like to ask RAN1 whether the following cases are possible - Higher layer provides the SL-PRS priority when SL-PRS is triggered by peer UE’s lower layer’s signalling - Lower layer signalling provides the SL-PRS priority when SL-PRS is triggered by peer UE’s lower layer signalling

Proposal 1: Send LS response to RAN2 (R2-2309343) regarding higher layer provision of SL-PRS configuration including priority and comb pattern of triggered SL-PRS.

## Correction on Step 6 for Scheme 2 SL-PRS resource allocation

For a dedicated resource pool, legacy step 6 is re-used for Scheme 2 SL-PRS resource selection procedure in section 8.2.4.2 as specified in the latest CR [4]. In legacy step 6, for PSSCH resource exclusion, the set of resource blocks and slots reserved in SCI format 1-A are determined according to clause 8.1.5. To apply the step for SL-PRS resource selection, SL-PRS resources and slots reserved in SCI-1B format should be used. The corresponding SL-PRS resources and slots determination based on the SCI-1B format is specified in section 8.2.4.2A [4].

Proposal 2: Make the following change in TS 38.214 to describe SL-PRS resources and slots reserved in SCI-1B format in Step 6 of Scheme 2 SL-PRS resource selection in a dedicated resource pool.

| Reason for change: | Correction on step 6 of SL-PRS resource allocation |
| --- | --- |
| Summary of change: | In clause 8.2.4.2, add modification on step 6 regarding the SL-PRS resource and slot determination based on 8.2.4.2A. |
| Consequences if not approved: | The determination of resources applied for SL-PRS resource exclusion is not clear. |

*** Unchanged parts are omitted *** #### 8.2.4.2 UE procedure for determining the subset of resources to be reported to higher layers in SL PRS resource selection in a dedicated SL PRS resource pool in sidelink resource allocation mode 2 In resource allocation mode 2 in a dedicated SL PRS resource pool, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this SL PRS/PSCCH transmission: *** Unchanged parts are omitted *** The UE shall perform this procedure according to clause 8.1.4, with the following modifications: - Partial sensing is not applicable in a dedicated SL PRS resource pool; - A candidate single-slot resource for transmission Rx,y is defined as the SL PRS resource with index x within the Set of SL-PRS resource ID(s) provided by the higher layer and in slot t'ySL - "SCI format 1-A” is replaced by “SCI format 1-B", - In step 5, the second condition is modified as follows: for any periodicity value allowed by the higher layer parameter reservationPeriodAllowed-Dedicated-SL-PRS-RP and any SL PRS resource ID in the set of SL PRS resource ID(s) provided by the higher layer, and a hypothetical SCI format 1-B received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating that SL-PRS resource ID, condition c in step 6 would be met. - In condition c of step 6 “determines according to clause 8.1.5 the set of resource blocks and slots” is replaced by “determines according to clause 8.2.4.2A the set of SL PRS resources and slots”. *** Unchanged parts are omitted ***

# Conclusion

In this contribution, the following proposals are made.

Proposal 1: Send LS response to RAN2 (R2-2309343) regarding higher layer provision of SL-PRS configuration including priority and comb pattern of triggered SL-PRS.

Proposal 2: Make the following change in TS 38.214 to describe SL-PRS resources and slots reserved in SCI-1B format in Step 6 of Scheme 2 SL-PRS resource selection in a dedicated resource pool.

| Reason for change: | Correction on step 6 of SL-PRS resource allocation |
| --- | --- |
| Summary of change: | In clause 8.2.4.2, add modification on step 6 regarding the SL-PRS resource and slot determination based on 8.2.4.2A. |
| Consequences if not approved: | The determination of resources applied for SL-PRS resource exclusion is not clear. |

*** Unchanged parts are omitted *** #### 8.2.4.2 UE procedure for determining the subset of resources to be reported to higher layers in SL PRS resource selection in a dedicated SL PRS resource pool in sidelink resource allocation mode 2 In resource allocation mode 2 in a dedicated SL PRS resource pool, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this SL PRS/PSCCH transmission: *** Unchanged parts are omitted *** The UE shall perform this procedure according to clause 8.1.4, with the following modifications: - Partial sensing is not applicable in a dedicated SL PRS resource pool; - A candidate single-slot resource for transmission Rx,y is defined as the SL PRS resource with index x within the Set of SL-PRS resource ID(s) provided by the higher layer and in slot t'ySL - "SCI format 1-A” is replaced by “SCI format 1-B", - In step 5, the second condition is modified as follows: for any periodicity value allowed by the higher layer parameter reservationPeriodAllowed-Dedicated-SL-PRS-RP and any SL PRS resource ID in the set of SL PRS resource ID(s) provided by the higher layer, and a hypothetical SCI format 1-B received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating that SL-PRS resource ID, condition c in step 6 would be met. - In condition c of step 6 “determines according to clause 8.1.5 the set of resource blocks and slots” is replaced by “determines according to clause 8.2.4.2A the set of SL PRS resources and slots”. *** Unchanged parts are omitted ***

# Reference

[1] RAN1 Chairman’s note, RAN1#114-bis, October 2023

[2] R1-231060 Moderator Summary #3 on resource allocation for SL PRS Qualcomm, October 2023

[3] R1-2308834/R2-2309343 LS on SL positioning MAC agreements, Huawei, August 2023

[4] R1-2310764 Corrections on the support for Expanded and Improved NR Positioning, Nokia, October 2023