3GPP TSG RAN WG1 #115			R1-2311664

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:	5

Source:	Apple

Title:	Discussion on RAN2 LS on RACH-less Handover

Document for:	Discussion/Decision

# Introduction

RAN2 sent an LS on RACH-less handover for NR NTN [1]. In the LS, three questions related to beam selection and power control for initial uplink transmission in NR NTN RACH-less handover are raised.

RAN1 sent a reply LS on RACH-less handover for NR NTN [2], answering the first question and part of the second question raised in [1]. RAN1 also sent a reply LS on RACH-less handover for NR NTN [3], answering the second question and part of the third question raised in [1].

Further discussions were conducted in RAN1 #114bis meeting [4] with the following observation.

Observation There is potential RAN1 discussion for the following aspects to support the RAN2 work on RACH-less handover. - The pre-allocated grant is provided with association to SSBs - The mapping between type-1 CG and SSBs in CG-SDT can be the baseline of how to configure pre-allocated grant mapped to SSBs

In this contribution, we discuss the remaining RAN1 related issues on RACH-less handover.

# Discussion

In RAN2 #123 meeting, the following agreements were made.

RAN2 #123 Agreements: - Single beam can be indicated in HO command to monitor target cell PDCCH for dynamic grant for initial UL transmission - The pre-allocated grant is provided with association to SSBs - The mapping between type-1 CG and SSBs in CG-SDT can be the baseline of how to configure pre-allocated grant mapped to SSBs (can rediscuss in case of different input from RAN1) - UE selects an SSB associated to the pre-allocated grant with RSRP above a configured threshold, use the selected SSB and the corresponding UL grant occasions for the initial UL transmission - ta-Report can be included in ServingCellConfigCommon in the RACH-less HO command - RAN2 understands that if pre-allocated grant is not configured and dynamic grant is used for first UL transmission, if UL HARQ mode is configured, HARQ mode A is recommended for the HARQ process (this is anyway up to NW implementation and there is no Stage2 and Stage3 spec impact) - The MAC entity applies the N_TA (value 0 or same as source cell) configured in the RACH-less HO command for the PTAG. FFS on when timerAlignmentTimer associated with this TAG starts - If no SSB mapping to pre-allocated grant has RSRP above the threshold, fallback to RACH HO (with new SSB selection), while T304 is running

It was observed [4] that the second and the third agreements in the above list may have RAN1 specification impact.

Here, we propose to have a new section in TS 38.213 for configured-grant based PUSCH transmission in RACH-less handover.

Proposal 1: RAN1 to adopt the following text proposal:

TS 38.213: Section 22 22. Configured-grant based PUSCH transmission in RACH-less handover A UE can be provided one or more configurations by respective one or more ConfiguredGrantConfig, for configured grant Type 1 PUSCH transmissions on the initial UL BWP [12, TS 38.331]. A UE can be provided by [RACHless-SSB-Subset] a number of SS/PBCH block indexes NPUSCHSS/PBCH to map to a number of valid PUSCH occasions for PUSCH transmissions over an association period. If the UE is not provided [RACHless-SSB-Subset], the UE determines NPUSCHSS/PBCH from the value of ssb-PositionsInBurst in ServingCellConfigCommon. A PUSCH occasion for a PUSCH transmission is defined by a time resource and a frequency resource and is associated with a DM-RS provided by cg-DMRS-Configuration for the configuration of PUSCH transmissions. An association period, starting from frame with SFN 0, for mapping NPUSCHSS/PBCH SS/PBCH block indexes, from the number of SS/PBCH block indexes, to valid PUSCH occasions and associated DM-RS resources is the smallest value in the set determined by the PUSCH configuration period provided by periodicity in ConfiguredGrantConfig according to Table 19.1-1 such that NPUSCHSS/PBCH SS/PBCH block indexes are mapped at least once to valid PUSCH occasions and associated DM-RS resources within the association period. A UE is provided a number of SS/PBCH block indexes associated with a PUSCH occasion and a DM-RS resource by [RACHless-SSB-PerCG-PUSCH]. If after an integer number of SS/PBCH block indexes to PUSCH occasions and associated DMRS resources mapping cycles within the association period there is a set of PUSCH occasions and associated DMRS resources that are not mapped to NPUSCHSS/PBCH SS/PBCH block indexes, no SS/PBCH block indexes are mapped to the set of PUSCH occasions and associated DMRS resources. An association pattern period includes one or more association periods and is determined so that a pattern between PUSCH occasions with associated DMRS resources and SS/PBCH block indexes repeats at most every 640 msec. PUSCH occasions and associated DMRS resources not associated with SS/PBCH block indexes after an integer number of association periods, if any, are not used for PUSCH transmissions. NPUSCHSS/PBCH SS/PBCH block indexes are mapped to valid PUSCH occasions and associated DMRS resources in the following order - first, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211] - second, in increasing order of PUSCH configuration period indexes A PUSCH occasion is valid if it does not overlap with a valid PRACH occasion as described in clause 8.1. A UE determines a power of a PUSCH transmission as described in clause 7.1.1, where the UE obtains PLb,f,c(qd) using a RS resource from an SS/PBCH block with index associated with the PUSCH transmission.

# Conclusion

In this contribution, we provided our views on RAN1 specification impact for RAN2 agreements on NR NTN RACH-less handover. Our proposal is as follows:

Proposal 1: RAN1 to adopt the following text proposal:

TS 38.213: Section 22 22. Configured-grant based PUSCH transmission in RACH-less handover A UE can be provided one or more configurations by respective one or more ConfiguredGrantConfig, for configured grant Type 1 PUSCH transmissions on the initial UL BWP [12, TS 38.331]. A UE can be provided by [RACHless-SSB-Subset] a number of SS/PBCH block indexes NPUSCHSS/PBCH to map to a number of valid PUSCH occasions for PUSCH transmissions over an association period. If the UE is not provided [RACHless-SSB-Subset], the UE determines NPUSCHSS/PBCH from the value of ssb-PositionsInBurst in ServingCellConfigCommon. A PUSCH occasion for a PUSCH transmission is defined by a time resource and a frequency resource and is associated with a DM-RS provided by cg-DMRS-Configuration for the configuration of PUSCH transmissions. An association period, starting from frame with SFN 0, for mapping NPUSCHSS/PBCH SS/PBCH block indexes, from the number of SS/PBCH block indexes, to valid PUSCH occasions and associated DM-RS resources is the smallest value in the set determined by the PUSCH configuration period provided by periodicity in ConfiguredGrantConfig according to Table 19.1-1 such that NPUSCHSS/PBCH SS/PBCH block indexes are mapped at least once to valid PUSCH occasions and associated DM-RS resources within the association period. A UE is provided a number of SS/PBCH block indexes associated with a PUSCH occasion and a DM-RS resource by [RACHless-SSB-PerCG-PUSCH]. If after an integer number of SS/PBCH block indexes to PUSCH occasions and associated DMRS resources mapping cycles within the association period there is a set of PUSCH occasions and associated DMRS resources that are not mapped to NPUSCHSS/PBCH SS/PBCH block indexes, no SS/PBCH block indexes are mapped to the set of PUSCH occasions and associated DMRS resources. An association pattern period includes one or more association periods and is determined so that a pattern between PUSCH occasions with associated DMRS resources and SS/PBCH block indexes repeats at most every 640 msec. PUSCH occasions and associated DMRS resources not associated with SS/PBCH block indexes after an integer number of association periods, if any, are not used for PUSCH transmissions. NPUSCHSS/PBCH SS/PBCH block indexes are mapped to valid PUSCH occasions and associated DMRS resources in the following order - first, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211] - second, in increasing order of PUSCH configuration period indexes A PUSCH occasion is valid if it does not overlap with a valid PRACH occasion as described in clause 8.1. A UE determines a power of a PUSCH transmission as described in clause 7.1.1, where the UE obtains PLb,f,c(qd) using a RS resource from an SS/PBCH block with index associated with the PUSCH transmission.

# References

- R1-2304322, “LS to on RACH-less handover,” May 2023.

- R1-2306217, “Reply LS on RACH-less handover,” May 2023.

- R1-2308568, “Reply LS on RACH-less Handover,” Aug. 2023.

- R1-2310496, “Summary of discussion on remaining issues for RACH-less handover,” Oct. 2023.