3GPP TSG RAN WG1 #115	R1-2311809

Chicago, IL, USA, November 13th – November 17th, 2023

# Title:	Draft reply LS on CPP

# Response to:	LS R1-2310789 (R2-2311565)

# Release:	Rel-18

# Work Item:	NR_pos_enh2-Core

Source:	3GPP TSG-RAN WG1 (Samsung)

To:	3GPP TSG-RAN WG2

Cc:	3GPP TSG-RAN WG4, 3GPP TSG-RAN WG3, 3GPP TSG-SA SA2

Contact Person:

Name: Emad Farag

E-mail Address: e.farag@samsung.com

# Attachments:

1. Overall Description:

RAN1 would like to thank RAN2 for their LS in R2-2311565 on CPP.

In that LS, RAN2 asked the following questions:

Question 1: Can the associated resource ID and resource Set ID in the report of RSCPD be different from the associated resource ID and resource Set ID in RSTD when UE report these measurements? The same question applies to RSCP and UE Rx-Tx time difference report.

RAN1 has agreed that the pair of the DL PRS resources used to obtain a DL RSCPD measurement are either the same as the pair of DL PRS resources used to obtain the associated DL RSTD measurement, or one of the pairs of DL PRS resources used to obtain the associated DL RSTD measurement.

RAN1 has also agreed that the DL PRS resource used to obtain a DL RSCP measurement is either the same DL PRS resource used to obtain the associated UE Rx-Tx time difference measurement, or one of the DL PRS resources used to obtain the associated UE Rx-Tx time difference measurement.

Hence, the answer to both questions is “no”.

Question 2: If answer of Q1 is yes, is the LOS/NLOS indication associated with the resource of RSCPD also required?

As the answer to question 1 is “no”, the DL PRS resource for carrier phase measurement is the resource or one of the resources used for RSTD. Hence the same LOS/NLOS indication applies.

Question3: For DL-TDOA, UE may provide up to 3 additional RSTD measurements per pair of TRPs. For CPP, is there any additional RSCP measurement per TRP or additional RSCPD measurement per pair of TRPs?

RAN1 has made the following agreement:

Agreement

Subject to UE’s capability, if a UE Rx-Tx time difference/DL RSTD measurement is obtained with Nsample (=2, 4) samples, as defined in TS 38.133, the UE Rx-Tx time difference/DL RSTD measurement can be associated with (i.e., reported together with) up to Nsample RSCP/RSCPD measurements.

- A single RSCP/RSCPD measurement is obtained within one sample

- Each RSCP/RSCPD measurement has its own timestamp.

- Note: It is up to RAN2 on how to define signalling support for the reporting of the timestamps of the RSCP/RSCPD measurements.

Hence, if a UE Rx-Tx time difference/DL RSTD measurement is obtained with Nsample (=2, 4) samples, the UE Rx-Tx time difference/DL RSTD measurement can be reported together with up to Nsample RSCP/RSCPD measurements.

Question4: On the indicated DL PRS resource set(s) occurring within indicated time window(s), is each indicated DL-PRS resourceSet associated with one indicated time window or multiple time windows?

Different PRS resource sets can be associated with different time windows. The association is based on the configuration.

2. Actions:

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account in their design.

3. Date of Next Meetings:

TSG RAN1 Meeting #116		26th FEB – 01st MAR 2024			Athens, GR

TSG RAN1 Meeting #116bis	15th APR – 19th APR 2024			China