3GPP TSG-RAN WG1 Meeting #115	R1-2312156

Chicago, USA, November 13-17, 2023

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on LS on NES CHO

Document for:	Discussion and Decision

Introduction

RAN1 has received an LS from RAN2 on Cell DTX/DRX activation/deactivation with the following agreement and request [1]:

Group common DCI format 2-X is reused to notify the UE that source cell is entering NES mode. • add one bit of DCI 2-X to trigger both use cases of Cell DTX/DRX activation and cell turning off. RAN2 send LS to RAN1 to request this signalling change. RAN2 respectfully requests RAN1 to specify signalling support corresponding to the above RAN2 agreement related to CHO enhancement.

We provide our analysis and observations for the issue and corresponding RAN1 actions for the relevant work in this contribution.

Discussion

In this section, analysis and RAN1 actions in addition to the RAN1 spec impact in detail [2] are provided.

The design on DCI format 2_9 were well discussed and the structure is defined as follows:

7.3.1.3.10 Format 2_9 DCI format 2_9 is used for activating or de-activating the cell DTX and/or DRX configuration of one or multiple serving cells for one or more UEs. The following information is transmitted by means of the DCI format 2_9 with CRC scrambled by cellDTRX-RNTI: - block number 1, block number 2,…, block number N where the starting position of a block is determined by the parameter positionInDCI-cellDTRX provided by higher layers for the UE. If the UE is configured with higher layer parameter XYZ, one or more blocks are configured for the UE by higher layers, with the following field defined for each block: - Cell DTX/DRX indication - 2 bits if XYZ, with the MSB corresponding to cell DTX configuration and the LSB corresponding to cell DRX configuration; otherwise 1 bit. The size of DCI format 2_9 is indicated by the higher layer parameter sizeDCI-2-9.

DCI format 2_9 has a block-wised structure, where each information block corresponds to a serving cell.

Observation 1: DCI format 2_9 has a block-wised structure, where each information block corresponds to a serving cell.

Also, according to the previous RAN2 discussion [4], NES-specific CHO is a procedure that executes on PCell. Since different UE may have different channel conditions (i.e. different UEs have different channel conditions in different CCs), gNB may configure different CCs as the PCell for different UEs.

Observation 2: NES-specific CHO is a procedure that is executed on PCell, while gNB may configure different CCs as the corresponding PCell for different UEs.

Thus, it is recommended that a new bit field (with 1 bit) is added for each information block, indicating that whether NES-specific CHO is triggered if UE is configured with the corresponding serving cell as PCell. The text proposal (TP) can be found in [2].

- It is recommended from RAN1 perspective that a new bit field is added for each information block, indicating that whether NES-specific CHO is triggered, if the UE is configured for that block with the corresponding serving cell as PCell.

Also, the new bit field should be configurable to support the requirements of different UEs (e.g. some UEs need to trigger NES-specific CHO when cell DTX/DRX is activated while others not). Especially, it would be possible that cell DTX/DTX operation could be turned ON or OFF while a UE can be separately indicated by an indication that CHO is triggered or not. Therefore, we propose that,

- The new bit field for NES-specific CHO triggering should be configurable independently from the indication of cell DTX/DRX operation.

Conclusions

In this contribution, we discuss the design details of NES CHO in DCI format 2_9. We have the following observations and proposals:

Observation 1: DCI format 2_9 has a block-wised structure, where each information block corresponds to a serving cell.

Observation 2: NES-specific CHO is a procedure that is executed on PCell, while gNB may configure different CCs as the corresponding PCell for different UEs.

- It is recommended from RAN1 perspective that a new bit field is added for each information block, indicating that whether NES-specific CHO is triggered, if the UE is configured for that block with the corresponding serving cell as PCell.

- The new bit field for NES-specific CHO triggering should be configurable independently from the indication of cell DTX/DRX operation.

- References

R2-2311589, LS to RAN1 on NES CHO

R1-2310864, Huawei, HiSilicon, Maintenance of Cell DTX/DRX for NES, November 13-17, 2023.

R1-2310454, Moderator (Intel Corporation), Discussion summary #5 for enhancements on cell DTX/DRX mechanism, October 9-13, 2023.

Moderator (InterDigital), Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning, October 9-13, 2023.

R1-2310865, Huawei, HiSilicon, UE features for Rel-18 NES, November 13-17, 2023.