3GPP TSG RAN WG1 Meeting #115	R1-2312324

Chicago, USA, 13-17 November, 2023

Agenda Item:	8.2.1.2

Source:	Moderator (Huawei)

Title:	FL summary#1 for AI 8.2.1.2 SL-U physical channel design framework

Document for:	Discussion and decision

- Introduction

The latest R18 WID on sidelink evolution (RP-230077) includes the following objective regarding support of sidelink on unlicensed spectrum (SL-U):

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

This contribution provides discussions related to SL-U physical channel design framework (AI 8.2.1.2), including summary of contributions, FL’s proposals, discussions, outcome of this meeting, etc. The related email thread is as below:

[115-R18-SL] Email discussion on NR sidelink evolution – Kevin (OPPO)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

- Issues

  - Issue#1: SL bandwidth part and resource pool

    - [M] Issue#1-1: TP, RP lowest RB aligns with RB set boundary

Background:

- Summary

  - Samsung proposes a TP to clarify lowest RB of resource pool aligns with RB set boundary.

- FL’s view

  - A proposal is given to reflect the above.

    - FL assumes this clarification applies to both contiguous RB and interlace RB, so FL updated the TP slightly based on Samsung’s TP, i.e., FL adds “… or ‘interlaceRB’ …”.

[M] Proposal 1-1

TP#1-1 in Section 4.1.1 of R1-23xxxxx (to be FLS) is endorsed for TS 38.214 Clause 8.

| Company | Agree? | Comments |
| --- | --- | --- |
| Apple | Yes |  |
| OPPO | Yes with comment | We have agreed one resource pool includes integer number of RB set. In addition to the TP, we think the highest RB of the resource should be also align with RB set boundary. Therefore, we propose the following modification to the TP part: For operation with shared spectrum channel access for frequency range 1, a sidelink resource pool can be (pre-)configured to include integer number of RB sets, and if the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB' or ‘interlaceRB’, the lowest RB of the sidelink resource pool is aligned with the lowest RB of a RB set in the SL BWP, the highest RB of the sidelink resource pool is aligned with the highest RB of a RB set in the SL BWP. A UE can be configured with intra-cell guard bands according to the higher layer parameter intraCellGuardBandsSL-List. The configured intra-cell guard band PRBs between any two adjacent RB sets can be used only for PSSCH transmission, if and only if, the UE has successfully performed channel access procedure in both adjacent RB sets, and the UE uses both of these RB sets for PSSCH transmission. |
| QC | Yes |  |
| xiaomi | yes |  |
| Panasonic | Yes |  |
| Spreadtrum | Yes |  |
| ZTE | Yes |  |
| Futurewei | Yes |  |
| Sharp | Yes with comments | In the very beginning of the sentence, we already mentioned “for operation with shared spectrum channel access”. Thus, the “if …” condition is redundant and could be removed for conciseness. For operation with shared spectrum channel access for frequency range 1, a sidelink resource pool can be (pre-)configured to include integer number of RB sets, and if the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB' or ‘interlaceRB’, the lowest RB of the sidelink resource pool is aligned with the lowest RB of a RB set in the SL BWP. |
| vivo | No | It can be handled by configuration, and does not need to be captured in RAN1 spec. |
| CATT, CICTCI | Yes |  |

  - Issue#2: Slot structure

    - [M] Issue#2-1: TP, PSCCH/PSSCH transmission on slots with PSFCH symbols

Background:

- Summary

  - Samsung, vivo proposed some TPs to clarify PSCCH/PSSCH transmission on slots with PSFCH symbols.

- FL’s view

  - A proposal is given to reflect the above.

[M] Proposal 2-1

TP#2-1 in Section 4.1.2 of R1-23xxxxx is endorsed for TS 38.214 Clause 8.1.2.1.

| Company | Agree? | Comments |
| --- | --- | --- |
| Apple | Yes |  |
| OPPO | Yes |  |
| QC | Yes |  |
| xiaomi | yes |  |
| Panasonic | Yes |  |
| Spreadtrum | Yes |  |
| ZTE | Yes |  |
| Futurewei | Yes |  |
| Sharp | Yes |  |
| CATT, CICTCI | Yes |  |

  - Issue#3: PSCCH/PSSCH

    - [M] Issue#3-1: TP, Usage of edge sub-channel

Background:

- Summary

  - LGE, Qualcomm point out: when the highest sub-channel of a candidate resource overlaps with a single RB set and intra-cell guardband PRBs, such sub-channel can be used for PSSCH transmission. This should be specified.

    - E.g., Qualcomm gave a figure below

- FL’s view

  - A proposal is given to reflect the above. FL adopts LGE’s TP directly.

[M] Proposal 3-1

TP#3-1 in Section 4.1.3 of R1-23xxxxx is endorsed for TS 38.214 Clause 8.

| Company | Agree? | Comments |
| --- | --- | --- |
| Apple | No | We think in this proposed case, a candidate resource is composed of more than 1 sub-channel. If a candidate resource is composed of just 1 sub-channel, and this sub-channel has overlap with intra-cell guardband PRBs, then this sub-channel cannot be used for PSCCH/PSSCH transmission. |
| OPPO | Comment | Two points need to be clarified: - The PSSCH transmission resource should include more than one sub-channels It is for contiguous RB based PSSCH transmission. |
| QC | Yes |  |
| Panasonic | Yes | Sub-channel(s) which include intra-cell guardband PRBs can be used PSSCH was agreed. It should be captured. |
| Spreadtrum | No | Share the similar view with OPPO. |
| NEC | Yes |  |
| ZTE | Comment | We agree with the TP, and some further clarification are needed in our opinion. For multiple subchannel CRB based transmission, it is still not clear how such subchannel is used. Using an a rate matching way or punctured way ? In our opinion, a rate matching way is more efficient as per Qualcomm suggestion, while we should also guarantee that the code rate after rate matching is not too high. So a further TP is proposed as below: A corresponding text proposal in 38.214 is provided: *** Unchanged parts are omitted *** ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 *** Unchanged parts are omitted *** If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB', the UE shall exclude candidate single-slot or candidate multi-slot resources with the lowest sub-channel including resource blocks of the intra-cell guardband PRBs, and UE shall exclude candidate single-slot or candidate multi-slot resources whose last N PRBs locate in guard band, where N is provided by higher layers, the intra-cell guardband PRBs are configured by higher layer parameter, intraCellGuardBandsSL-List. *** Unchanged parts are omitted ***. |
| Futurewei | Yes |  |
| Sharp | Yes |  |
| CATT, CICTCI | Comment | The pre-condition is that one candidate resource comprises more than one sub-channels as proposed by Apple, otherwise, it will be contradictory with the previous agreement that the sub-channel cannot be used for PSCCH transmission. |

    - [M] Issue#3-2: TP, RB set index for FRIV indication

Background:

- Summary

  - LGE point out for FRIV indication, RB set indexing for PSSCH allocation is ambiguous.

- FL’s view

  - A proposal is given to reflect the above.

[M] Proposal 3-2

TP#3-2 in Section 4.1.4 of R1-23xxxxx is endorsed for TS 38.214 Clause 8.1.5.

| Company | Agree? | Comments |
| --- | --- | --- |
| Apple | No | The TP #3-2 shows the wrong specification number. It is TS 38.214, not TS 38.213. Also, we do not think the added bullet is necessary, since “NRBset is the number of RB sets in a resource pool”. For FRIV for sub-channel, we also do not have such bullet for sub-channel indexing. |
| OPPO | Yes |  |
| QC | Comments | For legacy subchannel FRIV, the spec does not mention the subchannel is indexed in natural order. Not sure if we need to mention that for RB-set indexing here. |
| xiaomi | yes |  |
| Panasonic | Yes |  |
| ZTE | Yes |  |
| Futurewei | Yes |  |
| Sharp | Yes |  |
| vivo | No | Similar view as Apple and QC. |
| CATT, CICTCI | Yes |  |

  - Issue#4: PSFCH and SL-HARQ

    - [H] Issue#4-1: Order between PSFCH prioritization procedure and LBT procedure

Background:

- Summary

  - Alt 1: UE performs PSFCH prioritization after LBT result for PSFCH transmission is known

    - Support (1): [ZTE]

  - Alt 2: UE performs PSFCH prioritization before LBT result for PSFCH transmission is known

    - Support (3): LGE, vivo, [ZTE]

  - Alt 3: UE performs PSFCH prioritization regardless of LBT result for PSFCH transmission

    - Support (5): [Samsung], Huawei/HiSilicon, Spreadtrum, Panasonic, WILUS

- FL’s view

  - Quite some companies point out: after the LBT result is known, there is no sufficient time for the UE to perform PSFCH prioritization and prepare PSFCHs before the PSFCH transmission.

    - So, FL removes Alt1.

  - FL made some wording change to Alt 3 to avoid any confusion, i.e., “…regardless of  without considering…”

  - A proposal is given to reflect the above.

[H] Proposal 4-1

Regarding the order between performing LBT and PSFCH prioritization:

- Down-select one of followings

  - Alt 1: UE performs PSFCH prioritization after LBT result for PSFCH transmission is known

  - Alt 2: UE performs PSFCH prioritization before LBT result for PSFCH transmission is known

  - Alt 3: UE performs PSFCH prioritization without considering LBT result for PSFCH transmission

- When UE intends to transmit PSFCH, if LBT on all RB set(s) fail, down-select one of followings:

  - Alt A: UE drops PSFCH transmission

  - [ Alt B: UE performs prioritization of PSFCH reception and performs PSFCH reception ]

FL’s Note:

- FL adds bracket to Alt B since it seems infeasible considering UE processing time. Proponent of Alt B, please explain whether UE has sufficient time to do it after LBT result is known.

| Company | Agree? | Comments |
| --- | --- | --- |
| Apple | Yes | We are fine with Alt 3 and Alt A. |
| OPPO | Yes |  |
| QC | Yes | We support Alt 2. For high capability UE, it can prepare multiple sets of PSFCH waveform, and each set assume certain RB-set(s) combination. The PSFCH prioritization is done for each PSFCH waveform hypothesis based on the RB-sets combination. Based on the LBT outcome, the high-end UE could pick one of the PSFCH waveform hypo whose occupied RB-set(s) are all cleared. For low capable UE, only one PSFCH hypothesis is possible. So, the UE can only do all or nothing based on the LBT outcome. |
| xiaomi | yes |  |
| Panasonic | Yes | We support Alt 3 and Alt A. |
| Spreadtrum | Yes | Alt3 and Alt A. |
| NEC |  | We think Alt.B is feasible because LBT sensing procedure and PSFCH reception are both receiving procedure without processing time restriction. |
| ZTE | Alt2 | Alt 3 includes both Alt 1 or Alt2, if Alt 1 is removed, Alt 3 also needs to be removed considering the UE processing time. What’s the problem with Alt 2 if people are concerned on the processing time? The Alt A itself is not complete, after prioritization, if the decision is to perform transmission, the UE can switch to reception if the LBT on all RB sets fail and perform reception automatically, there is no need to re-do prioritization. Alt 2 and Alt A are tied, as per reasoning in the paragraph above, we support Regarding the order between performing LBT and PSFCH prioritization: - Down-select one of followings - Alt 1: UE performs PSFCH prioritization after LBT result for PSFCH transmission is known - Alt 2: UE performs PSFCH prioritization before LBT result for PSFCH transmission is known - Alt 3: UE performs PSFCH prioritization without considering LBT result for PSFCH transmission - When UE intends to transmit PSFCH, if LBT on all RB set(s) fail, down-select one of followings: - Alt A: UE drops PSFCH transmission and performs PSFCH reception |
| Futurewei | Yes | Alt 3/ Alt A |
| Sharp | Yes | We support Alt 3 and Alt A. And prefer to add “after performing PSFCH prioritization” as below to clarify the “UE intends to transmit PSFCH ” - When UE intends to transmit PSFCH after performing PSFCH prioritization, if LBT on all RB set(s) fail, down-select one of followings: - Alt A: UE drops PSFCH transmission - [ Alt B: UE performs prioritization of PSFCH reception and performs PSFCH reception ] |
| vivo | comment | We are not sure what the difference is between Alt 3 and Alt 2. If the prioritization is performed before LBT, obviously it is executed regardless of the LBT result which is unknow yet. |
| CATT, CICTCI | Yes | Prefer Alt 3 and Alt A. |

    - [H] Issue#4-2: PSFCH transmission over non-contiguous/contiguous RB sets

Background:

- Summary

  - Alt 1: UE selects contiguous RB set(s) including PSFCH with smallest priority value.

    - Support (8): OPPO, DCM, NOK, MTK, [Panasonic], IDC, Huawei/HiSilicon, WILUS

      - Alt 1-1-2 in proposal below is proposed by Nokia.

  - Alt 2: UE selects contiguous RB set(s) based on UE implementation

    - Support (0):

  - Alt 3: For Alt 1-1b, UE can transmit PSFCH on (pre-)configured common interlace in the RB set(s) between any non-contiguous RB sets

    - Support (1): ZTE

  - Alt 4: It is up to UE implementation to select one contiguous RB set which satisfies at least one of following conditions, (detailed conditions can be found in proposal below)

    - Support (1): vivo

- FL’s view

  - FL adds following red part to the main sentence to make it more accurate.

    - “When a UE does not support PSFCH transmission over non-contiguous RB sets, among the RB set(s) whose LBT were successful, down-select one of followings:”

    - Example#1

      - Assume the resource pool includes RB set {0,1,2,3,4}

      - Assume PSFCH locates in RB set {0,1,3,4} after legacy PSFCH prioritization

      - Assume LBT on RB set {0,3} succeeds, and LBT on RB set {1,4} fails

      - Then, UE needs to select RB set {0} or {3}, or do other behaviors, depending on the Alternatives below, i.e., there is no need to consider RB set 1/2/4 since anyway UE cannot transmit on such RB sets.

  - FL adds more details to Alt 1 to make it complete.

  - A proposal is given to reflect the above.

Agreement - Support PSFCH transmission over contiguous and non-contiguous RB sets - Support S-SSB transmission over contiguous and non-contiguous RB sets - Support of PSFCH/S-SSB transmission over contiguous and/or non-contiguous RB sets is subject to UE capability(ies)

[H] Proposal 4-2

When a UE does not support PSFCH transmission over non-contiguous RB sets, among the RB set(s) whose LBT were successful, down-select one of followings:

- Alt 1: UE selects contiguous RB set(s) including PSFCH(s) with smallest priority value.

  - If there are more than one contiguous RB set(s) including PSFCH(s) with the same smallest priority value, down-select one of followings:

    - Alt 1-1: UE selects contiguous RB set(s) including higher number of PSFCH(s)

      - If there are more than one contiguous RB set(s) including PSFCH(s) with the same number of PSFCH(s), down-select one of followings:

        - Alt 1-1-1: it is up to UE implementation to select one contiguous RB set among them

        - Alt 1-1-2: UE selects contiguous RB set(s) with lower RB set index

    - Alt 1-2: It is up to UE implementation to select one contiguous RB set among them

- [ Alt 3: For Alt 1-1b, UE can transmit PSFCH on (pre-)configured common interlace in the RB set(s) between any non-contiguous RB sets ]

  - FFS: UE behaviour under Alt 2-3a

  - Note: Alt 1-1b and Alt 2-3a in previous agreements are as below

    - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

- Alt 4: It is up to UE implementation to select one contiguous RB set which satisfies at least one of following conditions

  - This contiguous RB set includes PSFCH with the smallest priority value

    - If there are more than one contiguous RB set(s) including PSFCH(s) with the same smallest priority value, UE selects any one of contiguous RB set(s) including the highest number of PSFCHs

  - This contiguous RB set contains PSFCH which is intended for a COT initiating UE in a shared COT if the UE shares the COT

  - This contiguous RB set contains PSFCH within a COT if the UE initiates the COT

FL’s Note:

- FL adds bracket to Alt 3 since it seems infeasible in some cases.

  - In the above Example#1, since LBT succeeds only on RB set 0 and 3, and fails on RB set 1,2,4, the UE cannot transmit PSFCH on common interlace on RB set 1, 2. Then, how Alt 3 works?

  - In addition, how Alt 3 works for Alt 2-3a?

  - Proponent of Alt 3, please explain.

| Company | Agree? | Comments |
| --- | --- | --- |
| Apple | Yes | We are fine with Alt 1-2 and Alt 1-1-1. |
| OPPO | Yes | We prefer Alt 1-2. |
| QC | Yes | We support Alt 1-1-1 or Alt 1-2 |
| xiaomi | yes | We prefer the alt 1 and alt 1-2 due to the simplicity. |
| Panasonic | Yes | We support Alt 1-2. |
| Spreadtrum | Yes | We prefer Alt 1-2. |
| NEC | Yes | Alt.4 is fine. |
| ZTE |  | In the example above, the LBT results are known before the prioritization procedure, so that a priority based selection can be performed to pick RB set 0 containing PSFCH with smallest priority level, and this pre knowledge on PSFCH LBT operation is mandatory to ensure that because if the LBT results are not known in advance, RB set 0 or RB set 3 can not be selected in the first place (RB set 1 and RB set 4 does not necessarily needs to be ruled out). Thus the example is based on the Alt 1 in previous proposal, but Alt 1 is crossed out by FL due to the difficulty to prepare the PSFCH after the LBT result is known. Do we miss anything? If we go with Alt 2 in the previous proposal due to the concern on PSFCH preparation/prioritization process, consider the same example and only RB set 0 and 3 passes the prioritization, for RB set 1 and RB set 2, it’s our understanding to first attempt to transmit the common interlace and if the outcome is the transmission on RB set 1,2 fails due to LBT, the transmission on RB set 1 or 3 can be dropped without considering the priority, the benefit for this approach is better ensuring the feedback transmission/performance and occupying the COT. |
| Futurewei | Yes | Alt 1 |
| Sharp | Yes | We support Alt 1. Given higher number of PSFCHs could be PSFCHs with lower priorities in Alt 1-1, up to UE implementation for simplicity (i.e., Alt 1-2) is a bit preferred. |
| vivo | comment | Does this proposal consider only the case where type-1 channel access is used for PSFCH transmission? In the COT sharing case where the responding UE using type-2A, we have agreed that at least one PSFCH to the COT initiating UE should be sent, which does not seem to align with the alt 1/2/3. |

    - [H] Issue#4-3: PSFCH power control and prioritization

Background:

- Summary

  - Issue X1: When OCB is required, SL-U PSFCH occupies more than 1 PRB, PSFCH power control formulas may need some updates to reflect this

    - OPPO/Apple propose to adopt red changes in “PPSFCH,one=PO,PSFCH+10log102μ⋅MRBPSFCH+αPSFCH⋅PL [dBm]”

      - FL gave Alt1 in Proposal 4-3-1 below for this.

      - Alt A below is from last meeting and is simple.

      - Alt B is proposed by OPPO/Apple.

        - FL adds following red sentence to simplify the issue. Otherwise, RAN1 needs to consider the case where different resource pool is (pre-)configured with different value of numDedicatedPRBsForPSFCH and numDedicatedPRBsForPSFCH.

          - “UE expects the same (pre-)configured value of numDedicatedPRBsForPSFCH and numDedicatedPRBsForPSFCH across all resource pools”

    - LGE proposed another way to resolve this, FL gave Alt2 in Proposal 4-3-1 below for this.

  - Issue X2: Vivo point out “drop common PRB when it locates within 1 MHz with a dedicated PRB” should be performed after PSFCH prioritization, otherwise the OCB requirement may not be fulfilled.

    - Example#1

      - Step 1: Assume UE is scheduled to transmit 16 PSFCH.

      - Step 2: Assume after PSFCH prioritization, UE will transmit 4 PSFCH.

      - There are two Alts to drop common PRB

        - Alt 1: Drop common PRB after Step 1 and before Step 2

          - Then, many common PRBs might be dropped, and finally UE may not fulfill OCB requirement since only 4 PSFCH are transmitted.

        - Alt 2: Drop common PRB after Step 2

          - Vivo suggests to go with Alt2.

    - FL feels this is valid issue, and gave a proposal below to support Alt 2.

  - Issue X3: Huawei point out in the latest Editor CR, due to the cyan part below, although PPSFCH,first≤PPSFCH,second , since P_common = N*PPSFCH,first, final power on common PRB (P_common) might be larger than that of dedicate PRB  (P_dedicated=PPSFCH,second), which violates the Working Assumption.

    - FL feels this is valid issue, and gave a proposal below.

- FL’s view

  - Quite some companies gave very long/complicated TPs. However, it’s unclear what are the issues these companies trying to address. FL suggest we discuss issues and make agreements directly, and let Editor to capture them. It’s very hard to discuss long/complicated TPs directly.

  - If you feel there are more essential issues need to be resolved, please comment, clearly clarify what is the issue and solution (a detailed example is preferred to help quick understanding), instead of giving a long/complicated TP directly.

[H] Proposal 4-3-1

Regarding “SL-U PSFCH occupies more than 1 PRB”, down-select one of followings:

- Alt 1: Support following red changes to legacy PSFCH power control formula

  - PPSFCH,one=PO,PSFCH+10log102μ⋅MRBPSFCH+αPSFCH⋅PL [dBm], where MRBPSFCH is a reference number of resource blocks for a PSFCH transmission, down-select one of followings:

    - Alt A: MRBPSFCH is (pre-)configured per SL BWP, and the value range is {10, 11, 12, 13, 15, 16}

    - Alt B:

      - For Alt 1-1b: MRBPSFCH is equal to numDedicatedPRBsForPSFCH

      - For Alt 2-3a: MRBPSFCH is equal to numRefPRBOfInterlace

      - UE expects the same (pre-)configured value of numDedicatedPRBsForPSFCH and numDedicatedPRBsForPSFCH across all resource pools

- Alt 2: Support the followings

  - For Alt 1-1b:

    - PPSFCH,one refers to the power on one dedicated PRB

    - PPSFCH,first refers to the power on one common PRB

      - PPSFCH,first=PPSFCH,one-PPSFCH,offset, where PPSFCH,offset is provided by PSFCHPowerOffset.

    - PPSFCH,k(i) refers to the total power on all dedicated PRB(s) for a PSFCH transmission k

    - Let MRB,kPSFCHi denotes the number of dedicated PRBs of a PSFCH transmission k in PSFCH transmission occasion i

    - Let MRB,firstPSFCH denotes the number of PRBs in the common interlace

    - Then, use following formulas for PCMAX checking

      - PPSFCH,one*k=1Nsch,Tx,PSFCHMRB,kPSFCHi+PPSFCH,first*MRB,firstPSFCH≤PCMAX

      - PPSFCH,one*k=1Nmax,PSFCHMRB,kPSFCHi+PPSFCH,first*MRB,firstPSFCH≤PCMAX

      - where,

        - Nsch,Tx,PSFCH and Nmax,PSFCH are the same as legacy NR SL

        - assume the math symbols in above two formulas are converted to linear unit (i.e, Watt)

  - For Alt 2-3a:

    - PPSFCH,one refers to the power on one dedicated PRB

    - PPSFCH,k(i) refers to the total power on all dedicated PRB(s) for a PSFCH transmission k

    - Let MRB,kPSFCHi denotes the number of dedicated PRBs of a PSFCH transmission k in PSFCH transmission occasion i

    - Then, use following formulas for PCMAX checking

      - PPSFCH,one*k=1Nsch,Tx,PSFCHMRB,kPSFCHi≤PCMAX

      - PPSFCH,one*k=1Nmax,PSFCHMRB,kPSFCHi≤PCMAX

      - where,

        - Nsch,Tx,PSFCH and Nmax,PSFCH are the same as legacy NR SL

        - assume the math symbols in above two formulas are converted to linear unit (i.e, Watt)

  - TP#4-3-1 in Section 4.1.5 of R1-23xxxxx (to be FLS) is endorsed for TS 38.213 Clause 16.2.3.

- Note: Alt 1-1b and Alt 2-3a in previous agreements are as below

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

==

FL’s note:

- For Proposal 4-3-1:

  - FL feels some other companies are also trying to address this issue. But the solution is not clear to FL. So far, FL proposes Alt 1 based on OPPO/Apple’s Tdoc, and Alt 2 based on LGE’s Tdoc.

- FYI

| 8 | numRefPRBOfInterlace | Indicate reference number of PRBs of one interlace within 1 RB set | ENUMERATED {10, 11} | N/A | Per resource pool |
| --- | --- | --- | --- | --- | --- |
| 15 | numDedicatedPRBsForPSFCH | Indicates the value of K3 when each PSFCH transmission occupies "1 common interlace and K3 dedicated PRB(s)" | ENUMERATED {1,2,5} | N/A | Per resource pool |

[H] Proposal 4-3-2

TP#4-3-2 in Section 4.1.6 of R1-23xxxxx is endorsed for TS 38.213 clause 16.3.0.

[H] Proposal 4-3-3

TP#4-3-3 in Section 4.1.7 of R1-23xxxxx is endorsed for TS 38.213 clause 16.2.3.

| Company | Agree? | Comments |
| --- | --- | --- |
| Apple |  | Proposal 4-3-1, we support Alt 1, and think Alt 2 has a quite different definition from legacy way and should be avoided at this stage. To be fair, FL should also show the TP related to Alt 1, rather than only the TP related to Alt 2. Within Alt 1, we support Alt B and think Alt A has an issue. It is unclear for Alt 1-1b, PPSFCH,one is - the transmit power of dedicated PRBs only or - the transmit power of dedicated PRBs plus common PRBs (common interlace). Since the transmit power of per RB on common interlace and the transmit power of per RB on dedicated PRBs should be separately defined. Hence, we think PPSFCH,one is the transmit power of dedicated PRBs only (i.e., bullet 1 above.) In that sense, the value range of MRBPSFCH is incorrect. Also, we are not sure the intention of the third bullet under Alt B, and think this bullet should be removed. Proposal 4-3-2: we are fine. Proposal 4-3-3: we think this TP should be handled after Proposal 4-3-1 is determined, since both proposals depend on the PSFCH power control formula determination. |
| OPPO |  | Proposal 4-3-2: yes Proposal 4-3-3: no. it is common understanding that the power on common PRB is summary of all PSFCH (NTx,PSFCH selected PSFCH) after PSFCH selection procedure. It needs to be clarified. Therefore, we suggest the following modification: For operation with shared spectrum channel access, - if sl-PSFCH-Type = ‘type1’, a UE equally allocates power to all PRBs of an interlace used for a PSFCH transmission - if sl-PSFCH-Type = ‘type2’, a UE allocates power of all NTx,PSFCH PSFCH transmissions to PRBs such that - a power for each PRB in a subset of RRBs of a first interlace is PPSFCH,first, - a power for each PRB in a subset of RRBs of a second interlace is PPSFCH,second, and - PPSFCH,first=PPSFCH,second-PPSFCH,offset, where PPSFCH,offset is provided by sl-PSFCH-Type2-PowerOffset. |
| QC | ok | For proposal 4-3-1, we support Alt 2 |
| Xiaomi |  | For proposal 4-3-1, we support the Alt 1 and Alt B with the following modification. Firstly, in legacy sidelink, PPSFCH,one is the total power for one psfch transmission, and this definition shall be reused, so alt 1 shall be supported. For Alt 2-3a, there might exist that the prb number of different interlaces are different, to keep the same power for different psfch transmissions, so MRBPSFCH shall equal to numRefPRBOfInterlace, but for Alt 1-1b, it is not necessary to introduce the reference prb number, because the dedicated prb number is always same for different psfch transmissions based on pre-configuration, alt B in alt 1 shall be supported. Moreover, the offset between the common prb and dedicated prb for alt 1-1b shall be considered in the Alt1, and P (PSFCH,offset) should be converted to watts before adding，so we making the following revision with blue part: Regarding “SL-U PSFCH occupies more than 1 PRB”, down-select one of followings: - Alt 1: Support following red changes to legacy PSFCH power control formula - For Alt 2-3a, PPSFCH,one=PO,PSFCH+10log102μ⋅MRBPSFCH+αPSFCH⋅PL [dBm], - For Alt 1-1b, PPSFCH,one= 10 log10[ 10[PO,PSFCH+10log102μ⋅MRBPSFCH+αPSFCH⋅PL]÷10 +10MRBPSFCH×PPSFCH,offset÷10] [dBm]，wherePPSFCH,offset is provided by PSFCHPowerOffset. - where MRBPSFCH is a reference number of resource blocks for a PSFCH transmission, down-select one of followings: - Alt A: MRBPSFCH is (pre-)configured per SL BWP, and the value range is {10, 11, 12, 13, 15, 16} - Alt B: - For Alt 1-1b: MRBPSFCH is equal to numDedicatedPRBsForPSFCH - For Alt 2-3a: MRBPSFCH is equal to numRefPRBOfInterlace - UE expects the same (pre-)configured value of numDedicatedPRBsForPSFCH and numDedicatedPRBsForPSFCH across all resource pools |
| Panasonic |  | For proposal 4-3-1, we support Alt 2 with modification for MRB,firstPSFCH as "MRB,firstPSFCH denotes the number of PRBs in the common interlace in PSFCH transmission occasion i". The common PRB is shared by multiple PSFCH transmissions in a RB set, therefore double count of common interlace for multiple PSFCHs in a RB set should be avoided. We support proposal 4-3-2. |
| NEC | YES | Can support Alt.2 in proposal 4-3-1. |
| ZTE | Alt 2 is preferred | --For alt1，the power checking is not clear, only PPSFCH,one is defined, and how to determine the power for common PRB is still not clear. For alt 2, it is more Straightforward. And we think alt 2 procedure can be common for alt 1-1b format and alt 2-3a format. The technical issue with the currently proposed TP is the insufficient power level of dedicated/common PRB when the total power is inferior to Pc, max. We invite companies to have a check at ZTE’ revised contribution R1-2312312 |
| Futurewei | Yes |  |
| Sharp |  | We support direction of alt.2 in Proposal 4-3-1. Different interlaces may include different RBs within an RB set. Alt.1 use reference number to calculate PSFCH power, which may cause PSFCH power on one PRB in an interlace to be different form that in another interlace. |
| vivo | OK | Regarding the 4-3-1, the direction of Alt 2 is preferred. |
| CATT, CICTCI | Yes with Comment | For proposal 4-3-1, we prefer alt 2, and further think whether we need to consider the common RB dropping impacts when a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth. For proposal 4-3-2, OK For proposal 4-3-3, we agree that the power on common RBs are not accumulated, but current change have some problem. Since in some cases, two PSFCHs may be mapped into same dedicated PRBs, based on our understanding, the transmission power on dedicated PRBs need to be accumulated. It would be better to directly index the power control formular of PSFCH_first |

    - [M] Issue#4-4: Inter-UE coordination Scheme 2 (conflict indication)

Background:

- Summary

  - “One PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)” applies to R17 SL inter-UE coordination Scheme 2 (conflict indication)

    - Support (5): [Qualcomm], LGE, Samsung, Apple, vivo

    - Not Support (1): Panasonic

  - R17 SL inter-UE coordination Scheme 2 (conflict indication) for sl-PSFCH-Occasion = '1' uses the same transmission scheme (Alt 1-1b and Alt 2-3a) as HARQ-ACK in R18 SL-U

    - Support (6): LGE, Samsung, DCM, CATT, Apple, Panasonic

    - Not Support (1): Vivo (slightly)

- FL’s view

  - A proposal is given to reflect the above.

(38331)

sl-PSFCH-Occasion-r17                     INTEGER (0..1)                                                       OPTIONAL,   -- Need M

sl-PSFCH-Occasion Indicates the reference slot from which a PSFCH occasion for inter-UE coordination information transmission is derived. Value 0 corresponds to the slot where UE-B's SCI is transmitted and value 1 corresponds to the slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B's SCI.

==

[M] Proposal 4-4-1

Support:

- R17 SL inter-UE coordination Scheme 2 (conflict indication) for sl-PSFCH-Occasion = '1' uses the same transmission scheme (Alt 1-1b and Alt 2-3a) as HARQ-ACK in R18 SL-U

  - For Alt 1-1b, common interlace index for conflict indication and HARQ-ACK within the same RB set are the same

  - Note: Alt 1-1b and Alt 2-3a in previous agreements are as below

    - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

[M] Proposal 4-4-2

Support:

- “One PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)” applies to R17 SL inter-UE coordination Scheme 2 (conflict indication) for both sl-PSFCH-Occasion = '0' and sl-PSFCH-Occasion = '1'

| Company | Agree? | Comments |
| --- | --- | --- |
| Apple |  | Proposal 4-4-1: support Proposal 4-4-2: support |
| OPPO | Yes |  |
| QC | Yes |  |
| Panasonic |  | Proposal 4-4-1: support. Proposal 4-4-2: We didn't support it since it needs more PSFCH resources for inter-UE coordination Scheme 2 and PSFCH resources for HARQ-ACK should be reduced. However, if majority support it, we can accept proposal 4-4-2. |
| Spreadtrum | Yes |  |
| ZTE | OK |  |
| Futurewei | Yes |  |
| Sharp | Yes | As cited below, it seems spec has already supported the proposal 4-4-2. For the n-th candidate PSFCH transmission occasion, 1≤n≤NoccasionPSFCH, the UE determines a set of interlaces that includes a number Minterlace,kPSFCH,n of interlaces based on the n-th indication provided by sl-PSFCH-RB-SetList or sl-RB-SetPSFCHList for HARQ-ACK information or conflict information, respectively. The NPRBPSFCH PRBs in the second interlace are provided by sl-PSFCH-Type2-DedicatedPRB where, for the n-th candidate PSFCH transmission occasion, 1≤n≤NoccasionPSFCH, and for each interlace l, the UE determines MPRB,k, lPSFCH,n PRBs based on the n-th indication provided by sl-PSFCH-RB-SetList or sl-RB-SetPSFCHList for HARQ-ACK information or conflict information, respectively. |
| CATT, CICTCI | Yes |  |

    - [L] Issue#4-5: Inter-UE coordination Scheme 1 (preferred/non-preferred resources)

Background:

- Summary

  - When OCB is required,

    - Alt 1: R17 SL inter-UE coordination Scheme 1 (preferred/non-preferred resources) is not supported

      - CATT

    - Alt 2: R17 SL inter-UE coordination Scheme 1 (preferred/non-preferred resources) is supported

      - Apple, Sharp

- FL’s view

  - A proposal is given to reflect the above. If Alt 2 is chosen, RAN1 can discuss TP later taking Apple/Sharp’s TP as starting point, or even leave it to Editor.

[L] Proposal 4-5

When OCB is required, down-select one of followings:

- Alt 1: R17 SL inter-UE coordination Scheme 1 (preferred/non-preferred resources) is not supported

- Alt 2: R17 SL inter-UE coordination Scheme 1 (preferred/non-preferred resources) is supported

  - SCI format 2C is updated to include RB set related information

FL’s note:

- For Alt 2: SCI 2C is already very large, is it feasible to further include RB set related information while meeting 140bits limit? Proponents of Alt2, please clarify.

| Company | Agree? | Comments |
| --- | --- | --- |
| Apple |  | We support Alt 2 and think IUC scheme 1 should be supported to largely enjoy the benefit from IUC scheme 1, just like in IUC scheme 2 for SL-U. Regarding FL’s question, our calculation shows even with the modification of SCI 2C for IUC scheme 1, the SCI 2C payload size is below 140 bits: Below is our calculation: Legacy SCI 2C payload size (focus on IUC information which has a larger payload size than IUC request): 34 bits for “HARQ process number”, “NDI”, “RV”, “source ID”, “Destination ID”,”HARQ feedback enabled/disabled indicator”, “CSI request”, “Providing/Requesting indicator” 52 bits for “Resource combination” with N_{subchannel}^{SL}= 27, N_{rsv_period}=16. 8 bits for “First resource location” 17 bits for “Reference slot location” with u=3 (for FR-1, u can be further reduced to 2) 1 bit for “Resource set type” 10 bits for “Lowest sub-channel indices” Hence, the total is 122 bits. For IUC scheme 1 in SL-U, in our understanding: “Resource combination” is increased to 56 bits “Lowest subchannel indices (in an RB set)” is reduced to 8 bits, with N_{subchannel}^{RBset} = 10 “Lowest RB set indices” is newly added with payload size 6 bits, with N_{RBset}=5. With the modifications, the total SCI 2C payload size is 130 bits, within the limit. Note that only 8 bits additional overhead has limited impact on performance. Hence, we think Alt 2 is feasible. |
| OPPO | Yes |  |
| QC | Yes | We support Alt2 |
| xiaomi | yes | Considering the limited payload size in SCI, alt 1 shall be supported. |
| Panasonic | Yes |  |
| Spreadtrum | Yes |  |
| ZTE |  | Alt1 is preferred |
| Futurewei | Yes |  |
| Sharp | Yes | We support Alt 2. Firstly, compared to IUC scheme 2, IUC scheme 1 is more helpful to enhance reliability and reduce latency for SL transmission given preferred or non-preferred resources can be provided to UE-B. Regarding payload size concern, we summarized payload size change in terms of frequency allocation part in the case of providing IUC information. And it can be observed that no additional overhead occurs. Note that, in following table, 27 sub-channels are used to calculate size for legacy case, 10 sub-channels and 2 RB sets are used to calculate size for 15kHz case, and 5 sub-channels and 5 RB sets are used to calculate size for 30kHz case. Comparison of frequency related size in SCI format 2-C. SCI fields; Legacy case; 15kHz case; 30kHz case. Frequency size in Resource combinations (* see note 1); 26; 24; 24. Lowest subChannel indices 2∙log2N subChannel SL; 2∙log227=10; 2∙log210=8; 2∙log25=6. Lowest RB set indices 2∙log2NRBset; 0; 2∙log22=2; 2∙log25=6. Total; 36; 34; 36. Note 1: Frequency size here refers to 2∙log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) in legacy case and 2∙log2(NRBsetNRBset + 12NRBset + 16)+log2(N subChannel RBsetN subChannel RBset + 12N subChannel RBset + 16) in 15kHz case and 30kHz case.. |
| CATT, CICTCI | Yes | Prefer alt 1, and open for discussion |

    - [L] Issue#4-6: Reference slot n for PUCCH transmission to report HARQ in Mode 1

Background:

- Summary

  - Regarding “FFS: whether the specifications are already aligned with this” in previous agreement below

    - No specification changes are needed (3): Docomo, Spreadtrum, xiaomi

      - Main reason is: following red sentence in TS 38.213 is enough

    - Specification changes are needed (2): Apple, ZTE

- FL’s view

  - FL feels this issue can be discussed during Editor’s CR phase directly. So no proposal is given now.

Agreement In “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)”, regarding “the reference slot n for PUCCH transmission to report HARQ in Mode 1”: - slot n is updated as the slot containing the last candidate PSFCH occasion - FFS: whether the specifications are already aligned with this

TS 38.213 16.5 UE procedure for reporting HARQ-ACK on uplink … With reference to slots for PUCCH transmissions and for a number of PSFCH reception occasions ending in slot n, the UE provides the generated HARQ-ACK information in a PUCCH transmission within slot n+k, subject to the overlapping conditions in clause 9.2.5, where k is a number of slots indicated by a PSFCH-to-HARQ feedback timing indicator field, if present, in a DCI format indicating a slot for PUCCH transmission to report the HARQ-ACK information, or k is provided by sl-PSFCH-ToPUCCH for a transmission scheduled by a DCI format or for a SL configured grant type 2, or by sl-PSFCH-ToPUCCH-CG-Type1 for a SL configured grant type 1. k=0 corresponds to a last slot for a PUCCH transmission that would overlap with the last PSFCH reception occasion assuming that the start of the sidelink frame is same as the start of the downlink frame [4, TS 38.211]. …

| Company | Agree? | Comments |
| --- | --- | --- |
| Apple | No | According to “FFS” in the previous agreement, we think it should be discussed in RAN1 and does not rely on Editor on this issue. |
| QC | Yes |  |
| xiaomi | yes |  |
| Panasonic | Yes |  |
| Spreadtrum | Yes |  |
| ZTE | No | This issue is better to be handled in RAN1, for mode 1 Type-1 HARQ-ACK code book generation section, it is better clarified that the candidate PSSCH set MA is derived by the last associated PSFCH occasion. A corresponding text proposal in 38.213 is provided: *** Unchanged parts are omitted *** 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel *** Unchanged parts are omitted *** The cardinality of the set MA defines a total number M of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions corresponding to the HARQ-ACK information bits. For operation with shared spectrum channel access and when there are more than one candidate PSFCH occasions associated with one PSCCH/PSSCH transmission, each of the M candidate PSSCH transmissions is derived by the last associated PSFCH occasion. A UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits as ojACK = HARQ-ACK information bit for candidate PSSCH transmission with index j with corresponding PSFCH reception, for 0 ≤j<M, as described in clause 16.5. If the UE does not transmit a PSSCH in an occasion for candidate PSSCH transmission with corresponding PSFCH reception occasion, due to the UE not detecting a corresponding DCI format 3_0, the UE generates a NACK value for the occasion for candidate PSSCH transmission with corresponding PSFCH reception occasion. *** Unchanged parts are omitted ***. |
| Futurewei | Yes |  |
| Sharp | Yes |  |

    - [L] Issue #4-7: Avoid COT interruption due to PSFCH occasion(s)

Background:

- Summary

  - Avoid COT interruption due to PSFCH occasion(s)

    - Support (9): Qualcomm, LGE, Docomo, MTK, Panasonic, Vivo (via PSSCH), Quectel (via PSSCH), NEC (via PSSCH)

    - Not Support (1): Nokia

- FL’s view

  - A proposal is given to reflect the above.

    - Alt A is from last meeting.

    - Alt B is from vivo/etc. proposal.

    - Alt C will be the outcome if still no consensus in this meeting.

[L] Proposal 4-7

Regarding whether/how to avoid COT interruption due to PSFCH occasion(s):

- Alt A: For “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”

  - COT initiating UE can transmit PSFCH only on common interlace on a PSFCH occasion within the COT, when

    - COT initiating UE does not intend to transmit PSFCH and does not expect to receive PSFCH on the PSFCH occasion within a SL-BWP, and

    - COT initiating UE performs PSCCH/PSSCH transmission just before the PSFCH occasion in the same slot, and

    - COT initiating UE intends to transmit PSCCH/PSSCH or S-SSB after the PSFCH occasion

  - COT responding UE can transmit PSFCH only on common interlace on a PSFCH occasion within the COT based on COT sharing, when

    - COT responding UE does not intend to transmit PSFCH and does not expect to receive PSFCH on the PSFCH occasion within a SL-BWP, and

    - COT responding UE performs PSCCH/PSSCH transmission just before the PSFCH occasion in the same slot, and

    - COT responding UE intends to transmit PSCCH/PSSCH that can utilize the shared COT or S-SSB after the PSFCH occasion

  - The cyclic shift is up to UE implementation

  - No new rule for CPE for this transmission

- Alt B: COT initiating UE indicates in the COT-SI to enable/disable (pre-)configured PSFCH occasion(s) within its COT

  - COT initiating UE or responding UE to transmit PSSCH on the disabled PSFCH occasion(s)

- Alt C: RAN1 does not pursue specific enhancements to avoid COT interruption due to PSFCH occasion(s) in R18 NR SL

| Company | Agree? | Comments |
| --- | --- | --- |
| OPPO | Alt C |  |
| QC | Yes on Alt1 | We strongly feel the need to support Alt A |
| Panasonic | Yes | We support Alt A. |
| Spreadtrum | Yes | We support Alt A. |
| NEC | Yes | We support Alt.B. |
| ZTE | Alt A is preferred | Alt A is benefit for COT maintenance while introducing no new RRC parameter and little spec impact. |
| Sharp | Yes | We support Alt A. |
| CATT, CICTCI | Alt C |  |

  - Issue#5: S-SSB and synchronization

    - [M] Issue#5-1: TP, S-SSB power control

Background:

- Summary

  - OPPO/Samsung/Panasonic have different understanding about “…for transmission of each S-SS/PSBCH block in non-anchor RB-sets” in current 38.213 (copied below).

  - FL tries to use following Example#1 and Alt 1/2/3 to clarify this.

    - @OPPO/Samsung/Panasonic: I proposed following TP based on my understanding of your Tdocs. The intention is to make the TP clearer. If there are any misunderstandings, please comment and suggest new TPs.

  - Example#1

    - Assume SL BWP includes RB set {0,1,2,3,4}, and RB set 2 is anchor RB set

    - Assume UE’s COT includes RB set {1,2,3}

    - Assume UE attempts to transmit S-SSB on RB set {1,2,3}

    - Assume LBT on RB set {3} fails, so UE finally may only transmit S-SSB on RB set {1,2}

    - Then,

      - Alt 1: adopt red change: “…, the UE equally allocates power P'S-SSB(i) remaining from PCMAX, if any, for transmission of each S-SS/PSBCH block in all non-anchor RB-sets within the SL BWP”

        - Support: [Samsung]

        - This implies UE equally allocates power P'S-SSB(i) to RB set {0,1,3,4}

      - Alt 2: adopt red change: “…, the UE equally allocates power P'S-SSB(i) remaining from PCMAX, if any, for transmission of each S-SS/PSBCH block in non-anchor RB-sets where the UE attempted S-SS/PSBCH transmissions”

        - Support: [OPPO]

        - This implies UE equally allocates power P'S-SSB(i) to RB set {1,2,3}

      - Alt 3: adopt red change: “…, the UE equally allocates power P'S-SSB(i) remaining from PCMAX, if any, for transmission of each S-SS/PSBCH block in non-anchor RB-sets where the UE transmits S-SS/PSBCH block”

        - Support: [Panasonic]

        - This implies UE equally allocates power P'S-SSB(i) to RB set {1,2}

- FL’s view

  - A proposal is given to reflect the above.

Agreement Adopt following red change to RAN1#114’s agreement: Agreement Regarding “UE may transmit S-SSB repetition in more than one RB set”: - At least the power for S-SSB transmission on anchor RB set does not change due to the number of used RB sets - On anchor RB set, there is a (pre-)configured offset Poffset_anchor to limit the maximum power as below (changes to legacy NR SL is marked in red) - PS-SSB_anchor(i)=minPCMAX-Poffset_anchor,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm], where i is slot index as in legacy - value range of Poffset_anchor is: {10lg(N), [10lg(N)+2, 10lg(N)+4, …],10lg⁡(W)} - On non-anchor RB set - UE first allocates power to S-SSB repetitions on anchor RB set, assume the power of each S-SSB repetition is PS-SSB_anchor - Then, UE allocates remaining power Pleft equally to other S-SSB repetitions on all other used RB sets, where Pleft=PCMAX-N*PS-SSBanchor, where PCMAX and PS-SSB_anchor are converted to linear unit (i.e, Watt) in this formula - Note: for both anchor RB set and non-anchor RB set transmission, the same DL pathloss is taken into account - M is the total number of RB sets within this SL-BWP, N is the number of S-SSB repetitions within the anchor RB set, W is the maximum total number of S-SSB repetitions on RB sets within the SL-BWP - Note: the above power for S-SSB transmission refers to power of one S-SSB repetition - UE at least attempts to transmit on anchor RB set - Note: anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates - For above Alts, PCMAX is determined according to TS 38.101-1 for transmission of all S-SSB repetitions on all used RB sets.

## (Endorsed Editor CR for TS 38.213 in R1-2310739) ## 16.2 Power control ### 16.2.0 S-SS/PSBCH blocks A UE determines a power PS-SSB(i) for an S-SS/PSBCH block transmission occasion in slot i, in the anchor RB-set if applicable, on active SL BWP b of carrier f as PS-SSB(i)=minPCMAX-Poffset,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PO,S-SSB is a value of dl-P0-PSBCH-r17 if using the parameter is supported by the UE and the parameter is provided; else dl-P0-PSBCH-r16 if provided; otherwise, PS-SSB(i)=PCMAX - αS-SSB is a value of dl-Alpha-PSBCH, if provided; else, αS-SSB=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - MRBS-SSB=11 is a number of resource blocks for a S-SS/PSBCH block transmission with SCS configuration μ - Poffset is a value of sl-PowerOffsetAnchor, if provided; otherwise, Poffset=0. For operation with shared spectrum channel access, after allocating power PS-SSB(i) for transmission of each S-SS/PSBCH block in the anchor RB-set, the UE equally allocates power P'S-SSB(i) remaining from PCMAX, if any, for transmission of each S-SS/PSBCH block in non-anchor RB-sets - if dl-P0-PSBCH is not provided, a power P''S-SSBi for transmission of each S-SS/PSBCH block in a non-anchor RB-set is P''S-SSBi=P'S-SSBi - otherwise, a power P''S-SSBi for transmission of each S-SS/PSBCH block in a non-anchor RB-set is P''S-SSBi=min⁡(P'S-SSBi,P0, S-SSB+10log10(2μ⋅MRBS-SSB)+αS-SSB⋅PL)

[M] Proposal 5-1

For TS 38.213 Clause 16.2.0, adopt one of following TPs:

- Alt 1: “…, the UE equally allocates power P'S-SSB(i) remaining from PCMAX, if any, for transmission of each S-SS/PSBCH block in all non-anchor RB-sets within the SL BWP”

- Alt 2: “…, the UE equally allocates power P'S-SSB(i) remaining from PCMAX, if any, for transmission of each S-SS/PSBCH block in non-anchor RB-sets where the UE attempted S-SS/PSBCH transmissions”

- Alt 3: “…, the UE equally allocates power P'S-SSB(i) remaining from PCMAX, if any, for transmission of each S-SS/PSBCH block in non-anchor RB-sets where the UE transmits S-SS/PSBCH block”

| Reason for change: | When UE transmits S-SSB in more than one RB set, how to allocate remaining power from PCMAX to non-anchor RB set(s) is unclear. |
| --- | --- |
| Summary of change: | Clarify that xxx (to be replaced by chosen Alt). |
| Consequences if not approved: | When UE transmits S-SSB in more than one RB set, how to allocate remaining power from PCMAX to non-anchor RB set(s) is unclear. |

FL’s note:

- Alt 3 above means the UE does S-SSB power control after LBT result is known. Is this feasible? Does UE have enough time to do this? Proponents of Alt3, please clarify.

| Company | Agree? | Comments |
| --- | --- | --- |
| Apple | Yes | We are fine with Alt 2. |
| QC | Yes | We support Alt 2. For Alt 3, it is not feasible for UE to redistribute Tx power based on the LBT outcome. |
| Panasonic | Yes | S-SSB repetition can be transmitted only the slot in shared COT should be clarified. We are fine with Alt 2. |
| Spreadtrum | Yes | We prefer Alt 2. |
| ZTE | Alt2 | alt 2 is preferred, to maintain the COT，we think that the power on the non anchor should be as large as possible. |
| Futurewei | Yes | OK with Alt 2 |
| Lenovo | Yes | We are fine with Alt2. |
| Sharp | Yes | We support Alt 2. Also agree with QC’s assessment of the Alt 3. |

    - [M] Issue#5-2: TP, Transmit S-SSB on anchor RB set on additional S-SSB occasions

Background:

- Summary

  - Samsung point out: as per following conclusion and agreements (especially cyan part), when UE determines to transmit S-SSB on additional S-SSB occasions, UE at least attempts to transmit on anchor RB set. However, this is not captured yet. Samsung gave a TP to clarify this.

- FL’s view

  - A proposal is given to reflect the above.

    - FL gave a TP with addition of following red sentence, based on Samsung’s TP. The intention is to decouple the UE behavior on legacy S-SSB occasion and additional S-SSB occasion to avoid confusion.

      - “When a UE determines to transmit S-SS/PSBCH block on additional candidate S-SS/PSBCH block transmission occasions, the UE attempts to transmit at least S-SS/PSBCH blocks in the anchor RB set.”

Conclusion Regarding additional candidate S-SSB occasions, in the same S-SSB period, UE can attempt to transmit on additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s). Agreement When the SL-BWP contains multiple RB sets, support the followings: - When UE attempts to transmit S-SSB in a S-SSB occasion (e.g., R16/17 S-SSB occasion, R18 additional candidate S-SSB occasion) - UE may transmit S-SSB repetition in more than one RB set - Down-select one of the followings in RAN1#114: - Alt 1: At least the power for S-SSB transmission on anchor RB set does not change due to the number of used RB sets - FFS details, e.g., whether this can be satisfied by (pre-)configuration, whether the power for S-SSB transmission on other RB set(s) also does not change due to the number of used RB sets, etc. - Alt 2: The power for S-SSB transmission on each RB set does not change due to the number of used RB sets - FFS details, e.g., whether this can be satisfied by (pre-)configuration, etc. - FFS: Locations of S-SSB repetitions in each RB set are the same as the locations of S-SSB repetitions in the anchor RB set - FFS: how to (pre)configure resources for the S-SSB repetitions - Note: anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates - Note: whether UE can transmit S-SSBs over non-contiguous RB sets is subject to RAN4’s reply, details can be found in RAN1’s LS to RAN4 in R1-2304218, R1-2306198 Agreement Regarding “UE may transmit S-SSB repetition in more than one RB set”: - At least the power for S-SSB transmission on anchor RB set does not change due to the number of used RB sets - On anchor RB set, there is a (pre-)configured offset Poffset_anchor to limit the maximum power as below (changes to legacy NR SL is marked in red) - PS-SSB_anchor(i)=minPCMAX-Poffset_anchor,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm], where i is slot index as in legacy - value range of Poffset_anchor is: {10lg(N), [10lg(N)+2, 10lg(N)+4, …],10lg⁡(W)} - On non-anchor RB set - UE first allocates power to S-SSB repetitions on anchor RB set, assume the power of each S-SSB repetition is PS-SSB_anchor - Then, UE allocates remaining power Pleft equally to other S-SSB repetitions on all other used RB sets, where Pleft=PCMAX-N*PS-SSBanchor, where PCMAX and PS-SSB_anchor are converted to linear unit (i.e, Watt) in this formula - Note: for both anchor RB set and non-anchor RB set transmission, the same DL pathloss is taken into account - M is the total number of RB sets within this SL-BWP, N is the number of S-SSB repetitions within the anchor RB set, W is the maximum total number of S-SSB repetitions on RB sets within the SL-BWP - Note: the above power for S-SSB transmission refers to power of one S-SSB repetition - UE at least attempts to transmit on anchor RB set - Note: anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates - For above Alts, PCMAX is determined according to TS 38.101-1 for transmission of all S-SSB repetitions on all used RB sets

[M] Proposal 5-2

TP#5-2 in Section 4.1.8 of R1-23xxxxx is endorsed for TS 38.213 clause 16.1.

| Company | Agree? | Comments |
| --- | --- | --- |
| QC | Yes |  |
| Panasonic | Yes |  |
| Futurewei | Yes |  |
| Lenovo | Yes |  |

    - [L] Issue#5-3: S-SSB PAPR reduction

Background:

- Summary

  - S-SSB PAPR reduction was proposed by following companies:

    - Qualcomm, OPPO, MTK, vivo, etc.

- FL’s view

  - A proposal is given to reflect the above.

    - Option 1/2/4 are same as last meeting.

    - FL updated Option 3 based on OPPO’s Tdoc.

[L] Proposal 5-3

Regarding “UE may transmit S-SSB repetition in more than one RB set”, down-select one of the followings:

- Option 1: use different NIDSL across the different S-SSB repetitions to determine the initial scrambling seed of PSBCH, and the sequence shift for S-SSS and S-PSS

  - The S-SSB indicated by sl-AbsoluteFrequencySSB-r16 uses NIDSL

  - The kth repetition uses “NIDSL + k”, where 1≤k≤N⋅MRBset-1

    - N is the number of repetitions in one RB set

    - MRBset is the number of RB sets for S-SSB transmission

- Option 2: phase adjustment among repetitions

  - Phase adjustment sequence is ZC sequence with root index k=0, 1, ⋯, N∙MRBset-1, where

    - k is the S-SSB repetition number index, k is 0 for S-SSB indicated by sl-AbsoluteFrequencySSB-r16, and k equals to 1 to N⋅MRBset-1 for the repetitions of the S-SSB

      - Note the S-SSB indicated by sl-AbsoluteFrequencySSB-r16 is not applied with phase adjustment with k=0

    - N is the number of repetitions in one RB set

  - MRBset is the number of RB sets for S-SSB transmission

- Option 3: it is up to Tx UE implementation whether/how to reduce PAPR

- Option 4: use S-SSB repetition index to scramble different S-SSB repetition(s)

  - Scrambling sequence is Gold sequence with cinit(k)=k⋅floor(220/(N⋅MRBset))⋅210+NIDSL, k=0, 1,…, N⋅MRBset-1, where

    - k is the S-SSB repetition number index, k is 0 for S-SSB indicated by sl-AbsoluteFrequencySSB-r16, and k equals to 1 to N⋅MRBset-1 for the repetitions of the S-SSB,

    - N is the number of repetitions in one RB set.

    - MRBset is the number of RB sets for S-SSB transmission

  - Note: PSBCH/S-PSS/S-SSS indicated by sl-AbsoluteFrequencySSB-r16 remains unchanged

| Company | Agree? | Comments |
| --- | --- | --- |
| Apple |  | We support Option 3. |
| QC | Yes | We support option 1. |
| xiaomi |  | We think this is overoptimization issue . |
| Spreadtrum |  | We prefer option 3. |
| Futurewei | Yes | Option 3 |
| Lenovo | Yes | We support Option 3. |
| vivo | Yes | Option 1 |
| CATT, CICTCI |  | We support option 3 |

  - Issue#6: Others

    - [H] Issue#6-1: Higher layer signaling

Background:

- Summary

  - Introduce “Index 1 startRBResourcePool”: LGE, Huawei/HiSilicon,

    - LGE’s Tdoc: “According to TS 38.331, the description for the higher layer parameter sl-StartRB-Subchannel is “Indicates the lowest RB index of the subchannel with the lowest index in the resource pool with respect to the lowest RB index of a SL BWP”. On the other hand, according to NR-U interlace definition, it is possible that the lowest RB in a resource pool is not belonging to the sub-channel with the lowest index. In this case, sl-StartRB-Subchannel cannot be directly used for indicating the lowest RB index of the resource pool when transmissionStructureForPSCCHandPSSCH is set to ‘interlaceRB’.”

  - On “Index 3 intraCellGuardBandsSL-List”

    - Value range

      - Does not include 0: OPPO

  - On “Index 6 numRefSymbolLength”

    - Per SL BWP: Huawei/HiSilicon,

    - Per resource pool: LGE, Apple, NEC

  - On “Index 11 gapBetweenSSSBrepetition”

    - Value range

      - Remove 0: Huawei/HiSilicon, vivo, NEC

      - Keep 0: OPPO, Apple,

  - On “Index 18 SSSBPowerOffsetOfAnchorRBSet”

    - Value range

      - Remove [10lg(N)+2, 10lg(N)+4, …,]: Huawei/HiSilicon, Apple, NEC,

      - Keep [10lg(N)+1]: Nokia

      - Keep [10lg(N)+2, 10lg(N)+4, …,]: vivo

  - On “Index 19 sl-PSFCH-RB-SetList”

    - Value range

      - SEQUENCE (SIZE (1..4)) OF sl-PSFCH-RB-Set: Huawei/HiSilicon

      - SEQUENCE (SIZE (1… numPSFCHOccasions)) OF BIT STRING(SIZE)10..275): LGE

      - SEQUENCE (SIZE (1..N)) OF BIT STRING (SIZE (10..275)), N is the number of PSFCH occasions for one PSSCH configured by numPSFCHOccasion.: NEC

      - BIT STRING (SIZE (50..275)): Apple

      - SEQUENCE(SIZE(1..max numPSFCHOccasions)) OF BIT STRING (SIZE(51..275)): vivo

      - 10,…,100: Xiaomi

  - On “Index 23 PSFCHPowerOffset”

    - Value range

      - INTEGER (0,1,2,…,6) dB: Huawei/HiSilicon

      - INTEGER (3,6,10) dB: Nokia

      - INTEGER (0,1,2,…,63): Apple

  - Introduce “Index 24 RBSetIndexOfRP”: OPPO

  - Introduce “Index 25 sl-NumRBSetOfResourcePool”: CATT

  - ‘transmissionStructureForPSFCH’, ‘numDedicatedPRBsForPSFCH’ and power offset between common PRB and dedicated PRB (pre)configured are same across all the resource pools: ZTE

    - FL adds red sentence in “Index 14 transmissionStructureForPSFCH” and “Index 15 numDedicatedPRBsForPSFCH” to reflect this.

- FL’s view

  - A proposal is given to reflect the above. RAN1 needs to resolve yellow parts below.

FYI:

(TS 38.331) SL-ResourcePool-r16 ::= SEQUENCE { sl-PSCCH-Config-r16 SetupRelease { SL-PSCCH-Config-r16 } OPTIONAL, -- Need M sl-PSSCH-Config-r16 SetupRelease { SL-PSSCH-Config-r16 } OPTIONAL, -- Need M sl-PSFCH-Config-r16 SetupRelease { SL-PSFCH-Config-r16 } OPTIONAL, -- Need M sl-SyncAllowed-r16 SL-SyncAllowed-r16 OPTIONAL, -- Need M sl-SubchannelSize-r16 ENUMERATED {n10, n12, n15, n20, n25, n50, n75, n100} OPTIONAL, -- Need M dummy INTEGER (10..160) OPTIONAL, -- Need M sl-StartRB-Subchannel-r16 INTEGER (0..265) OPTIONAL, -- Need M sl-StartRB-Subchannel Indicates the lowest RB index of the subchannel with the lowest index in the resource pool with respect to the lowest RB index of a SL BWP. == SL-PSFCH-Config-r16 ::= SEQUENCE { sl-PSFCH-Period-r16 ENUMERATED {sl0, sl1, sl2, sl4} OPTIONAL, -- Need M sl-PSFCH-RB-Set-r16 BIT STRING (SIZE (10..275)) OPTIONAL, -- Need M SL-InterUE-CoordinationScheme2-r17 ::= SEQUENCE { sl-IUC-Scheme2-r17 ENUMERATED {enabled} OPTIONAL, -- Need R sl-RB-SetPSFCH-r17 BIT STRING (SIZE (10..275)) OPTIONAL, -- Need M sl-PSFCH-RB-Set Indicates the set of PRBs that are actually used for PSFCH transmission and reception. The leftmost bit of the bitmap refers to the lowest RB index in the resource pool, and so on. Value 0 in the bitmap indicates that the corresponding PRB is not used for PSFCH transmission and reception while value 1 indicates that the corresponding PRB is used for PSFCH transmission and reception (see TS 38.213 [13]). Sl-RB-SetPSFCH Indicates the set of PRBs that are actually used for inter-UE coordination information transmission and reception in Scheme 2. The leftmost bit of the bitmap refers to the lowest RB index in the resource pool, and so on.

[H] Proposal 6-1

For SL-U PHY, following higher layer parameters are endorsed.

| Index | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) |
| --- | --- | --- | --- | --- | --- |
| 3 | intraCellGuardBandsSL-List | List of intra-cell guard bands for operation with shared spectrum channel access. If not configured, the guard bands are defined according to 38.101-1 [15], see TS 38.214 [19], clause 7. For operation in licensed spectrum, this field is absent, and no UE action is required. | SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16 FFS: whether the value range of nrofCRBs can include 0. | N/A | Per SL BWP |
| 6 | numRefSymbolLength | Indicates a reference number of symbols for TBS determination | ENUMERATED {sym7, sym8, sym9, sym10, sym11, sym12, sym13, sym14} | N/A | [Per SL BWP or per resource pool] |
| 11 | gapBetweenSSSBrepetition | Indicate the gap between two adjacent S-SSB repetitions in frequency domain in one RB set, and the gap is between the lowest subcarrier of the upper PSBCH and the highest subcarrier of the lower PSBCH. | INTEGER([0], 1,2,3,…, 84) PRBs | N/A | Per RB set |
| 12 | numOfAdditionalSSSBOccasion | Indicate the number of additional candidate S-SSB occasion(s) for each R16/R17 NR SL S-SSB slot | INTEGER (0,1,2,3,4) | [N/A or 0] | Per SL BWP |
| 18 | SSSBPowerOffsetOfAnchorRBSet | Indicate the power offset for one S-SSB transmission on anchor RB set. anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates N is the number of S-SSB repetitions within the anchor RB set, W is the maximum total number of S-SSB repetitions on RB sets within the SL-BWP | EMUNERATED{{10lg(N), [10lg(N)+2, 10lg(N)+4, …,]10lg(W)}} | N/A | Per SL BWP |
| 19 | sl-PSFCH-RB-SetList | The n-th value in the list indicates the set of PRBs that are actually used for PSFCH transmission and reception of n-th PSFCH occasion of a PSCCH/PSSCH transmission. It shall be (pre-)configured such that N candidate PSFCH occasion(s) are associated with N different PRB sets. PRBs within intra-cell guard band are not used for PSFCH transmission. N is given by numPSFCHOccasions. | SEQUENCE (SIZE (1..N)) OF sl-PSFCH-RB-Set | N/A | Per resource pool |
| 21 | sl-StartRBsetCG-Type1 | Indicates starting RB set index of the initial PSSCH transmission of the sidelink configured grant Type 1 for interlace RB-based PSSCH transmission Note: M is the number of RB sets within this [SL resource pool]. | INTEGER (0,1,2,…,M-1) | N/A | [Per Configured Grant] |
| 22 | sl-RB-SetPSFCHList | The n-th value in the list indicates the set of PRBs that are actually used for inter-UE coordination information transmission and reception in Scheme 2. It shall be (pre-)configured such that N candidate PSFCH occasion(s) are associated with N different PRB sets. PRBs within intra-cell guard band are not used for PSFCH transmission. N is given by numPSFCHOccasions. | SEQUENCE (SIZE (1..N)) OF sl-RB-SetPSFCH | N/A | Per resource pool |
| 23 | PSFCHPowerOffset | Indicates the power offset between final Tx power on one common PRB (P_common) and final Tx power on one dedicate PRB (P_dedicated) when transmissionStructureForPSFCH is (pre-)configured as common interlace, i.e., P_common = P_dedicated – offset. The unit is dB. | INTEGER (0,1,2,…,10) | N/A | Per SL BWP |
| 1 | startRBResourcePool | Indicates the lowest RB index in the resource pool with respect to the lowest RB index of a SL BWP when tructureForPSCCHandPSSCH is set as interlaceRB. UE expects that the lowest RB in the resource pool is not within intra-cell guard band. | INTEGER (0,1,2,…,224) | N/A | Per resource pool |
| 24 | RBSetIndexOfRP | Indicates the RB set index in the resource pool Note: contiguous RB sets are configured for a resource pool | SEQUENCE (SIZE (1.. NRB-set,SLBWP)) OF INTEGER (0.. NRB-set,SLBWP-1) | N/A | Per resource pool |
| 25 | sl-NumRBSetOfResourcePool | Indicates the number of RB set(s) within one resource pool. | INTEGER (0,1,2,3,4,5) | N/A | Per resource pool |
| 14 | transmissionStructureForPSFCH | Indicate each PSFCH transmission occupies “1 common interlace and K3 dedicated PRB(s)”, or “1 dedicated interlace”. UE expects the same (pre-)configured value of transmissionStructureForPSFCH across all resource pools. | ENUMERATED {common interlace, dedicated interlace} | N/A | Per resource pool |
| 15 | numDedicatedPRBsForPSFCH | Indicates the value of K3 when each PSFCH transmission occupies “1 common interlace and K3 dedicated PRB(s)” UE expects the same (pre-)configured value of numDedicatedPRBsForPSFCH across all resource pools. | ENUMERATED {1,2,5} | N/A | Per resource pool |

| Company | Agree? | Comments |
| --- | --- | --- |
| Apple | No | For “numRefSymbolLength”, we think it is defined per resource pool. For “SSSBPowerOffsetOfAnchorRBSet”, we think its value should not include 10lg(N)+2, 10lg(N)+4, …,] For “sl-PSFCH-RB-SetList”, each RB set is 20 MHz, which is 50 PRBs for 15 kHz. Since a resource pool is composed of integer number of RB sets, the minimum number of PRBs of a resource pool is 50 PRBs. Hence, the minimum value should be 50, rather than 10. |
| Sharp |  | Since the parameter RBSetIndexOfRP (index 24) can cover sl-NumRBSetOfResourcePool (index 25), do we need both |

    - [M] Issue#6-2: Definition of SL RSSI

Background:

- Summary

  - Nokia, CATT, Sharp, OPPO, xiaomi propose to update the definition of SL RSSI.

- FL’s view

  - A proposal is given to reflect the above.

  - In addition, followings are RAN4 agreement/discussion history, companies can take them into account.

    - 202308 (RAN4#108): R4-2314349 WF, RAN4 discussed this issue and has 3 options

    - 202310 (RAN4#108bis)

      - R4-2317217, RAN4 further discussed this issue and reduced to 2 options

      - RAN4 finally agreed Option 1 in WF R4-2317356, i.e., “The legacy requirements for SL RSSI measurement could be reused for SL-U”, i.e., RAN4 already precluded Option 2/3 in above screenshot.

[M] Proposal 6-2

Regarding the definition of SL RSSI, down-select one of followings:

- Option 1: SL RSSI is re-defined as the linear average of the total received power observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH except the 1st candidate starting OFDM symbol and the 2nd candidate starting OFDM symbol.

- Option 2: SL RSSI is re-defined as max {SL RSSI-1, SL RSSI-2}, where

  - SL RSSI-1 is defined as the linear average of the total received power observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH except the 1st candidate starting symbol and the 2nd candidate starting symbols.

  - SL RSSI-2 is defined as the linear average of the total received power observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH starting from the 2nd OFDM symbol after the 2nd candidate starting symbol.

- Option 3: The OFDM symbols used for SL RSSI measurement start from the next symbol of the 2nd candidate starting symbol.

- Option 4: no update on the definition of SL RSSI

FL’s Note:

- Based on the RAN4 discussion and agreement above, it seems RAN4 already discussed this issue and agreed no update on the definition of SL RSSI. So FL feels RAN1 does not need to discuss this (i.e., Option 4 is the default understanding). Companies please take RAN4 situation into account and comment.

| Company | Agree? | Comments |
| --- | --- | --- |
| QC | yes | Option 4 should be good enough. When we configure two starting symbols for PSCCH/PSSCH, it means there is inter-RAT interference. In this case, we don’t believe congestion control works since RSSI cannot differentiate the interference from inter-RAT. Hence, no update is preferred. |
| Xiaomi | No | it is just agreed in RAN4 that the legacy requirements for SL RSSI coulde be resued for SL –u, but as shown in the following bule part, the consensus in RAN4 is that definition of SL RSSI is out of the RAN4’s scope and shall be discussed by RAN1. - We support the option3. Firstly, RSSI measurement can’t distinguish sidelink transmission and WIFI transmission.For the case that one slot is totally occupied by wifi, no matter which options, the measurement result is for wifi rather than sidelink. But for the case that sidelink transmission starts from the second staring symbol in one slot, only the measurement in option3 is for SL transmission. Therefore, comparing with other options, option3 has larger possibility to measure the SL transmission and more accurate. |
| NEC | yes |  |
| Futurewei | Yes | Option 4 |
| Sharp | Yes | We agree with xiaomi and support option 3. |
| CATT, CICICI | Comment | We support to discuss the SL-U RSSI measurement in RAN1 |

    - [L] Issue#6-3: Power control update due to PSD limit

Background:

- Summary

  - Power control update due to PSD limit are analyzed by following companies

    - Vivo, Panasonic, Docomo, Lenovo, [Apple], etc.

    - Note: vivo gave a very detailed TP, companies are encouraged to check.

- FL’s view

  - A proposal is given to reflect the above.

[L] Proposal 6-3

Considering PSD limit in unlicensed spectrum regulation, down-select one of followings:

- Alt 1: RAN1 does not pursue specific enhancements on power control formulas of PSCCH/PSSCH/S-SSB/PSFCH to reflect PSD limit

- Alt 2: RAN1 updates power control of PSCCH/PSSCH/S-SSB/PSFCH by taking PSD limit into account

  - Down-select one of followings:

    - Alt 2-1: The Tx power for each PSFCH RB is not larger than Ppsfch,PSD = PPSD/Nmax_psfch_per_MHz, where PPSD is the PSD limit per MHz, and Nmax_psfch_per_MHz is [the max number of PSFCH RBs in one MHz or is (pre-)configured (FFS value range)] .

    - Alt 2-2: The UE drops some of the PSFCHs iteratively based on the existing PSFCH prioritization rule until the PSD limit is met.

FL’s Note:

- Proponent of Alt 2-1, please give the value range

| Company | Agree? | Comments |
| --- | --- | --- |
| Apple | No | We tend to support the direction of Alt 2. But we are confused with the current Alt 2-1 and Alt 2-2, and feel they are not targeting the same issue. For Alt 2-1, it is discussing each PSFCH RB power due to PSD limit. It does not matter whether PSFCH is dropped or not. For Alt 2-2, it is discussing dropping of some PSFCHs. Also, we think PSD limit is imposed on PPSFCH,one directly. “If PPSFCH,one-10log10(2μ⋅MRBPSFCH) is larger than PSD limitation, then PPSFCH,one is updated/reduced to (10log10(2μ⋅MRBPSFCH)+PSD limit) dBm.” This can be added as a new alternative. |
| OPPO | Yes | Alt 1 is preferred |
| Panasonic | Yes | We support Alt 2 to clarify UE behaviour. |
| vivo | Yes | We support Alt 2. Our concern on Alt 1 is basically on the question of whether the existing UE power control formulation and prioritization rule should be respected or not, when PSD limit regulation is required. Could FL or someone clarify? If yes, how can a UE follow the existing spec and still achieves the PSD requirement? If no, the spec should be updated to say that a UE is allowed to not comply to the power control formulation and prioritization rule in order to meet the PSD limit. Regarding the Number of Nmax_psfch_per_MHz, we think it can be determined by the max number of PSFCH RBs in one MHz, and if configuration is preferred, the value range can be [2, 3, …, 10]. |

    - Others

If you have any other comments, please provide in box below.

| Company | Comments |
| --- | --- |
| Apple | We think in DCI 3_0, there is a mistake on the payload size on the “Lowest index of sub-channel allocation to the initial transmission” field, for “interlaceRB” case. It should be corrected. |
| Sharp | 1. PSSCH preparation time In Rel-15 NR-U, PUSCH preparation time (N2 timeline) was relaxed to take the CP extension into account. For SL-U, a similar relaxation has to be done for PSSCH preparation procedure time in RA mode 1, in order not to increase the UE complexity due to applying CPE. Since the gNB is not aware of the actual CPE starting position of any particular PSSCH transmission, we propose to use a CPE starting position corresponding to the largest CPE duration for a given SCS configuration, as defined in Table 5.3.1-3 of TS 38.211. - Reason for change: For SL-U, if the legacy PSSCH preparation procedure time as specified in clause 8.6 of TS 38.214 is reused “as is”, the time offset between the end of PDCCH reception and the start (of CPE) of the first scheduled PSSCH transmission would be reduced, imposing a more stringent requirement on PSSCH preparation procedure time for the UE comparing to Rel-16. - Summary of change: A new factor “Text” is introduced for PSSCH preparation procedure time, in a similar way to what was done for PUSCH preparation procedure time in NR-U, except that a pre-defined CPE index is used for each SCS configuration, assuming the gNB is not aware of the actual CPE starting position of the PSCCH/PSSCH transmission. - Consequences if not approved: An unintended more stringent requirement on PSSCH preparation procedure time is imposed on UE supporting RA mode 1 in SL-U. ## 8.6 UE PSSCH preparation procedure time For sidelink dynamic grant and for SL configured grant type 2 activation, if the first sidelink symbol in the sidelink allocation for a PSSCH for a transport block and the associated PSCCH, including the DM-RS and the duplicated symbol, as defined by the slot offset KSL of the scheduling DCI for dynamic grant or the activating DCI for SL configured grant type 2, is no earlier than at symbol L, where L is defined as the next sidelink symbol with its CP starting Tproc=(N2+d2,1)(2048+144)⋅κ2-μ⋅TC+Text after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the sidelink transmissions for dynamic grant or activating the SL configured grant type 2, then the UE shall transmit the PSSCH and the associated PSCCH. - N2 is based on µ of Table 8.6-1, where µ corresponds to the one of (µDL, µSL) resulting with the largest Tproc, where the µDL corresponds to the subcarrier spacing of the downlink with which the PDCCH carrying the DCI scheduling the PSSCH for dynamic grant or activating the SL configured grant type 2 was transmitted and µSL corresponds to the subcarrier spacing of the sidelink channel with which the PSSCH and the associated PSCCH are to be transmitted, and κ is defined in Clause 4.1 of [4, TS 38.211]. - For operation with shared spectrum channel access in FR1, Text is calculated according to [4, TS 38.211] with the index for Δi set to ‘6’ for μSL=0, to ‘8’ for μSL=1, and to ‘3’ for μSL=2. Otherwise Text=0. 2. Rate matching for 2nd stage SCI For rate matching of 2nd stage SCI, we think similar issue for number of allocated PRBs exists as TBS determination, e.g. for interlaced RB resource allocation, initial transmission and retransmissions with different number of RBs may occur. The straightforward way is to reuse nPRB for TBS determination as defined in clause 8.1.3.2 of 38.214, as following, ### 8.4.4 Rate Matching For 2nd-stage SCI transmission on PSSCH with SL-SCH, the number of coded modulation symbols generated for 2nd-stage SCI transmission prior to duplication for the 2nd layer if present, denoted as QSCI2', is determined as follows: QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2QmSCI2∙R, αl=0NsymbolPSSCH-1MscSCI2(l)+γ where […] - MscPSSCH(l) is the scheduled bandwidth total number nPRB of allocated PRBs of PSSCH transmission as defined in [6, TS38.214], expressed as a number of subcarriers. - MscPSCCH(l) is the number of subcarriers in OFDM symbol l that carry PSCCH and PSCCH DMRS associated with the PSSCH transmission. […] 3. Rate matching for 2nd stage SCI Current TS38.213 has specified that N candidate PSFCH occasions are applied to the case when interlace PSFCH transmission is configured, with leaving the uncertainty of whether N candidate PSFCH occasions are applicable to the case when legacy PSFCH format 0 is configured for operation with shared spectrum. For operation with shared spectrum, even if the legacy PSFCH format 0 is configured for PSFCH transmission, the UE is still required to perform LBT in order to access the channel. Without applying the N candidate PSFCH occasions, the UE may have no chance to transmit HARQ-ACK feedback once the channel is sensed to be busy. Therefore, as what has been agreed to the interlace PSFCH transmission, it is also beneficial to apply N candidate PSFCH occasions to the case when legacy PSFCH format 0 is configured for PSFCH transmission. Therefore we propose the following. Support: - “One PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)” applies to legacy PSFCH format 0 in SL-U. |

- Collection of Proposals

  - [Closed] Proposals before Monday online

FL’s comment:

- To help delegates have an overview, the following is a collection of Proposals for Round 1 discussions. Online session will probably go with this order.

- Based on input/offline/etc., the proposals and order may be updated before online session.

- Please provide input in the above box for each proposal.

  - If you want to propose TP, please also give all 3 CR fields, i.e., “reason for change”, “summary of changes”, “consequences if not approved”. Otherwise, companies may feel it’s hard to understand and FL may not add it in next update.

- Some Proposals have no TP yet since there are multiple alternatives. FL suggests to down-select first, and RAN1 can discuss TP later or leave it to Editor.

[M] Proposal 4-4-1

Support:

- R17 SL inter-UE coordination Scheme 2 (conflict indication) for sl-PSFCH-Occasion = '1' uses the same transmission scheme (Alt 1-1b and Alt 2-3a) as HARQ-ACK in R18 SL-U

  - For Alt 1-1b, common interlace index for conflict indication and HARQ-ACK within the same RB set are the same

  - Note: Alt 1-1b and Alt 2-3a in previous agreements are as below

    - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

[M] Proposal 4-4-2

Support:

- “One PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)” applies to R17 SL inter-UE coordination Scheme 2 (conflict indication) for both sl-PSFCH-Occasion = '0' and sl-PSFCH-Occasion = '1'

[H] Proposal 4-1

Regarding the order between performing LBT and PSFCH prioritization:

- Down-select one of followings

  - Alt 1: UE performs PSFCH prioritization after LBT result for PSFCH transmission is known

  - Alt 2: UE performs PSFCH prioritization before LBT result for PSFCH transmission is known

  - Alt 3: UE performs PSFCH prioritization without considering LBT result for PSFCH transmission

- When UE intends to transmit PSFCH, if LBT on all RB set(s) fail, down-select one of followings:

  - Alt A: UE drops PSFCH transmission

  - [ Alt B: UE performs prioritization of PSFCH reception and performs PSFCH reception ]

[H] Proposal 4-2

When a UE does not support PSFCH transmission over non-contiguous RB sets, among the RB set(s) whose LBT were successful, down-select one of followings:

- Alt 1: UE selects contiguous RB set(s) including PSFCH(s) with smallest priority value.

  - If there are more than one contiguous RB set(s) including PSFCH(s) with the same smallest priority value, down-select one of followings:

    - Alt 1-1: UE selects contiguous RB set(s) including higher number of PSFCH(s)

      - If there are more than one contiguous RB set(s) including PSFCH(s) with the same number of PSFCH(s), down-select one of followings:

        - Alt 1-1-1: it is up to UE implementation to select one contiguous RB set among them

        - Alt 1-1-2: UE selects contiguous RB set(s) with lower RB set index

    - Alt 1-2: It is up to UE implementation to select one contiguous RB set among them

- [ Alt 3: For Alt 1-1b, UE can transmit PSFCH on (pre-)configured common interlace in the RB set(s) between any non-contiguous RB sets ]

  - FFS: UE behaviour under Alt 2-3a

  - Note: Alt 1-1b and Alt 2-3a in previous agreements are as below

    - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

- Alt 4: It is up to UE implementation to select one contiguous RB set which satisfies at least one of following conditions

  - This contiguous RB set includes PSFCH with the smallest priority value

    - If there are more than one contiguous RB set(s) including PSFCH(s) with the same smallest priority value, UE selects any one of contiguous RB set(s) including the highest number of PSFCHs

  - This contiguous RB set contains PSFCH which is intended for a COT initiating UE in a shared COT if the UE shares the COT

  - This contiguous RB set contains PSFCH within a COT if the UE initiates the COT

[H] Proposal 6-1

For SL-U PHY, following higher layer parameters are endorsed.

| index | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) |
| --- | --- | --- | --- | --- | --- |
| 3 | intraCellGuardBandsSL-List | List of intra-cell guard bands for operation with shared spectrum channel access. If not configured, the guard bands are defined according to 38.101-1 [15], see TS 38.214 [19], clause 7. For operation in licensed spectrum, this field is absent, and no UE action is required. | SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16 FFS: whether the value range of nrofCRBs can include 0. | N/A | Per SL BWP |
| 6 | numRefSymbolLength | Indicates a reference number of symbols for TBS determination | ENUMERATED {sym7, sym8, sym9, sym10, sym11, sym12, sym13, sym14} | N/A | [Per SL BWP or per resource pool] |
| 11 | gapBetweenSSSBrepetition | Indicate the gap between two adjacent S-SSB repetitions in frequency domain in one RB set, and the gap is between the lowest subcarrier of the upper PSBCH and the highest subcarrier of the lower PSBCH. | INTEGER([0], 1,2,3,…, 84) PRBs | N/A | Per RB set |
| 12 | numOfAdditionalSSSBOccasion | Indicate the number of additional candidate S-SSB occasion(s) for each R16/R17 NR SL S-SSB slot | INTEGER (0,1,2,3,4) | [N/A or 0] | Per SL BWP |
| 18 | SSSBPowerOffsetOfAnchorRBSet | Indicate the power offset for one S-SSB transmission on anchor RB set. anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates N is the number of S-SSB repetitions within the anchor RB set, W is the maximum total number of S-SSB repetitions on RB sets within the SL-BWP | EMUNERATED{{10lg(N), [10lg(N)+2, 10lg(N)+4, …,]10lg(W)}} | N/A | Per SL BWP |
| 19 | sl-PSFCH-RB-SetList | The n-th value in the list indicates the set of PRBs that are actually used for PSFCH transmission and reception of n-th PSFCH occasion of a PSCCH/PSSCH transmission. It shall be (pre-)configured such that N candidate PSFCH occasion(s) are associated with N different PRB sets. PRBs within intra-cell guard band are not used for PSFCH transmission. N is given by numPSFCHOccasions. | SEQUENCE (SIZE (1..N)) OF sl-PSFCH-RB-Set | N/A | Per resource pool |
| 21 | sl-StartRBsetCG-Type1 | Indicates starting RB set index of the initial PSSCH transmission of the sidelink configured grant Type 1 for interlace RB-based PSSCH transmission Note: M is the number of RB sets within this [SL resource pool]. | INTEGER (0,1,2,...,M-1) | N/A | [Per Configured Grant] |
| 22 | sl-RB-SetPSFCHList | The n-th value in the list indicates the set of PRBs that are actually used for inter-UE coordination information transmission and reception in Scheme 2. It shall be (pre-)configured such that N candidate PSFCH occasion(s) are associated with N different PRB sets. PRBs within intra-cell guard band are not used for PSFCH transmission. N is given by numPSFCHOccasions. | SEQUENCE (SIZE (1..N)) OF sl-RB-SetPSFCH | N/A | Per resource pool |
| 23 | PSFCHPowerOffset | Indicates the power offset between final Tx power on one common PRB (P_common) and final Tx power on one dedicate PRB (P_dedicated) when transmissionStructureForPSFCH is (pre-)configured as common interlace, i.e., P_common = P_dedicated - offset. The unit is dB. | INTEGER (0,1,2,…,10) | N/A | Per SL BWP |
| 1 | startRBResourcePool | Indicates the lowest RB index in the resource pool with respect to the lowest RB index of a SL BWP when tructureForPSCCHandPSSCH is set as interlaceRB. UE expects that the lowest RB in the resource pool is not within intra-cell guard band. | INTEGER (0,1,2,…,224) | N/A | Per resource pool |
| 24 | RBSetIndexOfRP | Indicates the RB set index in the resource pool Note: contiguous RB sets are configured for a resource pool | SEQUENCE (SIZE (1.. NRB-set,SLBWP)) OF INTEGER (0.. NRB-set,SLBWP-1) | N/A | Per resource pool |
| 25 | sl-NumRBSetOfResourcePool | Indicates the number of RB set(s) within one resource pool. | INTEGER (0,1,2,3,4,5) | N/A | Per resource pool |
| 14 | transmissionStructureForPSFCH | Indicate each PSFCH transmission occupies "1 common interlace and K3 dedicated PRB(s)", or "1 dedicated interlace". UE expects the same (pre-)configured value of transmissionStructureForPSFCH across all resource pools. | ENUMERATED {common interlace, dedicated interlace} | N/A | Per resource pool |
| 15 | numDedicatedPRBsForPSFCH | Indicates the value of K3 when each PSFCH transmission occupies "1 common interlace and K3 dedicated PRB(s)" UE expects the same (pre-)configured value of numDedicatedPRBsForPSFCH across all resource pools. | ENUMERATED {1,2,5} | N/A | Per resource pool |

[H] Proposal 4-3-1

Regarding “SL-U PSFCH occupies more than 1 PRB”, down-select one of followings:

- Alt 1: Support following red changes to legacy PSFCH power control formula

  - PPSFCH,one=PO,PSFCH+10log102μ⋅MRBPSFCH+αPSFCH⋅PL [dBm], where MRBPSFCH is a reference number of resource blocks for a PSFCH transmission, down-select one of followings:

    - Alt A: MRBPSFCH is (pre-)configured per SL BWP, and the value range is {10, 11, 12, 13, 15, 16}

    - Alt B:

      - For Alt 1-1b: MRBPSFCH is equal to numDedicatedPRBsForPSFCH

      - For Alt 2-3a: MRBPSFCH is equal to numRefPRBOfInterlace

      - UE expects the same (pre-)configured value of numDedicatedPRBsForPSFCH and numDedicatedPRBsForPSFCH across all resource pools

- Alt 2: Support the followings

  - For Alt 1-1b:

    - PPSFCH,one refers to the power on one dedicated PRB

    - PPSFCH,first refers to the power on one common PRB

      - PPSFCH,first=PPSFCH,one-PPSFCH,offset, where PPSFCH,offset is provided by PSFCHPowerOffset.

    - PPSFCH,k(i) refers to the total power on all dedicated PRB(s) for a PSFCH transmission k

    - Let MRB,kPSFCHi denotes the number of dedicated PRBs of a PSFCH transmission k in PSFCH transmission occasion i

    - Let MRB,firstPSFCH denotes the number of PRBs in the common interlace

    - Then, use following formulas for PCMAX checking

      - PPSFCH,one*k=1Nsch,Tx,PSFCHMRB,kPSFCHi+PPSFCH,first*MRB,firstPSFCH≤PCMAX

      - PPSFCH,one*k=1Nmax,PSFCHMRB,kPSFCHi+PPSFCH,first*MRB,firstPSFCH≤PCMAX

      - where,

        - Nsch,Tx,PSFCH and Nmax,PSFCH are the same as legacy NR SL

        - assume the math symbols in above two formulas are converted to linear unit (i.e, Watt)

  - For Alt 2-3a:

    - PPSFCH,one refers to the power on one dedicated PRB

    - PPSFCH,k(i) refers to the total power on all dedicated PRB(s) for a PSFCH transmission k

    - Let MRB,kPSFCHi denotes the number of dedicated PRBs of a PSFCH transmission k in PSFCH transmission occasion i

    - Then, use following formulas for PCMAX checking

      - PPSFCH,one*k=1Nsch,Tx,PSFCHMRB,kPSFCHi≤PCMAX

      - PPSFCH,one*k=1Nmax,PSFCHMRB,kPSFCHi≤PCMAX

      - where,

        - Nsch,Tx,PSFCH and Nmax,PSFCH are the same as legacy NR SL

        - assume the math symbols in above two formulas are converted to linear unit (i.e, Watt)

  - TP#4-3-1 in Section 4.1.5 of R1-23xxxxx (to be FLS) is endorsed for TS 38.213 Clause 16.2.3.

- Note: Alt 1-1b and Alt 2-3a in previous agreements are as below

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

[H] Proposal 4-3-2

TP#4-3-2 in Section 4.1.6 of R1-23xxxxx is endorsed for TS 38.213 clause 16.3.0.

[H] Proposal 4-3-3

TP#4-3-3 in Section 4.1.7 of R1-23xxxxx is endorsed for TS 38.213 clause 16.2.3.

[M] Proposal 5-1

For TS 38.213 Clause 16.2.0, adopt one of following TPs:

- Alt 1: “…, the UE equally allocates power P'S-SSB(i) remaining from PCMAX, if any, for transmission of each S-SS/PSBCH block in all non-anchor RB-sets within the SL BWP”

- Alt 2: “…, the UE equally allocates power P'S-SSB(i) remaining from PCMAX, if any, for transmission of each S-SS/PSBCH block in non-anchor RB-sets where the UE attempted S-SS/PSBCH transmissions”

- Alt 3: “…, the UE equally allocates power P'S-SSB(i) remaining from PCMAX, if any, for transmission of each S-SS/PSBCH block in non-anchor RB-sets where the UE transmits S-SS/PSBCH block”

| Reason for change: | When UE transmits S-SSB in more than one RB set, how to allocate remaining power from PCMAX to non-anchor RB set(s) is unclear. |
| --- | --- |
| Summary of change: | Clarify that xxx (to be replaced by chosen Alt). |
| Consequences if not approved: | When UE transmits S-SSB in more than one RB set, how to allocate remaining power from PCMAX to non-anchor RB set(s) is unclear. |

[M] Proposal 6-2

Regarding the definition of SL RSSI, down-select one of followings:

- Option 1: SL RSSI is re-defined as the linear average of the total received power observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH except the 1st candidate starting OFDM symbol and the 2nd candidate starting OFDM symbol.

- Option 2: SL RSSI is re-defined as max {SL RSSI-1, SL RSSI-2}, where

  - SL RSSI-1 is defined as the linear average of the total received power observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH except the 1st candidate starting symbol and the 2nd candidate starting symbols.

  - SL RSSI-2 is defined as the linear average of the total received power observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH starting from the 2nd OFDM symbol after the 2nd candidate starting symbol.

- Option 3: The OFDM symbols used for SL RSSI measurement start from the next symbol of the 2nd candidate starting symbol.

- Option 4: no update on the definition of SL RSSI

[M] Proposal 5-2

TP#5-2 in Section 4.1.8 of R1-23xxxxx is endorsed for TS 38.213 clause 16.1.

[M] Proposal 1-1

TP#1-1 in Section 4.1.1 of R1-23xxxxx (to be FLS) is endorsed for TS 38.214 Clause 8.

[M] Proposal 2-1

TP#2-1 in Section 4.1.2 of R1-23xxxxx is endorsed for TS 38.214 Clause 8.1.2.1.

[M] Proposal 3-1

TP#3-1 in Section 4.1.3 of R1-23xxxxx is endorsed for TS 38.214 Clause 8.

[M] Proposal 3-2

TP#3-2 in Section 4.1.4 of R1-23xxxxx is endorsed for TS 38.214 Clause 8.1.5.

[L] Proposal 4-7

Regarding whether/how to avoid COT interruption due to PSFCH occasion(s):

- Alt A: For “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”

  - COT initiating UE can transmit PSFCH only on common interlace on a PSFCH occasion within the COT, when

    - COT initiating UE does not intend to transmit PSFCH and does not expect to receive PSFCH on the PSFCH occasion within a SL-BWP, and

    - COT initiating UE performs PSCCH/PSSCH transmission just before the PSFCH occasion in the same slot, and

    - COT initiating UE intends to transmit PSCCH/PSSCH or S-SSB after the PSFCH occasion

  - COT responding UE can transmit PSFCH only on common interlace on a PSFCH occasion within the COT based on COT sharing, when

    - COT responding UE does not intend to transmit PSFCH and does not expect to receive PSFCH on the PSFCH occasion within a SL-BWP, and

    - COT responding UE performs PSCCH/PSSCH transmission just before the PSFCH occasion in the same slot, and

    - COT responding UE intends to transmit PSCCH/PSSCH that can utilize the shared COT or S-SSB after the PSFCH occasion

  - The cyclic shift is up to UE implementation

  - No new rule for CPE for this transmission

- Alt B: COT initiating UE indicates in the COT-SI to enable/disable (pre-)configured PSFCH occasion(s) within its COT

  - COT initiating UE or responding UE to transmit PSSCH on the disabled PSFCH occasion(s)

- Alt C: RAN1 does not pursue specific enhancements to avoid COT interruption due to PSFCH occasion(s) in R18 NR SL

[L] Proposal 6-3

Considering PSD limit in unlicensed spectrum regulation, down-select one of followings:

- Alt 1: RAN1 does not pursue specific enhancements on power control formulas of PSCCH/PSSCH/S-SSB/PSFCH to reflect PSD limit

- Alt 2: RAN1 updates power control of PSCCH/PSSCH/S-SSB/PSFCH by taking PSD limit into account

  - Down-select one of followings:

    - Alt 2-1: The Tx power for each PSFCH RB is not larger than Ppsfch,PSD = PPSD/Nmax_psfch_per_MHz, where PPSD is the PSD limit per MHz, and Nmax_psfch_per_MHz is [the max number of PSFCH RBs in one MHz or is (pre-)configured (FFS value range)] .

    - Alt 2-2: The UE drops some of the PSFCHs iteratively based on the existing PSFCH prioritization rule until the PSD limit is met.

[L] Proposal 4-5

When OCB is required, down-select one of followings:

- Alt 1: R17 SL inter-UE coordination Scheme 1 (preferred/non-preferred resources) is not supported

- Alt 2: R17 SL inter-UE coordination Scheme 1 (preferred/non-preferred resources) is supported

  - SCI format 2C is updated to include RB set related information

[L] Proposal 5-3

Regarding “UE may transmit S-SSB repetition in more than one RB set”, down-select one of the followings:

- Option 1: use different NIDSL across the different S-SSB repetitions to determine the initial scrambling seed of PSBCH, and the sequence shift for S-SSS and S-PSS

  - The S-SSB indicated by sl-AbsoluteFrequencySSB-r16 uses NIDSL

  - The kth repetition uses “NIDSL + k”, where 1≤k≤N⋅MRBset-1

    - N is the number of repetitions in one RB set

    - MRBset is the number of RB sets for S-SSB transmission

- Option 2: phase adjustment among repetitions

  - Phase adjustment sequence is ZC sequence with root index k=0, 1, ⋯, N∙MRBset-1, where

    - k is the S-SSB repetition number index, k is 0 for S-SSB indicated by sl-AbsoluteFrequencySSB-r16, and k equals to 1 to N⋅MRBset-1 for the repetitions of the S-SSB

      - Note the S-SSB indicated by sl-AbsoluteFrequencySSB-r16 is not applied with phase adjustment with k=0

    - N is the number of repetitions in one RB set

  - MRBset is the number of RB sets for S-SSB transmission

- Option 3: it is up to Tx UE implementation whether/how to reduce PAPR

- Option 4: use S-SSB repetition index to scramble different S-SSB repetition(s)

  - Scrambling sequence is Gold sequence with cinit(k)=k⋅floor(220/(N⋅MRBset))⋅210+NIDSL, k=0, 1,…, N⋅MRBset-1, where

    - k is the S-SSB repetition number index, k is 0 for S-SSB indicated by sl-AbsoluteFrequencySSB-r16, and k equals to 1 to N⋅MRBset-1 for the repetitions of the S-SSB,

    - N is the number of repetitions in one RB set.

    - MRBset is the number of RB sets for S-SSB transmission

  - Note: PSBCH/S-PSS/S-SSS indicated by sl-AbsoluteFrequencySSB-r16 remains unchanged

  - Proposals for Monday online

[M] Proposal 4-4-1

Support:

- R17 SL inter-UE coordination Scheme 2 (conflict indication) for sl-PSFCH-Occasion = '1' uses the same transmission scheme (Alt 1-1b and Alt 2-3a) as HARQ-ACK in R18 SL-U

  - For Alt 1-1b, common interlace index for conflict indication and HARQ-ACK within the same RB set are the same

  - Note: Alt 1-1b and Alt 2-3a in previous agreements are as below

    - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

[M] Proposal 4-4-2

Support:

- “One PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)” applies to R17 SL inter-UE coordination Scheme 2 (conflict indication) for both sl-PSFCH-Occasion = '0' and sl-PSFCH-Occasion = '1'

  - PSFCH transmission and reception on such N occasion(s) of IUC scheme 2 is the same as HARQ-ACK

[H] Proposal 4-1

Regarding the order between performing LBT and PSFCH prioritization:

- Down-select one of followings

  - Alt 1: UE performs PSFCH prioritization after LBT result for PSFCH transmission is known

  - Alt 2: UE performs PSFCH prioritization before LBT result for PSFCH transmission is known

  - Alt 3: UE performs PSFCH prioritization without considering LBT result for PSFCH transmission

- When UE intends to transmit PSFCH after performing PSFCH prioritization, if LBT on all RB set(s) fail, down-select one of followings:

  - Alt A: UE drops PSFCH transmission

  - [ Alt B: UE performs prioritization of PSFCH reception and performs PSFCH reception ]

FL’s note for above proposal:

- FL adds “…after performing PSFCH prioritization…” based on Sharp’s comment.

- @vivo, ZTE:

  - To FL, one different between Alt 2 and Alt 3 is:

    - In Alt 3, in PSFCH prioritization section of TS 38.213, we’ll not mention LBT results.

    - In Alt 2, in PSFCH prioritization section of TS 38.213, we may need to mention PSFCH prioritization is done before LBT result is known.

==

[H] Proposal 4-2

When a UE does not support PSFCH transmission over non-contiguous RB sets, among the RB set(s) whose LBT were successful, down-select one of followings:

- Alt 1: UE selects contiguous RB set(s) including PSFCH(s) with smallest priority value.

  - If there are more than one contiguous RB set(s) including PSFCH(s) with the same smallest priority value, down-select one of followings:

    - Alt 1-1: UE selects contiguous RB set(s) including higher number of PSFCH(s)

      - If there are more than one contiguous RB set(s) including PSFCH(s) with the same number of PSFCH(s), down-select one of followings:

        - Alt 1-1-1: it is up to UE implementation to select one contiguous RB set among them

        - Alt 1-1-2: UE selects contiguous RB set(s) with lower RB set index

    - Alt 1-2: It is up to UE implementation to select one contiguous RB set among them

- [ Alt 3: For Alt 1-1b, UE can transmit PSFCH on (pre-)configured common interlace in the RB set(s) between any non-contiguous RB sets ]

  - FFS: UE behaviour under Alt 2-3a

  - Note: Alt 1-1b and Alt 2-3a in previous agreements are as below

    - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

- Alt 4: It is up to UE implementation to select one contiguous RB set which satisfies at least one of following conditions

  - This contiguous RB set includes PSFCH with the smallest priority value

    - If there are more than one contiguous RB set(s) including PSFCH(s) with the same smallest priority value, UE selects any one of contiguous RB set(s) including the highest number of PSFCHs

  - This contiguous RB set contains PSFCH which is intended for a COT initiating UE in a shared COT if the UE shares the COT

  - This contiguous RB set contains PSFCH within a COT if the UE initiates the COT

FL’s note for above proposal:

- @vivo: this proposal applies to both type 1 and 2 channel access.

  - If Alt 1 is chosen, and the selected PSFCH does not include the one to COT initiating UE, the UE cannot enjoy COT sharing and uses type 1. It still works.

==

[H] Proposal 6-1

For SL-U PHY, following higher layer parameters are endorsed.

| index | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) |
| --- | --- | --- | --- | --- | --- |
| 3 | intraCellGuardBandsSL-List | List of intra-cell guard bands for operation with shared spectrum channel access. If not configured, the guard bands are defined according to 38.101-1 [15], see TS 38.214 [19], clause 7. For operation in licensed spectrum, this field is absent, and no UE action is required. | SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16 FFS: whether the value range of nrofCRBs can include 0. | N/A | Per SL BWP |
| 6 | numRefSymbolLength | Indicates a reference number of symbols for TBS determination | ENUMERATED {sym7, sym8, sym9, sym10, sym11, sym12, sym13, sym14} | N/A | [Per SL BWP or per resource pool] |
| 11 | gapBetweenSSSBrepetition | Indicate the gap between two adjacent S-SSB repetitions in frequency domain in one RB set, and the gap is between the lowest subcarrier of the upper PSBCH and the highest subcarrier of the lower PSBCH. | INTEGER([0], 1,2,3,…, 84) PRBs | N/A | Per RB set |
| 12 | numOfAdditionalSSSBOccasion | Indicate the number of additional candidate S-SSB occasion(s) for each R16/R17 NR SL S-SSB slot | INTEGER (0,1,2,3,4) | [N/A or 0] | Per SL BWP |
| 18 | SSSBPowerOffsetOfAnchorRBSet | Indicate the power offset for one S-SSB transmission on anchor RB set. anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates N is the number of S-SSB repetitions within the anchor RB set, W is the maximum total number of S-SSB repetitions on RB sets within the SL-BWP | EMUNERATED{{10lg(N), [10lg(N)+2, 10lg(N)+4, …,]10lg(W)}} | N/A | Per SL BWP |
| 19 | sl-PSFCH-RB-SetList | The n-th value in the list indicates the set of PRBs that are actually used for PSFCH transmission and reception of n-th PSFCH occasion of a PSCCH/PSSCH transmission. It shall be (pre-)configured such that N candidate PSFCH occasion(s) are associated with N different PRB sets. PRBs within intra-cell guard band are not used for PSFCH transmission. N is given by numPSFCHOccasions. | Down-select one: - Alt1: SEQUENCE (SIZE (1..N)) OF sl-PSFCH-RB-Set - Alt2: BIT STRING (SIZE (50..275)) | N/A | Per resource pool |
| 21 | sl-StartRBsetCG-Type1 | Indicates starting RB set index of the initial PSSCH transmission of the sidelink configured grant Type 1 for interlace RB-based PSSCH transmission Note: M is the number of RB sets within this [SL resource pool]. | INTEGER (0,1,2,...,M-1) | N/A | [Per Configured Grant] |
| 22 | sl-RB-SetPSFCHList | The n-th value in the list indicates the set of PRBs that are actually used for inter-UE coordination information transmission and reception in Scheme 2. It shall be (pre-)configured such that N candidate PSFCH occasion(s) are associated with N different PRB sets. PRBs within intra-cell guard band are not used for PSFCH transmission. N is given by numPSFCHOccasions. | Down-select one: - Alt 1: SEQUENCE (SIZE (1..N)) OF sl-RB-SetPSFCH - Alt2: BIT STRING (SIZE (50..275)) | N/A | Per resource pool |
| 23 | PSFCHPowerOffset | Indicates the power offset between final Tx power on one common PRB (P_common) and final Tx power on one dedicate PRB (P_dedicated) when transmissionStructureForPSFCH is (pre-)configured as common interlace, i.e., P_common = P_dedicated - offset. The unit is dB. | INTEGER (0,1,2,…,10) | N/A | Per SL BWP |
| 1 | startRBResourcePool | Indicates the lowest RB index in the resource pool with respect to the lowest RB index of a SL BWP when tructureForPSCCHandPSSCH is set as interlaceRB. UE expects that the lowest RB in the resource pool is not within intra-cell guard band. | INTEGER (0,1,2,…,224) | N/A | Per resource pool |
| 24 | RBSetIndexOfRP | Indicates the RB set index in the resource pool Note: contiguous RB sets are configured for a resource pool | SEQUENCE (SIZE (1.. NRB-set,SLBWP)) OF INTEGER (0.. NRB-set,SLBWP-1) | N/A | Per resource pool |
| 25 | sl-NumRBSetOfResourcePool | Indicates the number of RB set(s) within one resource pool. | INTEGER (0,1,2,3,4,5) | N/A | Per resource pool |
| 14 | transmissionStructureForPSFCH | Indicate each PSFCH transmission occupies "1 common interlace and K3 dedicated PRB(s)", or "1 dedicated interlace". UE expects the same (pre-)configured value of transmissionStructureForPSFCH across all resource pools. | ENUMERATED {common interlace, dedicated interlace} | N/A | Per resource pool |
| 15 | numDedicatedPRBsForPSFCH | Indicates the value of K3 when each PSFCH transmission occupies "1 common interlace and K3 dedicated PRB(s)" UE expects the same (pre-)configured value of numDedicatedPRBsForPSFCH across all resource pools. | ENUMERATED {1,2,5} | N/A | Per resource pool |

FL’s note for above proposal:

- @Apple: for “Index 19 sl-PSFCH-RB-SetList”, FL adds following Alt 2 based on your comment. But I feel Alt2 is not complete? At least we need to mention “SEQUENCE (SIZE (1..N)) OF”? Please help give a complete Alt 2.

  - “Alt2: BIT STRING (SIZE (50..275))”

- @OPPO, CATT: please check Sharp’s comment below

  - Sharp: “Since the parameter RBSetIndexOfRP (index 24) can cover sl-NumRBSetOfResourcePool (index 25), do we need both”

==

[H] Proposal 4-3-1

Regarding “SL-U PSFCH occupies more than 1 PRB”, down-select one of followings:

- Alt 1: Support following red changes to legacy PSFCH power control formula

  - For Alt 2-3a, PPSFCH,one=PO,PSFCH+10log102μ⋅MRBPSFCH+αPSFCH⋅PL [dBm],

  - For Alt 1-1b, PPSFCH,one= 10 log10[ 10[PO,PSFCH+10log102μ⋅MRBPSFCH+αPSFCH⋅PL]÷10 +10MRBPSFCH×PPSFCH,offset÷10] [dBm]，wherePPSFCH,offset is provided by PSFCHPowerOffset

  - where MRBPSFCH is a reference number of resource blocks for a PSFCH transmission, down-select one of followings:

    - Alt A: MRBPSFCH is (pre-)configured per SL BWP, and the value range is {10, 11, 12, 13, 15, 16}

    - Alt B:

      - For Alt 1-1b: MRBPSFCH is equal to numDedicatedPRBsForPSFCH

      - For Alt 2-3a: MRBPSFCH is equal to numRefPRBOfInterlace

      - UE expects the same (pre-)configured value of numDedicatedPRBsForPSFCH and numDedicatedPRBsForPSFCH across all resource pools

  - TP#4-3-4 in Section 4.1.9 of R1-23xxxxx (to be FLS) for TS 38.213 Clause 16.2.3 is considered as starting point

- Alt 2: Support the followings

  - For Alt 1-1b:

    - PPSFCH,one refers to the power on one dedicated PRB

    - PPSFCH,first refers to the power on one common PRB

      - PPSFCH,first=PPSFCH,one-PPSFCH,offset, where PPSFCH,offset is provided by PSFCHPowerOffset.

    - PPSFCH,k(i) refers to the total power on all dedicated PRB(s) for a PSFCH transmission k

    - Let MRB,kPSFCHi denotes the number of dedicated PRBs of a PSFCH transmission k in PSFCH transmission occasion i

    - Let MRB,firstPSFCH denotes the number of PRBs in the common interlace

    - Then, use following formulas for PCMAX checking

      - PPSFCH,one*k=1Nsch,Tx,PSFCHMRB,kPSFCHi+PPSFCH,first*MRB,firstPSFCH≤PCMAX

      - PPSFCH,one*k=1Nmax,PSFCHMRB,kPSFCHi+PPSFCH,first*MRB,firstPSFCH≤PCMAX

      - where,

        - Nsch,Tx,PSFCH and Nmax,PSFCH are the same as legacy NR SL

        - assume the math symbols in above two formulas are converted to linear unit (i.e, Watt)

  - For Alt 2-3a:

    - PPSFCH,one refers to the power on one dedicated PRB

    - PPSFCH,k(i) refers to the total power on all dedicated PRB(s) for a PSFCH transmission k

    - Let MRB,kPSFCHi denotes the number of dedicated PRBs of a PSFCH transmission k in PSFCH transmission occasion i

    - Then, use following formulas for PCMAX checking

      - PPSFCH,one*k=1Nsch,Tx,PSFCHMRB,kPSFCHi≤PCMAX

      - PPSFCH,one*k=1Nmax,PSFCHMRB,kPSFCHi≤PCMAX

      - where,

        - Nsch,Tx,PSFCH and Nmax,PSFCH are the same as legacy NR SL

        - assume the math symbols in above two formulas are converted to linear unit (i.e, Watt)

  - TP#4-3-1 in Section 4.1.5 of R1-23xxxxx (to be FLS) is endorsed for TS 38.213 Clause 16.2.3.

- Note: Alt 1-1b and Alt 2-3a in previous agreements are as below

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

FL’s note for above proposal:

- For Alt 1:

  - FL add blue parts based on Xiaomi’s comment.

  - @Apple: previously, FL does not include TP for Alt 1 since it has multiple alternatives to be down-selected.

    - Anyway, since you commented to add TP for it, FL can do that.

    - @all, FL adds following red sentence for Alt1. The TP is copied from Apple’s Tdoc in R1-2311680 section 2.3.2. Since FL newly adds blue parts based on Xiaomi’s comment, FL assumes this TP can not be endorsed directly, and can be considered as starting point for Alt 1. @Apple/Xiaomi/etc, please try to have a stabilized Alt 1 and related TP, and also provide 3 CR fields (which was missing in Apple’s Tdoc).

      - “TP#4-3-4 in Section 4.1.9 of R1-23xxxxx (to be FLS) for TS 38.213 Clause 16.2.3 is considered as starting point”

==

[H] Proposal 4-3-2

TP#4-3-2 in Section 4.1.6 of R1-23xxxxx is endorsed for TS 38.213 clause 16.3.0.

[H] Proposal 4-3-3

TP#4-3-3 in Section 4.1.7 of R1-23xxxxx is endorsed for TS 38.213 clause 16.2.3.

FL’s note for above proposal:

- We can discuss Proposal 4-3-1 first, and hold on Proposal 4-3-2/4-3-3 a bit.

- For TP#4-3-3: FL adopts OPPO’s suggestion, i.e., “… all NTx,PSFCH PSFCH transmissions …”.

==

[M] Proposal 5-1

For TS 38.213 Clause 16.2.0, adopt one of following TPs:

- Alt 1: “…, the UE equally allocates power P'S-SSB(i) remaining from PCMAX, if any, for transmission of each S-SS/PSBCH block in all non-anchor RB-sets within the SL BWP”

- Alt 2: “…, the UE equally allocates power P'S-SSB(i) remaining from PCMAX, if any, for transmission of each S-SS/PSBCH block in non-anchor RB-sets where the UE attempted S-SS/PSBCH transmissions”

- Alt 3: “…, the UE equally allocates power P'S-SSB(i) remaining from PCMAX, if any, for transmission of each S-SS/PSBCH block in non-anchor RB-sets where the UE transmits S-SS/PSBCH block”

| Reason for change: | When UE transmits S-SSB in more than one RB set, how to allocate remaining power from PCMAX to non-anchor RB set(s) is unclear. |
| --- | --- |
| Summary of change: | Clarify that xxx (to be replaced by chosen Alt). |
| Consequences if not approved: | When UE transmits S-SSB in more than one RB set, how to allocate remaining power from PCMAX to non-anchor RB set(s) is unclear. |

[M] Proposal 6-2

Regarding the definition of SL RSSI, down-select one of followings:

- Option 1: SL RSSI is re-defined as the linear average of the total received power observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH except the 1st candidate starting OFDM symbol and the 2nd candidate starting OFDM symbol.

- Option 2: SL RSSI is re-defined as max {SL RSSI-1, SL RSSI-2}, where

  - SL RSSI-1 is defined as the linear average of the total received power observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH except the 1st candidate starting symbol and the 2nd candidate starting symbols.

  - SL RSSI-2 is defined as the linear average of the total received power observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH starting from the 2nd OFDM symbol after the 2nd candidate starting symbol.

- Option 3: The OFDM symbols used for SL RSSI measurement start from the next symbol of the 2nd candidate starting symbol.

- Option 4: no update on the definition of SL RSSI

[M] Proposal 5-2

TP#5-2 in Section 4.1.8 of R1-23xxxxx is endorsed for TS 38.213 clause 16.1.

[M] Proposal 1-1

TP#1-1 in Section 4.1.1 of R1-23xxxxx (to be FLS) is endorsed for TS 38.214 Clause 8.

FL’s note for above proposal:

- FL made following change based on Sharp’s comment to make it concise.

  - “…and if the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB' or ‘interlaceRB’,”

- @OPPO: FL feels the following cyan part should be clear enough, no need to add more.

  - “For operation with shared spectrum channel access for frequency range 1, a sidelink resource pool can be (pre-)configured to include integer number of RB sets, and the lowest RB of the sidelink resource pool is aligned with the lowest RB of a RB set in the SL BWP.”

==

[M] Proposal 2-1

TP#2-1 in Section 4.1.2 of R1-23xxxxx is endorsed for TS 38.214 Clause 8.1.2.1.

[M] Proposal 3-1

TP#3-1 in Section 4.1.3 of R1-23xxxxx is endorsed for TS 38.214 Clause 8.

FL’s note for above proposal:

- @Apple, OPPO, CATT: regarding your comment “if a candidate resource is composed of just 1 sub-channel, and this sub-channel has overlap with intra-cell guardband PRBs, then this sub-channel cannot be used for PSCCH/PSSCH transmission.” This is already specified in 38214 cyan part below.

“…If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB', the UE shall exclude candidate single-slot or candidate multi-slot resources with the sub-channel with the smallest index including resource blocks of the intra-cell guardband PRBs, configured by higher layer parameter, intraCellGuardBandsSL-List, or determined according to the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] when higher layer parameter, intraCellGuardBandsSL-List, is not configured.”

- @ZTE: FL feels your proposal is an optimization.

==

[M] Proposal 3-2

TP#3-2 in Section 4.1.4 of R1-23xxxxx is endorsed for TS 38.214 Clause 8.1.5.

FL’s note for above proposal:

- @LGE: please try to address Apple/QC/vivo’s concern.

==

[L] Proposal 4-7

Regarding whether/how to avoid COT interruption due to PSFCH occasion(s):

- Alt A: For “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”

  - COT initiating UE can transmit PSFCH only on common interlace on a PSFCH occasion within the COT, when

    - COT initiating UE does not intend to transmit PSFCH and does not expect to receive PSFCH on the PSFCH occasion within a SL-BWP, and

    - COT initiating UE performs PSCCH/PSSCH transmission just before the PSFCH occasion in the same slot, and

    - COT initiating UE intends to transmit PSCCH/PSSCH or S-SSB after the PSFCH occasion

  - COT responding UE can transmit PSFCH only on common interlace on a PSFCH occasion within the COT based on COT sharing, when

    - COT responding UE does not intend to transmit PSFCH and does not expect to receive PSFCH on the PSFCH occasion within a SL-BWP, and

    - COT responding UE performs PSCCH/PSSCH transmission just before the PSFCH occasion in the same slot, and

    - COT responding UE intends to transmit PSCCH/PSSCH that can utilize the shared COT or S-SSB after the PSFCH occasion

  - The cyclic shift is up to UE implementation

  - No new rule for CPE for this transmission

- Alt B: COT initiating UE indicates in the COT-SI to enable/disable (pre-)configured PSFCH occasion(s) within its COT

  - COT initiating UE or responding UE to transmit PSSCH on the disabled PSFCH occasion(s)

- Alt C: RAN1 does not pursue specific enhancements to avoid COT interruption due to PSFCH occasion(s) in R18 NR SL

[L] Proposal 6-3

Considering PSD limit in unlicensed spectrum regulation, down-select one of followings:

- Alt 1: RAN1 does not pursue specific enhancements on power control formulas of PSCCH/PSSCH/S-SSB/PSFCH to reflect PSD limit

- Alt 2: RAN1 updates power control of PSCCH/PSSCH/S-SSB/PSFCH by taking PSD limit into account

  - Down-select one of followings:

    - Alt 2-1: The Tx power for each PSFCH RB is not larger than Ppsfch,PSD = PPSD/Nmax_psfch_per_MHz, where PPSD is the PSD limit per MHz, and Nmax_psfch_per_MHz is (down-select one)

      - Alt A: the max number of PSFCH RBs in one MHz

      - Alt B: (pre-)configured (FFS value range) from {2,3,…,10}.

    - Alt 2-2: The UE drops some of the PSFCHs iteratively based on the existing PSFCH prioritization rule until the PSD limit is met.

    - Alt 2-3: If PPSFCH,one-10log10(2μ⋅MRBPSFCH)  is larger than PSD limitation, then PPSFCH,one is updated/reduced to (10log10(2μ⋅MRBPSFCH)+PSD limit) dBm.

FL’s note for above proposal:

- FL adds Alt 2-3 based on Apple’s comment.

- FL adds value range for Alt B based on vivo’s comment.

==

[L] Proposal 4-5

When OCB is required, down-select one of followings:

- Alt 1: R17 SL inter-UE coordination Scheme 1 (preferred/non-preferred resources) is not supported

- Alt 2: R17 SL inter-UE coordination Scheme 1 (preferred/non-preferred resources) is supported

  - SCI format 2C is updated to include RB set related information

[L] Proposal 5-3

Regarding “UE may transmit S-SSB repetition in more than one RB set”, down-select one of the followings:

- Option 1: use different NIDSL across the different S-SSB repetitions to determine the initial scrambling seed of PSBCH, and the sequence shift for S-SSS and S-PSS

  - The S-SSB indicated by sl-AbsoluteFrequencySSB-r16 uses NIDSL

  - The kth repetition uses “NIDSL + k”, where 1≤k≤N⋅MRBset-1

    - N is the number of repetitions in one RB set

    - MRBset is the number of RB sets for S-SSB transmission

- Option 2: phase adjustment among repetitions

  - Phase adjustment sequence is ZC sequence with root index k=0, 1, ⋯, N∙MRBset-1, where

    - k is the S-SSB repetition number index, k is 0 for S-SSB indicated by sl-AbsoluteFrequencySSB-r16, and k equals to 1 to N⋅MRBset-1 for the repetitions of the S-SSB

      - Note the S-SSB indicated by sl-AbsoluteFrequencySSB-r16 is not applied with phase adjustment with k=0

    - N is the number of repetitions in one RB set

  - MRBset is the number of RB sets for S-SSB transmission

- Option 3: it is up to Tx UE implementation whether/how to reduce PAPR

- Option 4: use S-SSB repetition index to scramble different S-SSB repetition(s)

  - Scrambling sequence is Gold sequence with cinit(k)=k⋅floor(220/(N⋅MRBset))⋅210+NIDSL, k=0, 1,…, N⋅MRBset-1, where

    - k is the S-SSB repetition number index, k is 0 for S-SSB indicated by sl-AbsoluteFrequencySSB-r16, and k equals to 1 to N⋅MRBset-1 for the repetitions of the S-SSB,

    - N is the number of repetitions in one RB set.

    - MRBset is the number of RB sets for S-SSB transmission

  - Note: PSBCH/S-PSS/S-SSS indicated by sl-AbsoluteFrequencySSB-r16 remains unchanged

- Collection of Text Proposals

  - TPs for Monday online

    - TP#1-1 for “Issue#1-1: TP, RP lowest RB aligns with RB set boundary”

| Reason for change: | The lowest RB may not be aligned with RB set boundary, resulting in unclear UE behaviour on whether/how to use sub-channel including such PRBs. |
| --- | --- |
| Summary of change: | Clarify that resource pool lower boundary is aligned with RB set boundary. |
| Consequences if not approved: | When the lowest RB is not aligned with RB set boundary, UE behaviour on whether/how to use sub-channel including such PRBs is unclear. |

---------------------------- Start of Text Proposal for TS 38.214-----------------------------

8 Physical sidelink shared channel related procedures

A UE can be configured by higher layers with one or more sidelink resource pools. A sidelink resource pool can be for transmission of PSSCH, as described in Clause 8.1, or for reception of PSSCH, as described in Clause 8.3 and can be associated with either sidelink resource allocation mode 1 or sidelink resource allocation mode 2.

In the frequency domain,

-	If the higher layer parameter transmissionStructureForPSCCHandPSSCH is not provided, or it is set to ‘contiguousRB', a sidelink resource pool consists of sl-NumSubchannel contiguous sub-channels. A sub-channel consists of sl-SubchannelSize contiguous PRBs, where sl-NumSubchannel and sl-SubchannelSize are higher layer parameters.

- If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘interlaceRB’, in the frequency domain, a sidelink resource pool consists of sl-NumSubchannel sub-channels, where each sub-channel consists of numInterlacePerSubchannel interlaces having contiguous interlace indices.

For operation with shared spectrum channel access for frequency range 1, a sidelink resource pool can be (pre-)configured to include integer number of RB sets, and the lowest RB of the sidelink resource pool is aligned with the lowest RB of a RB set in the SL BWP. A UE can be configured with intra-cell guard bands according to the higher layer parameter intraCellGuardBandsSL-List. The configured intra-cell guard band PRBs between any two adjacent RB sets can be used only for PSSCH transmission, if and only if, the UE has successfully performed channel access procedure in both adjacent RB sets, and the UE uses both of these RB sets for PSSCH transmission.

< Unchanged parts are omitted >

--------------------------------------- End of Text Proposal ----------------------------------

    - TP#2-1 for “Issue#2-1: TP, PSCCH/PSSCH transmission on slots with PSFCH symbols”

| Reason for change: | 1. The starting symbol for PSCCH/PSSCH transmission in a slot with PSFCH symbols is unclear. 2. It’s inaccurate to say that UE shall not use the second starting symbol in slots with PSFCH symbols since there is only one candidate starting symbols for PSCCH/PSSCH transmission in such slots. |
| --- | --- |
| Summary of change: | 1. Clarify the starting symbol for transmission in a slot with PSFCH symbols. 2. Delete the sentence “The UE shall not use the second starting symbol in slots with PSFCH symbols”. |
| Consequences if not approved: | 1. The starting symbol for transmission in a slot with PSFCH symbols is undefined. 2. It’s unclear where to transmit PSCCH/PSSCH in slots with PSFCH symbols. |

---------------------------- Start of Text Proposal for TS 38.214-----------------------------

8.1.2.1	Resource allocation in time domain

The UE shall transmit the PSSCH in the same slot as the associated PSCCH.

The minimum resource allocation unit in the time domain is a slot.

The UE shall transmit the PSSCH in consecutive symbols within the slot, subject to the following restrictions:

-	The UE shall not transmit PSSCH in symbols which are not configured for sidelink. A symbol is configured for sidelink, according to higher layer parameters sl-StartSymbol and sl-LengthSymbols, where sl-StartSymbol is the symbol index of the first symbol of sl-LengthSymbols consecutive symbols configured for sidelink.

-	Within the slot, PSSCH resource allocation starts at symbol sl-StartSymbol+1, except when startingSymbolFirst and startingSymbolSecond are provided for a SL-BWP. If startingSymbolFirst and startingSymbolSecond are provided for the SL-BWP, there are 2 candidate starting symbols, given by startingSymbolFirst and startingSymbolSecond respectively, for PSSCH transmission for slots without PSFCH symbols, and there is 1 candidate starting symbol, given by startingSymbolFirst, for PSSCH transmission for slots with PSFCH symbols. PSSCH resource allocation starts at the next symbol after each candidate starting symbol. In a slot, the UE may use the second candidate starting symbol, provided by startingSymbolSecond, only if it fails to access the channel prior to the first starting symbol provided by startingSymbolFirst.

< Unchanged parts are omitted >

--------------------------------------- End of Text Proposal ----------------------------------

    - TP#3-1 for “Issue#3-1: TP, Usage of edge sub-channel”

| Reason for change: | When the highest sub-channel of a candidate resource overlaps with a single RB set and intra-cell guardband PRBs, such sub-channel can be used for PSSCH transmission. This is not captured yet. |
| --- | --- |
| Summary of change: | Clarify that when the highest sub-channel of a candidate resource overlaps with a single RB set and intra-cell guardband PRBs, such sub-channel can be used for PSSCH transmission. |
| Consequences if not approved: | UE behaviour is ambiguous when the highest sub-channel of a candidate resource overlaps with a single RB set and intra-cell guardband PRBs. |

---------------------------- Start of Text Proposal for TS 38.214-----------------------------

8	Physical sidelink shared channel related procedures

< Unchanged parts are omitted >

For operation with shared spectrum channel access for frequency range 1, a sidelink resource pool can be (pre-)configured to include integer number of RB sets. A UE can be configured with intra-cell guard bands according to the higher layer parameter intraCellGuardBandsSL-List. The configured intra-cell guard band PRBs between any two adjacent RB sets can be used only for PSSCH transmission, if and only if, the UE has successfully performed channel access procedure in both adjacent RB sets, and the UE uses both of these RB sets for PSSCH transmission. When the highest sub-channel of PSSCH overlaps with a single RB set and intra-cell guard band PRBs, the UE can transmit PSSCH on the PRBs belonging to the allocated sub-channel(s) except for the intra-cell guard band PRBs within the highest sub-channel. The set of slots that may belong to a sidelink resource pool is denoted by (t0SL,t1SL,⋯,tTmax-1SL) where

< Unchanged parts are omitted >

--------------------------------------- End of Text Proposal ----------------------------------

    - TP#3-2 for “Issue#3-2: TP, RB set index for FRIV indication”

| Reason for change: | For FRIV indication, RB set indexing for PSSCH allocation is ambiguous. |
| --- | --- |
| Summary of change: | Clarify that for FRIV indication, within the resource pool, RB sets are numbered in increasing order from 0 to NRBset-1. |
| Consequences if not approved: | For FRIV indication, RB set indexing for PSSCH allocation is ambiguous. |

---------------------------- Start of Text Proposal for TS 38.214-----------------------------

8.1.5	UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 1-A

< Unchanged parts are omitted >

If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘interlaceRB’, the starting RB set nRBset,0start of the first resource is determined according to the clause [ABCDE]. The number of contiguously allocated RB sets for each of the N resources LRBset≥1 and the starting RB set indexes of resources indicated by the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received, are determined from "Frequency resource assignment" which is equal to a frequency RIV (FRIV) where.

If sl-MaxNumPerReserve is 2 then

FRIVRBset=nRBset,1start+i=1LRBset-1NRBset+1-i

If sl-MaxNumPerReserve is 3 then

FRIVRBset=nRBset,1start+nRBset,2start⋅NRBset+1-LRBset+i=1LRBset-1NRBset+1-i2

where

-	nRBset,1start denotes the starting RB set index for the second resource

-	nRBset,2start denotes the starting RB set index for the third resource

-	NRBset is the number of RB sets in a resource pool

-	LRBset is the number of RB sets for each of the indicated resources

-	For FRIV indication, within the resource pool, RB sets are numbered in increasing order from 0 to NRBset-1

If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘interlaceRB’, the resource is determined by an intersection of the interlaces corresponding to the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any.

< Unchanged parts are omitted >

--------------------------------------- End of Text Proposal ----------------------------------

    - TP#4-3-1 for “Issue#4-3: PSFCH power control and prioritization”

| Reason for change: | The number PRBs for PSFCH transmission(s) are not considered in PSFCH power control and the determination on the number of simultaneous PSFCH transmissions. Even though the number of dedicated PRBs increases, the total power for a PSFCH transmission may not increase. |
| --- | --- |
| Summary of change: | 1. For one PSFCH transmission power, the number of dedicated PRBs is taken into account. 2. Specify how to allocate power on PRBs of a common interlace and how to determine P_CMAX considering the PRBs of a common interlace further. |
| Consequences if not approved: | Coverage of a PSFCH is not extended regardless of the number of dedicated PRBs for a PSFCH transmission. |

---------------------------- Start of Text Proposal for TS 38.213-----------------------------

16.2.3	PSFCH

A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions for HARQ-ACK information and conflict information, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, with HARQ-ACK information or conflict information on all the resource pools in PSFCH transmission occasion i on active SL BWP b of carrier f as

-	if dl-P0-PSFCH is provided,

PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm]

where

-	PO,PSFCH is a value of dl-P0-PSFCH-r17, if using the parameter is supported by the UE and the parameter is provided;else dl-P0-PSFCH-r16 if provided

-	αPSFCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1

-	PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that

-	the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c

-	the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c

-	if Nsch,Tx,PSFCH≤Nmax,PSFCH

-	if PPSFCH,one+10log10Nsch,Tx,PSFCHk=1Nsch,Tx,PSFCHMRB,kPSFCHi≤PCMAX10log1010PCMAX10-10PPSFCH,first10∙MRB,firsstPSFCH, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

-	NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one +10log10MRB,kPSFCHi [dBm]

-	else

-	UE autonomously determines NTx,PSFCH PSFCH transmissions first with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, for 1≤i≤ 8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, for i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as

-	the largest value satisfying PPSFCH,one+10log10max1,i=1KMik=1MiMRB,kPSFCHi≤PCMAX10log1010PCMAX10-10PPSFCH,first10∙MRB,firsstPSFCH where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any

-	zero, otherwise

and

PPSFCH,k(i)=minPCMAX10log1010PCMAX10-10PPSFCH,first10∙MRB,firsstPSFCH-10log10NTx,PSFCHk=1NTx,PSFCHMRB,kPSFCHi,PPSFCH,one+10log10MRB,kPSFCHi [dBm]

where PCMAX	is defined in [8-1, TS 38.101-1] and is determined for the NTx,PSFCH PSFCH transmissions

-	else

-	the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2

-	if PPSFCH,one+10log10Nmax,PSFCHk=1Nmax,PSFCHMRB,kPSFCHi≤PCMAX10log1010PCMAX10-10PPSFCH,first10∙MRB,firsstPSFCH, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

-	NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one+10log10MRB,kPSFCHi [dBm]

-	else

-	the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, 1≤i≤8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as

-	the largest value satisfying PPSFCH,one+10log10max1,i=1KMik=1MiMRB,kPSFCHi≤PCMAX10log1010PCMAX10-10PPSFCH,first10∙MRB,firsstPSFCH where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any

-	zero, otherwise

and

PPSFCH,k(i)=min10log1010PCMAX10-10PPSFCH,first10∙MRB,firsstPSFCHPCMAX-10log10NTx,PSFCHk=1NTx,PSFCHMRB,kPSFCHi,PPSFCH,one+10log10MRB,kPSFCHi [dBm]

where PCMAX is determined for the  NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1]

-	else

PPSFCH,ki=PCMAX10log1010PCMAX10-10PPSFCH,offset10∙MRB,firsstPSFCH-10log10NTx,PSFCHk=1NTx,PSFCHMRB,kPSFCHi+MRB,firsstPSFCH[dBm]

where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥1 and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1].

where

-	PPSFCH,first=PPSFCH,one-PPSFCH,offset, where PPSFCH,offset is provided by sl-PSFCH-Type2-PowerOffset, if sl-PSFCH-Type = ‘type2’; else PPSFCH,first=0.

-	MRB,firsstPSFCH is the number of resource blocks in the first interlace with the index of sl-PSFCH-Type2-CommonInterlace, if sl-PSFCH-Type = ‘type2’; else MPSFCH,first =0.

-	MRB,kPSFCHi is the number of resource blocks of a PSFCH transmission k in PSFCH transmission occasion i.

For resource pools configured with PSFCH resources overlapping in time, the UE either expects not to be provided with dl-P0-PSFCH or dl-Alpha-PSFCH in any of the resource pools, or expects to be provided with the same values of dl-P0-PSFCH and the same values of dl-Alpha-PSFCH for all the resource pools.

For operation with shared spectrum channel access,

-	if sl-PSFCH-Type = ‘type1’, a UE equally allocates power to all PRBs of an interlace used for a PSFCH transmission

-	if sl-PSFCH-Type = ‘type2’, a UE allocates power to PRBs of a PSFCH transmission such that

-	a power for each PRB in a subset of PRBs of a first interlace is PPSFCH,first,

-	a power for a subset of PRBs of a second interlace isPPSFCH,secondPPSFCH,ki.

PPSFCH,first=PPSFCH,second-PPSFCH,offsetPPSFCH,offset

< Unchanged parts are omitted >

--------------------------------------- End of Text Proposal ----------------------------------

    - TP#4-3-2 for “Issue#4-3: PSFCH power control and prioritization”

| Reason for change: | The order of performing PSFCH prioritization and dropping PRB in common interlace is unclear. UE should drop the PRB of common interlace for each PSFCH transmission after performing PSFCH prioritization. Otherwise, the OCB requirement may not be fulfilled. |
| --- | --- |
| Summary of change: | Clarify that UE drops PRB of common interlace for each PSFCH transmission after performing PSFCH prioritization |
| Consequences if not approved: | The order of performing PSFCH prioritization and dropping PRB in common interlace is unclear. |

---------------------------- Start of Text Proposal for TS 38.213-----------------------------

16.3.0	UE procedure for transmitting PSFCH with control information

< Unchanged parts are omitted >

For operation with shared spectrum channel access, when sl-PSFCH-Type = ‘type2’, a PRB s1 in the first interlace is excluded from the resources for a PSFCH transmission, if |s1-s2|≤5 for μ=0 or |s1-s2|≤2 for μ=1 for any PRB s2 in the PRB subset when the PRB subset is selected for PSFCH transmission, and shigh-slow≥88 for μ=0 or shigh-slow≥44 for μ=1, where PRB shigh and PRB slow are the largest and smallest PRB indexes, respectively, in the resources for the PSFCH transmission assuming PRB s1 is excluded after performing PSFCH prioritization.

For a PSFCH transmission with HARQ-ACK information or conflict information, a UE determines a m0 value, for computing a value of cyclic shift α [4, TS 38.211], from a cyclic shift pair index corresponding to a PSFCH resource index and from NCSPSFCH using Table 16.3-1.

< Unchanged parts are omitted >

--------------------------------------- End of Text Proposal ----------------------------------

    - TP#4-3-3 for “Issue#4-3: PSFCH power control and prioritization”

| Reason for change: | In “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”, assume the UE transmits multiple PSFCHs, the power on common PRBs will be accumulated based on latest specification. Thus, the final Tx power on one common PRB (P_common) might be larger than the final Tx power on one dedicated PRB (P_dedicated), i.e., P_common > P_dedicated, which violates previous Working Assumption. |
| --- | --- |
| Summary of change: | Clarify that when the UE transmits multiple PSFCHs, the power on common PRBs are not accumulated. |
| Consequences if not approved: | The final Tx power on one common PRB (P_common) might be larger than the final Tx power on one dedicated PRB (P_dedicated), i.e., P_common > P_dedicated, which violates previous Working Assumption. |

---------------------------- Start of Text Proposal for TS 38.213-----------------------------

16.2.3	PSFCH

A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions for HARQ-ACK information and conflict information, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on all the resource pools in PSFCH transmission occasion i on active SL BWP b of carrier f as

< Unchanged parts are omitted >

For resource pools configured with PSFCH resources overlapping in time, the UE either expects not to be provided with dl-P0-PSFCH or dl-Alpha-PSFCH in any of the resource pools, or expects to be provided with the same values of dl-P0-PSFCH and the same values of dl-Alpha-PSFCH for all the resource pools.

For operation with shared spectrum channel access,

-	if sl-PSFCH-Type = ‘type1’, a UE equally allocates power to all PRBs of an interlace used for a PSFCH transmission

-	if sl-PSFCH-Type = ‘type2’, a UE allocates power of NTx,PSFCH PSFCH transmissions to PRBs such that

-	a power for each PRB in a subset of RRBs of a first interlace is PPSFCH,first,

-	a power for each PRB in a subset of RRBs of a second interlace is PPSFCH,second, and

-	PPSFCH,first=PPSFCH,second-PPSFCH,offset, where PPSFCH,offset is provided by sl-PSFCH-Type2-PowerOffset.

< Unchanged parts are omitted >

--------------------------------------- End of Text Proposal ----------------------------------

    - TP#5-2 for “Issue#5-2: TP, Transmit S-SSB on anchor RB set on additional S-SSB occasions”

| Reason for change: | As per previous conclusion and agreements, when UE determines to transmit S-SSB on additional S-SSB occasions, UE at least attempts to transmit on anchor RB set. However, this is not captured in specification yet. |
| --- | --- |
| Summary of change: | Clarify that when UE determines to transmit S-SSB on additional S-SSB occasions, UE at least attempts to transmit on anchor RB set |
| Consequences if not approved: | When UE determines to transmit S-SSB on additional S-SSB occasions, UE may not transmit on anchor RB set, which violates RAN1 agreement. |

---------------------------- Start of Text Proposal for TS 38.213-----------------------------

16.1	Synchronization procedures

A UE receives the following SL synchronization signals in order to perform synchronization procedures based on S-SS/PSBCH blocks: SL primary synchronization signals (S-PSS) and SL secondary synchronization signals (S-SSS) [4, TS 38.211].

A UE assumes that reception occasions of a physical sidelink broadcast channel (PSBCH), S-PSS, and S-SSS are in consecutive symbols [4, TS 38.211] and form a S-SS/PSBCH block.

For reception of a S-SS/PSBCH block,

-	for operation without shared spectrum channel access, or for operation with shared spectrum channel access and when sl-NumberRepeatedSSB is not provided and for RB-set j, a UE assumes a frequency location corresponding to the subcarrier with index 66 in the S-SS/PSBCH block [4, TS 38.211], is provided by

-	sl-AbsoluteFrequencySSB, for operation without shared spectrum channel access or when RB-set j is the anchor RB-set that is the RB set that includes the S-SS/PSBCH block

-	a corresponding value in sl-AbsoluteFrequencySSB-NonAnchorList when RB-set j is a non-anchor RB-set

-	for operation with shared spectrum channel access when sl-NumberRepeatedSSB is provided and in RB-set j, a UE assumes a frequency location corresponding to the subcarrier with index 66 in the S-SS/PSBCH block [4, TS 38.211] is provided by fstart,jS-SSB+ kS-SSB,j⋅Ngap,jS-SSB+MRBS-SSB⋅12⋅2μ∙15 kHz, where

-	fstart, jS-SSB is a frequency location of a lowest S-SS/PSBCH block in RB-set j, where fstart, jS-SSB is provided by

-	sl-AbsoluteFrequencySSB when RB-set j is the anchor RB-set,

-	a corresponding value in sl-AbsoluteFrequencySSB-NonAnchorList when RB-set j is a non-anchor RB-set

-	kS-SSB,j is an index of an S-SS/PSBCH block from repeated S-SS/PSBCH blocks in the frequency domain and within the RB-set j, where 0≤kS-SSB,j≤Nrepetition,jS-SSB-1, and Nrepetition,jS-SSB is provided by a value in sl-NumberRepeatedSSB corresponding to RB-set j;

-	Ngap,jS-SSB is a number of resource blocks, provided by sl-GapRepeatedSSB, for a gap between two adjacent repeated S-SS/PSBCH blocks;

-	MRBS-SSB=11 is a number of resource blocks for a S-SS/PSBCH block transmission with SCS configuration μ.

For operation with shared spectrum channel access, a UE attempts to transmit at least S-SS/PSBCH blocks in the slots including S-SS/PSBCH blocks in the anchor RB set. The UE applies CP extension to the first symbol of an S-SS/PSBCH block and within the first one or two symbols before the first symbol of the S-SS/PSBCH block according to an index [4, TS 38.211] provided by sl-CP-Extension-SSB. The UE assumes PRB(s) in an intra-cell guard band [6, TS 38.214] are not used for transmission of S-SS/PSBCH blocks.

The UE assumes that a S-PSS symbol, a S-SSS symbol, and a PSBCH symbol have a same transmission power. The UE assumes a same numerology of the S-SS/PSBCH as for a SL BWP of the S-SS/PSBCH block reception, and that a bandwidth of the S-SS/PSBCH is within a bandwidth of the SL BWP. The UE assumes the subcarrier with index 0 in the S-SS/PSBCH block is aligned with a subcarrier with index 0 in an RB of the SL BWP.

A UE is provided, by sl-NumSSB-WithinPeriod, a number NperiodS-SSB of S-SS/PSBCH blocks in a period of 16 frames. The UE assumes that a transmission of the S-SS/PSBCH blocks in the period is with a periodicity of 16 frames. The UE determines indexes of slots that include S-SS/PSBCH block as NoffsetS-SSB+NintervalS-SSB+1⋅iS-SSB, where

-	index 0 corresponds to a first slot in a frame with SFN of the serving cell satisfying (SFN mod 16)=0 or DFN satisfying (DFN mod 16) = 0

-	iS-SSB is a S-SS/PSBCH block index within the number of S-SS/PSBCH blocks in the period, with 0≤iS-SSB≤NperiodS-SSB-1

-	NoffsetS-SSB is a slot offset from a start of the period to the first slot including S-SS/PSBCH block, provided by sl-TimeOffsetSSB

-	NintervalS-SSB is a slot interval between S-SS/PSBCH blocks, provided by sl-TimeInterval

For operation with shared spectrum channel access and for each slot that includes S-SS/PSBCH blocks, a UE is provided, by sl-NumAdditionalOccasionPerSSB, a number NadditionalS-SSB of additional candidate S-SS/PSBCH block transmission occasions. When a UE determines to transmit S-SS/PSBCH blocks on additional candidate S-SS/PSBCH block transmission occasions, the UE attempts to transmit at least S-SS/PSBCH blocks in the anchor RB set. When NadditionalS-SSB>0, for S-SS/PSBCH block with index iS-SSB, the UE determines indexes of slots that include the additional candidate S-SS/PSBCH block transmission occasions as NoffsetS-SSB+NintervalS-SSB+1⋅iS-SSB + NgapS-SSB⋅(iS-SSB+1), where

-	NgapS-SSB is a slot gap, provided by sl-TimeGapAdditionalOccasion, for determining the additional candidate S-SS/PSBCH block transmission occasions, and

-	iS-SSB is an index of the additional candidate S-SS/PSBCH block transmission occasions, with 0≤iS-SSB≤NadditionalS-SSB-1.

For paired spectrum, an S-SS/PSBCH block can be transmitted/received only in a slot of an UL carrier. For unpaired spectrum, an S-SS/PSBCH block can be transmitted/received only in a slot of which all OFDM symbols are configured as UL by tdd-UL-DL-ConfigurationCommon of the serving cell if provided or sl-TDD-Configuration if provided or sl-TDD-Config of the received PSBCH if provided. If tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration are not provided for a spectrum indicated with only PC5 interface in Table 5.2E.1-1 in [TS 38.101-1], an S-SS/PSBCH block can be transmitted/received in any slot of the spectrum.

For transmission of an S-SS/PSBCH block, a UE includes a bit sequence a0, a1, a2, a3, …, a11 in the PSBCH payload to indicate sl-TDD-Config and provide a slot format over a number of slots.

< Unchanged parts are omitted >

--------------------------------------- End of Text Proposal ----------------------------------

    - TP#4-3-4 for “Issue#4-3: PSFCH power control and prioritization”

| Reason for change: | xxx |
| --- | --- |
| Summary of change: | xxx |
| Consequences if not approved: | xxx |

---------------------------- Start of Text Proposal for TS 38.213-----------------------------

16.2.3	PSFCH

A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions for HARQ-ACK information and conflict information, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on all the resource pools in PSFCH transmission occasion i on active SL BWP b of carrier f as

-	if dl-P0-PSFCH is provided,

PPSFCH,one=PO,PSFCH+10log102μ⋅MRBPSFCH+αPSFCH⋅PL [dBm]

where

-	PO,PSFCH is a value of dl-P0-PSFCH-r17, if using the parameter is supported by the UE and the parameter is provided;else dl-P0-PSFCH-r16 if provided

-	αPSFCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1

-	PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that

-	the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c

-	the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c

-	MRBPSFCH is equal to 1, if sl-PSFCH-Type is not provided; or is equal to a value of numRefPRBOfInterlace, if sl-PSFCH-Type = ‘type1’; or is equal to a value of numDedicatedPRBsForPSFCH, if sl-PSFCH-Type = ‘type2’

If sl-PSFCH-Type = ‘type2’,

PPSFCH, common, temp=PO,PSFCH+10log102μ∙MRB, common, tempPSFCH⋅R+αPSFCH⋅PL-β[dBm]

where

-	MRB, common, tempPSFCH is the number of PRBs of the interlace with index sl-PSFCH-Type2-CommonInterlace

-	β is provided by sl-PSFCH-Type2-PowerOffset

-	R is the number of RB sets for the transmissions on a first interlace

Otherwise, PPSFCH, common, temp=0.

-	if Nsch,Tx,PSFCH≤Nmax,PSFCH

-	if PPSFCH,one+10log10Nsch,Tx,PSFCH≤PCMAX-PPSFCH, common, temp, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

-	NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm]

-	else

-	UE autonomously determines NTx,PSFCH PSFCH transmissions first with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, for 1≤i≤ 8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, for i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as

-	the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX-PPSFCH, common, temp where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any

-	zero, otherwise

and

PPSFCH,k(i)=minPCMAX-10log10NTx,PSFCH-PPSFCH, common, temp,PPSFCH,one [dBm]

where PCMAX	is defined in [8-1, TS 38.101-1] and is determined for the NTx,PSFCH PSFCH transmissions

-	else

-	the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2

-	if PPSFCH,one+10log10Nmax,PSFCH≤PCMAX-PPSFCH, common, temp, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

-	NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm]

-	else

-	the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, 1≤i≤8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as

-	the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX-PPSFCH, common, temp where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any

-	zero, otherwise

and

PPSFCH,k(i)=minPCMAX-10log10NTx,PSFCH-PPSFCH, common, temp,PPSFCH,one [dBm]

where PCMAX is determined for the  NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1]

-	else

PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH)-PPSFCH, common, temp [dBm]

where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥1 and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1].

If sl-PSFCH-Type = ‘type2’, the power for a PSFCH transmission k on a second interlace is PPSFCH,k(i), and the power for a PSFCH transmission k on a first interlace is

Pcommon,ki=PPSFCH,k(i)-10⋅log10(2μ⋅MRBPSFCH/MRB, commonPSFCH)-β,

where MRB, commonPSFCH is the number of PRBs of a first interlace to be transmitted.

< Unchanged parts are omitted >

--------------------------------------- End of Text Proposal ----------------------------------

- Conclusions

TBD

- References

- R1-2310812	Remaining issues for Physical Channel Design Framework for SL-U	Nokia, Nokia Shanghai Bell

- R1-2310852	Maintenance of physical channel design for sidelink operation over unlicensed spectrum	Huawei, HiSilicon

- R1-2310969	Remaining issues on physical channel design of sidelink on unlicensed spectrum	Quectel

- R1-2311091	Remaining issues on physical channel design framework for sidelink on unlicensed spectrum	vivo

- R1-2311162	Remaining issues on Physical channel design for sidelink on unlicensed spectrum	Spreadtrum Communications

- R1-2311237	On maintenance of SL-U PHY channel designs and procedures	OPPO

- R1-2311337	Maintenance on physical channel design framework for sidelink on unlicensed spectrum	CATT, CICTCI

- R1-2311401	Remaining details on SL-U physical channel structures and procedures	xiaomi

- R1-2311425	Remaining issues on physical channel design framework	NEC

- R1-2311516	Discussion on physical layer structures and procedures for SL-U	ZTE, Sanechips

- R1-2311565	Physical channel design for sidelink on unlicensed spectrum	Panasonic

- R1-2311589	Remaining issues on physical layer design framework for sidelink on unlicensed spectrum	Lenovo

- R1-2311620	Remaining issues on channel design framework in SL-U	NTT DOCOMO, INC.

- R1-2311680	On Remaining Issues of Sidelink Physical Channel Design Framework for Unlicensed Spectrum	Apple

- R1-2311752	Remaining issues on physical channel design framework for SL-U	ETRI

- R1-2311767	Remaining issues on physical channel design framework for NR sidelink evolution on unlicensed spectrum	Sharp

- R1-2311796	Remaining issues of physical channel design for sidelink in unlicensed spectrum	Transsion Holdings

- R1-2311838	Remaining issues on physical channel design framework	Samsung

- R1-2311885	Remaining issues for SL U physical layer design	InterDigital, Inc.

- R1-2311976	Discussion on remaining issue for physical channel design framework	MediaTek Inc.

- R1-2312032	Physical Channel Design for Sidelink on Unlicensed Spectrum	Qualcomm Incorporated

- R1-2312122	Remaining issues on physical channel design framework for sidelink on unlicensed spectrum	LG Electronics

- R1-2312135	Discussion on channel design for SL-U	ASUSTeK

- R1-2312172	Remaining issues on PHY channel design for SL-U	WILUS Inc.

- Annex A: Contact Information

Please provide/update your contact information here so that delegates can better communicate with each other if needed.

| Company | Name | Email |
| --- | --- | --- |
| Huawei | Xiang Mi | Shawn.mixiang@huawei.com |
| HiSilicon | Fan Yang | james.yangfan@huawei.com |
| LGE | Daesung Hwang Seungmin Lee | daesung.hwang@lge.com edison.lee@lge.com |
| CATT, GOHIGH | Rui Zhao Shupeng Li Xiaoran Wen | zhaorui@goghigh.com.cn lsp@catt.cn wenxiaoran@gohigh.com.cn |
| OPPO | Zhenshan ZHAO Kevin Lin | zhaozhenshan@oppo.com kevin.lin@oppo.com |
| vivo | Zichao Ji | jizichao@vivo.com |
| ZTE,Sanechips | Yuzhou Hu | hu.yuzhou@zte.com.cn |
| Spreadtrum | Mimi Chen | mimi.chen@unisoc.com |
| Intel | Salvatore Talarico | salvatore.talarico@intel.com |
| CMCC | Pengyu Ji Jingwen Zhang | jipengyu@chinamobile.com zhangjingwen@chinamobile.com |
| Panasonic | Ayako Iwata | iwata.ayako@jp.panasonic.com |
| xiaomi | Wensu Zhao Qun Zhao | zhaowensu@xiaomi.com zhaoqun1@xiaomi.com |
| NTT DOCOMO | Shohei Yoshioka | shohei.yoshioka@docomo-lab.com syouhei.yoshioka.py@nttdocomo.com |
| Sharp | Liqing Liu | liu.liqing@sharp.co.jp |
| Nokia/NSB | Naizheng Zheng | naizheng.zheng@nokia-sbell.com |
| Ericsson | Ricardo Blasco Jose Leon Calvo | name.surname at company . com jose.leon.calvo@ericsson.com |
| Transsion | Xingya Shen | xingya.shen@transsion.com |
| InterDigital | Aata El Hamss | Aata.elhamss@interdigital.com |
| Lenovo | Xiaodong Yu Karthikeyan Ganesan Alexander Golitschek | Yuxd1@lenovo.com kganesan@lenovo.com aelbwart@lenovo.com |
| Samsung | Hongbo Si | hongbo.si@samsung.com |
| Futurewei | George Calcev | gcalcev@futurewei.com |
| Qualcomm | Giovanni Chisci Chih-Hao Liu Stelios Stefanatos | gchisci@qti.qualcomm.com chihliul@qti.qualcomm.com sstefana@qti.qualcomm.com |
| NEC | Zhaobang MIAO Jin YANG | miao_zhaobang@nec.cn yangjin@labs.nec.cn |
| WILUS | Minseok Noh | minseok.noh@wilusgroup.com |
| MediaTek | Tao Chen | tao.chen@mediatek.com |
| Fraunhofer | Tom Wirth | thomas.wirth@hhi.fraunhofer.de |
| China Telecom | Jing Guo | guojing6@chinatelecom.cn |
| Sony | Kazuyuki Shimezawa | kazuyuki.shimezawa@sony.com |
| Johns Hopkins University APL | Vijitha Weerackody | vijitha.weerackody@jhuapl.edu |
| National Spectrum Consortium | Amitav Mukherjee | Amitav.Mukherjee@nationalspectrumconsortium.org |
| ETRI | Junhyeong Kim | jhkim41jf@etri.re.kr |

- Annex B: Outcomes of RAN1 meetings

- RAN1#109-e (May 9 – 20, 2022)

Agreement

SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline

- Only one SL BWP is (pre-)configured within a carrier

- The SL BWP is (pre-)configured to include one or multiple SL resource pools

- At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets

  - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set

  - FFS: the applicable resource pool

  - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported

- PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets

  - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc.

- FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool

- FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc.

- FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc.

Agreement

For PSCCH and PSSCH in SL-U:

- Both R16/R17 NR SL contiguous RB-based and R16 NR-U interlace RB-based transmissions are considered as starting point

  - RAN1 strives to have unified design for both contiguous RB-based and interlace RB-based transmissions

  - FFS: whether/how to address IBE (In Band Emission) impact

Agreement

For PSCCH and PSSCH in SL-U:

- For interlace RB-based transmission (if supported), at least the following candidates can be discussed:

  - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission

    - FFS: Other resource allocation granularity, e.g., RB-level

  - 1 sub-channel equals K interlaces if sub-channel is supported

    - FFS details

  - Other candidates are not precluded

  - FFS: mapping of PSCCH to frequency resources

  - FFS: resource indication in time/frequency domain, e.g., how to handle using one RB set or multiple RB sets, etc.

Agreement

For slot structure in SL-U:

- At least R16/R17 NR SL slot-based PSCCH/PSSCH transmission is supported

  - FFS: whether/how to support additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission

Agreement

For PSFCH and SL-HARQ in SL-U:

- At least R16 NR SL PSFCH format 0 is supported

  - FFS whether to introduce new PSFCH format

- FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc.

- FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc.

- FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc.

- FFS: whether/how to address PSFCH and related PSSCH in different COTs

Agreement

For S-SSB and synchronization in SL-U:

- FFS the time domain locations of S-SSB resources, e.g., whether/how to introduce more candidate occasions compared with R16/R17 NR SL design, etc.

- Down-selection at least one of the following solutions to meet OCB and PSD requirement for S-SSB transmission

  - Option 1: Using interlaced RB transmission

  - Option 2: S-SSB multiplexing with other SL transmissions in the same slot

  - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain

  - Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth

- FFS: whether to support 4 symbols S-SSB

  - Note: 4 symbols S-SSB can be considered with options 1/2/3/4 above

- FFS whether the temporary exemption of OCB requirement is applicable for S-SSB transmission

- FFS whether any changes to R16/R17 NR SL synchronization procedure

- RAN1#110 (August 22 – 26, 2022)

Agreement

For PSCCH and PSSCH in SL-U:

- Both R16/R17 NR SL contiguous RB-based and interlace RB-based transmissions similar to R16 NR-U are supported

Agreement

For PSCCH and PSSCH in SL-U:

- For interlace RB-based transmission

  - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission

  - 1 sub-channel equals K interlace

    - FFS: whether K is fixed as 1 or (pre-)configured

  - Discuss whether one or both of the following alternatives are supported

    - Alt 1: 1 sub-channel is confined within 1 RB set

    - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool

Agreement

To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details.

Agreement

If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions

- FFS the number and locations of additional candidate S-SSB occasions

- FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior

Agreement

Regarding PSFCH transmission, at least the followings alternatives can be further studied

- Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s)

- Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts)

- Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs

- FFS details of above alternatives

Agreement

If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied

- Alt 1: PSFCH resources are (pre-)configured

- Alt 2: PSFCH resources are dynamically indicated

- Combination of above alternatives are not precluded

- FFS details of above alternatives

Agreement

For S-SSB and synchronization in SL-U:

- No changes on R16 NR SL S-PSS/S-SSS sequence generation

- Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS

Agreement

For PSCCH and PSSCH resource indication in time/frequency domain:

- For time domain: R16 NR SL TRIV is reused as baseline

- For frequency domain:

  - further study sub-channel indexing and resource indication

- FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

- RAN1#110b-e (October 10 – 19, 2022)

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding 1 sub-channel equals K interlace(s)

  - At least K=1 and K=2 is supported for 15 kHz SCS

  - At least K=1 is supported for 30 kHz SCS

  - FFS: details related to multiple RB sets

Working assumption:

Support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission.

- RAN1 strives to have unified design for PSCCH/PSSCH transmission from 1st or 2nd starting symbol

- The candidate starting symbol(s) are intended for AGC purpose

  - FFS: other potential uses of the candidate starting symbol(s)

- FFS other details, e.g., applicable scenarios (including SCS), position of 2nd starting symbol, TBS determination, PSCCH blind decoding complexity, processing time constraints, etc.

- FFS whether 2 candidate starting symbols is also supported for slots with PSFCH

Agreement

To meet OCB and PSD requirement for S-SSB transmission, down-select between the followings for 15 kHz and 30 kHz SCS:

- Option 1: Using interlaced RB transmission for S-PSS/S-SSS/PSBCH

- Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain

- FFS: whether/how the above options apply to all or subset of channel type of S-PSS/S-SSS/PSBCH

- Note: RAN1 further study the relationship between above options and temporary OCB exemption, and the discussion on temporary OCB exemption can continue even if option 1 or option 3 is supported

FFS: how to handle 60 kHz SCS (if needed, not limited to option 1 or option 3)

Agreement

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

- When more than one RB set is used for transmissions, down-select one of the followings

  - Option A: Support that the used interlace index(s) in different RB sets are always the same

  - Option B: Support that the used interlace index(s) in different RB sets can be different

- FFS details

Agreement

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

- Down-select one of the followings

  - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

  - Option 2: Support explicitly indicating at least the used sub-channel index(s)

    - At least RB set index(s) is not explicitly indicated

- FFS details

Agreement

For PSCCH and PSSCH in SL-U:

- PSCCH is transmitted within 1 sub-channel

- At least support Option 1 below

  - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH

    - Note: the lowest sub-channel may not be entirely contained in the lowest RB set

- FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below

  - Option 2: PSCCH locates in every RB set of corresponding PSSCH

- Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs.

- FFS other details

Agreement

Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets:

- Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission

  - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets

- Such PRBs are not used for PSCCH transmission

  - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission

Agreement

At least R16/R17 NR SL S-SSB slots are excluded from SL resource pool.

- Note: whether or not additional candidate S-SSB occasions are excluded from resource pool will be discussed after the details of additional candidate S-SSB occasions are clearer

Agreement

At least there is 1 PSFCH occasion per PSCCH/PSSCH transmission, FFS details

Agreement

To address PSFCH transmission dropping due to LBT failure, the followings are to be studied:

- Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

- Alt 2: PSFCH resources are dynamically indicated

- Alt 3: Convey SL-HARQ feedback information in PSCCH/PSSCH, e.g., new SCI or new MAC-CE

- Alt 4: drop PSFCH transmission

- Alt 5: Support trigger based HARQ feedback reporting for non-numerical HARQ FB and one shot HARQ FB

- Combination of above alternatives are not precluded

- FFS details of above alternatives

Agreement

Regarding additional candidate S-SSB occasions:

- Their number and time domain locations are (pre-)configured or pre-defined

- RAN1#111 (November 14 – 18, 2022)

Agreement

For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- Regarding the location of 1st starting symbol, down-select one of the followings:

  - Option 1: it is fixed as symbol#0

  - Option 2: it is indicated by sl-StartSymbol as in R16 NR SL

- Regarding the location of 2nd starting symbol, down-select one of the followings:

  - Option A: it is a fixed location

    - FFS the location, e.g., symbol#4, #7, etc.

  - Option B: it is a (pre-)configured location per resource pool

    - FFS the details of candidate locations

- Note: assume symbol index in a slot starts from #0

Agreement

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- TBS is determined based on a reference symbol length, down-select one of the followings:

  - Option 1: The reference symbol length is dynamically indicated by Tx UE

  - Option 2: The reference symbol length is determined based on 1st starting symbol

  - Option 3: The reference symbol length is determined based on 2nd starting symbol

  - Option 4: The reference symbol length is (pre-)configured

Agreement

Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, RAN1 continues studying the following updated alternatives:

- Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - FFS: value of K3

- Alt 2-1a: each PSFCH transmission occupies 1 interlace, and further apply frequency-domain OCC

  - FFS: details of FD-OCC, e.g., OCC length, RB-level, RE-level, etc.

- Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace

  - FFS: value of K1

- Alt 2-3a: each PSFCH transmission occupies 1 interlace

- Alt 2-4a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE uses different cyclic shifts on different PRBs in the interlace

- Alt 3-1a: each PSFCH transmission occupies 1 dedicated PRB and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

  - The above dedicated PRB and common PRBs are within 1 interlace

  - FFS: value of K2

- Alt 3-2a: each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of a RB set

- FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth

- FFS: whether IBE issue exists and whether/how to address it

- Note: in the above descriptions

  - The dedicated PRB/cyclic shift conveys ACK/NACK information

- Note: as previously agreed: to meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS.

Agreement

Slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH.

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and interlace, RAN1 further study the followings:

  - Option 1: 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

  - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool

  - Option 3: 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool

  - Option 4: 1 sub-channel is defined within 1 RB set or 2 adjacent RB sets, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool

  - Option 5: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly across different RB sets within the resource pool, then across different interlaces in the RB set

  - FFS: whether/how to use intra-cell guardband PRBs

Agreement

For S-SSB transmission, down-select one or more of the following for 15 kHz and 30 kHz SCS:

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

- Option 1-2: Using interlaced RB transmission for PSBCH only, and apply OCB exemption to S-PSS and S-SSS

- Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2)

  - FFS gap of 0

- Option 3-2: Repeat only S-PSS/S-SSS K times in frequency domain, and PSBCH is rate matched. There is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of K

  - FFS gap of 0

  - FFS PSBCH resource

- Option 3-3: keep the legacy S-PSS/S-SSS/PSBCH while repeating PSBCH N times in frequency domain and rate-matching PSBCH to S-PSS/S-SSS symbols, and there is a gap between the PSBCH repetition(s) to meet OCB requirements

  - FFS details, e.g. the length of gap is (pre-)configured or pre-defined, value of N

- Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH

- For Option 1-1 and 1-2 above

  - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set

- FFS: whether transient period issue exists and whether/how to address it

Agreement

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- Regarding Tx UE behaviour:

  - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings

    - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose

    - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose

    - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details

  - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose

- Regarding Rx UE behaviour, down-select one of the followings:

  - Option A: The Rx UE always monitors two AGC symbols in such slot

  - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol

    - FFS details

  - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol

  - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

Agreement

To address PSFCH transmission dropping due to LBT failure, RAN1 down-select one of followings, or support the combination of followings:

- Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

  - FFS other details, e.g., HARQ-ACK timeline

- Alt 2: PSFCH occasions are dynamically indicated

  - FFS: Whether/how to handle the case where some TB’s corresponding PSFCH cannot be transmitted within the same or different COT

  - FFS other details, e.g., dynamically indicate one or more PSFCH transmission(s), container of the indication, etc.

- FFS: Whether such PSFCH occasions are within the same or different COT of corresponding PSSCH

- FFS: Whether/how to address PSFCH collision if any

- FFS: Whether/how to handle the linearly decreased PSFCH capacity

Agreement

Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings:

- Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

- Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured

  - FFS details, e.g., value of K, details on gap length, etc.

- Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured

- Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period

- Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number

Agreement

Regarding additional candidate S-SSB occasions:

- In the same S-SSB period, RAN1 further study the followings:

  - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s)

  - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

  - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

  - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions

- FFS details

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and PRBs, further study the following options:

  - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size

    - FFS: whether/how to use sub-channel(s) which include intra-cell guardband PRBs

    - FFS: whether/how to handle the case when the number of PRBs of the resource pool cannot be divided by sub-channel size

  - Option 2 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set according to the sub-channel size

    - FFS: whether/how to use intra-cell guardband PRBs

    - FFS: whether/how to handle the case when the number of PRBs of one RB set cannot be divided by sub-channel size

  - Option 3 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set and/or guardband PRB according to the sub-channel size

    - FFS: how to use intra-cell guardband PRBs

    - FFS: how to use the subchannel including PRBs in guardband

Agreement

Regarding PSFCH transmission under 60 kHz SCS, further study the following alternatives:

- Alt 1: Each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs

  - FFS details

- Alt 2: Each PSFCH transmission occupies some dedicated PRBs

  - FFS details

Agreement

Regarding S-SSB, RAN1 further study the following:

- How to transmit S-SSB when a SL BWP contains multiple RB sets

- RAN1#112 (February 27 – March 3, 2023)

Agreement

For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP

  - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0

- The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP

  - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6

  - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol

- PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot

- Note: assume symbol index in a slot starts from #0

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

Agreement

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, RAN1 down-select one of followings, or support the combination of followings:

- Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - FFS: value of K3

- Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace

  - FFS: value of K1

- Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

- Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U

- Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

  - FFS: value of K2, K4

- FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth, e.g., drop common PRB or reduce power on common PRB in such case

- FFS: whether/how to reduce PAPR of PSFCH transmission

Agreement

To address PSFCH transmission dropping due to LBT failure:

- Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

  - Down-select one or support both of the followings

    - Option 1: Such PSFCH occasion(s) are (pre-)configured

    - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated

    - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc.

  - FFS other details

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and PRBs, down-select one of the followings during RAN1#112:

  - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size

    - FFS: how to deal with the remaining PRBs, e.g. for meeting OCB requirements

Agreement

For S-SSB transmission within 1 RB set, down-select to one or more of the following for 15 kHz and 30 kHz SCS:

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

  - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

- Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

  - FFS gap of 0

- Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH

  - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS.

Working assumption

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- At least for COT initiation, TBS is determined based on a reference number of symbols as follows:

  - Option 4: The reference number of symbols is determined by (pre-)configuration

  - FFS: value range

  - FFS: whether a different reference number of symbols is needed for transmission in a shared COT

Agreement

Down-select one or support both of the followings:

- Option 1: Additional candidate S-SSB occasions are excluded from resource pool

- Option 2: Additional candidate S-SSB occasions belong to resource pool

- Note: Companies are encouraged to consider aspects including: S-SSB resource overhead, Tx/Rx UE behavior (e.g., whether any blind detection in Option 2), applicable scenarios, etc.

Agreement

RAN1 further study the followings:

- Whether/how to maintain a COT when the COT contains multiple RB sets and includes S-SSB slot(s), e.g., whether to transmit S-SSB repetitions in more than one RB set, etc.

Agreement

RAN1 further study the followings:

- Whether any updates on power control are necessary considering PSD limit in unlicensed spectrum regulation.

- RAN1#112b-e (April 17 - 26, 2023)

Agreement

Considering PSD limit in unlicensed spectrum regulation, RAN1 further study whether updates on power control is needed especially for PSFCH.

Agreement

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS:

- RAN1 down-select one of followings in RAN1#113:

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - K3 is (pre-)configured, FFS value range

    - On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission

    - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB

      - FFS: whether any impact on meeting OCB requirement

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

  - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

    - K2=2

    - K4 is (pre-)configured, FFS value range

    - FFS: how to meet PSD limitation

    - FFS: whether to introduce any restrictions on the locations of K4 dedicated PRB(s) and/or K2 common PRBs, e.g., whether/how they are on the same interlace

- R16 NR SL PSSCH-PSFCH mapping is reused as baseline, FFS details

  - Note: companies are encouraged to give more details and analyze the specification impact

    - E.g., whether PSSCH transmissions on non-overlapped resources are mapped to non-orthogonal PSFCH resources, i.e., whether PSFCH collision may happen and whether/how to address it, etc.

    - E.g., whether introducing more than 6 CS pairs is needed

    - E.g., for group cast option 2, what’s the maximum group size that can be supported

    - E.g., how to support “more than 1 PSFCH occasion(s) per PSCCH/PSSCH”

- FFS: regardless of which Alt above is selected, whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR

- FFS: whether IBE issue exists and whether/how to address it

  - E.g., whether to introduce guardband PRB/RE between common PRB and dedicated PRB

Working assumption

Additional candidate S-SSB occasions are excluded from resource pool

Agreement

A SL-BWP is (pre-)configured with either contiguous RB-based or interlace RB-based PSCCH/PSSCH transmission, i.e., not both.

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U, considering 1 sub-channel equals K interlace(s), support the followings:

- Option A:

  - TBS is determined based on a reference number of PRBs of one interlace within 1 RB set (denoted as N_ref), down-select one of the followings in RAN1#113:

    - Option A1: N_ref is a fixed value, e.g., 10, 11

    - Option A2: N_ref is (pre-)defined

      - e.g., N_ref is the average number of PRBs per interlace, which is determined by total number of PRBs of the RP divided by the number of interlaces.

    - Option A3: N_ref is (pre-)configured

    - Option A4: N_ref is dynamically indicated by Tx UE

    - Note: The number of PRBs within a sub-channel can be different among sub-channels in a single resource pool subject to (pre-)configuration.

    - FFS: for TBS determination, whether/how to handle the impact of additional available  PRB(s) in intra-cell guard band(s) for PSCCH/PSSCH transmission across multiple RB sets

Agreement

Regarding more than 1 PSFCH occasion per PSCCH/PSSCH transmission, support the followings:

- One PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration

  - FFS value range of N

  - FFS detailed design of such N associated candidate PSFCH occasion(s)

    - E.g., they are in different slots of the same RB set, or in different RB sets of the same slot, or combination thereof, etc.

    - E.g., whether PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s)

- FFS: whether to support that COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions.

  - FFS: whether other associated candidate PSFCH occasion(s) within its COT are used for PSSCH transmissions, and applicable scenarios.

  - FFS: whether AGC issue and PSFCH/PSSCH collision issue exist, and whether/how to address them

- FFS other details

  - E.g., how to meet the HARQ RTT restriction

  - E.g., UE behavior on transmitting PSFCH

  - E.g., how to avoid the risk of losing the COT if the COT is interrupted by periodic PSFCH occasions

Agreement

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Option A: Support that for one PSSCH transmission, the used interlace index(s) in different used RB sets are always the same

- Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

  - Frequency domain resource of PSSCH transmission is determined by an intersection of the resource blocks of the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any

  - For a TB, the initial transmission and reservation of the resource(s) for retransmission(s) use the same number of sub-channel(s) and same number of RB set(s)

    - FFS: whether additionally support different number of RB set(s) in such case while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB

  - Use X bits for indicating sub-channel index(s), and use Y bits for indicating contiguous RB set index(s)

    - R16 NR SL FRIV is reused as baseline

    - FFS details, e.g., signaling design, bit size, whether to consider bitmap design, whether/how the used interlace(s) can be non-contiguous, etc.

- FFS others

  - E.g., considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource, and update resource selection and/or signaling from MAC to PHY, etc.

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, down-select one or more of the followings in RAN1#113:

- Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission

  - Note: PRBs within intra-cell guard band are not used for PSCCH transmission as per previous agreement

- Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission

- FFS details, e.g., conditions to apply the above Option(s)

- FFS impacts on definition of candidate resource, and resource selection

Agreement

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- TBS is determined based on a reference number of symbols (denoted as L_ref)

- Support the followings

  - Alt 1: Support Option 4 only

- Note: the options are as below

  - Option 4: The reference number of symbols is determined by (pre-)configuration

- FFS details, e.g., in TS 38.214 Clause 8.1.3.2, whether L_ref replaces Nsymbsh or sl-LengthSymbols or other usage of L_ref, whether NsymbPSFCH is determined in the same way as in legacy NR SL, etc.

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace:

- In a resource pool with multiple RB sets, sub-channel with the same index is mapped to K interlace(s) with the same index(s) in different RB sets.

- In a resource pool, support the following

  - At least for the agreed case where one SL resource pool can be (pre-)configured to include integer number of RB sets

    - Option 2: sub-channel#0 is mapped to K interlace(s) starting from interlace#0

      - sub-channel#1 is mapped to K interlace(s) starting from interlace#K, and so on

      - At least support that the above K interlace(s) are contiguous

        - FFS: whether/how to support the above K interlace(s) are non-contiguous

  - FFS: if RAN1 agrees to support that one SL resource pool can be (pre-)configured to include sub-set of PRBs of one RB set, the mapping between sub-channel and interlace for this case will be further discussed

- Interlace is indexed as per NR-U

Agreement

When the SL-BWP contains multiple RB sets, study the followings:

- When UE attempts to transmit S-SSB in a S-SSB occasion (e.g., R16/17 S-SSB occasion, R18 additional candidate S-SSB occasion)

  - Alt 1: UE may transmit S-SSB repetition in more than one RB set

    - FFS details, e.g., location of such S-SSB repetition(s) (e.g., (pre-)configured and/or pre-defined), whether/how to address potential power reduction and/or potential fluctuation of PSBCH-RSRP

    - FFS: the relationship with UE’s COT

    - FFS: the scenario that UE may or may not transmit S-SSB repetition in more than one RB set

    - Note: whether UE can transmit S-SSBs over non-contiguous RB sets is subject to RAN4’s reply, details can be found in RAN1’s LS to RAN4 in R1-2304218

Agreement

For S-SSB transmission within 1 RB set, for 15 kHz and 30 kHz SCS, Alt6 is supported:

- Alt 6: Support both Option 3-1(revised) and Option B, and enable one of them by (pre-)configuration

Note: the Options are as below

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

  - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

- Option 3-1(revised): Transmit legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), whether/how to reduce PAPR.

  - FFS gap of 0

- Option A: Legacy S-SSB

  - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption.

- Option B: Legacy S-SSB

  - RAN1 does not pursue further study on how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption.

Note: Option A and B are applicable in region with no OCB requirement, or with OCB exemption.

Conclusion

Regarding additional candidate S-SSB occasions, in the same S-SSB period, UE can attempt to transmit on additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s).

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U, support the following:

- Option 1: lowest sub-channel is the sub-channel with smallest sub-channel index

Agreement

Regarding Tx UE behavior, at least when it initiates a COT:

- For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT.

  - Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol

- FFS: whether/how to support that for the remaining slots of a COT, the Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission.

  - FFS applicable scenarios

    - e.g., at least for MCSt with no greater than 16us gap

    - e.g., at least for transmission with no greater than 16us gap from the previous transmission by any UE

  - FFS: Rx UE behavior

FFS: COT sharing case

- RAN1#113 (May 22-26, 2023)

Agreement

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS:

- One of the following alternatives is (pre-)configured:

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - K3 is (pre-)configured

      - Value range for K3 at least includes {1, 2, 5}

    - K3 dedicated PRB(s) are on the same interlace

    - There can be some guardband PRB(s) between common PRB and dedicated PRB

      - FFS details, e.g., whether/how to derive the number of guardband PRB(s), whether to additionally introduce a (pre-)configured gap (including 0), or whether this can be satisfied by (pre-)configuration and there is no additional specification impact (e.g., setting proper bit values in bitmap for PSFCH PRB allocation), etc.

      - FFS whether to additionally introduce guardband RE between common PRB and dedicated PRB

    - On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission

    - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB subject to meeting OCB requirements

    - FFS: whether to reduce power on common PRBs

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

- PSSCH transmissions on non-overlapped resources are mapped to orthogonal dedicated PRBs for PSFCH transmission

- FFS: whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR

- FFS: whether to drop common PRBs if the dedicated PRBs can already satisfy OCB requirement

Agreement

When the SL-BWP contains multiple RB sets, support the followings:

- When UE attempts to transmit S-SSB in a S-SSB occasion (e.g., R16/17 S-SSB occasion, R18 additional candidate S-SSB occasion)

  - UE may transmit S-SSB repetition in more than one RB set

    - Down-select one of the followings in RAN1#114:

      - Alt 1: At least the power for S-SSB transmission on anchor RB set does not change due to the number of used RB sets

        - FFS details, e.g., whether this can be satisfied by (pre-)configuration, whether the power for S-SSB transmission on other RB set(s) also does not change due to the number of used RB sets, etc.

      - Alt 2: The power for S-SSB transmission on each RB set does not change due to the number of used RB sets

        - FFS details, e.g., whether this can be satisfied by (pre-)configuration, etc.

    - FFS: Locations of S-SSB repetitions in each RB set are the same as the locations of S-SSB repetitions in the anchor RB set

    - FFS: how to (pre)configure resources for the S-SSB repetitions

    - Note: anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates

    - Note: whether UE can transmit S-SSBs over non-contiguous RB sets is subject to RAN4’s reply, details can be found in RAN1’s LS to RAN4 in R1-2304218, R1-2306198

Agreement

Regarding “Option 3-1(revised): Transmit legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement”:

- Support:

  - Alt 3: the value of gap is (pre-)configured, and the value of N is (pre-)configured

    - FFS: value range for gap and N

    - FFS: whether N for different RB sets can be different

    - FFS: whether to apply any restriction on sl-AbsoluteFrequencySSB-r16 for 60 kHz

- FFS: whether/how to support reducing PAPR of S-SSB transmission, at least considering the following options

  - Option 1: use different NIDSL across the different S-SSB repetitions to determine the initial scrambling seed of PSBCH, and the sequence shift for S-SSS and S-PSS

  - Option 2: phase adjustment among repetitions

  - Option 3: no specification impact to reduce PAPR

  - Option 4: use S-SSB repetition index to scramble different S-SSB repetition(s)

Agreement

Regarding the number and location(s) of additional candidate S-SSB occasions, support:

- Option 2 (12): Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion(s) in different time slot(s), and the gap between them is (pre-)configured

  - FFS details, e.g., value of K, details on gap length (including possibility of being 0), etc.

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U, support the followings:

- Option A3: N_ref is (pre-)configured

  - The value range for N_ref at least includes {10, 11}

  - FFS: whether to additionally include other values

Agreement

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, for TBS determination and 2nd SCI overhead, in TS 38.214 Clause 8.1.3.2:

- L_ref replaces sl-LengthSymbols

  - Value range of L_ref is {7, 8, 9, 10, 11, 12, 13, 14} symbols

- NsymbPSFCH is determined in the same way as in legacy NR SL

Working assumption

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Use FRIVsubCH to indicate used sub-channel index(s)

- FRIVsubCH is conveyed in 1st stage SCI

- FRIVsubCH is calculated as below

  - If sl-MaxNumPerReserve is 2 then

    - FRIVsubCH=nsubCH,1start+i=1LsubCHRBset-1N subchannel RBset+1-i

  - If sl-MaxNumPerReserve is 3 then

    - FRIVsubCH=nsubCH,1start+nsubCH,2start⋅N subchannel RBset+1-LsubCHRBset+i=1LsubCHRBset-1N subchannel RBset+1-i2

  - where

    - nsubCH,1start denotes the starting sub-channel index for the second resource

    - nsubCH,2start denotes the starting sub-channel index for the third resource

    - N subchannel RBset is the number of sub-channels for each RB set

    - LsubCHRBset is the number of used sub-channels for each RB set for each of the indicated resources

- FFS: whether/how to use spare states of FRIVsubCH to indicate using non-contiguous interlaces

- FFS: whether additionally support different number of RB set(s) in such case while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB

- FFS: whether additionally support using bitmap

Working assumption

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Use FRIVRBset to indicate used RB set index(s)

- FRIVRBset is conveyed in 1st SCI

- FRIVRBset is calculated as below

  - If sl-MaxNumPerReserve is 2 then

    - FRIVRBset=nRBset,1start+i=1LRBset-1NRBset+1-i

  - If sl-MaxNumPerReserve is 3 then

    - FRIVRBset=nRBset,1start+nRBset,2start⋅NRBset+1-LRBset+i=1LRBset-1NRBset+1-i2

  - where

    - nRBset,1start denotes the starting RB set index for the second resource

    - nRBset,2start denotes the starting RB set index for the third resource

    - NRBset is the number of RB sets in a resource pool

    - LRBset is the number of used RB sets for each of the indicated resources

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, support only option 3.

- FFS other details, e.g., impacts on resource selection, PSCCH mapping, etc.

- Note:

  - Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission

    - Note: PRBs within intra-cell guard band are not used for PSCCH transmission as per previous agreement

  - Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission

  - Nleft : the number of remaining PRBs of a sub-channel belonging to a RB set after excluding the PRBs belonging to intra-cell guardband

  - NPSCCH : the number of PRBs for PSCCH transmission

Agreement

When neither COT initiating UE nor responding UE intends to transmit PSFCH on some PSFCH occasion(s) within a COT, to avoid COT interruption, select one or more of the followings:

- Option 1: COT initiating UE or responding UE transmits PSSCH on such PSFCH occasion(s)

  - FFS details, e.g., how PSSCH Rx UE knows such transmission, etc.

- Option 2: COT initiating UE or responding UE transmits a PSFCH-like signal on such PSFCH occasion(s)

  - FFS details, e.g., signaling design, etc.

- Option 3: no optimization for this case

Agreement

Regarding one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration:

- Regarding locations of candidate PSFCH occasion(s):

  - Down-select at RAN1#114:

    - Alt 1 (15): Associated PSFCH occasion(s) are within the RB set(s) occupied by PSSCH transmissions

      - FFS details

    - Alt 2 (2): PSSCH transmission and its related PSFCH occasion(s) are in the same or different RB set(s) of the same resource pool

  - For one PSCCH/PSSCH transmission, at least support that its associated candidate PSFCH occasion(s) are in different slots of the same RB set(s)

    - FFS: whether to support its associated candidate PSFCH occasion(s) are in different RB sets of the same slot

Agreement

Regarding one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration:

- Value range of N at least includes {1, 2, 3, 4}

- When N>1, N associated candidate PSFCH occasion(s) for one PSCCH/PSSCH transmission have different time and/or frequency resource with the candidate PSFCH occasion(s) for another PSCCH/PSSCH transmission, at least if these two PSCCH/PSSCH transmissions are on non-overlapped resources

Agreement

Regarding one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration:

- Regarding UE behaviour on transmitting PSFCH:

  - Down-select at RAN1#114:

    - Alt 1: For one PSCCH/PSSCH transmission, PSCCH/PSSCH receiver UE attempts to transmit PSFCH on a candidate PSFCH occasion if and only if it fails to transmit on previous PSFCH occasion(s) due to LBT failure

    - Alt 2: For one PSCCH/PSSCH transmission, PSCCH/PSSCH receiver UE attempts to transmit PSFCH on a candidate PSFCH occasion if and only if it fails to transmit on previous PSFCH occasion(s) (e.g., due to LBT failure, or due to UL/SL prioritization, etc.)

    - Alt 3: Do not specify additional UE behavior on transmitting PSFCH due to LBT failure.

- FFS: UE behaviour on receiving PSFCH

- Regarding HARQ RTT restriction:

  - Further study whether/how to update the followings:

    - The minimum time gap Z=a+b between any two selected resources of a TB in case PSFCH is configured for this resource pool

    - The reference slot n for PUCCH transmission to report HARQ in Mode 1

- RAN1#114 (August 21-25, 2023)

Agreement

In “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”, regarding common interlace, index of common interlace is (pre-)configured per resource pool.

Agreement

Regarding PSFCH transmission,

- For “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”

  - Cyclic shift on each of K3 dedicated PRB(s) is the same

  - Cyclic shift on each PRB of common interlace is up to UE implementation

- For “Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace”

  - Support PRB-level cyclic shift hopping as in NR-U to reduce PAPR

Working Assumption

In “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”, regarding PSSCH-PSFCH mapping, down-select one of followings in RAN1#114:

- Alt 1: Map to a dedicated PRB subset

  - Step 1: UE determines the (pre-)configured dedicated PRB set MPRB,setPSFCH for current PSFCH occasion

  - Step 2: Index MPRB,setPSFCH dedicated PRBs, based on PRB index in an interlace first and interlace index second rule

    - FFS: whether/how to consider RB set index

  - Step 3: After indexing in Step 2, every K3 dedicated PRBs forms a dedicated PRB subset

  - Step 4: Legacy PSSCH-PSFCH mapping is reused with changes that each PSCCH/PSSCH candidate is mapped to one or multiple dedicated PRB subset(s) above

    - Note: “On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission” as per previous agreement

    - FFS: detailed steps to determine PSFCH resources for each PSFCH transmission

- Alt 2: Map to anchor RB, and extend to a dedicated PRB subset

  - Step 1: UE determines one anchor PRB for PSFCH transmission as per legacy PSSCH-PSFCH mapping rule

    - The set of anchor PRBs is the set of PRBs indicated as “1” as per (pre-)configured bitmap sl-PSFCH-RB-Set

  - Step 2: Assume anchor PRB is in PRB index k, then UE determines K3 dedicated PRB(s) are PRB {k, k+z, …, k+(K3-1)*z}

    - z is the number of interlaces, i.e., z=10 for 15kHz, z=5 for 30kHz

    - Note: above PRB {k+z, …, k+(K3-1)*z} are subset of PRBs indicated as “0” as per (pre-)configured bitmap sl-PSFCH-RB-Set

  - Note: it is up to (pre-)configuration of bitmap to ensure “PSSCH transmissions on non-overlapped resources are mapped to orthogonal dedicated PRBs for PSFCH transmission” as per previous agreement

- Note: UE expects the number of available dedicated PRBs based on (pre-)configured bitmap sl-PSFCH-RB-Set within one interlace is an integer multiple of K3

Agreement

Regarding “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)”:

- Support Alt 1 below, further support:

  - Legacy PSSCH-PSFCH mapping is reused by replacing “resource pool-level mapping” with “RB set-level mapping”,

- Note:

  - Alt 1: Associated PSFCH occasion(s) are within the RB set(s) occupied by PSSCH transmissions

  - Alt 2: PSSCH transmission and its related PSFCH occasion(s) are in the same or different RB set(s) of the same resource pool

Working Assumption

Regarding “UE may transmit S-SSB repetition in more than one RB set”, down-select one of the followings in RAN1#114:

- Alt 1: At least the power for S-SSB transmission on anchor RB set does not change due to the number of used RB sets

  - On anchor RB set, there is an offset Poffset_anchor to limit the maximum power as below (changes to legacy NR SL is marked in red)

    - PS-SSB_anchor(i)=minPCMAX-Poffset_anchor,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm]

    - FFS value range of Poffset_anchor

    - Note: if Poffset_anchor is always 0 then the parameter does not need to be defined

  - On non-anchor RB set, down-select one of the followings

    - Alt 1-1:

      - UE first allocates power to S-SSB repetitions on anchor RB set, assume the power of each S-SSB repetition is PS-SSB_anchor

      - Then, UE allocates remaining power Pleft equally to other S-SSB repetitions on all other used RB sets subject to DL pathloss, where Pleft=PCMAX-N*PS-SSB_anchor

    - Alt 1-2:

      - On non-anchor RB set, there is an offset Poffset_non_anchor to limit the maximum power as below (changes to legacy NR SL is marked in red)

        - PS-SSB_non_anchor(i)=minPCMAX-Poffset_non_anchor,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm]

        - FFS value range of Poffset_non_anchor

    - Alt 1-3:

      - UE first allocates power to anchor RB set, assume the power is PS-SSB_anchor

      - Then, UE allocates remaining power Pleft equally to all other used RB sets, where Pleft=PCMAX-N*PS-SSB_anchor (with N=1)

- Alt 2: The power for S-SSB transmission on each RB set does not change due to the number of used RB sets

  - Alt 2-1:

    - There is an offset Poffset to limit the maximum power as below (changes to legacy NR SL is marked in red)

      - PS-SSB(i)=minPCMAX-Poffset,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm]

      - Poffset=10lg⁡(M*N)

- In above Alts, M is the total number of RB sets within this SL-BWP, N is the number of S-SSB repetitions within one RB set

- Note: the above power for S-SSB transmission refers to power of one S-SSB repetition

- UE at least attempts to transmit on anchor RB set

  - Note: anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates

Agreement

- Support PSFCH transmission over contiguous and non-contiguous RB sets

- Support S-SSB transmission over contiguous and non-contiguous RB sets

- Support of PSFCH/S-SSB transmission over contiguous and/or non-contiguous RB sets is subject to UE capability(ies)

Agreement

Regarding “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)” and “For one PSCCH/PSSCH transmission, at least support that its associated candidate PSFCH occasion(s) are in different slots of the same RB set(s)”, support:

- Slot index of 1st PSFCH occasion (denoted as slot k) of a PSCCH/PSSCH transmission is determined in the same way as legacy NR SL

- The nth PSFCH occasion is in slot k+n-1*P

    - Alt 1: P is equal to the (pre-)configured PSFCH periodicity, i.e., P is provided by sl-PSFCH-Period

  - 1≤n≤N

- Within a slot including PSFCH, for each RB set, the (pre-)configured PRBs for PSFCH transmission on this RB set are divided into N different PRB sets (denoted as set#1, set#2, …, set#N), which are associated with N candidate PSFCH occasion(s)

  - Within this RB set, for one sub-channel on one slot of PSCCH/PSSCH transmission, its nth PSFCH occasion includes PRBs belonging to above set#n in slot k+n-1*P

  - FFS: whether to use 1 or N bitmaps to indicate resource for N candidate PSFCH occasion(s), respectively

Agreement

In “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)”, regarding UE behaviour on transmitting PSFCH:

- Support Alt 2 below

- Note:

  - Alt 1: For one PSCCH/PSSCH transmission, PSCCH/PSSCH receiver UE attempts to transmit PSFCH on a candidate PSFCH occasion if and only if it fails to transmit on previous PSFCH occasion(s) due to LBT failure

  - Alt 2: For one PSCCH/PSSCH transmission, PSCCH/PSSCH receiver UE attempts to transmit PSFCH on a candidate PSFCH occasion if and only if it fails to transmit on previous PSFCH occasion(s) (e.g., due to LBT failure, or due to UL/SL prioritization, etc.)

  - Alt 3: Do not specify additional UE behavior on transmitting PSFCH due to LBT failure.

Agreement

Regarding “Option 2 (12): Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion(s) in different time slot(s), and the gap between them is (pre-)configured”, support:

- K is (pre-)configured per SL-BWP from {0, 1, 2, 3, 4}

- Gap_1 is (pre-)configured per SL-BWP from {0, 1, 2, 3, 4, …, 639} slots

  - Gap_2 is fixed to Gap_1

- Note:

  - Gap_1 is the gap between a R16/R17 NR SL S-SSB slot and its 1st corresponding additional candidate S-SSB occasion

  - Gap_2 is the gap between any two adjacent additional candidate S-SSB occasions corresponding to one R16/R17 NR SL S-SSB slot when K>1

Agreement

Regarding “Option 3-1(revised): Transmit legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement”, and “Alt 3: the value of gap is (pre-)configured, and the value of N is (pre-)configured”, support:

- Gap is (pre-)configured per RB set from {[0], 1, 2, 3, …, 84} PRBs

- N is (pre-)configured per RB set from {2, 3, 4, 5, …, 9}

- The gap is between the lowest subcarrier of the upper PSBCH and the highest subcarrier of the lower PSBCH

Working assumption

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- The PSCCH modulation symbols are mapped sequentially over the PRBs of a sub-channel, regardless the number of interlace within one sub-channel

- The PSSCH modulation symbols are mapped sequentially over the PRBs among all the allocated PRBs for PSSCH transmission, regardless the number of interlace within one sub-channel and number of allocated sub-channels

Note: this working assumption will be automatically confirmed if no concern is raised before the end of RAN1#114.

Agreement

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

  - Alt A: MAC layer indicates both LsubCH and LRBset to PHY layer, where LRBset is the number of used RB sets for one PSCCH/PSSCH transmission

    - Regarding LsubCH in TS 38.214 Clause 8.1.4, down-select one of the followings in RAN1#114:

      - Sub-Alt 1: LsubCH is “the number of sub-channels within each RB set to be used for the PSSCH/PSCCH transmission in a slot”

        - Note: LsubCHRBset=LsubCH

      - Note: LsubCHRBset is the number of used sub-channels within each RB set for one PSCCH/PSSCH transmission

- Resources from the RB set where C-LBT failure was detected are not reported to MAC layer.

- Note: RAN1 assumes “MAC informs PHY of the RB set information where SL C-LBT failure was detected” as per RAN2’s LS in R1-2306174

Agreement

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- Regarding Tx UE behaviour:

  - If PSCCH/PSSCH transmission starts from 1st starting symbol, the PSCCH/PSSCH transmission has at least 1 AGC symbol

- Regarding Rx UE behaviour:

  - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

- Note: AGC symbol is duplication of next symbol

Agreement

In “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)”, regarding Rx UE behaviour on receiving PSFCH for a PSCCH/PSSCH transmission, support:

- For unicast:

  - FFS: Monitor:

    - Rx UE attempts to monitor candidate PSFCH occasion(s) until one PSFCH is detected or all candidate PSFCH occasion(s) are monitored.

    - If one PSFCH is detected, Rx UE can omit monitoring following candidate PSFCH occasion(s).

  - Report:

    - If Rx UE receives PSFCH, Rx UE reports same value as a value of HARQ-ACK information that the UE determines from the PSFCH reception to higher layers, otherwise re-ports NACK to higher layer.

- FFS: For groupcast option 1 (NACK only):

  - FFS: Monitor:

    - Rx UE attempts to monitor all candidate PSFCH occasions.

    - If NACK is detected, Rx UE can omit monitoring following candidate PSFCH occasion(s).

  - Report:

    - If Rx UE does not detect any PSFCH in all candidate PSFCH occasions, Rx UE reports ACK to higher layers; otherwise, reports NACK to higher layers.

- For groupcast option 2 (ACK/NACK):

  - FFS: Monitor:

    - Rx UE attempts to monitor PSFCH transmission occasions until PSFCH from all transmitters have been detected or all candidate PSFCH occasions are monitored.

    - If Rx UE detects PSFCH from one PSFCH transmitter, it can omit PSFCH detection for following PSFCH transmission occasions for this PSFCH transmitter.

  - Report:

    - If ACK has been detected from at least one PSFCH occasion of each of all expected PSSCH receivers, Rx UE reports ACK to higher layers; otherwise, reports NACK to higher layers.

Agreement

In “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”, regarding mapping between PSSCH and K3 dedicated PRB(s):

- Alt 1: Map to a dedicated PRB subset

  - Within one RB set, for mapping between “one sub-channel on one slot” to “PRBs for PSFCH”

    - Step 1: For nth PSFCH occasion, UE determines the (pre-)configured dedicated PRB set set#n

      - 1≤n≤N, N refers to “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)”

    - Step 2: Index dedicated PRBs in set#n, based on PRB index in an interlace first and interlace index second rule

    - Step 3: After indexing in Step 2, every K3 dedicated PRBs forms a dedicated PRB subset

      - The total number of dedicated PRB subsets is MRBSetPSFCH,subset(i) , and UE expects that MRBSetPSFCH,subset(i) is a multiple of (NSubCHRBSet∙NPSSCHPSFCH).

        - NSubCHRBSet is the number of sub-channels within one RB set

        - NPSSCHPSFCH is the (pre-)configured PSFCH periodicity

        - i is RB set index

    - Step 4: Legacy PSSCH-PSFCH mapping is reused with changes that “one sub-channel on one slot” is mapped to one or multiple dedicated PRB subset(s) above

      - Note: “On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission” as per previous agreement

      - Let Msubch,slotPSFCH,subset(i) denote the number of dedicated PRB subset(s) for “one sub-channel on one slot” on RB set i, i.e.,

        - Msubch,slotPSFCH,subset(i)= MRBSetPSFCH,subset(i)/ (NSubCHRBSet∙NPSSCHPSFCH)

  - For a PSCCH/PSSCH transmission, UE determines PSFCH resources as below

    - For this PSCCH/PSSCH transmission, the total number of PSFCH resources is RPRB_subset, CSPSFCH=X·NCSPSFCH, where NCSPSFCH is the number of (pre-)configured cyclic shift pairs

      - if sl-PSFCH-CandidateResourceType is configured as startSubCH,

        - Ntype PSFCH=1

        - X=Msubch,slotPSFCH,subset(i)

        - and the Msubch,slotPSFCH,subset(i) dedicated PRB subset(s) are associated with the lowest sub-channel index of lowest RB set of the corresponding PSSCH

      - if sl-PSFCH-CandidateResourceType is configured as allocSubCH,

        - Ntype PSFCH=Nsubch PSSCH

        - X is the total number of all dedicated PRB subset(s) corresponding all allocated sub-channels on all allocated RB sets of the corresponding PSSCH

        - and the X dedicated PRB subset(s) are associated with the Nsubch PSSCH sub-channels of the corresponding PSSCH, where Nsubch PSSCH is the total number of all allocated sub-channels on all allocated RB sets of the corresponding PSSCH

    - The PSFCH resources are indexed according to dedicated PRB subset(s) first, RB set second, cyclic shift pair index third rule.

      - The PSFCH resources are within the RB set(s) of PSCCH/PSSCH transmission

    - UE determines an index of a PSFCH resource for a PSFCH transmission with HARQ-ACK information in response to a PSSCH reception as PID+MID mod RPRB_subset, CSPSFCH, where PID,MID are same as legacy

- UE expects the number of available dedicated PRBs based on (pre-)configured bitmap sl-PSFCH-RB-Set within one interlace within one RB set is an integer multiple of K3

Agreement

Regarding “UE may transmit S-SSB repetition in more than one RB set”:

- At least the power for S-SSB transmission on anchor RB set does not change due to the number of used RB sets

  - On anchor RB set, there is a (pre-)configured offset Poffset_anchor to limit the maximum power as below (changes to legacy NR SL is marked in red)

    - PS-SSB_anchor(i)=minPCMAX-Poffset_anchor,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm], where i is slot index as in legacy

    - value range of Poffset_anchor is: {10lg(N), [10lg(N)+2, 10lg(N)+4, …],10lg⁡(W)}

  - On non-anchor RB set

    - UE first allocates power to S-SSB repetitions on anchor RB set, assume the power of each S-SSB repetition is PS-SSB_anchor

    - Then, UE allocates remaining power Pleft equally to other S-SSB repetitions on all other used RB sets, where Pleft=PCMAX-N*PS-SSBanchor, where PCMAX and PS-SSB_anchor are converted to linear unit (i.e, Watt) in this formula

    - Note: for both anchor RB set and non-anchor RB set transmission, the same DL pathloss is taken into account

- M is the total number of RB sets within this SL-BWP, N is the number of S-SSB repetitions within the anchor RB set, W is the maximum total number of S-SSB repetitions on RB sets within the SL-BWP

- Note: the above power for S-SSB transmission refers to power of one S-SSB repetition

- UE at least attempts to transmit on anchor RB set

  - Note: anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates

- For above Alts, PCMAX is determined according to TS 38.101-1 for transmission of all S-SSB repetitions on all used RB sets

Agreement

Regarding “For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, support only option 3” and “Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission”:

- Candidate resource, whose lowest sub-channel includes intra-cell guardband PRBs, is excluded

  - Such exclusion is performed in PHY layer, and such candidate resource is excluded in Step 1

Agreement

For contiguous RB-based PSSCH transmission:

- MAC layer indicates only LsubCH to PHY layer as in legacy NR SL

  - LsubCH is “the number of sub-channels within all used RB sets to be used for the PSSCH/PSCCH transmission in a slot”

Working assumption

In “Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace”, regarding mapping between PSSCH and 1 dedicated interlace:

- Alt 2: Map to a dedicated PRB subset interlace

  - Within one RB set, for mapping between “one sub-channel on one slot” to “PRBs interlace for PSFCH”

    - Step 1: For nth PSFCH occasion, UE determines the (pre-)configured dedicated PRB set interlace set#n using legacy PRB-level bitmap

      - 1≤n≤N, N refers to “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)”

    - Step 2: Index dedicated PRBs interlace(s) in set#n, based on PRB index in an interlace first and interlace index second rule

    - Step 3: After indexing in Step 2, every K3 dedicated PRBs forms a dedicated PRB subset

      - The total number of dedicated PRB subsets interlace(s) is MRBSetPSFCH,subset(i) , and UE expects that MRBSetPSFCH,subset(i) is a multiple of (NSubCHRBSet∙NPSSCHPSFCH).

        - NSubCHRBSet is the number of sub-channels within one RB set

        - NPSSCHPSFCH is the (pre-)configured PSFCH periodicity

        - i is RB set index

    - Step 3: Legacy PSSCH-PSFCH mapping is reused with changes that “one sub-channel on one slot” is mapped to one or multiple dedicated PRB subset interlace(s) above

      - Note: “On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission” as per previous agreement

      - On the dedicated interlace, multiple CS pairs can be used as in legacy NR SL PSFCH transmission

      - Let Msubch,slotPSFCH,subset(i) denote the number of dedicated PRB subset interlace(s) for “one sub-channel on one slot” on RB set i, i.e.,

        - Msubch,slotPSFCH,subset(i)= MRBSetPSFCH,subset(i)/ (NSubCHRBSet∙NPSSCHPSFCH)

  - For a PSCCH/PSSCH transmission, UE determines PSFCH resources as below

    - For this PSCCH/PSSCH transmission, the total number of PSFCH resources is RInterlace, CSPSFCH=X ∙NCSPSFCH RPRB_subset, CSPSFCH=X·NCSPSFCH, where NCSPSFCH is the number of (pre-)configured cyclic shift pairs

      - if sl-PSFCH-CandidateResourceType is configured as startSubCH,

        - Ntype PSFCH=1

        - X=Msubch,slotPSFCH,subset(i)

        - and the Msubch,slotPSFCH,subset(i) dedicated PRB subset interlace(s) are associated with the lowest sub-channel index of lowest RB set of the corresponding PSSCH

      - if sl-PSFCH-CandidateResourceType is configured as allocSubCH,

        - Ntype PSFCH=Nsubch PSSCH

        - X is the total number of all dedicated PRB subset interlace(s) corresponding all allocated sub-channels on all allocated RB sets of the corresponding PSSCH

        - and the X dedicated PRB subset interlace(s) are associated with the Nsubch PSSCH sub-channels of the corresponding PSSCH, where Nsubch PSSCH is the total number of all allocated sub-channels on all allocated RB sets of the corresponding PSSCH

    - The PSFCH resources are indexed according to dedicated PRB subset interlace (s) first, RB set second, cyclic shift pair index third rule.

      - The PSFCH resources are within the RB set(s) of PSCCH/PSSCH transmission

    - UE determines an index of a PSFCH resource for a PSFCH transmission with HARQ-ACK information in response to a PSSCH reception asPID+MIDmod RInterlace, CSPSFCH  PID+MID mod RPRB_subset, CSPSFCH,  where PID,MID are same as legacy

- UE expects the number of available dedicated PRBs based on (pre-)configured bitmap sl-PSFCH-RB-Set within one interlace within one RB set is an integer multiple of K3

- UE expects all the PRBs of one interlace within 1 RB set are available for PSFCH transmission

Agreement

In “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)”, regarding Rx UE behaviour on receiving PSFCH for a PSCCH/PSSCH transmission, support:

- For unicast:

  - Monitor:

    - Alt 1: Rx UE attempts to monitor all candidate PSFCH occasion(s)

      - If one PSFCH is detected, Rx UE can omit monitoring following candidate PSFCH occasion(s), if any.

    - PSFCH prioritization rule is used

- For groupcast option 2 (ACK/NACK):

  - Monitor:

    - Alt 1: Rx UE attempts to monitor all PSFCH transmission occasions.

      - If Rx UE detects PSFCH from a PSFCH transmitter, it can omit PSFCH detection for following PSFCH transmission occasions for this PSFCH transmitter, if any.

    - PSFCH prioritization rule is used

## RAN1#114bis (October 9-13, 2023)

Agreement

Regarding “Within a slot including PSFCH, for each RB set, the (pre-)configured PRBs for PSFCH transmission on this RB set are divided into N different PRB sets (denoted as set#1, set#2, …, set#N), which are associated with N candidate PSFCH occasion(s)”:

- use N bitmaps to indicate resource for N candidate PSFCH occasion(s), respectively

  - It shall be (pre-)configured such that N candidate PSFCH occasion(s) are associated with N different PRB sets

Agreement

Regarding frequency locations of S-SSB repetitions within anchor RB set:

- the S-SSB indicated by sl-AbsoluteFrequencySSB is the lowest S-SSB in anchor RB set

Agreement

Regarding frequency locations of S-SSB repetitions within a non-anchor RB set:

- the lowest S-SSB in the non-anchor RB set is indicated by new RRC parameters, where each RRC parameter corresponds to one non-anchor RB set

Agreement

Regarding PRBs within intra-cell guard band:

- Such PRBs are not used for PSFCH transmission

- Such PRBs are not used for S-SSB transmission

Note: spec impact (if any) can be discussed.

Agreement

- DCI format 3_0 is updated to include the field of “lowest index of the RB set allocation to the initial PSSCH transmission”.

- For interlace RB-based PSSCH transmission, Sidelink configured grant Type 1 configuration provides a new RRC parameter sl-StartRBsetCG-Type1 to indicate starting RB set index of the initial PSSCH transmission of the sidelink configured grant Type 1

Agreement

In “Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace”:

- The power for a PSFCH transmission is equally allocated over the occupied PRBs.

Agreement

TP#4-5 in Section 4.2.5 of R1-2310350 is endorsed for TS 38.213 clause 16.3.0.

Agreement

TP#1-1 in Section 4.2.1 of R1-2310350 is endorsed for TS 38.214 clause 7, and clause 8.1.4.

Agreement

For SL-U PHY, following higher layer parameters are endorsed.

| row | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) |
| --- | --- | --- | --- | --- | --- |
| 1 | startRBResourcePool | Indicates the lowest RB index in the resource pool with respect to the lowest RB index of a SL BWP. UE expects that the lowest RB in the resource pool is not within intra-cell guard band. | INTEGER (0..224) | N/A | Per resource pool |
| 2 | numInterlacePerSubchannel | Indicates the number of interlaces per sub-channel within a resource pool, i.e. 1 sub-channel =K interlace(s). The applicable values are related to the subcarrier spacing as below: For SCS = 15 kHz: K=1 or 2 For SCS = 30 kHz: K=1 | ENUMERATED{1, 2} | N/A | Per resource pool |
| 3 | intraCellGuardBandsSL-List | List of intra-cell guard bands for operation with shared spectrum channel access. If not configured, the guard bands are defined according to 38.101-1 [15], see TS 38.214 [19], clause 7. For operation in licensed spectrum, this field is absent, and no UE action is required. | SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16 | N/A | Per SL BWP |
| 4 | startingSymbolFirst | Indicates the location of 1st starting symbol within a slot | ENUMERATED {sym0, sym1, sym2, sym3, sym4, sym5, sym6} | sym0 | Per SL BWP |
| 5 | startingSymbolSecond | Indicates the location of 2nd starting symbol within a slot Note： o the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6 o within a slot, the 2nd starting symbol is later than the 1st starting symbol • PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot | ENUMERATED {sym3,sym4,sym5,sym6,sym7} | N/A | Per SL BWP |
| 6 | numRefSymbolLength | Indicates a reference number of symbols for TBS determination | ENUMERATED {sym7, sym8, sym9, sym10, sym11, sym12, sym13, sym14} | N/A | [Per SL BWP or per resource pool] |
| 7 | transmissionStructureForPSCCHandPSSCH | Indicate a SL-BWP is (pre-)configured with contiguous RB-based or interlace RB-based PSCCH/PSSCH transmission. Note: Legacy PSCCH/PSSCH are applicable in region with no OCB requirement, or with OCB exemption | ENUMERATED {contigousRB, interlaceRB} | N/A | Per SL BWP |
| 8 | numRefPRBOfInterlace | Indicate reference number of PRBs of one interlace within 1 RB set | ENUMERATED {10, 11} | N/A | [Per SL BWP or per resource pool] |
| 9 | transmissionStructureForSSSB | Indicate use legacy S-SSB or repetition for S-SSB transmission. Note: Legacy S-SSB are applicable in region with no OCB requirement, or with OCB exemption. | ENUMERATED {nonRepetitionSSB, repetitionSSB} | N/A | Per SL BWP |
| 10 | numOfSSSBrepetition | Indicate the number of S-SSB repetitions in frequency domain in one RB set Note: Legacy S-SSB are applicable in region with no OCB requirement, or with OCB exemption. | INTEGER (2,3,…,9) | N/A | Per SL BWP RB set |
| 11 | gapBetweenSSSBrepetition | Indicate the gap between two adjacent S-SSB repetitions in frequency domain in one RB set, and the gap is between the lowest subcarrier of the upper PSBCH and the highest subcarrier of the lower PSBCH. | INTEGER([0], 1,2,3,…, 84) PRBs | N/A | Per SL BWP RB set |
| 12 | numOfAdditionalSSSBOccasion | Indicate the number of additional candidate S-SSB occasion(s) for each R16/R17 NR SL S-SSB slot | INTEGER (0,1,2,3,4) | [N/A or 0] | Per SL BWP |
| 13 | gapOfAdditionalSSSBOccasion | Indicate the gap between each R16/R17 NR SL S-SSB slot and its first corresponding additional candidate S-SSB occasion, and the gap between adjacent two additional candidate S-SSB occasion corresponding to a R16/R17 NR SL S-SSB slot | INTEGER (0,1,2,…,639) slot | N/A | Per SL BWP |
| 14 | transmissionStructureForPSFCH | Indicate each PSFCH transmission occupies "1 common interlace and K3 dedicated PRB(s)", or "1 dedicated interlace". | ENUMERATED {common interlace, dedicated interlace} | N/A | Per resource pool |
| 15 | numDedicatedPRBsForPSFCH | Indicates the value of K3 when each PSFCH transmission occupies "1 common interlace and K3 dedicated PRB(s)" | ENUMERATED {1,2,5} | N/A | Per resource pool |
| 16 | numPSFCHOccasions | Indicates one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) | ENUMERATED {1,2,3,4} | 1 | Per resource pool |
| 17 | PSFCHCommonInterlaceIndex | Indicate the index of common interlace to meet OCB requirements when transmissionStructureForPSFCH is set to common interlace, e.g., value 0 means interlace 0 is used as common interlace | INTEGER (0,1,2,...,9) | N/A | Per resource pool |
| 18 | SSSBPowerOffsetOfAnchorRBSet | Indicate the power offset for one S-SSB transmission on anchor RB set. anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates N is the number of S-SSB repetitions within the anchor RB set, W is the maximum total number of S-SSB repetitions on RB sets within the SL-BWP | EMUNERATED{{10lg(N), [10lg(N)+2, 10lg(N)+4, …,]10lg(W)}} | N/A | Per SL BWP |

Working assumption

- In “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”:

  - Assume the UE transmits N PSFCH

    - Denote the final Tx power on one common PRB is P_common

    - Denote the final Tx power on one dedicated PRB is P_dedicated

    - P_common <= P_dedicated

      - (pre-)configure an offset between P_common and P_dedicated

  - Send an LS to RAN4 asking whether there is any difficulty for supporting the following cases

    - P_common < P_dedicated

    - P_common = P_dedicated

Agreement

The draft LS to RAN4 is endorsed in R1-2310594. Final LS in R1-2310595.

Agreement

For the contiguous RB-based PSCCH/PSSCH,

- For the case where the highest sub-channel of a candidate resource overlaps with a single RB set and intra-cell guardband PRBs

  - a reference number of PRBs of one sub-channel is used for TBS determination, and it is equal to the (pre-)configured sub-channel size.

    - TP#3-1 in Section 4.7.1 of R1-2310354 is endorsed for TS 38.214 clause 8.1.3.2.

  - Note: the above sub-channel “cannot be used for PSCCH transmission, and can be used for PSSCH transmission” as per previous agreement

Agreement

For SL-U PHY, following higher layer parameters are endorsed.

| row | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) |
| --- | --- | --- | --- | --- | --- |
| 19 | sl-PSFCH-RB-SetList | The n-th value in the list indicates the set of PRBs that are actually used for PSFCH transmission and reception of n-th PSFCH occasion of a PSCCH/PSSCH transmission. It shall be (pre-)configured such that N candidate PSFCH occasion(s) are associated with N different PRB sets. | FFS | N/A | Per resource pool |
| 20 | sl-AbsoluteFrequencySSBNonAnchorList | Each parameter in this list indicates the lowest S-SSB in a non-anchor RB set. Note: anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates. Note: NonAnchorRBsetsNum refers to the number of non-anchor RB sets within this SL BWP. | SEQUENCE (SIZE (1…NonAnchorRBsetsNum)) OF ARFCN-ValueNR | N/A | Per SL BWP |
| 21 | sl-StartRBsetCG-Type1 | Indicates starting RB set index of the initial PSSCH transmission of the sidelink configured grant Type 1 for interlace RB-based PSSCH transmission Note: M is the number of RB sets within this [SL resource pool]. | INTEGER (0,1,2,...,M-1) | N/A | [Per Configured Grant] |

Agreement

Support:

- Alt A: R17 SL inter-UE coordination Scheme 2 (conflict indication) at least for sl-PSFCH-Occasion = '0' uses the same transmission scheme (Alt 1-1b and Alt 2-3a) as HARQ-ACK in R18 SL-U

  - For Alt 1-1b,

    - Alt A2: Common interlace index for conflict indication and HARQ-ACK within the same RB set are the same

  - Note: Alt 1-1b and Alt 2-3a in previous agreements are as below

    - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

Agreement

TP#3-2 in Section 4.9.1 of R1-2310355 is endorsed for TS 38.213 clause 16.4.

Agreement

Adopt following red change to RAN1#114’s agreement:

Agreement In “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)”, regarding Rx UE behaviour on receiving PSFCH for a PSCCH/PSSCH transmission, support: - For unicast: - FFS: Monitor: - Rx UE attempts to monitor candidate PSFCH occasion(s) until one PSFCH is detected or all candidate PSFCH occasion(s) are monitored. - If one PSFCH is detected, Rx UE can omit monitoring following candidate PSFCH occasion(s). - Report: - If Rx UE receives PSFCH, Rx UE reports same value as a value of HARQ-ACK information that the UE determines from the PSFCH reception to higher layers, otherwise re-ports NACK to higher layer. - FFS: For groupcast option 1 (NACK only): - FFS: Monitor: - Rx UE attempts to monitor all candidate PSFCH occasions. - If NACK is detected, Rx UE can omit monitoring following candidate PSFCH occasion(s). - Report: - If Rx UE does not detect any PSFCH in all candidate PSFCH occasions, Rx UE reports ACK to higher layers; otherwise, reports NACK to higher layers. - For groupcast option 2 (ACK/NACK): - FFS: Monitor: - Rx UE attempts to monitor PSFCH transmission occasions until PSFCH from all transmitters have been detected or all candidate PSFCH occasions are monitored. - If Rx UE detects PSFCH from one PSFCH transmitter, it can omit PSFCH detection for following PSFCH transmission occasions for this PSFCH transmitter. - Report: - If ACK has been detected from at least one PSFCH occasion of each of all expected PSSCH receivers, Rx UE reports ACK to higher layers; otherwise, reports NACK to higher layers.

Agreement

Adopt following red change to RAN1#114’s agreement:

Agreement Regarding “UE may transmit S-SSB repetition in more than one RB set”: - At least the power for S-SSB transmission on anchor RB set does not change due to the number of used RB sets - On anchor RB set, there is a (pre-)configured offset Poffset_anchor to limit the maximum power as below (changes to legacy NR SL is marked in red) - PS-SSB_anchor(i)=minPCMAX-Poffset_anchor,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm], where i is slot index as in legacy - value range of Poffset_anchor is: {10lg(N), [10lg(N)+2, 10lg(N)+4, …],10lg⁡(W)} - On non-anchor RB set - UE first allocates power to S-SSB repetitions on anchor RB set, assume the power of each S-SSB repetition is PS-SSB_anchor - Then, UE allocates remaining power Pleft equally to other S-SSB repetitions on all other used RB sets, where Pleft=PCMAX-N*PS-SSBanchor, where PCMAX and PS-SSB_anchor are converted to linear unit (i.e, Watt) in this formula - Note: for both anchor RB set and non-anchor RB set transmission, the same DL pathloss is taken into account - M is the total number of RB sets within this SL-BWP, N is the number of S-SSB repetitions within the anchor RB set, W is the maximum total number of S-SSB repetitions on RB sets within the SL-BWP - Note: the above power for S-SSB transmission refers to power of one S-SSB repetition - UE at least attempts to transmit on anchor RB set - Note: anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates - For above Alts, PCMAX is determined according to TS 38.101-1 for transmission of all S-SSB repetitions on all used RB sets

Agreement

In “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)”, regarding “the minimum time gap Z=a+b between any two selected resources of a TB in case PSFCH is configured for this resource pool”:

- Alt 2: Z = a’ + b, where

  - For unlicensed operation, a’ is defined as: a’ is the time gap between the end of the last symbol of a PSSCH transmission of the first resource and the start of the first symbol of the last corresponding PSFCH reception determined by sl-MinTimeGapPSFCH and sl-PSFCH-Period for the pool of resources; and

  - b remains the same as legacy NR SL

- Note: the meaning of a and b in legacy NR SL is

  - a is the time gap between the end of the last symbol of a PSSCH transmission of the first resource and the start of the first symbol of the corresponding PSFCH reception determined by sl-MinTimeGapPSFCH and sl-PSFCH-Period for the pool of resources

  - b is the time required for PSFCH reception and processing plus sidelink retransmission preparation including multiplexing of necessary physical channels and any TX-RX/RX-TX switching time.

Agreement

In “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)”, regarding “the reference slot n for PUCCH transmission to report HARQ in Mode 1”:

- slot n is updated as the slot containing the last candidate PSFCH occasion

  - FFS: whether the specifications are already aligned with this