3GPP TSG RAN WG1 #115	R1- 2312652

Chicago, USA, November 13th – 17th, 2023

Agenda Item:	8.3.5

Source:	Ericsson

Title:	Feature Lead summary #5 for Positioning for RedCap UEs

Document for:	Discussion, Decision

# Introduction

This document summarizes the proposals received as part of Agenda Item 8.3.5 for RAN1#115 for the Rel-18 work item on expanded and improved NR positioning [1]. The objectives relevant for this agenda item are as follow:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

# [HIGH] Text Proposals

## Summary of TPs

Several contributions provided text proposals related to previous agreement already captured in specifications. For these proposals it is possible to discuss directly the TPs.

| Source | Proposal | FL Summary |
| --- | --- | --- |
| [2] | P3 | Clarification of the collision rules for the case where the time between hop allows to go back to active BWP |
| [2] | P9/P10 | Use of SRS-po with FH with non-redcap UEs. Proposal 9: The features of PRS Rx hopping and SRS Tx hopping (including the feature of transmitting SRS outside the active UL BWP) should also be applicable to non-RedCap UEs, and Rel-18 eRedCap UEs. |
| [3] | P6 | Simplification of the equation for mapping to physical resources for the case of tx hopping |
| [5] | TP1 for 211 | Clarification that the hopping parameter N_hop is only applicable to the SRS for positioning. |
| [5] | TP2 for 211 | Further clarification to the configuration of tx hopping including use of c_srs, hop index and n0 |
| [5] | TP3 for 211 | Clarification of slot configuration for SRS with tx hopping |
| [5] | TP1 for 214 | Clarification of “cycle” in 38.214 |
| [5] | TP2 for 214 P5 | Collision with the mimo and positioning SRS with tx hopping |
| [5] | P5 | SRS with tx hopping power control |
| [6] | P6 |  |
| [6] | P7 |  |
| [7] | P2 | Parameters for the SRS with tx hopping not listed in 38.214 |
| [11] | P3 | SRS Hopping across slot boundary is not allowed. |
| [11] | P5 | Collision rules with UL signals |
| [11] | P7 | Clarification of “cycle” in 38.214 |
| [11] | P10 | Corrects a typo in 38.211 |
| [12] | P6 | SRS Hopping across slot boundary is not allowed. |
| [14] | P3 | Same TP as [7] P2 |
| [20] | TP1 | Collision rules with PUSCH and PUCCH and SRS with tx hopping |
| [20] | TP2 | Collision rules for A-SRS with positioning and tx hopping |
| [20] | TP3 | Collision rules with PRACH and SRS with tx hopping |
| [20] | TP4 | SRS with tx hopping power control |
| [20] | TP5 | UTW configuration per serving cell |

## UE behaviour between SRS with tx hopping hops

### Text proposal

| TP 2.1-1 |  |
| --- | --- |
| reason for change: | The UE behaviour between hops in SRS with tx hopping is not clear as to whether it should return to the active BWP if needed. |
| summary of change: |  |
| Consequences if not approved: |  |
| ------------ Start of Text Proposal for TS 38.214 clause 6.2.1.4.1 ------------ < Unchanged parts are omitted > ##### 6.2.1.4.1 SRS frequency hopping for positioning The reduced capability UE may be configured via [higher layer parameter], subject to UE capability, to perform transmit frequency hopping separate from the UL BWP configuration and outside of the UL BWP, where the UE may be configured with subcarrier spacing, CP and bandwidth that are different from the UL active BWP. The reduced capability UE transmit frequency hopping is configured within one SRS resource for positioning, that may be configured with a bandwidth larger than the maximum bandwidth of the reduced capability UE, in RRC_CONNECTED or RRC_INACTIVE mode. The reduced capability UE transmit frequency hopping, may be configured with overlapping or non-overlapping frequency hops in the frequency domain. The reduced capability UE may be configured, via [higher layer parameter], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is not expected to be configured with one [cycle] of the transmit frequency hopping that is partially overlapped with the time window. If the SRS symbol(s), including the switching time to or from the active bandwidth part, of the transmit frequency hopping collides with the transmission of PUSCH or PUCCH or the reception of DL signals and channels, and if the UE determines the SRS to be dropped, the colliding SRS symbol(s) are dropped. If the [time offset] between two consecutive hops exceeds the sum of the switching time to and from the active BWP, the UE is expected to switch back to the active BWP after UE transmits each hop. < Unchanged parts are omitted > ---------------------------- End of Text Proposal for TS 38.214 ---------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TP in the table below

TP 2.1-1:

| Company | Comment |
| --- | --- |
| ZTE | For the last paragraph, even if there is no any other scheduling in the active BWP, does UE have to switch back? What is the benefit? Also, what’s the relationship between this proposal and the one in section 5.3.1 ? |
| Ericsson | Ok to capture it once we have the agreement in 5.3.1. |

## Support of Tx hopping for non-redcap UEs

### Text proposal

| TP 2.2-1 |  |
| --- | --- |
| reason for change: | The features of PRS Rx hopping and SRS Tx hopping (including the feature of transmitting SRS outside the active UL BWP) should also be applicable to non-RedCap UEs, and Rel-18 eRedCap UEs. |
| summary of change: | Add supports to non-redcap UEs in the SRS frequency hopping for positioning sections |
| Consequences if not approved: | SRS with tx hopping for positioning is limited to redcap UEs. |
| ------------ Start of Text Proposal for TS 38.214 ------------ < Unchanged parts are omitted > ##### 5.1.6.5.1 PRS receiver frequency hopping The UE may be configured to measure and report, subject to UE capability, via [higher layer parameter] the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, or UE Rx-Tx time difference using receiver frequency hopping for a DL PRS resource, with a requested bandwidth of all hops that may be greater than the maximum UE channel bandwidth. The UE performing receiver frequency hopping may report via [higher layer parameter] one measurement associated with one received frequency hop or one measurement based on multiple hops of the DL PRS. [In RRC_CONNECTED mode], the UE is expected to use a single instance of a configured measurement gap to receive all hops of the DL PRS using receiver frequency hopping. < Unchanged parts are omitted > ##### 6.2.1.4.1 SRS frequency hopping for positioning The UE may be configured via [higher layer parameter], subject to UE capability, to perform transmit frequency hopping separate from the UL BWP configuration and outside of the UL BWP, where the UE may be configured with subcarrier spacing, CP and bandwidth that are different from the UL active BWP. The UE transmit frequency hopping is configured within one SRS resource for positioning, that may be configured with a bandwidth larger than the maximum bandwidth of the UE, in RRC_CONNECTED or RRC_INACTIVE mode. The UE transmit frequency hopping, may be configured with overlapping or non-overlapping frequency hops in the frequency domain. The UE may be configured, via [higher layer parameter], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is not expected to be configured with one [cycle] of the transmit frequency hopping that is partially overlapped with the time window. If the SRS symbol(s), including the switching time to or from the active bandwidth part, of the transmit frequency hopping collides with PUSCH or PUCCH including the switching time to or from the active bandwidth part, and if the UE determines the SRS to be dropped, the colliding SRS symbol(s) are dropped. < Unchanged parts are omitted > ---------------------------- End of Text Proposal for TS 38.214 ---------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TP in the table below

TP 2.2-1:

| Company | Comment |
| --- | --- |
| vivo | Out of scope, based on the WID, this feature is only for RedCap UEs. |
| Huawei, HiSilicon | Support. There could be multiple feature extended beyond original case, like NR-U features extended to licensed spectrum, RedCap features extended to non-RedCap UEs. |
| ZTE | OK |
| New H3C | OK |
| Ericsson | OK |

## Frequency hopping text in 38.211

### Text proposal

| TP 2.3-1 |  |
| --- | --- |
| reason for change: | In the current TS 38.211, the new offset for Tx frequency hopping denoted by noffset2FH is added to k0(pi)such that k0(pi)=k0(pi)+noffsetFH+noffsetRPFS+noffset2FH, where noffset2FH=n0+nSRSTxHopping mod Nhop-n0mSRS,0-moverlaphopNscRB. The equation might have already included n_shift of Alt.2 into the offset, however the n_shift is still in k0(pi), such that k0(pi)=nshiftNscRB+kTC(pi)+koffsetl'+fcoh(nf,ns,fμ,l') mod KTC, given by Clause 6.4.1.4.3. |
| summary of change: | The frequency domain position equation is broken in two cases, when tx hopping is supported and when it is not supported. |
| Consequences if not approved: | 38.211 specification for tx hopping is not clear. |
| ---------------------------- Start of Text Proposal for TS 38.211 clause 6.2.4.1 ----------------------------- < Unchanged parts are omitted > The frequency-domain starting position k0(pi) is defined by k0(pi)=k0(pi)+noffsetFH+noffsetRPFS+noffset2FH If the higher layer parameter SRShoppingNrofHops is configured, k0(pi)=nFirstHopRB-n0idxmSRS,0-moverlaphopNscRB+kTC(pi)+koffsetl', noffset2FH=n0idx+nSRS mod NhopmSRS,0-moverlaphopNscRB, noffsetFH=0, noffsetRPFS=0, Otherwise, k0(pi)=nshiftNscRB+kTC(pi)+koffsetl'+fcoh(nf,ns,fμ,l') mod KTC noffset2FH=0 Where kTCpi=kTC+3KTC4 mod KTCif NapSRS=8, pi∈1003, 1007, and nSRScs,max=6kTC+KTC2 mod KTCif NapSRS=8, pi∈1002, 1006, and nSRScs,max=6kTC+KTC4 mod KTCif NapSRS=8, pi∈1001, 1005, and nSRScs,max=6kTC+KTC2 mod KTCif NapSRS=8, pi∈1001, 1003, 1005, 1007, and nSRScs,max=12kTC+KTC2 mod KTCif NapSRS=8, pi∈1001, 1003, 1005, 1007, nSRScs,max=8, and nSRScs≥nSRScs,max2kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, nSRScs,max∈{8, 12},and nSRScs≥nSRScs,max2kTCotherwise n0idx is the initial hop index < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ---------------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TP in the table below

TP 2.3-1:

| Company | Comment |
| --- | --- |
| vivo | Only support to change ‘n0’ to ‘n0idx’ to distinguish with ‘nb’ MIMO SRS. For other changes, we prefer to illustrate the n_shift as section 2.6, other than to introduce the two equations. The frequency domain shift value nshift adjusts the SRS allocation with respect to the reference point grid and is contained in the higher-layer parameter freqDomainShift in the SRS-Resource IE or the SRS-PosResource IE. If SRShoppingNrofHops for SRS-PosResource is provided, the frequency domain shift value nshift represents the starting RB of the first hop in time domain. The transmission comb offset kTC∈0,1,…,KTC-1 is contained in the higher-layer parameter transmissionComb in the SRS-Resource IE or the SRS-PosResource IE and nb is a frequency position index.. |
| ZTE | Do not support. We do not think n_shift is counted twice since the value of n0 has been subtracted when calculating noffset2FH. In current agreed CR, n_shift is the starting PRB of the first hop in time domain and n0 is the frequency hop index of the first hop in time domain. |
| Futurewei | RAN1 have yet to agree on the following: n0idx is the initial hop index The TP is pending the outcome of discussion in 5.1.1. |
| Nokia/NSB | We think at least n0 should be fixed. |
| Ericsson | Ok with updating n0 to something clearer. The other changes are not critical. |

## [closed] Clarification for tx hopping in 38.211

### Text proposal

| TP 2.4-1 |  |
| --- | --- |
| reason for change: | It needs to be distinguished that frequqency hopping here is for SRS for positioning, or MIMO SRS, which improves specification readability and reduce ambiguity. |
| summary of change: | Section 6.4.1.4.1 in TS 38.211 1.Add the description that ‘ Nhop is for SRS for postioning’. 2. Typo fixed. |
| Consequences if not approved: | SRS for postioning frequency hopping and MIMO SRS frequency hopping may be confused by the reader. |
| ---------------------------- Start of Text Proposal for TS 38.211 ------------------ < Unchanged text omitted > ##### 6.4.1.4.1 SRS resource An SRS resource is configured by the SRS-Resource IE or the SRS-PosResource IE and consists of - NapSRS∈1,2,4,8 antenna ports pii=0NapSRS-1, where the number of antenna ports is given by the higher layer parameter nrofSRS-Ports if configured, otherwise NapSRS=1, and pi=1000+i when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet not set to 'nonCodebook', or determined according to [6, TS 38.214] when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet set to 'nonCodebook' - Nhop, the number of hops for SRS Tx hopping for an SRS resource configured by SRS-PosResource given by the higher layer parameter SRShoppingNrofHops if configured, otherwise Nhop=1. - NsymbSRS∈1,2,4,8,10,12,14 consecutive OFDM symbols given by the field nrofSymbols contained in the higher layer parameter resourceMapping. If Nhop>1, NsymbSRS is the number of consecutive OFDM symbol per hops. - l0, the starting position in the time domain given by l0=Nsymbslot-1-loffset where the offset loffset∈0,1,…,13 counts symbols backwards from the end of the slot and is given by the field startPosition contained in the higher layer parameter resourceMapping and loffset≥NsymbSRS-1. If Nhop>1 l0 is the starting position of each hop in the time domain, determined by the field startPosition for each SRS transmission hop. - k0, the frequency-domain starting position of the sounding reference signal < Unchanged text omitted > < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ---------------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TP in the table below

TP 2.4-1:

| Company | Comment |
| --- | --- |
| Vivo | Support |
| InterDigital | Support |
| ZTE | Support |
| CATT | OK with the change for clarification. |
| Nokia/NSB | OK |
| New H3C | OK |

### Conclusion for RAN1#115

The following was agreed :

Agreement Endorse the TP 2.4-1 in section 2.5.1 of R1-2312343

## Clarifications for tx hopping in 38.211

### Text proposal

- FL Note: the changes on the use of a parameter for n_0 in this text proposal need to be discussed separately.

| TP 2.5-1 |  |
| --- | --- |
| reason for change: | 1.The specification lacks the description of C_SRS of the previous agreement. - For the hop bandwidth common to all hops - Configuration re-uses C_SRS - The values of C_SRS in legacy SRS for positioning such that the maximum bandwidth is: 104 PRBs, 48 PRBs, 132 PRBs, 64 PRBs, for 15,30,60,120 KHz respectively when B_SRS equal 0. 2. Based on current description, it is still not clear how to get nSRSTxHopping , e.g., how to get the relationship between and each hop. A straightforward way is to configure hop time index together with the starting slot offset and starting symbol for each hop. In addition, change the definition of ‘nSRSTxHopping’ to ‘hop index counter in time domain’ to distinguish from the hop index represented by n0. 3. The specification lacks the description of where n0 comes from. 4. The specification lacks the description of nshift which represents the starting RB of the first hop in time domain, especially considering corresponding description is deleted in TS38.214. When the reduced capability UE is configured to perform transmit frequency hopping it expects to be configured via [higher layer parameter] with the starting PRB of the first frequency hop. |
| summary of change: | Section 6.4.1.4.3 in TS 38.211 1.Add the description of C_SRS following the previous agreement. 2.Add the description of nSRSTxHopping based on higher-layer configuration. And change the definition of ‘nSRSTxHopping’ to ‘hop index counter in time domain’. 3.Add the description of n0 based on higher-layer configuration. 4.Add the description of nshift represents the starting RB of the first hop in time domain. |
| Consequences if not approved: | Incomplete descriptions for SRS for postioning frequency hopping. |
| ---------------------------- Start of Text Proposal for TS 38.211 ------------------ < Unchanged text omitted > ##### 6.4.1.4.3 Mapping to physical resources Throughout this clause, when the higher layer parameter SRShoppingNrofHops is provided for SRS-PosResource, the sounding reference signal sequence definitions applies to a given hop. When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol l' and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with to resource elements in a slot for each of the antenna ports according to aKTCk'+k0pi, l'+l0(pi)=1NapβSRSrpi(k',l')if k'=0, 1, …, Msc,bSRS-1 and l'=0,1,…,NsymbSRS-10otherwise The length of the sounding reference signal sequence is given by Msc,bSRS=mSRS,bNscRBKTCPF where mSRS,b is given by a selected row of Table 6.4.1.4.3-1 with where is given by the field b-SRS contained in the higher-layer parameter freqHopping if configured, otherwise BSRS=0. The row of the table is selected according to the index given by the field c-SRS contained in the higher-layer parameter freqHopping. If SRShoppingNrofHops for SRS-PosResource is provided, the row of the table is selected according to the index CSRS with the limitation of the maximum bandwidth of104 PRBs, 48 PRBs, 132 PRBs, 64 PRBs, for 15,30,60,120 KHz respectively when BSRS=0. The quantity PF ∈2, 4 is given by the higher-layer parameter FreqScalingFactor if configured, otherwise PF=1. When FreqScalingFactor is configured, the UE expects the length of the SRS sequence to be a multiple of 6. The frequency-domain starting position k0(pi) is defined by k0(pi)=k0(pi)+noffsetFH+noffsetRPFS+noffset2FH where k0(pi)=nshiftNscRB+kTC(pi)+koffsetl'+fcoh(nf,ns,fμ,l') mod KTC and kTCpi=kTC+3KTC4 mod KTCif NapSRS=8, pi∈1003, 1007, and nSRScs,max=6kTC+KTC2 mod KTCif NapSRS=8, pi∈1002, 1006, and nSRScs,max=6kTC+KTC4 mod KTCif NapSRS=8, pi∈1001, 1005, and nSRScs,max=6kTC+KTC2 mod KTCif NapSRS=8, pi∈1001, 1003, 1005, 1007, and nSRScs,max=12kTC+KTC2 mod KTCif NapSRS=8, pi∈1001, 1003, 1005, 1007, nSRScs,max=8, and nSRScs≥nSRScs,max2kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, nSRScs,max∈{8, 12},and nSRScs≥nSRScs,max2kTCotherwise noffsetFH=b=0BSRSmSRS,bNscRBnb noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF noffset2FH=n0+nSRSTxHopping mod Nhop-n0mSRS,0-moverlaphopNscRB and - kF∈0,1,…,PF-1 is given by the higher-layer parameter StartRBIndex if configured, otherwise kF=0; - khop is given by Table 6.4.1.4.3-3 with khop=nSRSb'=bhopBSRSNb' mod PF Nbhop=1 if the higher-layer parameter EnableStartRBHopping is configured, otherwise khop=0. - moverlaphop∈0,1,2,4 is given by the higher-layer parameter YYY - nSRSTxHopping=0,1,…,NhopsSRS-1 is the hop index counter in time domain, given by the higher-layer parameter [ZZ]. - n0 is the initial hop index, given by the higher-layer parameter [ZZZ] The quantity fcohnf,ns,fμ,l' is given by - if the higher-layer parameter combOffsetHopping is not configured: fcohnf, ns,fμ,l'=0 - if the higher-layer parameter combOffsetHopping is configured: fcohnf ,ns,fμ,l'= scohSRSm=07c8nf mod 128Nslotframe,μNsymbslot+ns,fμNsymbslot+l0+l''+m2mmod ncohSRS where scohSRSn and ncohSRS is the nth entry and the cardinality of the set Scoh={scohSRS0, scohSRS1, …,scohSRSncohSRS-1} respectively, where Scoh is given by the higher-layer parameter combOffsetHoppingSubset if configured, otherwise Scoh={0, 1,…,KTC-1}. The pseudo-random sequence ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDcoh at the beginning of each radio frame for which nf mod 128=0, where the comb hopping identity nIDcoh is contained in the higher-layer parameter combOffsetHopping. If the higher-layer parameter hoppingWithRepetition is configured, l''=l'RR, otherwise l''=l'. If SRShoppingNrofHops is configured: - The reference point for k0(pi)=0 is the lowest subcarrier of the configured bandwidth for SRS with Tx hopping configured by the parameter XXX in TxhoppingBandwidth. otherwise: - If NBWPstart≤nshift the reference point for k0(pi)=0 is subcarrier 0 in common resource block 0, otherwise the reference point is the lowest subcarrier of the BWP. If the SRS is configured by the IE SRS-PosResource, the quantity koffsetl' is given by Table 6.4.1.4.3-2, otherwise koffsetl'=0. The frequency domain shift value nshift adjusts the SRS allocation with respect to the reference point grid and is contained in the higher-layer parameter freqDomainShift in the SRS-Resource IE or the SRS-PosResource IE. If SRShoppingNrofHops for SRS-PosResource is provided, the frequency domain shift value nshift represents the starting RB of the first hop in time domain. The transmission comb offset kTC∈0,1,…,KTC-1 is contained in the higher-layer parameter transmissionComb in the SRS-Resource IE or the SRS-PosResource IE and nb is a frequency position index. Frequency hopping of the sounding reference signal is configured by the parameter bhop∈0,1,2,3, given by the field b-hop contained in the higher-layer parameter freqHopping if configured, otherwise bhop=0. If bhop≥BSRS, frequency hopping is disabled and the frequency position index nb remains constant (unless re-configured) and is defined by for all NsymbSRS OFDM symbols of the SRS resource. The quantity is given by the higher-layer parameter freqDomainPosition if configured, otherwise nRRC=0, and the values of mSRS,b and Nb for b=BSRS are given by the selected row of Table 6.4.1.4.3-1 corresponding to the configured value of . If bhop<BSRS, frequency hopping is enabled and the frequency position indices nb are defined by nb=4nRRCmSRS,b mod Nbb≤bhopFbnSRS+4nRRCmSRS,b mod Nbotherwise where Nb is given by Table 6.4.1.4.3-1, and where Nbhop=1 regardless of the value of Nb. The quantity nSRS counts the number of SRS transmissions. For the case of an SRS resource configured as aperiodic by the higher-layer parameter resourceType, it is given by nSRS=l'sR within the slot in which the NsymbSRS symbol SRS resource is transmitted. The quantity s is given by s=2 if the higher-layer parameter nrofSRS-Ports-n8 equals ‘ports8tdm’, otherwise s=1. The quantity R≤NsymbSRS is the repetition factor given by the field repetitionFactor if configured, otherwise R=NsymbSRS. For the case of an SRS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, the SRS counter is given by nSRS=Nslotframe,μnf+ns,fμ-ToffsetTSRSNsymbSRSsR+l'sR for slots that satisfy . The periodicity in slots and slot offset are given in clause 6.4.1.4.4. < Unchanged text omitted > --------------------------------------- End of Text Proposal ---------------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TP in the table below

TP 2.5-1:

| Company | Comment |
| --- | --- |
| vivo | OK |
| ZTE | The following change should not be agreed for now, the two options in previous agreement should be decided first. n0 is the initial hop index, given by the higher-layer parameter [ZZZ] |
| CATT | Since we say nSRSTxHopping=0,1,…,NhopsSRS-1 is the hop index counter in time domain, we prefer that we can add similar descriptions for n0, which should be the initial hop index in frequency domain. |
| Futurewei | The TP is pending the outcome of discussion in 5.1.1. RAN1 have yet to agree on the following: n0 is the initial hop index, given by the higher-layer parameter [ZZZ] |
| Nokia/NSB | OK |
| Ericsson | This is not a critical correction in our view. |

## [closed] Slot configuration for SRS with tx hopping

### Text proposal

| TP 2.6-1 |  |
| --- | --- |
| reason for change: | The specification lacks the description of SRS slot configuration for each hop. The starting position of each hop can be configured independently and does not depend on the relative relationship with the first hop. When reflecting the SRS slot configurtation in TS38.214, we only need to follow the existing formula without generating a new formula. |
| summary of change: | Section 6.4.1.4.4 in TS 38.211 Add description of SRS slot configuration for each hop. |
| Consequences if not approved: | Incomplete descriptions of SRS slot configuration for each hop. |
| ##### ------------ Start of Text Proposal for TS 38.211 ----------- ##### 6.4.1.4.4 Sounding reference signal slot configuration Throughout this clause, when the higher layer parameter SRShoppingNrofHops is provided for SRS-PosResource, the sounding reference signal slot configuration applies to a given hop. For an SRS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, a periodicity (in slots) and slot offset are configured according to the higher-layer parameter periodicityAndOffset-p or periodicityAndOffset-sp in the SRS-Resource IE, or periodicityAndOffset-p or periodicityAndOffset-sp in the SRS-PosResource IE. Candidate slots in which the configured SRS resource may be used for SRS transmission are the slots satisfying SRS is transmitted as described in clause 6.2.1 of [6, TS 38.214]. < Unchanged parts are omitted > ---------------------------- End of Text Proposal for TS 38.211 ---------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TP in the table below

TP 2.6-1:

| Company | Comment |
| --- | --- |
| vivo | OK |
| ZTE | OK |
| New H3C | OK |

### Conclusion for RAN1#115

The following was agreed :

Agreement Endorse the TP 2.6-1 in section 2.7.1 of R1-2312343

## Clarifications of [cycle] in 38.214

### Text proposal

| TP 2.7-1a |  |
| --- | --- |
| reason for change: | It is not clear what the [cycle] means since there is no such definition in the specification. |
| summary of change: | Section 6.2.1.4.1 in TS 38.214: Add a clear decription to represent the meaning of ‘one [cycle]’ of Tx frequency hopping. |
| Consequences if not approved: | Unclear description of ‘one [cycle]’ of Tx frequency hopping. |
| ##### ------------ Start of Text Proposal for TS 38.214 ----------- ##### 6.2.1.4.1 SRS frequency hopping for positioning The reduced capability UE may be configured via [higher layer parameter], subject to UE capability, to perform transmit frequency hopping separate from the UL BWP configuration and outside of the UL BWP, where the UE may be configured with subcarrier spacing, CP and bandwidth that are different from the UL active BWP. The reduced capability UE transmit frequency hopping is configured within one SRS resource for positioning, that may be configured with a bandwidth larger than the maximum bandwidth of the reduced capability UE, in RRC_CONNECTED or RRC_INACTIVE mode. The reduced capability UE transmit frequency hopping, may be configured with overlapping or non-overlapping frequency hops in the frequency domain. The reduced capability UE may be configured, via [higher layer parameter], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is not expected to be configured with one [cycle] Nhop hops given by higher layer parameter [XX]of the transmit frequency hopping that is partially overlapped with the time window. If the SRS symbol(s), including the switching time to or from the active bandwidth part, of the transmit frequency hopping collides with PUSCH or PUCCH including the switching time to or from the active bandwidth part, and if the UE determines the SRS to be dropped, the colliding SRS symbol(s) are dropped. ---------------------------- End of Text Proposal for TS 38.214 ---------------------------- |  |

| TP 2.7-1b |  |
| --- | --- |
| reason for change: |  |
| summary of change: |  |
| Consequences if not approved: |  |
| ##### ------------ Start of Text Proposal for TS 38.214 ----------- ##### 6.2.1.4.1 SRS frequency hopping for positioning <omitted text> The reduced capability UE may be configured, via [higher layer parameter], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping, where the window is not partially overlapped with the transmit frequency hopping of an SRS resource. The UE is not expected to be configured with one [cycle] of the transmit frequency hopping that is partially overlapped with the time window. <omitted text> ---------------------------- End of Text Proposal for TS 38.214 ---------------------------- |  |

| TP 2.7-1c |  |
| --- | --- |
| reason for change: | [cycle] is put within square brackets in the spec, but based on RAN1’s agreements, it is clear that one cycle contains all hops and UE is not expected to be configured with a SRS for positioning hopping cycle partially overlapping with UTW. |
| summary of change: | remove the square brackets of [cycle] |
| Consequences if not approved: | unstable spec |
| ##### ------------ Start of Text Proposal for TS 38.214 ----------- ##### 6.2.1.4.1 SRS frequency hopping for positioning The reduced capability UE may be configured via [higher layer parameter], subject to UE capability, to perform transmit frequency hopping separate from the UL BWP configuration and outside of the UL BWP, where the UE may be configured with subcarrier spacing, CP and bandwidth that are different from the UL active BWP. The reduced capability UE transmit frequency hopping is configured within one SRS resource for positioning, that may be configured with a bandwidth larger than the maximum bandwidth of the reduced capability UE, in RRC_CONNECTED or RRC_INACTIVE mode. The reduced capability UE transmit frequency hopping, may be configured with overlapping or non-overlapping frequency hops in the frequency domain. The reduced capability UE may be configured, via [higher layer parameter], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is not expected to be configured with one cycle of the transmit frequency hopping that is partially overlapped with the time window. ##### If the SRS symbol(s), including the switching time to or from the active bandwidth part, of the transmit frequency hopping collides with PUSCH or PUCCH including the switching time to or from the active bandwidth part, and if the UE determines the SRS to be dropped, the colliding SRS symbol(s) are dropped. ---------------------------- End of Text Proposal for TS 38.214 ---------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TPs in the table below, including which TP should be selected.

TP 2.7-1 (a or b or c):

| Company | Comment |
| --- | --- |
| Vivo | Support TP 2.7-1a. |
| InterDigital | Support TP 2.7-1a with the following modification for clarificadtion. The UE is not expected to be configured with one [cycle] Nhop hops given by higher layer parameter [XX]of the transmit frequency hopping that are partially overlapped with the time window. |
| ZTE | We are ok with either TP 2.7-1a or TP 2.7-1c. |
| CATT | We prefer TP 2.7-1a, which is clear and simple solution for the issue of [cycle]. |
| Futurewei | TP 2.7-1c |
| NewH3C | OK |

### Round 2

During the online session on Wednesday, some issues with TP 2.7.1a were highlighted.

- Is the whole sequence of hops supposed to fit in one single occasion of the UTW, or the is it enough that hops and UTW can be configured such that different hops can be fitted in different occasion of the UTW

From the FL perspective, it should be sufficient to explain that a transmission SRS resource with tx positioning cannot be both in and out of the UTW. This will ensure the UE can expect the same level of prioritization for a whole transmission. the case of using one more multiple UTW can be resolved by the network implementation, following the UE capabilities.

My proposal is thus to reword the TP as follow:

TP 2.7-2

##### ------------ Start of Text Proposal for TS 38.214 ----------- ##### 6.2.1.4.1 SRS frequency hopping for positioning The reduced capability UE may be configured via [higher layer parameter], subject to UE capability, to perform transmit frequency hopping separate from the UL BWP configuration and outside of the UL BWP, where the UE may be configured with subcarrier spacing, CP and bandwidth that are different from the UL active BWP. The reduced capability UE transmit frequency hopping is configured within one SRS resource for positioning, that may be configured with a bandwidth larger than the maximum bandwidth of the reduced capability UE, in RRC_CONNECTED or RRC_INACTIVE mode. The reduced capability UE transmit frequency hopping, may be configured with overlapping or non-overlapping frequency hops in the frequency domain. The reduced capability UE may be configured, via [higher layer parameter], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is not expected to be configured to transmit the SRS with transmit frequency hopping so that symbols from the same SRS with transmit frequency hopping resource can be inside or outside of the UL time window. If the SRS symbol(s), including the switching time to or from the active bandwidth part, of the transmit frequency hopping collides with PUSCH or PUCCH including the switching time to or from the active bandwidth part, and if the UE determines the SRS to be dropped, the colliding SRS symbol(s) are dropped. ---------------------------- End of Text Proposal for TS 38.214 ----------------------------

TP 2.7-2:

| Company | Comment |
| --- | --- |

Conclusion

Comb offset hopping is not supported for positioning SRS with frequency hopping for RedCap UEs.

## [closed]Collision of SRS with tx hopping with the mimo SRS

### Text proposal

| TP 2.8-1b |  |
| --- | --- |
| reason for change: | The description of collision between SRS for positioning frequency hopping and MIMO SRS is missing. |
| summary of change: | Section 6.2.1.4.1 in TS 38.214: Add the description of collision between SRS for positioning frequency hopping and MIMO SRS. |
| Consequences if not approved: | Incomplete description of collision between SRS for positioning frequency hopping and MIMO SRS. |
| < Unchanged text omitted > For operation in the same carrier, the reduced capability UE is not expected to be configured on overlapping symbols with a SRS resource of the transmit frequency hopping configured by the higher layer parameter [XX] including the switching time to or from the active bandwidth part and a SRS resource with resourceType of both SRS resources as 'periodic'. < Unchanged text omitted > For operation in the same carrier, the reduced capability UE is not expected to be activated or triggered to transmit SRS on overlapping symbols with a SRS resource of the transmit frequency hopping configured by the higher layer parameter [XX] including the switching time to or from the active bandwidth part and a SRS resource with resourceType of both SRS resources as 'semi-persistent' or 'aperiodic'. < Unchanged text omitted > |  |

### Round 1

Companies are encouraged to comment on the proposed TP in the table below

TP 2.8-1:

| Company | Comment |
| --- | --- |
| vivo | Support. For MIMO SRS and Pos SRS, UE is not expected to be configured them on overlapping symbols, and UE doesn’t drop MIMO SRS or Pos SRS. We can reuse similar rules for MIMO SRS and Pos SRS frequency hopping and additional consider switching time. |
| ZTE | It is better to discuss the proposal in section 5.3 first, and then we can discuss the TP. |
| CATT | OK with the TP. |
| Ericsson | OK |

## [closed] Parameters for the SRS for positioning with tx hopping

### Text proposal

| TP 2.9-1 |  |
| --- | --- |
| reason for change: | All SRS frequency hopping related configuration parameters need to be captured in 6.2.1.4 in 38.214. |
| summary of change: | Section 6.2.1.4 in 38.214: Clarify SRS frequency hopping related configuration parameters. |
| Consequences if not approved: | SRS frequency hopping configuration is not clear in 38.214. |
| ##### ------------ Start of Text Proposal for TS 38.214 ----------- The reduced capability UE may be configured via [higher layer parameter], subject to UE capability, to perform transmit frequency hopping separate from the UL BWP configuration and outside of the UL BWP, where the UE may be configured with subcarrier spacing, CP and bandwidth that are different from the UL active BWP. The reduced capability UE transmit frequency hopping is configured within one SRS resource for positioning, that may be configured with a bandwidth larger than the maximum bandwidth of the reduced capability UE, in RRC_CONNECTED or RRC_INACTIVE mode. The reduced capability UE transmit frequency hopping, may be configured with overlapping or non-overlapping frequency hops in the frequency domain. When the reduced capability UE is configured to perform transmit frequency hopping it expects to be provided with the following parameters:. - The starting PRB of the first frequency hop in [higher layer parameter] - The starting slot offset and starting symbol for each hop in [higher layer parameter] - The number of symbols in each hops in [higher layer parameter] - The hop bandwidth in [higher layer parameter] - The overlap between hops, if present, in [higher layer parameter] - The number of hops in [higher layer parameter]. ---------------------------- End of Text Proposal for TS 38.214 ---------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TP in the table below

TP 2.9-1:

| Company | Comment |
| --- | --- |
| vivo | Not essential, these parameters are reflected in TS38.211. |
| ZTE | OK |
| NewH3C | OK |
| Ericsson | OK |
| FL | Let’s bring this for quick online agreement. |

### Conclusion

Agreement Endorse the TP 2.9-1 in section 2.10.1 of R1-2312344, with deletion of “s” in this text: “in each hops”.

## [closed] Hopping across slot boundary

### Text proposal

| TP 2.10-1 a |  |
| --- | --- |
| reason for change: | The agreed “UE does not expect to be configured for any hops across slot boundaries, i.e.the starting position + duration of a hop cannot exceed a slot duration” was not captured in current spec. |
| summary of change: | Capture the following agreement in the spec. |
| Consequences if not approved: | Without any clarifications, UE may be configured with a hop across slot boundary. |
| ##### ------------ Start of Text Proposal for TS 38.214 ----------- ##### 6.2.1.4.1 SRS frequency hopping for positioning The reduced capability UE may be configured via [higher layer parameter], subject to UE capability, to perform transmit frequency hopping separate from the UL BWP configuration and outside of the UL BWP, where the UE may be configured with subcarrier spacing, CP and bandwidth that are different from the UL active BWP. The reduced capability UE transmit frequency hopping is configured within one SRS resource for positioning, that may be configured with a bandwidth larger than the maximum bandwidth of the reduced capability UE, in RRC_CONNECTED or RRC_INACTIVE mode. The reduced capability UE transmit frequency hopping, may be configured with overlapping or non-overlapping frequency hops in the frequency domain. When the reduced capability UE is configured to perform transmit frequency hopping, the UE does not expect to be configured for any hop across slot boundaries The reduced capability UE may be configured, via [higher layer parameter], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is not expected to be configured with one [cycle] of the transmit frequency hopping that is partially overlapped with the time window. If the SRS symbol(s), including the switching time to or from the active bandwidth part, of the transmit frequency hopping collides with PUSCH or PUCCH including the switching time or from the active bandwidth part, and if the UE determines the SRS to be dropped, the colliding SRS symbol(s) are dropped. ---------------------------- End of Text Proposal for TS 38.214 ---------------------------- |  |

| TP 2.10-1 b |  |
| --- | --- |
| reason for change: | For SRS with Tx hopping, each hop must be contained within a slot. |
| summary of change: | Add the following description in TS 38.214: UE does not expect to be configured for any hops across slot boundaries, i.e., the starting position + duration of a hop cannot exceed a slot duration. |
| Consequences if not approved: | The configuration of the starting position and hop duration for SRS with Tx hopping is not clear. |
| ##### ------------ Start of Text Proposal for TS 38.214 clause 6.2.1.4.1 ----------- The reduced capability UE may be configured via [higher layer parameter], subject to UE capability, to perform transmit frequency hopping separate from the UL BWP configuration and outside of the UL BWP, where the UE may be configured with subcarrier spacing, CP and bandwidth that are different from the UL active BWP. The reduced capability UE transmit frequency hopping is configured within one SRS resource for positioning, that may be configured with a bandwidth larger than the maximum bandwidth of the reduced capability UE, in RRC_CONNECTED or RRC_INACTIVE mode. The reduced capability UE transmit frequency hopping, may be configured with overlapping or non-overlapping frequency hops in the frequency domain. When the reduced capability UE is configured to perform transmit frequency hopping, it does not expect to be configured with [StartingSymbol] and [Length] for a hop that exceeds a slot duration. ---------------------------- End of Text Proposal for TS 38.214---------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TPs in the table below

TP 2.10-1 (a or b):

| Company | Comment |
| --- | --- |
| vivo | We slightly prefer TP 2.10-1b. |
| CATT | We prefer TP 2.10-1b. |
| Ericsson | Ok with 1b. |

### Conclusion

Agreement Endorse the TP 2.10-1b in section 2.11.1 of R1-2312344

## Collision with UL signals and SRS with tx hopping

### Text proposal

| TP 2.11-1 |  |
| --- | --- |
| reason for change: | (1) Replace “PUSCH or PUCCH” with “other UL signals” to include the case when MIMO SRS and SRS for hopping have collision. (2) Based on RAN’s agreement, if the SRS symbol(s), including the retuning time to/from the active BWP, collides with PUSCH or PUCCH, when UE determines that SRS with Tx hopping is to be dropped, the colliding SRS symbol(s) are dropped. However, in the current spec wording, switching time is counted twice. (3) If the SRS hop is outside the active BWP, the rule defined in Rel-17 for RRC_INACTICE state outside initial BWP can be reused, i.e. SRS is lower priority. |
| summary of change: | (1) Replace “PUSCH or PUCCH” with “other UL signals”; (2) delete “including the switching time to or from the active bandwidth part” following “collides with PUSCH or PUCCH”; (3) add the collision case when the SRS hop is outside the active BWP |
| Consequences if not approved: | SRS symbols even without collision with data may be dropped. |
| ##### ------------ Start of Text Proposal for TS 38.214 ----------- ##### 6.2.1.4.1 SRS frequency hopping for positioning <Omitted> If the SRS symbol(s), including the switching time to or from the active bandwidth part, of the transmit frequency hopping collides with other UL signals , and if the UE determines the SRS to be dropped, the colliding SRS symbol(s) are dropped. If the positioning SRS symbol(s) is outside the active BWP, including the switching time to or from the active bandwidth part, of the transmit frequency hopping collides with other UL signals, the UE determines the SRS to be dropped on the colliding symbol(s). ---------------------------- End of Text Proposal for TS 38.214---------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TP in the table below

TP 2.11-1:

| Company | Comment |
| --- | --- |
| Vivo | We perfer the modification in section 2.9 to resolve the collision between SRS for positioning and SRS for MIMO. That is, if we reuse collision rules between MIMO SRS and Pos SRS, UE should be not expected to be configured MIMO SRS and Pos SRS frequency hopping in the overlapping symbols, which is different with ‘dropping rules’ here. Then, regarding the last sentence, the description can be covered by previous description. |
| ZTE | We support this TP. Without clarification, it is unclear whether SRS or other UL signal is prioritized. However, we think it is better to discuss the proposal in section 5.3 first which may imply the corresponding TP. Basically, the TP is related to the proposals in section 5.3.2 and 5.3.3. |
| Ericsson | Agree with ZTE, this can wait until we have clear agreements in place. |

## [closed] Editiorial change in 38.211

### Text proposal

| TP 2.12-1 |  |
| --- | --- |
| reason for change: | Correction of typo in 38.211 |
| summary of change: | Correction of typo in 38.211 |
| Consequences if not approved: |  |
| ##### ------------ Start of Text Proposal for TS 38.211 ----------- ##### 6.4.1.4.1 SRS resource An SRS resource is configured by the SRS-Resource IE or the SRS-PosResource IE and consists of - NapSRS∈1,2,4,8 antenna ports pii=0NapSRS-1, where the number of antenna ports is given by the higher layer parameter nrofSRS-Ports if configured, otherwise NapSRS=1, and pi=1000+i when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet not set to 'nonCodebook', or determined according to [6, TS 38.214] when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet set to 'nonCodebook' - Nhop, the number of hops for SRS Tx hopping given by the higher layer parameter SRShoppingNrofHops if configured, otherwise Nhop=1. - NsymbSRS∈1,2,4,8,10,12,14 consecutive OFDM symbols given by the field nrofSymbols contained in the higher layer parameter resourceMapping. If Nhop>1, NsymbSRS is the number of consecutive OFDM symbol per hop. - l0, the starting position in the time domain given by l0=Nsymbslot-1-loffset where the offset loffset∈0,1,…,13 counts symbols backwards from the end of the slot and is given by the field startPosition contained in the higher layer parameter resourceMapping and loffset≥NsymbSRS-1. If Nhop>1 l0 is the starting position of each hop in the time domain, determined by the field startPosition for each SRS transmission hop. - k0, the frequency-domain starting position of the sounding reference signal <Unrelated part omitted> ##### 6.4.1.4.3 Mapping to physical resources Throughout this clause, when the higher layer parameter SRShoppingNrofHops is provided for SRS-PosResource, the sounding reference signal sequence definition applies to a given hop. <Unrelated part omitted> ---------------------------- End of Text Proposal for TS 38.211 ---------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TP in the table below

TP 2.12-1:

| Company | Comment |
| --- | --- |
| vivo | OK |
| CATT | OK |
| Ericsson | OK |
| FL | Let’s bring this for quick online agreement. |

### Conclusion

Agreement Endorse the TP 2.12-1 in section 2.13.1 of R1-2312344

## Collision rules with PUSCH and PUCCH

### Text proposal

| TP 2.13-1 |  |
| --- | --- |
| reason for change: | Current specification for the SRS for positioning only partially captures the UE behaviour for the SRS for positioning with Tx hopping for the case of collision with PUSCH. |
| summary of change: | Includes the collision rules for SRS collisions with PUSCH and PUCCH, for the cases of - PUSCH priority 1 - PUCCH for the different PUCCH content colliding with periodic or SP SRS. |
| Consequences if not approved: | Collision rules specified in 38.214 are incomplete. |
| ---------------------------- Start of Text Proposal for TS 38.214 ----------------------------- < Unchanged parts are omitted > If a PUSCH transmission with a priority index 1 or a PUCCH transmission with a priority index 1, including potential retuning time to or from the active UL bandwidth part if the SRS transmission is configured with frequency hopping for positioning as described in clause 6.2.1.4.1, would overlap in time with an SRS transmission on a serving cell, the UE does not transmit the SRS in the overlapping symbol(s). < Unchanged parts are omitted > For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent or periodic SRS is configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1SINR report(s) , including symbols for potential retuning time to or from the active UL bandwidth part if the SRS transmission is configured with frequency hopping for positioning as described in clause 6.2.1.4.1. A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR including symbols for potential retuning time to or from the active UL bandwidth part if the SRS transmission is configured with frequency hopping for positioning as described in clause 6.2.1.4.1. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS, including potential retuning time between hops when the SRS transmission is configured with frequency hopping for positioning as described in clause 6.2.1.4.1, is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s). < Unchanged parts are omitted > #### 6.2.1.4 UE sounding procedure for positioning purposes < Unchanged parts are omitted > For operation on the same carrier, if an SRS configured by the higher parameter SRS-PosResource collides with a scheduled PUSCH, including potential retuning time to or from the active UL bandwidth part if the SRS transmission is configured with frequency hopping for positioning as described in clause 6.2.1.4.1, the SRS is dropped in the symbols where the collision occurs. ##### 6.2.1.4.1 SRS frequency hopping for positioning The reduced capability UE may be configured via [higher layer parameter], subject to UE capability, to perform transmit frequency hopping separate from the UL BWP configuration and outside of the UL BWP, where the UE may be configured with subcarrier spacing, CP and bandwidth that are different from the UL active BWP. The reduced capability UE transmit frequency hopping is configured within one SRS resource for positioning, that may be configured with a bandwidth larger than the maximum bandwidth of the reduced capability UE, in RRC_CONNECTED or RRC_INACTIVE mode. The reduced capability UE transmit frequency hopping, may be configured with overlapping or non-overlapping frequency hops in the frequency domain. The reduced capability UE may be configured, via [higher layer parameter], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is not expected to be configured with one [cycle] of the transmit frequency hopping that is partially overlapped with the time window. --------------------------------------- End of Text Proposal ---------------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TP in the table below

TP 2.13-1:

| Company | Comment |
| --- | --- |
| vivo | It seems that the previous description of collision rules in section 6.2.1.4.1 is enough to capture collision rules with PUSCH and PUCCH. |
| ZTE | The TP conflicts with the one in section 2.12. We have to select one. For this TP, it assumes the hopping SRS even outside active BWP is in the same CC with the active BWP. For the TP in section 2.12, it reuses the rule when SRS is outside active BWP in RRC_INACTIVE state. Also, the proposal 5.3.3 is discussing the same issue. |
| Ericsson | We support the TP but are ok to wait for stabilizing the discussion in section 5. Another way would be to clarify that the rules in legacy apply only to the case of no hopping, and keep the wording in 6.2.1.4.1. |

## Collision rules with aperiodic SRS for positioning

### Text proposal

| TP 2.14-1 |  |
| --- | --- |
| reason for change: | Current specification for the SRS for positioning do not capture the rules for aperiodic SRS for positioning with frequency hopping. |
| summary of change: | Includes the collision rules for SRS collisions for the case of aperiodic SRS for positioning with frequency hopping. |
| Consequences if not approved: | SRS for positioning with Tx hopping collision rules are undefined for aperiodic SRS. |
| ---------------------------- Start of Text Proposal for TS 38.214 ----------------------------- ### 6.2.1 UE sounding procedure < Unchanged parts are omitted > In case a SRS resource with resourceType set as 'aperiodic' is triggered on the OFDM symbol(s) configured with periodic/semi-persistent SRS transmission, the UE shall transmit the aperiodic SRS resource and only the periodic/semi-persistent SRS symbol(s) overlapping within the symbol(s), including potential RF retuning time for SRS for positioning with frequency hopping as described in clause 6.1.2.4.1, are dropped, while the periodic/semi-persistent SRS symbol(s) that are not overlapped with the aperiodic SRS resource are transmitted. In case a SRS resource with resourceType set as 'semi-persistent' is triggered on the OFDM symbol(s) configured with periodic SRS transmission, the UE shall transmit the semi-persistent SRS resource and only the periodic SRS symbol(s) overlapping within the symbol(s) , including potential RF retuning time for SRS for positioning with frequency hopping as described in clause 6.1.2.4.1, are dropped, while the periodic SRS symbol(s) that are not overlapped with the semi-persistent SRS resource are transmitted. ## *** Unchanged parts are omitted *** --------------------------------------- End of Text Proposal ---------------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TP in the table below

TP 2.14-1:

| Company | Comment |
| --- | --- |

## Collision rules for PRACH and SRS for positioning with tx hopping

### Text proposal

| TP 2.15-1 |  |
| --- | --- |
| reason for change: | Collision rules for SRS for positioning with FH colliding with PRACH are not clearly defined. |
| summary of change: | Clarifies that for collision rules of positioning SRS with FH and PRACH, the retuning time of the SRS FH should be included in the colliding slots. |
| Consequences if not approved: | SRS for positioning scheduling rules for the case of FH are not clear |
| ---------------------------- Start of Text Proposal for TS 38.213 ----------------------------- < Unchanged parts are omitted > ## 8.1 Random access preamble ## *** Unchanged parts are omitted *** For single cell operation or for operation with contiguous carrier aggregation in a same frequency band or for operation with non-contiguous carrier aggregation in a same frequency band if the UE is not provided with intraBandNC-PRACH-simulTx-r17, a UE does not transmit PRACH and PUSCH/PUCCH/SRS, including RF retuning time applicable to SRS tx hopping when configured, in a same slot with respect to the smallest SCS configuration between the SCS configuration for the UL BWP with the PRACH and the SCS configuration for the UL BWP with the PUSCH/PUCCH/SRS transmissions or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, N=16 for μ=5, N=32 for μ=6, and μ is the smallest SCS configuration between the SCS configuration for the UL BWP with the PRACH and the SCS configuration for the UL BWP with the PUSCH/PUCCH/SRS transmissions. For a PUSCH transmission with repetition Type B, this applies to each actual repetition for PUSCH transmission [6, TS 38.214]. ## *** Unchanged parts are omitted *** --------------------------------------- End of Text Proposal ---------------------------------- |  |

| TP 2.15-1b |  |
| --- | --- |
| reason for change: | Collision rules for SRS for positioning with FH colliding with PRACH are not clearly defined. |
| summary of change: | Clarifies that for collision rules of positioning SRS with FH and PRACH, the retuning time of the SRS FH should be included in the colliding slots. |
| Consequences if not approved: | SRS for positioning scheduling rules for the case of FH are not clear |
| ---------------------------- Start of Text Proposal for TS 38.213 ----------------------------- < Unchanged parts are omitted > ## 8.1 Random access preamble ## *** Unchanged parts are omitted *** For single cell operation or for operation with contiguous carrier aggregation in a same frequency band or for operation with non-contiguous carrier aggregation in a same frequency band if the UE is not provided with intraBandNC-PRACH-simulTx-r17, a UE does not transmit PRACH and PUSCH/PUCCH/SRS, including RF retuning time applicable to SRS tx hopping when configured as described in clause 6.1.2.4.1 [6, TS 38.214], in a same slot with respect to the smallest SCS configuration between the SCS configuration for the UL BWP with the PRACH and the SCS configuration for the UL BWP with the PUSCH/PUCCH/SRS transmissions or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, N=16 for μ=5, N=32 for μ=6, and μ is the smallest SCS configuration between the SCS configuration for the UL BWP with the PRACH and the SCS configuration for the UL BWP with the PUSCH/PUCCH/SRS transmissions. For a PUSCH transmission with repetition Type B, this applies to each actual repetition for PUSCH transmission [6, TS 38.214]. ## *** Unchanged parts are omitted *** --------------------------------------- End of Text Proposal ---------------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TP in the table below

TP 2.15-1:

| Company | Comment |
| --- | --- |
| Vivo | Generally OK. Maybe additional reference to TS38.214 section 6.2.1.4.1 is needed. For example, including RF retuning time applicable to SRS tx hopping when configured as described in clause 6.1.2.4.1 [6, TS 38.214] |
| CATT | OK with the updated TP from vivo. |
| New H3C | OK with TP form VIVO |
| Ericsson | OK with the vivo update. |
| FL | Update from the vivo’s proposal is in TP 2.15-1b above |

## UTW configuration per serving cells

### Text proposal

| TP #2.16-1 |  |
| --- | --- |
| reason for change: | The UTW definition states that it applies to all SRSs for positioning with Tx hopping in the serving cell it is configured. This is not captured in the current 38.214 specification. |
| summary of change: | - Clarifies that the UTW applies to the configured SRS with tx hopping resources in the serving cell. |
| Consequences if not approved: | UTW application range is unclear. |
| ---------------------------- Start of Text Proposal for TS 38.214 ----------------------------- < Unchanged parts are omitted > ##### 6.2.1.4.1 SRS frequency hopping for positioning < Unchanged parts are omitted > The reduced capability UE may be configured, via [higher layer parameter], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels in a serving cell and is only expected to transmit the SRS for positioning using frequency hopping. The UE is not expected to be configured with one [cycle] of the transmit frequency hopping that is partially overlapped with the time window. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ---------------------------------- |  |

### Round 1

Companies are encouraged to comment on the proposed TP in the table below

TP 2.16-1:

| Company | Comment |
| --- | --- |
| vivo | We think the previous description is clear. |
| ZTE | OK |
| New H3C | OK |
| support | Support. In our view this clarifies at which level the UTW is configured, i.e at the serving cell level, for any BWP. |

## [closed] Power control for SRS with tx hopping

### Text proposal

| TP #2.17-1 a |  |
| --- | --- |
| reason for change: | Current specification for the SRS for positioning power control do not account for the configuration of the SRS with Tx hopping outside of the active BWP |
| summary of change: | Include configuration of the SRS for positioning with tx hopping in the power control equations. |
| Consequences if not approved: | SRS for positioning with Tx hopping power control is undefined. |
| ---------------------------- Start of Text Proposal for TS 38.213 ----------------------------- < Unchanged parts are omitted > ### 7.3.1 UE behaviour ## *** Unchanged parts are omitted *** If a UE transmits SRS for positioning with Tx hopping based on a configuration by SRS-PosResourceSet in [TBD RAN2], , the active UL BWP b refers to configuration provided by [bwp] in [TBD RAN2]. If a UE transmits SRS based on a configuration by SRS-PosResourceSet in SRS-PosRRC-InactiveConfig-ValidityArea in RRC_INACTIVE state [12, TS 38.331], the active UL BWP b refers to the BWP provided by bwp in SRS-PosRRC-InactiveConfig-ValidityArea. If the UE is not provided pathlossReferenceRS-Pos in SRS-PosResourceSet, or if the UE is provided pathlossReferenceRS-Pos in SRS-PosResourceSet and the UE cannot accurately measure a pathloss, the UE calculates PLb,f,c(qd) using an RS resource from an SS/PBCH block with same index as the one the UE used to obtain MIB; otherwise, the UE uses the RS indicated by pathlossReferenceRS-Pos to calculate PLb,f,c(qd). ## *** Unchanged parts are omitted *** --------------------------------------- End of Text Proposal ---------------------------------- |  |

| TP #2.17-1 b |  |  |  |
| --- | --- | --- | --- |
| Reason for change | Based on the following agreement, SRS for positioning with Tx hopping is configured outside UL BWP. So, for power control of SRS with Tx hopping, ‘the active BWP b’ should refer to the BWP configuration for SRS with Tx hopping. However, such description is not captured in TS38.213. Agreement SRS for positioning with Tx hopping can be configured outside of the active UL BWP - The configuration may include SCS, CP size and bandwidth (position and size), which can use a SCS, CP size and bandwidth different from the UL active BWP. |
| Summary of change | Section 7.3.1 in TS 38.213: Add description regarding ‘the active BWP b’ for SRS for positioning frequency hopping power control. |
| Consequences if not approved | Incorrect description for SRS for positioning frequency hopping power control regarding the BWP reference. |
| < Unchanged text omitted > If a UE transmits SRS based on a configuration by SRS-PosResourceSet outside initial UL BWP of carrier f of serving cell c in RRC_INACTIVE state, the active UL BWP b refers to the BWP configuration provided by bwp-NUL or bwp-SUL in SRS-PosRRC-InactiveConfig for the corresponding carrier. If a UE transmits SRS on multiple SRS resources for positioning bandwidth aggregation according to linkage [6, TS 38.214], the UE calculates PSRS,b,f,ci,qs using the same values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and PLb,f,cqd for each of the multiple SRS resources. If a UE transmits SRS based on a configuration by SRS-PosResourceSet in SRS-PosRRC-InactiveConfig-ValidityArea in RRC_INACTIVE state [12, TS 38.331], the active UL BWP b refers to the BWP provided by bwp in SRS-PosRRC-InactiveConfig-ValidityArea. If the UE is not provided pathlossReferenceRS-Pos in SRS-PosResourceSet, or if the UE is provided pathlossReferenceRS-Pos in SRS-PosResourceSet and the UE cannot accurately measure a pathloss, the UE calculates PLb,f,c(qd) using an RS resource from an SS/PBCH block with same index as the one the UE used to obtain MIB; otherwise, the UE uses the RS indicated by pathlossReferenceRS-Pos to calculate PLb,f,c(qd). If a reduced capability UE performs SRS transmit frequency hopping based on a configuration by [SRS-PosResourceSet] in [higher layer parameter] outside of the active UL BWP of carrier f of serving cell c in RRC_CONNECTED state [12, TS 38.331], the active UL BWP b refers to the BWP configuration provided by [bwp] in [higher layer parameter]. If a reduced capability UE performs SRS transmit frequency hopping based on a configuration by [SRS-PosResourceSet] in [higher layer parameter] outside of the intial UL BWP of carrier f of serving cell c in RRC_INACTIVE state [12, TS 38.331], the active UL BWP b refers to the BWP configuration provided by [bwp] in [higher layer parameter]. < Unchanged text omitted > |  |  |

### Round 1

Companies are encouraged to comment on the proposed TPs in the table below

TP 2.17-1 (a or b):

| Company | Comment |
| --- | --- |
| vivo | TP 2.17-1b is preferred, since for RRC_CONNECTED and RRC_INACTIVE, the corresponding [higher layer parameter] of BWP configurations are different, separate descriptions for RRC_CONNECTED and RRC_INACTIVE are clearer. |
| ZTE | Either one is fine |
| CATT | We prefer the TP 2.17-1b to address the power control issue. |
| Ericsson | Ok with 1b, since it also covers rrc inactive. |

### Conclusion

Agreement Endorse the TP 2.17-1b in section 2.18.1 of R1-2312344

# Measurement reporting

The proposals on measurement reporting revolve around the following issues:

- hop indication for multi hop reporting

- use of single-hop or multi-hop reporting

- number of hops included in a measurement request

- special case for multi-RTT measurements

## [closed] Single / multi hop indication

### Summary of contributions

Indication of a report being for single-hop or multi-hop measurement was proposed in [2,3,6,9,11,12,13,18]. In [16] the indication is not supported.

Company views are summarized in the table below:

| Company | Proposal |
| --- | --- |
| [2] | Proposal 7: Support the indication on either a single-hop or multi-hop measurement is reported. |
| [3] | Proposal 1: RAN1 supports the following text proposal on Clause 5.1.6.5.1 of TS 38.214. ##### 5.1.6.5.1 PRS receiver frequency hopping The reduced capability UE may be configured to measure and report, subject to UE capability, via [higher layer parameter] the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, or UE Rx-Tx time difference using receiver frequency hopping for a DL PRS resource, with a requested bandwidth of all hops that may be greater than the maximum reduced capability UE bandwidth. The reduced capability UE performing receiver frequency hopping may report via [higher layer parameter] one measurement associated with one received frequency hop or one measurement based on multiple hops of the DL PRS. In the measurement report via [higher layer parameter], the reduced capability UE includes the number of consecutive hops of the DL PRS used for the measurement. <omitted text>. Proposal 10: Support at least one of the following features: - Alt.1: The gNB explicitly indicates frequency hops used for a reported positioning measurement. - Alt 2: The gNB indicates if the reported positioning measurement is from a single hop or multiple hops. |
| [6] | Proposal 1 • For DL PRS or UL SRS for positioning with frequency hopping, positioning measurements based on single hop and multiple hops are separately reported. • A single indication of “single-hop” and “multiple-hop” can be included in the measurement report. |
| [9] | Proposal 2: An indicator is needed to indicate the measurement based on multiple hops or based on one hop. Proposal 3: Adopt the TP #1 regarding measurement report in clause 5.1.6.5.1 of TS 38.214 [3][4]. |
| [11] | Proposal 9: For DL Rx hopping, the measurement report from UE indicates which of a single-hop or multi-hop measurement is being reported. |
| [13] | Proposal 3: The UE/gNB can report either a single-hop or multi-hop measurement and the measurement report indicates which of a single-hop or multihop measurement is being reported. |
| [12] | Proposal 2: For measurements based on DL PRS with Rx frequency hopping or UL SRS with Tx hopping, support that: • UE/gNB can report either a single-hop or multi-hop measurement. • Indication on which of a single-hop or multi-hop measurement is reported. |
| [16] | Proposal 3: which of a single-hop or multi-hop measurement is not being reported. |
| [18] | Proposal 3: For DL Rx hopping, • A UE may report either a measurement based on receiving multiple hops, or the measurement based on single hop. o Introduce a signaling that indicated whether a “measurement based on receiving multiple hops” or “measurement based on single hop” is being reported • Up to RAN4 to define any conditions on when the measurements are reported, what/if accuracy requirements are needed to be specified if there collisions or hops are being dropped. |

### Round 1

The discussion is a continuation from the previous meeting and it seems the views are more or less stable. The majority support the indication of multi or single-hop.

Proposal 3.1-1:

For measurements based on DL PRS with Rx frequency hopping or UL SRS with Tx hopping:

- UE/gNB can report either a single-hop or multi-hop measurement.

- Indication of which of a single-hop or multi-hop measurement is reported.

Companies are encouraged to comment on the proposed TP in the table below

Proposal 3.1-1:

| Company | Comment |
| --- | --- |
| Vivo | OK |
| InterDigital | Support |
| Huawei, HiSilicon | Support. |
| Qualcomm | Support |
| ZTE | Support. |
| CATT | Support. We think it is necessary to introduce such an indication in the last bullet. |
| New H3C | OK |
| LGE | Although we still think the benifit of indication in the second bullet is ambiguous, we can compromise for the progress. |
| Ericsson | OK |
| FL | It seems we can bring this one online to agreement. |

### Conclusion for RAN1#115

The following was agreed online:

Agreement For measurements based on DL PRS with Rx frequency hopping or UL SRS with Tx hopping: - UE/gNB can report either a single-hop or multi-hops measurement. - Indication of which of a single-hop or multi-hops measurement is optionally reported. - Note: mapping of the indicator to performance requirement(s), or impact to performance requirement(s), is up to RAN4

## [MEDIUM] Hop ID / number of hops in measurement reports

### Summary of contributions

This issue has been ongoing for several meeting and proposals in [3,11,12,13,18] are similar from previous meeting. The proposals are all centered on reporting either the which hops were used to produced the reported measurements, or how many hops were used.  In [12,18] no further enhancements are supported.

Company views are summarized in the table below:

| Company | Proposal |
| --- | --- |
| [3] | Proposal 1: RAN1 supports the following text proposal on Clause 5.1.6.5.1 of TS 38.214. ##### 5.1.6.5.1 PRS receiver frequency hopping The reduced capability UE may be configured to measure and report, subject to UE capability, via [higher layer parameter] the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, or UE Rx-Tx time difference using receiver frequency hopping for a DL PRS resource, with a requested bandwidth of all hops that may be greater than the maximum reduced capability UE bandwidth. The reduced capability UE performing receiver frequency hopping may report via [higher layer parameter] one measurement associated with one received frequency hop or one measurement based on multiple hops of the DL PRS. In the measurement report via [higher layer parameter], the reduced capability UE includes the number of consecutive hops of the DL PRS used for the measurement. <omitted text>. Proposal 10: Support at least one of the following features: - Alt.1: The gNB explicitly indicates frequency hops used for a reported positioning measurement. - Alt 2: The gNB indicates if the reported positioning measurement is from a single hop or multiple hops. |
| [11] | Proposal 8: For UL SRS Tx hopping, support TRP to report either a single-hop or multi-hop measurement  The measurement report includes how many and which hops are used. |
| [12] | Proposal 1: For DL Rx hopping or UL Tx hopping, NO need to include the following contents in a measurement report: • Indication of which hop is used for a single-hop measurement; • Indication of which hops are used for multiple-hop measurement. |
| [13] | Proposal 4: If multi-hop based measurement is supported, the UE can either indicate hop indices of received hops or number of hops used to determined the measurement |
| [18] | Proposal 2: Do not support an additional indication of how many received hops / which received hops where used in the measurement report. • Note: A UE may, up to UE implementation, to use the quality metric and the ReportingGranularityfactor to inform the LMF that a more accurate measurement is being reported. |

### Round 1

Similar to the previous meeting, based on the proposals received there is no consensus to support reporting the number of hops, or which hops were used in the measurement by the UE or gNB. However, a majority of proposal support indicating the hop index (for single hop measurement) or the number of hops  (or list of indices) in the measurement report.

Proposal 3.1-1:

For measurements based on DL PRS with Rx frequency hopping or UL SRS with Tx hopping:

- the UE/gNB can additionally report

  - Indication of which hop(s) were used to produce the DL positioning measurement for UE reported DL positioning measurements

  - Indication of which hop(s) were used to produce the UL positioning measurement for gNB reported UL positioning measurements.

Companies are encouraged to comment on the proposal in the table below:

Proposal 3.2-1:

| Company | Comment |
| --- | --- |
| Vivo | Not support. For measurement reporting, the proposal in section 3.1.2 is enough. |
| InterDigital | Support |
| Huawei, HiSilicon | No need. |
| Qualcomm | We do not see the need to report which “hops” were used for measurement. |
| ZTE | We prefer to have progress on proposal in 3.1.2 first. |
| New H3C | No need |
| LGE | Do not support |

## [MEDIUM] Single-hop as fallback for multi-hop measurement

### Summary of contributions

In [9] the proposal to use single hop as a fall back to multiple hop when measurement for multihop fails is brought again for discussion.

| Company | Proposal |
| --- | --- |
| [9] | Proposal 1: When the measurement based on multiple hops is reported, the measurement based on one hop should not be reported. When the measurement based on multiple hops fails, the measurement based on one hop can be reported as a fallback. |

### Round 1

Proposal 3.3-1: When the measurement based on multiple hops is reported, the measurement based on one hop should not be reported. When the measurement based on multiple hops fails, the measurement based on one hop can be reported as a fallback.

Companies are encouraged to comment on the proposal in the table below:

Proposal 3.3-1:

| Company | Comment |
| --- | --- |
| Vivo | Up to RAN4. |
| Huawei, HiSilicon | We are fine for this. |
| Qualcomm | We are Ok with the first sentence, but not sure if the 2nd is really needed; to us it looks related to RAN4 requirements, i.e. under which conditions the UE should report a measurement; the wording “measurement fails” is not descriptive enough and we prefer to not go into RAN4 territory. Maybe a more generic wording for the 2nd sentence would be more appropriate (example shown below): When the UE is requested to perform multi-hop measurements, a UE may report a measurement based on one hop if the UE doesnt report a multi-hop measurement. |
| ZTE | With proposal in 3.1.2, we don’t think this proposal is needed. A UE can either report a single-hop measurement result or a multi-hop measurement result. |
| New H3C | Ok in general |
| LGE | Same view with ZTE |

## [LOW] Carrier phase measurements

### Summary of contributions

| Company | Proposal |
| --- | --- |
| [3] | Proposal 12: The gNB can report CP measurement made from the combined hops of SRS transmission. Proposal 13: The RedCap UE can report CP measurement subject to the UE capability. |

### Round 1

from the FL perspective, there is nothing prohibiting the use of Carrier phase positioning  in a redcap UE which supports CPP, but we should not aim at specific enhancement for CPP.  It is not obvious that such an agreement would be needed, but we can try to collect more views:

Proposal 3.4-1: For carrier phase measurements:

- for UL CP measurements based on SRS for positioning with tx hopping, The gNB can report CP measurement made from the combined hops of SRS transmission.

- For DL CP measurements The RedCap UE can report CP measurement subjects to the UE capability.

Companies are encouraged to comment on the proposal in the table below:

Proposal 3.4-1:

| Company | Comment |
| --- | --- |
| Vivo | We perfer not to combine carrier phase measurement with Rel-18 technology. |
| InterDigital | We need more time to discuss the details. |
| Qualcomm | Do not support |
| ZTE | Prefer not to discuss any more enhancement in Rel-18 |
| LGE | Not suppuort for Rel-18 |
| Ericsson | Do not support |

# DL-PRS Frequency Hopping

## [MEDIUM] DL PRS antenna port indication across repetitions

### Proposals

| Company | Proposal |
| --- | --- |
| [2] | Proposal 8: The assistance data includes an indicator per PRS resource set on whether the PRS resource repetitions are from the same antenna port. |

### Round 1

this is a new proposal. from the FL view, a PRS resource transmission is single-port, therefore all transmitted symbols for a given PRS resource occasion should come from the same port.

Proposal 4.1-1:   The assistance data includes an indicator per PRS resource set on whether the PRS resource repetitions are from the same antenna port.

Companies are encouraged to comment on the proposal in the table below:

Proposal 4.1-1:

| Company | Comment |
| --- | --- |
| vivo | We share the similar view as FL. |
| Huawei, HiSilicon | We think that the proposal is about PRS repetition across slots, whether the PRS repetitions are regarded as the same antenna port. |
| Qualcomm | Even though we generally interpret the specification in a similar way as the FL, we sympathize with a potential issue that may arise: if the DL PRS repetitions are “too far away” maybe there can be additional phase mismatches that could not be compensated with a simple frequency domain overlap. We are open to discuss more this issue to reach a common understanding. |
| ZTE | Do not support. We do not think any assistance-data-level enhancement is needed for PRS Rx hopping. |

## [closed] Assistance data for DL PRS with RX hopping

### Proposals

| Company | Proposal |
| --- | --- |
| [13] | Proposal 6: Support configurable number of Rx hops for DL PRS with Rx frequency hopping. Proposal 7: Support configurable amount of overlap between Rx hops for DL PRS with Rx frequency hopping. |

### Round 1

this proposal is similar to proposals made in previous meeting to provide more information in the DL PRS assistance data for Rx hopping and include the number of Rx hops and overlap. In general the comments from previous meeting was to leave this to UE implementation.

Proposal 4.2-1: for DL PRS Rx hopping, the assistance data includes

- the number of Rx hops for DL PRS with rx hopping

- the PRB overlap between Rx hops

Companies are encouraged to comment on the proposal in the table below:

Proposal 4.2-1:

| Company | Comment |
| --- | --- |
| vivo | Not support. The hop request with total hop bandwidth is enough. |
| InterDigital | Support |
| Huawei, HiSilicon | Do not support. |
| Qualcomm | Do not support |
| New H3C | Do not support |
| LGE | Do not support. |
| FL | Considering the lack of support we can close the issue |

## Assistance data for RSTD with rx hopping

### Summary of proposals

| Company | Proposal |
| --- | --- |
| [13] | Proposal 1: The UE should be configured with the same number of PRS repetitions transmitted from the reference transmission point and target transmission point when Rx hopping is enabled. Proposal 2: When Rx hopping is enabled, the UE determines RSTD based on a hop pair basis where a hop pair consists of two PRSs with the same repetition occasion index each transmitted from different transmission point |

### Round 1

this proposal aims at clarifying how the UE is supposed to do the RSTD measurement with Rx hopping. From the FL perspective, it could be left to implementation.  It seems that both proposal can be alternatives to ensure that PRSs pairs are using the same repetitions.

Proposal 4.3-1: For redcap UEs performing RSTD measurement using DL PRS rx hopping:

- Alt1: The UE should be configured with the same number of PRS repetitions transmitted from the reference transmission point and target transmission point when Rx hopping is enabled.

- Alt2: When Rx hopping is enabled, the UE determines RSTD based on a hop pair basis where a hop pair consists of two PRSs with the same repetition occasion index each transmitted from different transmission point.

Companies are encouraged to comment on the proposal in the table below:

Proposal 4.3-1:

| Company | Comment |
| --- | --- |
| vivo | Same view as FL |
| InterDigital | Just to clarify, the proposals are meant to be separate proposals, not alternatives. They are meant to coexist. Each alternative should be changed to a proposal. |
| Qualcomm | Do not support |
| LGE | We don’t think it is necessary. |

## [closed] DL PRS repetition factor

### Proposal

| Company | Proposal |
| --- | --- |
| [20] | Proposal 4 Extend the value range of PRS repetition factor to {1, 2, 4, 6, 8, 16, 32, 64, 128} or {1, 2, 4, 6, 8, 16, 32, 40, 80, 160} for RedCap positioning. |

### Round 1

the proposal is to extend the values available for PRS repetition so that Rx hopping can be performed while maintaining coverage. Let’s first collect views on the issue.

Proposal 4.4-1: Extend the value range of PRS repetition factor to {1, 2, 4, 6, 8, 16, 32, 64, 128} or {1, 2, 4, 6, 8, 16, 32, 40, 80, 160} for RedCap positioning.

Companies are encouraged to comment on the proposal in the table below:

Proposal 4.2-1:

| Company | Comment |
| --- | --- |
| vivo | Large repetition factors will affect the performance of communication. We are OK to not extend repetition factor or only extend repetition factor to 64. |
| ZTE | Not sure why we need to extend the value range of PRS repetition factor? 6-hop is enough to achieve 100MHz PRS reception. |
| CATT | We prefer not to introduce new repetion factors in maintenence phase, unless the significant performance gains can be found. |
| New H3C | We don’t need introduce new factors |
| LGE | Considering the duration of a MG, we are OK with this proposal. |
| FL | Considering the lack of support from already 4 companies, we can drop the issue. |

# UL SRS Tx Hopping

## UL SRS Tx hopping configuration

### [closed] Parameter for initial hop index n0

#### Background

| Company | Proposal |
| --- | --- |
| [1] | Proposal 1: For SRS positioning with Tx hopping wrapping pattern, support: - alt2: n0 is the initial frequency hop index defined as n0=floornFirstHopRB/( mhopSRS-moverlaphop) |
| [2] | Proposal 1: Support Alt2 for the definition of the initial frequency hop index. Proposal 2: Endorse the following TP to clause 6.4.1.4.3 of TS 38.211. |
| [3] | Proposal 5: RAN1 supports a revised alt 2 of the WA RAN1#114-bis - alt2: n0is the initial frequency hop index defined as n0=floornFirstHopRB/( mhopSRS-moverlaphop) - Note: The reference point for starting PRB of the first hop nFirstHopRB and nshift is defined as lowest RB provided by the agreed configuration that may include SCS, CP size and bandwidth (position and size) - nFirstHopRB is the starting PRB of the first hop - In k0, nshift is replaced by nFirstHopRB-n0mhopSRS-moverlaphopNscRB, where nFirstHopRB≥n0mhopSRS-moverlaphop |
| [4] | Proposal 2: Alt2 seems to be a more proper solution than Alt.1. nshift should be replaced by nFirstHopRB-n0mhopSRS-moverlaphop. |
| [5] | Proposal 2 - Regarding frequency hop index of the initial hop, support Alt1: n0 is the frequency hop index of the initial hop (new configured parameter). |
| [6] | Proposal 3 - For the SRS for positioning with Tx hopping wrapping pattern, Alt. 2 is supported: - n0 is the initial frequency hop index defined as n0=floornFirstHopRB/( mhopSRS-moverlaphop) |
| [7] | Proposal 1: n0 is the initial frequency hop index defined as n0=floornFirstHopRB/( mhopSRS-moverlaphop) |
| [9] | Proposal 7: The working assumption in RAN1#114bis for the SRS for positioning with Tx hopping wrapping pattern should be confirmed, where alt2 is used for the calculation of n0. |
| [10] | Proposal 2: Support Alt-2 for determining the start point in frequency domain for each hop in the working assumption. Proposal 3: Support special handling for determining the start point in frequency domain for the hop across bandwidth boundary, and minnoffsetFH, Fe-mhopSRSNscRB can be applied for the final offset determination of each hop. Proposal 4: Support limit the start point of the first hop to ensure no hop is across the boundary of hopping band when wrapping is happened, and maximize the number of UEs multiplexed within a same time and frequency resource. |
| [11] | Proposal 1: Confirm the following working assumption with Alt.2. |
| [12] | Proposal 3 Confirm the working assumption with Alt. 2: |
| [13] | Proposal 8: Regarding “FFS: whether/how special handling for the last hop overlap”, the UE is expected to be configured with the bandwidth within which all hops in the pattern are contained, i.e., no special handling for the last hop overlap is needed |
| [14] | Proposal 1: - The initial frequency hop index n0 should be defined and calculated according to the other parameters. |
| [15] | Proposal 3: For the Tx hopping pattern for SRS for positioning: alt2: n0 is the initial frequency hop index defined as n0=floornFirstHopRB/( mhopSRS-moverlaphop) - Note: The reference point for starting PRB of the first hop nFirstHopRB and nshift is defined as lowest RB provided by the agreed configuration that may include SCS, CP size and bandwidth (position and size) |
| [16] | Proposal 1: the working assumption for the new offset nFH is confirmed with alt.1. |
| [17] | Proposal 1: For the SRS for positioning with Tx hopping wrapping pattern, the frequency hop index of the initial hop should be configured with new parameter. |
| [18] | Proposal 4: For UL SRS Tx hopping, confirm the related WA and support either Alt. 1 or Alt. 2 (small preference towards Alt. 2). |
| [19] | Proposal 2-1: Support alt. 2 as the solution for the initial frequency hop index n0 |
| [20] | - Proposal 7 The starting hop index is defined as n0=floornFirstHopRB/( mhopSRS-moverlaphop). - No new parameter is defined. |

#### Round 1

Every contribution in the agenda item had a proposal on this issue, and all but [5,16,17] support alt2, that is to say, that the variable n0=floornFirstHopRB/( mhopSRS-moverlaphop) is not a new parameter.  Let’s see if this is agreeable to the companies supporting alt1.

There was also TPs on this topic. Once we have an agreement, we can further work on an agreeable TP.

Proposal 5.1.1-1 for SRS for positioning with Tx hopping  n0 is the initial frequency hop index defined as  n0=floornFirstHopRB/( mhopSRS-moverlaphop)

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.1.1-1:

| Company | Comment |
| --- | --- |
| vivo | Before discuss this issue, we would like to check with the majority as following two questions: - Whether the bandwidth position and bandwidth size of hopping is per SRS resource - Whether allow a UE to be configured with multiple SRS resources with different hop numbers or hop bandwidth. Agreement SRS for positioning with Tx hopping can be configured outside of the active UL BWP - The configuration may include SCS, CP size and bandwidth (position and size), which can use a SCS, CP size and bandwidth different from the UL active BWP If bandwidth position and bandwidth size of hopping is configured for all the hops in the carrier, and different hop numbers or hop bandwidth can be configured to different SRS resources or SRS resource sets. The alt 1 benefits are clear since the bandwidth size may larger than hop bandwidth of some SRS resources. For example, for Alt 2, it may have limitations in some cases when the bandwidth of the ‘virtual BWP’ is greater than the total hop bandwidth, ie., Alt 2 will limit the use of the pure staircase pattern. As shown in the following figure, it can be observed that if the bandwidth of ‘virtual BWP’ is enough, for Alt 2, if the starting PRB of the first hop is within certain range, n0 cannot be equal to 0 based on the generation formula, so the SRS frequency hopping pattern can only be a wrapped staircase pattern; however, for Alt 1, even if with the same range of starting PRB of the first hop, due to the sufficient bandwidth of ‘virtual BWP’, n0 can be configured as 0, so a pure staircase pattern is applied. In general, the performance of a pure staircase pattern is better than a wrapped staircase pattern. |
| Huawei, HiSilicon | Support. Reply to vivo: We do not see the logic of configuration Alt.1 by the network. |
| ZTE | OK. To be clear, if this proposal is selected, this would mean that the goal is to occupy the configured bandwidth of “virtual BWP” as much as possible. |
| CATT | OK with the proposal. |
| Futurewei | Support the proposal. It is not clear what the benefits for vivo’s configuration scenario. Typically, the network will configure the number of hops to occupy the full wide bandwidth. |
| vivo2 | We would like to further explain our understanding of the difference between Alt 1 and Alt 2 for section 5.1.1. Based on the following figure, once the virtual BWP is larger than the total hop bandwidth（e,g., twice as follows）, Alt 1 can have 8 candidate patterns for hopping, but Alt 2 has only 4 patterns since the upper resource can not be used due to equation. In this case, the Alt 1 has more benefit for multiplexing. In addition, if the virtual BWP is equal to hop bandwidth, there is no difference between alt 1 and alt2. But, considering the hop bandwidth and hop number are per resource configuration, and virtual BWP is configured for all SRSs, it is difficult to avoid the virtual BWP being larger than the hop bandwidth. So, we prefer the majority can further consider Alt 1 and Alt 2 |
| New H3C | OK |
| LGE | Do not support. First of all, I think the proposal 5.1.5 saying that the virtual BWP configuration is common to all configured SRS resource seems agreeable. Then, If the multiple SRS-pos resource is configured in a single „virtual BWP“, then there is a clear limiation that some SRS-pos resource which have smaller hop BW is limited to be configured in the bottom part of „virtual BWP“ as following: These limitations will result in a lot of wasted resources when implementing UE multiplexing. For the perspective of resource utilization, this proposal has the indisputable defect. Instead, with Alt 1, gNB can explicit and flexbile schedule by introduce only one RRC parameter. |

#### Conclusion for RAN1#115

The following was agreed:

Agreement For SRS for positioning with Tx hopping n_0 is the initial frequency hop index defined as n_0=floor(n_FirstHop^RB/(m_hop^SRS-m_overlap^hop)) - No new parameter is defined Note: the corresponding working assumption from RAN1#114bis is confirmed with this agreement.

### [closed] Comb offset hopping

#### Background

In [6] it is mentioned that comb offset hopping, introduced for SRS for MIMO, is not to be supported for SRS for positioning. From the FL perspective, this may not be required, as the parameter combOffsetHoppingWithRepetition is not available in SRS-config for SRS for positioning resources.

| Company | Proposal |
| --- | --- |
| [6] | Proposal 4 - Comb offset hopping is not supported for positioning SRS with frequency hopping for RedCap UEs. |

#### Round 1

Proposal 5.1.2-1  Comb offset hopping is not supported for positioning SRS with frequency hopping for RedCap UEs.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.1.2-1:

| Company | Comment |
| --- | --- |
| vivo | Current description in TS38.211 is enough to avoid supporting comb offset hopping. |
| ZTE | Basically we do not think this is needed but ok to have a conclusion on this issue instead of an agreement. |
| FL | Let’s have a conclusion to close the discussion |

#### conclusion

Conclusion Comb offset hopping is not supported for positioning SRS with frequency hopping for RedCap UEs.

### [closed] Slot offset configuration

#### Background

For the slot offset values of the hops following the first hop when Tx hopping is for periodic or semi persistent , the values and applicability to SP-SRS have not yet been decided.

Agreement For SRS Tx hopping, the configuration parameters values are: - For the hop bandwidth common to all hops - Configuration re-uses C_SRS - The values of C_SRS in legacy SRS for positioning such that the maximum bandwidth is: 104 PRBs, 48 PRBs, 132 PRBs, 64 PRBs, for 15,30,60,120 KHz respectively when B_SRS equal 0. - For the starting RB of the first hop in time domain: - Configuration re-uses the IE freqDomainShift - The range is {0,268} RBs - For the single overlap common to all hops for the SRS resource - The value can be 0,1,2,4 RBs - Note: This is a new IE - For the starting slot offset and starting symbol for the SRS resource with tx hopping (first hop in time) - The value range is {0,1,2…, nrof slot in periodicity} in slots for the slot offset - Note: this is for the periodic [and semi-persistent] SRS - Starting symbol: {0,1,2,…13} in symbol - Starting slot reuses the SRS-PeriodicityAndOffset IE - Starting symbol reuses the starting position startPosition in the IE resourceMapping - The starting slot offset and symbol for each of the hops following the first hop in time, - FFS: The value range is {0,1,2…, nrof slot in periodicity} in slots for the slot offset - Note: this is for the periodic [and semi-persistent] SRS - Starting symbol: {0,1,2,…13} in symbol - this is a new IE - The number of consecutive symbols in a hop common to all hops - Values are 1,2,4,8 and 12 symbols - Configuration re-uses the IE nrofsymbols in resourcemapping - The number of hops - Values are 2,3,4,5,6 - This is a new IE

| Company | Proposal |
| --- | --- |
| [5] | Proposal 1: • Regarding the starting slot offset and symbol for each of the hops following the first hop in time, the value range can be {0,1,2…, nrof slot in periodicity} in slots for the slot offset. • The value range of {0,1,2…, nrof slot in periodicity} in slots for the slot offset for each hop applies to both the periodic and semi-persistent SRS. |
| [4] | Proposal 1 The relative position scheme has smaller parameter size the absolute scheme, the value range of the slot offset range can be {0,1,2,…,6}. |
| [13] | Proposal 9 : From the agreement about the configuration parameter values for hopping for SRS for positioning, remove square brackets as follows, “Note: this is for the periodic [and semi-persistent] SRS” due to the agreement to support both aperiodic and semi-persistent SRS for Tx frequency hopping |

#### Round 1

The proposal in [5] is to remove the FFS.  In [4] it is proposed to have a reduced slot offset range that is relative to the previous hop. [13] proposes to remove the brackets for SP SRS case.

Proposal 5.1.3-1  For the values of the starting slot offset for each of the hops following the first hop in time:

- Alt1: the value range can be {0,1,2…, nrof slot in periodicity} in slots for the slot offset.

- Alt2: the slot offset for each hop is relative to the preceding hop with range (0,1,2…,6)

- The value range slot offset for each hop applies to both the periodic and semi-persistent SRS.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.1.3-1:

| Company | Comment |
| --- | --- |
| vivo | Alt 1. In our view, Alt 1 is easier. With Alt 1, when reflecting the SRS slot configuration/formula in TS38.214, we only need to follow the existing formula without generating a new formula. |
| Huawei, HiSilicon | Alt 2. For Alt.1. this would be duplicated and also not aligned with aperiodic SRS slot indication. |
| ZTE | Alt.1 is preferred. |
| CATT | Atl. 1. |
| New H3C | Alt.1 |
| LGE | We are okay with both alternatives, but slightly prefer Alt 1 for the simplicity. |

#### Conclusion

The following was agreed:

Agreement For the values of the starting slot offset for each of the hops following the first hop in time: - Alt1: the value range can be {0,1,2…, nrof slot in periodicity -1} in slots for the slot offset. - The value range slot offset for each hop applies to both the periodic and semi-persistent SRS. - The periodicity in PeriodicityandOffset configured for each hop for a SRS resource with Tx hopping must be the same.

### [HIGH] Hop index configuration

#### Background

In both [5][11] it is proposed to clarify that the hop index is in the time domain. Additionally, [5] proposes to make the hop index a parameter. From the FL perspective, it is not clear that a parameters need to be provided. The hop index can be inferred by other parameters, e.g. the slot offset, since a wrapped staircase has been agreed.

| Company | Proposal |
| --- | --- |
| [5] | Proposal 3: • Hop index in time domain can be configured together with the starting slot offset and starting symbol for each hop. |
| [11] | Proposal 2: Adopt TP#1 for TS 38.211 for SRS Tx hopping of RedCap UE Summary of change: Clarify that nSRSTxHopping in 38.211 is the SRS hop transmission counter in time domain |

#### Round 1

We can start with the proposal in [5,11]:

Proposal 5.1.4-1  for the SRS for positioning with Tx hopping:

- The hop index is in the time domain

- Hop index in time domain can be configured together with the starting slot offset and starting symbol for each hop.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.1.4-1:

| Company | Comment |
| --- | --- |
| vivo | Support. |
| ZTE | Do not support, there is no clear benefit on introducing a new RRC parameter. |
| CATT | OK |
| LGE | There is a typo: Proposal 5.1.34-1. Simliar view with FL. We don’t see any benefit of hop index in the time domain parameter. FL: thanks! Typo corrected. |

### [closed] Parameters for SRS configuration with frequency hopping

#### Background

In [1] and [17] it is proposed to clarify that a separate configuration from the active BWP is used to configure tx hopping, and this configuration hosts all SRS resources with tx hopping.

| Company | Proposal |
| --- | --- |
| [1] | Proposal 6: The virtual BWP for positioning SRS with frequency hopping is configured independently from any data BWP. |
| [17] | Proposal 7: Configuration of SRS for positioning with Tx hopping including SCS, CP size and reference point for bandwidth determination is common to all configured SRS for positioning with Tx hopping resource(s). |

#### Round 1

This proposal will help clarifying how to put the configuration in the RRC parameter list hierarchy.  We can start with the following proposal:

Proposal 5.1.5-1 The Configuration of SRS for positioning with Tx hopping including SCS, CP size and reference point for bandwidth determination is common to all configured SRS for positioning with Tx hopping resource(s).

- The configuration for positioning SRS with frequency hopping is configured independently from any data BWP.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.1.5-1:

| Company | Comment |
| --- | --- |
| vivo | We are OK to conclude and reply LS regarding ‘The virtual BWP for positioning SRS with frequency hopping is configured independently from any data BWP’. And we are OK to the intention that SRS for positioning with Tx hopping including SCS, CP size and reference point for bandwidth is configured for all the SRS. But we would like to check whether it means the hop numbers and hoping bandwidth of all SRSs are same. |
| ZTE | OK |
| CATT | OK with the proposal. |
| New H3C | OK |
| LGE | Support To vivo: We don't think that means the number of hops and hop bandwidth are common to all SRS for positioning resources. |

#### Conclusion

The following was agree:

Agreement The configuration of SRS for positioning with Tx hopping including SCS, CP size and reference point for bandwidth determination is common to all configured SRS for positioning with Tx hopping resource(s). - The configuration for positioning SRS with frequency hopping is outside any data BWP configuration.

## UL time window (option 1)

### [MEDIUM] UTW covering the duration of a SRS occasion

#### Background

| Company | Proposal |
| --- | --- |
| [13] | Proposal 10: In the agreement related to coexistence of UTW and prioritization rule, reword the note as follows, “Note: UE is not expected to be configured with a SRS for positioning hopping cycle wrapping pattern partially overlapping with UTW.” |
| [17] | Proposal 2: Support to update the agreement as following: Agreement For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping, support both of the following options - Option 1: UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit FH SRS for positioning. - FFS details of an UL time window - Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning - Option 2: new collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels/. Option 2 can apply without or outside UL time window (i.e. option 1) - FFS: details on the collision rules Note: it is understood that option 2 is a component of the feature for UL SRS Tx hopping (FG 41-5-2), and option 1 is a separate feature group. Note: UE is not expected to be configured with a SRS for positioning hopping cycle, including the retuning time required ahead of the first hop and after the last hop, partially overlapping with UTW.. |

#### Round 1

From the FL side, it’s not clear wheather we need to spend time rewording the agreement, or if a TP agreement is enough.  We can start with the proposal in [17]

Proposal 5.2.1-1:

Support to update the agreement as following:

Agreement For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping, support both of the following options - Option 1: UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit FH SRS for positioning. - FFS details of an UL time window - Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning - Option 2: new collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels/. Option 2 can apply without or outside UL time window (i.e. option 1) - FFS: details on the collision rules Note: it is understood that option 2 is a component of the feature for UL SRS Tx hopping (FG 41-5-2), and option 1 is a separate feature group. Note: UE is not expected to be configured with a SRS for positioning hopping cycle, including the retuning time required ahead of the first hop and after the last hop, partially overlapping with UTW.

Proposal 5.2.1-1:

| Company | Comment |
| --- | --- |
| InterDigital | Ok to start with the TP. |
| LGE | Support. This should be discussed, otherwise there would be an ambiguity of UE behavior for the collision between the retuning time and the other UL signal/channel outside UTW. |
| FL | Seems it does not seem to be a major concern, we can agree on it online and get working on the TP. |

### [CLOSED] UL time window activation/deactivation mechanisms

#### Background

In [7,9], dynamic activation mechanisms for UTW are proposed:

| Company | Proposal |
| --- | --- |
| [7] | Proposal 3: UL time window activation/deactivation mechanisms (e.g., though DCI or MAC CE) should be supported. |
| [9] | Proposal 5: Support serving gNB to activate/deactivate the UTW by MAC CE. |

#### Round 1

From the FL perspective, this is not a critical issue to be resolve and could be seen as an enhancement. However, we can collect some views on the proposal:

Proposal 5.2.2-1 support activation/deactivation of the UL time window via

- Alt1: DCI

- Alt2: MAC-CE

Proposal 5.2.2.2-1:

| Company | Comment |
| --- | --- |
| vivo | It seems to be enhancement, we prefer not to pursue in R18. |
| InterDigital | It’s too late at this stage to discuss this proposal. |
| ZTE | We do not support any more enhancement for the UL time window. |
| LGE | We are okay with both Alts for the gNB scheduling flexbility. |
| FL | Let’s close the issue, since there are already 3 company opposing. |

### [CLOSED] UL time window location for aperiodic SRS

#### Background

In [7] it is proposed to discuss how the UTW can be placed for aperiodic SRS. We have previously agreed a semi-static UTW which is periodic. This proposal proposes to dynamically place the UTW for the special case of aperiodic SRS. From the FL perspective, this is a non-critical optimization.

| Company | Proposal |
| --- | --- |
| [7] | Proposal 4: For aperiodic SRS transmission, the following two schemes to determine the UL time window can be considered. - Configure the length of the uplink transmission window through high-level parameters, and DCI indicates the starting position of the uplink transmission window. - UE determines the starting position and length of the uplink time window based on the SRS resource time position in the SRS resource set triggered by DCI. |

#### Round 1

Proposal 5.2.3-1: For aperiodic SRS transmission, the following two schemes to determine the UL time window can be considered.

- Configure the length of the uplink transmission window through high-level parameters, and DCI indicates the starting position of the uplink transmission window.

- UE determines the starting position and length of the uplink time window based on the SRS resource time position in the SRS resource set triggered by DCI.

Proposal 5.2.3-1:

| Company | Comment |
| --- | --- |
| vivo | We prefer not to pursue in R18 |
| ZTE | Do not support |
| LGE | Support By dynamically scheduling the UTW, the gNB can have advantages in terms of flexibility in scheduling the UTW and minimizing unnecessary dropping of other UL signals/channels due to the UTW. |
| FL | Let’s close the issue, since there are already 2 company opposing expressing concerns about it being non-essential. |

### [CLOSED] UL time window and Measurement gaps

#### Background

In [7] it is proposed that when the UL time window overlaps with a measurement gap, the window is not valid.  This proposal was already captured during previous meetings but not discussed during offline sessions.

| Company | Proposal |
| --- | --- |
| [7] | Proposal 5: When the UL time window overlaps with the measurement gap, the UL time window becomes invalid. |

#### Round 1

Proposal 5.2.4-1: When the UL time window overlaps with the measurement gap, the UL time window becomes invalid.

Proposal 5.2.4-1:

| Company | Comment |
| --- | --- |
| ZTE | Both UL time window and measurement gap are configured by gNB, we do not think the case in Proposal 5.2.4-1 is valid. It is up to gNB’s implementation. |
| LGE | Same view with ZTE. |
| FL | Since it has been pointed that the issue can be avoided by implementation, let’s not discuss it further – it is not essential enough. |

### [closed] UL time window request

#### Background

[9] proposes to have LMF signaling to request UTW for a UE.

| Company | Proposal |
| --- | --- |
| [9] | Proposal 4: Support LMF to request UTW for a UE, and serving gNB to configure UTW for the UE through RRC signaling. |

#### Round 1

Proposal 5.2.5-1: Support LMF to request UTW for a UE, and serving gNB to configure UTW for the UE through RRC signaling.

Proposal 5.2.5-1:

| Company | Comment |
| --- | --- |
| vivo | We prefer not to pursue in R18 |
| ZTE | Do not introduce any more enhancement. |
| LGE | Okay |

### [MEDIUM] UL time window configuration

#### Background

In [17] it is proposed to clarify that only UL slot are applicable for the UTW. In [18] the configuration of the UTW is within the part configuring the SRS for positioning with frequency hopping, also refered sometimes as the “virtual BWP”.

| Company | Proposal |
| --- | --- |
| [17] | Proposal 3: A UE expects that UL time window is configured only within UL slot(s). |
| [18] | Proposal 5: For to the UL time window for SRS frequency hopping, - With regards to the configuration of the window: - It is part of the configuration that includes the SRS for positioning with frequency hopping |

#### Round 1

Proposal 5.2.6-1: The UTW is configured as part of the configuration that includes the SRS for positioning with frequency hopping

- The UTW only applies to UL slots

Proposal 5.2.6-1:

| Company | Comment |
| --- | --- |
| Vivo | Does it mean the SRS transmission in flexible symbols within UTW will follow the priority rule outside UTW? |
| ZTE | How about the flexible symbols? |
| LGE | To vivo, ZTE: Our intention of this proposal is to prevent the UE from having ambiguity about DL signal/channel within UTW. If we support to configure UTW within flexible slot as well, there also could be ambiguity when DL signal/channel is configured in the flexible slot. That’s why we suggest to configure UTW only within UL slot(s) for the simplest solution. We are open to further discuss about flexbile slot/symbol(s) for this issue. |

### [MEDIUM] UE behaviour in UTW with no SRS transmission

#### Background

This topic was already seen in previous meeting.  In [13,15,17] the case of an “empty” UTW is discussed, and it is proposed in that case that UL transmission of other signal is allowed in the empty UTW.

| Company | Proposal |
| --- | --- |
| [13] | Proposal 5: UL signals or channels can be transmitted during the UL time window if SRS for positioning is not scheduled to be transmitted during the UL time window. ` |
| [15] | Proposal 2: Within a UTW, if no SRS is to be transmitted, other UL channels can be transmitted. |
| [17] | Proposal 5: Within the UTW, UE can transmit other uplink transmission in slot(s) where no actual transmission of SRS-pos with frequency hopping is configured/indicated. |

#### Round 1

Proposal 5.2.7-1: UL signals or channels can be transmitted during the UL time window if SRS for positioning is not scheduled to be transmitted during the UL time window.

Proposal 5.2.7-1:

| Company | Comment |
| --- | --- |
| vivo | More clarification is needed, in this case, why we need configure UTW? |
| InterDigital | Support. We agreed to support aperiodic SRS for positioning and semi-static SRS for positioning with frequency hopping in RAN1#114b. UTWs are configured semi-statically. This situation will create unused UTWs. As the FL summarized, the proposal is to use “empty” UTW for transmission of UL channels. |
| ZTE | Do not agree, it is not aligned with our previous agreement. |
| LGE | Support. Similar view with InterDigital. Periodic UTW would be hard to cover the AP/SP SRS for positioning with Tx hopping transmission and this can cause waste of resource. Our intention of UTW was to protect the SRS for positioning with Tx hopping, not to prevent the transmission of other UL signal/channel. We don’t think there is reason to prevent the transmission of other UL signal/channel within ‚empty‘ UTW where there is no SRS for positioning with Tx hopping to be protected. |

### [LOW] SCS for UTW

#### Background

In [17] it is propose to link the SCS within the UTW to the minimum SCS among the SCSs of all configured BWP.

From the FL side, the SCS for the SRS with tx hopping is configured separately from the UL BWPs for data. Therefore, it feels natural that the UTW will use the same SCS as the SRS for tx hopping.

| Company | Proposal |
| --- | --- |
| [17] | Proposal 4: SCS of UTW is determined with respect to the minimum SCS among the SCSs of all configured UL BWPs. |

#### Round 1

Proposal 5.2.8-1: SCS of UTW is determined with respect to the minimum SCS among the SCSs of all configured UL BWPs.

Proposal 5.2.8-1:

| Company | Comment |
| --- | --- |
| vivo | Share the similar view as FL. |
| ZTE | UTW can be configured under the configuration of SRS Tx hopping and share the same SCS. |
| LGE | We would like to discuss proposal 5.1.5-1 first before discussing this proposal. If proposal 5.1.5-1 is agreeded, then we agree with FL’s comment. But if not, we think there will be an ambiguity of UE behavior when the multiple SRS resources are configured within single UTW. |

## Additional Collision rules (option 2)

### Background

### [closed] condition for returning to active BWP between hops

#### Background

In [10] it is proposed to discuss the conditions for returning to the active BWP between hops. From the FL side, it does not seem necessary to specify such conditions. Instead, it should be enough to have the dropping and collision rules specified, and let the UE return to the active BWP accordingly.

| Company | Proposal |
| --- | --- |
| [10] | Proposal 5: To handle the collision between gap and UL channel/signals, support UE switches back to the activate BWP only when both the two conditions meets: 1) The time between hop and the UL channel/signals exceed the retuning time to the active BWP 2) The time between the UL channel/signals and hop exceed the retuning time from the active BWP |

#### Round 1

We can check the views on the proposal:

Proposal 5.3.1-1 To handle the collision between gap and UL channel/signals, support UE switches back to the activate BWP only when both the two conditions meets:

1) The time between hop and the UL channel/signals exceed the retuning time to the active BWP

2) The time between the UL channel/signals and hop exceed the retuning time from the active BWP

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.3.1-1

| Company | Comment |
| --- | --- |
| ZTE | Does the above two conditions mean the gap between two hop is no less than the sum of switching time to and from active BWP? |
| LGE | Same view with FL. It does not seem necessary. To ZTE: In my understanding, the intention of this proposal is to exclude the cases that time between hop and UL signal/channel is less than the switching time to or from active BWP, even though the gap between two hop is no less than the sum of switching time to and from active BWP. |
| FL | After a couple of bilateral discussion offline, maybe we should clarify the behaviour oft he UE between hops, as it does not seem that the view is very aligned. Let’s try to bring the proposal offline. |

#### Conclusion

Agreement The UE is expected to switch back to the active BWP when the time between two consecutive hops exceeds twice the switching time to/from the BWP. -Note: this is applicable when UTW is configured or not configured.

### [closed] Collisions with MIMO SRS

#### Background

[11] and [18] discuss the collision with MIMO SRS.  [11] discuss the special case of collision while the SRS with tx hopping is transmitting outside of the BWP, and propose that in that case SRS is considered low priority.

| Company | Proposal |
| --- | --- |
| [11] | Proposal 4: For RedCap UEs positioning transmitting the positioning SRS with frequency hopping, regarding the collisions between other UL signals/channels including MIMO SRS and the positioning SRS with frequency hopping, - If the positioning SRS hop is within the active BWP, the existing dropping rule for RRC_CONNECTED state is reused. - If the positioning SRS hop is outside the active BWP, Rel-17 defined rule for SRS outside initial BWP in RRC_INACTIVE state is reused, i.e. SRS has low priority. |
| [18] | Proposal 6: For the collisions between SRS for Positioning with frequency and MIMO SRS: • The existing rules can be applied with the additional clarification that SRS symbols include the retuning time to/from the active BWP. |

#### Round 1

We can check the views on the proposal from [11]:

Proposal 5.3.2-1  For RedCap UEs positioning transmitting the positioning SRS with frequency hopping, regarding the collisions between other UL signals/channels including MIMO SRS and the positioning SRS with frequency hopping,

•	If the positioning SRS hop is within the active BWP, the existing dropping rule for RRC_CONNECTED state is reused.

•	If the positioning SRS hop is outside the active BWP, Rel-17 defined rule for SRS outside initial BWP in RRC_INACTIVE state is reused, i.e. SRS has low priority.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.3.2-1

| Company | Comment |
| --- | --- |
| vivo | Regarding collision with MIMO SRS, the existing rules can be applied with the additional clarification that SRS symbols include the retuning time to/from the active BWP. So, the TP 2.8-1 can be adopted instead of this proposal. |
| ZTE | Support. |
| LGE | Same view with vivo. |

### [LOW] SRS with tx hopping priorities

#### Background

In [9] it is proposed to introduce priority levels for the SRS with tx hopping:

| Company | Proposal |
| --- | --- |

| [9] | Proposal 6: Support the following collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels. • UE can be configured with two priority states. − State 1: UL SRS is higher priority than all PUCCH/PUSCH/CSI-RS. − State 2: UL SRS is lower priority than all PUCCH/PUSCH/CSI-RS. |
| --- | --- |

#### Round 1

We can check the views on the proposal from [9]. From the FL perspective, this is not an essential item but we can at least gather some comments:

Proposal 5.3.3-1  For SRS with Tx hopping, the UE can be configured with two priority states.

−	State 1: UL SRS is higher priority than all PUCCH/PUSCH/CSI-RS.

−	State 2: UL SRS is lower priority than all PUCCH/PUSCH/CSI-RS.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.3.3-1

| Company | Comment |
| --- | --- |
| vivo | It is configured for UTW? |
| ZTE | Not clear about the intention of this proposal |
| LGE | We can see the intention of this proposal but we think this intention might be satisfied by the priority rule of UTW suggested in the proposal 5.2.7-1 |
| Ericsson | This is a new enhancements and we don’t support it at this stage. |

### [closed] timeline for A-SRS

#### Background

The timeline for validation of A-SRS is discussed in [6,15].

| Company | Proposal |
| --- | --- |
| [6] | Proposal 5 - For aperiodic positioning SRS with frequency hopping, switching time is included in the minimal time interval between the last symbol of PDCCH triggering A-SRS and the first symbol of SRS resource in the first hop if the first hop is outside the active UL BWP. - Agree on TP#1 for the timeline of aperiodic positioning SRS with frequency hopping for RedCap UEs. |
| [15] | Proposal 1: For aperiodic SRS for positioning with Tx hopping, the UE should not expect the DCI that triggers SRS with frequency hopping to occur at a specified duration before the retuning time to the active BWP. |

#### Round 1

The issue needs to be settled to complete the aperiodic SRS design with Tx hopping. We can start the discussion based on the proposal in [6], and proceed with a TP once we have an agreement:

Proposal 5.3.4-1  For aperiodic positioning SRS with frequency hopping, switching time is included in the minimal time interval between the last symbol of PDCCH triggering A-SRS and the first symbol of SRS resource in the first hop if the first hop is outside the active UL BWP.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.3.4-1

| Company | Comment |
| --- | --- |
| vivo | Does it mean we change the minimal time interval as follows the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 symbols, an additional time duration Tswitch , and switching time from or to active BWP. |
| InterDigital | Support |
| ZTE | It seems that Proposal 5.3.5.2 includes this proposal. Prefer to treat it together. |
| LGE | OK |
| Ericsson | In our understanding, N2 between PDCCH and aperiodic SRS transmission is typically due to PDCCH processing. Since the UE decision to leave the BWP cannot be taken before decoding PDCCH, we think an additional time for switching must be added after N2. |

#### Conclusion

Agreement For aperiodic positioning SRS with frequency hopping, switching time to/from active UL BWP is added in the minimal time interval between the last symbol of PDCCH triggering A-SRS and the first symbol of the triggered SRS in the first hop.

### [closed] timeline for PUSCH collisions

#### Background

For the timeline for PUSCH collisions the proposals are as follow:

- Reuse the rule with at least N2 symbols between the end of PDCCH carrying the DCI for PUSCH transmission and the start of the SRS transmission

  - Supported by [2,12,16,18,20]

  - [12,18,20] includes also switching time added to the N2 symbols.

| Company | Proposal |
| --- | --- |
| [2] | Proposal 4: The timeline for determining the collision is defined as within the N2 symbols after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, i.e., if UE determines the presence of PUSCH that collides the SRS transmission within N2, the colliding SRS symbol(s) are dropped. ‐ Endorse the following TP to clause 6.2.1.4.1 of TS 38.214. |
| [11] | Proposal 6: Do not further introduce timeline for determination of colliding channels/signals. |
| [12] | Proposal 4: When SRS for Tx frequency hopping symbol(s), including the retuning time to/from the active BWP, collides with PUSCH, the UE should apply the collision rule considering that: • DCI for which the time interval between the last symbol of PDCCH and the starting symbol of the SRS for Tx frequency hopping is at least a processing time T. • UL channels/signals considered active at least a processing time T before the starting symbol of the SRS for Tx frequency hopping. where the processing time T include N2 symbols of preparation time and an additional time duration for switching from active BWP. |
| [16] | Proposal 2: when the UE determines the potential PUSCH/PUCCH transmission later than N symbol before a SRS transmission symbol, then such SRS transmission symbol is expected to be transmitted and the potential PUSCH/PUCCH is not required to transmit. |
| [18] | Proposal 7: Related to the timeline for the determination of SRS for positioning frequency hopping and colliding PUSCH or PUCCH channels: For an SRS transmission for positioning with frequency hopping starting in symbol N_(c_1 ) and a PUSCH or PUCCH transmission starting in symbol N_S, where T_(SRS_h ), is the retuning time to/from the active BWP, the UE shall apply the dropping rules taking into account: PUSCH or PUCCH channels determined to be colliding at least N_2 symbols and an additional time duration T_(SRS_h ) before N_(c_1 ), and determined to be colliding at least N_2 symbols before N_S. |
| [20] | Proposal 5 For the case of dynamic PUSCH controlled by UL DCI, the UE is not expected to transmit the PUSCH if the DCI scheduling the UL transmission is received less than Y symbols before the first symbol of SRS with Tx hopping, including symbols required for retuning. Proposal 6 For the case of CG PUSCH , the UE is not expected to transmit the PUSCH if the CG activating the UL transmission is received less than Y symbols before the first symbol of SRS with Tx hopping, including symbols required for retuning. |

#### Round 1

The issue seems straightforward. We can re-use the proposal form [18] to start the discussion.

Proposal 5.3.5-1: for the determination of collision between PUSCH and the SRS with tx hopping:

- For an SRS transmission for positioning with frequency hopping starting in symbol Nc1 and a PUSCH or PUCCH transmission  starting in symbol NS, where TSRSh, is the retuning time to/from the active BWP, the UE shall apply the dropping rules taking into account:

  - PUSCH or PUCCH channels determined to be colliding at least N2 symbols and an additional time duration TSRSh before Nc1, and determined to be colliding at least N2 symbols before NS.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.3.5-1

| Company | Comment |
| --- | --- |
| vivo | Not support. In RRC_INACTIVE state, there are still scheduled PUSCH/PDSCH and SRS transmission outside initial BWP with switching time, however, no timeline related collision rule was specified. For SRS frequency hopping, we don’t find too much difference from the rules in RRC_INACITVE. In addtion, we have some concerns about the potential benefit of timeline since the neighbour cell still needs to do a blind measurement to determine if SRS is present even with the timeline. In this case, why serving cell can not do blind measurement to determine SRS or data is present. |
| ZTE | Generally we prefer not to discuss any enhancement for the timeline of collision determination during maintenance phase and leave it up to implementation. |
| Ericsson | Support in Principle. For the part “and determined to be colliding at least N_2 symbols before N_S”, we are not sure is is necessary. What matters is the the UE has receive the information pertaining to scheduling ahead enough in time compared to the SRS, in our view. |

#### Conclusion

Agreement For the determination of collision between PUSCH or PUCCH and the SRS with tx hopping: - For a transmission of an SRS for positioning symbol with frequency hopping in a hop in symbol and a PUSCH or PUCCH transmissionstarting in symbol, where is the switching time to/from the active BWP, the UE shall apply the dropping rules taking into account: - DCI(s) for which the time interval between the last symbol of PDCCH and the SRS in symbol is at least ( + ) symbols - DCI(s) for which the time interval between the last symbol of PDCCH and the colliding PUSCH/PUCCH in symbol is at least N2 symbols - Note: as in legacy - For the calculation of N2, use the smallest SCS between the SCS configured for the SRS with tx hopping and the SCS of the PUSCH

### [closed] overlapping SRS resources with Tx hopping

#### Background

| Company | Proposal |
| --- | --- |
| [2] | Proposal 5: UE does not expect to be configured with more than one SRS frequency hopping resources overlap. - Endorse the following TP to clause 6.2.1.4.1 of TS 38.214. |
| [3] | Proposal 8: For the same resourceType of SRS-PosResource and SRS-Resource, the UE is not expected to be configured with SRS-Resource on the symbol(s) of an SRS resource configured with SRS-PosResource including retuning time. |
| [20] | Proposal 8 For SRS for positioning with Tx hopping, the UE is not expected to be configured with Tx hopping so that one SRS resource with Tx hopping transmission begins before the end of the preceding Tx hopping transmission. |

#### Round 1

The received proposals extend the exisiting rule in 38.214, ie. SRS resources should not overlap when they are of the same type. The main change is the resource may need to also include retuning time as proposed in [3]. We can start with the proposal in [3] and discuss a TP once we have an agreement:

Proposal 5.3.6-1:  For the same resourceType of SRS-PosResource and SRS-Resource, the UE is not expected to be configured with SRS-Resource on the symbol(s) of an SRS resource configured with SRS-PosResource including retuning time.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.3.6-1

| Company | Comment |
| --- | --- |
| vivo | OK. And the TP 2.8-1 in section 2.9 can be adopted accordingly. |
| ZTE | OK |
| LGE | OK |
| FL | Seem we can directly discuss this online and get an agreement on TP 2.9. |

## [MEDIUM] timing change within the SRS resource with tx hopping

### Background

Two proposals have been brought forward, in [3] when timing changes during the SRS transmission, the transmission is either interrupted, or the timing change is relayed to the gNB. In [17] the timing change is only applied once the ongoing SRS transmission is completed.

| Company | Proposal |
| --- | --- |
| [3] | Proposal 11: If the UE Tx timing is changed while performing Tx frequency hopping in the same SRS Tx hopping cycle/occasion, RAN1 supports either one of the following alternatives: Alt 1: The UE drops the remaining hops of SRS transmission. Alt 2: The UE does not drop the remaining hops of SRS transmission but reports information on the hops affected by the Tx timing change so that the gNB can selects hops to combine them. |
| [17] | Proposal 6: If UE is indicated to apply an adjustment for the uplink transmission timing at a slot where SRS for positioning with Tx hopping is configured/indicated, the UE is expected to apply a corresponding adjustment for the uplink transmission timing at the slot after the slot in which last hop is configured. |

#### Round 1

Proposal 5.4-1:  : When the UE Tx timing is to be changed while performing Tx frequency hopping in the same SRS Tx hopping cycle/occasion (downselect)

- Alt 1: The UE drops the remaining hops of SRS transmission.

- Alt 2: The UE does not drop the remaining hops of SRS transmission but reports information on the hops affected by the Tx timing change so that the gNB can selects hops to combine them.

- Alt3: the UE is expected to apply a corresponding adjustment for the uplink transmission timing at the slot after the slot in which last hop is configured.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.4-1

| Company | Comment |
| --- | --- |
| ZTE | Do not prefer to further discuss this issue. The impact of Tx timing change was also not treated in legacy positioning. |
| LGE | For the positioning with frequency hopping, if Tx timing is changed while performing Tx frequency hopping, there will be significant positioning accuracy performance degradation. We think it is needed to discuss this issue. We prefer Alt 3. To ZTE: In our understanding, there is a different between SRS for positioning with Tx hopping for RedCap UEs and legacy positioing: coherent combining vs non-coherent combining. For the coherent combining, Tx timing error between combined hops will significantly affect to positioing performance. |

## [MEDIUM] LMF request for SRS FH parameters

### Summary of contributions

In [12,18], proposals are put forward to include requested SRS tx hopping configuration parameters in the SRS LMF request.

| Company | Proposal |
| --- | --- |
| [18] | Proposal 8: An LMF should be able to request from the serving gNB, using the “Requested SRS Transmission Characteristics IE”, to configure an SRS for positioning with “SRS frequency hopping” configuration is needed. Proposal 9: An LMF should be able to request, using the “Requested SRS Transmission Characteristics IE”, • Total Bandwidth (reuse the existing field in NRPPa) • The amount of overlap between 2 consecutive hops, hopping bandwidth of each hop. |
| [12] | Proposal 5: For the LMF to request SRS Tx frequency hopping configuration for a UE, if the UE capability of supporting SRS Tx frequency hopping is provided to the LMF, support an indication to enable SRS Tx frequency hopping on top of the SRS total bandwidth. • Details are up to RAN3. |

### Round 1

Proposal 5.4-1: As part of the LMF request for SRS for positioning configuration, support the following to be included in the request:

- An indicator to request SRS with Tx hopping

- The amount of overlap between hops

- Total bandwidth across hops

  - The exisiting field for bandwidth in the SRS for positioning LMF request can be re-used

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.4-1:

| Company | Comment |
| --- | --- |
| vivo | OK |
| ZTE | This proposal is related with whether UE capabilities for SRS Tx hopping is reported to LMF. Prefer to delay the discussion and wait fot the outcome of UE feature. |

## [MEDIUM] SRS transmission priority for Multi-RTT with Rx/Tx hopping

### Summary of contributions

In [3] a text proposal is put forward to add a condition that when configured for PRS measurement performed for Multi-RTT the UE should prioritize SRS transmission.

| Company | Proposal |
| --- | --- |
| [3] | Proposal 7: RAN1 supports the following text proposal on Clause 6.2.1.4.1 of TS 38.214. ##### 6.2.1.4.1 SRS frequency hopping for positioning <omitted text> The reduced capability UE may be configured, via [higher layer parameter], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is not expected to be configured with one [cycle] of the transmit frequency hopping that is partially overlapped with the time window. If the reduced capability UE is expected to perform a UE Rx-Tx time difference measurement and uses receiver frequency hops of a DL PRS resource, transmission of SRS using transmit frequency hopping on an SRS resource has higher priority than other channels and signals. <omitted text>. |

### Round 1

As the issue was never discussed, we should first capture some more comments on the proposed TP to see whether to move forward.

Issue 3.3:

| Company | Comment |
| --- | --- |

# UE features

The following proposals in [3,10] discuss issues suitable for UE feature discussion in AI 8.16.3.

| Company | Proposal |
| --- | --- |
| [3] | Proposal 2: RAN1 supports that the UE reports the required number of symbols per frequency hop of DL PRS resources per TRP considering the coverage performance. Proposal 3: UE provides the LMF with the maximum number of frequency hops that it can measure, to guarantee the measurement error less than a certain threshold considering UE mobility, for a given DL PRS resource configuration including repetitions. Proposal 4: UE provides the LMF with the maximum number of frequency hops that it needs to measure to guarantee a certain level of the measurement accuracy for a given DL PRS resource configuration including repetitions. |
| [10] | Proposal 1: For frequency hopping of positioning RS, support UE reporting the capability related to the phase offset to facilitate the overlap size’s configuration. |

# Other issues

| Company | Proposal |
| --- | --- |
| [6] | Proposal 8 - For HD-FDD RedCap UE, collision handling between DL PRS and UL channels/signals within a configured PPW needs to be addressed at least for the case where DL PRS is configured without Rx frequency hopping. FL note: there is one TP in [2] proposal 3 that may resolve the issue Proposal 2 • For DL PRS with Rx frequency hopping for RedCap UEs, only MG-based measurement is supported. FL note: at this stage of the WI hopefully we do not need to have a conclusion on this issue. |
| [8] | - For RedCap UEs positioning, support the SRS frequency hopping across multiple BWPs within one SRS resource. FL note: this contradict the exisiting agreements to hop within a resource. |
| [14] | Proposal 2: • RAN1 should make the explicit conclusion to support frequency hopping for eRedCap UEs FL note: while eRedCap support is not precluded, it is not clear what RAN1 needs to do to support these UEs. as we are in the maintenance phase, we should not start any new issue. |
| [20] | Proposal 1 For measurements using Rx hopping, do not support M=1 sample measurements in Rel-18. Proposal 2 For measurements using Rx hopping, do not consider the use of TEG framework. Proposal 3 (for conclusion, if needed) RAN1 will not discuss further the case of no overlap in DL PRS Rx hopping. |

# Offline sessions

## Monday Morning Offline

No consensus was reached in the offline discussion.

## Wednesday Morning Offline

(no more discussion on this in RAN1#115)

Proposal 5.1.4-1  for the SRS for positioning with Tx hopping:

- The hop index is in the time domain

- Hop index in time domain can be configured together with the starting slot offset and starting symbol for each hop.

- The slot offset and symbol offset for each hop are such that the order of transmission of the hops in time follows the hop indices (e.g., hops are transmitted starting with 0 then 1,2,3….)

During the online session, 3 proposals were discussed to try to break the logjam:

Proposal1 (Alt2) offline consensus For SRS for positioning with Tx hopping n0 is the initial frequency hop index defined as n_0=floor(n_FirstHop^RB/(m_hop^SRS-m_overlap^hop)) - No new parameter is defined Proposal2 (Alt1) For SRS for positioning with Tx hopping, the initial frequency hop index n0 is configured by RRC in the range [0, number of hops -1]. Proposal3 For SRS for positioning with Tx hopping, the initial frequency hop index n_0 can be optionally configured by RRC in the range [0, number of hops -1]. - If not configured, n_0=floor(n_FirstHop^RB/(m_hop^SRS-m_overlap^hop))

From the FL perspective, a choice has to be made. Otherwise, we will miss the RRC deadline and by default the solution will be proposal 1 above. The majority of companies support not to introduce new parameters.

To clarify the situation, please add your companies to the supported alternative in the list below:

- Proposal 1 (no new parameter): FW, QC,MTK,IDG,CATT,intel, E///,DCM,ZTE,IIT,apple,HW

- Proposal 2 (new parameter): LGE, vivo, samsung

- Proposal 3 (new parameter if configured, otherwise n0 is defined as per proposal 1):

  - HW, LGE,

## Thursday morning

UTW with no SRS

Proposal 5.2.7-1: UL signals or channels can be transmitted during the UL time window if SRS for positioning is not scheduled to be transmitted during the UL time window.

LMF request

Proposal 5.2.5-1: Support LMF to request UTW for a UE, and serving gNB to configure UTW for the UE through RRC signaling.

Timing changes

Proposal 5.4-1:  : When the UE Tx timing is to be changed while performing Tx frequency hopping in the same SRS Tx hopping cycle/occasion (downselect)

- Alt 1: The UE drops the remaining hops of SRS transmission.

- Alt 2: The UE does not drop the remaining hops of SRS transmission but reports information on the hops affected by the Tx timing change so that the gNB can selects hops to combine them.

- Alt3: the UE is expected to apply a corresponding adjustment for the uplink transmission timing at the slot after the slot in which last hop is configured.

# Online sessions

## Monday Morning Offline

Proposal 5.1.1-1 for SRS for positioning with Tx hopping  n0 is the initial frequency hop index defined as  n0=floornFirstHopRB/( mhopSRS-moverlaphop)

- No new parameter is defined

Proposal 5.1.3-1  For the values of the starting slot offset for each of the hops following the first hop in time:

- Alt1: the value range can be {0,1,2…, nrof slot in periodicity -1} in slots for the slot offset.

- Alt2: the slot offset for each hop is relative to the preceding hop with range (0,1,2…,6)

- The value range slot offset for each hop applies to both the periodic and semi-persistent SRS.

- The periodicity in PeriodicityandOffset configured for each hop for a SRS resource with Tx hopping must be the same.

Proposal 5.1.4-1  for the SRS for positioning with Tx hopping:

- The hop index is in the time domain

- Hop index in time domain can be configured together with the starting slot offset and starting symbol for each hop.

Proposal 5.1.5-1 The Configuration of SRS for positioning with Tx hopping including SCS, CP size and reference point for bandwidth determination is common to all configured SRS for positioning with Tx hopping resource(s).

- The configuration for positioning SRS with frequency hopping is configured independently from any data BWP.

TP that could be endorsed:

Proposal: endorse the TP 2.4-1 in section 2.5.1 of R1-2312343

Proposal: endorse the TP 2.6-1 in section 2.7.1 of R1-2312343

Proposal: endorse the TP 2.7-1a in section 2.8.1 of R1-2312343

## Tuesday session

SRS configuration:

14 contributions for alt 2, 3 for alt1.

Alt2 imposes that the starting PRB is at most Nhop from the lowest available subcarrier, otherwise n0>Nhop

Proposal 5.1.1-1 (alt2) for SRS for positioning with Tx hopping  n0 is the initial frequency hop index defined as  n0=floornFirstHopRB/( mhopSRS-moverlaphop)

- No new parameter is defined

Slot offset for hops after the first hop:

Proposal 5.1.3-1  For the values of the starting slot offset for each of the hops following the first hop in time:

- Alt1: the value range can be {0,1,2…, nrof slot in periodicity -1} in slots for the slot offset.

- Alt2: the slot offset for each hop is relative to the preceding hop with range (0,1,2…,6)

- The value range slot offset for each hop applies to both the periodic and semi-persistent SRS.

- The periodicity in PeriodicityandOffset configured for each hop for a SRS resource with Tx hopping must be the same.

Proposal 5.1.4-1  for the SRS for positioning with Tx hopping:

- The hop index is in the time domain

- Hop index in time domain can be configured together with the starting slot offset and starting symbol for each hop.

- The slot offset and symbol offset for each hop are such that the order of transmission of the hops in time follows the hop indices (e.g., hops are transmitted starting with 0 then 1,2,3….)

Proposal 5.1.5-1 The Configuration of SRS for positioning with Tx hopping including SCS, CP size and reference point for bandwidth determination is common to all configured SRS for positioning with Tx hopping resource(s).

- The configuration for positioning SRS with frequency hopping is configured independently from any data BWP.

UTW:

Proposal 5.2.1-1:

Support to update the agreement as following:

Agreement For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping, support both of the following options - Option 1: UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit FH SRS for positioning. - FFS details of an UL time window - Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning - Option 2: new collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels/. Option 2 can apply without or outside UL time window (i.e. option 1) - FFS: details on the collision rules Note: it is understood that option 2 is a component of the feature for UL SRS Tx hopping (FG 41-5-2), and option 1 is a separate feature group. Note: UE is not expected to be configured with a SRS for positioning hopping cycle, including the retuning time required ahead of the first hop and after the last hop, partially overlapping with UTW.

Proposal 5.2.6-1: The UTW is configured as part of the configuration that includes the SRS for positioning with frequency hopping

- The UTW only applies to UL slots does not apply to downlink symbols within the window duration, i.e. only the UL and flexible symbol are affected by the UTW prioritization.

Collisions:

Proposal 5.3.1-1 To handle the collision between gap and UL channel/signals, support UE switches back to the activate BWP only when both the two conditions meets:

1) The time between hop and the UL channel/signals exceed the retuning time to the active BWP

2) The time between the UL channel/signals and hop exceed the retuning time from the active BWP

For the mimo SRS collision, at least the collision rules while the hopping occurs outside of the active BWP are needed:

Proposal 5.3.2-1  For RedCap UEs positioning transmitting the positioning SRS with frequency hopping, regarding the collisions between other UL signals/channels including MIMO SRS and the positioning SRS with frequency hopping,

•	If the positioning SRS hop is within the active BWP, the existing dropping rule for RRC_CONNECTED state is reused.

•	If the positioning SRS hop is outside the active BWP, Rel-17 defined rule for SRS outside initial BWP in RRC_INACTIVE state is reused, i.e. SRS has low priority.

Proposal 5.3.4-1  For aperiodic positioning SRS with frequency hopping, switching time is included in the minimal time interval between the last symbol of PDCCH triggering A-SRS and the first symbol of SRS resource in the first hop if the first hop is outside the active UL BWP.

Proposal 5.3.5-1: for the determination of collision between PUSCH and the SRS with tx hopping:

- For an SRS transmission for positioning with frequency hopping starting in symbol Nc1 and a PUSCH or PUCCH transmission  starting in symbol NS, where TSRSh, is the retuning time to/from the active BWP, the UE shall apply the dropping rules taking into account:

  - PUSCH or PUCCH channels determined to be colliding at least N2 symbols and an additional time duration TSRSh before Nc1, and determined to be colliding at least N2 symbols before NS.

Proposal 5.3.6-1:  For the same resourceType of SRS-PosResource and SRS-Resource, the UE is not expected to be configured with SRS-Resource on the symbol(s) of an SRS resource configured with SRS-PosResource including retuning time.

TP that could be endorsed:

Proposal: endorse the TP 2.4-1 in section 2.5.1 of R1-2312343

Proposal: endorse the TP 2.6-1 in section 2.7.1 of R1-2312343

Proposal: endorse the TP 2.7-1a in section 2.8.1 of R1-2312343

## Wednesday session

Proposal 3.1-1: (offline consensus)

For measurements based on DL PRS with Rx frequency hopping or UL SRS with Tx hopping:

- UE/gNB can report either a single-hop or multi-hop measurement.

- Indication of which of a single-hop or multi-hops measurement is (optionally) reported.

  - Note: impact on /mapping of the indicator to performance requirement(s) is up to RAN4.

Proposal1 (Alt2) offline consensus

For SRS for positioning with Tx hopping n0 is the initial frequency hop index defined as n_0=floor(n_FirstHop^RB/(m_hop^SRS-m_overlap^hop))

- No new parameter is defined

## thursday session

(Offline consensus)

Proposal 5.3.4-1  For aperiodic positioning SRS with frequency hopping, switching time to/from active UL BWP is added in the minimal time interval between the last symbol of PDCCH triggering A-SRS and the first symbol of the triggered SRS in the first hop.

Offline consensus

Proposal 5.3.5-1: for the determination of collision between PUSCH or PUCCH and the SRS with tx hopping:

- For a transmission of an SRS for positioning symbol with frequency hopping in a hop in symbol Nc1 and a PUSCH or PUCCH transmission  starting in symbol NS, where TSRSh is the switching time to/from the active BWP, the UE shall apply the dropping rules taking into account:

  - [PUSCH or PUCCH channels determined to be colliding]

    - at least (N2 + TSRSh) symbols before the SRS is transmitted Nc1

    - at least N2 symbols before NS.

  - For the calculation of N2, use the smallest SCS between the SCS configured for the SRS with tx hopping and the SCS of the PUSCH

Rewording for  [PUSCH or PUCCH channels determined to be colliding]

Proposal 5.3.5-1b: for the determination of collision between PUSCH or PUCCH and the SRS with tx hopping:

- For a transmission of an SRS for positioning symbol with frequency hopping in a hop in symbol Nc1 and a PUSCH or PUCCH transmission  starting in symbol NS, where TSRSh is the switching time to/from the active BWP, the UE shall apply the dropping rules taking into account:

  - DCI(s) for which the time interval between the last symbol of PDCCH and the SRS in symbol Nc1 is  at least (N2 + TSRSh) symbols

  - the time interval between the last symbol of PDCCH and the PUSCH in symbol NS.is at least N2 symbols

  - For the calculation of N2, use the smallest SCS between the SCS configured for the SRS with tx hopping and the SCS of the PUSCH

Offline consensus

Proposal 5.3.1-1 The UE is expected to switch back to the active BWP when the time between two consecutive hops exceeds twice the switching time to/from the BWP.

-FFS case when UTW is configured.

Proposal 5.1.2-1 (for conclusion) Comb offset hopping is not supported for positioning SRS with frequency hopping for RedCap UEs.

Proposal: endorse the TP 2.9-1 in section 2.10.1 of R1-2312344

Proposal: endorse the TP 2.10-1b in section 2.11.1 of R1-2312344

Proposal: endorse the TP 2.12-1 in section 2.13.1 of R1-2312344

Proposal: endorse the TP 2.15-1 in section 2.16.1 of R1-2312344

Proposal: endorse the TP 2.16-1 in section 2.17.1 of R1-2312344

Proposal: endorse the TP 2.17-1b in section 2.18.1 of R1-2312344

## Friday session

Offline consensus

Proposal: endorse the TP 2.8-1b in section 2.9.1 of R1-23NNNN

Offline consensus

Proposal 5.2.1-1:

Support to update the agreement as following:

Agreement For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping, support both of the following options - Option 1: UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit FH SRS for positioning. - FFS details of an UL time window - Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning - Option 2: new collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels/. Option 2 can apply without or outside UL time window (i.e. option 1) - FFS: details on the collision rules Note: it is understood that option 2 is a component of the feature for UL SRS Tx hopping (FG 41-5-2), and option 1 is a separate feature group. Note: UE is not expected to be configured with a SRS for positioning hopping cycle, including the switching time from/to active BWP required ahead of the first hop and after the last hop, partially overlapping with UTW.

Proposal: endorse the TP 2.1-1 in section 2.2.1 of R1-2312652

Proposal: endorse the TP 2.16-1b in section 2.17.1 of R1-2312652

# Conclusion

# References

R1-2310823, On remaining open issues and maintenance for RedCap UE Positioning, FUTUREWEI

R1-2310842, Maintenance of RedCap positioning, Huawei, HiSilicon

R1-2310981, Remaining issues on Positioning for RedCap UEs, Nokia, Nokia Shanghai Bell

R1-2310989, Remaining issues of positioning for RedCap UEs, New H3C Technologies Co., Ltd.

R1-2311100, Remaining issues on positioning for RedCap UEs, vivo

R1-2311146, Remaining details of Positioning for RedCap Ues, Intel Corporation

R1-2311168, Remaining issues on positioning for RedCap UEs, Spreadtrum Communications

R1-2311227, Remaining issues of positioning for RedCap UEs, OPPO

R1-2311345, Maintenance issues on positioning for RedCap UEs, CATT

R1-2311418, Remaining issues on positioning for RedCap UEs, NEC

R1-2311462, Maintenance on Positioning for RedCap UEs, ZTE

R1-2311485, Maintenance on RedCap UE positioning, CMCC

R1-2311601, Remaining issues on positioning for RedCap UEs, InterDigital, Inc.

R1-2311625, Remaining issues on positioning for RedCap UEs, NTT DOCOMO, INC.

R1-2311687, Remaining Issues On Positioning for RedCap UEs, Apple

R1-2311847, Maintenance on Positioning for RedCap UEs, Samsung

R1-2311913, Remaining issues on positioning support for RedCap UEs, LG Electronics

R1-2312039, Maintenance for Positioning for Reduced Capabilities UEs, Qualcomm Incorporated

R1-2312094, Maintenance for RedCap UE for positioning, MediaTek Korea Inc.

R1-2312191, Remaining issues on positioning for RedCap Ues, Ericsson