3GPP TSG RAN WG1 #116	R1-2400962

Athens, Greece, February 26th – March 1st, 2024

Agenda item:		9.6.1

Title:			LP-WUS and LP-SS design

Source: 			Nokia, Nokia Shanghai Bell

Document for:		Discussion and Decision

# Introduction

In this document, we will discuss in detail about our view and proposals on the design of LP-WUS and LP-SS design for wake-up signal. The proposal for both LP-SS and LP-WUS are made under two underlying assumptions, namely, the complexity of the gNB transmission and the low power nature of the receiving device, i.e., LR.

# Agreements in WID

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol. - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported. - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping, and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2.

# Waveform Design

## Signal bandwidth and the location within carrier BW

One elementary parameter for the LP-WUS design is the bandwidth of the signal as it will affect many aspects of the LP-WUS operation and LR design. Assuming higher bandwidth would allow higher signal power, but also result more noise power, so the receiver SNR may remain unchanged, thus the typically considered receivers may not benefit from this in terms of performance. However, by increasing the bandwidth, robustness against narrowband interference and fading can be improved. Also, higher bandwidth per (e.g., OOK) symbol may result in lower spectrum efficiency, unless we use DFT-s-OFDM to improve the utilization by transmitting fewer OOK symbols within a NR CP-OFDM symbol.

Wider LP-WUS/LP-SS BW can offer robustness against fading but impacts the spectrum efficiency.

On other hand Rel-17 RedCap devices are restricted to a RF/BB bandwidth of 20MHz and introduction of enhanced support of RedCap devices with further restricted BW, down to 5MHz, has been specified for Rel-18, thus limited LP-WUS BW could also enable wider adoption. Limiting the BW of LP-WUS transmission could enable wider use of LP-WUS in different type of deployments, with different transmission bandwidths, down to 5MHz and even below, which are currently supported in NR. Therefore, the trade-off between LP-WUS BW and e.g. data rate should be also considered.

The scalability of LP-WUS/LP-SS BW would benefit multitude of device types and deployments.

Consider scalable design for LP-WUS/LP-SS to support multiple BW options depending on the deployment scenario. For initial evaluations, 5MHz LP-WUS BW can be used.

The position of LP-WUS/LP-SS transmissions within the carrier should be flexible, i.e., can be configurable by the gNB, which can then be informed to the MR through unicast or broadcast messages. This way, the spectral utilization can be optimized by the NW.

The position of LP-WUS/LP-SS within the carrier BW should be flexible and configurable by the NW.

## Modulation Type

In WID, two modulations were proposed for OOK signalling, namely, OOK1 and OOK4. Even though they both fall under the ON/OFF keying structure, the way the signal is generated within the underlying OFDM framework of NR introduces subtlety in the time domain (TD). We will discuss briefly about the two schemes and list out the highlighting differences.

### OOK1

In this scheme, the modulation order, denoted as M, is always fixed to M=1, without having DFT spreading to generate the waveform. However, if the modulation order has to be increased, the only way to do so is to increase the LP-WUS SCS higher than NR SCS, i.e., μWUS>μNR, i.e., to get four bits (equivalent to M=4) in OOK1, the SCS of LP-WUS must be M×2μNR=2μWUS. Thus, to achieve higher modulation order, the SCS of LP-WUS should be increased by M times to that of NR SCS. This will introduce additional overhead at the gNB implementation as this may introduce leakage from LP-WUS transmission to adjacent NR transmissions.

| (a) Without pulse shaping, i.e., rectangular window | (b) Gaussian pulse shaping for LP-WUS signal |
| --- | --- |

Fig. 1 Impact of different LP-WUS SCS on legacy UEs receiving NR at 15KHz SCS (with and without Filtering)

The leakage caused by the higher SCS used for LP-WUS compared to adjacent NR transmission may cause interference to the legacy NR receivers, since the SCS of the NR transmission defines the FFT length. To evaluate the impact of using different SCS for LP-WUS compared to NR transmission, we evaluate the impact by considering a 5MHz LP-WUS spectrum using different SCS, namely, 15, 30, 60, and 120 on the NR signal using 15KHz SCS. The LP-WUS BW used in this modeling is 4.32MHz with the rest of whole 20MHz BW is set to zeros to study the leakage observed by the NR receiver. Fig. 1 illustrates the spectral leakage experienced by the legacy NR receiver due to LP-WUS transmission performed with SCS different from the NR signal. In this experiment, each bit of Manchester encoded LP-WUS payload is transmitted over a CP-OFDM symbol spanning 4.32MHz BW. Additional GB of 320KHz is provided on either side of LP-WUS transmission to relax the filtering requirements at LR. Note that only LP-WUS BW has the signal, and the rest of the tones are set to zeros. z

Fig. 2 Generation of OOK1 with higher SCS using TD insertion.

LP-WUS waveform is created by populating one part of Manchester encoded bit to a CP-OFDM symbol occupying LP-WUS BW of 4.32MHz in the respective SCS numerology. Once the time domain waveform is created for all symbols, then, depending on the LP-WUS SCS, 2,4, or 8 LP-WUS symbols are grouped for each FFT process at the NR receiver using 15KHz SCS, respectively for SCS 30,60, and 120KHz. Fig. 1a demonstrates the spectral leakage of LP-WUS transmission created with multiple SCS, namely, 15,30,60,and 120KHz, together with transmit side rectangular pulse shaping, and Fig. 1b illustrates the filtered LP-WUS BW before transmission, which is generated by the approach mentioned in Fig. 2.

The generation of LP-WUS together with NR signal is presented in Fig. 2, wherein two separate signals, namely, the incumbent NR signal and the LP-WUS signal using different SCS are added in TD. The SCS used by the LP-WUS is determined by the modulation order M used and the TD signal is created like NR signal, i.e., by using IFFT operation, followed by CP insertion. Note that the tones corresponding to the complementary signals in the frequency domain are filled with zeros, i.e., NR IFFT process will set zeros for LP-WUS BW region and LP-WUS will set zeros for NR regions. Upon taking respective IFFTs, TD signals of both LP-WUS and NR signal are added with necessary filtering. As the LP-WUS signal is generated separately and added to the incumbent NR signal, the CP insertion follows the respective SCS numerology of LP-WUS signal itself.

### OOK4

The major drawback with the OOK1 scheme is the complexity involved in the generation of waveform at the gNB side. This is attributed mainly due to the usage of different SCS for LP-WUS signal compared to that of incumbent NR transmission. On the contrary, OOK4 waveform generation uses the existing HW available at the gNB and generates the similar OOK style waveform with greatly reduced complexity at the gNB side. The OOK4 generation involves the following process.

Fig. 3 OOK4 signal generation together with NR

The desired time domain waveform, which may contain multiple ON/OFF keying depending on M, is created by having the samples matching the desired LP-WUS BW for a duration of one NR CP-OFDM symbol. Let this be NWUS DFT samples, which is less than the NR FFT size of N≫NWUS.

Once the desired TD waveform is generated with NWUS samples, DFT is performed for NWUS size, which is then followed by DFT shift process to ensure DC centering.

The DFT spread waveform is then mapped to the respect LP-WUS region within NR carrier BW, which is then followed by IFFT shift and IFFT process using N point IFFT process.

The reason for doing DC centering, i.e., DFT shift, before mapping is to align the DC in the center as the LR uses the center of LP-WUS transmission as the local oscillator (LO) frequency for reception.

As the waveform is created together with the NR signal, the CP insertion in the final TD transmission follows the NR process. An illustration of OOK4 generation is presented in Fig. 3.

Thus, the generation of OOK4 signal is relatively easier compared to OOK1 scheme, which involves TD insertion of the LP-WUS signal. It is worth noting that the DFT shift, or DC centering operation is identical to multiplying -1n before each sample before DFT operation, i.e., on the desired TD samples xn.

The generation of OOK4 signal at gNB is relatively easier compared to OOK1 scheme for modulation order M>1.

OOK4 modulation scheme should be prioritized for the WI as it reduces the generation complexity significantly compared to OOK1 scheme.

## Modulation order and Encoding

### Modulation Order

During SI phase, the modulation order M used for evaluation considers M={1,2,4} as they provide better robustness against time and frequency offset to certain extent. The maximum tolerable time offset by the OOK modulation scheme for M=4 was observed as ≈4μsec, beyond which, the degradation is quite significant. Thus, the WI should consider these values for further evaluation. Furthermore, as M value increases, the ON duration of OOK signal reduces noticeably, which will then be comparable to that of the CP duration. For example, if  NR uses SCS 30KHz, which has the CP duration of 3.33μsec, the ON duration of OOK signal with M=8 is ≈4.16μsec. Thus, the CP insertion may cause misdetection at LR as the OOK ON duration is now comparable to the CP insertion and every NR CP-OFDM symbol boundary.

#### Impact of CP insertion

Fig. 4 Effect of CP insertion on OOK4 transmission (as observed by LR after demodulation and down sampling)

The effect of CP insertion due to combined generation of NR and LP-WUS signal is illustrated in Fig. 4, where the CP insertion can be observed as a glitch in the OOK transmission at the CP-OFDM boundaries. In this evaluation, we use square pulse for ON duration with different modulation order, i.e., M, values. As can be seen that the CP insertion does not create any issue with the OOK4 generation when modulation order is M=1 as shown in Fig. 4(a). However, as the modulation order increases, the effect of CP insertion is more pronounced when the final LP-WUS symbol within equivalent NR CP-OFDM symbol is different from the first LP-WUS symbol of the same symbol. That is the reason, the CP glitch is not observed on all symbol transitions but only on those with flipped OOK symbol at the ends.

As the modulation order increases, i.e., M=8, the effect of CP insertion occurs after every M transitions, which in this case is M=8 as shown in Fig. 4(d). The size of CP insertion increases relatively to the ON duration of OOK. Thus, LR may need to perform additional corrections at every NR symbol boundary to account for CP insertion. As M increases, the corrections must be performed after M transitions, thus imposing conditional operations in LR, which adds complexity. Furthermore, the CP insertion may accidentally be treated as valid data when the modulation order is higher. Thus, to reduce the LR complexity, limiting the modulation order to M=1,2,4 is a wiser option.

#### Coverage issue with higher order modulations

Furthermore, the use of higher M≥4 significantly degrades the OOK detection performance, since with higher modulation orders, the ON duration reduces and so does the total energy. Furthermore, it reduces the overlay sequence length, which in turn reduces the correlation gain of SD and non-coherent combining gain of ED, thus leading to poor coverage for LP-WUS transmission. Additionally, during SI, it is observed that the impact of timing offset affects significantly for M>4, therefore, the modulation order M>4 should be deprioritized to ensure better coverage for LRs.

As the modulation order M increases, the ON duration of OOK symbol length is comparable to that of CP insertion, thus restricting M≤4 is beneficial to reduce the complexity at LR. Using M>4 impairs the detectability due to reduced ON duration to provide receive side SNR improvements.

The considered modulation order M is limited to the values considered in the SI, i.e., M∈{1,2,4}. Further discuss whether the M is fixed or configurable and adaptive.

### Data Encoding

Even though various encoding schemes are proposed in the literature, by far the most used scheme is the Manchester encoding scheme, which is also considered in the SI phase. The use of Manchester encoding provides the following benefits.

Each ON and OFF duration is characterized by a transition in the signal level, i.e., even when the data bits are all ones or zeros, Manchester encoding introduces transition in the signal, thus avoiding constant signal level at the receiver, which may cause synchronization failure.

Due to the presence of both ON/OFF transition for both bit values, i.e., {0,1}, the threshold can be easily obtained at each symbol level to facilitate the decision at each symbol instant. This reduces the LR complexity by favouring hard decision after each transition to decide the bit value.

It ensures better averaging for threshold when M is higher, i.e., when M=4, the ON duration reduces significantly, and the noise average performed with the envelope detector (ED) may not converge to the true variance due to lack of sample averaging. In those cases, averaging across all OFF duration within LP-WUS payload can help to achieve better noise variance estimate, which can then be used for threshold calculation.

Long sequences of ones or zeros are avoided and instead each MC encoded symbol has a constant DC level (ideally half of the level of the ON symbol) so even if the signal is fed through a DC blocker the signal does not loose content. A DC blocker would deteriorate the received signal if it could contain long sequences of ones or zeros as the output after the DC blocker would go towards the zero in both cases making it difficult to the decode the symbols.

Since each MC encoded symbol has a transition between ON and OFF in the middle, this transition can be used to recover the clock of the transmitted signal.

The detection threshold between ON and OFF symbols can easily be found by averaging signal, which is an advantage for very simple receivers which can find the comparator threshold value simply by LP filtering the received signal as shown in Fig. 5.

Fig. 5 MC allows simple ED based OOK receiver to acquire comparator threshold

Since the considered HW architectures assumes sampling of the MC-OOK signal, then we think other more efficient signal encodings may work as well. Despite the above-mentioned advantages, it has some drawbacks too, which are listed below.

Due to the insertion of two symbols for every input bit, for a fixed LP-WUS BW, the throughput is reduced by half.

If there is a preamble before the actual data transmission, the threshold to evaluate whether the symbol is {0,1} can be determined from the preamble, as it uses a known ON/OFF sequence. Thus, the need for Manchester encoding may depend on the availability of preamble in the LP-WUS transmission or not.

The use of Manchester encoding for LP-WUS facilitate the threshold evaluation to determine the ON/OFF decision at each symbol. However, if preamble is appended to LP-WUS transmission, or other encoding of transmitted codewords is applied, the need for Manchester encoding may not be imperative.

The use of Manchester encoding for OOK scheme should be considered if there is no preamble field in LP-WUS frame structure.

We suggest exploring if there are more spectrum efficient alternatives to Manchester encoding. One proposal could be to prefix the LP-SS / LP-WUS signal with a preamble for symbol synchronization and encode the rest of the signal with 8b/10b encoding as known from other telecommunication technologies. 8b/10b encoding can, like MC, also achieve DC balance and at the same time provide enough state changes to allow reasonable clock recovery.

Explore if there are more efficient alternatives to Manchester encoding. e.g., a preamble followed by 8b/10b or in general k/n bit encoding for rest of the signal.

## Pulse shaping and Sequence overlaying

### Effect of pulse shaping

To improve the detection performance of OOK with envelope detector (ED) pulse shaping can be applied to the ON duration of the transmission. When the applied pulse shaping is accounted in the receiver, the benefits of pulse shaping can improve the receiver SNR, and band limiting of the transmitted pulse in the ON duration, i.e., by restricting the spectral leakage of LP-WUS transmission to the adjacent NR transmission. Fig. 6 compares the various pulse shapes and their behaviour assuming matched filter at the LR. As can be seen that the width of the pulse reduces with the increase in M and so does the correlation gain, which is depicted in Fig. 6b.

| (a) Impact of M on pulse shape width | (b) Matched filtering performance |
| --- | --- |

Fig. 6 A comparison of different pulse shapes for various M

Even though pulse shaping has the above-mentioned benefits, namely, reducing the spectral leakage, improves receiver SNR even with non-coherent receivers such as ED, it has the following shortcomings as well. As the pulse shaping concentrates the total energy within the CP-OFDM symbol to multiple locations depending on M and the LP-WUS bits, the impact on PAPR of the overall gNB transmission should be analyzed. For instance, the worst case PAPR corresponding to the above listed pulse shapes are provided in the Table 1. By worst case, we mean that the information bits can be {1,0,0,0}, which leads to only one ON duration within an equivalent NR CP-OFDM symbol over which the PAPR is evaluated.

Note that for M=1, the pulse is the only existing shape within an equivalent CP-OFDM symbol used for evaluating PAPR. In the case of higher modulation orders, i.e., M≥1, both ON duration and OFF duration exists in the same symbol. Thus, the PAPR grows significantly when the modulation order is increases due to the reason that half of the energy is allocated to the ON duration when using Manchester encoded data transmission. However, one should note that the PAPR is evaluated by considering the total transmission, i.e., both NR transmission and LP-WUS transmission together. Thus, the actual effect may be very different from what we have evaluated here.

|  | M=1 | M=2 | M=4 w/o MC | M=4 with MC |
| --- | --- | --- | --- | --- |
| Rectangular pulse | 0.7dB | 6.9dB | 12.8dB | 6.9dB |
| Gaussian pulse | 6.2dB | 12.2dB | 18.3dB | 12.2dB |
| Tukey pulse | 2.6dB | 8.6dB | 14.8dB | 8.6dB |
| Hanning pulse | 6.1dB | 12.1dB | 18.3dB | 12.1dB |

Table 1 Worst case PAPR for pulse shaping over different modulation orders (with and without MC).

Use of pulse shaping benefit the LR to obtain better receive SNR even with simple detectors by enabling using receiver side filter matched with the pulse shape used for transmission.

Despite improving the receiver side SNR of non-coherent receivers, PAPR caused by the pulse shaping must be evaluated to minimize the stress on gNB side transmission.

| (a) AWGN performance δt=0,4,5μsec | (b) TDL-C 300ns performance δt=0,4,5μsec |
| --- | --- |

Fig. 7 Effect of time offset on pulse shaped LP-WUS reception by SD.

The use of pulse shaping not only benefits in reducing the spectral leakage but also provides robustness against the timing error. This is illustrated in figure Fig. 7 using two different channel models, namely, AWGN and TDL-C 300ns. The timing offset evaluated in the simulation are δt∈0,3.2,4.2μsec, respectively. It is worthy to note that the pulse shaping improves the robustness against the time offset for non-coherent detectors like ED, which is considered in this evaluation. The LR is operating at 4.32MHz sampling rate, matching the transmission BW.

As the modulation order increases, i.e., M, takes the value of 4, the degradation in the performance is almost ≥5dB for a change of 1μsec in the time offset, irrespective of the pulse shaping used at the transmitter for ON duration. However, for lower modulation orders, the deviation is less than ≤2dB. Thus, the effect of timing offset reduces the ON duration length from which the energy is collected to verify against the threshold to determine whether the bit is one or zero. Therefore, the modulation order should be limited to M≤4 even with pulse shaping is used at both the transmitter and the corresponding matched filter is used at the receiver.

The use of pulse shaping in the ON duration can improve the detection performance against time offset compared to the rectangular pulse, i.e., without pulse shaping counterpart.

Evaluate further the options of applying pulse shaping in the ON duration of OOK symbols accounting impact on the gNB transmission.

### Discussion on LR Types

As mentioned in the WID, the use of overlay sequence in the ON duration should provide no additional information than the one carried in the OOK signal, it leaves us with the following possibilities on the LR types, namely,

Envelope-only detectors (ED)

  - The LR is operating in ED only mode, i.e., only ON/OFF keying is supported by the receiver. The LR has no IQ branch to perform coherent/sequence detection.

Dual mode receivers, i.e., ED + SD mode

  - In this option, the LR is predominantly operating in ED mode when the signal conditions are conducive enough to use OOK detection. However, when the signal quality degrades, the LR switches to sequence detection mode, where it uses IQ branches to perform coherent detection to improve the coverage. Thus, the LR has dual operating mode and depending on the signal quality or condition, the LR switches the detection mode. In this case, the overlay sequence in the ON duration provides coverage extension.

Sequence-only detectors (SD)

  - In this option, with sequence detection (SD) only, the LR uses IQ branches always to perform coherent detection. It consumes more power due to the better accuracy of crystal oscillators (XO) used to drive the PLLs. Even though the LR with IQ branch can receive the OOK signal, either by using non-coherent reception or by using the overlay sequence, the power consumed by that operation make them unfavourable.

Depending on the LR type adopted by the UE, the embedded sequence may be used either to improve the coverage or to provide information.

Now assuming that the exact LR implementation approach in the UE is not restricted, there could be a mix of different types of LRs present in the deployment. In IDLE/Inactive mode especially, where network may need to prepare to support different LR types, it would be beneficial if the different type of LRs can share the same LP-WUS (and LP-SS) design.

Categorizing LR will/can benefit the design of overlay sequence requirements and what information it can carry. However, the LP-WUS design should aim at providing the same information to all LR device types using the same time-frequency resources.

The time-frequency resources used for LP-WUS should be the same irrespective of the device type used as LR, i.e., LR type specific LP-WUS transmission should be avoided.

### Design of Overlay sequence

Using the above-mentioned types of LR implementation architectures, let us discuss different possibilities of using sequences in the ON duration to provide the same information bits to sequence only detectors but using fewer LP-WUS symbols to reduce the power consumption. To do so, we introduce two variables, namely, Nosq and Nslice, which corresponds to the number of overlay sequence with a time slice of ON duration and Nslice is the number of time slices made within the ON duration to interpret as separate sequence, i.e., segments within the ON duration. Thus, depending on Nosq and Nslice, we can characterize the above three schemes as depicted in Fig. 8 as follows.

#### Sequences carrying information.

Fig. 8 Overlay sequence types (use cases)

##### Cell-specific overlay sequence, i.e., Nslice=1 and Nosq=1

Overlaying a sequence in the ON duration is illustrated in Fig. 8. In this option, the sequence is used only to improve coverage of the LP-WUS detection, i.e., the sequence used in the ON duration is the same on all ON durations of the LP-WUS transmission. Thus, the SD based receiver can correlate with the same sequence and determine the peaks in every ON duration while evaluating zeros in the OFF duration as it contains only the receiver noise. This imposes the SD based LR to be simple and can be triggered only when the ED mode experiences coverage issue. It has the following benefits, which are listed below.

As the overlay sequence is based on cell-id, sequence correlation is simple at LR, i.e., it expects a peak at every ON duration of OOK transmission.

Since the whole ON duration is used for sequence transmission, depending on the modulation order M of the OOK sequence, the correlation gain and hence the coverage can change. When M=1, the LP-WUS signal can have the best coverage.

This approach is beneficial for dual mode LRs, where ED is used when the signal conditions are good and when the signal quality deteriorates, SD mode can be opted to improve the coverage.

However, on the contrary, it has some drawbacks.

Since the ON duration carries the overlay sequence, the information transmitted for OOK based receivers are usually long. Thus, using the overlay sequence only in the ON duration may turn the SD to perform correlation for the entire LP-WUS duration, that is targeted for ED. It increases the power consumption at the SD type LRs.

Due to the cell-specific nature of the overlay sequence, this option is useful for LP-SS and for LRs equipped with ED and SD operating modes. That is, predominantly ED will be used for OOK sequence detection, and upon the detection failure, i.e., when the coverage is not sufficient for OOK detection, SD receiver will be activated for sequence detection. Upon detection failure by ED, SD receiver will be activated to perform sequence detection. However, due to the robustness of ED against frequency offset, SD receiver may incur huge frequency offset while performing sequence correlation.

The sequence overlaid in the ON duration of OOK signal must be robust against frequency offset, since the sequence detectors are activated upon ED failure, thus, cannot expect to get frequency synchronization in between the switching operation.

##### Multiple sequences in ON duration carrying more bits, i.e., Nslice=1 and Nosq>1

In the second case, where multiple sequences can be allocated to carry more than one bit of information, i.e., using 2Nosq ovelay sequences, we can convey Nosq bits depending on which sequence is used for transmission. This is addressed in this section. The size of the sequence remains the same as that of the OOK ON duration, i.e., M. The following are the merits of this approach, namely,

The number of overlay sequences can be configured based on the number of bits carried in each OOK ON duration.

As the OOK modulation order increases, the number of samples within ON duration reduces, thus imposing difficulty in finding a better set of 2Nosq sequences with cross correlation property.

Since multiple bits can be conveyed in each ON duration of LP-WUS, sequence detectors may not be active for the complete LP-WUS transmission. Thus, the SD can receive fewer ON duration of LP-WUS signal to recover the complete message carried by the overall LP-WUS, thereby, reducing the power consumption of SD based receivers.

Unlike the previous scheme, this has fewer drawbacks, namely,

As the number of sequences used for correlation increases with the number of bits carried within each ON duration of LP-WUS, the HW complexity and the processing complexity increases exponentially. Additionally, the SD based receivers may require buffering if the processing capability does not meet the real-time requirements of sequence detection.

Using multiple overlay sequences in the ON duration to convey more bits may increase the complexity of sequence detectors exponentially. Thus, limiting to fewer bits would provide a better trade-off between the complexity and the power consumption.

The number of overlay sequences used to provide more information in a single ON duration of OOK signal should consider the underlying modulation order, i.e., M, used by OOK signal.

The correlation gain and cross-correlation between sequences diminish with the increase in M used by the underlying OOK sequence, since increasing M reduces the ON duration length, i.e., sequence length of embedded sequence itself.

##### Multiple time slices to carry more information in ON duration, i.e., Nslice>1 and Nosq>1

Finally, the third option discussed in Fig. 8 uses only two sequences, i.e., Nosq=2, namely, one for bit ONE and another for ZERO bit. However, additional information bits can be achieved by multiplexing 2Nosq sequences in the time domain, i.e., 2Nslice segments, within each ON duration. The generation of ON duration is done by concatenating sequences corresponding to the respective bits in time domain and then perform OOK4 style generation. It is worth noting that the length of sequence is scaled by the number of information bits carried in each ON duration, i.e., if NON is the number of samples in the ON duration, then the sequence length for Nosq bits will be ≈NONNosq, i.e., scaled by the number of information bits carried in each ON duration. Even though this can reduce the active time of SD based LR, the performance benefit is significantly limited by the correlation gain, since the correlation gain is proportional to the length of the sequence.

| (a) Pulse shape used in ON duration | (b) Coherent detection using overlay sequence |
| --- | --- |

Fig. 9 Coherent detection of overlay sequence together with pulse shaping

As the LR is primarily a stripped-down version of the main radio (MR), the accuracy of crystals cannot be guaranteed. Thus, to ensure robust detection of LP-WUS message, the overlay sequence should be robust against the frequency offset and time offset. The time offset can be overcome by using additional margin for correlation, i.e., by collecting few samples before and after the expected location of the signal and performing correlation to determine the peak. However, the frequency offset degrades the correlation gain as the sequence is longer, thus, leading to a contradiction.

Slicing the ON duration to multiplex more information bit sequences will degrade the sequence detector performance significantly as the correlation gain is proportional to the length of the sequence, which is compromised in this approach.

Packing multiple bits within a ON duration of OOK signal by time slicing may lead to significant degradation in the correlation gain when the underlying OOK signal uses higher M>1.

Thus, the sequence used as an overlay sequence should be robust against frequency offset and offers good cross correlation and autocorrelation properties. One such sequence, satisfying the earlier mentioned requirements, is the polyphase sequence, which comes under CAZAC family. The use of Zadoff-Chu sequence can be considered for initial evaluation. Even though m-sequences are used for secondary synchronization signal (SSS) sequences, the correlation gain diminishes quickly with even fewer KHz of frequency offset. The LR, considered to receive the LP-WUS, cannot be assured to have tighter synchronization to receive such a demanding sequence.

Polyphase sequence like Zadoff-Chu, which also falls under CAZAC family, should be considered as an overlay sequence due to its robustness against frequency offset.

The effect of combining the pulse shaping together with the overlay sequence does not affect the OOK performance but hampers the correlation gain depending on the shape of the pulse used for ON duration. The benefit of using pulse shaping overlaid with the sequence on coherent receiver is highlighted in Fig. 9. The sequence correlation provides sharper peaks due to coherent reception, which is highlighted in Fig. 9b for the corresponding pulse shapes considered in Fig. 9a.

| (a) Non-coherent detection (ED style) | (b) Coherent detection (SD style) |
| --- | --- |

Fig. 10 Effect of frequency offset on pulse shaped overlay sequence (δf∈0,10,90ppm) under TDL-C 300ns.

The impact of frequency offset on the detection performance of various pulse shapes is presented in Fig. 10 for coherent and non-coherent detectors, i.e., ED and SD respectively. As can be seen from Fig. 10a that the frequency offset of 90ppm does not change the relative behavior of the performance with respect to each pulse shape compared to that of 0ppm. Note that the effect of pulse shaping has no benefit on the frequency offset experienced by the LR, which is also highlighted in Fig. 10a. On the contrary, the sequence detectors, which uses coherent receivers, performance is degraded when the LR frequency offset increases. However, the pulse shaped versions can offer better performance than the square pulse under frequency offset considered in this evaluation. The performance for all cases is degrading as a function of the modulation order.

##### Constellation mapping over ON duration sequence

Alternatively, instead of increasing the number of sequences in the ON duration or to divide the ON duration into multiple short time segments with an overlay sequence, we can transmit multiple bits by mapping them on a single sequence. This way, the LR equipped with dual mode, which switches to SD mode occasionally, may use the correlation benefit of single sequence. Whereas the SD only mode, which assumes better synchronization through SSBs, or high accuracy crystals can detect and decode the constellation used for the ON duration sequence to obtain more bits.

It is worth noting that the dual mode receivers operate primarily in ED mode, where the frequency is not a big concern. Thus, when the signal quality degrades, they switch to SD mode, and we cannot ensure better accuracy on the clocks. Thus, they can use a simple correlation using single sequence and obtain the OOK information using correlation. On the contrary, SD only receivers, for which we can assume better clocks and synchronization circuitry, embedded constellation can provide additional information and they obtain that with fewer number of OOK symbols.

Embedding a constellation symbol over the ON duration sequence provides more bits per ON duration with a single sequence used for correlation.

Due to the correlation gain, the SNR is much better than the sequence encoding, since the sequence encoding trades-off either the LR complexity or the time duration, i.e., with lower correlation gain, to increase the number of bits.

Embedding a constellation symbol on the overlay sequence, like chip sequence, provides more information bits per ON duration for SD receivers compared to OOK based ED receivers.

# LP-WUS channel structure

In this section, we will discuss the LP-WUS channel format, fields that can be present, and contents used for transmission. The consideration is made from the perspective of ED receiver. It is assumed that the LP-WUS carries the wake-up information for a single or multiple paging occasion (POs), which includes multiple UEs that are grouped together in a different groups for a given PO or for multiple POs, wherein each PO includes multiple UE groups. Under the assumption that the low-power wake-up radio (LR) decodes the LP-WUS message to determine the paging status, i.e., when the respective group/sub-group is paged, wakes up the MR to monitor the paging message. To facilitate LR to receive LP-WUS, we must consider certain channel structure of LP-WUS, which enables the LR to identify the presence of LP-WUS, then followed by decoding of the same.

(a) Payload based scheme using NPO POs addressed by LP-WUS

(b) Sequence based scheme to address NPO POs within LP-WUS

Fig. 11 LP-WUS channel structure options

Using the Fig. 11 as reference, let us discuss each field used in the LP-WUS channel structure and identify the use case or merits in detail. In Fig. 11, two different types of channel structure are proposed, namely, payload based and sequence-based scheme. Let us discuss each of the fields in detail.

## Preamble

The preamble containing AGC field may help to improve both detectability and reliability of WUS signal irrespective of whether the LR is continuously or periodically listening for an on-demand wakeup. The benefit of having each field in the LP-WUS signal shown in Fig. 11 are as follows.

Settling of comparator threshold voltage: In case of OOK and comparator-based receivers, the initial part of the preamble may be used for settling of comparator threshold voltage. If the preamble is a toggling sequence of 0’s and 1’s or if the preamble is a Manchester encoded sync word, the threshold voltage may be found by LP filtering the received preamble signal. In this case, the length of the preamble should be long enough to ensure reliable settling of the low pass filter equipped in the LR. On the other hand, the low pass filter settling time shall be much longer than the ON/OFF duration to ensure that the threshold voltage is maintained across the ON and OFF symbols.

Detecting the alignment of the WUS message: To find the start of wake-up message payload, the preamble may contain a synchronization pattern, which could assist devices to establish timing alignment with the gNB. Furthermore, different synchronization patterns could be used to differentiate LP-WUS messages e.g., how many active POs exist in the LP-WUS message part.

Preamble for threshold: If preamble can provide the threshold to determine the ON to OFF level ratio, the Manchester encoding can be ignored from the LP-WUS message part as the primary benefit of using Manchester encoding is to obtain the threshold to differentiate between ON and OFF signal. An additional benefit is that it gives frequent ON/OFF symbol transitions, which allows the LRs to find and maintain accurate symbol timing / clock. In a long sequence of constant ON or OFF symbols, the symbol timing may start to slide, if the LR has a high clock error relative to the received signal.

RAN1 should evaluate whether LP-WUS requires a preamble or not and if required, the preamble design should be discussed.

## LP-WUS message field

The LP-WUS message field can be classified as payload based or sequence based. Each one has its own benefit that depends on the message size used. They are discussed briefly below.

### Payload based LP-WUS message content.

An illustration of the payload type used for LP-WUS message is presented in Fig. 11a, where the LP-WUS message is formed by concatenating the bit fields corresponding to multiple POs, where each PO i has Gi groups with a total of NGi bits, i.e., each bit dictates whether the corresponding group is paged or not with a bit value of ONE or ZERO, respectively. Thus, the LP-WUS message contains NGi×NPO bits if the LP-WUS addresses NPO POs.

#### Header field

As the payload contains certain number of bits that are allocated to each POs depending on the number of groups associated with each POs, the total number of POs addressed by the LP-WUS message is fixed. However, to ensure better utilization of LP-WUS resources, a header field can be included in the LP-WUS message, which informs the number of POs, or the POs addressed in the current LP-WUS message using a bit mask. If the header is not present, then the LP-WUS message remains fixed for number of POs and if certain PO, say, j, is not paged, then all the groups i.e., NGj within the PO j will carry zeros, indicating none of the groups within the PO j is paged, thereby wasting the LP-WUS resources, which could be used otherwise efficiently. Thus, the scalability of payload-based scheme can be achieved with a header field defining the number of POs addressed in the current LP-WUS message structure. The encoding of header field can be isolated from LP-WUS message itself, in that case, it can inform the modulation order, use of MC encoding as well.

The header field may indicate certain payload structure that can benefit both NW and LR to reduce resource utilization for LP-WUS and complexity, respectively.

#### CRC

The CRC field can optionally be included in the LP-WUS message structure. The use of CRC facilitates the LR to verify the content of LP-WUS message is error free. However, if there is an error in either CRC or the message bit, LR must wake-up the MR to monitor paging PDCCH message. Thus, the use case of CRC in the LP-WUS message becomes critical to identify whether the message is intact or not. However, if the CRC is not included, then it may impose additional restrictions on the LP-WUS/LP-SS structure as follows.

Optionally, LR can utilize LP-SS to ensure the detectability of LP-WUS message if LP-SS is co-located in the time or within the coherence time experienced by the LR. It is worth noting that the LP-SS is periodic but not LP-WUS, i.e., LP-WUS may or may not be transmitted at a given LP-WUS monitoring occasion (LP-WUS MO).

Alternatively, if there is a preamble, then LR can utilize that to ensure the presence of LP-WUS message in each LP-WUS MO and depending on the quality of preamble detection, LR can determine the detectability of LP-WUS payload.

Despite all the above alternatives proposed to overcome the usage of CRC bits, the benefit of having CRC assures the correct detection of LP-WUS for certainty, i.e., when the CRC succeeded, then the LP-WUS message is intact. On the other hand, if there is no CRC, even if the LR uses other fields such as, LP-SS, or preamble in LP-WUS, the certainty of the detection cannot be ensured to the fullest.

The presence of CRC ensures the certainty of the LP-WUS detection, which is otherwise cannot be ensured to the fullest by LR.

RAN1 should evaluate whether LP-WUS requires a CRC field or not and if required, then the size and the polynomial used should be defined.

#### FEC (channel coding)

The use of simple forward error correction (FEC) for LP-WUS message structure may benefit the error detection and correction capability at LR, which will extend the coverage of LP-WUS a bit further depending on the coding rate. However, this may increase the complexity of LR as it requires a simple decoder to extract the information bits before checking CRC if there is any.

The use of FEC for LP-WUS message can improve the coverage by a factor proportional to the FEC coding rate but may increase the complexity of LR.

#### Payload

The payload is a bit string which is constructed by concatenating multiple groups, each associated with different POs or a single PO depending on the design. A simple illustration is provided Fig. 11a, where we have NPO PO associated message, where each PO may contain different number of groups, which are addressed by separately. Upon concatenating the bit string from multiple POs, FEC can be performed that expands the number of bits carried by the payload from Kwus,bits≈Khdr+Kcrc+i∈1,..,NPONGi to Nwus,bits=c×Kwus,bits, where Kcrc, Khdr and c denotes the number of bits used for CRC, number of bits used for header field, and the FEC code rate, respectively. Thus, the channel coded data stream is then Manchester encoded to increase the total number of bits to 2×Nwus,bits, which will then be transmitted after using desired modulation order M that is common for all the groups in each POs. Thus, the total number of CP-OFDM symbols used by the payload-based scheme is given by 2×Nwus,bitsM. The number of bits used as payload depends on the number of POs addressed by the LP-WUS and the number of groups within each PO.

The modulation order used for payload-based scheme is common for all POs and the same goes to the Manchester encoding as well.

When the number of groups present in a PO is fewer, or when the allocated bits per PO is fewer, payload-based scheme provides better robustness with FEC encoding as it is applied over the entire LP-WUS message.

RAN1 should evaluate the content and the structure of LP-WUS payload.

### Sequence based message.

This is presented in Fig. 11b, where there is no requirement of CRC bits to the LP-WUS message. However, header field can optionally be present, depending on the use case it serves.

#### Sequence message construction

In this approach, whether LP-WUS addresses multiple POs or a single PO, the number of bits per PO can be fixed, which can either be the same or different. Let us assume that each PO i consists of Gi groups, thus requiring only ⌈log2NGi⌉ bits. However, in the sequence based mapping, the number of bits associated with each PO i is allocated with NPOi bits, i.e., NPOi>⌈log2NGi⌉, thus mapping a lower number of bits to higher number of bits per PO. This mapping increases the Hamming distance between the different bit words per PO, thus increasing the robustness. In one possible option, each group Gi associated with a PO looks for certain pattern inside the PO specific region, thus the detection robustness is proportional to the length of the sequence allocated to a PO segment. This is illustrated in Fig. 11b, where PO1 contains 2 groups, and PO2 contains 2 groups and PONPO contains 3 groups.

Unlike the payload-based approach, the use of Manchester encoding, if used, can be altered within each PO segment as each PO segment is independent of one another and the same goes for the modulation order as well. This is illustrated in Fig. 11b, where PO1 uses the modulation order M=2 with Manchester encoding, whereas M=1 without Manchester encoding is considered by PO2. Thus, the sequence-based mapping provides better flexibility and facilitates the grouping of UEs depending on the signal quality and conditions.

Unlike payload-based mapping, sequence-based LP-WUS is flexible enough to modify the use of Manchester encoding and the modulation order per PO.

The sequence length determines the robustness of the detection, thus a LR associated with a group insider a PO, can monitor only the segment associated with that PO for a sequence, thereby reducing the active duration and power.

As the performance of payload-based and sequence-based LP-WUS structure depends on the length of the message content, RAN1 should first decide the range of the information content of the LP-WUS message.

#### Header

The header field can be used in the LP-WUS message before the sequence type mapping to inform the number of POs addressed in the LP-WUS message. In such cases, the length of LP-WUS message can be reduced to minimize the overhead of LP-WUS transmission due to unnecessary indication of no group paged within a PO. Alternatively, one can repeat the sequence associated with PO i in the LP-WUS message segment allocated for PO i+1 when PO i+1 has no group to be paged. By doing so, LRs belonging to PO group i may use the repetition to improve the detectability as it can corroborate the signal by combining with the repeated signal.

The header field can be used to improve the sequence mapping by combining/repeating when the number of POs with a valid paged group is small when compared to the total number of POs present in the LP-WUS message.

# LP-SS Design

As the LR is designed to reduce the power, the LP-WUS design considers the waveform using OOK with or without an embedded sequence. To integrate the OOK signal in the OFDM framework, to minimize impact on the legacy UEs, OOK scheme follows OFDM symbol structure and generation. Due to the inferior performance of OOK scheme under low S(I)NR cell-edge cases, additional sequence is embedded in the ON duration of the OOK signal, thus ensuring coverage of OOK receivers with additional sequence detection capabilities.

Despite limiting the usage of power consuming receiver blocks in the LR, it may not be effective for a LR to monitor LP-WUS continuously all the time, since the energy consumption is proportional to the ON duration. Thus, for an efficient operation, LRs are operated in a duty cycled manner, which turns on at discreet instants to reduce the power consumption.

The OOK only receiver, i.e., using ED receiver, reduces the overall power consumption by restricting the use of IQ branches with higher sampling rate.  Due to the removal or trim-down of major power consuming components such as PLLs, LNAs, and clock sources or synthesizers, LRs may require periodic calibration of clocks before receiving the LP-WUS signal for improved detection performance, thus, mandating a periodic low-power synchronization signal (LP-SS) to be transmitted from a stable clock source, such as gNB. Thus, the LP-SS can be useful to provide following uses as listed below.

Assists in timing synchronization

  - To detect higher modulation orders, i.e., M>2, used in LP-WUS, tighter synchronization in time is required, if otherwise, may lead to significant degradation in the performance.

  - LRs operating in SD mode requires higher timing accuracy than ED based LRs, thus, having LP-SS with embedded sequence may assist SD based LRs as well.

Assists in frequency synchronization

  - LRs can have multiple parallel filters with reduced BW to receive LP-SS and can determine the frequency offset.

  - Even though SD based LRs can utilize SSBs, but having an embedded sequence in LP-SS like LP-WUS would benefit SD receivers as well.

Used for RRM measurements

  - As the LP-WUS transmission is not periodic, to ensure the detectability of LP-WUS by LRs, LP-SS can be used as reference. This is under the assumption that the LP-SS and LP-WUS separation is not larger than the coherence time of LRs.

  - Some measure to ensure that the LR is within the cell coverage. This is primarily used by LRs to ensure the detectability of LP-WUS message.

## LP-SS waveform properties

### Modulation type and order

As discussed earlier in Section 3.2, the modulation used for LP-SS should be the same as LP-WUS, thus ensuring a unified design between both channels. As can be seen from the discussions in Section 3.2, the modulation type should consider using OOK4 as it facilitates both LR and gNB to reduce the complexity of reception and generation, respectively.

Unified waveform design between LP-SS and LP-WUS would be beneficial from LR implementation, thereby choosing OOK4 modulation type should be prioritized.

The modulation order used for LP-WUS could range from M∈{1,2,4}, with or without Manchester encoding, which dictates that LP-SS can have the same modulation range. Unlike LP-WUS that carries only the paging message with additional fine time and frequency synchronization fields as preamble, LP-SS is a periodic signal used mainly to obtain coarse synchronization of both time and frequency. Furthermore, the design of LP-SS is important as it is the only physical channel that LRs can use to decide the coverage and the detectability of LP-WUS signal. Thus, to ensure better coverage and detectability of LP-SS, the modulation order should be limited to M∈{1,2} with Manchester encoding to ensure proper threshold determination.

As the initial sampling clock of LR may have ~100ppm error, using higher modulation order, i.e., M=4, may prolong the initial synchronization duration of LRs unless there is an assistance from MR.

The modulation order used by LP-SS should be restricted to M={1,2} with Manchester encoding to ensure better coverage and to facilitate accurate measurements for RRM purposes.

### Use of overlay sequence

The use of overlay sequence in the ON duration of LP-SS is not considered in the WID. The main reason for not considering an overlay sequence is that SD receivers can utilize SSBs to obtain synchronization. However, it has following constraints.

We cannot ensure complete overlapping of LP-WUS/LP-SS and SSBs in the same BW within the gNB carrier BW. Due to this, the SD receivers may retune the XOs whenever there is a need to receive SSBs to either retune XO or to perform RRM measurements.

The SCS used by the NR data and LP-WUS message may be different than the one used by SSB, depending on the deployment scenario. This may impose additional burden on LRs to adjust the reception strategy, i.e., one for SSBs and another for LP-WUS, which may ruin the power saving benefit of SD based LRs.

The SSS sequence within SSB are based on m-sequence, which has very low processing gain (correlation gain) when the receiver is having a frequency offset comparable to 0.5× SCS. Such an accurate XOs cannot be assumed for LRs even with SD only mode. This may increase the initial synchronization of LRs unless there is an assistance from MR to synchronize the XO.

As the ED based LRs does not care about the content of ON duration, having a sequence overlaid in the ON duration benefits both type of LR implementations, i.e., ED and SD based receivers. Furthermore, the overlay sequence is used in the ON duration of LP-WUS, therefore, there is no harm in using an overlay sequence in the ON duration of LP-SS.

As the LP-WUS carries an overlay sequence in the ON duration, it would be beneficial to follow the same approach for LP-SS as well as it may avoid LRs with SD to receive SSBs.

As the LP-WUS SCS is aligned with the incumbent NR SCS, the SCS of SSBs may not be the same as NR SCS in some deployment scenarios. In such cases, LRs with SD receivers may not receiver SSBs to tune the XOs without additional complexity.

As the LP-SS benefit all kind of LRs, i.e., SD only, ED only, and dual mode receivers, i.e., ED + SD, overlaying a sequence in the ON duration and aligning the waveform design to LP-WUS should be selected.

## LP-SS channel structure

The primary aim LP-SS is to achieve initial synchronization of both time and frequency. Once the initial time and frequency is obtained, fine tuning can either be performed by LRs using subsequent LP-SSs itself or with the preamble in LP-WUS, if present. Additionally, the LP-SS is also used for RRM measurements, therefore, it should be cell-specific, i.e., upon receiving the LP-SS, a LR should identify the cell it is intended to synchronize with.

Thus, to achieve initial synchronization and to utilize the cell-specific information to perform reliable RRM measurements, the LP-SS channel structure must be designed to assist the LR to acquire both time and frequency synchronization with minimal complexity. The OOK signal is robust against the frequency offset but the timing accuracy is imperative as it demands ≈4μsec timing error tolerance to receive M=4 modulation order. Additionally, the same XO will be used to drive both RF and sampling clock, thus it is crucial for LRs with ED only receivers to estimate and correct the frequency offset to a certain extent.

Fig. 12 LP-SS Design in time/frequency grid (hopping pattern based on cell-id).

In this section, we propose an approach to achieve ED based receivers to achieve both time and frequency offset to a certain extent. Ideally, the synchronization signals are transmitted over multiple symbols to improve the detectability under low SNRs or can be combined across multiple periods to improve receive SNR. However, due to the limited HW capability of LRs with ED only receivers, we cannot assume such a sophistication in the reception as it may also increase power consumption. Thus, to aid both time and frequency error estimation, the LP-SS can be designed to use a fraction of LP-WUS BW with frequency hopping pattern, which is a function of cell-id as shown in Fig. 12.

An illustration of LP-SS signal designed for cell-id Ncid=5 is shown in Fig. 12, where both time and frequency contents are varied in accordance with the serving cell id using M=2 as modulation order with Manchester encoding. The LP-SS waveform uses ON/OFF pattern in time domain following the cell-id encoded in BCD format and the waveform is restricted to a fraction of the available LP-WUS BW, thus facilitating LP-SS transmission to have different frequency hopping on different symbols. The ON/OFF pattern used in time domain illustrates the OOK sequence, which can be identified using the cell-id, thus ensuring the detection of LP-SS means the association with the desired serving cell.

The OOK pattern uses the cell-id or cell specific information and is known to MR, which can then be configured or fed to LR for detection of LP-SS. As M=2 with Manchester encoding is used for LP-SS in Fig. 12,  the position of the pulse can be placed in either first half or second half to determine 0/1 as information bit. The ON duration can be pulse shaped as discussed in previous sections, to ensure better detectability with envelope detectors. Pulse shaping can provide significant benefit compared to the square pulse by improving the received SNR with filter matched to the pulse shape.

To assist the LR in estimating the frequency offset, the gNB transmits the LP-SS signal with narrow section of the LP-WUS BW, which can be a factor, say, ζ, of the total LP-WUS BW. The value of α can take values between e.g., ζ∈0.5,1×BWWUS, where BWWUS is the total available BW for LP-WUS/LP-SS transmission. Assuming LP-SS occupies multiple symbols in a slot, say, L, symbols, the frequency location of the LP-SS can be altered over different symbols, thereby aiding the LR to determine the symbol containing highest energy as the actual frequency offset. The hopping pattern illustrate the location of energy in the spectrum over different symbols, where Δf0, ±∆f1, ±∆f2 present the different (center) frequency locations of the transmitted LP-SS. As the LR is using a fixed center frequency, the fading experienced by the LR over the given LR BW is same for all frequency hopping performed over symbols. Thus, the effect of fading may not affect the estimation performance.

The LR can use a fixed LPF with slightly reduced BW compared to that of LP-WUS reception. Upon receiving the LP-SS signal over multiple symbols with frequency hopping, LRs can estimate the energy and find the frequency offset from the symbol index for which the received energy is the maximum, since the hopping pattern is known to LR in advance. Thus, ED based receiver can benefit from this transmission scheme by avoiding power hungry LO switching and retuning. To verify the performance of above scheme, we evaluated in a TDL-C 300ns channel with SCS 30KHz by varying the SNR. The initial frequency offset assumed for LR is ≈200ppm and the performance comparison is provided by considering 4.32MHz LP-WUS BW and {6,12} PRBs for LP-SS BW. The outcome of this experiment is that the residual error with only ED cannot be worse than ±max { mini,j|Δfi-Δfj| } on an average.

Performing frequency hopping together following the bit field in symbol using Manchester encoding enables LP-SS to provide both time and frequency synchronization to LRs with ED without involving frequency retuning.

LP-SS should provide both time and frequency synchronization to LRs of all types with minimal real-time constraints on the reception.

## RRM Measurements

Other than the initial synchronization, LP-SS should provide resources upon UE can carry out measurements for evaluation RRM measurement offloading. These measurements could also be used to ensure the reliability of LR detection so that MR can be used instead if reception quality of LR is low. In RAN1 SI discussions, various measurement quantities were considered. When considering the feasibility of different measurement metrics,  it would be also necessary to consider the underlying LR device type. The use of LP-SS for ED based LRs and PSS/SSS or overlay sequence for SD based LR can result different feasibility evaluation and in the end, definition for the measurement metrics.

Depending on the type of LR considered, measurement metrics may vary, since the underlying signal used for measurements may not be the same.

Evidently, for ED based LR that is not capable of detecting PSS/SSS, there is a need to agree and define the LP-SS based measurement quantities. To determine the level of coverage or the quality of reception at LR, certain metrics can be used. For example, relative metric like RSRQ or SINR can be used to indicate the reception signal quality and are robust against certain receiver impairments. On addition or alternatively, it would be preferable also to consider absolute quantities, such as, RSRP for LP-SS based measurements, while it is good to note that absolute metrics are more sensitive to receiver impairments. Thus, the usability of LP-SS signal design to obtain certain measurement metrics must be considered in the WI.

Both relative and absolute quantities could be considered for LP-SS based measurements, namely, RSRP, RSSI, RSRQ-WUS, SINR.

For LR capable of receiving PSS/SSS, measurement quantities are obtained using PSS/SSS. Even though existing measurement quantities defined for MR based on SSB are available, such as SS-RSRP, reusing the same quantities may not be preferable. To improve the reporting and measurement configuration, separating LR based measurement metrics from MR based values might be a better option.

Fig. 13 illustrates the performance of SNR estimation under timing and frequency errors. The modulation waveform used is OOK1 with M=1, i.e., in each CP-OFDM symbol, only one transition is allowed, therefore, MC encoded OOK1 waveform spans two CP-OFDM symbols. Since OOK1 spans wider BW with additional guard band to prevent leakage from adjacent NR transmission, the SINR estimation is robust against frequency offset up until 100ppm as shown in Fig. 13a. The maximum deviation observed by the SINR estimation using OOK1 signal is ≈±3dB. Due to the estimation bias, the error looks like one-sided, however, if the estimation is performed with bias compensation, the overall performance can be improved slightly.

| (a) Effect of varying frequency offset | (b) Effect of varying time offset |
| --- | --- |

Fig. 13 Estimation performance under impairments using SCS15, M=1 with OOK1/OOK4

As discussed earlier that the frequency error does not alter the estimation performance noticeably, the estimation suffers in the presence of timing offset at the receiver. In this experiment, we introduced uniform timing offset in the reception of OOK1 signal with known bit pattern to estimate the SINR. The performance degrades with the larger time offset ΔT≥4μs in Fig. 13b. Note that the effect of channel model has no impact on the estimation performance, whereas the SCS does. Even though higher SCS is not shown here, as the SCS increases, the available number of samples used for estimation reduces accordingly, thereby, making it error prone. The benefit of using relative measurements instead of absolute one is that it avoids unnecessary bias offsets introduced by AGC calibrations.

Any relative measurements using a known preamble containing certain bit pattern is beneficial for RRM related measurements compared to the absolute ones, since the calibration of LR may not be feasible when there is limited hardware. Thus, SNR may be a viable alternative.

# Conclusion

In this document, we presented our initial thoughts on the WI scope and the topics that are relevant to be considered and discussed during the WI phase.

## Observations

- Wider LP-WUS/LP-SS BW can offer robustness against fading but impacts the spectrum efficiency.

The scalability of LP-WUS/LP-SS BW would benefit multitude of device types and deployments.

The generation of OOK4 signal at gNB is relatively easier compared to OOK1 scheme for modulation order M>1.

As the modulation order M increases, the ON duration of OOK symbol length is comparable to that of CP insertion, thus restricting M≤4 is beneficial to reduce the complexity at LR. Using M>4 impairs the detectability due to reduced ON duration to provide receive side SNR improvements.

The use of Manchester encoding for LP-WUS facilitate the threshold evaluation to determine the ON/OFF decision at each symbol. However, if preamble is appended to LP-WUS transmission, or other encoding of transmitted codewords is applied, the need for Manchester encoding may not be imperative.

Use of pulse shaping benefit the LR to obtain better receive SNR even with simple detectors by enabling using receiver side filter matched with the pulse shape used for transmission.

Despite improving the receiver side SNR of non-coherent receivers, PAPR caused by the pulse shaping must be evaluated to minimize the stress on gNB side transmission.

The use of pulse shaping in the ON duration can improve the detection performance against time offset compared to the rectangular pulse, i.e., without pulse shaping counterpart.

Categorizing LR will/can benefit the design of overlay sequence requirements and what information it can carry. However, the LP-WUS design should aim at providing the same information to all LR device types using the same time-frequency resources.

The sequence overlaid in the ON duration of OOK signal must be robust against frequency offset, since the sequence detectors are activated upon ED failure, thus, cannot expect to get frequency synchronization in between the switching operation.

Using multiple overlay sequences in the ON duration to convey more bits may increase the complexity of sequence detectors exponentially. Thus, limiting to fewer bits would provide a better trade-off between the complexity and the power consumption.

Slicing the ON duration to multiplex more information bit sequences will degrade the sequence detector performance significantly as the correlation gain is proportional to the length of the sequence, which is compromised in this approach.

Packing multiple bits within a ON duration of OOK signal by time slicing may lead to significant degradation in the correlation gain when the underlying OOK signal uses higher M>1.

Embedding a constellation symbol over the ON duration sequence provides more bits per ON duration with a single sequence used for correlation.

Due to the correlation gain, the SNR is much better than the sequence encoding, since the sequence encoding trades-off either the LR complexity or the time duration, i.e., with lower correlation gain, to increase the number of bits.

The header field may indicate certain payload structure that can benefit both NW and LR to reduce resource utilization for LP-WUS and complexity, respectively.

The presence of CRC ensures the certainty of the LP-WUS detection, which is otherwise cannot be ensured to the fullest by LR.

The use of FEC for LP-WUS message can improve the coverage by a factor proportional to the FEC coding rate but may increase the complexity of LR.

The modulation order used for payload-based scheme is common for all POs and the same goes to the Manchester encoding as well.

When the number of groups present in a PO is fewer, or when the allocated bits per PO is fewer, payload-based scheme provides better robustness with FEC encoding as it is applied over the entire LP-WUS message.

Unlike payload-based mapping, sequence-based LP-WUS is flexible enough to modify the use of Manchester encoding and the modulation order per PO.

The sequence length determines the robustness of the detection, thus a LR associated with a group insider a PO, can monitor only the segment associated with that PO for a sequence, thereby reducing the active duration and power.

The header field can be used to improve the sequence mapping by combining/repeating when the number of POs with a valid paged group is small when compared to the total number of POs present in the LP-WUS message.

As the initial sampling clock of LR may have ~100ppm error, using higher modulation order, i.e., M=4, may prolong the initial synchronization duration of LRs unless there is an assistance from MR.

As the LP-WUS carries an overlay sequence in the ON duration, it would be beneficial to follow the same approach for LP-SS as well as it may avoid LRs with SD to receive SSBs.

As the LP-WUS SCS is aligned with the incumbent NR SCS, the SCS of SSBs may not be the same as NR SCS in some deployment scenarios. In such cases, LRs with SD receivers may not receiver SSBs to tune the XOs without additional complexity.

Performing frequency hopping together following the bit field in symbol using Manchester encoding enables LP-SS to provide both time and frequency synchronization to LRs with ED without involving frequency retuning.

Depending on the type of LR considered, measurement metrics may vary, since the underlying signal used for measurements may not be the same.

Both relative and absolute quantities could be considered for LP-SS based measurements, namely, RSRP, RSSI, RSRQ-WUS, SINR.

Any relative measurements using a known preamble containing certain bit pattern is beneficial for RRM related measurements compared to the absolute ones, since the calibration of LR may not be feasible when there is limited hardware. Thus, SNR may be a viable alternative.

## Proposals

- Consider scalable design for LP-WUS/LP-SS to support multiple BW options depending on the deployment scenario. For initial evaluation, 5MHz LP-WUS BW can be used.

The position of LP-WUS/LP-SS within the carrier BW should be flexible and configurable by the NW.

OOK4 modulation scheme should be prioritized for the WI as it reduces the generation complexity significantly compared to OOK1 scheme.

The considered modulation order M is limited to the values considered in the SI, i.e., M∈{1,2,4}. Further discuss whether the M is fixed or configurable and adaptive.

The use of Manchester encoding for OOK scheme should be considered if there is no preamble field in LP-WUS frame structure.

Explore if there are more efficient alternatives to Manchester encoding. e.g., a preamble followed by 8b/10b or in general k/n bit encoding for rest of the signal.

Evaluate further the options of applying pulse shaping in the ON duration of OOK symbols accounting impact on the gNB transmission.

The time-frequency resources used for LP-WUS should be the same irrespective of the device type used as LR, i.e., LR type specific LP-WUS transmission should be avoided.

The number of overlay sequences used to provide more information in a single ON duration of OOK signal should consider the underlying modulation order, i.e., M, used by OOK signal.

The correlation gain and cross-correlation between sequences diminish with the increase in M used by the underlying OOK sequence, since increasing M reduces the ON duration length, i.e., sequence length of embedded sequence itself.

Polyphase sequence like Zadoff-Chu, which also falls under CAZAC family, should be considered as an overlay sequence due to its robustness against frequency offset.

Embedding a constellation symbol on the overlay sequence, like chip sequence, provides more information bits per ON duration for SD receivers compared to OOK based ED receivers.

RAN1 should evaluate whether LP-WUS requires a preamble or not and if so, the design of it should be discussed.

RAN1 should evaluate whether LP-WUS requires a CRC field or not and if required, then the size and the polynomial used should be defined.

RAN1 should evaluate the content and the structure of LP-WUS payload.

As the performance of payload-based and sequence-based LP-WUS structure depends on the length of the message content, RAN1 should first decide the range of the information content of the LP-WUS message.

Unified waveform design between LP-SS and LP-WUS would be beneficial from LR implementation, thereby choosing OOK4 modulation type should be prioritized.

The modulation order used by LP-SS should be restricted to M={1,2} with Manchester encoding to ensure better coverage and to facilitate accurate measurements for RRM purposes.

As the LP-SS benefit all kind of LRs, i.e., SD only, ED only, and dual mode receivers, i.e., ED + SD, overlaying a sequence in the ON duration and aligning the waveform design to LP-WUS should be selected.

LP-SS should provide both time and frequency synchronization to LRs of all types with minimal real-time constraints on the reception.