3GPP TSG RAN WG1 #116-bis		R1-2402346

Changsha, Hunan Province, China, April 15th – 19th, 2024

Source:	OPPO

Title:	Discussion on IoT-NTN uplink capacity/throughput enhancement

Agenda Item:	9.11.4

Document for:	Discussion and Decision

Introduction

IoT NTN is being deployed to support massive IoT devices, while the system capacity faces enormous challenge due to the large beam footprint size from hundreds to thousands of kilometers in NTN scenario. Regarding this matter, the WID proposes to study and specify, if beneficial, the application of orthogonal cover code (OCC) to NPUSCH format 1 an NPRACH to improve the uplink capacity [1].

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2] - Multi-tone support for 15 kHz SCS should also be considered Note: Impact of impairment shall be taken into account

In this contribution, we provide our view and analysis on the enhancements to support UE multiplexing via OCC for NPUSCH format 1 and NPRACH. The OCC structures of the candidate OCC schemes for NPUSCH format 1 are firstly discussed and the corresponding evaluation results are provided. Then, we discuss the intra-symbol group OCC design for NPRACH format 1 and present our preliminary evaluation results.

Uplink capacity enhancement for IoT-NTN

OCC for NPUSCH format 1

Regarding multiplexing UEs via OCC for NPUSCH format 1, according to the WID, both single-tone and multi-tone NPUSCH transmission for 3.75kHz and 15kHz SCS should be taken into account. Considering OCC can be applied in time or frequency domain, RAN1 agreed to further study the following OCC schemes for single-tone and multi-tone NPUSCH format 1 in the last meeting:

Agreement For single-tone NPUSCH format 1 transmissions with both 3.75kHz and 15kHz SCS, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level - Repetition-level - RV-level For multi-tone NPUSCH format 1 transmissions, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level (including Nslot level) - Repetition-levels - RV-level - Intra-symbol pre-DFT spreading OCC

Then, we discuss the block-wise spreading design on the following candidate OCC schemes:

- Slot-level OCC

For multi-tone NPUSCH repetition, the inter-slot repetition is firstly performed within NPUSCH, i.e., across Nslots slots, and then across NPUSCH. The details in TS36.211 are specified as follows:

10.1.3.6 Mapping to physical resources Each NPUSCH codeword can be mapped to one or more than one resource units, NRU, as given by clause 16.5.1.2 of TS 36.213 [4], each of which shall be transmitted MrepNPUSCH times. The block of complex-valued symbols z(0),...,z(Msymbap-1) shall be multiplied with the amplitude scaling factor βNPUSCH in order to conform to the transmit power PNPUSCH specified in [4], and mapped in sequence starting with z(0) to subcarriers assigned for transmission of NPUSCH. The mapping to resource elements k,l corresponding to the subcarriers assigned for transmission and not used for transmission of reference signals, shall be in increasing order of first the index k, then the index l, starting with the first slot in the assigned resource unit. After mapping to Nslots slots, the Nslots slots shall be repeated MidenticalNPUSCH-1 additional times, before continuing the mapping of z(⋅) to the following slot, where MidenticalNPUSCH=minMrepNPUSCH/2,4NscRU>11NscRU=1 Nslots=1Δf=3.75 kHz2Δf=15 kHz

In this case, the repeated Nslots slots can be reused for multiplexing UEs with OCC. Take 12-tone NPUSCH transmission for example, when MidenticalNPUSCH=2, 2 UEs can be multiplexed on every MidenticalNPUSCH∙Nslots slots with OCC, and the slot-level OCC structure is depicted in Fig 1. The OCC spreads across Nslots slots, so the slot-level OCC structure is flexible enough to multiplex any number of UEs theoretically.

Fig 1. Slot-level OCC with OCC length = 2

However, according to the above description in the current spec, only multi-tone NPUSCH transmission supports inter-Nslots slots repetition within NPUSCH. Thus, for single-tone NPUSCH transmission, this feature should also be supported to enable slot-level OCC scheme.

Observation 1: For multi-tone NPUSCH transmission, the inter-Nslots slots repetitions within NPUSCH can be reused enable slot-level OCC scheme.

Observation 2: For single-tone NPUSCH transmission, the inter-Nslots slots repetitions within NPUSCH should also be supported to enable slot-level OCC scheme.

- Symbol-level OCC

For symbol-level OCC, the symbols within an OCC group (e.g., a slot) are firstly divided into NSF OCC blocks. Then, inter-symbol repetition is performed within the OCC group and OCC spreads across the NSF OCC blocks. Fig 2 illustrates the symbol-level OCC structure with OCC length NSF=2. The symbol-level OCC scheme applies to both single-tone and multi-tone NPUSCH transmission, but the multiplexed UE number can only be the value from {2,3,6}.

Observation 3: Symbol-level OCC applies to both single-tone and multi-tone NPUSCH transmission, but the multiplexed UE number can only be the value from {2,3,6}.

Fig 2. Symbol-level OCC with OCC length = 2

- Intra-symbol pre-DFT spreading OCC

According to the agreement, for multi-tone NPUSCH transmission, intra-symbol pre-DFT spreading OCC can be additionally taken into account. For OCC in frequency domain, block-wise spreading is applied to the allocated subcarriers, and UEs are multiplexed on the OCC blocks of subcarriers within an OCC group. The intra-symbol pre-DFT spreading OCC structure with OCC length NSF=2 is illustrated in Fig 3. Since only 3-tones/6-tones/12-tones are supported by NPUSCH format 1, the multiplexed UE number can only be the value from {3,6,12}.

Fig 3. Intra-symbol pre-DFT spreading OCC with OCC length = 2

Observation 4: For intra-symbol pre-DFT spreading OCC, the multiplexed UE number can only be the value from {3,6,12}.

The preliminary evaluation results of candidate OCC schemes, including slot-level OCC and symbol-level OCC, are provided in Fig 4-5, and the detailed simulation parameters are listed in Table 1 in the Appendix. Note that the REs used for TB size determination and NPUSCH repetition for different OCC schemes are the same as the NPUSCH transmission without OCC. In addition, the legacy DMRS on NPUSCH format 1 does not support UE multiplexing. Regarding this issue, in our OCC evaluation, slot-level OCC is also applied to DMRS to ensure the orthogonality of channel estimation between the OCC-multiplexed UEs.

When 10% BLER is taken as the performance metric, Fig 4 shows that, for single-tone NPUSCH transmission, slot-level OCC-2 and symbol-level OCC-2 presents 0.2dB and 0.4dB performance degradation on the required SNR respectively. When the multiplexed UE number increases to 4, slot-level OCC-4 shows a further required SNR loss of 1.4dB. However, from the NPUSCH capacity perspective, the aggregated throughput increases linearly with the OCC-multiplexed UE number. Besides, for multi-tone NPUSCH transmission, slot-level OCC-2 and symbol-level OCC-2 presents 0.6dB and 0.5dB performance degradation on the required SNR respectively, where a bit more loss than single-tone NPUSCH transmission is observed due to the timing drift. Similarly, the NPUSCH capacity also increases linearly with the OCC-multiplexed UE number.

Fig 4. Performance evaluation of non-OCC, slot-level OCC, and symbol-level OCC for single-tone NPUSCH

Fig 5. Performance evaluation of non-OCC, slot-level OCC, and symbol-level OCC for multi-tone NPUSCH

Observation 5: For single-tone NPUSCH transmission:

- Slot-level OCC-2 and symbol-level OCC-2 presents 0.2dB and 0.4dB performance degradation on the required SNR respectively.

- Slot-level OCC-4 shows a further required SNR loss of 1.4dB.

- The aggregated throughput increases linearly with the OCC-multiplexed UE number.

Observation 6: For multi-tone NPUSCH transmission:

- Slot-level OCC-2 and symbol-level OCC-2 presents 0.6dB and 0.5dB performance degradation on the required SNR respectively.

- The aggregated throughput increases linearly with the OCC-multiplexed UE number.

OCC for NPRACH

In RAN1#116b meeting, multiplexing UEs via OCC for NPRACH was preliminarily discussed, but no agreement was made due to limited TUs. As proposed in the FL summary [2], three candidate NPRACH OCC schemes can be considered by RAN1, including intra-symbol group OCC, inter-symbol group OCC and inter-repetition OCC. The legacy NPRACH transmission is based on single-subcarrier frequency-hopping symbol groups, where a symbol group consists of a CP and a sequence of N identical symbols as depicted in Fig 6.

Fig 6. Single-subcarrier frequency-hopping symbol groups for NPRACH

Based on the above symbol group structure, the N identical symbols within a symbol group can be reused to implement intra-symbol group OCC. That is, the OCC spreading is performed across the N identical symbols within the same symbol group. Fig 7 provides the preliminary evaluation results of intra-symbol group OCC and the detailed simulation parameters are listed in Table 2 in the Appendix. When 99% miss detection rate is taken as the performance metric, intra-symbol group OCC presents the similar performance as non-OCC on the required SNR when the false alarm rate is limited to 0.1%.

Fig 7. Performance evaluation of non-OCC and intra-symbol group OCC for NPRACH format 1

Observation 7: Intra-symbol group OCC presents the similar performance as non-OCC on the required SNR when the false alarm rate is limited to 0.1%.

Conclusion

In this contribution, we present our view and analysis on enhancements to support UE multiplexing with OCC for NPUSCH format 1 and NPRACH. The following observations and proposals are made:

Observation 1: For multi-tone NPUSCH transmission, the inter-Nslots slots repetitions within NPUSCH can be reused enable slot-level OCC scheme.

Observation 2: For single-tone NPUSCH transmission, the inter-Nslots slots repetitions within NPUSCH should also be supported to enable slot-level OCC scheme.

Observation 3: Symbol-level OCC applies to both single-tone and multi-tone NPUSCH transmission, but the multiplexed UE number can only be the value from {2,3,6}.

Observation 4: For intra-symbol pre-DFT spreading OCC, the multiplexed UE number can only be the value from {3,6,12}.

Observation 5: For single-tone NPUSCH transmission:

- Slot-level OCC-2 and symbol-level OCC-2 presents 0.2dB and 0.4dB performance degradation on required SNR respectively.

- Slot-level OCC-4 shows a further required SNR loss of 1.4dB.

- The aggregated throughput increases linearly with the OCC-multiplexed UE number.

Observation 6: For multi-tone NPUSCH transmission:

- Slot-level OCC-2 and symbol-level OCC-2 presents 0.6dB and 0.5dB performance degradation on required SNR respectively.

- The aggregated throughput increases linearly with the OCC-multiplexed UE number.

Observation 7: Intra-symbol group OCC presents the similar performance as non-OCC on the required SNR when the false alarm rate is limited to 0.1%.

Reference

[1]       RP-234077, “New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Deutsche Telekom, RAN#102, December 11-15, 2023

[2]       R1-2401607, “FL Summary #1 for IoT-NTN”, Sony, RAN1#116 meeting, February 26 – March 1, 2024

Appendix

Simulation assumptions

Table 1. LLS parameters for NPUSCH format 1

|  | Parameter | value |  |
| --- | --- | --- | --- |
| scenario | orbit | GEO | LEO600 |
|  | Elevation angle | 12.5 degree | 30degree |
| Channel and impairments | carrier frequency | 2GHz |  |
|  | Channel model | NTN-TDL-C The channels from different UE are independent. |  |
|  | Frequency error | Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. |  |
|  | Timing error | Uniform random selection from [-29Ts, +29Ts] for all UEs Timing drift 80us/s for LEO600 and 0 for GEO. |  |
| transmitter | SCS | 15KHz | 15kHz |
|  | Number of tones | Single tone | 12 tones |
|  | Waveform | DFT-s-OFDM |  |
|  | Frequency hopping | w/o frequency hopping |  |
|  | MIMO scheme | SISO |  |
|  | DMRS configuration | For baseline evaluations: OS#3 per slot for 15kHz For OCC evaluations: Slot-level OCC for DMRS | For baseline evaluations: OS#3 per slot for 15kHz For OCC evaluations: Slot-level OCC for DMRS |
|  | Number of resource unit (NRU) | 1 | 1 |
|  | Modulation order (Qm) | QPSK | QPSK |
|  | TBS (ITBS) | ITBS = 4 (56bits) | ITBS = 6 (88bits) |
|  | Number of repetitions (Nrep) | 8 |  |
|  | OCC length | 2, 4 |  |
|  | OCC sequence | OCC-2: [+1 +1] [+1 -1] OCC-4: [+1 +1 +1 +1] [+1 -j -1 +j] [+1 -1 +1 -1] [+1 +j -1 -j] |  |
|  | Number of UE | 2, 4 |  |
|  | Velocity of UE | 3km/h |  |
| receiver | Receiver algorithm | MMSE |  |
|  | Channel estimation | Real channel estimation |  |

Table 2. LLS parameters for NPRACH format 1

|  | Parameter | value |
| --- | --- | --- |
| scenario | orbit | GEO |
|  | Elevation angle | 12.5 degree |
| Channel and impairments | carrier frequency | 2GHz |
|  | Channel model | NTN-TDL-C The channels from different UE are independent. |
|  | Frequency error | Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. |
|  | Timing error | Uniform random selection from [-97Ts, +97Ts] for all UEs Timing drift 0 for GEO. |
| transmitter | SCS | 3.75kHz |
|  | Number of tones | Single tone |
|  | Waveform | NPRACH format 1 |
|  | Frequency hopping | As per NPRACH |
|  | MIMO scheme | SISO |
|  | Number of repetitions (NrepNPRACH) | 8 |
|  | OCC length | 5 |
|  | OCC sequence | In Table 6.3.2.4.1-2 in TS38.211 |
|  | Number of UE | 4 with same transmit power |
|  | Velocity of UE | 3km/h |
| KPI | Missed detection rate | SNR@ 1% miss detection rate |
|  | False alarm rate | 0.1% false alarm rate |