3GPP TSG RAN WG1 #116bis		R1-2403216

Changsha, Hunan Province, China, April 15th – 19th, 2024

Title:	Draft reply LS on Parallel Tx Capability

Response to:	R1-2401943 (R2-2401741)

Release:	Release 17

Work Item:	TEI17, NR_newRAT-Core, NR_2step_RACH, NR_IIOT_URLLC_enh

Source:	NTT DOCOMO, INC. (To be RAN1)

To:	RAN2

Cc:

Contact Person:

Name:	[Naoya Shibaike]

E-mail Address:	[naoya[dot]shibaike[dot]eg[at]nttdocomo[dot]com]

Attachments:	None

1	Overall description

RAN1 would like to thank RAN2 for the LS on Parallel Tx Capability and would like to provide the following response.

Q1: For the application of  “Simultaneous PUSCH and PUCCH transmissions of same priority on different inter-band cells” capability, which understanding is aligned with RAN1’s intention?

- Understanding #1: This capability can also be applied to the NR-DC band combination with the inter-band CA operation on the MCG/SCG;

- Understanding #2: This capability can only be applied to the inter-band CA band combination

[Answer#1]: RAN1’s understanding is aligned with Understanding#1 above.

Q2: Whether Features listed in the Table 1 follow understanding #1 or understanding #2?

Table 1: Parallel Tx Capability with Inter-band CA

| FG Index | Ran1 Description in the Feature list Table | Current Ran2 Field description |
| --- | --- | --- |
| 4-25(R15) | Parallel SRS and PUCCH/PUSCH transmission across CCs in inter-band CA | parallelTxSRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of SRS and PUCCH/ PUSCH across CCs in an inter-band CA band combination. |
| 4-26(R15) | Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | parallelTxPRACH-SRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band CA band combination. |
| 9-3(R16) | Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell | parallelTxMsgA-SRS-PUCCH-PUSCH-r16 Indicates whether the UE supports parallel transmission of MsgA and SRS/ PUCCH/ PUSCH across CCs in an inter-band CA band combination. A UE supporting this feature shall also indicate support of parallelTxPRACH-SRS-PUCCH-PUSCH. |
| 25-18(R17) | Support simultaneous PUCCH and PUSCH transmissions of different priority on different cells for inter-band CA. | parallelTxPUCCH-PUSCH-r17 Indicates whether the UE supports simultaneous PUCCH and PUSCH transmissions of different priority on different cells for inter-band CA. |

[Answer#2]: In RAN1 understanding, Features listed in the Table 1 follow Understanding#1

Q3: Whether Features listed in the Table 2 can also be applied to the NR-DC band combination with the intra-band non-contiguous CA operation on the MCG/SCG or only to intra-band non-contiguous CA band combination?

Table 2: Parallel Tx Capability with Intra-band non-contiguous CA

| FG Index | Ran1 Description in the Feature list Table | Current Ran2 Field description |
| --- | --- | --- |
| 39-2(R17) | Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in intra-band non-contiguous CA | parallelTxPRACH-SRS-PUCCH-PUSCH-intraBand-r17 Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an intra-band non-contiguous CA band combination. |
| 39-4(R17) | Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in intra-band non-contiguous CA(Prerequisite 9-3) | parallelTxMsgA-SRS-PUCCH-PUSCH-intraBand-r17 Indicates whether the UE supports parallel transmission of MsgA and SRS/ PUCCH/ PUSCH across CCs in an intra-band non-contiguous CA band combination. The UE indicating support of this field shall also indicate support of parallelTxMsgA-SRS-PUCCH-PUSCH-r16 and parallelTxPRACH-SRS-PUCCH-PUSCH-intraBand-r17. |
| 39-1(R17) | Parallel SRS and PUCCH/PUSCH transmission across CCs in intra-band non-contiguous CA | parallelTxSRS-PUCCH-PUSCH-intraBand-r17 Indicates whether the UE supports parallel transmission of SRS and PUCCH/ PUSCH across CCs in an intra-band non-contiguous CA band combination. |

[Answer#3]: In RAN1 understanding, Features listed in the Table 2 follow Understanding#1

RAN1 respectfully asks RAN2 to take above into consideration for future discussion.

2	Actions

To RAN2

ACTION: RAN1 respectfully asks RAN2 to take above into consideration for future discussion.

3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1#117		May 20 – May 24, 2024		Fukuoka City, Fukuoka, JP

TSG RAN WG1#118		August 19 – August 23, 2024 	Maastricht, NL