3GPP TSG RAN WG1 #116bis                                                              R1-2403639

Changsha, Hunan Province, China, April 15th – 19th, 2024

Agenda Item:	9.4.2.4

Source:	Moderator (Spreadtrum Communications)

Title:	FL summary#3 on CW waveform characteristics for A-IoT

Document for:	Discussion and decision

Introduction

This feature lead summary (FLS) concerns the Rel-19 study item (SI) on solutions for Ambient IoT (Internet of Things) in NR [1]. The detailed objectives can be seen in appendix (section 7.2). This Rel-19 WGs level study item was preceded by a Rel-18 RAN study item [2, 3] and a Rel-19 SA1 study item [4, 5].

The final CW (carrier wave) FLS in last RAN1 meeting can be found in [6], and the discussion in RAN#103 can be found in [7]. The achieved agreements in RAN1#116bis and RAN#103 are attached in section 7.3 and 7.4, respectively. The agreements achieved in this meeting will be added in section 7.5.

The contact info for 9.4.2.4 is attached in section 7.1, please feel free to update the contact info, if any.

This document summarizes contributions [8]-[37] submitted to agenda item 9.4.2.4, i.e., discussion on the characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device. The section arrangement and numbering in this document based on the submitted contributions, the issues that have been discussed extensively are placed at the front. If the summary missed or misunderstood companies’ views, FL apologizes for that, and please feel free to contact with FL or correct your position/views.

The issues in this document are tagged and color coded with High Priority or Medium Priority or Low priority. The issues that were in the focus of this round of the discussion are tagged FL6.

Follow the naming convention in this example:

- .. characteristics for A-IoT -v000.docx

- .. characteristics for A-IoT -v001-CompanyA.docx

- .. characteristics for A-IoT -v002-CompanyA-CompanyB.docx

- .. characteristics for A-IoT -v003-CompanyB-CompanyC.docx

There is NO need to send an info email to the reflector to inform that you have uploaded a new version of this document.

CW characteristics

According the SID [1], the device’s UL transmission is backscattered on a carrier wave provided externally. For evaluation, the study include whether/how to model carrier wave from node(s) inside or outside the connectivity topology. In addition, the necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device should be studied, including for interference handling at Ambient IoT UL receiver, and at NR base station (BS). Based on the SID and all the contributions, CW waveform, transmission cases and interference analysis are discussed by the majority. Therefore, these three aspects are treated in this section. Other issues are discussed in section 3.

CW Waveform

In the last meeting RAN1#116, it has been agreed that at least single-tone unmodulated sinusoid waveform is a candidate waveform for carrier wave (CW) for D2R backscattering, and multi-tone waveforms for carrier wave can also be studied.

Agreement For R19 A-IoT study item, at least single-tone unmodulated sinusoid waveform is a candidate waveform for carrier wave for D2R backscattering. Agreement For R19 A-IoT study item, multi-tone waveforms for carrier wave for D2R backscattering can be studied.

In this meeting, Contribution [8, 9, 10, 12, 13, 14, 15, 16, 18, 19, 20, 21, 22, 24, 25, 26, 27, 28, 29, 34, 35, 36] continue discussed CW waveform. Based on the contribution, the CW waveform discussion includes single tone and multi-tone, single tone and two kinds of multi-tones:

Single-tone:

  - Single-tone unmodulated sinusoid waveform

Multi-tone:

  - Multiple unmodulated single-tone, (e.g., two unmodulated sinusoid waveform located in different frequency point)

  - OFDM waveform, (e.g., conventional with each sub-carrier being either modulated or unmodulated or non-contiguous modulated multi-tone OFDM signal)

Candidate waveform for study [Closed]

- Single-tone

Contribution [8, 9, 10, 11, 13, 14, 16, 17, 19, 20, 21, 22, 26, 27, 30, 35, 36] continue share views for single-tone unmodulated sinusoid waveform. The pros and cons views are summarized below.

Table 2.1-1 views for single-tone CW waveform

| Waveforms | Pros | Cons |
| --- | --- | --- |
| Single tone unmodulated sinusoid waveform | - Low complexity, sufficient suppression for Carrier-wave interference [9, 10, 13, 14, 16, 17, 19, 20, 21, 36, 27, 36] - High spectrum efficiency of backscatter modulation [10] - Beneficial for harmonized design [10, 13] - In an ideal AWGN channel without fading, a single-tone CW signal outperforms a multi-tone CW signal for FSK-modulated backscattered signal [8] | - The single tone waveform may be impacted by deep fading[14, 19, 20, 21, 35] - Worse performance for OOK detection per LP-WUS experience [27] |

Furthermore, contribution [10, 11, 17, 18, 20, 34] propose to prioritize single-tone unmodulated sinusoid waveform for externally provided carrier wave for D2R backscattering.

- Multi-tone

Contribution [8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 24, 25, 26, 27, 30, 34, 35, 36] shared views for multi-tone waveforms, the views are summarized below.

Table 2.1-2 Views for multi-tone CW waveforms

| Waveforms | Pros | Cons |
| --- | --- | --- |
| Multiple unmodulated single-tone | - Beneficial to combat the frequency selective fading [13, 14, 15, 16, 21, 25, 26, 30, 34, 35, 36] - Better performance for OOK detection due to a flatter spectrum in frequency domain [27] - Beneficial to energy harvest [30, 34, 35 36] | - Increase complexity in interference cancellation [9, 10, 13, 21, 27, 30, 35] - Wider bandwidth, more complicated design of BS/intermediate node.[10, 16, 17, 18] - For non-OFDM based multi-tone CW, aliasing phenomenon will be generated [9] - Decrease the number of frequency shift values available for supporting FDMA of multiple devices.[34] - Possible for harmonized design, but with disadvantages[10] - Trade-off between increased BW and diversity gain [10] |
| OFDM based CW | - Reusing current hardware and signal [8, 20, 21, 26] - Beneficial to combat the frequency selective fading [8, 13, 14, 15, 16, 21, 25, 26, 30, 34, 35, 36] - Better performance for OOK detection due to a flatter spectrum in frequency domain [27] - Beneficial to energy harvest [30, 34, 35, 36] | - More challenging to handle self-interference and CLI [10, 13, 16, 21, 17, 27, 35] - Not feasible [16] - more complicated design of BS/intermediate node[10, 16, 17, 27] - Requires wider BW [10, 17] - Decrease the number of frequency shift values available for supporting FDMA of multiple devices.[34] - Leads to divergence from a harmonized design [10] |

For the above two kinds of multi-tone CW signals, Contributions [9, 10, 16, 18, 21, 27, 34, 35] propose to deprioritize/preclude OFDM waveform, while contributions [8, 15, 24, 30] propose to study both. Contribution [8] proposed a non-contiguous modulated multi-tone OFDM signal, and provided analysis for it. Contribution [11] observed that non-linear components in Ambient IoT devices can cause multi-tone waveforms (such as OFDM) to lose orthogonality. If multi-tone CW is supported, contribution [14] prefer mapping CW signal to non-continuous subcarriers.

For multiple unmodulated single-tone, contribution [16] thinks that two unmodulated sinusoid waveforms in two frequency tones can be considered, while contribution [27] proposes to study the number of unmodulated single-tone, and [27] further indicated that there should be a guardband between two adjacent tones. Contribution [25] indicated that the guardband should be at least 1.6 MHz for diversity gain. Contribution [35] prefer to study two-tone CW for backscattering from same node, or two-tone CW for backscattering from different nodes. Contribution [10] suggest to study the possible benefit of coverage enhancement.

Based on the above, it seems multiple unmodulated single-tone is supported by the majority, while contiguous modulated multi-tone OFDM signal is not. FL suggests to consider the following proposals.

FL1 High Priority Proposal 2.1-1a: For multi-tone CW waveform study, multiple unmodulated single-tone is studied in R19 SI.

FFS: the number of tones, whether/how large gap is needed between tones.

| Company | Y/N | Comments |
| --- | --- | --- |
| xiaomi | Yes |  |
| NTT Docomo | Y |  |
| FUTUREWEI | concerns | What is the assumption of the Ambient IoT system bandwidth for D2R? The number of multi-tone CW depends on the system bandwidth available for D2R transmission. The number of tones can only be determined if the system bandwidth is known. If agreed, the number of tones should be limited to 2 in the main bullet as a starting point, minimizing cases. |
| Apple | Y | FL please have a clear definition/boundary between multi-tone CW and non-contiguous OFDM. To us, the difference is related to distance between the multiple CW tones and the D2R signal BW. |
| TCL | Y | We agree with this proposal. To avoid aliasing phenomena, the gap between should be further studied and discussed. |
| Samsung | Y |  |
| CEWiT | Yes |  |
| Panasonic | Y |  |
| Qualcomm | Ok in principle |  |
| Sharp | Y |  |
| LGE | Y |  |
| OPPO |  | Multiple single-tone CW should be deprioritized while single-tone CW can be studied as the baseline. Multiple single-tone requires much complicated design of the reader no matter it is BS or a UE. multiple tones also requires wider bandwidth. Furthermore, it is not clear whether/how type-ii device to use amplification inside to backscatter the waveform on UL band. We suggest to deprioritize the study on multi-tone. |
| FL | Based on the replies so far, the following proposal is considered for Monday online discussion: FL2 High Priority Proposal 2.1-1a: For multi-tone CW waveform study, multiple unmodulated single-tone is studied in R19 SI. - Two unmodulated single-tones as a starting point - FFS: Other number of tones, whether/how large gap is needed between tones. - Note: ‘Unmodulated’ means without IFFT processing. |  |
| FL3 | The following agreements was achieved after online discussion, this issue is closed for this meeting. Agreement For CW waveform for D2R backscattering, multiple unmodulated single-tone is studied compared to single-tone in R19 SI. - Two unmodulated single-tones as a starting point - FFS: Other number of tones - FFS: how large gap is needed between tones |  |

FL1 High Priority proposal 2.1-2a: For multi-tone CW waveform study, contiguous modulated multi-tone OFDM signal is not studied in R19 SI.

FFS: non-contiguous modulated multi-tone OFDM signal.

| Company | Y/N | Comments |
| --- | --- | --- |
| xiaomi | Yes |  |
| NTT Docomo | Y |  |
| FUTUREWEI | Y | Suggest removing the FFS and revise the main bullet as follows: For multi-tone CW waveform study, contiguous modulated multi-tone OFDM signal is not studied in R19 SI. - FFS: non-contiguous modulated multi-tone OFDM signal. |
| Apple |  | FL please have a definition/example of modulated multi-tone OFDM signal. To us, modulated OFDM CW is similar to DL transmission, where OOK is modulated in time domain, and the frequency domain information (modulated sequence) is NOT decoded by reader. |
| TCL | Y |  |
| New H3C | Y |  |
| Samsung | Y |  |
| Panasonic | Y |  |
| Qualcomm |  | If the CW is outside the topology, the reason to use the modulated signal is not clear. Also, if modulated signal is used, it may not be known by the reader. We think modulated OFDM signal can be deprioritized. |
| Sharp | Y |  |
| LGE | Y |  |
| OPPO |  | Similar views/comments as above. Furthermore, multi-tone OFDM signal is difficult for CLI interference cancellation. |
| FL2 | @Apple, a note is added. @Qualcomm, may be you are right, but may be helpful for the harmonized design study. FL2 High Priority proposal 2.1-2a: For multi-tone CW waveform study, contiguous modulated multi-tone OFDM signal is not studied in R19 SI. - FFS: non-contiguous modulated multi-tone OFDM signal. - Note: ‘Modulated’ means with IFFT processing. |  |
| FL3 | The following agreements was achieved after online discussion, this issue is closed for this meeting. Agreement For CW waveform for D2R backscattering, contiguous multi-tone OFDM signal is not studied in R19 SI. |  |

Aspects for CW waveform study [Open]

For single-tone waveform, contribution [19, 21, 22, 35] propose to study frequency hopping for single tone to combat fading channel. Contribution [26] prefer to locate the CW is in the center of transmission bandwidth of D2R channel.  Contribution [29, 30, 35] think CP-OFDM based generation of single-tone CW waveform can be studied, and [35] further indicated that Non-OFDM-based transmitter can also be studied. Contribution [22] suggest to study in-band baseband generation of carrier wave reusing OFDM transmitter.

For the multi-tone waveforms, contribution [8] thinks that RAN 1 should further study the different aspects affecting the selection of the CW signal, i.e., the bandwidth allocated to the A-IoT devices, channel conditions, band deployment (in-band, out-of-band, guard band), backscatter modulation/detection assumptions, and interference mitigation techniques. Contribution [12] points out that waveform study depending on the scenario or topology, and RAN1 should identify the aspects and metrics that should drive the selection of the activation signal carrier-wave waveform. Contribution [19] propose to study the trade-off between signal restoration performance and the complexity of the filter required for CW interference while contribution [24] share the similar views that receiver complexity, D2R receiver interference suppression performance, spectrum efficiency, impact to legacy NR, etc should be considered. Contribution [26] focuses on efficient interference mitigation for CW waveform study.

For both single-tone and multi-tone, contribution [10] analyzed spectral efficiency, harmonized design, complexity and performance of the carrier-wave interference suppression.

Based on the above, it is suggest to have a common understanding on the aspects for CW waveform study.

FL1 High Priority Proposal 2.1-3a: For both single-tone and multi-tone CW waveform study, at least the following aspects should be studied.

- Performance (e.g., frequency selective fading, w/wo frequency hopping, etc)

- Spectrum efficiency

- Complexity of CW interference suppression at D2R receiver

- Complexity of CW transmitter (CW generation)

- Harmonized design

| Company | Aspect(s) | Comments |  |
| --- | --- | --- | --- |
| xiaomi | Yes |  |  |
| NTT Docomo |  | Agree on these aspects |  |
| FUTUREWEI | 5, 3 | There is a difference between capturing some sentences in a table in the TR and evaluations/numerical results. The need for some of these may also depend on the outcome of the previous questions. |  |
| Apple | OK | To study performance, spectrum efficiency etc, can we align the D2R transmission basic assumption such as BW, data rate etc. |  |
| TCL | Y | Frequency shifter may impact on the performance of frequency hopping, thus, we think this method can be discussed until frequency shifter has been agreed in section 9.4.1.2. |  |
| New H3C | OK |  |  |
| Samsung | OK |  |  |
| CEWiT | Yes |  |  |
| Panasonic | Y |  |  |
| Qualcomm |  | It may need to clarify in the main bullet of the proposal that it is for the waveform and transmission of CW for backscattering. For 5, does it means harmonized design for device 1/2a which requires CW for backscattering? |  |
| Sharp | OK |  |  |
| LGE | OK |  |  |
| OPPO |  | It is better to decide whether to prioritize to study multi-tone CW first, and then we can discuss it in this proposal about multi-tone CW. |  |
| FL2 | @Qualcomm, it is for the waveform and transmission of CW for backscattering. For 5, may also include device 2b. The following proposal (without update) can be tried during Monday online. FL2 High Priority Proposal 2.1-3a: For both single-tone and multi-tone CW waveform (if agreed) study, at least the following aspects should be studied. - Performance (e.g., frequency selective fading, w/wo frequency hopping, etc) - Spectrum efficiency - Complexity of CW interference suppression at D2R receiver - Complexity of CW transmitter (CW generation) - Harmonized design |  |  |
| Lenovo | Y |  |  |
| Huawei, HiSilicon | Agree |  | We wanted clarity on the complexity of the CW transmitter generation. This should not be relevant when the CW is outside the topology. In general, the complexity of the transmitter is not obviously part of the SI, and we doubt it is a major point to consider. In #1, we suggest to delete the parentheses, as it’s not sure yet that frequency fading is the only or primary aspect to look at. |
| FL3 | As there is no time to discuss this proposal online, companies can continue provide comments for the following proposal, if any. FL3 High Priority Proposal 2.1-3a: For both single-tone and multi-tone CW waveform (if agreed) study, at least the following aspects should be studied. - Performance (e.g., frequency selective fading, w/wo frequency hopping, etc) - Spectrum efficiency - Complexity of CW interference suppression at D2R receiver - Complexity of CW transmitter (CW generation) - Harmonized design |  |  |
| NTT Docomo | Agree |  |  |
| China Telecom | Agree | For 1, a clarification is needed that what performance metric can be utilized for frequency selective fading and other potential aspects. Considering no information is carried in the CW, the SNR cannot be evaluated. |  |
| Huawei, HiSilicon | Agree | We still have an issue with #4 on the complexity of the CW transmitter. This aspect should not be relevant when the CW is outside the topology. In general, the complexity of the transmitter is not obviously part of the SI, and we doubt it is a major point to consider. For #1, we suggest to delete the brackets. |  |
| vivo | Agree |  |  |
| IIT Kanpur, Indian Institute of Technology Madras | Agree | We agree with the study. |  |
| Ericsson | comment | We share the same concern of QC. We are fine to have harmonized design between Type 1 and 2a in mind for CW waveform study. It is not clear to us how to harmonize the CW wave design for active devices. |  |
| Samsung |  | After checking the proposed agreements again, I have come to think that this agreement discusses all aspects of CW’s waveform, interference and transmission. For #3 and 4, discussing them in section 2.3 and 2.2.1 respectively seems more appropriate. |  |
| CEWiT | Agree |  |  |
| FL4 | @CT, according to the contributions, BER for detection performance can be considered. Qualitative analysis would also fine. @HW, both 4 and bracket of 1 are removed @Ericsson, as mentioned by some contribution, if device 2b cannot support multi-tone transmission, the reader may need to decode two kinds of D2R signals. We can discuss this during today’s offline. @Samsung, thanks for the suggestion, the other possibility is to include all the interference discussions here. Based on the above, the following proposal is considered. FL4 High Priority Proposal 2.1-3a: For both single-tone and multi-tone CW waveform study, at least the following aspects should be studied. - Performance - Spectrum efficiency - Complexity of CW interference suppression at D2R receiver - For both self-interference and CLI - Suppression method and corresponding suppression level or level range - Complexity of CW transmitter (CW generation) - Harmonized design |  |  |
| FL5 | After online discussion, the following proposal is considered for the 3rd round FL5 High Priority Proposal 2.1-3b Study both unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering by considering at least the following aspects. - For D2R - Reception performance - Spectrum utilization of backscattered signal corresponding to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and CW cancellation capability value/range (if any) - For scenarios ’A1’, ’A2’ and ’B’ - Relative complexity of CW generation |  |  |
| Samsung |  | As mentioned in the online discussion, there is also an option that single tone CW employs frequency hopping. Since single-tone CW that uses frequency hopping and that does not will yield very different evaluation results from the above aspects. Therefore, it is necessary to distinguish between them. |  |
|  | FL5 High Priority Proposal 2.1-3b Study both unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering by considering at least the following aspects. - For D2R - Reception performance - Spectrum utilization of backscattered signal corresponding to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and CW cancellation capability value/range (if any) - For scenarios ’A1’, ’A2’ and ’B’ - Relative complexity of CW generation - Note: CW frequency hopping can be at least for single-tone unmodulated single-tone waveform study |  |  |
| FL6 | After offline, the following proposal seems stable FL6 High Priority Proposal 2.1-3c Study both unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering by considering at least the following aspects. - For D2R - Reception performance - Spectrum utilization of backscattered signal corresponding to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and CW cancellation capability value/range (if any) - For scenarios ’A1’, ’A2’ and ’B’ - Relative complexity of CW generation |  |  |

CW transmission

CW transmission cases without frequency shift [Open]

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, 6 CW transmission cases were agreed in the last meeting RAN1#116.

Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. - Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum - Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum - Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. - Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum - Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum - Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Contribution [8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 25, 26, 27, 29, 31, 32, 33, 35, 36] continues discussed CW transmission. The agreed cases in RAN1#116 can be illustrated by the following figure. In order to avoid misleading for the band of R2D transmission, the R2D arrow has been removed compared to the figure in last meeting [6].

| Case 1-1 | Case 1-2 | Case 1-4 |  |
| --- | --- | --- | --- |

Figure 2.2-1 CW transmission for topo 1

| Case 2-2 | Case 2-3 | Case 2-4 |  |
| --- | --- | --- | --- |

Figure 2.2-2 CW transmission for topo 2

Views from contributions for the above cases are summarized below.

Table 2.2-1 views for CW transmission cases

| Topology 1 |  | Topology 2 |  |
| --- | --- | --- | --- |
| Case 1-1 Inside DL | - FDD BS needs to support full-duplex capability in DL [8, 13, 20, 21, 25, 29, 31, 33, 36] - (D2R in DL)The spectrum regulation needs to be checked and clarified. [13,14,16, 17, 19, 20, 25, 31, 33, 36] - Dense deployment of gNBs to illuminate the devices[8] - Higher legal transmit power[10, 16] - Separate CW source not needed[16] - Interference will be worse[20] - Spectrum efficiency is lower and timeline scheduling overhead will be introduced[20] - No interference between the CW and NR signal is observed on BS side [23] - Self-interference at BS reception[27, 36] - Interference to Uu DL reception[25, 27, 36] | Case 2-2 inside UL | - UE should have high capability, e.g., the UE needs to operate in full-duplex mode, UE needs to receive signal in UL. (self-interference) [8, 10, 13, 14, 16, 20, 21, 27, 29, 31, 33, , 36 ] - May need RF-IC capability, RF interference cancelation and BB filtering [16, 21, 27, 31, 36] - No regulation issue[13, 16, 36] - Separate CW source not needed[16] - Lower CW Tx power[16] - May need to define UE behavior for receiving D2R transmission in UL spectrum.[31] - Interference to Uu UL reception[36] |
| Case 1-2 Inside UL | - FDD BS needs to support full-duplex capability in UL [8, 13, 14, 21, 29, 33, 36] - Dense deployment of gNBs to illuminate the devices[8] - BS transmits CW in an UL band may conflict regulatory restraints [9,10, 13, 14, 16, 17, , 31, 36] - No regulation limitation.[15] - Separate CW source not needed[16] - Lower CW Tx power [16] - RF-IC capability required, if CW source and receiver for backscatter share the same node [16] - Guard band between the CW and the NR signal to suppress the inter system interference[23] - May create interference on the UL transmission of legacy UEs[25] - Resulting in reduced coverage for the IoT devices.[25] - Self-interference at BS reception[27, 36] - Interference to Uu UL reception[27, 36] | Case 2-3 Outside DL | - Backscattered signal is transmitted in DL band may not align with regulation. [9,13, 14, 16, 17, 23, 33, 36] - The intermediate UE receives backscattered signal in a DL band, same as in current NR network [13] - Higher CW Tx power[16] - Need separate CW source[16] - Channel state information acquisition is needed[11] - Cross link interference (CLI)[12] - Lower requirements on RF-IC capability[16] - the UE needs to use both DL and UL bands if R2D in transmitted in UL[25] - Potential severe interference to the legacy UE reception[27] - Create interference with the NR signal on DL[31,33] - Coordination between the CWN and the UE[31] |
| Case 1-4 Outside UL | - Align with spectrum regulation[13, 16, 31, 36] - Relaxing the full duplex capability, align with current BS capability[8, 12, 13, 25, ] - Lower requirements on RF-IC capability[16] - Improve the coverage by adapting the distance between the outside CW emitter and tag.[25] - Cross link interference (CLI)[12] - Lower CW Tx power[16] - Need separate CW source[16] - The coverage performance must be confirmed[20] - The timing of the CW transmission has to be adjusted[25] - Coordination between the CWN and the BS[31] - Interference to legacy UL(36) | Case 2-4 Outside UL | - UE should have high capability, e.g., UE needs to receive signal in UL. [13] - Not required if two UEs are used for CW and D2R [36] - Align with the spectrum regulation [13, 16, 33, 36] - Improve the coverage by controlling the distance between the outside CW emitter and tag. - Full duplex capability may be needed if an intermediate UE simultaneously receives D2R in an UL band and transmits to gNB[8] - Channel state information acquisition is needed[11] - Cross link interference (CLI)[12] - Lower requirements on RF-IC capability[16] - Need separate CW source[16] - Lower CW Tx power[16] - The coverage performance must be confirmed[20] - UE receiving the backscattering signal in the UL can create interference[31, 36] - Coordination between the CWN and the UE[31] |

In addition to the above views, contribution [31] thinks that CW are transparent to a device for the case of, 1)whether CW is from inside/outside the topology; 2) whether the device is connected to topology 1 or topology 2. Contribution [8] indicates that supporting both Case 1-4 and Case 2-3 may require devices to differentiate the two topologies. Contribution [32] points out that the CW transmission spectrum would have impact on interference handling. In order to reduce complexity for UE, contribution [17] propose to study TDMed transmission.

For spectrum regulation, the understandings are little different for some cases, contribution [15] observed that NCR transmission is allowed in both UL and DL spectrum based on RAN4 requirement, then CW deployed inside the topology and transmitted in UL spectrum has no regulation limitation. Contribution [18] mentioned that the workaround solution of regulation constraint of the gNB transmission on the UL spectrum was to have the certification of the gNB transmission on the UL spectrum as one UE. However, contributions [9, 10, 13, 14, 16, 17, 31, 36] doubt it and think CW by BS in UL may not allowed. Contribution [13] suggest to send an LS to RAN4 or RAN to check spectrum regulation issues. Contribution [22] proposes to study whether reduced transmit power (e.g., maximum 23/26 dBm as UE) could satisfy the regulation for case 1-2.

In general, for CW transmission case study, the following aspects are provided by companies:

- Capability/complexity (e.g., gNB/UE needs to support full-duplex capability for inside CW transmission)

- Spectrum regulation

- CW node deployment (e.g., density, co-located/ non-co-located)

- Coverage (e.g., transmit power limitation, distance between CW and A-IoT)

- Interference type and handling at receiver side

- Interference to legacy NR

Based on the above, the following proposal is considered to have a common understanding on the aspects for CW transmission case study. For 5 listed below, details can be discussed in section 2.3.  For the aspect of interference to legacy NR, this is to be discussed in RAN4.

FL1/2 High Priority Proposal 2.2-1a:  For the CW transmission case study, at least the following aspects should be studied

- Capability/complexity (e.g., gNB/UE needs to support full-duplex capability for inside CW transmission)

- Spectrum regulation

- CW node deployment (e.g., density, co-located/ non-co-located)

- Coverage (e.g., transmit power limitation, distance between CW and A-IoT)

- Interference type and handling at receiver side

| Company | Aspect(s) | Comments |
| --- | --- | --- |
| xiaomi |  | We are generally fine with the proposal, but we think the transmission timing of the carrier wave shall also be studied, so we make the following revision with the blue part: For the CW transmission case study, at least the following aspects should be studied - Capability/complexity (e.g., gNB/UE needs to support full-duplex capability for inside CW transmission) - Spectrum regulation - CW node deployment (e.g., density, co-located/ non-co-located) - Coverage (e.g., transmit power limitation, distance between CW and A-IoT) - Interference type and handling at receiver side - The transmission timing of the carrier wave |
| NTT Docomo |  | Agree in general to study these aspects. One question for clarification: what does ‘co-located/non-located’ intends to study? |
| FUTUREWEI | At least 2 | Unclear that coverage should be discussed in this agenda |
| Samsung | 1, 2, 3 | 5 can be discussed in 2.3. Coverage of CW would be reflected in that of D2R. |
| CEWiT |  | We share the same view as Xiaomi. Carrier wave transmission timing should also be studied along with above aspects mentioned in FL proposal. |
| Qualcomm |  | For 2, we are not sure how to discuss regulation issue. Probably it is outside 3GPP scope, since the regulation may be different for different countries/regions. For 3, is it for outside CW case? For 5, what is the definition of the interference type? |
| LGE |  | Similar view as Xiaomi/CEWiT that the aspect of CW transmission timing needs to be studied. |
| OPPO | 2 | The 2nd aspect should be discussed and determine first. |
| Lenovo | 3 | CW node type may need to be discussed together with CW node deployment. CW ON/OFF signaling should be discussed |
| Huawei, HiSilicon | Not agree #2, #3, #4, #5 | #2: Regarding the spectrum regulation, it is not appropriate for RAN1 to open this. We cannot make any decisions for the regulations, and studying such aspects would be beyond the scope of 3GPP. Moreover, this is not defined in the SID to study spectrum related aspects. #3, #4: Delete this. CW node deployment goes beyond the reach of this agenda item. This may not need to be studied explicitly, e.g. companies report interference level in 9.4.1.1. #5: There is no SID scope to develop interference handling methods. However, a simple listing of the steps by which the interference is handled in general, together with the typical range of dB cancellation could be captured. But we are also fine to drop the bullet entirely. FL could clarify what “interference type” can be, and why it needs particular study rather than just straight identification. |
| FL3 | @xiaomi @LGE @CEWiT, FL’ s understanding is that the transmission timing of the carrier wave is more related to section 2.4, i.e., CW characteristics which need control. But your suggestion is accepted, let’s check other companies’ views for this. @Docomo, ‘co-located/non-located’ means whether a standalone CW node is needed. @Qualcomm, for 2, we can check other companies’ views during offline. For 3, it is for both inside and outside CW case, some contributions mentioned that case 1-1 requires dense deployment of gNBs to illuminate the devices. @Qualcomm, @HW, For 5, interference type includes self-interference and CLI. @Lenovo, CW ON/OFF signaling may related to CW control. Based on the above, the following proposal is considered FL3 High Priority Proposal 2.2-1a: For the CW transmission case study, at least the following aspects should be studied - Capability/complexity (e.g., gNB/UE needs to support full-duplex capability for inside CW transmission) - Spectrum regulation - CW node deployment (e.g., density, co-located/ non-co-located) - Coverage (e.g., transmit power limitation, distance between CW and A-IoT) - Interference type and handling at receiver side - The transmission timing of the carrier wave |  |
| Docomo |  | We feel the newly added bullet ‘aspects 6’ should belong to the discussion of CW node control. Other aspects seem related to study of e.g. pros and cons of the cases, but aspect 6 seems not. |
| China Telecom | 2,6 | For 2, The first thing needs to be confirmed is that whether CW can be regarded as a ‘DL’ based on its transmission direction, or as a ‘UL’ based on its transmission effect/result. I think the alignment should be achieved when we discuss the spectrum regulation first. For 6, seems it is related to other aspects and not need to be discussed here. |
| Huawei, HiSilicon | Not agree #2, #3, #4, #5 | #2: Regarding the spectrum regulation, it is not appropriate for RAN1 to open this. We cannot make any decisions for the regulations, and studying such aspects would be beyond the scope of 3GPP. Moreover, this is not defined in the SID to study spectrum related aspects. #3, #4: Delete this. CW node deployment goes beyond the reach of this agenda item. This may not need to be studied explicitly, e.g. companies report interference level in 9.4.1.1. #5: There is no SID scope to develop interference handling methods. However, a simple listing of the steps by which the interference is handled in general, together with the typical range of dB cancellation could be captured. But we are also fine to drop the bullet entirely. For #5 and #6, they both have dedicated proposals in Section 2.3 and 2.4, and we do not think we need to repeat them here. |
| vivo | 2, 3, [5] | For 2 Spectrum regulation, we agree that spectrum regulation is more important than other aspects, since it is feasibility issue rather than performance/complexity issue. Besides, we also share similar view with Huawei that, 3GPP may not be able to define or interpret the regulations, it is up to administrations in different regions. We do not know how RAN1 can proceed the study. For 3, co-located and non-collocated seems already covered by definition of the cases for CW transmission, e.g., inside topology or outside topology. Besides, density seems the same metric as coverage. For 5, for interference other than CW interference to receiver of R2D, we think it belongs to RAN4 co-existence study, we can reuse the results from RAN4 when available. |
| IIT Kanpur, Indian Institute of Technology Madras | Agree | We agree with the updated FL3 proposal. |
| Samsung |  | Similarly to the above comment, I suggest that this agreement aims to discuss only the carrier wave transmission itself. It would be appropriate to discuss aspect #5 in section 2.1.2 since interference type and complexity for the cancellation depend on the waveform. And aspect #6 can be discussed in section 2.4. |
| CEWiT |  | We agree with the modified FL proposal. |
| FL4 | Different companies share different views on whether those aspects should be studied, the following proposal is to be discussed in the offline FL4 High Priority Proposal 2.2-1a: For the CW transmission case study, at least the following aspects should be studied - Capability/complexity at D2R receiver side (e.g., gNB/UE needs to support full-duplex capability for inside CW transmission) - Spectrum regulation - CW node deployment (e.g., density, co-located/ non-co-located) - Coverage (e.g., transmit power limitation, distance between CW and A-IoT) - Interference type and handling at receiver side - The transmission timing of the carrier wave |  |
| FL5 | The following proposal is considered based on the replies FL5 High Priority Proposal 2.2-1b: Study the CW transmission cases by considering the following aspects. - Capability/complexity at D2R receiver side (e.g., whether the BS/intermediate UE needs to support full-duplex capability) - Interference type at D2R receiver side(e.g., self-interference or CLI) |  |
| Samsung |  | For the first bullet, it seems that the discussion on it could be included in CW interference suppression at D2R receiver and relative complexity of CW generation in the previous agreement (if agreed) |
| FL6 | After offline, the following proposal seems stable FL6 High Priority Proposal 2.2-1c: Study the CW transmission cases by considering at least the following aspects. - Capability/complexity at D2R receiver side - CW interference type at D2R receiver side (e.g., self-interference or CLI) |  |

In addition to the views collected above, several contributions propose to prioritize part of cases, their preference is summarized in the following table.

Table 2.2-2 Priority views for CW cases

| Topology 1 |  | Topology 2 |  |
| --- | --- | --- | --- |
| Case 1-1 Inside DL | [10] [23] | Case 2-2 Inside UL | [10] [18] [19] [20] [26] [22] [23] [25] [26] [36] |
| Case 1-2 Inside UL | [9] [18] [19] [20] [26] [23] [25] [26] | Case 2-3 Outside DL | [9] [21] |
| Case 1-4 Outside UL | [8] [21] [10] [14] [17] [18] [19] [20] [26] [21] [22] [25] [26] [33] [36] | Case 2-4 Outside UL | [10] [14] [17] [18] [19] [20] [26] [22] [25] [26] [33] [36] |

Besides, contribution [11] proposes to deprioritize 1-2, while contribution [27] prefers to preclude case 2-2/2-3 due to the self-interference issue at the intermediate UE.

According to the table, very few companies want study case 1-1 and case 2-3. However, in the last RAN1 meeting, down-selection/prioritize for these cases was tried, but 6 cases were agreed finally. In this regards, FL would like to ask companies’ views for further down-selection/prioritize for CW cases.

FL1/2/3 Medium Priority Question 2.2-2a: For CW transmission cases without frequency shift, whether further down-selection among the agreed 6 cases is needed, and if needed, which cases are preferred?

| Company | Y/N | Comments |
| --- | --- | --- |
| NTT Docomo |  | We are open to study all the cases. While if down-selection is needed, our preference is case 1-4, case 2-2, case 2-4. |
| Apple | Yes | Prefer further down-selection. CW transmission in DL spectrum should be deferred until other agenda confirm that D2R can happen in DL spectrum. |
| TCL | Y |  |
| Samsung | Y | Considering radio wave regulations, it seems appropriate to de-prioritize cases where the device transmits in DL spectrum. Therefore, we prefer cases 1-4, 1-2, 2-2, and 2-4. |
| CEWiT | Yes | Prefer further down selection in case of no frequency shift is supported. Our preference is case 1-4, case 2-2, case 2-4. |
| Panasonic | Y |  |
| Qualcomm |  | We feel it’s hard to down-select the cases in RAN1 due to different understandings/concerns among the companies. Also for Case 1-1, we think D1T1-A1/D1T1-A2 should be both considered. |
| Sharp | Y | We prefer to only study cases 1-4, 2-2 and 2-4. |
| LGE | Y | For the case with no frequency shift or small frequency shift, further down selection can be considered. Cases 1-2/1-4/2-2/2-4 are preferred. |
| Lenovo | Y | Since the passive device is topology agnostic, same spectrum whether DL or UL should be supported between topology 1 and topology 2. |
| Huawei, HiSilicon | Down-selection preferable | We are fine to capture all the options in the TR, but we would like to have some down-selection for reducing the cases that we would need to evaluate. We could target one inside topology and one outside topology case, for each of the topologies. |
| IIT Kanpur, Indian Institute of Technology Madras | Y | We prefer prioritizing the following cases for study - Case 1-1, Case 1-4, Case 2-2 and Case 2-4. |
| Ericsson |  | A clarification, does FL mean without large frequency shift? We think in these cases, small frequency shift up to tens of kHz can be supported. We prefer to prioritize Case 1-4, Case 2-2, and Case 2-4. |
| FL5/ FL6 | Based on the above, all the responses want to make down-selection, however, the supported cases are as follows: - Case 1-1: NTT, [Qualcomm], IIT Kanpur - Case 1-2: Samsung, LGE, - Case 1-4:Samsung, CEWiT, Sharp, LGE, IIT Kanpur, Ericsson - Case 2-2: NTT, Samsung, CEWiT, Sharp, LGE, IIT Kanpur, Ericsson - Case 2-3: - Case 2-4: NTT, Samsung, CEWiT, Sharp, LGE, IIT Kanpur, Ericsson The following proposal is considered. FL5/FL6 Medium Priority proposal 2.2-2b: For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following case for CW transmission is deprioritized. - Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum |  |

CW transmission cases with large frequency shift [Closed]

In the last RAN1 meeting, CW transmission cases without frequency shift was discussed. In the contribution submitted for this meeting, several contributions [8, 10, 13, 17, 19, 20, 21, 22, 23, 24, 25, 26 27, 29, 30, 31, 32, 34, 35, 36] discussed CW transmission with frequency shift capability (e.g., CW transmitted in DL band, and D2R transmitted in UL band).

Contribution [22, 29, 30, 35] generally propose to study the CW transmission cases with large frequency shift.  Contribution [31] points out that case 1-1 is OK if device support large frequency shift. Contribution [32] thinks that whether an A-IoT device can be capable of shifting frequency of the carrier wave would be a factor of deciding which band or spectrum to transmit carrier wave.

The proposed CW transmission cases with large frequency shift are summarize below

Table 2.2-3 Priority views for CW cases with large frequency shift

| Cases for topo1 | Supporters | Cases for topo2 | Supporters |
| --- | --- | --- | --- |
| Inside CW in DL, D2R in UL | [10] [21] [23] [25] [27] [31] [36] | Inside CW in DL, D2R in UL | [25] |
| Inside CW in UL, D2R in DL | [23] | Inside CW in UL, D2R in DL | [21] [23] [27] [36] |
| Outside CW in DL, D2R in UL | [8] [25] | Outside CW in DL, D2R in UL | [8] [10] [25] |
| Outside CW in UL, D2R in DL | [36] | Outside CW in UL, D2R in DL | [27] |

On the contrary, contribution [20] points out that if frequency shifter is considered, the power consumption and device complexity will be increased, which brings uncertainty to meet the target RAN requirements. Therefore, contribution [20] suggests to de-prioritize this study. Contribution [10] thinks that although the cases with frequency shift can be captured in the TR, the study de-prioritizes further effort on CW transmission cases with large frequency shift.

In addition, contributions [19, 13, 26] propose to revisit the issue after the conclusion of the discussion on the tag’s large frequency shift capability in Section 9.4.1.2. Contribution [17] assumed that A-IoT devices are not support large frequency shift as the baseline for study.

It seems that the majority are interested in the cases that D2R backscattering is transmitted in the different carrier as CW for D2R backscattering, and the following cases are preferred by most companies

- Topo 1: CW is transmitted from inside the topology, transmitted in DL spectrum. D2R is transmitted in UL

- Topo 2: CW is transmitted from inside the topology, transmitted in UL spectrum. D2R is transmitted in DL

| For topo 1: CW is transmitted from inside the topology, transmitted in DL spectrum. D2R is transmitted in UL | For topo 2: CW is transmitted from inside the topology, transmitted in UL spectrum. D2R is transmitted in DL |
| --- | --- |

Figure 2.2-3 part of CW transmission cases with large frequency shift capability

Based on the above, the following proposal is suggested.

FL1/2/3/5 Medium Priority Proposal 2.2-3a:

For the case that D2R backscattering is transmitted in the different band as CW for D2R backscattering, and for topology 1, at least the following case for CW transmission is studied.

Case 1-1a: CW is transmitted from inside the topology, transmitted in DL spectrum

For the case that D2R backscattering is transmitted in the different band as CW for D2R backscattering, and for topology 2, at least the following case for CW transmission is studied.

Case 2-2a: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum

Note: This study can be captured in the TR only when the support of frequency shifter is confirmed in 9.4.1.2.

| Company | Y/N | Comments |
| --- | --- | --- |
| xiaomi | Yes |  |
| NTT Docomo | Yes |  |
| FUTUREWEI | Y |  |
| Apple | N | Defer the study, until 9.4.1.2 confirm its capability for device type 1. |
| Samsung |  | We need to check the feasibility of a large frequency shifter in tags and radio wave regulations first. |
| CEWiT | Y |  |
| Qualcomm |  | Share similar view as Apple, it can be deferred after more progress in 9.4.1.2. |
| LGE | N | We share the same view with Apple and Qualcomm. For the case where CW and D2R are transmitted in different bands, capability/feasibility of device 1 on supporting such large frequency shift should be checked first. |
| OPPO |  | Similar view with Samsung and apple. The feasibility for device, especially for type 1, should be considered first. |
| Lenovo |  | Agree with Apple, Qualcomm. This issue can be deferred after reaching agreements in 9.4.1.2. We prefer same design for topology 1 and topology 2. |
| Huawei, HiSilicon |  | We would prefer to deprioritize this for now, at least until we have clarity on whether the device can support frequency shifting or not, which is being discussed in AI 9.4.1.2. With low priority, we would be fine to study the following cases: For Topology 1, the CW is transmitted from inside the topology, transmitted in DL spectrum. For Topology 2, the CW is transmitted from outside the topology, transmitted in DL spectrum. |
| Vivo |  | Similar view as companies that cases with large frequency shift capability can be deferred or deprioritized. |
| IIT Kanpur, Indian Institute of Technology Madras | Yes | We agree with the study. |
| Ericsson |  | ‘different band’ in the two main sentences can be changed to ‘different carriers’, so as to align the wording in the agreements in RAN1#116. We think the intention is not to discuss inter-band backscattering. For Topology 1, CW transmitted outside the topology in DL spectrum can also be considered. The reason is that the distance between CW node and a device is much smaller than the distance between a reader and a device. CW node outside of topology can reduce the Gnb density. |
| FL5 | Based on the above, the views are quite divergent. FL suggests to postpone this study, until 9.4.1.2 confirm its capability for device type 1. This issue is closed this meeting. |  |

CW interference to A-IoT receiver [Closed]

Contribution [8, 11 13, 14, 22, 23, 29, 27, 16, 18, 19, 25, 28, 37, 36] analyzed interference caused by CW at A-IoT receiver side. There are two aspects for interference analysis, i.e., interference type and interference handling methods. Views from contributions are summarized below:

Table 2.3-1 CW interference and handling at A-IoT receiver side

| CW transmission | Interference type | Handling methods |
| --- | --- | --- |
| Inside CW cases case 1-1/1-2/2-2 | Self-interference [9, 10, 13, 15, 16, 17, 19, 22, 26, 27, 29, 31, 36, 37, 35 | - A-IoT frequency shift (large shift by shifter, or small shift by coding schemes, square waves) [8, 9, 11 13, 14, 22, 23, 29, 27, 18, 19, 25] - Receiver hardware design [13, 14, 29] (Circulator[16, 25], Tx to RX antenna separation[16], cancellation circuit[16], High-pass filter[16], directional coupler [25], up to implementation [17, 23], RF or BB domain[26]) - Synchronization between the CW node and the reader[8] - Design of known information, such as the preamble[37] - TDM between CW/backscattered wave and UE’s DL/UL[17] - Outside CW[16, 18] - Guard band between CW and D2R[23] - Two BSs are used for CW and D2R[36] - Transmission power[22] |
| Outside CW case case 1-4/2-3/2-4 | Cross-link/direct link interference [9, 10, 12, 13, 17, 29, 37, 35] | - A-IoT frequency shift [11, 18, 25, 29, 13, 14, 22] - Management/control between the CW node and the reader.[12, 17, 18] - QCL relationships between reference signals, the cross link interference (CLI) framework or pre-coding of the CW signal [12] - Hardware design [13, 14, 25] (RF front end (circulator, etc.)[25], RF/analog interference cancellation [25], filtering[25]) - Beam isolation[26, 25] - simplified RF-IC cancellation circuitry [16] - Transmission power[22] |

In addition to the above, detailed analysis for interference and handling method for different waveforms are conducted in contribution [9] and [10]. Contribution [9] further proposed to study the interference cancellation method and aliasing problem, e.g., power control and resource configuration. Contribution [8] points out that the receiver needs to decode the backscattered signals amid a potentially stronger (and interfering) carrier wave signal transmitted by the CW node. Contribution [12] observed that CW signal can induce very high interference levels (up to 50 Db) at the reader in bi-static setup. While contribution [15] observed that the interference strength in case 1-1 is higher than that of case 1-2 because from devices’ perspective, the transmission power and the number of antenna ports in DL spectrum may be larger than that of transmission in UL spectrum. In addition, contribution [26] mentioned that multipath propagation causing signal fading and inter-symbol interference. Some contributions (e.g., [10] and [16]) provided interference suppression level for the handling method.

To enable efficient inference mitigation and cancellation, contribution [26] suggest to study the procedure for CW node selection and transmission. Contribution [27, 37] also propose to FFS the handling methods. Contribution [19] suggests to study time duration, period, central frequencies, and frequency hopping pattern of CW for interference handling. For the CW transmission time, contribution [24] point out that CW shall not be an always-on transmission.

For evaluation, contribution [37] thinks that interference should be included in evaluation assumptions for LLS, and contribution [35] proposes to prioritize Case 1-1, Case 2-2 and Case 2-4 for evaluation.

Based on the above, it is observed that for CW interference to A-IoT receiver, interference type identification and interference handling method are the main discussion point. Therefore, FL propose to consider the following proposal at this stage.

FL1 High Priority Proposal 2.3-1a:  At least the following aspects for CW interference handling at A-IoT receiver side are studied:

- Interference type caused by CW

- CW interference handling at A-IoT receiver side (e.g., handling method and the corresponding interference suppression level or level range.)

| Company | Y/N | Comments |
| --- | --- | --- |
| xiaomi | Yes |  |
| NTT Docomo | Yes |  |
| FUTUREWEI | Y |  |
| TCL | Y |  |
| New H3C | Y |  |
| Samsung | Y | We tend to agree with this proposal. For CW interference handling at AioT receiver side, the complexity of interference suppression at receiver side also should be considered. For instance, the complexity of implementing a filter to eliminate interference from single-tone CW waveform might be significant. |
| CEWiT | Yes |  |
| Panasonic | Y |  |
| Qualcomm |  | Not clear what is definition of the interference type. To cancel the interference of non-self-CW (not transmitted by reader itself), the estimation of the CW needs to be considered. The cancelling 20Db of the non-self-CW (not generated by the reader itself) could be more challenging than cancelling 60Db of the self-CW. |
| Sharp | Y |  |
| LGE | Y |  |
| OPPO |  | Different cases may have different types interferences. When case 1-4 and 2-4 is supported, and the CWN is under control, the interference can also be handled at CWN side. |
| FL2 | @Qualcomm, according to the contribution, the identified interference type includes self-interference and cross link interference. FL2 High Priority Proposal 2.3-1a: At least the following aspects for CW interference handling at A-IoT receiver side are studied: - Interference type caused by CW (e.g., self-interference and cross-link interference) - CW interference handling at A-IoT receiver side (e.g., handling method and the corresponding interference suppression level or level range.) |  |
| Lenovo | Y | To avoid the confusion of which receiver we are talking about, device receiver or reader receiver. We suggest the following: FL1 High Priority Proposal 2.3-1a: At least the following aspects for CW interference handling at A-IoT the receiver side of D2R signal are studied: |
| Huawei, HiSilicon |  | The second bullet needs to be clarified. There is no SID scope to develop interference handling methods. However, a simple listing of the steps by which the interference is handled in general, together with the typical range of Db cancellation could be captured. But we are also fine to drop the second bullet entirely. |
| FL3 | Based on Lenovo’s suggestion, the following version is considered. FL3 High Priority Proposal 2.3-1a: At least the following aspects for CW interference handling at A-IoT the receiver side of D2R signal are studied: - Interference type caused by CW (e.g., self-interference and cross-link interference) - CW interference handling at A-IoT receiver side (e.g., handling method and the corresponding interference suppression level or level range.) |  |
| China Telecom | Y |  |
| Huawei, HiSilicon |  | We repeat our comment regarding the 2nd bullet. There is no SID scope to develop interference handling methods. However, a simple listing of the steps by which the interference is handled in general, together with the typical range of Db cancellation could be captured. But we are also fine to drop the second bullet entirely. |
| Vivo | Y |  |
| IIT Kanpur, Indian Institute of Technology Madras | Y | We agree with the updated FL3 proposal |
| Ericsson |  | We have a small wording suggestion made in blue below to clarify the aggressor. FL3 High Priority Proposal 2.3-1a: At least the following aspects for CW interference handling at A-IoT the receiver side of D2R signal are studied: - Interference type caused by CW node (e.g., self-interference and cross-link interference) - CW interference handling at A-IoT receiver side (e.g., handling method and the corresponding interference suppression level or level range.) |
| Samsung |  | I believe “D2R transmission” is more appropriate since we usually refer to is as D2R transmission instead of D2R signal. Interference suppression complexity should be included in this agreement. |
| CEWiT | Y | We agree with the modified version of proposal. |
| FL4 | Based on the Samsung’s suggestion for FL3 High Priority Proposal 2.1-3a and FL3 High Priority Proposal 2.2-1a, the following proposal can be combined into FL4 High Priority Proposal 2.1-3a. We can check whether this is accept by other company. FL4 High Priority Proposal 2.3-1a: At least the following aspects for CW interference handling at A-IoT the receiver side of D2R signal transmission are studied: - Interference type caused by CW node (e.g., self-interference and cross-link interference) - CW interference handling at A-IoT receiver side (e.g., handling method and the corresponding interference suppression level or level range.) |  |
| FL5 | This issue is combined in section 2.1.2, this section is closed. |  |

CW characteristics which need control [Open]

Some contributions discussed CW control related issues. Contribution [8, 31] proposes to study possible signaling and/or data impacts of the control of a CW node, Contribution[17, 22, 23, 24, 28, 35] proposes to study a part or full of the following aspects for CW control: timing(e.g., always on or on-demand), power, T/F resource. In addition, contribution [28] thinks that the reader or network should be able to control the reference signal that is sent on the CW. Contribution [19] thinks tags can expect the carrier wave to be appropriately emitted in a manner that does not hinder D2R transmission from them. Contribution [13, 19, 21, 22, 30] discussed which CW node should be controlled, e.g., inside node or outside node. Contribution [13] suggests RAN1 to study and clarify how an intermediate UE performs CW transmission.

Contribution [15] indicates that the transmission time of CW used for D2R backscattering should at least include the continuous time from the end of R2D to the completion of D2R transmission. Contribution [18] think that carrier-wave should be transmitted after the R2D signal to allow the A-IoT device processing the R2D signals. While contribution [16] proposed that not support UE to be the CW node without R2D transmission or D2R reception with AIoT device.

In addition to that, contribution [14, 16] think that control of CW signal transmission between the external CW node and the base station, may be up to implementation and/or out of scope for A-IoT study. Contribution [19] understands that CW control should be discussed in next phases of AIoT work.

The issue of whether CW node can be controlled was discussed in RAN#103, the following agreements was achieved in RAN#103.

RAN#103 agreement: - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc. - No SID revision is necessary

According to the agreements and the discussion in RANP [7], the SI can consider “characteristics which would need control” and does not introduce the study of how to do that control.

Based on the above, FL proposes to collect companies’ views for which CW characteristics which would need control.

FL1/FL2 Medium Priority Question 2.4-1a:  From RAN1 perspective, which CW characteristics would need control?

| Company | Comments |  |
| --- | --- | --- |
| xiaomi | We think at least following aspects need to be controlled，because these waveform characteristics will impact interference, e.g., self- interference, inter-system interference between AIoT and NR. - when CW is transmitted or not transmitted - power - bandwidth - spectrum |  |
| NTT Docomo | We think following at least aspects of control of CW characteristics can be studied. - Time resource - Frequency resource - Power |  |
| Apple | The following needs to be controlled: - Which CW node to transmit (outside of topology). - When CW is transmitted/not transmitted. (ON/OFF). - Time resource/frequency resource/Power when it is ON. |  |
| TCL | The following aspects should at least consider we think - Transmission power - Time and frequency resource |  |
| New H3C | The following needs to be controlled: - transmission power - Time/frequency resource |  |
| Samsung | We believe that control is needed for time/frequency resources, transmission power, spectrum, and frequency hopping patterns. However, we want to make it clear that this agreement does not imply that the method of controlling CW nodes (e.g., signaling) is discussed at the SI. This can rather be discussed in the WI. |  |
| Panasonic | The following aspects should be considered: Transmission power Time and frequency allocations Beam or directional transmission if the CW supports it. |  |
| Sharp | Time and frequency resources for CW transmission, TX power, |  |
| LGE | we share the similar view with DOCOMO and Sharp. Transmission timing, F-domain resource, and TX power for CW transmission need to be studied. |  |
| Lenovo | Transmission resource including frequency and/or time domain, transmit power of CW |  |
| FL3 | The views are summarized below. 1. Timing (e.g., when CW is transmitted or not transmitted, Time resources) Xiaomi, NTT, Apple, TCL, New H3C, Samsung, Panasonic, sharp, LGE, Lenovo, 2. Transmission Power Xiaomi, NTT, Apple, TCL, New H3C, Samsung, Panasonic, sharp, LGE, Lenovo, 3. Frequency resources (includes bandwidth) Xiaomi, NTT, Apple, TCL, New H3C, Samsung, Panasonic, sharp, LGE, Lenovo, 4. Spectrum Xiaomi, Samsung 5. Which CW node to transmit (outside of topology). Apple 6. frequency hopping patterns Samsung 7. Beam or directional transmission if the CW supports it. Panasonic Based on the above, the following proposal is considered FL3/4 Medium Priority Question 2.4-2a From RAN1 perspective, at least the following CW characteristics would need control: - Timing (e.g., when CW is transmitted or not transmitted, Time resources) - Transmission Power - Frequency resources (includes bandwidth) - Spectrum - Which CW node to transmit (outside of topology). - frequency hopping patterns - Beam or directional transmission if the CW supports it. FFS what kind of control is needed or suggested Note: How to do that control (including signaling design) is not studied in R19 SI phase. |  |
| Company | Y/N | Comments |
| Docomo |  | Since we focus on FR1, we think control of beam may not be needed. |
| China Telecom | Y | For Beam or directional transmission, considering gNB has the ability and UE doesn’t have in the FR1, we can separately study them based on node type, so Beam or directional transmission can be included. |
| vivo |  | For ‘Which CW node to transmit (outside of topology)’, we think is already covered by ‘when CW is transmitted or not transmitted’ under Timing. We would like to clarify the meaning of ‘bandwidth’ under frequency resource, since only single tone and multiple non-contiguous single tone CW are agreed now, does it mean number for tones for CW. |
| IIT Kanpur, Indian Institute of Technology Madras | Y | We agree with the updated FL3 proposal. |
| CEWiT | Y | We agree with the modified version of proposal. |
| FL5 | @vivo, “ Which CW node to transmit (outside of topology).” is removed, companies can discuss this aspects in the first sub-bullet. For ”BW”, the brackets is removed. The following proposal is considered. FL5 Medium Priority Question 2.4-2b From RAN1 perspective, at least the following CW characteristics would need control: - Timing (e.g., when CW is transmitted or not transmitted, Time resources) - Transmission Power - Frequency resources - Spectrum - Frequency hopping patterns - Beam or directional transmission if the CW supports it. FFS what kind of control is needed or suggested Note: How to do that control (including signaling design) is not studied in R19 SI phase. |  |
| Samsung |  | FL5 Medium Priority Question 2.4-2b From RAN1 perspective, at least the following CW characteristics would need study: - Timing (e.g., when CW is transmitted or not transmitted, Time resources) - Transmission Power - Frequency resources - Spectrum - Frequency hopping patterns - Beam or directional transmission if the CW supports it. FFS what kind of control is needed or suggested Note: How to do that control (including signaling design) is not studied in R19 SI phase. |
| FL6 | Based on the reply, the following proposal is considered. FL6 Medium Priority Question 2.4-2c From RAN1 perspective, at least the following CW characteristics would need study: - Timing (e.g., when CW is transmitted or not transmitted, Time resources) - Transmission Power - Frequency resources - Spectrum - Frequency hopping patterns - Beam or directional transmission if the CW supports it. FFS what kind of control is needed or suggested Note: How to do that control (including signaling design) is not studied in R19 SI phase. |  |

Others

Coexistence with NR/LTE

Some contribution discussed coexistence related issues, e.g., CW interference to legacy NR/LTE and NR/LTE interference to A-IoT system.

- CW interference to legacy NR/LTE

Contribution [15, 17, 20, 27] shared very detailed analysis for CW interference to legacy NR case by case, and think interference handling, includes gNB-gNB CLI, UE-device CLI and so on should be considered. Contribution [18, 25, 29, 36] proposes to FFS the CW interference on NR users. Contribution [19] suggests to study the impact of CW interference on legacy UEs assuming legacy DL transmission FDMed with CW. Contribution [8] points out that if CW is transmitted in an UL band, and it interferes with the gNB receiving the legacy UL transmissions. Contribution [8] suggest RAN1 to study the handling of interference from a CW node to a gNB in Topology 2 Case 2-4. Contribution [18] suggests to study inter-device interference cancelation.

In addition to the interference cases discussed above, some contribution also shared CW handling method at legacy side, e.g., contribution [10] recommended that sub-carrier orthogonality between the external carrier-wave and NR uplink or downlink signal, contribution [12] [22] [23] suggest to introduce sufficient guard band, contribution [23] thinks that dropping rule would be helpful. In addition to that, up to BS’s implementation (scheduling or hardware design) [17][20][22] and CW power control [28] are considerable.

- NR/LTE interference to A-IoT system

Some contribution also discussed interference from legacy to A-IoT system, e.g., contribution [15, 21, 25] discussed interference to A-IoT system case by case, e.g., for case 1-1, interference to A-IoT system mainly comes from NR DL transmission because both R2D and D2R transmission of A-IoT system are in DL spectrum. Contribution [36] observed interference source (e.g., from legacy NR DL or UL) for each CW transmission case.

For the above two coexistence issues, contribution [10] indicated that the CW interference to legacy NR is expected to be evaluated by RAN4. Contribution [35] also points out that RAN1/4 will discuss how to the evaluation scenarios and the coexistence evaluation study, e.g., co-channel coexistence/adjacent channel coexistence, will be studied in RAN4. Contribution [35] further indicated that the interference handling schemes may be studied in RAN1, if needed.

FL: According to the SID scope (as copied below), RAN4 will study co-existence including evaluation and interference scenarios. In addition, as far as FL knows, RAN4’ contributions in simultaneous meetings, are focused on coexistence.  Therefore, FL thinks that there is no need to discuss these parts in RAN1, unless RAN4 ask RAN1 to involve.

- RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception

Other aspects

Energy harvesting

In RAN#103, the following agreement was reached

RAN#103 agreement: - Confirm that study of design of energy harvesting signal/waveform is out of SI scope in Rel-19 - The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study [RAN2, RAN1] - Duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds - Note: this value can be revisited in future RAN plenary meetings, if necessary - TR 38.848 clause 5.6 statement on latency remains the case with respect to a single device, i.e.: “NOTE: The time for charging the Ambient IoT device storage (if present) is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implementation issue only.” - No SID revision is necessary

In this meeting, some contribution share views for energy harvesting.

Table 3.2-1 Views for energy harvesting

| Views | Details |
| --- | --- |
| No further study | Contribution [11] indicates that RAN1 should not further discuss technical design of energy harvesting signal/waveform for Device 1, Device 2a, and Device 2b. Contribution [15] proposed that design of signal/waveform for CW for energy harvesting is discussed as low priority in RAN1. |
| FFS standardization impact | Contribution [22]: Study the standardization impact if the carrier wave transmission is used for RF energy harvesting in additional to provide carrier wave for backscattered communication |
| CW for backscattering can be used for EH | Contribution [23]: It is preferred that the CW can also be used for energy harvesting from the cost and deployment complexity aspect. Study the UE behaviour when the device is in a state of energy harvesting. Contribution [32]: The study of carrier wave assumes that carrier wave transmission can be used for energy harvesting for A-IoT devices. |
| Resource allocation for EH CW | Contribution [35]: CW for EH and R2D are in the same carrier frequency for device 1/2a/2b, No new waveform is needed for CW for energy harvesting, Study the resource allocation of CW for EH |

FL: Based on the above, energy harvesting will not be discussed in RAN1#116bis.

Miscellaneous

Contribution [9] propose to clarify the D2R frequency spectrum in given cases according to CW transmission spectrum.

FL: The D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, the figure is updated in section 2.2.1. Whether to study the case that the D2R backscattering is transmitted in different carrier as CW for D2R backscattering is discussed in 2.2.2.

Contribution [10] propose to prioritize CW case 1-1/1-4/2-2/2-4 for evaluation purposes. Contribution [15] thinks the specific values of bandwidth and power for CW for backscattering need to be considered for D2R backscattering evaluation.

FL: this can be discussed in 9.4.1.1.

Contribution[12] indicates that RAN1 to study or specify mono-static and bi-static terminology by considering whether the device(s) acting as CW node and the device(s) acting as readers for backscattered signal from tag are the same or not, respectively.

FL: For the point indicated by [12], FL understand that this is already included in the CW cases in section 2.2.1.

Contribution [15] thinks that spectrum used for CW, backscatter and R2D transmission should be considered together. Contribution [15] proposed that CW, backscatter and R2D transmission in same spectrum (e.g. case 1-2 (a)) to reduce A-IoT device complexity, interference to NR UE.

Contribution [32] thinks that RAN1 to study whether and how an A-IoT device reports its capability on frequency shifting to base station.

Contribution [28] suggests RAN1 to study both ambient and dedicated external carrier waves.

Contribution [34] studied time delay for forwarding, timing adjustment, collision handling between R1 and R2 and so on for different deployment cases listed in 9.4.1.1(e.g., D1T1-A1, D1T1-A2)

FL: For the above, as mentioned by very few companies, they will not be discussed with high priority at least in this meeting.

Proposals for offline discussion

Tuesday

FL4 High Priority Proposal 2.1-3a: study both single-tone and multiple single-tone CW waveforms for backscattering by considering the following aspects.

- For D2R

  - reception performance

  - relative spectrum efficiency of backscattered signal correspond to the CW waveforms

- CW interference suppression at D2R receiver

  - Including complexity and suppression level(if any)

  - For scenarios ’AI’, ’A2’ and ’B’

Note: Companies are encouraged to perform a qualitative analysis of the above aspects

Note: The digital baseband processing of CW self-interference handling is not modelled in link level simulation.

FL4 High Priority Proposal 2.2-1a:  study the CW transmission cases by considering the following aspects.

- Capability/complexity at D2R receiver side (e.g., gNB/UE needs to support full-duplex capability for inside CW transmission)

- Spectrum regulation

- CW node deployment (e.g., density, co-located/ non-co-located)

- Coverage (e.g., transmit power limitation, distance between CW and A-IoT)

- Interference type(e.g., self-interference or CLI) and handling at receiver side

- The transmission timing of the carrier wave

FL4 High Priority Proposal 2.3-1a:  At least the following aspects for CW interference handling at A-IoT the receiver side of D2R transmission are studied:

- Interference type caused by CW node (e.g., self-interference and cross-link interference)

- CW interference handling at A-IoT receiver side (e.g., handling method and the corresponding interference suppression level or level range.)

After offline discussion. The following proposals are candidate proposals for Wednesday online treatment. FL4 High Priority Proposal 2.1-3a: Study both single-tone and multiple single-tone CW waveforms for backscattering by considering the following aspects. - For D2R - Reception performance - Relative spectrum efficiency of backscattered signal correspond to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and suppression level(if any) - For scenarios ’A1’, ’A2’ and ’B’ Note: Companies are encouraged to perform a qualitative analysis of the above aspects. Note: The digital baseband processing of CW self-interference handling is not modelled in link level simulation. FL4 High Priority Proposal 2.2-1a: Study the CW transmission cases by considering the following aspects. - Capability/complexity at D2R receiver side (e.g., whether the gNB/UE needs to support full-duplex capability) - Interference type(e.g., self-interference or CLI)

Wednesday

FL4 Proposal 2.1-3a

Study both unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering by considering at least the following aspects.

- For D2R

  - Reception performance

  - RSpectrum utilization of backscattered signal corresponding to the CW waveforms

- CW interference suppression at D2R receiver

  - Including complexity and CW cancellation capability value/range (if any)

  - For scenarios ’A1’, ’A2’ and ’B’

- Relative complexity of CW generation

FL4 High Priority Proposal 2.2-1a:  Study the CW transmission cases by considering the following aspects.

- Capability/complexity at D2R receiver side (e.g., whether the gNB/UE needs to support full-duplex capability)

- Interference type(e.g., self-interference or CLI)

FL5 Medium Priority proposal 2.4-2b (No discussion in offline)

From RAN1 perspective, at least the following CW characteristics would need study:

- Timing (e.g., when CW is transmitted or not transmitted, Time resources)

- Transmission Power

- Frequency resources

- Spectrum

- Frequency hopping patterns

- Beam or directional transmission if the CW supports it.

FFS what kind of control is needed or suggested

Note: How to do that control (including signaling design) is not studied in R19 SI phase.

FL5 Medium Priority proposal 2.2-2b (No discussion in offline)

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following case for CW transmission is deprioritized.

- Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum

Back up (if any) for proposal 2.2-1a

Study the CW transmission cases by considering at least the following aspects.

- Required Capability at D2R receiver side

  - For CW transmission and D2R reception

- Complexity at D2R receiver side

  - For CW interference suppression

  - CW interference type (e.g., self-interference or CLI)

After discussion, the following proposals are candidates for online discussion Proposal 2.1-3b (offline consensus) Study both unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering by considering at least the following aspects. - For D2R - Reception performance - Spectrum utilization of backscattered signal corresponding to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and CW cancellation capability value/range (if any) - For scenarios ’A1’, ’A2’ and ’B’ - Relative complexity of CW generation Proposal 2.2-1a (offline consensus) Study the CW transmission cases by considering at least the following aspects. - Capability/complexity at D2R receiver side - CW interference type at D2R receiver side (e.g., self-interference or CLI)

Thursday

Proposal 2.2-1a ()

Characterize the agreed CW transmission cases at least according to the following aspects

- Capability/complexity at D2R receiver side

- CW interference at D2R receiver side (e.g., self-interference, CLI)

- [CW interference at NR side]

Proposal 2.4-2b (No discussion in offline)

From RAN1 perspective, at least the following CW characteristics would need study:

- Timing (e.g., when CW is transmitted or not transmitted, Time resources)

- Transmission Power

- Frequency resources

- Spectrum

- Frequency hopping patterns

- Beam or directional transmission if the CW supports it.

FFS what kind of control is needed or suggested

Note: How to do that control (including signaling design) is not studied in R19 SI phase.

The following agreement is for reference

RAN#103 agreement: - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc. - No SID revision is necessary

Proposal 2.2-2b (No discussion in offline)

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following case for CW transmission is deprioritized.

- Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum

Proposals for online discussion

Monday

| Proposal 1 | Proposal 2.1-1a: For multi-tone CW waveform study, multiple unmodulated single-tone is studied in R19 SI. - Two unmodulated single-tones as a starting point - FFS: Other number of tones, whether/how large gap is needed between tones - Note: ‘Unmodulated’ means without IFFT processing |
| --- | --- |
| Corresponding agreements 1 (if any) | Agreement For CW waveform for D2R backscattering, multiple unmodulated single-tone is studied compared to single-tone in R19 SI. - Two unmodulated single-tones as a starting point - FFS: Other number of tones - FFS: how large gap is needed between tones |
| Proposal 2 | Proposal 2.1-2a: For multi-tone CW waveform study, contiguous modulated multi-tone OFDM signal is not studied in R19 SI. - FFS: non-contiguous modulated multi-tone OFDM signal - Note: ‘Modulated’ means with IFFT processing |
| Corresponding agreements 2 (if any) | Agreement For CW waveform for D2R backscattering, contiguous multi-tone OFDM signal is not studied in R19 SI. |
| Proposal 3 | Proposal 2.1-3a: For both single-tone and multi-tone CW waveform (if agreed) study, at least the following aspects should be studied. - Performance (e.g., frequency selective fading, w/wo frequency hopping, etc.) - Spectrum efficiency - Complexity of CW interference suppression at D2R receiver - Complexity of CW transmitter (CW generation) - Harmonized design |
| Corresponding agreements 3 (if any) | Not treated on Monday |
| Proposal 4 | Proposal 2.3-1a: At least the following aspects for CW interference handling at A-IoT receiver side are studied. - Interference type caused by CW - CW interference handling at A-IoT receiver side (e.g., handling method and the corresponding interference suppression level or level range) |
| Corresponding agreements 4 (if any) | Not treated on Monday |
| Proposal 5 | Proposal 2.2-1a: For the CW transmission case study, at least the following aspects should be studied. - Capability/complexity (e.g., gNB/UE needs to support full-duplex capability for inside CW transmission) - Spectrum regulation - CW node deployment (e.g., density, co-located/ non-co-located) - Coverage (e.g., transmit power limitation, distance between CW and A-IoT) - Interference type and handling at receiver side |
| Corresponding agreements 5 (if any) | Not treated on Monday |

Wednesday

| Proposal 1 | FL4 Proposal 2.1-3a: Study both single-tone and multiple single-tone CW waveforms for backscattering by considering the following aspects. - For D2R - Reception performance - Relative spectrum efficiency of backscattered signal correspond to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and suppression level(if any) - For scenarios ’A1’, ’A2’ and ’B’ Note: Companies are encouraged to perform a qualitative analysis of the above aspects. Note: The digital baseband processing of CW self-interference handling is not modelled in link level simulation. |
| --- | --- |
| Corresponding agreements 1 (if any) | Versions after online treatment FL4 Proposal 2.1-3a Study both unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering by considering at least the following aspects. - For D2R - Reception performance - Spectrum utilization of backscattered signal corresponding to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and CW cancellation capability value/range (if any) - For scenarios ’A1’, ’A2’ and ’B’ - Relative complexity of CW generation |
| Proposal 2 | FL4 Proposal 2.2-1a: Study the CW transmission cases by considering the following aspects. - Capability/complexity at D2R receiver side (e.g., whether the BS/intermediate UE needs to support full-duplex capability) - Interference type at D2R receiver side(e.g., self-interference or CLI) |
| Corresponding agreements 2 (if any) | Not treated on Monday |
| Proposal 3 | Proposal 2.4-2a From RAN1 perspective, at least the following CW characteristics would need control: - Timing (e.g., when CW is transmitted or not transmitted, Time resources) - Transmission Power - Frequency resources (includes bandwidth) - Spectrum - Which CW node to transmit (outside of topology) - frequency hopping patterns - Beam or directional transmission if the CW supports it. FFS: What kind of control is needed or suggested. Note: How to do that control (including signaling design) is not studied in R19 SI phase. |
| Corresponding agreements 3 (if any) | Not treated on Monday |

Thursday

The following proposals are considered for Thursday online

Proposal 2.1-3b (offline consensus)

Study both unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering by considering at least the following aspects.

- For D2R

  - Reception performance

  - Spectrum utilization of backscattered signal corresponding to the CW waveforms

- CW interference suppression at D2R receiver

  - Including complexity and CW cancellation capability value/range (if any)

  - For scenarios ’A1’, ’A2’ and ’B’

- Relative complexity of CW generation

Proposal 2.2-1a (the 3rd aspect is unstable)

Characterize the agreed CW transmission cases at least according to the following aspects.

- Capability/complexity at D2R receiver side

- CW interference at D2R receiver side (e.g., self-interference, CLI)

- [CW interference at NR side]

Proposal 2.4-2b (No discussion in offline)

From RAN1 perspective, at least the following CW characteristics would need study:

- Timing (e.g., when CW is transmitted or not transmitted, Time resources)

- Transmission Power

- Frequency resources

- Spectrum

- Frequency hopping patterns

- Beam or directional transmission if the CW supports it.

FFS what kind of control is needed or suggested

Note: How to do that control (including signaling design) is not studied in R19 SI phase.

Proposal 2.2-2b (No discussion in offline)

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following case for CW transmission is deprioritized.

- Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum

Reference

|  | RP-240826 | Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#103, Maastricht, Netherlands. | RAN WGs SID (Revised SID) |
| --- | --- | --- | --- |
|  | RP-223396 | Study on Ambient IoT (Internet of Things) in RAN | RAN SID |
|  | TR 38.848 | Study on Ambient IoT (Internet of Things) in RAN | RAN TR |
|  | S1-220118 | Study on Energy Harvesting enabled Communication Services in 5GS. | SA1 SID |
|  | TR 22.840 | Study on Ambient power-enabled Internet of Things | SA1 TR |
|  | R1-2401855 | Final FL summary on CW waveform characteristics for A-IoT, RAN1#116, February 2024. | Moderator (Spreadtrum) |
|  | RP-240854 | Moderator's summary on R19 Ambient IoT, RAN#103, Maastricht, Netherlands. | Moderator (Huawei) |
|  | R1-2401975 | Waveform characteristics of carrier wave provided externally to the Ambient IoT device | Ericsson |
|  | R1-2401979 | Discussion on waveform characteristics of external carrier-wave for Ambient IoT | TCL |
|  | R1-2402016 | On external carrier wave for backscattering based Ambient IoT device | Huawei, HiSilicon |
|  | R1-2402045 | Discussion on External Carrier Waveform Characteristics for Ambient IoT | FUTUREWEI |
|  | R1-2402077 | Waveform characteristics of carrier-wave provided externally to the Ambient IoT device | Nokia |
|  | R1-2402110 | Discussion on waveform characteristics of external carrier-wave for Ambient IoT | Spreadtrum Communications |
|  | R1-2402136 | Discussions on waveform characteristics of carrier-wave for A-IoT | Intel Corporation |
|  | R1-2402189 | Discussion on carrier wave for Ambient IoT | ZTE, Sanechips |
|  | R1-2402247 | Discussion on CW waveform and interference handling at AIoT UL receiver | vivo |
|  | R1-2402333 | Discussion on Waveform characteristics of carrier-wave provided externally to the A-IoT device | OPPO |
|  | R1-2402388 | Discussion on the waveform characteristics of carrier-wave for the Ambient IoT device | CATT |
|  | R1-2402471 | Considerations for waveform characteristics of carrier-wave | Samsung |
|  | R1-2402515 | Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device | China Telecom |
|  | R1-2402570 | Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device | CMCC |
|  | R1-2402587 | Discussion on external carrier wave for ambient IoT | Lenovo |
|  | R1-2402671 | Discussion on waveform characteristics of carrier-wave | Xiaomi |
|  | R1-2402739 | Discussion on waveform characteristics of externally provided carrier-wave | Sharp |
|  | R1-2402860 | Discussion on carrier-wave for Ambient IoT | InterDigital, Inc. |
|  | R1-2402886 | Views on carrier waveform and interference handling for AIoT | Apple |
|  | R1-2402951 | On carrier-wave waveform characteristics for A-IoT | MediaTek |
|  | R1-2402972 | External carrier wave for Ambient IoT | Sony |
|  | R1-2403002 | Discussion on waveform characteristics of carrier-wave for Ambient IoT device | Panasonic |
|  | R1-2403023 | Discussion on waveform characteristics of carrier-wave provided externally to the A-IoT device | ETRI |
|  | R1-2403062 | Discussion on Waveform characteristics of carrier-wave provided externally to the Ambient IoT device | CEWiT |
|  | R1-2403093 | Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device | Google |
|  | R1-2403094 | Considerations for waveform characteristics of carrier-wave | Semtech Neuchatel SA |
|  | R1-2403122 | Considerations on carrier-wave transmission for Ambient IoT | LG Electronics |
|  | R1-2403199 | Waveform characteristics of carrier-wave provided externally to the Ambient IoT device | Qualcomm Incorporated |
|  | R1-2403249 | Study on waveform characteristics of carrier-wave for Ambient IoT | NTT DOCOMO, INC. |
|  | R1-2403399 | Discussion on Carrier wave related aspects for AIoT | IIT Kanpur, Indian Institute of Technology Madras |

Appendix

Contact info for 9.4.2.4

Based on feedback in RAN1#116. Please feel free to update it, if any.

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| NTT Docomo | Weiqi Sun | sunwq@docomolabs-beijing.com.cn |
| FUTUREWEI | Brian Classon | brian@classonconsulting.com |
| China Telecom | Yi Gu | guy6@chinatelecom.cn |
| xiaomi | Yajun Zhu Fu ting Wensu Zhao | zhuyajun@xiaomi.com futing@xiaomi.com zhaowensu@xiaomi.com |
| OPPO | Teng Ma | mateng1@oppo.com |
| CEWiT | Deepak PM Vishakha Singh | deepakpm@cewit.org.in vish@cewit.org.in |
| TCL | Rongling Jian | rongling.jian@tcl.com |
| Samsung | Hyemin Choe | hams.choe@samsung.com |
| Nokia/NSB | Ganesh Venkatraman | Ganesh.venkatraman@nokia.com |
| Spreadtrum | Mimi Chen | mimi.chen@unisoc.com |
| CATT | Fang-Chen Cheng Ren Da | fcc@catt.cn renda@catt.cn |
| SONY | Martin Beale | martin.beale@sony.com |

SID(Revised in RAN#103)

- 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI This study targets a further assessment at RAN WG-level of Ambient IoT, a new 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power. General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case. Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum. The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. - RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential. - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. - RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception RAN2 and RAN3 are expected to identify RAN-CN functional split in coordination with SA2. Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies.

RAN1#116 agreements for CW

Agreement For R19 A-IoT study item, at least single-tone unmodulated sinusoid waveform is a candidate waveform for carrier wave for D2R backscattering. Agreement For R19 A-IoT study item, multi-tone waveforms for carrier wave for D2R backscattering can be studied. Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. - Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum. - Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum. - Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum. Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. - Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum. - Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum. - Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum.

RAN#103 Agreements

RAN#103 agreement: - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc. - No SID revision is necessary RAN#103 agreement: - Confirm that study of design of energy harvesting signal/waveform is out of SI scope in Rel-19 - The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study [RAN2, RAN1] - Duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds - Note: this value can be revisited in future RAN plenary meetings, if necessary - TR 38.848 clause 5.6 statement on latency remains the case with respect to a single device, i.e.: “NOTE: The time for charging the Ambient IoT device storage (if present) is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implementation issue only.” - No SID revision is necessary RAN#103 agreement: - RAN design targets for user experienced data rate, maximum message size, and moving speed of device: those can be used as assumptions in coverage evaluations, i.e. the coverage evaluations are done under the conditions that meet those targets. - Evaluations of RAN design targets for latency and connection/device density are allowed by the Rel-19 SID and observations on those evaluations can be captured in the TR38.769 - Note: this is as per the SID: “NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically.”

RAN1#116bis Agreements for CW

Agreed on Monday