3GPP TSG RAN WG1 #117			R1-2404203

Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

Agenda Item:	9.11.1

Source:	Moderator (Thales)

Title:	FL Summary #2: NR-NTN downlink coverage enhancements

Document for:	Discussion, Decision

## Introduction

This feature lead summary (FLS) document aims to collect and align on company views on Release-19 NR-NTN downlink coverage enhancements. It contains a summary of the contributions under 9.11.1 at TSG-RAN WG1 #117, together with identified key issues. The goal of this FLS is to facilitate consensus-building and offer recommendations for prioritizing discussions, including considerations for potential postponements of certain issues.

A total of 31 TDocs have been submitted to current meeting for discussion. The source contributions are cited in references [1]-[31]. The companies’ proposals are listed in the appendix.

# Topic#1 System Level Study (SLS) results

## Companies’ contributions summary

The following proposals and observations on SLS are submitted to current RAN1 meeting:

| Company | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 2: The unequal distribution of user density among different beam footprints should be considered in the system level coverage enhancement. Proposal 3: The following method is used in the system level evaluation under the agreed framework with N1, N2 and N3 beam footprints: - In each simulation drop: - Step 1: Randomly select N3 beam footprints among 1058 beam footprints, where N3=106 for Set1-1, Set1-3/and N3=16 for Set 1-2; - Step 2: Randomly drop X UEs per each selected N3 beam footprint, where X is reported by companies; - Step 3: Generate traffic based on the agreed traffic model, if user perceived throughput and cell throughput are simulated. |
| Eutelsat Group | Observation 1: The conclusions of [2] for FR1-NTN antenna performance are an improvement over the original assumptions and are equally applicable to FR2-NTN. Proposal 1: The single antenna gain, antenna spacing and consequent results from ‘option 3’ in [2] should be scaled in frequency (from S-band to Ka band) and adopted as the basis of evaluation in FR2-NTN (or at least used as an option). Observation 2: The values suggested in [3] for FR2-NTN are generally in line with industry expectations. Observation 3: The beam pattern and frequency reuse in FR2-NTN will have similar considerations to FR1-NTN. Observation 4: Similar to FR1-NTN, FR2-NTN traffic patterns will not follow terrestrial density or traffic patterns. Observation 5: As is the case for FR1-NTN, FR2-NTN access latency and coverage can be traded off to achieve a range of latency-coverage objectives a satellite operator may have. The trade-off may vary over the lifetime as the constellation is densified. |
| Fraunhofer IIS, Fraunhofer HHI | Observation 1: A beam dwell time of 2ms might not be enough to transmit common control signaling (e.g., SSB, Type0/0A-PDCCH, SIB1, and SIB19) and other data transmissions. Observation 2: A maximum dwell time of 16ms could be achieved for set1-1 and set1-3 if an SSB periodicity of 160ms was considered. Observation 3: Set 1-2 is not enough to ensure receiving the necessary common control signaling (e.g., SSB, Type0/0A-PDCCH, SIB1, and SIB19) even by considering an SSB periodicity of 160ms. Observation 4: Increasing the beam dwell time increases the beam revisit time, which requires increasing the SSB periodicity. Observation 6: More satellite footprints could be covered using the active satellite beams by sweeping each active satellite beam in an SSB TDM-mulitplexed manner. Observation 7: The SSB periodicity of 20ms could be achieved for set1-1 and set1-3 by sweeping each active satellite beam in an SSB TDM-mulitplexed manner, where each SSB burst consists of 4 SSBs and the beam dwell time is 5ms required to transmit the SSB burst. Observation 8: The SSB periodicity of 20ms could not be achieved for set1-2 even by sweeping each active satellite beam in an SSB TDM-mulitplexed manner, where an SSB periodicity of at least 80ms might be required. |
| Spreadtrum Communications | Observation 8: If the total number of simultaneously active beams is 16, extending the default SSB periodicity is needed to achieve 100% coverage ratio. Observation 9: If the total number of simultaneously active beams is 106, extending the default SSB periodicity may be not needed to achieve 100% coverage ratio. |
| THALES | Observation 7: Set1-2 FR1 with % simultaneously active beams of 1,5% requires SSB periodicity of at least 80ms. That is, a default periodicity of 20ms is not doable for such deployment scenario. Observation 8: Set 1-1 and Set 1-3 FR1 with % simultaneously active beams of 10% increasing the SSB periodicity from up to 160ms allows an increase of the cell available capacity for user traffic and the cell throughput by 30%. Observation 9: Set1-2 FR1 can work with a default SSB periodicity of 20ms if and only if the % simultaneously active beams is at least equal to 4.1% (only 394 beam are illuminated). Observation 10: WGS84 flattened sphere Earth model used LEO600 with 4.417° 3 dB beamwidth for 38.821 Set1. If beam layout defined in Table 6.1.1.1-4 in TR 38.821 beamwidth is used, 547 beams (13 tiers) is the largest possible before outside beams stretch over the horizon. Observation 11: Observations 7, 8 and 9 are confirmed with the System level simulations results. Observation 12: System level simulations results and analytical evaluation results lead to the same conclusion on the necessity to extend the default SSB periodicity to support satellite beam hopping in NTN. |
| ZTE | Observation 1: Based on the SLS evaluation results, it is observed that the number of active beams is not enough to serve N3 footprints for LEO600km Set1-1 to Set1-3 FR1 scenarios. Observation 2: Based on the SLS evaluation results, it is observed that the number of active beams that can be dedicatedly used for N2 footprints is zero for LEO600km Set1-1 to Set1-3 FR1 scenarios. Observation 3: The 1058 beam footprints cannot be served with a maximum revisit time of 20ms with the necessary common channel transmission. Observation 4: For Set 1-1 and Set 1-2, the minimum dwell time is 4ms due to repetition of common channel. Observation 5: For Set 1-3, the minimum dwell time is 13ms due to repetition of common channels. Observation 6: To support MSG1 detection, a minimum dwell time of 13.2ms is needed for a give beam. Observation 7: If a complete initial access procedure is involved, a minimum dwell time of 31.5ms is needed for a give beam. Observation 8: For Set 1-1, a minimum SSB periodicity of 640ms is needed to support necessary DL common channels transmission and random-access procedure. Observation 9: For Set 1-2, a minimum SSB periodicity of 640ms is needed to support necessary DL common channels transmission. Observation 10: For Set 1-3, a minimum SSB periodicity of 320ms is needed to support necessary DL common channels if link level enhancement is introduced for coverage. |
| Apple | Observation 11: In LEO-600 set 1-1 and set 1-3 in FR1 with N1=N2=0 and N3=1058, the coverage ratio is 100%, with dwell time = 2 ms, revisit time = 20 ms and the periodicity of SSB and SIB1 = 20 ms. Observation 12: In LEO-600 set 1-1 and set 1-3 in FR1 with N1=0, N2=846 and N3=212, the coverage ratio is 100%, with dwell time = 6 ms (on N3 beam footprints), revisit time = 20 ms and periodicity of SSB and SIB1 = 20 ms. Observation 13: In LEO-600 set 1-2 in FR1 with N1=N2=0 and N3=1058, the coverage ratio is 100%, with dwell time = 2.4 ms, revisit time = 160 ms and periodicity of SSB and SIB1 = 160 ms. Observation 14: In LEO-600 set 1-2 in FR1 with N1=0, N2=846 and N3=212, the coverage ratio is 100%, with dwell time= 8.4 ms (on N3 beam footprints), revisit time =160 ms and periodicity of SSB and SIB1 = 160 ms. Observation 15: In LEO-600 set 1-2 in FR1 with N1=N3=0 and N2= 320, the coverage ratio is 30%, with dwell time = 1 ms (on N2 beam footprints), revisit time = 20 ms and periodicity of SSB and SIB1 = 20 ms. |
| LG Electronics | Observation 1: The required simultaneously active beam ratio only for SSB with 15kHz SCS is at least 1.43% when the number of SSBs within 20msec periodicity is 1. For other DL transmission including common channels, the active beam ratio of 1.5% in Set 1-2 is not feasible. Observation 2: The required simultaneously active beam ratio only for SSB with 30kHz SCS is at least 0.71% when the number of SSBs within 20msec periodicity is 1. For other DL transmission including common channels, the active beam ratio of 1.5% in Set 1-2 could be feasible. However, it would be necessary to increase BW to be more than 5MHz since the BW of the SSB with 30kHz SCS occupies 7.2MHz. |
| CATT | - Under the random beam scheduling method, the interference is more severe when the number of physical beams is 106 compared to that of 16 physical beams. - In realistic deployment of LEO 600km at FR1, if one satellite can provide 16 beams activation, the coverage ratio can be optimized from 6% to 96.9%. - In realistic deployment of LEO 600km at FR1, if one satellite can provide 106 beams activation, the coverage ratio can be optimized from 40% to 80.2%. - SSB periodicity with 320ms can provide better performance in coverage ratio when only 16 beams are activated. |
| CMCC | Observation 10. No obvious difference between N2 and N3 is observed, - N2 beam footprints can support at least system information and UE initial access with wide/narrow beam - N3 beam footprints can support both active traffic and system information with narrow beam Observation 11. For LEO-600 Set1-1/1-2/1-3, even if all the active beams are used for common channels, it cannot satisfy 100% coverage ratio with the default common channel periodicity. Observation 12. It is observed that the coverage ratio can be improved with longer duration of common channel periodicity from UE side. Observation 13. For LEO-600 Set1-1/1-2/1-3, longer revisit time can be considered to improve the coverage ratio. |
| ETRI | Observation 3: The additional reference satellite parameters scenarios for LEO600km Set1-1 and Set1-3 in FR1 can satisfy the SSB periodicity of 20 ms. Observation 4: The additional reference satellite parameters scenarios for LEO600km Set1-2 in FR1 cannot satisfy the SSB periodicity of 20 ms. Observation 5: Increasing the time period of common signals can accommodate the 1058 cells with the additional reference satellite parameters for LEO600km Set1 in FR1, as well as reduce overhead. |
| Baicells | Observation 3: For NTN DL coverage, coverage ratio for common messages should be no less than that for user traffic, and therefore is more challenging and needs more attention. Observation 4: Limited to the legacy specification, the maximum coverage ratio for SSB in a cell is 40%, 6%, and 40% for FR1 Set1-1, Set1-2, and Set1-3 respectively. |
| OPPO | Proposal 1: For system level study based on analytical evaluation: - The dwell time and revisit time of the beam footprints in state “off” are 0. - The dwell time and revisit time of the beam footprints in state “common message only” and “active traffic” are reported by companies. Observation 1: For DL coverage evaluation at system level, the majority of beam footprints are in state “off”. Observation 2: For LEO600 set 1-1/set 1-3, the coverage ratio of the beam footprints in state “common message only” and state “active traffic” can achieve 100% when 80%-90% beam footprints in state “off” is assumed. - The coverage ratio larger than 100% implies that satellite resource exceeds demand. Observation 3: For LEO600 set 1-2, most of scenarios can achieve 100% DL coverage except the high cell load scenario. - Extending the revisit time or increasing satellite beam number per cell can be considered to improve the DL coverage at system level. - Extending the revisit time faces backward compatibility issue. - When the revisit time of the N3 beam footprints in state “active traffic” is 160ms, the coverage ratio can achieve 100% for all scenarios. |
| MediaTek Inc. | Observation 1: For narrow beams with NSSB=4 Beam hopping: - Set 1-2: SSB periodicity of 320 ms can provide coverage of 96.8% and efficiency per beam of 96.9%. - Set 1-1/1-3: SSB periodicity of 80 ms can provide coverage of 96.6% and efficiency per beam of 87.5%. Observation 2: For wide beam covering 4 narrow beams with NSSB=4 Beam hopping: - Set 1-2, SSB periodicity of 80 ms can provide coverage of 96.8% and efficiency per beam of 96.9%. - Set 1-1/1-3, SSB periodicity of 80 ms can provide coverage of 100% and efficiency per beam of 87.5%. |
| Nokia, Nokia Shanghai Bell | Observation 1: Areas covered by N1 cells without being covered by other cells will be considered without coverage. Proposal 1: RAN1 to prioritize FR1 in the downlink coverage enhancements studies for Rel-19. Proposal 2: RAN1 to discuss what is the maximum amount of area which can be out of coverage. |

## Companies SLS results summary

The main companies results on DL coverage evaluation at system level are recapped within the following sub-sections:

### Minimum Dwell time

[Ericsson]:

Comparison of dwell times for different beam layout configurations when the revisit time = 20 ms.

| System parameter | System configuration |  |  |
| --- | --- | --- | --- |
|  | Beam lay out with ABS = 0.0668 (i.e., ~ 50 km beam size) + frequency reuse factor = 2 | Beam lay out with ABS'=0.0945 (i.e., 70.7 km beam size) + frequency reuse factor = 2 | Beam lay out with ABS = 0.1157 (i.e., and 86.6 km beam size) + frequency reuse factor = 2 |
| Total number of beam footprints | 1058 | 529 | 353 |
| (N1, N2+N3) - N1 beam footprints are in state “off”. - N2 beam footprints are in state “common messages only”. - N3 beam footprints are in state “active traffic” | (210, 848) | (105, 424) | (35, 318) |
| Fraction of area not in service | 20% | 20% | 10% |
| Dwell time | 2.5 ms to 5 ms | 5 ms to 10 ms | 6.67 ms to 12 ms |
| Revisit time | 20 ms | 20 ms | 20 ms |
| SSB periodicity | 20 ms | 20 ms | 20 ms |

[Fraunhofer IIS, Fraunhofer HHI]:

- A maximum dwell time of 16ms could be achieved for set1-1 and set1-3 if an SSB periodicity of 160ms was considered.

[ZTE]:

- For Set 1-1 and Set 1-2, the minimum dwell time is 4ms due to repetition of common channel if link level enhancement is performed based on required SNR of SSB detection.

- For Set 1-3, the minimum dwell time is 13ms due to repetition of common channels.

- To support MSG1 detection, a minimum dwell time of 13.2ms is needed for a give beam.

- If a complete initial access procedure is involved, a minimum dwell time of 31.5ms is needed for a give beam.

[Apple]:

- In LEO-600 set 1-1 and set 1-3 in FR1 with N1=N2=0 and N3=1058, the coverage ratio is 100%, with dwell time = 2 ms, revisit time = 20 ms and the periodicity of SSB and SIB1 = 20 ms.

- In LEO-600 set 1-1 and set 1-3 in FR1 with N1=0, N2=846 and N3=212, the coverage ratio is 100%, with dwell time = 6 ms (on N3 beam footprints), revisit time = 20 ms and periodicity of SSB and SIB1 = 20 ms.

- In LEO-600 set 1-1 and set 1-3 in FR1 with N1=0, N2=846 and N3=212, the coverage ratio is 100%, with dwell time = 6 ms (on N3 beam footprints), revisit time = 20 ms and periodicity of SSB and SIB1 = 20 ms.

- In LEO-600 set 1-2 in FR1 with N1=N2=0 and N3=1058, the coverage ratio is 100%, with dwell time = 2.4 ms, revisit time = 160 ms and periodicity of SSB and SIB1 = 160 ms.

- In LEO-600 set 1-2 in FR1 with N1=0, N2=846 and N3=212, the coverage ratio is 100%, with dwell time= 8.4 ms (on N3 beam footprints), revisit time =160 ms and periodicity of SSB and SIB1 = 160 ms.

- In LEO-600 set 1-2 in FR1 with N1=N3=0 and N2= 320, the coverage ratio is 30%, with dwell time = 1 ms (on N2 beam footprints), revisit time = 20 ms and periodicity of SSB and SIB1 = 20 ms.

### Resource utilization and common signals/messages overhead

[vivo]:

- For Set1-1 FR1,  the common signals/messages overhead can be significantly reduced by using a proper and flexible beam hopping design taking into account, e.g., combining wider beam for SSB and narrower beam for others, compared to a fixed beam configuration.

Table 1. The overhead of common signalling/messages for Set1-1 FR1

| SIB1 periodicity |  | 20ms | 40ms | 80ms | 160ms |
| --- | --- | --- | --- | --- | --- |
| Overhead | Set1-1 with fixed 1058 beam footprints | 35% | 23% | 17% | 14% |
|  | Set1-1 with flexible beam footprints (wider beam for SSB) | 20% | 12% | 8% | 6% |

- For Set1-2 FR1,  the common signals/messages overhead can be significantly reduced by using a proper and flexible beam hopping design taking into account, e.g., combining wider beam for SSB and narrower beam for others, compared to a fixed beam configuration.

Table 2. The overhead of common signalling/messages for Set1-2 FR1

| SIB1 periodicity |  | 20ms | 40ms | 80ms | 160ms |
| --- | --- | --- | --- | --- | --- |
| Overhead | Set1-2 with fixed 1058 beam footprints | N/A | N/A | N/A | 95% |
|  | Set1-2 with flexible beam footprints (wider beam for SSB) | N/A | 78% | 53% | 40% |

- For Set1-3 FR1,  the common signals/messages overhead can be significantly reduced by using a proper and flexible beam hopping design taking into account, e.g., combining wider beam for SSB and narrower beam for others, compared to a fixed beam configuration.

Table 3. The overhead of common signalling/messages for Set1-3 FR1

| SIB1 periodicity |  | 20ms | 40ms | 80ms | 160ms |
| --- | --- | --- | --- | --- | --- |
| Overhead | Set1-3 with fixed 1058 beam footprints | N/A | 59% | 35% | 24% |
|  | Set1-3 with flexible beam footprints (wider beam for SSB) | 94% | 52% | 30% | 20% |

[Thales]:

- Set 1-1 and Set 1-3 FR1 with % simultaneously active beams of 10% increasing the SSB periodicity from up to 160ms allows an increase of the cell available capacity for user traffic and the cell throughput by 30%.

- Evaluation results for Set1-1 FR1 Set1-3 FR1: the following table shows the resource utilization for different SSB periodicities. Further, the average throughput per beam/cell is determined based on the available resource (for user traffic) per cell

Table 4 Resource utilization for Set1-1 FR1 and Set1-3 FR1

| Number of slot within SSB periodicity | PSSB | 20 | 40 | 80 | 160 |
| --- | --- | --- | --- | --- | --- |
| Ratio of Common message Resource utilization per cluster of cells | RUsigcluster | 0,40 | 0,20 | 0,10 | 0,05 |
| Ratio of Available resource (for user traffic) per cluster of cells | RUtrafficcluster | 0,60 | 0,80 | 0,90 | 0,95 |
| Available resource (for user traffic) per cluster of cells (PRB) | ARcluster | 14,88 | 19,94 | 22,47 | 23,73 |
| Available resource (for user traffic) per cell (PRB) | ARcell | 1,49 | 2,00 | 2,25 | 2,38 |
| Average Beam throughput (for user traffic) (kbps) |  | 50,47 | 67,64 | 76,23 | 80,52 |

- Evaluation results for Set1-2 FR1: The following table shows the resource utilization for Set1-2 FR1.

Table 5 Resource utilization for Set1-2 FR1

| Number of slot within SSB periodicity | PSSB | 20 | 40 | 80 | 160 |
| --- | --- | --- | --- | --- | --- |
| Ratio of Common message Resource utilization per cluster of cells | RUsigcluster | N/A | N/A | 0,67 | 0,34 |
| Ratio of Available resource (for user traffic) per cluster of cells | RUtrafficcluster | N/A | N/A | 0,33 | 0,66 |
| Available resource (for user traffic) per cluster of cells (PRB) | ARcluster | N/A | N/A | 8,23 | 16,62 |
| Available resource (for user traffic) per cell (PRB) | ARcell | N/A | N/A | 0,12 | 0,25 |
| Average Beam throughput (for user traffic) (kbps) |  | N/A | N/A | 4,22 | 8,51 |

- Evaluation results for Set1-1 FR2: The following table shows the resource utilization for Set1-1 FR2.

Table 6 Resource utilization for Set1-1 FR2

| Number of slot within SSB periodicity | PSSB | 20 | 40 | 80 | 160 |
| --- | --- | --- | --- | --- | --- |
| Ratio of Common message Resource utilization per cluster of cells | RUsigcluster | 0,05 | 0,03 | 0,01 | 0,01 |
| Ratio of Available resource (for user traffic) per cluster of cells | RUtrafficcluster | 0,95 | 0,97 | 0,99 | 0,99 |
| Available resource (for user traffic) per cluster of cells (PRB) | ARcluster | 249 | 256 | 260 | 262 |
| Available resource (for user traffic) per cell (PRB) | ARcell | 3,75 | 3,85 | 3,91 | 3,93 |
| Average Beam throughput (for user traffic) (Mbps) |  | 22,49 | 23,13 | 23,45 | 23,61 |

The following figures illustrate the simulations results for parameter scenarios sets 1-1 FR1, set 1-2 FR1 and set 1-3 FR1, with different SSB periodicities using the two beam hopping algorithms:

Figure 1 Common control traffic/signalling overhead:

### Coverage ratio

[Huawei]:

- The following tables show the evaluated cases/solutions, and corresponding coverage ratio metric for the LEO600km Set 1-2 and Set 1-1, respectively , [R1-2403938]:

| # Case list | Case1 | Case2 | Case3 | Case4 |
| --- | --- | --- | --- | --- |
| # Note | Beam hopping of SSBs within the same SS burst | Beam hopping of common control signal/channel with increased SSB periodicity |  |  |
| # Periodicity of common control channels (ms) for the N2 beam footprints | 20 | 160 | 320 | 640 |
| # Periodicity of common control channels (ms) for the N3 beam footprints | 20 | 160 | 320 | 640 |
| # The number of served beam footprints (i.e. N2+N3) | 64 | 512 | 1024 | 1058 |
| # The number of unserved beams, i.e. N1 | 994 | 546 | 34 | 0 |
| # Dwell time of common control channels for every 4*16 beam footprints | 10ms | 10ms | 10ms | 10ms |
| # Common control channel overhead | 50% | 50% | 50% | 25.8% |
| # Common control channel coverage ratio # (i.e. (N2+N3)/(N1+N2+N3)) | 6.049% | 48.393% | 96.786% | 100% |

| # Case list | Case1 | Case2 | Case3 | Case4 |
| --- | --- | --- | --- | --- |
| # Note | Beam hopping of SSBs within the same SS burst | Beam hopping of common control signal/channel with increased SSB periodicity |  |  |
| # Periodicity of common control channels (ms) for the N2 beam footprints | 20 | 40 | 80 | 320 |
| # Periodicity of common control channels (ms) for the N3 beam footprints | 20 | 40 | 80 | 320 |
| # The number of served beam footprints (i.e. N2+N3) | 424 | 848 | 1058 | 1058 |
| # The number of unserved beams, i.e. N1 | 634 | 210 | 0 | 0 |
| # Common control channel overhead | 50% | 50% | 31.2% | 15.6% |
| # Common control channel coverage ratio # (i.e. (N2+N3)/(N1+N2+N3)) | 40.08% | 80.16% | 100% | 100% |

[CATT]:

- In realistic deployment of LEO 600km at FR1, if one satellite can provide 16 beams activation, the coverage ratio can be optimized from 6% to 96.9%.

- In realistic deployment of LEO 600km at FR1, if one satellite can provide 106 beams activation, the coverage ratio can be optimized from 40% to 80.2%.

- SSB periodicity with 320ms can provide better performance in coverage ratio when only 16 beams are activated.

Table 7 Parameter configuration and assumption for system level evaluation

|  | Case1 (baseline) | Case2 (beam hopping) | Case3 (baseline) | Case4 ((beam hopping)) | Case5 (baseline) | Case6 ((beam hopping)) | Case7 (baseline) | Case8 ((beam hopping)) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Active beams | 16 | 16 | 16 | 16 | 106 | 106 | 106 | 106 |
| Period of SSB | 20ms | 320ms | 20ms | 320ms | 20ms | 40ms | 20ms | 40ms |
| Period of SIB | 320ms | 320ms | 320ms | 320ms | 160ms | 160ms | 160ms | 160ms |
| Traffic type | VOIP | VOIP | IM | IM | VOIP | VOIP | IM | IM |
| Users per users(X) | 10 | 10 | 1 | 1 | 10 | 10 | 1 | 1 |
| Beam hopping | off | on | off | on | off | on | off | on |

The evaluation results are shown in the following table:

Table 8 Parameter configuration for link budget analysis

|  | Case1 |  | Case2 | Case3 | Case4 | Case5 | Case6 | Case7 | Case8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N1 | 993 |  | 33 | 993 | 33 | 633 | 209 | 633 | 209 |
| N2 | 0 |  | 864 | 0 | 960 | 0 | 0 | 0 | 530 |
| N3 | 64 |  | 160 | 64 | 64 | 424 | 848 | 424 | 318 |
| Coverage ratio （N2+N3/ total beam footprints） | 6.05% |  | 96.88% | 6.05% | 96.88% | 40.11% | 80.23% | 40.11% | 80.23% |
| Resource utilization | 28.5% |  | 90% | 60.1% | 97.60% | 31% | 82% | 68.8% | 90.35% |

[CMCC]:

- For LEO-600 Set1-1/1-2/1-3, even if all the active beams are used for common channels, it cannot satisfy 100% coverage ratio with the default common channel periodicity.

- It is observed that the coverage ratio can be improved with longer duration of common channel periodicity from UE side.

- For LEO-600 Set1-1/1-2/1-3, longer revisit time can be considered to improve the coverage ratio.

Table 9 System level evaluation for LEO-600

| Cases | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 | Case 6 | Case 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite parameter set | Set1-1/1-3 | Set1-1/1-3 | Set1-1/1-3 | Set 1-2 | Set 1-2 | Set 1-2 | Set 1-2 |
| Total number of beam footprints | 1058 | 1058 | 1058 | 1058 | 1058 | 1058 | 1058 |
| Total number of simultaneously active beams | 106 | 106 | 106 | 16 | 16 | 16 | 16 |
| Periodicity of common control channels | 20ms | 40ms | 80ms | 20ms | 80ms | 160ms | 320ms |
| Number of beam footprints being served | 424 | 828 | 1058 | 64 | 256 | 512 | 1024 |
| Coverage ratio (number of beam footprints being served/total number of beam footprints) | 40.08% | 80.16% | 100% | 6.05% | 24.2% | 48.4% | 96.8% |

[Baicells]:

- Limited to the legacy specification, the maximum coverage ratio for SSB in a cell is 40%, 6%, and 40% for FR1 Set1-1, Set1-2, and Set1-3 respectively.

[OPPO]:

- For LEO600 set 1-1/set 1-3, the coverage ratio of the beam footprints in state “common message only” and state “active traffic” can achieve 100% when 80%-90% beam footprints in state “off” is assumed.

- The coverage ratio larger than 100% implies that satellite resource exceeds demand.

- For LEO600 set 1-2, most of scenarios can achieve 100% DL coverage except the high cell load scenario.

- Extending the revisit time or increasing satellite beam number per cell can be considered to improve the DL coverage at system level.

- Extending the revisit time faces backward compatibility issue.

- When the revisit time of the N3 beam footprints in state “active traffic” is 160ms, the coverage ratio can achieve 100% for all scenarios.

[MediaTek]:

- For narrow beams with NSSB=4 Beam hopping:

- Set 1-2: SSB periodicity of 320 ms can provide coverage of 96.8% and efficiency per beam of 96.9%.

- Set 1-1/1-3: SSB periodicity of 80 ms can provide coverage of 96.6% and efficiency per beam of 87.5%.

- For wide beam covering 4 narrow beams with NSSB=4 Beam hopping:

- Set 1-2, SSB periodicity of 80 ms can provide coverage of 96.8%  and efficiency per beam of 96.9%.

- Set 1-1/1-3, SSB periodicity of 80 ms can provide coverage of 100%  and efficiency per beam of 87.5%.

Table 10 System-level analysis for set 1-2, 1058 narrow beams

| SSB Periodicity X = [20, 40, 80, 160, 320, 640] ms |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario | Nbeam,total | Nbeam,active | Coverage ratio | Nbeam,served | Revisit time for common channels [ms] | Efficiency per beam |
| Set 1-2 | 1058 | 16 | 6.0%, 12.1%, 24.2%, 48.4%, 96.8%, 100% | 64, 128, 256, 512, 1024, > 1058 | 20, 40, 80, 160, 320, 640 | 50.0%, 75.0%, 87.5%, 93.8%, 96.9%, 98.4% |
| Set 1-1 Set 1-3 | 1058 | 106 | 40.1%, 80.2%, 100%, 100%, 100% | 424, > 848, > 1058, > 1058, 1058, > 1058, | 20, 40, 80, 160, 320, 640 | 50.0%, 75.0%, 87.5%, 93.8%, 96.9%, 98.4% |

### CDF of the received SINR

[Samsung]:

[R1-2404132] For SLS, the following simulation methodologies are considered for each scenario based on newly agreed assumptions in RAN1#116bis.

- 10 UEs are dropped randomly in a given number of beams (i.e. active beams) that are selected randomly or coordinately out of 1057 possible beams.

- Satellite uses a 20x20 phased array antenna and directs the signal towards the center of the selected beams. (Antenna pattern is shown in Appendix D)

- Total available power of the satellite is divided among the active beams.

- The signal power received by UE and also the interference powers from all other beams are calculated.

- SINR geometry CDFs are generated for multiple drops.

Table 1 CDFs of SIR and SINR for randomly activated beam

|  | SIR geometry | SINR geometry |
| --- | --- | --- |
| Set 1-1 |  |  |
| Set 1-2 |  |  |
| Set 1-3 |  |  |

Table 3 CDFs of SIR and SINR for coordinated activated beam

|  | SIR geometry | SINR geometry |
| --- | --- | --- |
| Set 1-1 |  |  |
| Set 1-2 |  |  |
| Set 1-3 |  |  |

- SSB performance requirement could not be met for all sets based on 5% percentile SINR result in case of randomly activated beam.

- SSB performance requirement could not be met for all sets based on 5% percentile SINR result in case of randomly activated beam.

[Ericsson]:

- For FRF = 1, larger ABS with the same HPBW reduces the aggregate interference and hence improves the downlink SINR in majority UE locations (excluding the cell-edge users), while marginally decreasing the SINR for the cell-edge users.

- For FRF = 3 and BH with TRF = 3, larger ABS with the same HPBW reduces interference and hence improves the downlink SINR in a limited number of UE locations (i.e., around the cell center), while reducing the SINR for majority UE

- Larger ABS allows to reduce the required time reuse factor which in turn yields increased beam illumination ON-time for a given Trevisit in the context of beam hopping.

Figure 2. CDFs of the downlink geometry SIR for FRF = 1, FRF = 3 and BH with TRF = 3. Beam layouts with BS = 0.0668 , ABS'=0.0945 and with ABS'=0.1157 are considered.

Figure 3. CDFs of the downlink geometry SINR for FRF = 1, FRF = 3 and BH with TRF = 3. Beam layouts with ABS = 0.0668, ABS'=0.0945 and with ABS'=0.1157 are considered.

[Thales]:

The following figures illustrate the simulations results for parameter scenarios sets 1-1 FR1, set 1-2 FR1 and set 1-3 FR1, with different SSB periodicities using the two beam hopping algorithms:

Figure 1  DL SINR

[CATT]:

The evaluated SINR curves are:

Figure 3 CDF of SINR for FR1 set1-1 and set1-2

- Under the random beam scheduling method, the interference is more severe when the number of physical beams is 106 compared to that of 16 phyisical beams.

### CDF of the cell throughput and UPT

[Huawei]:

The UPT and cell throughput performance with 3 UEs with IM traffic per active beam, 1 UE with FTP traffic per active beam and 20 UEs with VoIP traffic per active beam are shown in the following Figures:

Figure 2 UPT Performance @Set 1-2

Figure 3 cell throughput Performance @Set 1-2

[vivo]:

- For Set1-1 FR1 with FTP traffic,

- The average SE performance of the configurations with fixed beam footprints is comparable to the results of IMT-2020 self-evaluation for NTN.

- The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can significantly achieve better performance than the configurations with fixed beam footprints, e.g., 500% performance improvement.

- For Set1-1 FR1 with VoIP traffic,

- The performance of the configurations with fixed beam footprints cannot achieve the VoIP transmission rate in TN.

- The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can achieve better performance, which is close to the VoIP transmission rate in TN.

Table 6. The comparison of the average throughput of UE, considering SIB1 periodicity 20ms

| VoIP traffic (AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) and 20 ms data arriving interval) | Set1-1 with fixed 1058 beam footprints | Set1-1 with flexible beam footprints (wider beam for SSB) | The VoIP transmission rate in TN [5]. |
| --- | --- | --- | --- |
| Average throughput performance (Kbps) of UE | 31.2 | 62.7 | 66 |

- For Set1-2 FR1 with FTP traffic,

- The average SE performance of the configurations with fixed beam footprints is worse than the results of IMT-2020 self-evaluation for NTN.

- The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can significantly achieve better performance than the configurations with fixed beam footprints, e.g., 276% performance improvement.

- For Set1-2 FR1 with VoIP traffic,

- The performance of the configurations with fixed beam footprints cannot achieve the VoIP transmission rate in TN.

- The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can achieve better performance, 37% performance improvement than the configuration with fixed beam footprints.

Table 7. The comparison of the average throughput of UE, considering SIB1 periodicity 160ms

| VoIP traffic (AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) and 20 ms data arriving interval) | Set1-2 with fixed 1058 beam footprints | Set1-2 with flexible beam footprints (wider beam for SSB) | The VoIP transmission rate in TN [5]. |
| --- | --- | --- | --- |
| Average throughput performance (Kbps) of UE | 42 | 57.5 | 66 |

- For Set1-3 FR1 with FTP traffic,

- The average SE performance of the configurations with fixed beam footprints is worse than the results of IMT-2020 self-evaluation for NTN.

- The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can significantly achieve better performance than the configurations with fixed beam footprints, e.g., 150% performance improvement.

- For Set1-3 FR1 with VoIP traffic,

- The performance of the configurations with fixed beam footprints cannot achieve the VoIP transmission rate in TN.

- The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can achieve better performance, 74% performance improvement than the configuration with fixed beam footprints.

Table 8. The comparison of the average throughput of UE, considering SIB1 periodicity 160ms

| VoIP traffic (AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) and 20 ms data arriving interval) | Set1-3 with fixed 1058 beam footprints | Set1-3 with flexible beam footprints (wider beam for SSB) | The VoIP transmission rate in TN [5]. |
| --- | --- | --- | --- |
| Average throughput performance (Kbps) of UE | 35 | 61 | 66 |

[Thales]:

- System level simulations results and analytical evaluation results lead to the same conclusion on the necessity to extend the default SSB periodicity to support satellite beam hopping in NTN.

- DL Average throughput per cell:

Figure 5 DL Average throughput per cell

- DL Average throughput per UE:

Figure 6 DL Average throughput per UE

### SSB periodicity evaluation

[Huawei]:

- Beam hopping transmission of common control signal/channel within increased SSB periodicity increases the common control channel coverage ratio to improve system level coverage for all the three LEO600km satellite parameter sets.

- Beam hopping transmission of common control signal/channel within increased SSB periodicity can also reduce the system common control channel overhead for all the three LEO600 parameter sets.

- Due to the reduced common control channel overhead by extending the SSB and common control channel periodicity:

    - The UPT for traffic type of VoIP, IM and FTP3 can be improved,

    - The cell throughput for IM and FTP is improved.

[Ericsson]:

- It is possible to keep using the legacy 20ms SSB periodicity for performing a Beam Hopping based scheduling, while guaranteeing the transmission of physical channels and signals required to be transmitted during initial access (SSB, SIB1, SIB19, TRS, CSI-RS) and at least msg1 and msg2 of the RACH procedure.

- Using an SSB supporting longer than 20ms for perfoming a Beam Hopping based scheduling will result in a specification impact (i.e., according to TS 38.213 clause 4.1, during initial access UEs assume an SSB periodicity of 20ms). A modification on this procedure is foreseen to impact legacy UEs which may not be able to access an NTN Beam Hopping cell supporting an SSB periodicity longer than 20ms during initial access.

[Fraunhofer IIS]:

- Set 1-2 is not enough to ensure receiving the necessary common control signaling (e.g., SSB, Type0/0A-PDCCH, SIB1, and SIB19) even by considering an SSB periodicity of 160ms

- The SSB periodicity of 20ms could be achieved for set1-1 and set1-3 by sweeping each active satellite beam in an SSB TDM-mulitplexed manner, where each SSB burst consists of 4 SSBs and the beam dwell time is 5ms required to transmit the SSB burst.

- The SSB periodicity of 20ms could not be achieved for set1-2 even by sweeping each active satellite beam in an SSB TDM-mulitplexed manner, where an SSB periodicity of at least 80ms might be required.

[Spreadtrum]:

- If the total number of simultaneously active beams is 16, extending the default SSB periodicity is needed to achieve 100% coverage ratio.

- If the total number of simultaneously active beams is 106, extending the default SSB periodicity may be not needed to achieve 100% coverage ratio.

[THALES]:

- Set1-2 FR1 with % simultaneously active beams of 1,5% requires SSB periodicity of at least 80ms. That is, a default periodicity of 20ms is not doable for such deployment scenario.

- Set 1-1 and Set 1-3 FR1 with % simultaneously active beams of 10% increasing the SSB periodicity from up to 160ms allows an increase of the cell available capacity for user traffic and the cell throughput by 30%.

- Set1-2 FR1 can work with a default SSB periodicity of 20ms if and only if the % simultaneously active beams is at least equal to 4.1% (only 394 beam are illuminated).

- System level simulations results and analytical evaluation results lead to the same conclusion on the necessity to extend the default SSB periodicity to support satellite beam hopping in NTN.

[ZTE]:

- For Set 1-1, a minimum SSB periodicity of 640ms is needed to support necessary DL common channels transmission and random-access procedure.

- For Set 1-2, a minimum SSB periodicity of 640ms is needed to support necessary DL common channels transmission.

- For Set 1-3, a minimum SSB periodicity of 320ms is needed to support necessary DL common channels if link level enhancement is introduced for coverage.

Analysis of periodicity and coverage ratio of DL common channels transmission

| Scenario | Note | Nbeam,total | Nbeam,active | Nbeam served in 20ms | Coverage ratio | Revisit time(ms) | Minimum Periodicity (ms) for common channel |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Set 1-1 | Common channel | 1058 | 106 | 424 | 40.08% (=424/1058) | 50 (=1058/424*20) | 80 (>50) |
|  | Common channel and RACH |  |  | 53 | 5.01% (=53/1058) | 400 (=1058/53*20) | 640 (>400) |
| Set 1-2 | Common channel | 1058 | 16 | 64 | 6.05% (=64/1058) | 331 (=1058/64*20) | 640 (>331) |
| Set 1-3 | Common channel with repetition | 1058 | 106 | 106 | 10.02% (=106/1058) | 200 (=1058/106*20) | 320(>200) |
|  | Common channel without repetition |  |  | 424 | 40.08% (=424/1058) | 50 (=1058/424*20) | 80 (>50) |

[Apple]:

- In LEO-600 set 1-1 and set 1-3 in FR1 with N1=N2=0 and N3=1058, the coverage ratio is 100%, with dwell time = 2 ms, revisit time = 20 ms and the periodicity of SSB and SIB1 = 20 ms.

- In LEO-600 set 1-1 and set 1-3 in FR1 with N1=0, N2=846 and N3=212, the coverage ratio is 100%, with dwell time = 6 ms (on N3 beam footprints), revisit time = 20 ms and periodicity of SSB and SIB1 = 20 ms.

- In LEO-600 set 1-1 and set 1-3 in FR1 with N1=0, N2=846 and N3=212, the coverage ratio is 100%, with dwell time = 6 ms (on N3 beam footprints), revisit time = 20 ms and periodicity of SSB and SIB1 = 20 ms.

- In LEO-600 set 1-2 in FR1 with N1=N2=0 and N3=1058, the coverage ratio is 100%, with dwell time = 2.4 ms, revisit time = 160 ms and periodicity of SSB and SIB1 = 160 ms.

- In LEO-600 set 1-2 in FR1 with N1=0, N2=846 and N3=212, the coverage ratio is 100%, with dwell time= 8.4 ms (on N3 beam footprints), revisit time =160 ms and periodicity of SSB and SIB1 = 160 ms.

- In LEO-600 set 1-2 in FR1 with N1=N3=0 and N2= 320, the coverage ratio is 30%, with dwell time = 1 ms (on N2 beam footprints), revisit time = 20 ms and periodicity of SSB and SIB1 = 20 ms.

[LG]:

- The required simultaneously active beam ratio only for SSB with 15kHz SCS is at least 1.43% when the number of SSBs within 20msec periodicity is 1. For other DL transmission including common channels, the active beam ratio of 1.5% in Set 1-2 is not feasible.

- The required simultaneously active beam ratio only for SSB with 30kHz SCS is at least 0.71% when the number of SSBs within 20msec periodicity is 1. For other DL transmission including common channels, the active beam ratio of 1.5% in Set 1-2 could be feasible. However, it would be necessary to increase BW to be more than 5MHz since the BW of the SSB with 30kHz SCS occupies 7.2MHz.

[CMCC]:

- For LEO-600 Set1-1/1-2/1-3, even if all the active beams are used for common channels, it cannot satisfy 100% coverage ratio with the default common channel periodicity.

- It is observed that the coverage ratio can be improved with longer duration of common channel periodicity from UE side.

[ETRI]:

- The additional reference satellite parameters scenarios for LEO600km Set1-1 and Set1-3 in FR1 can satisfy the SSB periodicity of 20 ms.

- The additional reference satellite parameters scenarios for LEO600km Set1-2 in FR1 cannot satisfy the SSB periodicity of 20 ms.

- Increasing the time period of common signals can accommodate the 1058 cells with the additional reference satellite parameters for LEO600km Set1 in FR1, as well as reduce overhead.

[MediaTek]:

- For narrow beams with NSSB=4 Beam hopping:

- Set 1-2: SSB periodicity of 320 ms can provide coverage of 96.8% and efficiency per beam of 96.9%.

- Set 1-1/1-3: SSB periodicity of 80 ms can provide coverage of 96.6% and efficiency per beam of 87.5%.

- For wide beam covering 4 narrow beams with NSSB=4 Beam hopping:

- Set 1-2, SSB periodicity of 80 ms can provide coverage of 96.8%  and efficiency per beam of 96.9%.

- Set 1-1/1-3, SSB periodicity of 80 ms can provide coverage of 100%  and efficiency per beam of 87.5%.

## Initial proposals on SLS results

### Proposal 1-1

Proposal 1-1-v0: Adopt the SLS results in section 1.2 as part of the outcomes of the study on DL coverage enhancements at system level in NTN

Companies are encouraged to share views on Proposal 1-1-v0

| Companies | Comments |
| --- | --- |
| Qualcomm | It’s unclear what results are to be adopted. |
| MediaTek | We are generally fine with the intention, but some summary would be preferable. |
| DCM | We are not sure what is intention of this proposal. We do not see any agreement in terms of SLS. |
| Huawei, HiSilison | The proposal seems a little bit general. We propose to consider the following observations based on the evaluation results in section 1.2.3 and other inputs from companies: Proposed observation 1: the coverage ratio of the served beam footprints, where at least SSBs and common channels (SIB1 and SIB19) are provided for UE’s access, needs to be improved for NTN system level coverage enhancement. Proposed observation 2: Beam hopping transmission of common control signal/channel within increased SSB periodicity increases the coverage ratio to improve system level coverage for all the three LEO600km satellite parameter sets: - For set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms; - For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms; Proposed observation 3: Beam hopping transmission of common control signal/channel within increased SSB periodicity can reduce the system common control channel overhead for all the three LEO600 parameter sets. Proposed observation 4: The unequal distribution of user density among different beam footprints should be considered in the system level coverage enhancement. Proposed observation 5: 20ms periodicity SSB can be transmitted for in the beam footprints which are supposed to be illuminated/served in Rel-17/18 baseline, while beam hopping transmission of common control signal/channel with increased SSB periodicity can be used to extend the coverage to other beam footprints in the total 1058 beam footprints. |
| TCL | We are generally fine with the intention, but it’s unclear for us which results can be used. |
| Lenovo | Not sure the details. |
| Ericsson | We are also not sure with the intention of the proposal. It is difficult to glean the insights or key take-aways from this proposal in its current form. |
| CATT | We support the intention of FL, for detailed wording, we think Huawei’ proposal is quite constructive. Since a lot of companies provided the results, we need to identify the benefits and key insights if introduce the specific system level enhancement. Based on the provided results so far, the benefits include the coverage ratio, throughput gain, signalling overhead reduction and resource utilization efficiency. |
| Nokia | We are supportive of the intention, but would be nice to have concrete summary to relate to. |
| ZTE | Generally fine to list observations separately for different aspects. For minimum dwell time of set 1-1 and 1-2, the 4 ms is for the case where link level enhancement is performed based on the required SNR of SSB detection. If the CNR of set 1-1 and 1-2 is adopted as reference, no repetition of common channel is needed. Hence, ZTE’s view in 1.2.1 is updated. |
| Apple | It is unclear for us how the results can be used. |

# Topic#2 Potential enhancements at system level

### Companies’ contributions summary

The following proposals on Support of larger SSB periodicity are submitted to current RAN1 meeting:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 4: Beam hopping transmission of common control signal/channel within increased SSB periodicity is supported to increase the system level coverage ratio in NR NTN. Proposal 5: For NR NTN UE, the default SSB periodicity assumed by UE during initial access is enlarged to support higher system level coverage ratio in NR NTN. Proposal 6: At least SSB periodicity of 320ms are introduced in SIB1 for NR NTN UE to enable the wider system coverage in Rel-19 NR NTN. Proposal 7: Shorter SSB periodicity, e.g. 20ms, and extended SSB periodicities are used for beam footprints with high user density and very low user density respectively. |
| Ericsson | Proposal 6 At least the legacy SSB periodicity of 20ms during initial access is supported for Beam Hopping. Proposal 7 If SSB periodicities longer than 20ms during initial access are intended to be supported, then the following aspects shall be studied: - Specification impacts - Backward compatibility with legacy UEs - Sync-raster - MIB periodicity - PBCH combining - Performance Loss/Gain. |
| Eutelsat | Proposal 4: The impact of extending the SS burst periodicity beyond the present assumptions should be considered. Proposal 5: Finally, we would encourage RAN1 to consider if techniques being proposed to enhance FR1-NTN downlink coverage can be applied to FR2-NTN, either directly or with the essential adaptations that could be required. |
| Fraunhofer IIS, Fraunhofer HHI | Proposal 2: RAN1 to study the impact of increasing the periodicity of SSB during initial access on legacy UEs. |
| Spreadtrum Communications | Proposal 3: Cell DTX/DRX mechanism in RRC idle/inactive mode also should be considered. Proposal 4 Cell DTX/DRX mechanism in RRC idle/inactive mode needs to consider the specification impacts on the following UE activities. - RRM measurement - System information updates - Paging message reception - RACH procedure - SDT Proposal 5: The following two schemes for NES can be considered for achieving dynamic and flexible power sharing between satellite beams in RRC CONNECTED mode. - Cell DTX - Adaptation of transmission power of signals and channels |
| Samsung | Proposal 3: RAN1 does not consider study for potential SSB enhancement in Rel-19. |
| vivo | Proposal 1: For LEO600km Set1-1/2/3 FR1, applying proper flexible beam size and hopping design can significantly improve the system level performance to an acceptable level. Proposal 2: For LEO600km Set1-1/2/3 FR1, the change of SSB design including periodicity is not necessary for the DL coverage enhancement. |
| Thales | Proposal 3: The following potential enhancements are considered to enable an optimized Satellite beam illumination plan/beam hopping: - Extending the default SSB periodicity - Release 18 network energy saving techniques (e.g., cell DTX/DRX, etc.) with modifications to fit NR NTN (e.g., beam-based operations): - Support the configuration of more than one cell DTX patterns to a UE. - Support dynamic group-common signaling for the change of cell DTX patterns. Proposal 4: RAN1 to study coverage enhancements techniques allowing for a reduced dwell time of a satellite beam (time duration during which the beam is active) such as techniques with a reduced number of repetitions. |
| ZTE | Proposal 1: A minimum SSB periodicity of 640ms can be considered. |
| Apple | Proposal 5: RAN1 considers the system level downlink coverage enhancement, using Release 18 network energy saving techniques (e.g., cell DTX/DRX, etc.) with modifications to fit NR NTN (e.g., beam-based operations). |
| CATT | - The beam hopping mechanism with SSB periodicity extension can effectively improve satellite coverage and should be considered in system level enhancement. |
| Lenovo | Proposal 1: Default SSB periodicity is kept to be 20ms for NR NTN. |
| CMCC | Proposal 5. To improve the coverage ratio of active beams, long revisit time/period and dwelling time should be introduced. Proposal 6. Under the assumption of long revisit time, the SSB periodicity should be extended. - For Set 1-1/1-3, the default SSB detection period should be extended. - For Set 1-2, the SSB periodicity should also be extended. |
| Xiaomi | Proposal 1:The following solutions can be studied to solve the SSB sweeping problem due to the limited simultaneously active beam ratio. - Solution 1: Increase the SSB number and change the SSB pattern for S band - Solution 2: Associated the SSB index with the specific area/cell by NW implementation - Solution 3: Change the UE’s assumption on default SSB periodicity for initial cell selection |
| ETRI | Proposal 2: RAN1 to study whether the longer time period for common signals is necessary. |
| NTT DOCOMO, INC. | Proposal 3: - If longer SSB periodicity is within the scope, discuss what is the specification/UE impact beyond the existing specification. |
| MediaTek Inc. | Proposal 2: RAN1 to study potential impact of beams ON/OFF on procedures for DL synchronization, acquisition of system information for configuration of beams, and initial cell access via RACH procedure. Proposal 3: RAN1 to study potential impact of beams ON/OFF on paging procedures. |
| CEWiT | Proposal 12: System level enhancements for NTN DL coverage enhancement, at least for N2 beams consider following options: Option 1: Increase the SSB periodicity more than 20 ms. Possible values are 80ms, 160ms. - Backward compatibility issues should be considered. Option 2: Increasing the number of beams in an SSB burst. Option 3: Increasing the beam width to cover the desired coverage area with possible SSB beams - Necessary link gain loss should be compensated by link level enahcements. Option 4: Support Cell DRX/DTX from Rel-18 NES to turn ON/OFF the beams as a system level enhancement to improve the DL coverage for Rel-19 NR NTN. - FFS: any NTN specific modification necessary. |
| CSCN | Proposal 2: The extended periodicity of common signals/channels such as SSB/SIBs could be considered for downlink coverage enhancement at a system level. |
| Nokia, Nokia Shanghai Bell | Proposal 4: It is RAN1s understanding that legacy UEs will not be compatible with any new DL coverage enhancement signalling procedures. |

The following proposals on support of wider beam for SSB are submitted to current RAN1 meeting

| Company | Proposals |
| --- | --- |
| Ericsson | Proposal 1 RAN1 to consider beam layout with at least one increased value of ABS (e.g., 0.1157, equivalently, beam size of 86.6 km) in addition to the baseline ABS of 0.0668 (equivalently beam size of 50 km) with the same HPBW. Proposal 2 RAN1 to discuss the baseline techniques such as FRF = 1 and FRF = 3 while comparing the performance of beam hopping technique in the downlink coverage enhancements and the specification impact point of view. Proposal 3 RAN1 to consider beam layout with larger ABS without increasing the HPBW in the system-level evaluations of the downlink coverage enhancements study. Proposal 4 RAN1 aim to consider the low or moderate time reuse factors (that allows reasonable Tdwell) in the context of beam hopping to limit the specification impact. Proposal 5 RAN1 to revisit the UE distribution model to reflect the relevant NTN deployment aspects, including the non-uniformity of the traffic distribution in system-level evaluation of beam hopping. |
| vivo | Observation 2: For Set1-1 FR1 with FTP traffic, - The average SE performance of the configurations with fixed beam footprints is comparable to the results of IMT-2020 self-evaluation for NTN. - The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can significantly achieve better performance than the configurations with fixed beam footprints, e.g., 500% performance improvement. Observation 3: For Set1-1 FR1 with VoIP traffic, - The performance of the configurations with fixed beam footprints cannot achieve the VoIP transmission rate in TN. - The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can achieve better performance, which is close to the VoIP transmission rate in TN. Observation 4: For Set1-2 FR1, the common signals/messages overhead can be significantly reduced by using a proper and flexible beam hopping design taking into account, e.g., combining wider beam for SSB and narrower beam for others, compared to a fixed beam configuration. Observation 5: For Set1-2 FR1 with FTP traffic, - The average SE performance of the configurations with fixed beam footprints is worse than the results of IMT-2020 self-evaluation for NTN. - The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can significantly achieve better performance than the configurations with fixed beam footprints, e.g., 276% performance improvement. Observation 6: For Set1-2 FR1 with VoIP traffic, - The performance of the configurations with fixed beam footprints cannot achieve the VoIP transmission rate in TN. - The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can achieve better performance, 37% performance improvement than the configuration with fixed beam footprints. Observation 7: For Set1-3 FR1, the common signals/messages overhead can be significantly reduced by using a proper and flexible beam hopping design taking into account, e.g., combining wider beam for SSB and narrower beam for others, compared to a fixed beam configuration. Observation 8: For Set1-3 FR1 with FTP traffic, - The average SE performance of the configurations with fixed beam footprints is worse than the results of IMT-2020 self-evaluation for NTN. - The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can significantly achieve better performance than the configurations with fixed beam footprints, e.g., 150% performance improvement. Observation 9: For Set1-3 FR1 with VoIP traffic, - The performance of the configurations with fixed beam footprints cannot achieve the VoIP transmission rate in TN. - The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can achieve better performance, 74% performance improvement than the configuration with fixed beam footprints. |
| ZTE | Observation 11: If a wide beam covering 4 footprints is used, the CNR values for Set 1-1/1-2 and Set 1-3 are -7.9dB and -15.9dB, respectively. Observation 12: If a wide beam covering 4 footprints is used, all necessary DL common channels including the SSB cannot be correctly received with the reduced CNR values. Observation 13: If a wide beam covering is used, the necessity to extend the periodicity of common still holds due to the extended dwell time from the enhancement of DL common channel. |
| LG Electronics | Observation 5: When the wide beam footprint is supported with multiple active component beam footprints with the reduced TX power, the satellite still needs to support the case where a large number of beam footprints are simultaneously active. This approach may not be suitable for Set 1-2. Observation 6: When the wide beam footprint is supported with a single active component beam footprint due to the RF limit, the active beam ratio for SSB transmission only could be 0.38%. However, the SSB detection would not be guaranteed in the large portion of the wide beam footprint due to the limited beam width of the antenna gain main lobe. Observation 7: When the wide beam footprint is supported with a single active component beam footprint due to the RF limit, if the phased array antenna is used at the serving satellite, the SSB detection could be guaranteed. For the normal beam footprint with 50km diameter, the amplitude and the phase of each antenna elements need to be adjusted to minimize inter-cell or inter-beam footprint interference. Proposal 1: Support wide beam footprint with a single active component beam footprint at least for Set 1-2. - Phased array antenna is a baseline. - FFS: Whether or how to support parabolic antenna. - A single wide beam footprint consists of at least 4 component beam footprints with 50km diameter. - A wide beam footprint is used to transmit at least SSB and common channel(s). - Component beam footprints are used to transmit the user DL traffic by using UE-specific scrambling ID. |
| CMCC | Proposal 8. To extend the coverage ratio, the following system level enhancements can be considered, - Spatial domain enhancements - Broader/wide beam size to serve larger area based on the link level evaluation - Time domain enhancements - Long revisit period and illumination window for each beam/footprint should be defined to improve the coverage ratio |
| NEC | Proposal 1: Support using different beam sizes to provide DL service of different PHY channels to meet the link-level DL coverage requirements, and to improve the rate of (N2+N3)/(N1+N2+N3) by flexible power-sharing at the system level. Proposal 2: Support beam-level time domain configurations to flexible sharing of the DL transmission power. |
| Sharp | Proposal 1: Link level results can be used to determine the maximum number of simultanouse transmitted beams for each channel type. Proposal 2: Beam groups should be defined based on the determined maximum number of simultaneouse beams for SSBs and PBCHs, etc. Proposal 3: An SSB index may be configured/assocaitated with a beam group/pattern, which may be transparent to NTN UEs. Proposal 4: For RACH procedure, methods of beam determination/reduction should be considered within a beam group to reduce the number of simultaneous beams for PDSCH Msg2, and Msg 4. Proposal 5: Beam group/patterns can be defined for efficient gNB scheduling. A distributed beam pattern can reduce the interference among simultaneous transmitted beams. Proposal 6: Active simultaneous transmitting beams can be limited within a beam group at any given time. DL monitoring pattern can be introduced to minimize the UE monitoring periods. |
| IIT Kharagpur, CEWIT | Proposal 2: For NTN system level enhancement for the DL coverage enhancement, at least for common channel wider beams can be used. - FFS Effect on the link margin and possibility to overcome using link level enhancements. |
| MediaTek Inc. | Observation 1: For narrow beams with NSSB=4 Beam hopping: - Set 1-2: SSB periodicity of 320 ms can provide coverage of 96.8% and efficiency per beam of 96.9%. - Set 1-1/1-3: SSB periodicity of 80 ms can provide coverage of 96.6% and efficiency per beam of 87.5%. Observation 2: For wide beam covering 4 narrow beams with NSSB=4 Beam hopping: - Set 1-2, SSB periodicity of 80 ms can provide coverage of 96.8% and efficiency per beam of 96.9%. - Set 1-1/1-3, SSB periodicity of 80 ms can provide coverage of 100% and efficiency per beam of 87.5%. Observation 3: Using wide beam covering N narrow beams allows to reduce SSB periodicity by a factor N. Observation 4: The DL synchronization latency should be at least smaller than the typical on-coverage time of a narrow beam to avoid systemic failure. Proposal 4: Wide beam should be ON to at least transmit CD-SSB and SIB1, and CORESET 0. Otherwise, there is no coverage on these wider beams, and no coverage on narrow beams within the wide beam. Proposal 5: After initial DL synchronization and system information acquisition, initial cell access can be done on the wide beam re-using legacy specifications, or on narrow beams with NCD-SSBs. |

Other potential enhancements at system level

The following proposals other potential enhancements at system level are submitted to current RAN1 meeting:

| Companies | Proposals |
| --- | --- |
| Eutelsat Group | Proposal 5: Finally, we would encourage RAN1 to consider if techniques being proposed to enhance FR1-NTN downlink coverage can be applied to FR2-NTN, either directly or with the essential adaptations that could be required. |
| Spreadtrum | Proposal 3: Cell DTX/DRX mechanism in RRC idle/inactive mode also should be considered. Proposal 4 Cell DTX/DRX mechanism in RRC idle/inactive mode needs to consider the specification impacts on the following UE activities. - RRM measurement - System information updates - Paging message reception - RACH procedure - SDT Proposal 5: The following two schemes for NES can be considered for achieving dynamic and flexible power sharing between satellite beams in RRC CONNECTED mode. - Cell DTX - Adaptation of transmission power of signals and channels |
| Thales | Proposal 3: The following potential enhancements are considered to enable an optimized Satellite beam illumination plan/beam hopping: - Extending the default SSB periodicity - Release 18 network energy saving techniques (e.g., cell DTX/DRX, etc.) with modifications to fit NR NTN (e.g., beam-based operations): - Support the configuration of more than one cell DTX patterns to a UE. - Support dynamic group-common signaling for the change of cell DTX patterns. |
| InterDigital | Proposal 1: The network uses NES cell DTx and cell DRx mechanisms to activate and de-activate satellite beams in a periodic manner. Proposal 2: Configuration and simultaneous activation of multiple cell DTx and cell DRx patterns are supported for NTN operation. Proposal 3: Fractional power sharing among the satellite beams is supported. Proposal 4: Satellite beams may adapt spatial footprint based upon coverage requirements and active traffic without UE feedback |
| Apple | Proposal 5: RAN1 considers the system level downlink coverage enhancement, using Release 18 network energy saving techniques (e.g., cell DTX/DRX, etc.) with modifications to fit NR NTN (e.g., beam-based operations). |
| LG | Proposal 4: For system-level enhancement in Rel-19 NR NTN, RAN1 carefully investigates followings: - Prioritize dynamic and flexible power sharing mechanism in per-cell basis, but not in per-satellite-beam basis. - FFS: Whether or how to support the case when more than one satellite beams of a cell are activated in a time. - Whether or how to further restrict gNB or UE behavior outside the cell DTX Active Period. - Whether or how to reduce the DL reference power of a subset of cells served by a satellite. - Whether or how to skip DL transmissions (e.g., SIB19) other than PDCCH and SPS PDSCH during the cell DTX Non-Active Period. - Whether or how to support TDD configuration for NR NTN operation in FDD carrier. - Whether or how to support dynamic and flexible power sharing between DL transmission and UL transmission at the serving satellite side. |
| Lenevo | Proposal 7: To consider other NES techniques only after stable discussion in AI 9.5. |
| Xiaomi | Proposal 2: Time domain NES solutions such as on-demand SIB1, cell DRX/DRX can be studied for DL coverage enhancement in NTN. Proposal 3: Beam based dynamic bandwidth adjustment for dynamic power sharing between beams can be studied for DL coverage enhancement in NTN. Proposal 4: Beam based DL reference signal power indication can be studied for DL coverage enhancement in NTN. |
| NTT DOCOMO | Proposal 1 (for conclusion): With respect to R18 NES techniques, no system-level enhancement beyond them is necessary from RAN1 perspective |
| Panasonic | Proposal 4: RAN1 should explore the connection between beam-hopping patterns and Cell DTX/DRX patterns. Proposal 5: RAN1 to explore the impact of DTX/DRX per SSB beam and related UE IDLE/INACTIVE mode UE behavior. |
| Qualcomm | Proposal 4: Support the configuration of more than one cell DTX patterns to a UE. Proposal 5: Support dynamic group-common signaling for the change of cell DTX patterns. |

## Initial proposals Potential enhancements at system level

### Proposal 2-1

Based on the companies’ results of system level evaluation, the following proposal is made:

Proposed observation 2-1-v0: It is observed that extending SSB periodicity is a necessary enhancement to support beam hopping in NTN

Companies are encouraged to provide views on Proposal 2-1-v0:

| Companies | Comments |
| --- | --- |
| LGE | In our understanding, to support beam hopping in NTN, wide beam approach could be used. If the intention of the observation is to check the interpretation of the WID regarding the SSB channel enhancement part, it would be better to have more neutral expression as follow: - With the SSB periodicity of 20 msec, beam hopping including common channel signalling may not be feasible in some NTN scenarios |
| Qualcomm | We don’t see that increasing SSB periodicity is a necessity condition for beam hopping. |
| MediaTek | We have similar view as LGE. We would have preference to first discuss SSB periodicity enhancements case by case for the set 1-1, set 1-2, and set 1-3 satellites considering narrow beam and wide beam. |
| vivo | As shown in our contribution, with the SSB periodicity of 20 msec, applying proper flexible beam size and hopping design can achieve 100% coverage ratio and improve the system level performance to an acceptable level. In other words, extending SSB periodicity is not essential for all kinds of deployments and configurations. Thus, we think LGE’s suggestion is more reasonable. |
| Huawei, HiSilicon | We support the proposal. And we think we can start the discussion from the observations we suggested in the comments for proposal 1-1. |
| Eutelsat Group | Extending the SSB periodicity should be considered |
| Lenovo | We also think wide beam/narrow beam should be discussed before decision on increase of default SSB periodicity. |
| Panasonic | Clarification is needed, if there is spec impact expected. Extending the SSB periodicity up to 160 ms should be already feasible. Only during initial access, the UE side assumption is 20ms SSB periodicity. If UE detects SSB independently without combining multiple SSBs, it would work. |
| Xiaomi | We think extending SSB periodicity is a candidate solution for Set 1-2. And wider beams for SSB is another candidate solution. |
| ETRI | Support the proposal as a candidate solution to support beam hopping. |
| Ericsson | We agree with Qualcomm. We are not sure if one could draw such a stronger conclusion in a broader way with respect to beam hopping. By optimizing the adjacent beam spacing corresponding to the mapping of beam footprints to the cell coverage regions on the earth, beam hopping can be supported reasonably well with the default SSB periodicity. And, there can be some beam hopping settings where the larger than the default SSB periodicity is desirable. |
| HONOR | Wider beams for SSB can be used in beam hopping in NTN. We don’t see that increasing SSB periodicity is a necessity condition for beam hopping. |
| CATT | In general we support FL proposal. From proposal 1-1, we could see the benefits of SSB periodicity extension. Then we can support the SSB periodicity is one potential candidate scheme of beam hopping. For other schemes, it is not precluded. If using wide beam, the link budget should be guaranteed at least. |
| Nokia | We find it strange that a bold statement like the one in the proposal is suggested without introducing the impacts that would be seen (like loss of backwards compatibility, longer cell search time, etc). The need for having beam hopping comes from hardware limitations (and power limitations), which originates from poor assumptions for the system evaluations. This was already raised at RAN1#116, but some companies ignored such comments. In our preference, the result of such poor conditions will simply be that some cells will experience “black-out”. |
| ZTE | Agree. Without extending SSB periodicity, the coverage ratio will be still limited due to the required dwell time of common channels. |
| Apple | We think wide beam should be discussed before we can conclude on expanding the default SSB periodicity. |
| CEWiT | We can postpone such conclusion to the future meetings. Rather proposal 2-2 should be agreed first with all possible system level solutions and then results should be compared across all possible cases rather than concluding the observation on SSB periodicity. We can add this or relevant proposal in on SSB periodicity increase in proposal 2-2 as one more possible options. |

### Proposal 2-2

Based on the companies’ inputs on the potential enhancements at system level, the following proposal is made:

Proposal 2-2-v0: The following potential DL coverage enhancements at system level are for further study/evaluation: - Using Release 18 network energy saving techniques (e.g., cell DTX/DRX, etc.) with modifications to fit NR NTN (e.g., beam-based operations): - Support the configuration of more than one cell DTX patterns to a UE. - Support dynamic group-common signaling for the change of cell DTX patterns. - Using wider beam for SSB

Companies are encouraged to provide views on Proposal 2-2-v0:

| Companies | Comments |
| --- | --- |
| LGE | Regarding the sub-bullets of the first bullet, we are OK put them as examples. Otherwise, from our side, it is too early to consider group-common signalling to support multiple cell DTX patterns. |
| Qualcomm | OK |
| MediaTek | We are fine with second bullet. On first bullet, it needs more discussion |
| DCM | Similar feeling with LGE. Besides, which kind of spec impact is relevant to the 2nd bullet? |
| vivo | It is a little confusing to us to say “support” in the sub-bullet of the 1st bullet, given that the 1st bullet itself is for study/evaluation. We suggest removing these two sub-bullets. |
| Huawei, HiSilicon | Generally, NES can have some power saving benefits, but the relation between coverage enhancement is not fully investigated. For example, how the cell DTX and cell DRX enhancement can enhance the coverage is still not very clear. In our views, the Cell DTX and DRX enhancement would need to be used with SSB periodicity extension so that the coverage ratio can be improved. It is not clear for us how wider the wider beam can be used. When the beam is wider, the coverage area can be larger however the antenna gain shall be reduced. Therefore, the wider beam size is still restricted. The wider beam is restricted by the link budget in downlink, e.g. the PSS detection performance. Per our simulation, only 3dB margin is observed for PSS detection, and therefore the beam area could be only doubled. In this sense, for the wider beam proposal, we think at least it needs to be clear that the link budget of PSS detection and PBCH decoding should not be impacted. Using wider beam for SSB without impact on downlink channel performance, including PSS detection and PBCH decoding |
| TCL | We are fine with the second bullet, for the first one, more discussion is needed, e.g., it’s too early to make a decision for using a group-common signalling for the change of cell DTX patterns. |
| Lenovo | Generally fine with the proposal. Also think group common DCI can be discussed later. |
| Panasonic | We are OK with the first bullet. The second bullet seems to be a Network implementation detail. |
| Ericsson | Does the intention of the second bullet “Using wider beam for SSB” is to utilize wider beam only for SSB transmission while precluding the transmission and/or reception of other channels/signals, e.g., SIB1, PRACH, etc., during the initial access? |
| HONOR | We are fine with the proposal. |
| CATT | For listed schemes, technically they are not precluded in evaluation aspect. The key thing is that we need to identify the benefit in coverage enhancement for introducing these new schemes. Secondly the simulation assumptions and output metrics should be carefully configured to align with agreed system level assumptions, otherwise, it is hard to move forward. Based on our initial analysis, cell DTX to coverage enhancement is unclear since it is originally used in energy saving. For the wide beam, how to set the EIRP to align the set 1-1/1-2/1-3? Under same total EIRP, we don’t think it can resolve the coverage issue in set 1-1/1-2/1-3. |
| Nokia | We are not OK with first bullet. We can live with second bullet (if it is only for further evaluation) – and we should keep in mind that wider beam will probably suffer from coverage shortage due to less power per m2 |
| ZTE | It seems premature to agree on the detailed solutions before finalizing the evaluations. Further justification based on the evaluation results may be needed for the detailed enhancements. For example, in NTN, beam-based operation is less used than in TN. Not sure why beam-based operations should be enhanced specifically to fit NTN. |
| Apple | We are fine with the proposal in general. As LGE and DCM, we think the sub-bullets in the first bullet should be put as examples. It is pre-mature to agree on the detailed solutions. |
| CEWiT | Include the SSB periodicity change options in this proposal. The following potential DL coverage enhancements at system level are for further study/evaluation: - Using Release 18 network energy saving techniques (e.g., cell DTX/DRX, etc.) with modifications to fit NR NTN (e.g., beam-based operations): - Support the configuration of more than one cell DTX patterns to a UE. - Support dynamic group-common signaling for the change of cell DTX patterns. - Using wider beam for SSB - Using wider SSB periodicity |

# Topic#3 Link Level Study results

## Companies’ contributions summary

The following proposals on Topic#3 are submitted to current RAN1 meeting:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Observation 10: For LLS assumption for SIB1 PDSCH, TBS sizes are 984 bits and 1608 bits for SIB1 payload option1 and option 2. Observation 11: TBS for SIB19 is 736bits for LLS assumption in R19 NR NTN. Observation 12: According to current specification, PBCH cannot be combined if the total time span exists 80ms. Observation 13: With parameter LEO60km Set1-1 and 1-2 considered, no coverage gaps are found for all DL channels. Observation 14: With parameter LEO600km Set 1-3, the SSB cannot even work due to the very low EIRP density. Proposal 8: Doppler frequency offset should be considered in the LLS assumption for PSS detection. Proposal 9: Do not consider link level coverage enhancement for downlink channels in a scenario/configuration where the EIRP density is reduced so much that the SSB cannot work. Proposal 10: Refine the simultaneously active beams number from 106 to 42 in Set 1-3 to make sure SSB does not have coverage gap. |
| Ericsson | Observation 10 For Set 1-1 and Set 1-2, SSB, PDCCH and PDSCH meet the requirements of the link budget in terms of the BLER target. The coverage gap for PDCCH w.r.t SSB is about 6.5 dB while the coverage gap for PDSCH w.r.t SSB is in the range 0.1 to 6.1 dB, depending on the type of message carried on the channel. Observation 11 For Set 1-3, only SSB and PDSCH Msg 2 meet the link budget requirements and the other channels, including PDCCH, PDSCH Msg 4, PDSCH SIB1 and PDSCH SIB 19, could not meet the requirements. Proposal 8 Given that multiple cases of PDSCH and PDCCH suffer from the coverage requirements to meet the BLER target under the settings of Set 1-3, RAN1 to revisit the settings of Set 1-3 or to prioritize Set 1-1/1-2 in the study. |
| Spreadtrum Communications | Observation 1: For Set1-1 FR1/ Set1-2 FR1, coverage enhancement on PDCCH is unnecessary. Observation 2: For Set1-3 FR1, coverage enhancement on PDCCH can be considered due to the 2.6 dB coverage gap. Observation 3: For Set1-1 FR1/ Set1-2 FR1/ Set1-3 FR1, coverage enhancement on PDSCH for Msg2 is unnecessary since there is no coverage gap. Observation 4: For Set1-1 FR1/ Set1-2 FR1/ Set1-3 FR1, coverage enhancement on PDSCH for Msg4 is unnecessary since there is no coverage gap. Observation 5: For Set1-1 FR1/ Set1-2 FR1, coverage enhancement on PDSCH for VoIP is unnecessary since there is no the coverage gap. Observation 6: For Set1-3 FR1, coverage enhancement on PDSCH for VoIP should be considered due to the 1.4 dB coverage gap. Observation 7: For Set1-1 FR1/ Set1-2 FR1/ Set1-3 FR1, coverage enhancement on PDSCH for low date rate service is unnecessary since there is no coverage gap. |
| Samsung | Observation 1: PDSCH (Msg 2) performance requirement can be met for set 1-1, set 1-2 and set 1-3. Observation 2: PDCCH and PDSCH (Msg 4, SIB1 and SIB 19) performance requirements can be met for set 1-1 and set 1-2. Observation 3: PDCCH and PDSCH (Msg 4, SIB1 and SIB 19) performance requirements cannot be met for set 1-3. |
| vivo | Observation 10: For LEO600km Set1-1 FR1 and Set1-2 FR1, based on link budget analysis, no coverage issue is observed for any channels. Observation 11: For LEO600km Set1-3 FR1, based on link budget analysis, 3~6 dBs performance gaps from the coverage target are observed for some of the channels and signals, namely PDCCH, PDSCH-1Mbps, Msg4, SIB1 with 800/1280 bits payload size. Observation 12: The performance loss of SIB1/SIB19 can be easily solved by enabling repetition transmission or retransmission. Observation 13: The performance loss of Msg4/PDSCH-1Mbps can be easily solved by enabling retransmission. Observation 14: The performance loss of PDCCH can be solved by a proper beam hopping pattern design at the system level aspects. Proposal 3: For LEO600km Set1-1/2/3 FR1, additional link level enhancement for DL coverage is not necessary. |
| Thales | Observation 14: For PDSCH with 3 kbps with parameter Set-1-1/ Set-1-2/ Set-1-3 for LEO-600 operating at LOS the existing specification can meet the performance requirement. Observation 15: For PDSCH with 1 Mbps with parameter Set-1-3 for LEO-600 operating at LOS the existing specification cannot meet the performance requirement with a gap of -5,29 dB. Observation 16: For PDSCH with VoIP with parameter Set-1-1/ Set-1-2/ Set-1-3 for LEO-600 operating at LOS the existing specification can meet the performance requirement. Observation 17: For PDCCH with parameter Set-1-3 for LEO-600 operating at LOS the existing specification cannot meet the performance requirement with a gap of -2,19dB. |
| ZTE | Observation 14: For PSS detection, large frequency offset and timing drift should be considered since no compensation can be performed in initial DL synchronization. Observation 15: For PSS detection, the required SNR for single-shot SSB detection is -3.3 dB, which shows an SNR margin of 1.4 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3. Even with 4-SSB combination, the required SNR is around -6.1 dB, which is still higher than CNR for Set 1-3. Observation 16: For PBCH detection, the required SNR for single-shot SSB detection is -7.2 dB, which shows an SNR margin of 5.3 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3. Observation 17: For PBCH detection, with 4-SSB combination, the required SNR is around -13.2 dB, which is lower than CNR for Set 1-3. However, the required SNR for PSS detection, which is -6.1 dB, will be the bottleneck for SSB detection. Observation 18: The required SNR for PDCCH is about -6.8 dB with single repetition, which shows an SNR margin of 4.9 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3. Observation 19: The required SNR for Msg2 PDSCH is about -11.2 dB, which shows an SNR margin of 9.3 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, and 1.3 dB with respect to CNR for LEO-600 Set 1-3. Observation 20: The required SNR for Msg4 PDSCH is about -4.5 dB, which shows an SNR margin of 2.6 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3. Observation 21: The required SNR for SIB1 is about -5.5/-3.2 dB for option 1/2, which shows an SNR margin of 3.6/1.3 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3. Observation 22: The required SNR for SIB19 is about -6.4 dB, which shows an SNR margin of 4.5 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3. Observation 23: The required SNR for PDSCH of VoIP with 2 repetitions is about -10.8 dB, which shows an SNR margin of 8.9 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, and 0.9 dB with respect to CNR for LEO-600 Set 1-3. Observation 24: The performance of 3kbps data service can be better than VoIP due to lower data rate. The performance of 1Mbps data service will be similar to Msg4 PDSCH without repetition due to similar data rate. |
| Apple | Observation 1: At 2% BLER, the required SNR for PDSCH with VoIP is -12.7 dB with 7 PRBs for 8 repetitions. Observation 2: At 10% BLER, the required SNR for PDSCH with 3 kbps is -10.4 dB with 1 PRB and 120/1024 coding rate with 8 repetitions. Observation 3: At 10% BLER, the required SNR for PDSCH with 1 Mbps is -4.9 dB with 25 PRBs and 193/1024 coding rate without repetition. Observation 4: At 10% BLER, the required SNR for Msg2 PDSCH is -6.1, -8.8 and -11.2 dB, with 3, 6 and 12 PRBs, 3 DMRS symbols and 120/1024 coding rate with scaling factor of 1, ½ and ¼, respectively. Observation 5: At 10% BLER, the required SNR for Msg4 PDSCH is -4.8 dB, -7.4 dB and -10.0 dB with 23 PRBs and 193/1024 coding rate with repetitions of 1, 2 and 4, respectively. Observation 6: At 10% BLER, the required SNR for SIB1 PDSCH is -5.8 dB and -3.7 dB with 24 PRBs for payload sizes of 800 bits and 1280 bits, respectively. Observation 7: At 10% BLER, the required SNR for PDSCH with SIB19 is -6.7 dB with 24 PRBs and 120/1024 coding rate. Observation 8: At 1% BLER, the required SNR for PDCCH is -7.9, -5.5, -2.4 and 1.7 dB at aggregation level of 8, 4, 2, 1, respectively. Observation 9: At 1% BLER, the required SNR for PBCH is -12.1 dB with a combination of 4 SSBs. Observation 10: PDSCH with 1 Mbps, PDSCH Msg2 (scaling factor of 1 or ½), PDSCH Msg4, PDSCH SIB1, PDSCH SIB19 and PDCCH have the following coverage gaps for LEO-600 set 1-3 parameters: - PDSCH with 1 Mbps: 5.0 dB - PDSCH Msg2 with scaling factor of 1: 3.8 dB - PDSCH Msg2 with scaling factor of ½: 1.1 dB - PDSCH Msg4: 5.1 dB - PDSCH SIB1: 4.1 dB - PDSCH SIB19: 3.2 dB - PDCCH: 2.0 dB |
| LG | Observation 13: For SSB detection performance as a reference, the value of Doppler shift needs to be determined based on the realistic scenario in terms of the relative locations among satellite, GW, and UE. For the elevation angle of 30 degree and the assumption that the UE is far away from the GW, 21 ppm can be a baseline. Observation 14: For PDSCH transmissions scheduled by non-fallback DCI, the existing PDSCH repetition schemes such as DL slot aggregation and repetition would be sufficient for NR FR1-NTN. Observation 15: For PDSCH transmissions scheduled by non-fallback DCI, to reduce the total active time for a given beam footprint, the existing scaling factor for TBS determination can be used even for SIBs. Moreover, the reserved state of the DCI indication for the scaling factor of TBS determination could be replaced with 1/8. |
| CATT | Observation 1: SSB detection without combination requires SNR about -5.5 dB, with 3.6 dB margin compared to CNR for LEO-600km set1-1 and set1-2. Observation 2: Even if 4 SSBs are combined, the link budget requirements of CNR=-9.9dB for set1-3 cannot be met. Observation 3: PDCCH channels can meet the link budget result of CNR=-1.9dB for Set1-1 and Set1-2, and do not require link-level enhancement. Observation 8: All cases of PDSCH can meet the link budget result of CNR=-1.9dB for Set1-1 and Set1-2, and do not require link-level enhancement. |
| CMCC | Observation 1. Regarding LEO-600 Satellite with Set 1-1/1-2 parameters in LOS environment, there is no coverage gap of DL physical channels considering -5.5dBi UE antenna gain. Observation 2. For LEO-600 Satellite with Set 1-1/1-2 parameters in LOS environment, it is observed that around 5dB coverage margin of PBCH with single SSB detection And for PBCH with 4 SSB combination, if Doppler frequency drift is not considered, 10.5dB coverage margin is observed, if Doppler frequency drift of 0.27 ppm/s is assumed, 8.89 coverage margin is observed. Observation 3. For LEO-600 Satellite with Set 1-1/1-2 parameters in LOS environment, when the Doppler frequency drift is considered in LLS for channels/signals before SIB19 acquisition, there is no coverage gap for PDSCH carrying SIB1 or SIB19. Observation 4. For LEO-600 satellite with Set1-3 parameters, there are coverage gaps for PDSCH both in initial access and connected state, including PDSCH carrying SIB1, PDSCH carrying SIB19, PDSCH Msg4, PDSCH for 1Mbps low-data rate services. - For initial access, 3.76 dB coverage gap of PDSCH carrying SIB1 (800bits) and 5.77dB coverage gap for PDSCH carrying SIB1 (1280 bits) is observed, and Msg4 PDSCH needs to be enhanced with 4.49 dB coverage gap. - To support 1Mbps low-data rate service, 7.73 dB coverage gap needs to be enhanced for PDSCH. Observation 5. For LEO-600 satellite with Set1-3 parameters, there is around 3dB coverage gap for PBCH when single SSB detection is applied no matter the Doppler frequency drift is considered or not. While 2.52dB coverage margin can be observed for PBCH when 4 SSB combination is considered without frequency drift, and barely no margin if considering Doppler frequency drift. Observation 6. For LEO-600 satellite with Set1-3 parameters, DL coverage needs to be enhanced with 3.52 dB for PDCCH. Observation 7. For LEO-600 in FR1, with default 20ms SSB periodicity, 4 SSBs combination can provide better coverage performance and coverage margins are observed for different satellite parameters. |
| Xiaomi | Observation 1: Without considering the maximum Doppler frequency drift in the LLS evaluation, for channels before SIB19 acquisition, there is no bottleneck channel based on the evaluation results for Set1-1 FR1& Set1-2 FR1. Observation 2: For Set1-3 FR1, there is performance gap for DL channels including PDCCH, SIB1 PDSCH, SIB19 PDSCH and Msg4 PDSCH even without considering the maximum Doppler frequency drift in the LLS evaluation. Observation 3: For PDCCH coverage enhancement, the possible solutions can be repetition, payload reduction and AL increasing. Observation 4: For common PDSCH coverage enhancement, a straightforward solution can be repetition. |
| ETRI | Observation 1: The CNRs for the satellite payload parameters Set 1-1, Set 1-2, and Set 1-3 are equal to -1.9 dB, -1.9 dB and -9.9 dB respectively when steering loss is not considered. Observation 2: LEO600km Set 1-3 in FR1 provides 8dB lower CNR value compared to Set 1-1 & Set 1-2 regardless of the values of steering losses. |
| Baicells | Observation 1: Lower CNR means lower MCS or lager repetition factor/aggregation level, and does mean capacity gain in the NTN DL power sharing scenario. Observation 2: For NTN DL coverage, the working point of CNR can be optimized according to power sharing schemes and should not necessarily be a very low value. Link level coverage enhancement to achieve a very low working point of SNR is not mandatory especially for UE-specific channels due to no capacity gain. |
| Oppo | Observation 4: For LEO600 set1-1 and set1-2, there is no coverage issue for DL physical channels and at least 2.6 dB link margin is obtained. Observation 5: For LEO600 set1-3, PDCCH and Msg4 PDSCH present 4.5 dB and 5.4 dB coverage performance gap respectively to be compensated, so link level enhancements are needed. |
| NTT DOCOMO | Observation 2: - Even though there is no performance gap compared to CNR in Set 1-1, there may not be sufficient margin to reduce EIRP per beam for more active beams and/or to apply wider beam. |
| MediaTek | Observation 5: For SSB evaluation, the following was observed with impairments: - NR PSS: 1 1% detection failure is met at SNR=-4.5 dB and -10.3 dB for 1 UE Rx antenna element, "1-shot" or "4 combining of SSB instances within 20 msec" respectively - NR PBCH: 1% BLER for 1 UE Rx antenna element is met - at SNR=-7.6 dB, 1-shot. - at SNR=-12.2 dB, with SSB periodicity 20 ms and 4 SSB combining over 80 ms (=4*20 ms). - at SNR=-11.8 dB, with SSB periodicity 80 ms and 4 SSB combining over 320 ms (=4*80 ms). - at SNR=-11.7 dB, with SSB periodicity 320 ms and 4 SSB combining over 1280 ms ms (=4*320 ms). |
| Panasonic | Observation 2: According to our link level simulation, the following observations are made: - For set 1-1 and set 1-2 (Target CNR is -1.9dB) - All evaluated channels satisfy the target without repetitions. - For set 1-3 (Target CNR is -9.9dB) - 8 repetitions are necessary for PDSCH VoIP and PDSCH Msg4. - PDCCH with 24PRB and AL8 (because of 5MHz bandwidth) can not meet the target. - SIB1 (800 bits), SIB1 (1280 bits) and SIB19 (616 bits) can not meet the target. Multiple transmissions or repetitions (8 transmissions for SIB1 and 4 transmissions for SIB19) are necessary. |
| Qualcomm | Observation 1: In NR NTN, UE reception of SIBs before the acquisition of satellite ephemeris (SIB19) may suffer larger timing and frequency offsets than in TN. The issue becomes more severe if SSB transmission periodicity is increased. Observation 2: To support Set1-3, coverage enhancement is needed at least for the following channels: - PDCCH - PDSCH carrying SIBs Observation 3: At 1% BLER, removing the reserved bits for the PDCCH with CRC scrambled by SI-RNTI can improved the performance by about 1.3 dB for NR NTN TDL-C. |
| CEWiT | Observation 1: The simulation results yield a required SNR value of -5.38dB for 1% target BLER of PDCCH with 8 CCE aggregations. Observation 2: For PDCCH with AL 8 has a coverage gap of -4.52dB for the set 1-3 parameters with LEO 600 satellites. Observation 4: Supporting dynamic aggregation levels for each PDCCH repetition can yield diversity gain which leads to DL coverage enhancements in NR NTN. |
| Nokia | Observation 5: SSB is out of scope for the DL coverage enhancements but should establish the baseline for evaluation of channels in potential need for DL coverage enhancements. Observation 6: The channels that may need DL coverage enhancements are PDCCH AL8, PDSCH carrying SIB1 and PDSCH carrying Msg4 Observation 7: For PDSCH Msg4, there is little room to expand the frequency-domain resources due to large TBS and low code rate. Observation 8: For configuration or indication of repetition factor of Type-0 PDCCH, transmission prior to SIB1 transmission must be considered such as MIB. |

## Companies LLS results summary

The LLS results from different companies are summarized within the following subsections:

### PDCCH coverage evaluation

The recap of PDCCH performance evaluation is given in the following table:

| Company | Channel/signal | Payload size or TBS size (bit) | Required SNR (dB) | Gap (dB) |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Set1-1,1-2 | Set1-3 |  |
|  |  |  |  | CNR -1,9 dB | CNR -9,9 dB |  |
| Huawei | Common PDCCH |  | -5.6 | -3.7 | 4.3 |  |
| Ericsson | PDCCH | 40 | -4.8 | -2.9 | 5.1 | iBLER Target 1% |
| Spreadtrum | PDCCH | 40 | -7.2 | -5.3 | 2.7 |  |
| Samsung | PDCCH | 40 | -5 | -3.1 | 4.9 | No repetition, 1% BLER |
| vivo | PDCCH |  | -4.91 | -3.01 | 4.99 |  |
| Thales | PDCCH |  | -7.7 | -5.81 | 2.19 | AL of 8 |
| ZTE | PDCCH | 40 | -6.8 | -4.9 | 3.1 | with 4 repetitions PDCCH there will be no gap for Set1-3 (Gap = -2dB) |
| Apple | PDCCH | 40 | -7.9 | -6 | 2 | 24 PRBs; no rep; 1% BLER |
| CATT | PDCCH | 40 | -7.9 | -6 |  | AL of 8, 2 symbols, 24 PRBs |
| CMCC | PDCCH | 40 | -6.38 | -4.48 | 3.52 |  |
| Xiaomi | PDCCH | 40 | -6.8 | -4.9 | 3.1 | AL=8/1% BLER SNR (dB): 2 rep = -11, 4rep = -12,9, 8 rep = -14,6 |
| OPPO |  | 40 | -5.4 | -3.5 | 4.5 |  |
| NTT DOCOMO | PDCCH | 40 | -5.04 | -3.14 |  | Aggregation level 8 |
| Panasonic | PDCCH | 40 | -4.6 | -2.7 | 5.3 | 24PRB, AL8 |
| Qualcomm | PDCCH | 39 | -5.9 | -4 | 4 | Assuming CFO 200 Hz |
| CEWiT | PDCCH | 40 | -5.38 | -3.48 | 4.52 | PDCCH with 8 CCE aggregations |
| Nokia | PDCCH | 40 | -5.28 | -3.48 | 4.52 | PDCCH AL=8 |

### PDSCH Msg2 coverage evaluation

The recap of PDSCH Msg2 performance evaluation is given in the following table:

| Company | Channel/signal | Payload size or TBS size (bit) | Required SNR (dB) | Gap (dB) |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Set1-1,1-2 | Set1-3 |  |
|  |  |  |  | CNR -1.9 dB | CNR -9.9 dB |  |
| Huawei | Msg2 | 72 | -8.3 | -6.4 | 1.6 |  |
| Ericsson | PDSCH Msg2 | 72 | -11.2 | -9.3 | -1.3 | iBLER Target 10% |
| Spreadtrum | PDSCH Msg2 | 72 | -11.6 | -9.7 | -1.7 |  |
| Samsung | PDSCH Msg2 | 72 | -14.6 | -12.7 | -4.7 | 24 PRB, MCS 1, 10% BLER |
| vivo | Msg2 |  | -14.13 | -12.23 | -4.18 |  |
| ZTE | Msg2 | 72 | -11.2 | -9.3 | -1.3 |  |
| Apple | Msg2 | 72 | -11.2 | -9.3 | -1.3 | 12 PRBs; no rep; 10% BLER; 120/1024 CR; for scaling factor = 0.25 |
| CATT | Msg2 | 72 | -5.4 | -3.5 |  |  |
| CMCC | Msg2 | 72 | -11.59 | -9.69 | -1.69 |  |
| Xiaomi | Msg2 | 72 | -13.4 | -11.5 | -3.5 | scaling factor=0.25 |
| Oppo | Msg2 | 72 | -10.6 | -8.7 | -0.7 | the TBS scaling factor of 0.25 |
| NTT DOCOMO | Msg2 | 72 | -5.38 | -3.48 |  |  |
| Panasonic | Msg2 |  | -10.1 | -8.2 | -0.2 |  |
| Nokia | Msg2 | 64 | -13.41 | -11.51 | -3.51 |  |

### PDSCH Msg4 coverage evaluation

The recap of PDSCH Msg4 performance evaluation is given in the following table:

| Company | Channel/signal | Payload size or TBS size (bit) | Required SNR (dB) | Gap (dB) |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Set1-1,1-2 | Set1-3 |  |
|  |  |  |  | CNR -1.9 dB | CNR -9.9 dB |  |
| Huawei | PDSCH Msg4 | 1040 | -4.0 | -2.1 | 5.9 |  |
|  |  |  |  |  |  | TBS=1128bits |
| Ericsson | PDSCH Msg4 | 1040 | -7.8 | -5.9 | 2.1 | iBLER Target 10% |
| Spreadtrum | PDSCH Msg4 | 1040 | -10.2 | -8.3 | -0.3 |  |
| Samsung | PDSCH Msg4 | 1040 | -4.7 | -2.8 | 5.2 | 24 PRB, MCS 1, 10% BLER |
| vivo | PDSCH Msg4 |  | -4.74 | -2.84 | 5.16 |  |
| ZTE | PDSCH Msg4 | 1040 | -4.5 | -2.6 | 5.4 |  |
| Apple | PDSCH Msg4 | 1064 | -4.8 | -2.9 | 5.1 | 23 PRBs; no rep; 10% BLER; 193/1024 CR |
| CATT | PDSCH Msg4 | 1064 | -3.5 | -1.6 |  |  |
| CMCC | PDSCH Msg4 |  | -5.41 | -3.51 | 4.49 | MCS2 for Msg.4 (1040bit) |
| Xiaomi | PDSCH Msg4 |  | -6.5 | -4.6 | 3.4 |  |
| Oppo | PDSCH Msg4 | 1040 | -4.5 | -2.6 | 5.4 | 1040 bits for Msg4 PDSCH  MCS2 25 PRBs |
| NTT DOCOMO | PDSCH Msg4 |  | -3.88 | -1.98 |  |  |
| Panasonic | PDSCH Msg4 | 1040 | -3.5 | -1.6 | 6,4 |  |
| Nokia | PDSCH Msg4 | 1040 | -4.95 | -3.05 | 4.95 | 1040 bits for Msg4 PDSCH  MCS2 25 PRBs |

### PDSCH SIB1 coverage evaluation

The recap of PDSCH SIB1 performance evaluation is given in the following table:

| Company | Channel/signal | Payload size or TBS size (bit) | Required SNR (dB) | Gap (dB) |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Set1-1,1-2 | Set1-3 |  |
|  |  |  |  | CNR -1.9 dB | CNR -9.9 dB |  |
| Huawei | SIB1 (option 1) | 800 | -4.0 | -2.1 | 5.9 | TBS=984bits |
|  |  |  |  |  |  | -800 |
| Ericsson | SIB1 (option 1) | 800 | -6.2 | -4.3 | 3.7 | iBLER Target 10% |
| Samsung | SIB1 (option 1) | 800 | -6 | -4.1 | 3.9 | 24 PRB, MCS 1, 10% BLER |
| vivo | SIB1 (option 1) | 800 | -5.73 | -3.83 | 4.17 |  |
| ZTE | SIB1 (option 1) | 800 | -5.5 | -3.6 | 4.4 |  |
| Apple | SIB1 (option 1) | 888 | -5.8 | -3.9 | 4.1 | 24 PRBs; no rep; 10% BLER; 157/1024 CR |
| CATT | SIB1 (option 1) | 800 | -4.3 | -2.4 |  |  |
| CMCC | SIB1 (option 1) | 800 | -6.14 | -4.24 | 3.76 |  |
| Xiaomi | SIB1 (option 1) | 800 | -6.1 | -4.2 | 3.8 | w/o soft combining |
| Oppo | SIB1 (option 1) | 800 | -13.3 | -11.4 | -3.4 | combination of 8 SIBs considered |
| NTT DOCOMO | SIB1 (option 1) | 800 | -4.97 | -3.07 |  |  |
| Panasonic | SIB1 (option 1) | 800 | -4.9 | -3 | 5 | With 8 rep required SNR = -12.0 dB |
| Qualcomm | SIB1 (option 1) | 704 | -5.3 | -3.4 | 4.6 |  |
|  | SIB1 (option 1) | 800 | -4.75 | -2.85 | 5.15 |  |
| Nokia | SIB1 (option 1) | 800 | -4.25 | -2.35 | 5.65 |  |

| Company | Channel/signal | Payload size or TBS size (bit) | Required SNR (dB) | Gap (dB) |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Set1-1,1-2 | Set1-3 |  |  |
|  |  |  |  | CNR -1.9 dB | CNR -9.9 dB |  |  |
| Huawei | SIB1(option 2) | 1280 | -2.5 | -0.6 | 7.4 | TBS=1608bits (1280). |  |
| Ericsson | PDSCH SIB1 (Opt 2) | 1280 | -4.9 | -3.0 | 5.0 | iBLER Target 10% |  |
| Samsung | SIB1 (option 2) | 1280 | -3.9 | -2 | 6 |  |  |
| vivo | SIB1(option 2) | 1280 | -3.75 | -1.85 | 6.15 |  |  |
| ZTE | SIB1(option 2) | 1280 | -3.2 | -1.3 | 6.7 |  |  |
| Apple | SIB1(option 2) | 1280 | -3.7 | -1.8 | 6.2 | 24 PRBs; no rep; 10% BLER; 251/1024 CR |  |
| CATT | SIB1(option 2) | 1280 | -2.1 | -0.2 |  |  |  |
| CMCC | SIB1(option 2) | 1280 | -4.13 | -2.23 | 5.77 |  |  |
| Xiaomi | SIB1(option 2) | 1280 | -4.2 | -2.3 | 5.7 | w/o soft combining. w/ soft combining=4 |  |
| NTT DOCOMO | SIB1(option 2) | 1280 | -2.97 | -1.07 |  |  |  |
| Panasonic | SIB1(option 2) | 1280 | -3.0 | -1.1 | 6.9 | With 8 rep required SNR = -10.5 dB |  |
| Nokia | SIB1(option 2) | 1280 | -2.41 | -0.51 | 7.49 |  |  |

### PDSCH SIB19 coverage evaluation

The recap of PDSCH SIB19 performance evaluation is given in the following table:

| Company | Channel/signal | Payload size or TBS size (bit) | Required SNR (dB) | Gap (dB) |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Set1-1,1-2 | Set1-3 |  |
|  |  |  |  | CNR -1.9 dB | CNR -9.9 dB |  |
| Huawei | SIB19 | 616 | -4.6 | -2.7 | 5.3 | TBS=736bits (616) |
| Ericsson | PDSCH SIB19 | 616 | -6.8 | -4.9 | 3.1 | iBLER Target 10% |
| Samsung | SIB19 | 616 | -7 | -5.1 | 2.9 | 24 PRB, MCS 1, 10% BLER |
| vivo | SIB19 |  | -5.99 | -4.09 | 3.91 |  |
| ZTE | SIB19 | 616 | -6.4 | -4.5 | 3.5 | TBS:672 |
| Apple | SIB19 | 616 | -6.7 | -4.8 | 3.2 | 24 PRBs; no rep; 10% BLER; 120/1024 CR |
| CATT | SIB19 | 616 | -5.4 | -3.5 |  |  |
| CMCC | SIB19 | 616 | -6.85 | -4.95 | 3.05 | MCS0 for SIB19 (616bits) |
| Xiaomi | SIB19 | 616 | -7.2 | -5.3 | 2.7 |  |
| Oppo | SIB | 800 | -13.3 | -11.4 | -3.4 | combination of 8 SIBs considered |
| NTT DOCOMO | SIB19 | 616 | -5.90 | -4.00 |  |  |
| Panasonic | SIB19 | 616 | -5.7 | -3.8 | 4,2 | With 4 combination the gap is -0.6dB for SET 1-3 |
| Qualcomm | SIB19 |  | higher than -9.9 dB |  | >0 |  |
| Nokia | SIB19 | 616 | -5.2 | -3.3 | 4.7 |  |

### PDSCH VoIP coverage evaluation

The recap of PDSCH VoNR performance evaluation is given in the following table:

| Company | Channel/signal | Payload size or TBS size (bit) | Required SNR (dB) | Gap (dB) |  | comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Set1-1,1-2 | Set1-3 |  |
|  |  |  |  | CNR -1.9 dB | CNR -9.9 dB |  |
| Huawei | VoIP |  | -13.8 | -11.9 | -3.9 |  |
| Spreadtrum | VoIP | 184 | -8.4 | -6.5 | 1.5 | # repetitions: 8 |
| vivo | VoIP |  | -14.08 | -12.18 | -4.18 |  |
| Thales | VoIP | 184 | -12,4 | -10,51 | -2,51 | 8 repetitions |
| ZTE | VoIP | 184 | -10.8 | -8.9 | -0.9 | 2 repetitions |
| Apple | VoIP | 192 | -12.7 | -10.8 | -2.8 | 7 PRBs; 8 rep; 2% BLER; 120/1024 CR |
| CATT | VoIP | 184 | -4.0 | -2.1 |  | PDSCH for VoIP (2% rBLER) |
| CMCC | VoIP | 184 | -12.36 | -10.46 | -2.46 | 16 repetitions for VoIP |
| Xiaomi | VoIP | 184 | -11.8 | -9.9 | -1.9 | rep=4 |
| Oppo | VoIP | 184 | -11.2 | -9.3 | -1.3 | 184bits for VoIP PDSCH  MCS0, 6 PRBs. 20 repetitions |
| Panasonic | VoIP | 184 | -10.7 | -8.8 | -0.8 | # repetitions: 8 |

### PDSCH Low data rate coverage evaluation

The recap of PDSCH Low data rate performance evaluation is given in the following table:

| Company | Channel/signal | Payload size or TBS size (bit) | Required SNR (dB) | Gap (dB) |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Set1-1,1-2 | Set1-3 |  |
|  |  |  |  | CNR -1.9 dB | CNR -9.9 dB |  |
| Spreadtrum | 3kbps | 24 | -10.0 | -8.1 | -0.1 |  |
| vivo | 3kbps |  | -14.73 | -12.83 | -4.83 |  |
| Thales | 3kbps |  | -10,2 | -8,31 | -0,31 | 8 repetitions |
| ZTE | The performance of 3kbps data service can be better than VoIP due to lower data rate |  |  |  |  |  |
| Apple | 3kbps | 24 | -10.4 | -8.5 | -0.5 | 1 PRB; 8 rep; 10% BLER; 120/1024 CR |
| CATT | 3kbps | 96 | -5.4 | -3.5 |  |  |
| CMCC | 3kbps | 72 | -11.75 | -9.85 | -1.85 | 16 repetitions for 3kbps data rate |
| Xiaomi | 3kbps |  | -12 | -10.1 | -2.1 | rep=4 |
| NTT DOCOMO | 3kbps |  | -11.07 | -9.17 |  |  |

### PDSCH 1Mbps coverage evaluation

The recap of PDSCH performance evaluation is given in the following table:

| Company | Channel/signal | Payload size or TBS size (bit) | Required SNR (dB) | Gap (dB) |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Set1-1,1-2 | Set1-3 |  |
|  |  |  |  | CNR -1.9 dB | CNR -9.9 dB |  |
| Huawei | 1Mbps |  | -5.3 | -3.4 | 4.6 |  |
| vivo | 1Mbps |  | -4.90 | -3.00 | 5.00 |  |
| Thales | 1Mbps |  | -4,6 | -2,71 | 5,29 | 0 repetition |
| ZTE | The performance of 1Mbps data service will be similar to Msg4 PDSCH without repetition due to similar data rate |  |  |  |  |  |
| Apple | 1Mbps | 1128 | -4.9 | -3.0 | 5.0 | 25 PRBs; no rep; 10% BLER 193/1024 CR |
| CATT | 1Mbps | 1024 | -3.5 | -1.6 |  |  |
| CMCC | 1Mbps |  | -2.17 | -0.27 | 7.73 | MCS10 for 1Mbps data rate |
| NTT DOCOMO | 1Mbps |  | -3.74 | -1.84 |  |  |

### SSB coverage evaluation

The recap of SSB performance evaluation is given in the following table:

| Company | Channel/signal | Payload size or TBS size (bit) | Required SNR (dB) | Gap (dB) |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Set1-1,1-2 | Set1-3 |  |
|  |  |  |  | CNR -1.9 dB | CNR -9.9 dB |  |
| Huawei | PSS single shot detection |  | -3.7 | -1.8 | 6.2 | maximum timing offset for PSS could reach 48ppm, while the maximum frequency offset is 24ppm |
|  | PSS (4 combinex20ms) |  | -5.7 | -3.8 | 4.2 |  |
|  | PBCH (single shot) |  | -7.3 | -5.4 | 2.6 |  |
|  | PBCH (4repx20ms) |  | -12.5 | -10.6 | -2.6 |  |
| Ericsson | SSB of period 20 ms (4 combinations) |  | -11.3 | -9.4 | -1.4 | BLER Target 1% |
| ZTE | PSS (single shot detection) |  | -3.3 | -1.4 | 6.6 | Max Time drift for PSS: 48 ppm |
|  |  |  |  |  |  | Max Frequency offset for PSS: 24 ppm |
|  | PSS (combination of 4 ) |  | -6.1 | -4.2 | 3.8 |  |
|  | PBCH (single shot) |  | -7.2 | -5.3 | 2.7 |  |
|  | PBCH (combination of 4) |  | -13.2 | -11.3 | -3.3 |  |
| Apple | 4 SSBs in 80 ms. | 56 | -12.1 | -10.2 | -2.2 | Combination of 4 SSBs in 80ms; 1% BLER |
| CATT | PBCH (single shot) |  | -5.5 | -3.6 | 4.4 | Max Doppler shift: 24 ppm without pre-compensation |
|  | PBCH (combination of 4) |  | -9.0 | -7.1 | 0.9 |  |
| CMCC | PBCH (single shot) | 56 | -6.85 | -4.95 | 3.05 | Max Doppler shift: 24 ppm and 0.27ppm/s frequency drift |
|  | PBCH (combination of 4) | 56 | -10.79 | -8.89 | -0.89 |  |
| Xiaomi | 4 SSBs in 80 ms. |  | -12.7 | -10.8 | -2.8 | Combination of 4 SSBs in 80ms; 1% BLER |
| MediaTek | PSS single shot detection |  | -4.5 | -2.6 | 5.4 | 1 UE Rx antenna, 1% detection failure |
|  | PSS (4 combination within 20ms) |  | -10.3 | -8.4 | -0.4 | Doppler shift drift was not included |
|  | PBCH single shot detection | 56 | -7.6 | -5.7 | 2.3 |  |
|  | PBCH (4 combination within 80ms =4*20ms) | 56 | -12.2 | -10.3 | -2.3 |  |
|  | PBCH (4 combination within 320ms=4*80ms) | 56 | -11.8 | -9.9 | -1.9 |  |
|  | PBCH (4 combination within 1280ms= 4*320ms) | 56 | -11.7 | -9.8 | -1.8 |  |
| Nokia | SSB of period 20 ms (4 combinations) |  | -10.87 | -8.97 | -0.97 | BLER Target 1% |

## Initial proposals on LLS results

Based on the preliminary results collected in the previous section, the following proposed observations are made:

### PDCCH coverage evaluation

#### Proposal 3-1

Proposed Observation 3-1-v0: Based on LLS results on PDCCH coverage evaluation collected from different sources: - It is observed that the required SNR for PDCCH is in average equal to -6dB (17 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - 17 sources observed that there is no coverage gap with Set1-1/1-2 FR1 - The coverage margin is round 4 dB - With parameter LEO600km Set1-3: - 15 sources observed that there is a PDCCH coverage gap of 3.9dB in average

Companies are encouraged to share views on Proposed Observation 3-1-v0

| Companies | Comments |
| --- | --- |
| Qualcomm | OK |
| MediaTek | We are generally fine to capture companies evaluation results |
| DCM | OK |
| vivo | The coverage gap is concluded based on the assumption without combining nor beam level optimization, such as power sharing or multiple beams illuminating a same area. The above description should be clarified/noted in the observation. |
| Huawei, HiSilicon | There are several companies raising concerns on the LEO600km Set 1-3. In this sense, it is not reasonable to make observation for Set 1-3 on this, and count these companies showing concerns on Set 1-3 into the 15 companies. Furthermore, a number of companies observed that for Set1-3, there is still coverage gap for PSS detection. Before we get to the conclusion or observation of LL performance of different channels, we need to first clarity whether the assumptions for Set1-3 is reasonable or whether we can retune the Set 1-3 with less active beams to make sure the PSS detection has no coverage issue. It doesn’t make sense to discuss the coverage gaps for Set1-3, if there is still coverage gap of PSS detection for Set 1-3. |
| TCL | Ok |
| Panasonic | We are fine with the proposed observation. |
| Xiaomi | OK |
| ETRI | OK |
| Ericsson | OK |
| HONOR | OK |
| CATT | It is hard to conclude the observations for LEO600km Set1-3. If SSB is not workable, how can we evaluate the PDCCH evaluation gap? |
| Nokia | In principle OK, but perhaps reference the AL used for the PDCCH evaluation (since it is an observation) |
| ZTE | OK |
| Apple | We support the proposed observation. |

### PDSCH Msg2 overage evaluation

#### Proposal 3-2

Proposed Observation 3-2-v0: Based on LLS results on PDSCH Msg2 coverage evaluation collected from different sources: - It is observed that the required SNR for PDSCH carrying Msg2 is in average equal to – 10.9 dB (14 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - 14 sources observed that there is no coverage gap for PDSCH with Msg2: - The coverage margin is round 9 dB - With parameter LEO600km Set1-3 FR1: - 1 source observed that there is a coverage gap of 1.6 dB for PDSCH with Msg2 - 11 sources observed that there is no coverage gap for PDSCH with Msg2: - The coverage margin is round 2 dB

Companies are encouraged to share views on Proposed Observation 3-2-v0

| Companies | Comments |
| --- | --- |
| MediaTek | We are generally fine to capture companies evaluation results |
| DCM | OK |
| Vivo | Similar comments as for proposal 3-1. The coverage gap is concluded based on the assumption without repetition or retransmission. The above description should be clarified/noted in the observation. |
| Huawei, HiSilicon | Similar comments for proposal 3-1. It is controversial for the conclusion in the Set 1-3. |
| TCL | Ok |
| Panasonic | We are fine with the proposed observation. |
| Xiaomi | OK |
| ETRI | OK |
| Ericsson | OK |
| HONOR | OK |
| Nokia | OK |
| ZTE | OK |
| Apple | Okay |

### PDSCH Msg4 overage evaluation

#### Proposal 3-3

Proposed Observation 3-3-v0: Based on LLS results on PDSCH coverage evaluation collected from different sources: - It is observed that the required SNR for PDSCH carrying Msg4 is in average equal to – 5.2 dB (14 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - 14 sources observed that there is no coverage gap for PDSCH with Msg2: - The coverage margin is round 3.3 dB - With parameter LEO600km Set1-3 FR1: - 11 sources observed that there is a coverage gap for PDSCH with Msg4: - The coverage gap is round 4.8 dB - 1 source observed that there is no coverage gap for PDSCH with Msg4 with a coverage margin of 0.3 dB

Companies are encouraged to share views on Proposed Observation 3-3-v0

| Companies | Comments |
| --- | --- |
| MediaTek | We are generally fine to capture companies evaluation results |
| DCM | OK |
| vivo | Same comments as for proposal 3-2. The coverage gap is concluded based on the assumption without repetition or retransmission. The above description should be clarified/noted in the observation. |
| Huawei, HiSilicon | Similar comments for proposal 3-1. It is controversial for the conclusion in the Set 1-3. |
| TCL | Ok |
| Panasonic | We are fine with the proposed observation. |
| Xiaomi | There are some typos in Proposal 3-3-v0, suggest the following update: Based on LLS results on PDSCH Msg4 coverage evaluation collected from different sources: - It is observed that the required SNR for PDSCH carrying Msg4 is in average equal to – 5.2 dB (14 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - 14 sources observed that there is no coverage gap for PDSCH with Msg2 Msg4: - The coverage margin is round 3.3 dB - With parameter LEO600km Set1-3 FR1: - 11 sources observed that there is a coverage gap for PDSCH with Msg4: - The coverage gap is round 4.8 dB - 1 source observed that there is no coverage gap for PDSCH with Msg4 with a coverage margin of 0.3 dB |
| ETRI | OK |
| Ericsson | OK |
| HONOR | OK |
| Nokia | Could be OK |
| ZTE | OK |
| Apple | Okay |

### PDSCH SIB1 coverage evaluation

#### Proposal 3-4

Proposed Observation 3-4-v0: Based on LLS results on PDSCH SIB1 coverage evaluation collected from different sources: - For PDSCH carrying SIB1 option 1 (with a payload size of 800bits) it is observed that the required SNR is in average equal to – 5.8 dB (14 sources) - For PDSCH carrying SIB1 option 2 (with a payload size of 1280bits) it is observed that the required SNR is in average equal to – 3.4 dB (12 sources) - With parameter LEO600km Set1-1 FR1 and 1-2FR1: - 14 sources observed that there is no coverage gap for PDSCH with SIB1 option 1: - The coverage margin is round 3.9 dB - 12 sources observed that there is no coverage gap for PDSCH with SIB1 option 2: - The coverage margin is round 1.5 dB - With parameter LEO600km Set1-3 FR1: - 11 sources observed that there is a coverage gap for PDSCH with SIB1 option 1: - The coverage gap is round 4.5 dB - 1 source observed that there is no coverage gap for PDSCH with SIB1 option 1: - The coverage margin is 3.4 dB - 10 sources observed that there is a coverage gap for PDSCH with SIB1 option 2: - The coverage gap is round 6.3 dB

Companies are encouraged to share views on Proposed Observation 3-4-v0

| Companies | Comments |
| --- | --- |
| MediaTek | We are generally fine to capture companies evaluation results |
| DCM | OK |
| vivo | The conditions of the concluded coverage gap should be captured/noted, such as the whether or note soft combination/repetition is applied (and if yes, how many times), etc. |
| Huawei, HiSilicon | Similar comments for proposal 3-1. It is controversial for the conclusion in the Set 1-3. |
| TCL | Ok |
| Panasonic | We are fine with the proposed observation. |
| Xiaomi | OK |
| ETRI | OK |
| Ericsson | OK |
| HONOR | OK |
| Nokia | OK |
| ZTE | OK |
| Apple | OK |

### PDSCH SIB19 coverage evaluation

#### Proposal 3-5

Proposed Observation 3-5-v0: Based on LLS results on PDSCH SIB19 coverage evaluation collected from different sources: - It is observed that the required SNR for PDSCH carrying SIB19 is in average equal to – 6.9 dB (14 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - 12 sources observed that there is no coverage gap for PDSCH with SIB19: - The coverage margin is round 4.2 dB - With parameter LEO600km Set1-3 FR1: - 10 sources observed that there is a coverage gap for PDSCH with SIB19: - The coverage gap is round 3.5 dB

Companies are encouraged to share views on Proposed Observation 3-5-v0

| Companies | Comments |
| --- | --- |
| MediaTek | We are generally fine to capture companies evaluation results |
| DCM | OK |
| Vivo | Same comments as proposal 3-4 |
| Huawei, HiSilicon | Similar comments for proposal 3-1. It is controversial for the conclusion in the Set 1-3. |
| TCL | Ok |
| Panasonic | We are fine with the proposed observation. |
| Xiaomi | OK |
| ETRI | OK |
| Ericsson | OK |
| HONOR | OK |
| Nokia | OK |
| ZTE | OK |
| Apple | OK |

### PDSCH VoIP coverage evaluation

#### Proposal 3-6

Proposed Observation 3-6-v0: Based on LLS results on PDSCH for VoIP coverage evaluation collected from different sources: - It is observed that the required SNR for PDSCH for VoIP is in average equal to – 11 dB (11 sources) - With parameter LEO600km Set1-1 and 1-2 FR1: - When PDSCH repetition is enabled, 11 sources observed that there is no coverage gap for VoIP: - The coverage margin is round 9 dB - With parameter LEO600km Set1-3 FR1: - When PDSCH repetition is enabled, 9 sources observed that there is no coverage gap for VoIP: - The coverage margin is round 2.3 dB - 1 source observed that even with 8 PDSCH repetitions there is a coverage gap of 1.5 dB

Companies are encouraged to share views on Proposed Observation 3-6-v0

| Companies | Comments |
| --- | --- |
| MediaTek | We are generally fine to capture companies evaluation results |
| DCM | OK |
| vivo | Same comments as proposal 3-4 |
| Huawei, HiSilicon | Similar comments for proposal 3-1. It is controversial for the conclusion in the Set 1-3. |
| TCL | Ok |
| Panasonic | We are fine with the proposed observation. |
| Xiaomi | OK |
| ETRI | OK |
| HONOR | We fine with the observation. |
| Nokia | OK |
| ZTE | OK |
| Apple | OK |

### PDSCH Low data rate coverage evaluation

#### Proposal 3-7

Proposed Observation 3-7-v0: Based on LLS results on PDSCH 3kbps coverage evaluation collected from different sources: - It is observed that the required SNR for PDSCH for low data rate is in average equal to – 11 dB (8 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - When PDSCH repetition is enabled, 8 sources observed that there is no coverage gap for PDSCH with 3kbp: - The coverage margin is round 8.8 dB - With parameter LEO600km Set1-3 FR1: - When PDSCH repetition is enabled, 6 sources observed that there is no coverage gap for PDSCH with 3kbp: - The coverage margin is round 1.6 dB

Companies are encouraged to share views on Proposed Observation 3-7-v0

| Companies | Comments |
| --- | --- |
| MediaTek | We are generally fine to capture companies evaluation results |
| DCM | OK |
| vivo | Same comments as proposal 3-4 |
| Huawei, HiSilicon | Similar comments for proposal 3-1. It is controversial for the conclusion in the Set 1-3. |
| TCL | Ok |
| Panasonic | We are fine with the proposed observation. |
| Xiaomi | OK |
| ETRI | OK |
| HONOR | OK |
| Nokia | OK |
| ZTE | OK |
| Apple | OK |

### PDSCH 1Mbps coverage evaluation

#### Proposal 3-8

Proposed Observation 3-8-v0: Based on LLS results on PDSCH 1Mbps coverage evaluation collected from different sources: - It is observed that the required SNR for PDSCH with 1Mbps data rate is in average equal to – 4.1 dB (7 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - 7 sources observed that there is no coverage gap for PDSCH with 1Mbps: - The coverage margin is round 2.2 dB - With parameter LEO600km Set1-3 FR1: - 5 sources observed that, there is a coverage gap for PDSCH with 1Mbps: - The coverage gap is round 5.5 dB

Companies are encouraged to share views on Proposed Observation 3-8-v0

| Companies | Comments |
| --- | --- |
| MediaTek | We are generally fine to capture companies evaluation results |
| DCM | DCM |
| Vivo | Same comments as proposal 3-4 |
| Huawei, HiSilicon | Similar comments for proposal 3-1. It is controversial for the conclusion in the Set 1-3. |
| TCL | Ok |
| Panasonic | We are fine with the proposed observation. |
| ETRI | OK |
| HONOR | OK |
| Nokia | OK |
| ZTE | OK |
| Apple | OK |

### SSB coverage evaluation

#### Proposal 3-9

Proposed Observation 3-9-v0: Based on LLS results on SSB coverage evaluation collected from different sources: - With parameter LEO600km Set1-1 and 1-2, the required SNR for SSB detection is ranging from -10.8dB to -3.6dB and thereby no coverage gap is observed for LEO600km Set1-1 and 1-2. - With parameter LEO600km Set1-3: - 7 sources observed that with 4 SSB combinations the SSB can be successfully detected with a margin ranging from 0.2dB to 6.68dB - 2 sources observed that there is a coverage gap of 3.4 dB and 0.78dB respectively even with 4 SSB combinations. - 2 sources observed that there is a coverage gap of 2.95 dB and 1.8dB respectively only with a single SSB detection.

Companies are encouraged to share views on Proposed Observation 3-9-v0:

| Companies | Comments |
| --- | --- |
| LGE | We have some questions on the evaluation assumption for the SSB. In our understanding, it is assumed that the elevation angle of 30 degree to derive the CNR. On the other hand, when we see the value of Doppler shift for SSB evaluation (24ppm), it is associated with the elevation angle of 10 degree according to the formula specified in TR38.811. Is it still fair comparison? |
| MediaTek | We are generally fine to capture companies evaluation results |
| DCM | We are not sure whether agreeing this observation is possible before determining simulation assumptions for SSB. |
| vivo | Same comments as proposal 3-4 regarding the basic assumption (e.g., with or without soft combining, etc.) |
| Huawei, HiSilicon | We think PSS detection performance and PBCH decoding performance is essential for the link level coverage study. Otherwise, we are not even sure the proposed coverage enhancement solutions could work or not. Regarding the detailed proposal, it is not correct to mix the PSS detection performance with PBCH decoding performance. For PBCH decoding, the companies observed margin in Set 1-3 did not consider PSS detection in the simulation. This should not be ignored otherwise the study is not complete. |
| Panasonic | We are fine with the proposed observation. |
| Xiaomi | It is better to capture the evaluation results for PSS and PBCH respectively. |
| CATT | For SSB channel evaluation, the PSS/SSS and PBCH should be separated because the Doppler shift and timing error are quite different. It is very important to have realist assumptions and realistic results in NTN case. |
| Nokia | We are not sure what the purpose of this proposed observation would be. The target from WID it to NOT touch the SSB. |
| ZTE | It seems that different companies have different assumptions, especially on the impact of Doppler shift. And this leads to significant difference of the evaluation results. Therefore, it is premature to have such observation which mix different assumptions. Moreover, for the observation, seems that at least 3 companies find coverage gap between SSB and CNR of set 1-3, which is not correctly reflected. |
| Apple | We share the same view as LGE. The value of Doppler shift (24 ppm) for SSB evaluation in some companies’ simulation is based on 10 degrees elevation angles, which is not aligned with the satellite parameter assumption (30 degrees elevation angles). We also agree with DCM that any observations should be based on agreed simulation assumptions. |

# Topic#4 Potential enhancements at link level

## Companies’ contributions summary

The following proposals on Topic#1-3 are submitted to current RAN1 meeting:

| Companies | Proposals |
| --- | --- |
| Spreadtrum Communications | Proposal 1: M-TRP based PDCCH repetition in R16 can be as a start point for PDCCH coverage enhancement in NR NTN. Proposal 2: The enhancement of PDSCH coverage in RRC CONNECTED mode needs to consider the impact on spectral efficiency of system. |
| Samsung | Proposal 1: PDSCH (Msg2) enhancement is not considered in Rel-19 NTN. Proposal 2: RAN1 discusses whether to consider enhancements for PDCCH and PDSCH (SIB1) in Rel-19 NTN considering if set 1-3 can be supported by existing SSB scheme. |
| vivo | Proposal 3: For LEO600km Set1-1/2/3 FR1, additional link level enhancement for DL coverage is not necessary. |
| Thales | Proposal 5: RAN1 to introduce enhancements on the DL coverage of the following channels: - PDCCH - PDSCH with 1 Mbps |
| ZTE | Proposal 2: The required SNR for SSB detection is larger than the CNR for Set 1-3 even with 4-SSB combination, i.e., CNR for Set 1-3 should not be the target for link level enhancement. Proposal 3: For LEO-600 Set 1-1 and Set 1-2, link level enhancement on PDCCH is not needed. Proposal 4: For LEO-600 Set 1-3, the link budget is not enough for PDCCH detection. Up to 4 repetitions may be needed to mitigate the SNR gap. Proposal 5: For all the satellite parameter sets for LEO-600, link level enhancement on PDCCH is not needed. Proposal 6: For LEO-600 Set 1-1 and Set 1-2, link level enhancement on msg4 PDSCH is not needed. Proposal 7: For LEO-600 Set 1-3, the link budget is not enough for msg4 PDSCH detection. Repetitions may be needed to mitigate the SNR gap. Proposal 8: For LEO-600 Set 1-1 and Set 1-2, link level enhancement on SIB1 is not needed. Proposal 9: For LEO-600 Set 1-3, the link budget is not enough for SIB1 detection. Repetitions may be needed to mitigate the SNR gap. Proposal 10: For LEO-600 Set 1-1 and Set 1-2, link level enhancement on SIB19 is not needed. Proposal 11: For LEO-600 Set 1-3, the link budget is not enough for SIB19 detection. Repetitions may be needed to mitigate the SNR gap. Proposal 12: For all the satellite parameter sets for LEO-600, link level enhancement on PDSCH of VoIP is not needed. Proposal 13: For LEO-600 Set 1-1 and 1-2, data service with 3kbps and 1Mbps can be supported without enhancement. Proposal 14: For LEO-600 Set 1-3, data service with 3kbps can be supported without enhancement, but data service with 1Mbps cannot be supported. |
| InterDigital | Proposal 5: The network uses higher aggregation factor for PDCCH transmission while the satellite beams are in power sharing mode. Proposal 6: PDCCH can be repeated multiple times to compensate the PDCCH coverage loss due to power sharing among the satellite beams. Proposal 7: PDSCH can be transmitted with a higher number of repetitions with fractional power transmission from satellite beams. |
| Apple | Proposal 1: At least the coverage of PDCCH, PDSCH with Msg 4, PDSCH with SIB1 and PDSCH with SIB19 need to be enhanced. - Further evaluate whether the coverage of PDSCH with Msg2 needs to be enhanced. Proposal 2: To enhance the coverage of PDSCH Msg 4 and PDSCH carrying SIB1, consider introducing PDSCH repetition. Proposal 3: To enhance the coverage of PDSCH, the schemes (e.g., TB over multiple slots, DMRS bundling, etc.) used to enhance PUSCH performance should be examined for PDSCH. Proposal 4: To enhance the coverage of PDCCH, consider the following solutions: - Enhance PDCCH repetition (e.g., for PDCCH scheduling system information) - Increase CORESET symbol number and aggregation level - Reduce DCI size (e.g., 2-stage DCI) |
| LG | Proposal 3: For link-level enhancement in Rel-19 NR NTN, RAN1 carefully investigates followings: - PDSCH repetition for PDSCH transmissions scheduled by fallback DCI - How to indicate PDSCH repetition number - Scaling factor indicated by DCI for TBS determination for PDSCH - Whether or how to apply it to SIB transmissions - Whether or how to support the lower value (e.g., 1/8) - Search space linkage for PDCCH carrying DCI format for SIB1. |
| CATT | Proposal 2: Link level enhancement seems not be urgent from the evaluation result. |
| Lenevo | Proposal 8: Consider repetition of PDCCH/PDSCH, TBoMS/DMRS bundling for PDCSH, larger aggregation level/number of CORESET symbols for PDCCH for DL coverage enhancement. |
| CMCC | Proposal 1. When additional loss (e.g. steering loss) is not considered, DL coverage enhancements are not required for LEO-600 with Set 1-1/1-2 satellite parameters to serve the target service. Proposal 2. To support target services under LEO-600 scenario with set1-3 reference parameters, DL coverage enhancements need to be considered for PBCH with single SSB detection, PDSCH carrying SIB1, PDSCH carrying SIB19, Msg4 PDSCH and PDSCH for 1Mbps low data rate service. Proposal 3. For the detection of SSBs in NR NTN, SSB combinations can be considered to improve the coverage performance. |
| Xiaomi | Observation 3: For PDCCH coverage enhancement, the possible solutions can be repetition, payload reduction and AL increasing. Observation 4: For common PDSCH coverage enhancement, a straightforward solution can be repetition. |
| Google | Proposal 1: Support to indicate the EPRE ratio between the PDCCH and TRS to facilitate the AGC for PDCCH reception. Proposal 2: Consider the PDCCH repetition operations for some PDCCH candidates in a search space for PDCCH coverage enhancement. Proposal 3: Support to indicate the power offset between PDSCH and TRS by DCI with regard to dynamic power sharing. Proposal 4: Compared to the coverage enhancement for PDSCH scheduled by fallback DCI, the coverage enhancement for PDSCH scheduled by DCI format 1_1/1_2/1_3 should be deprioritized. Proposal 5: Support to transmit the SSB by multiple repetitions - At least the number of repetitions for SSS and PBCH within an SSB should be the same |
| Oppo | Proposal 3: RAN1 to consider link-level enhancement of PDCCH and Msg4 PDSCH. Observation 4: For LEO600 set1-1 and set1-2, there is no coverage issue for DL physical channels and at least 2.6 dB link margin is obtained. Observation 5: For LEO600 set1-3, PDCCH and Msg4 PDSCH present 4.5 dB and 5.4 dB coverage performance gap respectively to be compensated, so link level enhancements are needed. |
| NTT DOCOMO | Proposal 2: - For discussion on NTN DL coverage enhancement, prioritize Set 1-1 FR1. Proposal 5: - R19 NTN DL coverage enhancement introduces link level enhancements for: - Msg2 PDSCH; Msg4 PDSCH; SIB1 PDSCH; SIB19 PDSCH; Paging PDSCH; PDCCH Proposal 6: - R19 NTN DL coverage enhancement introduced for SIB1 / SIB19 is not restricted to SIB1 / SIB19 only. |
| Qualcomm | Proposal 1: Consider coverage enhancements for PDSCH schedule by a PDCCH in a CSS. - At least for PDSCH scheduled by PDCCH format 1_0 with CRC scrambled by SI-RNTI. Proposal 2: Consider coverage enhancement for PDCCH. Proposal 3: Consider coverage enhancement for the PDCCH that schedules a SIB1 PDSCH without NW signaling. |
| CEWiT | Proposal 1: PDCCH with 8 CCE aggregations can provide sufficient link margin for set 1-1 and set 1-2 parameters. Hence, it is not necessary to do coverage enhancement for LEO 600 with set 1-1 and set 1-2 parameters. Proposal 2: Coverage improvement is needed for set 1-3 for LEO 600 due to a coverage gap of -4.52 dB observed in PDCCH with 8 CCE aggregations. Proposal 3: In the NTN DL coverage enhancement, link level improvement should be considered, considering reduced EIRP per beam for practical deployment case. Link level enhancement should target 5 dB improvement at least for the PDCCH channel. Proposal 4: The maximum number of repetitions for PDCCH can be extended beyond two for NR NTN DL coverage enhancements. Proposal 5: The higher aggregation level can be supported, such as 32 and 64, to improve the link margin of PDCCH for DL coverage enhancement of NR NTN. Proposal 6: For PDCCH, dynamic aggregation level per repetition can be configured to improve the DL coverage of NR NTN. Proposal 7: Each DCI format can be segmented depends on the size of the DCI to provide better coverage for NR NTN Proposal 8: Study the effect of choosing the number of repetitions, aggregation level and number of segmentations based on the elevation angle, for PDCCH, to improve both coverage and spectral efficiency of NR NTN. Proposal 9: Study the adoption of number of DMRS based on the elevation angle to improve the spectral efficiency in NR NTN. Proposal 10: The maximum number of repetitions can be increased to 16 or 32 to improve the coverage of NR NTN. |
| Nokia | Proposal 5: RAN1 to focus on PDSCH carrying SIB1 for DL coverage enhancements. Proposal 6: RAN1 to focus on PDSCH carrying Msg4 for DL coverage enhancements. Proposal 7: RAN1 to evaluate different approaches for improving DL coverage for PDSCH for initial access. Proposal 8: RAN 1 to focus on PDCCH for common channels (PDCCH Type 0/1) for DL coverage enhancements. Proposal 9: The methods of signaling for PDSCH conveying SIB1 repetition and time resource determination need further studies. Proposal 10: RAN1 to discuss solutions for UE to report assistant information for aiding gNB scheduling of Msg4 repetition factor. Proposal 11: For coverage improvement of common PDCCH (both Type-0 and Type-1), and to take backward compatibility into account, RAN1 considers time-domain repetitions as a potential solution. Proposal 12: Type-0 PDCCH repetitions may be performed inter-slot or intra-slot with repetitions factor configure/indicated via MIB. Proposal 13: For Type-1 PDCCH with CRC scrambled with RA-RNTI, repetition factor is configured or indicated via SIB1. |

## Initial proposals on potential enhancements at link level

### DL Coverage bottleneck channels

#### Proposal 4-1

Based on the companies’ proposals and observations the following working assumption is made:

Proposed working assumption 4-1-v0: RAN1 to consider link-level enhancements for at least the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB 1/SIB19.

Companies are encouraged to provide views on working assumption 4-1-v0:

| Companies | Comments |
| --- | --- |
| Qualcomm | OK |
| DCM | Generally fine. We would like to clarify that enhancement for SIB1/SIB19 may be applicable to any other SIBs. For example, if repetition is applicable to SIB1/SIB19, the same way should be applicable to e.g., SIB6, etc. No need to preclude it from specification. Our concern is, this proposal may imply that any other SIBs cannot be enhanced. But in actual system, some other SIBs also are necessary feature and thus sufficient coverage should be supported. |
| vivo | In our outstanding, though these channels suffer performance loss, it doesn’t mean that these channels need to be enhanced at link-level aspects. - For Msg4, the evaluated performance loss can be easily solved by enabling retransmission. - For SIB1and SIB19, the evaluated performance loss can be easily solved by repetition transmission or retransmission. - For PDCCH, the evaluated performance loss can be solved by a proper beam hopping pattern design at the system level aspects, e.g., in our T-doc, using four simultaneously activated beams targeting the same area in the beam hopping design to achieve 6dB gain. In other words, even when enhancements are deemed necessary, we don’t think link level enhancement should be prioritized. |
| Huawei, HiSilicon | We cannot achieve this proposal if the PSS detection performance in Set 1-3 is not justified. |
| TCL | Ok |
| Lenovo | Fine. |
| Panasonic | For Set 1-1, 1-2, enhancements are not required while for set 1-3 enhancements of PDCCH, PDSCH with msg4 and PDSCH with SIB1/19 are required. Whether to support set 1-3 needs to be discussed because the target SNR is very low (-9.9dB) and beam hopping might not work well with large number of repetitions which causes too long dwell time. |
| Xiaomi | It is better to clarify whether the PDCCH is common PDCCH or UE-specific PDCCH. |
| ETRI | Support |
| Ericsson | We are not sure how these channels are selected for coverage enhancement. In proposals 3-1, 3-4 and 3-5, it is written that there is no coverage gap for these channels for Set 1-1 and Set 1-2. On the other hand, it is proposed to consider these channels for coverage enhancement. |
| HONOR | OK |
| CATT | It is too early to conclude the link level enhancement before we finish the SSB evaluation. |
| Nokia | OK. But we would prefer if we also have a note that main emphasis should be on maintaining backwards compatibility with “legacy UEs” – at least for the channels that are of broadcast type.. |
| ZTE | Based on the inputs, there is no coverage gap for set 1-1 and set 1-2. For set 1-3, whether SSB can work is questionable. Hence, it’s too early to consider enhancing the listed channels as there is no reference SNR for link level enhancement. |
| ZTE | As commented for 4-1, it’s too early to consider detailed enhancements. |
| Apple | Support |

### DL Coverage enhancements techniques

#### Proposal 4-2

Based on the companies’ proposals and observations the following working assumption is made:

Proposed working assumption 4-2-v0: The following potential link-level enhancements techniques are for further study/evaluation: - To enhance the coverage of PDSCH Msg 4 and PDSCH carrying SIB1, consider introducing PDSCH repetition, with the maximum number of repetitions being at least X (X to be defined). - To enhance the coverage of PDCCH, consider the following solutions: - Increase CORESET symbol number and aggregation level. - Enhance PDCCH repetition (e.g., for PDCCH scheduling system information) - Reduce DCI size (e.g., 2-stage DCI)

Companies are encouraged to provide views on working assumption 4-2-v0:

| Companies | Comments |
| --- | --- |
| LGE | On the PDSCH for Msg3 or SIB1, we also consider to use the concept of scaling factor to reduce TB size for a given PRB number, and MCS. So, I’d like to list up the potential enhancement schemes for PDSCH as well. On increasing CORESET symbol duration, this approach may affect to the system-level evaluation as well since it will cause the increase on the PDCCH symbols and the decrease on the PDSCH symbols. Regarding “Reduce DCI size”, there would be two possibilities. One is that the UE will assume that the reserved fields are known bits when it decodes the DCI format. The other would be just reduce the input size of the Polar encoder. For the second case, it will definitely affect to the legacy UE and the existing DCI format budget. |
| Qualcomm | For the first bullet, propose to clarify “ To enhance the coverage of PDSCH scheduled by a PDCCH in a CSS” |
| DCM | Which type of enhancement is necessary should be discussed later. |
| vivo | As our comments in proposal 4-1, so far we do not see an urgent need for additional PDCCH enhancements because the coverage gap may be complemented through a proper system level design, or by other existing mechanisms. |
| TCL | This can be discussed after coverage bottleneck channel(s) are identified. |
| Lenovo | So far we think 2-stage DCI may have large spec impact and have error propagation issue. We prefer this aspect to be discussed later. |
| Panasonic | Agree with the proposal regarding the PDCCH should be enhanced. We agreed with LGE regarding the increase of CORESET symbol number, which the increase in CORESET size may increase the size of PDCCH and reduce the size of PDSCH, which will affect the system level evaluation. For PDCCH repetition, it can be enhanced for more repetition’s capability, based on the mechanism from Rel-17. For DCI reduction, the proposal should further clarify which DCI it needs to be reduced. DCI 0_0 and 1_0 should be excluded to keep backward combability |
| Xiaomi | We think it is too early to discuss the solutions. |
| Ericsson | We agree with the views expressed by multiple companies that to delay this proposal until we conclude the channels for enhancement. |
| HONOR | This can be discussed later. |
| Nokia | OK. But we would prefer if we also have a note that main emphasis should be on maintaining backwards compatibility with “legacy UEs” – at least for the channels that are of broadcast type.. |
| Apple | Support |

# Proposals for offline session on Tuesday

## Initial proposals on SLS results

Proposed observation 1:

Based on the results of DL coverage evaluation at system level collected from different sources, extending the default value of SSB periodicity (different from 20ms) is beneficial in NTN implementing beam hopping. The benefits of such extension as observed by several companies are:

- Reduction of common control channel overhead:

  - With Set 1-1 FR1 and Set 1-3 FR1, the ratio of common message (SSB, SIB1, SIB19) is round 40% when SSB periodicity of 20ms is in use, this ratio could be reduced to round 5% when 160ms SSB periodicity is used.

  - Set1-2 FR1 with a ratio of simultaneously active beams of 1,5% requires SSB periodicity of at least 80ms. That is, a default periodicity of 20ms is not doable for such deployment scenario.

- The UPT for traffic type of VoIP, IM and FTP3 is improved,

- The cell throughput for IM and FTP is improved.

Proposed observation 2:

It is observed that extended SSB periodicity increases the coverage ratio and thereby improves system level coverage for all the three LEO600km satellite parameter sets:

- For set 1-1/1-3, the coverage ratio (N2+N3/ total beam footprints) can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms,

- For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms.

## Initial proposals on LLS results

Based on the preliminary results collected in the previous section, the following proposed observations are made:

### PDCCH coverage evaluation

#### Proposal 3-1

Proposed Observation 3-1-v0: Based on LLS results on PDCCH coverage evaluation collected from different sources: - It is observed that the required SNR for PDCCH is in average equal to -6dB (17 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - 17 sources observed that there is no coverage gap with Set1-1/1-2 FR1. - The coverage margin is round 4 dB. - With parameter LEO600km Set1-3: - 15 sources observed that there is a PDCCH coverage gap of 3.9dB in average.

### PDSCH Msg2 overage evaluation

#### Proposal 3-2

Proposed Observation 3-2-v0: Based on LLS results on PDSCH Msg2 coverage evaluation collected from different sources: - It is observed that the required SNR for PDSCH carrying Msg2 is in average equal to – 10.9 dB (14 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - 14 sources observed that there is no coverage gap for PDSCH with Msg2: - The coverage margin is round 9 dB - With parameter LEO600km Set1-3 FR1: - 1 source observed that there is a coverage gap of 1.6 dB for PDSCH with Msg2 - 11 sources observed that there is no coverage gap for PDSCH with Msg2: - The coverage margin is round 2 dB

### PDSCH Msg4 overage evaluation

#### Proposal 3-3

Proposed Observation 3-3-v1: Based on LLS results on PDSCH Msg4 coverage evaluation collected from different sources: - It is observed that the required SNR for PDSCH carrying Msg4 is in average equal to – 5.2 dB (14 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - 14 sources observed that there is no coverage gap for PDSCH with Msg4: - The coverage margin is round 3.3 dB - With parameter LEO600km Set1-3 FR1: - 11 sources observed that there is a coverage gap for PDSCH with Msg4: - The coverage gap is round 4.8 dB - 1 source observed that there is no coverage gap for PDSCH with Msg4 with a coverage margin of 0.3 dB

### PDSCH SIB1 coverage evaluation

#### Proposal 3-4

Proposed Observation 3-4-v0: Based on LLS results on PDSCH SIB1 coverage evaluation collected from different sources: - For PDSCH carrying SIB1 option 1 (with a payload size of 800bits) it is observed that the required SNR is in average equal to – 5.8 dB (14 sources) - For PDSCH carrying SIB1 option 2 (with a payload size of 1280bits) it is observed that the required SNR is in average equal to – 3.4 dB (12 sources) - With parameter LEO600km Set1-1 FR1 and 1-2FR1: - 14 sources observed that there is no coverage gap for PDSCH with SIB1 option 1: - The coverage margin is round 3.9 dB - 12 sources observed that there is no coverage gap for PDSCH with SIB1 option 2: - The coverage margin is round 1.5 dB - With parameter LEO600km Set1-3 FR1: - 11 sources observed that there is a coverage gap for PDSCH with SIB1 option 1: - The coverage gap is round 4.5 dB - 1 source observed that there is no coverage gap for PDSCH with SIB1 option 1: - The coverage margin is 3.4 dB - 10 sources observed that there is a coverage gap for PDSCH with SIB1 option 2: - The coverage gap is round 6.3 dB

### PDSCH SIB19 coverage evaluation

#### Proposal 3-5

Proposed Observation 3-5-v0: Based on LLS results on PDSCH SIB19 coverage evaluation collected from different sources: - It is observed that the required SNR for PDSCH carrying SIB19 is in average equal to – 6.9 dB (14 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - 12 sources observed that there is no coverage gap for PDSCH with SIB19: - The coverage margin is round 4.2 dB - With parameter LEO600km Set1-3 FR1: - 10 sources observed that there is a coverage gap for PDSCH with SIB19: - The coverage gap is round 3.5 dB

### PDSCH VoIP coverage evaluation

#### Proposal 3-6

Proposed Observation 3-6-v0: Based on LLS results on PDSCH for VoIP coverage evaluation collected from different sources: - It is observed that the required SNR for PDSCH for VoIP is in average equal to – 11 dB (11 sources) - With parameter LEO600km Set1-1 and 1-2 FR1: - When PDSCH repetition is enabled, 11 sources observed that there is no coverage gap for VoIP: - The coverage margin is round 9 dB - With parameter LEO600km Set1-3 FR1: - When PDSCH repetition is enabled, 9 sources observed that there is no coverage gap for VoIP: - The coverage margin is round 2.3 dB - 1 source observed that even with 8 PDSCH repetitions there is a coverage gap of 1.5 dB

### PDSCH Low data rate coverage evaluation

#### Proposal 3-7

Proposed Observation 3-7-v0: Based on LLS results on PDSCH 3kbps coverage evaluation collected from different sources: - It is observed that the required SNR for PDSCH for low data rate is in average equal to – 11 dB (8 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - When PDSCH repetition is enabled, 8 sources observed that there is no coverage gap for PDSCH with 3kbp: - The coverage margin is round 8.8 dB - With parameter LEO600km Set1-3 FR1: - When PDSCH repetition is enabled, 6 sources observed that there is no coverage gap for PDSCH with 3kbp: - The coverage margin is round 1.6 dB

### PDSCH 1Mbps coverage evaluation

#### Proposal 3-8

Proposed Observation 3-8-v0: Based on LLS results on PDSCH 1Mbps coverage evaluation collected from different sources: - It is observed that the required SNR for PDSCH with 1Mbps data rate is in average equal to – 4.1 dB (7 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - 7 sources observed that there is no coverage gap for PDSCH with 1Mbps: - The coverage margin is round 2.2 dB - With parameter LEO600km Set1-3 FR1: - 5 sources observed that, there is a coverage gap for PDSCH with 1Mbps: - The coverage gap is round 5.5 dB

### SSB coverage evaluation

#### Proposal 3-9

Proposed Observation 3-9-v0: Based on LLS results on SSB coverage evaluation collected from different sources: - With parameter LEO600km Set1-1 and 1-2, the required SNR for SSB detection is ranging from -10.8dB to -3.6dB and thereby no coverage gap is observed for LEO600km Set1-1 and 1-2. - With parameter LEO600km Set1-3: - 7 sources observed that with 4 SSB combinations the SSB can be successfully detected with a margin ranging from 0.2dB to 6.68dB - 2 sources observed that there is a coverage gap of 3.4 dB and 0.78dB respectively even with 4 SSB combinations. - 2 sources observed that there is a coverage gap of 2.95 dB and 1.8dB respectively only with a single SSB detection.

# Proposals for online session on Tuesday

Proposed Observation 3-1-v0:

Based on LLS results on PDCCH coverage evaluation collected from different sources:

- It is observed that the required SNR for PDCCH is in average equal to -6dB (17 sources)

  - With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 17 sources observed that there is no coverage gap with Set1-1/1-2 FR1.

      - The coverage margin is round 4 dB.

  - With parameter LEO600km Set1-3 FR1:

  - 15 sources observed that there is a PDCCH coverage gap of 3.9dB in average.

Proposed Observation 3-2-v0:

Based on LLS results on PDSCH Msg2 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying Msg2 is in average equal to – 10.9 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 14 sources observed that there is no coverage gap for PDSCH with Msg2:

      - The coverage margin is round 9 dB

- With parameter LEO600km Set1-3 FR1:

  - 1 source observed that there is a coverage gap of 1.6 dB for PDSCH with Msg2

  - 11 sources observed that there is no coverage gap for PDSCH with Msg2:

      - The coverage margin is round 2 dB

Proposed observation 1:

Based on the results of DL coverage evaluation at system level collected from different souorces, extending the default value of SSB periodicity (different from 20ms) is beneficial in NTN implementing beam hopping. The benefits of such extension as observed in 8 sources are:

- Reduction of common control channel overhead:

  - With Set 1-1 FR1 and Set 1-3 FR1, the ratio of common message (SSB, SIB1, SIB19) is round 40% when SSB periodicity of 20ms is in use, this ratio could be reduced to round 5% when 160ms SSB periodicity is used.

  - Set1-2 FR1 with a ratio of simultaneously active beams of 1,5% requires SSB periodicity of at least 80ms. That is, a default periodicity of 20ms is not doable for such deployment scenario.

- The UPT for traffic type of VoIP, IM and FTP3 is improved,

- The cell throughput for IM and FTP is improved.

Proposed observation 2:

Based on the results of DL coverage evaluation at system level collected from 6 sources, it is observed that extended SSB periodicity increases the coverage ratio and thereby improves system level coverage for all the three LEO600km satellite parameter sets:

- For set 1-1/1-3, the coverage ratio (N2+N3/ total beam footprints) can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms,

- For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms.

Proposed Observation 3-3-v1:

Based on LLS results on PDSCH Msg4 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying Msg4 is in average equal to – 5.2 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 14 sources observed that there is no coverage gap for PDSCH with Msg4:

      - The coverage margin is round 3.3 dB

- With parameter LEO600km Set1-3 FR1:

  - 11 sources observed that there is a coverage gap for PDSCH with Msg4:

    - The coverage gap is round 4.8 dB

  - 1 source observed that there is no coverage gap for PDSCH with Msg4 with a coverage margin of 0.3 dB

Proposed Observation 3-4-v0:

Based on LLS results on PDSCH SIB1 coverage evaluation collected from different sources:

- For PDSCH carrying SIB1 option 1 (with a payload size of 800bits) it is observed that the required SNR is in average equal to – 5.8 dB (14 sources)

- For PDSCH carrying SIB1 option 2 (with a payload size of 1280bits) it is observed that the required SNR is in average equal to – 3.4 dB (12 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2FR1:

  - 14 sources observed that there is no coverage gap for PDSCH with SIB1 option 1:

      - The coverage margin is round 3.9 dB

  - 12 sources observed that there is no coverage gap for PDSCH with SIB1 option 2:

      - The coverage margin is round 1.5 dB

- With parameter LEO600km Set1-3 FR1:

  - 11 sources observed that there is a coverage gap for PDSCH with SIB1 option 1:

      - The coverage gap is round 4.5 dB

  - 1 source observed that there is no coverage gap for PDSCH with SIB1 option 1:

      - The coverage margin is 3.4 dB

  - 10 sources observed that there is a coverage gap for PDSCH with SIB1 option 2:

      - The coverage gap is round 6.3 dB

Proposed Observation 3-5-v0:

Based on LLS results on PDSCH SIB19 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying SIB19 is in average equal to – 6.9 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 12 sources observed that there is no coverage gap for PDSCH with SIB19:

      - The coverage margin is round 4.2 dB

- With parameter LEO600km Set1-3 FR1:

  - 10 sources observed that there is a coverage gap for PDSCH with SIB19:

    - The coverage gap is round 3.5 dB

Proposed Observation 3-6-v0:

Based on LLS results on PDSCH for VoIP coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for VoIP is in average equal to – 11 dB (11 sources)

- With parameter LEO600km Set1-1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 11 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is round 9 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 9 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is round 2.3 dB

  - 1 source observed that even with 8 PDSCH repetitions there is a coverage gap of 1.5 dB

Proposed Observation 3-7-v0:

Based on LLS results on PDSCH 3kbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for low data rate is in average equal to – 11 dB (8 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 8 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is round 8.8 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 6 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is round 1.6 dB

Proposed Observation 3-8-v0:

Based on LLS results on PDSCH 1Mbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH with 1Mbps data rate is in average equal to – 4.1 dB (7 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - 7 sources observed that there is no coverage gap for PDSCH with 1Mbps:

  - The coverage margin is round 2.2 dB

- With parameter LEO600km Set1-3 FR1:

  - 5 sources observed that, there is a coverage gap for PDSCH with 1Mbps:

  - The coverage gap is round 5.5 dB

Proposed Observation 3-9-v0:

Based on LLS results on SSB coverage evaluation collected from different sources:

- With parameter LEO600km Set1-1 and 1-2, the required SNR for SSB detection is ranging from -10.8dB to -3.6dB and thereby no coverage gap is observed for LEO600km Set1-1 and 1-2.

- With parameter LEO600km Set1-3:

  - 7 sources observed that with 4 SSB combinations the SSB can be successfully detected with a margin ranging from 0.2dB to 6.68dB

  - 2 sources observed that there is a coverage gap of 3.4 dB and 0.78dB respectively even with 4 SSB combinations.

  - 2 sources observed that there is a coverage gap of 2.95 dB and 1.8dB respectively only with a single SSB detection.

# Proposals for offline session on Wednesday

Proposed observation 1:

Based on the results of DL coverage evaluation at system level collected from different sources, extending the default value of SSB periodicity (different from 20ms) is beneficial in NTN implementing beam hopping. The benefits of such extension as observed in 8 sources are:

- Reduction of common control channel overhead:

  - With Set 1-1 FR1 and Set 1-3 FR1, the ratio of common message (SSB, SIB1, SIB19) is around 40% assuming 5 MHz BW when SSB periodicity of 20ms is in use, this ratio could be reduced to around 5% when 160ms SSB periodicity is used.

  - Set1-2 FR1 with a ratio of simultaneously active beams of 1,5% requires SSB periodicity of at least 80ms. That is, a default periodicity of 20ms is not doable for such deployment scenario.

[Note: increase SSB periodicity will lead to in crease of initial access delay and degradation of the performance of DL reception and of uplink signal integrity]

- The UPT for traffic type of VoIP, IM and FTP3 is improved,

- The cell throughput for IM and FTP is improved.

Proposed observation 2:

Based on the results of DL coverage evaluation at system level collected from 6 sources, it is observed that extended SSB periodicity increases the coverage ratio and thereby improves system level coverage for all the three LEO600km satellite parameter sets:

- For set 1-1/1-3, the coverage ratio (N2+N3/ total beam footprints) can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms,

- For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms.

Proposed Observation 3-3-v1:

Based on LLS results on PDSCH Msg4 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying Msg4 is in average equal to – 5.2 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 14 sources observed that there is no coverage gap for PDSCH with Msg4:

      - The coverage margin is round 3.3 dB

- With parameter LEO600km Set1-3 FR1:

  - 11 sources observed that there is a coverage gap for PDSCH with Msg4:

    - The coverage gap is round 4.8 dB

  - 1 source observed that there is no coverage gap for PDSCH with Msg4 with a coverage margin of 0.3 dB

Proposed Observation 3-4-v0:

Based on LLS results on PDSCH SIB1 coverage evaluation collected from different sources:

- For PDSCH carrying SIB1 option 1 (with a payload size of 800bits) it is observed that the required SNR is in average equal to – 5.8 dB (14 sources)

- For PDSCH carrying SIB1 option 2 (with a payload size of 1280bits) it is observed that the required SNR is in average equal to – 3.4 dB (12 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2FR1:

  - 14 sources observed that there is no coverage gap for PDSCH with SIB1 option 1:

      - The coverage margin is round 3.9 dB

  - 12 sources observed that there is no coverage gap for PDSCH with SIB1 option 2:

      - The coverage margin is round 1.5 dB

- With parameter LEO600km Set1-3 FR1:

  - 11 sources observed that there is a coverage gap for PDSCH with SIB1 option 1:

      - The coverage gap is round 4.5 dB

  - 1 source observed that there is no coverage gap for PDSCH with SIB1 option 1:

      - The coverage margin is 3.4 dB

  - 10 sources observed that there is a coverage gap for PDSCH with SIB1 option 2:

      - The coverage gap is round 6.3 dB

Proposed Observation 3-5-v0:

Based on LLS results on PDSCH SIB19 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying SIB19 is in average equal to – 6.9 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 12 sources observed that there is no coverage gap for PDSCH with SIB19:

      - The coverage margin is round 4.2 dB

- With parameter LEO600km Set1-3 FR1:

  - 10 sources observed that there is a coverage gap for PDSCH with SIB19:

    - The coverage gap is round 3.5 dB

Proposed Observation 3-6-v0:

Based on LLS results on PDSCH for VoIP coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for VoIP is in average equal to – 11 dB (11 sources)

- With parameter LEO600km Set1-1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 11 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is round 9 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 9 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is round 2.3 dB

  - 1 source observed that even with 8 PDSCH repetitions there is a coverage gap of 1.5 dB

Proposed Observation 3-7-v0:

Based on LLS results on PDSCH 3kbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for low data rate is in average equal to – 11 dB (8 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 8 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is round 8.8 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 6 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is round 1.6 dB

Proposed Observation 3-8-v0:

Based on LLS results on PDSCH 1Mbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH with 1Mbps data rate is in average equal to – 4.1 dB (7 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - 7 sources observed that there is no coverage gap for PDSCH with 1Mbps:

  - The coverage margin is round 2.2 dB

- With parameter LEO600km Set1-3 FR1:

  - 5 sources observed that, there is a coverage gap for PDSCH with 1Mbps:

  - The coverage gap is round 5.5 dB

Proposed Observation 3-9-v0:

Based on LLS results on SSB coverage evaluation collected from different sources:

- With parameter LEO600km Set1-1 and 1-2, the required SNR for SSB detection is ranging from -10.8dB to -3.6dB and thereby no coverage gap is observed for LEO600km Set1-1 and 1-2.

- With parameter LEO600km Set1-3:

  - 7 sources observed that with 4 SSB combinations the SSB can be successfully detected with a margin ranging from 0.2dB to 6.68dB

  - 2 sources observed that there is a coverage gap of 3.4 dB and 0.78dB respectively even with 4 SSB combinations.

  - 2 sources observed that there is a coverage gap of 2.95 dB and 1.8dB respectively only with a single SSB detection.

# Proposals for online session on Thursday

Proposed observation 1:

Based on the results of DL coverage evaluation at system level collected from different sources, extending the default value of SSB periodicity (different from 20ms) is beneficial in NTN implementing beam hopping. The benefits of such extension as observed in 8 sources are:

- Reduction of common control channel overhead:

  - With Set 1-1 FR1 and Set 1-3 FR1, the ratio of common message (SSB, SIB1, SIB19) is around 40% assuming 5 MHz BW when SSB periodicity of 20ms is in use, this ratio could be reduced to around 5% when 160ms SSB periodicity is used.

  - Set1-2 FR1 with a ratio of simultaneously active beams of 1,5% requires SSB periodicity of at least 80ms. That is, a default periodicity of 20ms is not doable for such deployment scenario.

[Note: Increasing SSB periodicity will lead to increase of initial access delay and may introduce degradation of the performance of DL reception and of uplink signal integrity]

- The UPT for traffic type of VoIP, IM and FTP3 is improved,

- The cell throughput for IM and FTP is improved.

Companies are encouraged to provide views on Proposed observation 1:

| Companies | Comments |
| --- | --- |
| LGE | It is unclear to us that some observations are made for extending SSB periodicity which definitely needs a confirmation from the plenary meeting for whether the extending SSB periodicity is a scope of WID or not. Note that there was no consensus at the last RAN1 meeting for this issue. We are fine to make observations only for the SSB periodicity of 20msec. Meanwhile, according to WID, wide beam technique is explicitly mentioned. Moreover, in this meeting, we saw some analysis for the wide beam technique as well. In those points of views, it seems necessary to have observations for the wide beam technique. Our suggestions for wide beam technique are as follows: Based on the results of DL coverage evaluation at system level collected from different sources, - For wide beam footprint covering N narrow beam footprints, followings are observed: - Reduction of common control channel overhead: - With Set 1-1 FR1 and Set 1-3 FR1, the ratio of common message (SSB, SIB1, SIB19) could be reduced by 1/N when wide beam footprints are used. - Set1-2 FR1 with a ratio of simultaneously active beams of 1,5% requires wide beam footprints covering at least 4 narrow beam footprints. |
| vivo | We agree with LGE that the observation of wide beam mechanism should also be captured. Regarding the proposed observation-1, for the 1st subbullet - With Set 1-1 FR1 and Set 1-3 FR1, the ratio of common message (SSB, SIB1, SIB19) is around 40% assuming 5 MHz BW when SSB periodicity of 20ms is in use, this ratio could be reduced to around 5% when 160ms SSB periodicity is used. We are not sure how come the highlighted number of 5%. Based on our analysis, it is around 14%. It should be updated either as a range [5, 14], or as an average number, e.g., 10%. For the 2nd subbullet - Set1-2 FR1 with a ratio of simultaneously active beams of 1,5% requires SSB periodicity of at least 80ms. That is, a default periodicity of 20ms is not doable for such deployment scenario. The sentence of “That is, a default periodicity of 20ms is not doable for such deployment scenario” should be reomved from the 2nd sub-bullet. This might be the inference from some companies but not the common fact observed by RAN1. |
| MediaTek | We have similar views as LG and Vivo on wide beam observation can be captured. Another bullet could be captured - For wide beam covering 4 narrow beams with NSSB=4 Beam hopping, Set 1-2 with SSB periodicity of 80 ms can provide coverage of 96.8% and efficiency per beam of 96.9%. |
| Ericsson | We sincerely thank the FL for the introduction of our system-level evaluations that were not included in the previous round in the FL summary. However, the current proposal does not capture accurate enough the system-level evaluations presented in Section 1.2, including the observations gleaned from our results. Hence, we are not supportive of this proposal in its current form. According to the available results, there are at least the following ways of implementation: 1) Utilize the fixed satellite beam size of 50 km, but with increased SSB periodicity to facilitate the larger revisit time and consequently the increased dwell time for beam illumination compared to the default SSB periodicity of 20 msec. 2) Beam layout is chosen such that each beam covers the larger cell area on the earth (compared to that of beam size of 50 km) such that the required number of beam footprints under the satellite footprint are reduced, which also facilitates the reasonable dwell times for beam illumination at least for certain settings. Note that both implementations are feasible according to the agreements made in the previous RAN1 meetings. Hence, we would like to revise the proposal as Proposed observation 1: - Based on the results of DL coverage evaluation at system level collected from different sources, extending the default value of SSB periodicity (different from 20ms) is beneficial in NTN implementing beam hopping. The benefits of such extension as observed in 8 sources are: is beneficial in terms of reduction of common control channel overhead according to 8 sources: - Reduction of common control channel overhead: - With Set 1-1 FR1 and Set 1-3 FR1, the ratio of common message (SSB, SIB1, SIB19) is around 40% assuming 5 MHz BW when SSB periodicity of 20ms is in use, this ratio could be reduced to around 5% when 160ms SSB periodicity is used. - Set1-2 FR1 with a ratio of simultaneously active beams of 1,5% requires SSB periodicity of at least 80ms. That is, a default periodicity of 20ms is not doable for such deployment scenario. Aspects that require to be further investigated are: specification impacts, impact on initial access delay, backward compatibility with legacy UEs, SSB detection at the sync-raster points, MIB periodicity, PBCH combining, performance Loss/Gain. - [X] sources observed, for Set 1-1, that the legacy SSB periodicity of 20ms during initial access is usable with NTN beam hopping based on choosing the beam layout such that each beam covers the larger cell area on the earth. [Note: Increasing SSB periodicity will lead to increase of initial access delay and may introduce degradation of the performance of DL reception and of uplink signal integrity] |
| Huawei, HiSilicon | - Regarding the second sub-bullet, there are some comments from vivo. We think it would be better to focus on the common channel overhead reduction itself. Therefore, we propose the following update on the second sub-bullet; - For the concern on the DL reception performance/uplink signal integrity due to the timing/frequency drift when larger SSB periodicity is used, as we commented in the morning, we have the following feedbacks: - For the PDCCH and PDSCH reception before the SIB19 acquisition: the PDCCH and PDSCH for SIB1 and SIB19 are scheduled within the same 10ms time duration as the SSB, in the analysis. Therefore, the performance of PDCCH and PDSCH has almost no impact due to the frequency and timing drift; - For the PDCCH and PDSCH after the SIB19 acquisition: the UE can obtain the doppler frequency offset and timing drift based on the ephemeris information from SIB19 and the GNSS information. This shall be anyway done for the NTN UE for uplink transmission. Therefore, the residual frequency offset and timing drift after the compensation shall be limited. Therefore, the performance loss simulated in our contribution [R1-2403938] is marginal. - Furthermore, TRS can be used anyway for timing/frequency synchronization if needed. Based on the above reasons, we think the potential impact on the performance of DL reception and of uplink signal integrity due to the timing/frequency drift can be mitigated by other implementations in NTN. Therefore, we propose some update on the note. - With respect to the impact on the legacy UEs, we have already provided the comments to resolve the concerns in offline discussion in this morning, which were also echoed by satellite operators. In the beam footprints which can be accessed in Rel-18 baseline, the 20ms SSB periodicity can be transmitted still. The legacy UEs can be still served. For other beam footprints, there is anyway no UEs served in the Rel-18 baseline. So, the legacy UEs are not impacted if the SSB periodicity is extended in these beam footprints. - For the initial access latency, similarly, the access latency in the beam footprints which are illuminated/served in Rel-18 baseline shall not be impacted because 20ms SSB periodicity shall be still transmitted in these beam footprints. For other beam footprints, there is no chance for NTN UE to access in these beam footprints in Rel-18 baseline, therefore, there shall be no initial access latency impact in these beam footprints. - For wider beam solutions, we think it should be a separate observation because the observation here focuses on the SSB periodicity increasement. Based on the above reasons and some offline discussions, we propose the following updates in purple: Proposed observation 1: Based on the results of DL coverage evaluation at system level collected from different sources, extending the default value of SSB periodicity (different from 20ms) is beneficial in NTN implementing beam hopping. The benefits of such extension as observed in 8 sources are: - Reduction of common control channel overhead: - With Set 1-1 FR1 and Set 1-3 FR1, the ratio of common message (SSB, SIB1, SIB19) is around 40% assuming 5 MHz BW when SSB periodicity of 20ms is in use, this ratio could be reduced to around 5% when 160ms SSB periodicity is used. - With Set 1-2FR1, the ratio of common message (SSB, SIB1, SIB19) is around 50% assuming 5 MHz BW when SSB periodicity of 20ms is in use, this ratio could be reduced to around 25.8% when 640ms SSB periodicity is used. - Set1-2 FR1 with a ratio of simultaneously active beams of 1,5% requires SSB periodicity of at least 80ms. That is, a default periodicity of 20ms is not doable for such deployment scenario. [Note: Increasing SSB periodicity will lead to increase of initial access delay and may introduce degradation of the performance of DL reception and of uplink signal integrity] [Note: The potential impact on the performance of DL reception and of uplink signal integrity due to the timing/frequency drift can be mitigated by other implementations in NTN] - The UPT for traffic type of VoIP, IM and FTP3 is improved, - The cell throughput for IM and FTP is improved. |
| CATT | For the wording change from Huawei, we support it because of clearer description. Regarding some technical points from other companies, we have the following comments: - If using wide beam for evaluation, the EIRP per beam decrease should be mentioned. The SINR target should be guaranteed at least. Meanwhile, it is one separate enhanced scheme, which can be formulated with another observation. - For the backward compatibility issue with legacy UEs, it depends on the deployment. In some beams, 20ms SSB periodicity can be kept, and some beams can have large SSB periodicity. Any new features would have the problem to support legacy UE. In another aspect, Rel-19 NTN can be chosen by the satellite operator as one starting point, and actually we didn’t find any commercial network has already deployed the R17 or R18 NTN. - For the latency issue, there are many factors to be taken into account. For example, the propagation delay between UE and satellite, the beam sweeping pattern and system information transmission ways all impact the latency. Based on the PRACH reception, gNB can schedule one active beam to speed up the initial access. We can’t say SSB periodicity will always increase the latency. |
| ZTE | Agree to achieve the observation. And we have following further comments: - Regarding wide beam, we think it is another potential solution to improve the system level performance, which is parallel with extending SSB periodicity. If wide beam is applied, the EIRP will degrade, which means a different assumption of satellite parameter. To avoid mixing the result of different solutions and satellite assumptions, we think it is better to only focus on SSB periodicity extension in this observation. The wide beam solution can be considered separately. - Regarding the note, we think that the impact of timing and frequency drift can be mitigated by implementations. Hence, there will not always be performance degradation. Moreover, the access delay is due to the application of beam hopping instead of extending periodicity. If the revisit time of beam hopping need to be larger than 20ms to cover enough area, the access delay is inevitable no matter whether the default SSB periodicity is extended or not. Therefore, the note should be updated. |

Proposed observation 2:

Based on the results of DL coverage evaluation at system level collected from 6 sources, it is observed that extended SSB periodicity increases the coverage ratio and thereby improves system level coverage for all the three LEO600km satellite parameter sets:

- For set 1-1/1-3, the coverage ratio (N2+N3/ total beam footprints) can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms,

- For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms.

Companies are encouraged to provide views on Proposed observation 2:

| Companies | Comments |
| --- | --- |
| LGE | It is unclear to us that some observations are made for extending SSB periodicity which definitely needs a confirmation from the plenary meeting for whether the extending SSB periodicity is a scope of WID or not. Note that there was no consensus at the last RAN1 meeting for this issue. We are fine to make observations only for the SSB periodicity of 20msec. Meanwhile, according to WID, wide beam technique is explicitly mentioned. Moreover, in this meeting, we saw some analysis for the wide beam technique as well. In those points of views, it seems necessary to have observations for the wide beam technique. Our suggestions for wide beam technique are as follows: Based on the results of DL coverage evaluation at system level collected from different sources, - For wide beam footprint covering N narrow beam footprints - For set 1-1/1-3, the coverage ratio (N2+N3/ total beam footprints) can be improved from 40% to 100% if the number of narrow beam footprints covered by a wide beam footprint is increased from 1 to 4, - For Set 1-2, the coverage ratio can be improved from 6% to 24.2% if the number of narrow beam footprints covered by a wide beam footprint is increased from 1 to 4. |
| Vivo | We agree with LGE that the observation of wide beam mechanism should also be captured. Moreover, increasing the coverage ratio does not direclty mean the system level coverage is improved, where the latter one depends on many other aspects. We suggest to remove the “and thereby improves system level coverage” from the main bullet, just focus on the observation of coverage ratio. For the case of 20ms SSB periodicity, it is not clear why only 10% and 1.5% is captured. At least in our analysis, the ratio can be much higher. |
| Ericsson | Similar comments as Proposed observation 1 are applicable. We would like to propose the following revision to the proposal: Proposed observation 2: - Based on the results of DL coverage evaluation at system level collected from 6 sources, it is observed that extended SSB periodicity increases the coverage ratio and thereby improves system level coverage for all the three LEO600km satellite parameter sets: - For set 1-1/1-3, the coverage ratio (N2+N3/ total beam footprints) can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms, - For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms. - [X] sources observed that the legacy SSB periodicity of 20ms during initial access is usable with NTN beam hopping based on choosing the beam layout such that each beam covers the larger cell area on the earth: - For set 1-1, the coverage ratio (N2+N3/ total beam footprints) can be achieved at least in the range of 80% to 90% with the default SSB periodicity of 20 ms. |
| Huawei, HiSilicon | We support the observation. Regarding the comments about the wider beam solution, it should be a separate observation. |
| CATT | We support this observation. For system level enhancement, the coverage ratio improvement is one very essential target. Since the limited satellite number and limited satellite beam per satellite, increasing the coverage ratio is very necessary to satellite operator. |
| ZTE | Agree with the observation. Regarding wide beam, as commented for proposed observation 1, can be considered separately. |

Proposed Observation 3-4-v1:

Based on LLS results on PDSCH SIB1 coverage evaluation collected from different sources:

- For PDSCH carrying SIB1 option 1 (with a payload size of 800bits) it is observed that the required SNR is in average equal to – 5.8 dB (14 sources)

- For PDSCH carrying SIB1 option 2 (with a payload size of 1280bits) it is observed that the required SNR is in average equal to – 3.4 dB (12 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2FR1:

  - 14 sources observed that there is no coverage gap for PDSCH with SIB1 option 1:

      - The coverage margin is around 3.9 dB compared to CNR of -1.9 dB

  - 12 sources observed that there is no coverage gap for PDSCH with SIB1 option 2:

      - The coverage margin is around 1.5 dB compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 11 sources observed that there is a coverage gap for PDSCH with SIB1 option 1:

      - The coverage gap is around 4.1 dB compared to CNR of -9.9 dB

  - 1 source observed that there is no coverage gap for PDSCH with SIB1 option 1:

      - The coverage margin is 3.4 dB compared to CNR of -9.9 dB

  - 10 sources observed that there is a coverage gap for PDSCH with SIB1 option 2:

      - The coverage gap is around 6.5 dB compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB1.

Companies are encouraged to provide views on Proposed Observation 3-4-v1:

| Companies | Comments |
| --- | --- |

Proposed Observation 3-5-v1:

Based on LLS results on PDSCH SIB19 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying SIB19 is in average equal to – 6.9 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 12 sources observed that there is no coverage gap for PDSCH with SIB19:

      - The coverage margin is around 4.2 dB compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 10 sources observed that there is a coverage gap for PDSCH with SIB19:

      - The coverage gap is around 3.5 dB compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB19.

Companies are encouraged to provide views on Proposed Observation 3-5-v1:

| Companies | Comments |
| --- | --- |

Proposed Observation 3-6-v1:

Based on LLS results on PDSCH for VoIP coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for VoIP is in average equal to – 11 dB (11 sources)

- With parameter LEO600km Set1-1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 11 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is around 9.1 dB compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 9 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is around 2.3 dB compared to CNR of -9.9 dB

  - 1 source observed that even with 8 PDSCH repetitions there is a coverage gap of 1.5 dB compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH for VoIP.

Companies are encouraged to provide views on Proposed Observation 3-6-v1:

| Companies | Comments |
| --- | --- |

Proposed Observation 3-7-v1:

Based on LLS results on PDSCH 3kbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for low data rate is in average equal to – 11 dB (8 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 8 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is around 9.1 dB compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 6 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is around 1.6 dB compared to CNR of -9.9 dB

Companies are encouraged to provide views on Proposed Observation 3-7-v1:

| Companies | Comments |
| --- | --- |

Proposed Observation 3-8-v0:

Based on LLS results on PDSCH 1Mbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH with 1Mbps data rate is in average equal to – 4.1 dB (7 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - 7 sources observed that there is no coverage gap for PDSCH with 1Mbps:

  - The coverage margin is around 2.2 dB compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 5 sources observed that, there is a coverage gap for PDSCH with 1Mbps:

  - The coverage gap is round 5.5 dB compared to CNR of -9.9 dB

Companies are encouraged to provide views on Proposed Observation 3-8-v1:

| Companies | Comments |
| --- | --- |

Proposed Observation 3-9-v1:

Based on LLS results on SSB coverage evaluation collected from different sources:

- With parameter LEO600km Set1-1 and 1-2, the required SNR for SSB detection is ranging from -10.8dB to -3.6dB and thereby no coverage gap is observed for LEO600km Set1-1 and 1-2.

- With parameter LEO600km Set1-3:

  - 7 sources observed that with 4 SSB combinations the SSB can be successfully detected with a margin ranging from 0.2dB to 6.68dB

  - 2 sources observed that there is a coverage gap of 3.4 dB and 0.78dB respectively even with 4 SSB combinations.

  - 2 sources observed that there is a coverage gap of 2.95 dB and 1.8dB respectively only with a single SSB detection.

Companies are encouraged to provide views on Proposed Observation 3-9-v1:

| Companies | Comments |
| --- | --- |
| LGE | In our understanding, it is assumed that the elevation angle of 30 degree to derive the CNR. On the other hand, when we see the value of Doppler shift for SSB evaluation (24ppm), it is associated with the elevation angle of 10 degree according to the formula specified in TR38.811. Regarding the time drift due to Doppler frequency offset, it is unclear whether 48ppm would be a typical scenario. It seems that it comes from the summation of two 24 ppm (one for feeder link and the other for service link). However, in our understanding, depending on the relative direction between the satellite and the UE or GW, the sign of Doppler frequency offset could be positive or negative. In our understanding, 48 ppm can be achieved when both UE and GW are on the same location which is associated with the elevation angle of 10 degree. However, in this case, since the distance between GW and UE is so close, NTN operation may not be used. In our understanding, in typical scenarios, GW and UE will be located oppositely with respect to the satellite. In this case, the time drift due to Doppler frequency offset would be less than 24ppm. In short, the observations cannot be captured in this stage since proper evaluation assumptions are not considered. |

# Updated Proposals for online session on Thursday

Proposed observation 1-v1:

Based on the results of DL coverage evaluation at system level collected from different sources, extending the default value of SSB periodicity (different from 20ms) is beneficial in NTN implementing beam hopping is beneficial in terms of reduction of common control channel overhead according to 8 sources:

- With Set 1-1 FR1 and Set 1-3 FR1, the ratio of common message (SSB, SIB1, SIB19) is around 40% assuming 5 MHz BW when SSB periodicity of 20ms is in use, this ratio could be reduced to less than 14% when 160ms SSB periodicity is used.

- With Set 1-2 FR1, the ratio of common message (SSB, SIB1, SIB19) is greater than 50% assuming 5 MHz BW when SSB periodicity of 20ms is in use, this ratio could be reduced to around 25.8% when 640ms SSB periodicity is used.

Proposed observation 2-v1:

Based on the results of DL coverage evaluation at system level collected from 6 sources, it is observed that extended SSB periodicity increases the coverage ratio and thereby improves system level coverage for all the three LEO600km satellite parameter sets:

- For set 1-1/1-3, the coverage ratio (N2+N3/ total beam footprints) can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms,

- For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms.

Proposed Observation 3-4-v1:

Based on LLS results on PDSCH SIB1 coverage evaluation collected from different sources:

- For PDSCH carrying SIB1 option 1 (with a payload size of 800bits) it is observed that the required SNR is in average equal to – 5.8 dB (14 sources)

- For PDSCH carrying SIB1 option 2 (with a payload size of 1280bits) it is observed that the required SNR is in average equal to – 3.4 dB (12 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2FR1:

  - 14 sources observed that there is no coverage gap for PDSCH with SIB1 option 1:

      - The coverage margin is around 3.9 dB compared to CNR of -1.9 dB

  - 12 sources observed that there is no coverage gap for PDSCH with SIB1 option 2:

      - The coverage margin is around 1.5 dB compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 11 sources observed that there is a coverage gap for PDSCH with SIB1 option 1:

      - The coverage gap is around 4.1 dB compared to CNR of -9.9 dB

  - 1 source observed that there is no coverage gap for PDSCH with SIB1 option 1:

      - The coverage margin is 3.4 dB compared to CNR of -9.9 dB

  - 10 sources observed that there is a coverage gap for PDSCH with SIB1 option 2:

      - The coverage gap is around 6.5 dB compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB1.

Proposed Observation 3-5-v1:

Based on LLS results on PDSCH SIB19 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying SIB19 is in average equal to – 6.9 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 12 sources observed that there is no coverage gap for PDSCH with SIB19:

      - The coverage margin is around 4.2 dB compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 10 sources observed that there is a coverage gap for PDSCH with SIB19:

      - The coverage gap is around 3.5 dB compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB19.

Proposed Observation 3-6-v1:

Based on LLS results on PDSCH for VoIP coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for VoIP is in average equal to – 11 dB (11 sources)

- With parameter LEO600km Set1-1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 11 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is around 9.1 dB compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 9 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is around 2.3 dB compared to CNR of -9.9 dB

  - 1 source observed that even with 8 PDSCH repetitions there is a coverage gap of 1.5 dB compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH for VoIP.

Proposed Observation 3-7-v1:

Based on LLS results on PDSCH 3kbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for low data rate is in average equal to – 11 dB (8 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 8 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is around 9.1 dB compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 6 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is around 1.6 dB compared to CNR of -9.9 dB

Proposed Observation 3-8-v0:

Based on LLS results on PDSCH 1Mbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH with 1Mbps data rate is in average equal to – 4.1 dB (7 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - 7 sources observed that there is no coverage gap for PDSCH with 1Mbps:

  - The coverage margin is around 2.2 dB compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 5 sources observed that, there is a coverage gap for PDSCH with 1Mbps:

  - The coverage gap is round 5.5 dB compared to CNR of -9.9 dB

Proposed Observation 3-9-v1:

Based on LLS results on SSB coverage evaluation collected from different sources:

- With parameter LEO600km Set1-1 and 1-2, the required SNR for SSB detection is ranging from -10.8dB to -3.6dB and thereby no coverage gap is observed for LEO600km Set1-1 and 1-2.

- With parameter LEO600km Set1-3:

  - 7 sources observed that with 4 SSB combinations the SSB can be successfully detected with a margin ranging from 0.2dB to 6.68dB

  - 2 sources observed that there is a coverage gap of 3.4 dB and 0.78dB respectively even with 4 SSB combinations.

  - 2 sources observed that there is a coverage gap of 2.95 dB and 1.8dB respectively only with a single SSB detection.

# Conclusion

# Appendix I

| TDoc | Source |  |
| --- | --- | --- |
| R1-2403938 | Huawei, HiSilicon | Observation 1: Beam hopping transmission of common control signal/channel within increased SSB periodicity increases the common control channel coverage ratio to improve system level coverage for all the three LEO600km satellite parameter sets. Observation 2: Beam hopping transmission of common control signal/channel within increased SSB periodicity can also reduce the system common control channel overhead for all the three LEO600 parameter sets. Observation 3: Due to the reduced common control channel overhead by extending the SSB and common control channel periodicity: - The UPT for traffic type of VoIP, IM and FTP3 can be improved, - The cell throughput for IM and FTP is improved. Observation 4: Only the legacy UEs in the illuminated beam footprints in Rel-17/Rel-18 baseline need to be considered, and meanwhile for other beam footprints, larger SSB periodicity can be used to increase the number of beam footprints where SSB and common channel are provided for system level coverage ratio improvement. Observation 5: Legacy UEs work well in the beam footprints which are supposed to be illuminated/served in Rel-17/18 baseline by transmitting 20ms periodicity SSB in these footprints, meanwhile beam hopping transmission of common control signal/channel with increased SSB periodicity can be used to extend the coverage to other beam footprints in the total 1058 beam footprints. Observation 6: Performance of SIB1 and SIB19 meets the coverage requirement in Set1-1/2, with extended SSB periodicity. Observation 7: PDSCH 1Mbps/VoIP meet the coverage requirements in Set1-1 and Set1-2, under the time and frequency error impairment after a time duration of 640ms, which is the worst case if SSB periodicity is extended to 640ms, where the performance degradation is at most 0.2dB. Observation 8: With worst impairments of 0.9dB, UE-specific PDCCH still satisfy coverage requirement in both Set1-1 and Set1-2, in context of SSB periodicity extension. Observation 9: UE can utilize TRS to help improve performance for data transmission when the SSB periodicity is extended. Observation 10: For LLS assumption for SIB1 PDSCH, TBS sizes are 984 bits and 1608 bits for SIB1 payload option1 and option 2. Observation 11: TBS for SIB19 is 736bits for LLS assumption in R19 NR NTN. Observation 12: According to current specification, PBCH cannot be combined if the total time span exists 80ms. Observation 13: With parameter LEO60km Set1-1 and 1-2 considered, no coverage gaps are found for all DL channels. Observation 14: With parameter LEO600km Set 1-3, the SSB cannot even work due to the very low EIRP density. Proposal 1: Support improving the percentage of served beam footprints, where at least SSBs and common channels (SIB1 and SIB19) are provided for UE’s access, for NTN system level coverage enhancement. Proposal 2: The unequal distribution of user density among different beam footprints should be considered in the system level coverage enhancement. Proposal 3: The following method is used in the system level evaluation under the agreed framework with N1, N2 and N3 beam footprints: - In each simulation drop: - Step 1: Randomly select N3 beam footprints among 1058 beam footprints, where N3=106 for Set1-1, Set1-3/and N3=16 for Set 1-2; - Step 2: Randomly drop X UEs per each selected N3 beam footprint, where X is reported by companies; - Step 3: Generate traffic based on the agreed traffic model, if user perceived throughput and cell throughput are simulated. Proposal 4: Beam hopping transmission of common control signal/channel within increased SSB periodicity is supported to increase the system level coverage ratio in NR NTN. Proposal 5: For NR NTN UE, the default SSB periodicity assumed by UE during initial access is enlarged to support higher system level coverage ratio in NR NTN. Proposal 6: At least SSB periodicity of 320ms are introduced in SIB1 for NR NTN UE to enable the wider system coverage in Rel-19 NR NTN. Proposal 7: Shorter SSB periodicity, e.g. 20ms, and extended SSB periodicities are used for beam footprints with high user density and very low user density respectively. Proposal 8: Doppler frequency offset should be considered in the LLS assumption for PSS detection. Proposal 9: Do not consider link level coverage enhancement for downlink channels in a scenario/configuration where the EIRP density is reduced so much that the SSB cannot work. Proposal 10: Refine the simultaneously active beams number from 106 to 42 in Set 1-3 to make sure SSB does not have coverage gap. |
| R1-2403989 | Ericsson | Observation 1 The adopted UV plane is not a fitting methodology to allow an accurate inter-beam interference calculation, nor a placement of high number of spot beam tier layouts. Observation 2 The adopted ABS definition couples the satellite antenna beam width, which is a fixed simulation configuration, with the ABS. As an effect, the system cannot be optimized in terms of number of beam footprints within the satellite coverage area and SNR/SI balancing. Observation 3 If one increases the ABS by n, then the hexagonal cell area increases by n in the UV plane. Then, the number of satellite beam footprints required to provide the coverage to the specific region of interest can be decreased by increasing the ABS in the UV plane. Observation 4 For FRF = 1, larger ABS with the same HPBW reduces the aggregate interference and hence improves the downlink SINR in majority UE locations (excluding the cell-edge users), while marginally decreasing the SINR for the cell-edge users. Observation 5 For FRF = 3 and BH with TRF = 3, larger ABS with the same HPBW reduces interference and hence improves the downlink SINR in a limited number of UE locations (i.e., around the cell center), while reducing the SINR for majority UE locations, including the ell-edge users. Observation 6 Larger ABS allows to reduce the required time reuse factor which in turn yields increased beam illumination ON-time for a given Trevisit in the context of beam hopping. Observation 7 The existing UE distribution model does not reflect the expected NTN deployment in terms of cell user density, cell population/traffic imbalance. Observation 8 It is possible to keep using the legacy 20ms SSB periodicity for performing a Beam Hopping based scheduling, while guaranteeing the transmission of physical channels and signals required to be transmitted during initial access (SSB, SIB1, SIB19, TRS, CSI-RS) and at least msg1 and msg2 of the RACH procedure. Observation 9 Using an SSB supporting longer than 20ms for perfoming a Beam Hopping based scheduling will result in a specification impact (i.e., according to TS 38.213 clause 4.1, during initial access UEs assume an SSB periodicity of 20ms). A modification on this procedure is foreseen to impact legacy UEs which may not be able to access an NTN Beam Hopping cell supporting an SSB periodicity longer than 20ms during initial access. Observation 10 For Set 1-1 and Set 1-2, SSB, PDCCH and PDSCH meet the requirements of the link budget in terms of the BLER target. The coverage gap for PDCCH w.r.t SSB is about 6.5 dB while the coverage gap for PDSCH w.r.t SSB is in the range 0.1 to 6.1 dB, depending on the type of message carried on the channel. Observation 11 For Set 1-3, only SSB and PDSCH Msg 2 meet the link budget requirements and the other channels, including PDCCH, PDSCH Msg 4, PDSCH SIB1 and PDSCH SIB 19, could not meet the requirements. Proposal 1 RAN1 to consider beam layout with at least one increased value of ABS (e.g., 0.1157, equivalently, beam size of 86.6 km) in addition to the baseline ABS of 0.0668 (equivalently beam size of 50 km) with the same HPBW. Proposal 2 RAN1 to discuss the baseline techniques such as FRF = 1 and FRF = 3 while comparing the performance of beam hopping technique in the downlink coverage enhancements and the specification impact point of view. Proposal 3 RAN1 to consider beam layout with larger ABS without increasing the HPBW in the system-level evaluations of the downlink coverage enhancements study. Proposal 4 RAN1 aim to consider the low or moderate time reuse factors (that allows reasonable Tdwell) in the context of beam hopping to limit the specification impact. Proposal 5 RAN1 to revisit the UE distribution model to reflect the relevant NTN deployment aspects, including the non-uniformity of the traffic distribution in system-level evaluation of beam hopping. Proposal 6 At least the legacy SSB periodicity of 20ms during initial access is supported for Beam Hopping. Proposal 7 If SSB periodicities longer than 20ms during initial access are intended to be supported, then the following aspects shall be studied: - Specification impacts - Backward compatibility with legacy UEs - Sync-raster - MIB periodicity - PBCH combining - Performance Loss/Gain. Proposal 8 Given that multiple cases of PDSCH and PDCCH suffer from the coverage requirements to meet the BLER target under the settings of Set 1-3, RAN1 to revisit the settings of Set 1-3 or to prioritize Set 1-1/1-2 in the study. |
| R1-2403993 | Eutelsat Group | Observation 1: The conclusions of [2] for FR1-NTN antenna performance are an improvement over the original assumptions and are equally applicable to FR2-NTN. Proposal 1: The single antenna gain, antenna spacing and consequent results from ‘option 3’ in [2] should be scaled in frequency (from S-band to Ka band) and adopted as the basis of evaluation in FR2-NTN (or at least used as an option). Observation 2: The values suggested in [3] for FR2-NTN are generally in line with industry expectations. Proposal 2: With some minor adjustment the proposed values in [3] as modified by Table 1 should be used for FR2-NTN analysis. Observation 3: The beam pattern and frequency reuse in FR2-NTN will have similar considerations to FR1-NTN. Observation 4: Similar to FR1-NTN, FR2-NTN traffic patterns will not follow terrestrial density or traffic patterns. Observation 5: As is the case for FR1-NTN, FR2-NTN access latency and coverage can be traded off to achieve a range of latency-coverage objectives a satellite operator may have. The trade-off may vary over the lifetime as the constellation is densified. Proposal 4: The impact of extending the SS burst periodicity beyond the present assumptions should be considered. Observation 6: In the above discussion, we have shown at least some of the work on FR1-NTN DL enhancements can be directly or indirectly applied to FR2-NTN. Proposal 5: Finally, we would encourage RAN1 to consider if techniques being proposed to enhance FR1-NTN downlink coverage can be applied to FR2-NTN, either directly or with the essential adaptations that could be required. |
| R1-2404003 | Fraunhofer IIS, Fraunhofer HHI | Observation 1: A beam dwell time of 2ms might not be enough to transmit common control signaling (e.g., SSB, Type0/0A-PDCCH, SIB1, and SIB19) and other data transmissions. Observation 2: A maximum dwell time of 16ms could be achieved for set1-1 and set1-3 if an SSB periodicity of 160ms was considered. Observation 3: Set 1-2 is not enough to ensure receiving the necessary common control signaling (e.g., SSB, Type0/0A-PDCCH, SIB1, and SIB19) even by considering an SSB periodicity of 160ms. Observation 4: Increasing the beam dwell time increases the beam revisit time, which requires increasing the SSB periodicity. Observation 5: Increasing the periodicity of SSB impacts initial access in terms of latency and synchronization. Observation 6: More satellite footprints could be covered using the active satellite beams by sweeping each active satellite beam in an SSB TDM-mulitplexed manner. Observation 7: The SSB periodicity of 20ms could be achieved for set1-1 and set1-3 by sweeping each active satellite beam in an SSB TDM-mulitplexed manner, where each SSB burst consists of 4 SSBs and the beam dwell time is 5ms required to transmit the SSB burst. Observation 8: The SSB periodicity of 20ms could not be achieved for set1-2 even by sweeping each active satellite beam in an SSB TDM-mulitplexed manner, where an SSB periodicity of at least 80ms might be required. Proposal 1: The number of beams active at the same time/the same dwell time could be grouped and defined by one SSB pattern/SSB index. Proposal 2: RAN1 to study the impact of increasing the periodicity of SSB during initial access on legacy UEs. |
| R1-2404041 | Spreadtrum Communications | Observation 1: For Set1-1 FR1/ Set1-2 FR1, coverage enhancement on PDCCH is unnecessary. Observation 2: For Set1-3 FR1, coverage enhancement on PDCCH can be considered due to the 2.6 dB coverage gap. Observation 3: For Set1-1 FR1/ Set1-2 FR1/ Set1-3 FR1, coverage enhancement on PDSCH for Msg2 is unnecessary since there is no coverage gap. Observation 4: For Set1-1 FR1/ Set1-2 FR1/ Set1-3 FR1, coverage enhancement on PDSCH for Msg4 is unnecessary since there is no coverage gap. Observation 5: For Set1-1 FR1/ Set1-2 FR1, coverage enhancement on PDSCH for VoIP is unnecessary since there is no the coverage gap. Observation 6: For Set1-3 FR1, coverage enhancement on PDSCH for VoIP should be considered due to the 1.4 dB coverage gap. Observation 7: For Set1-1 FR1/ Set1-2 FR1/ Set1-3 FR1, coverage enhancement on PDSCH for low date rate service is unnecessary since there is no coverage gap. Observation 8: If the total number of simultaneously active beams is 16, extending the default SSB periodicity is needed to achieve 100% coverage ratio. Observation 9: If the total number of simultaneously active beams is 106, extending the default SSB periodicity may be not needed to achieve 100% coverage ratio. Proposal 1: M-TRP based PDCCH repetition in R16 can be as a start point for PDCCH coverage enhancement in NR NTN. Proposal 2: The enhancement of PDSCH coverage in RRC CONNECTED mode needs to consider the impact on spectral efficiency of system. Proposal 3: Cell DTX/DRX mechanism in RRC idle/inactive mode also should be considered. Proposal 4 Cell DTX/DRX mechanism in RRC idle/inactive mode needs to consider the specification impacts on the following UE activities. - RRM measurement - System information updates - Paging message reception - RACH procedure - SDT Proposal 5: The following two schemes for NES can be considered for achieving dynamic and flexible power sharing between satellite beams in RRC CONNECTED mode. - Cell DTX - Adaptation of transmission power of signals and channels |
| R1-2404132 | Samsung | Observation 1: PDSCH (Msg 2) performance requirement can be met for set 1-1, set 1-2 and set 1-3. Observation 2: PDCCH and PDSCH (Msg 4, SIB1 and SIB 19) performance requirements can be met for set 1-1 and set 1-2. Observation 3: PDCCH and PDSCH (Msg 4, SIB1 and SIB 19) performance requirements cannot be met for set 1-3. Proposal 1: PDSCH (Msg2) enhancement is not considered in Rel-19 NTN. Proposal 2: RAN1 discusses whether to consider enhancements for PDCCH and PDSCH (SIB1) in Rel-19 NTN considering if set 1-3 can be supported by existing SSB scheme. Observation 4: SSB performance requirement could not be met for all sets based on 5% percentile SINR result in case of randomly activated beam. Observation 5: SSB performance requirement could not be met for all sets based on 5% percentile SINR result in case of randomly activated beam. Observation 6: Antenna element spacing of 0.667causes an unwanted higher interference than Antenna element spacing of 0.5. Proposal 3: RAN1 does not consider study for potential SSB enhancement in Rel-19. |
| R1-2404194 | vivo | Observation 1: For Set1-1 FR1, the common signals/messages overhead can be significantly reduced by using a proper and flexible beam hopping design taking into account, e.g., combining wider beam for SSB and narrower beam for others, compared to a fixed beam configuration. Observation 2: For Set1-1 FR1 with FTP traffic, - The average SE performance of the configurations with fixed beam footprints is comparable to the results of IMT-2020 self-evaluation for NTN. - The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can significantly achieve better performance than the configurations with fixed beam footprints, e.g., 500% performance improvement. Observation 3: For Set1-1 FR1 with VoIP traffic, - The performance of the configurations with fixed beam footprints cannot achieve the VoIP transmission rate in TN. - The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can achieve better performance, which is close to the VoIP transmission rate in TN. Observation 4: For Set1-2 FR1, the common signals/messages overhead can be significantly reduced by using a proper and flexible beam hopping design taking into account, e.g., combining wider beam for SSB and narrower beam for others, compared to a fixed beam configuration. Observation 5: For Set1-2 FR1 with FTP traffic, - The average SE performance of the configurations with fixed beam footprints is worse than the results of IMT-2020 self-evaluation for NTN. - The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can significantly achieve better performance than the configurations with fixed beam footprints, e.g., 276% performance improvement. Observation 6: For Set1-2 FR1 with VoIP traffic, - The performance of the configurations with fixed beam footprints cannot achieve the VoIP transmission rate in TN. - The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can achieve better performance, 37% performance improvement than the configuration with fixed beam footprints. Observation 7: For Set1-3 FR1, the common signals/messages overhead can be significantly reduced by using a proper and flexible beam hopping design taking into account, e.g., combining wider beam for SSB and narrower beam for others, compared to a fixed beam configuration. Observation 8: For Set1-3 FR1 with FTP traffic, - The average SE performance of the configurations with fixed beam footprints is worse than the results of IMT-2020 self-evaluation for NTN. - The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can significantly achieve better performance than the configurations with fixed beam footprints, e.g., 150% performance improvement. Observation 9: For Set1-3 FR1 with VoIP traffic, - The performance of the configurations with fixed beam footprints cannot achieve the VoIP transmission rate in TN. - The proposed configurations utilizing proper and flexible beam, e.g., combining wider beam for SSB and narrower beam for others, can achieve better performance, 74% performance improvement than the configuration with fixed beam footprints. Observation 10: For LEO600km Set1-1 FR1 and Set1-2 FR1, based on link budget analysis, no coverage issue is observed for any channels. Observation 11: For LEO600km Set1-3 FR1, based on link budget analysis, 3~6 dBs performance gaps from the coverage target are observed for some of the channels and signals, namely PDCCH, PDSCH-1Mbps, Msg4, SIB1 with 800/1280 bits payload size. Observation 12: The performance loss of SIB1/SIB19 can be easily solved by enabling repetition transmission or retransmission. Observation 13: The performance loss of Msg4/PDSCH-1Mbps can be easily solved by enabling retransmission. Observation 14: The performance loss of PDCCH can be solved by a proper beam hopping pattern design at the system level aspects. Proposal 1: For LEO600km Set1-1/2/3 FR1, applying proper flexible beam size and hopping design can significantly improve the system level performance to an acceptable level. Proposal 2: For LEO600km Set1-1/2/3 FR1, the change of SSB design including periodicity is not necessary for the DL coverage enhancement. Proposal 3: For LEO600km Set1-1/2/3 FR1, additional link level enhancement for DL coverage is not necessary. |
| R1-2404201 | THALES | Observation 1: In NR NTN, the satellite beam illumination plan using satellite optimized beam hopping should take into account several constraints: - The total available payload power - The total number of satellite beams: hundreds of beams maybe needed for the coverage of target service area. - The nominal DL power density per satellite beam: e.g. a satellite EIRP density of 34 dBW/MHz at LEO600km orbit [TR 38.821] - The configuration of the 5G common channels and signals, such as the SSB, SIB1, other SIBs e.g. SIB19, the PDCCH and PRACH occasions. - The traffic demand and the number of RRC active users in each beam - The configuration of UE specific signals and channels: periodic/semi-static CSI-RS, group-common/UE-specific PDCCH, SPS PDSCH, PUCCH carrying SR, PUCCH/PUSCH carrying CSI reports, PUCCH carrying HARQ-ACK for SPS, CG-PUSCH, SRS, and positioning RS (PRS) - Discontinuous Transmission (DTX) and Discontinuous Reception (DRX) - The dwell time of a satellite beam which depends on the configuration of common channels and signals, the configuration of UE specific channels and signals and the user traffic type. - The quality of service (QoS) and the QoE provided to the end user: - It should not be allowed to not illuminate the beam for long duration: - which could be seen from RRC connected UEs perspective as a radio link failure leading to call/PDU session drops. - And will introduce large latency for network access - It should not be allowed to illuminate the beam for short duration (short satellite beam dwell time) leading to a degraded user throughput. Observation 2: Different solutions should be combined together to enable an efficient beam hopping in 5G NTN: - Take benefit of beam management techniques specified in 5G NR and adopted as baseline in 5G NTN - Satellite payload power saving techniques are also needed: Leveraging the new techniques that are being studied and specified in 3GPP Release-18 and Release-19 as part of Network Energy Saving (NES) work item - Downlink coverage enhancements (CE) at link level: to extend the number of satellite beams that could be simultaneously activated/illuminated the nominal available power per beam could be dynamically split between several beams (e.g between 4 beams which lead to a power reduction of 6dB). DL CE is needed to cope with such DL Power reduction. Observation 3: The minimum size of NR beam is the satellite beam’s size Observation 4: One-beam per cell and multiple-beam per cell are supported in existing NR specifications and are baseline for NR NTN (Release-17 agreement). Observation 5: In existing specifications, number of NR beams is chosen according to the beam width and meant to cover the whole cell. The maximum number of beams L defined by 3GPP and dependent on frequency band is only equal to L=4 in S-band. In NTN, for an optimized illumination plan larger cells may need to be deployed and thereby, L=4 may not be sufficient in S-Band. Observation 6: Based on RAN1 DL NTN CE Study in Release-18, a power reduction up to 6dB (to be confirmed by Rel-19 Link level study) can be applied without impact on the performance on SSB detection Observation 7: Set1-2 FR1 with % simultaneously active beams of 1,5% requires SSB periodicity of at least 80ms. That is, a default periodicity of 20ms is not doable for such deployment scenario. Observation 8: Set 1-1 and Set 1-3 FR1 with % simultaneously active beams of 10% increasing the SSB periodicity from up to 160ms allows an increase of the cell available capacity for user traffic and the cell throughput by 30%. Observation 9: Set1-2 FR1 can work with a default SSB periodicity of 20ms if and only if the % simultaneously active beams is at least equal to 4.1% (only 394 beam are illuminated). Observation 10: WGS84 flattened sphere Earth model used LEO600 with 4.417° 3 dB beamwidth for 38.821 Set1. If beam layout defined in Table 6.1.1.1-4 in TR 38.821 beamwidth is used, 547 beams (13 tiers) is the largest possible before outside beams stretch over the horizon. Observation 11: Observations 7, 8 and 9 are confirmed with the System level simulations results. Observation 12: System level simulations results and analytical evaluation results lead to the same conclusion on the necessity to extend the default SSB periodicity to support satellite beam hopping in NTN. Observation 13: The Carrier-to-noise ratio (CNR) for the satellite payload parameters Set 1-1, Set 1-2 and Set 1-3 is equal to -1,89 dB, -1,89 dB and -9,89 dB respectively. Observation 14: For PDSCH with 3 kbps with parameter Set-1-1/ Set-1-2/ Set-1-3 for LEO-600 operating at LOS the existing specification can meet the performance requirement. Observation 15: For PDSCH with 1 Mbps with parameter Set-1-3 for LEO-600 operating at LOS the existing specification cannot meet the performance requirement with a gap of -5,29 dB. Observation 16: For PDSCH with VoIP with parameter Set-1-1/ Set-1-2/ Set-1-3 for LEO-600 operating at LOS the existing specification can meet the performance requirement. Observation 17: For PDCCH with parameter Set-1-3 for LEO-600 operating at LOS the existing specification cannot meet the performance requirement with a gap of -2,19dB. Proposal 1: RAN1 to determine the maximum power reduction that can be applied in DL without impacting SSB design Proposal 2: RAN1 to consider techniques/enhancement enabling an optimized Satellite beam illumination plan/beam hopping. These enhancements should allow legacy UEs to be able to continue accessing a network implementing these new Rel-19 features, with the possible exception of techniques developed specifically for greenfield deployments. Proposal 3: The following potential enhancements are considered to enable an optimized Satellite beam illumination plan/beam hopping: - Extending the default SSB periodicity - Release 18 network energy saving techniques (e.g., cell DTX/DRX, etc.) with modifications to fit NR NTN (e.g., beam-based operations): - Support the configuration of more than one cell DTX patterns to a UE. - Support dynamic group-common signaling for the change of cell DTX patterns. Proposal 4: RAN1 to study coverage enhancements techniques allowing for a reduced dwell time of a satellite beam (time duration during which the beam is active) such as techniques with a reduced number of repetitions. Proposal 5: RAN1 to introduce enhancements on the DL coverage of the following channels: - PDCCH - PDSCH with 1 Mbps |
| R1-2404214 | ZTE | Observation 1: Based on the SLS evaluation results, it is observed that the number of active beams is not enough to serve N3 footprints for LEO600km Set1-1 to Set1-3 FR1 scenarios. Observation 2: Based on the SLS evaluation results, it is observed that the number of active beams that can be dedicatedly used for N2 footprints is zero for LEO600km Set1-1 to Set1-3 FR1 scenarios. Observation 3: The 1058 beam footprints cannot be served with a maximum revisit time of 20ms with the necessary common channel transmission. Observation 4: For Set 1-1 and Set 1-2, the minimum dwell time is 4ms due to repetition of common channel. Observation 5: For Set 1-3, the minimum dwell time is 13ms due to repetition of common channels. Observation 6: To support MSG1 detection, a minimum dwell time of 13.2ms is needed for a give beam. Observation 7: If a complete initial access procedure is involved, a minimum dwell time of 31.5ms is needed for a give beam. Observation 8: For Set 1-1, a minimum SSB periodicity of 640ms is needed to support necessary DL common channels transmission and random-access procedure. Observation 9: For Set 1-2, a minimum SSB periodicity of 640ms is needed to support necessary DL common channels transmission. Observation 10: For Set 1-3, a minimum SSB periodicity of 320ms is needed to support necessary DL common channels if link level enhancement is introduced for coverage. Observation 11: If a wide beam covering 4 footprints is used, the CNR values for Set 1-1/1-2 and Set 1-3 are -7.9dB and -15.9dB, respectively. Observation 12: If a wide beam covering 4 footprints is used, all necessary DL common channels including the SSB cannot be correctly received with the reduced CNR values. Observation 13: If a wide beam covering is used, the necessity to extend the periodicity of common still holds due to the extended dwell time from the enhancement of DL common channel. Proposal 1: A minimum SSB periodicity of 640ms can be considered. Observation 14: For PSS detection, large frequency offset and timing drift should be considered since no compensation can be performed in initial DL synchronization. Observation 15: For PSS detection, the required SNR for single-shot SSB detection is -3.3 dB, which shows an SNR margin of 1.4 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3. Even with 4-SSB combination, the required SNR is around -6.1 dB, which is still higher than CNR for Set 1-3. Observation 16: For PBCH detection, the required SNR for single-shot SSB detection is -7.2 dB, which shows an SNR margin of 5.3 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3. Observation 17: For PBCH detection, with 4-SSB combination, the required SNR is around -13.2 dB, which is lower than CNR for Set 1-3. However, the required SNR for PSS detection, which is -6.1 dB, will be the bottleneck for SSB detection. Proposal 2: The required SNR for SSB detection is larger than the CNR for Set 1-3 even with 4-SSB combination, i.e., CNR for Set 1-3 should not be the target for link level enhancement. Observation 18: The required SNR for PDCCH is about -6.8 dB with single repetition, which shows an SNR margin of 4.9 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3. Proposal 3: For LEO-600 Set 1-1 and Set 1-2, link level enhancement on PDCCH is not needed. Proposal 4: For LEO-600 Set 1-3, the link budget is not enough for PDCCH detection. Up to 4 repetitions may be needed to mitigate the SNR gap. Observation 19: The required SNR for Msg2 PDSCH is about -11.2 dB, which shows an SNR margin of 9.3 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, and 1.3 dB with respect to CNR for LEO-600 Set 1-3. Proposal 5: For all the satellite parameter sets for LEO-600, link level enhancement on PDCCH is not needed. Observation 20: The required SNR for Msg4 PDSCH is about -4.5 dB, which shows an SNR margin of 2.6 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3. Proposal 6: For LEO-600 Set 1-1 and Set 1-2, link level enhancement on msg4 PDSCH is not needed. Proposal 7: For LEO-600 Set 1-3, the link budget is not enough for msg4 PDSCH detection. Repetitions may be needed to mitigate the SNR gap. Observation 21: The required SNR for SIB1 is about -5.5/-3.2 dB for option 1/2, which shows an SNR margin of 3.6/1.3 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3. Proposal 8: For LEO-600 Set 1-1 and Set 1-2, link level enhancement on SIB1 is not needed. Proposal 9: For LEO-600 Set 1-3, the link budget is not enough for SIB1 detection. Repetitions may be needed to mitigate the SNR gap. Observation 22: The required SNR for SIB19 is about -6.4 dB, which shows an SNR margin of 4.5 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3. Proposal 10: For LEO-600 Set 1-1 and Set 1-2, link level enhancement on SIB19 is not needed. Proposal 11: For LEO-600 Set 1-3, the link budget is not enough for SIB19 detection. Repetitions may be needed to mitigate the SNR gap. Observation 23: The required SNR for PDSCH of VoIP with 2 repetitions is about -10.8 dB, which shows an SNR margin of 8.9 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, and 0.9 dB with respect to CNR for LEO-600 Set 1-3. Proposal 12: For all the satellite parameter sets for LEO-600, link level enhancement on PDSCH of VoIP is not needed. Observation 24: The performance of 3kbps data service can be better than VoIP due to lower data rate. The performance of 1Mbps data service will be similar to Msg4 PDSCH without repetition due to similar data rate. Proposal 13: For LEO-600 Set 1-1 and 1-2, data service with 3kbps and 1Mbps can be supported without enhancement. Proposal 14: For LEO-600 Set 1-3, data service with 3kbps can be supported without enhancement, but data service with 1Mbps cannot be supported. |
| R1-2404261 | InterDigital, Inc. | Observation 1: NES Cell DTx and cell DRx mechanisms may provide a suitable baseline for NTN scenarios to perform power sharing among satellite beams and to periodically activate and de-activate a subset of satellite beams. Observation 2: A single DTx/DRx pattern may not be suitable for NTN due to wide coverage of satellite beams supporting UEs with different activity levels. Observation 3: Turning on and off the satellite beams in binary fashion may result in QoS degradations. Observation 4: Satellite payloads need to perform flexible power sharing among the beams to keep a balance between the QoS provisioning and satellite power constraints. Observation 5: Spatial domain adaptation techniques standardized in Rel-18 NES work may have very large overhead for NTN scenarios and use cases to adapt the beam footprint. Observation 6: The UEs need to be indicated at least the gNB employed spatial domain adaptation to be able to successfully decode PDSCH when the network is employing spatial adaptations with or without power domain adaptations. These observations and the discussion have led to the following proposals: Proposal 1: The network uses NES cell DTx and cell DRx mechanisms to activate and de-activate satellite beams in a periodic manner. Proposal 2: Configuration and simultaneous activation of multiple cell DTx and cell DRx patterns are supported for NTN operation. Proposal 3: Fractional power sharing among the satellite beams is supported. Proposal 4: Satellite beams may adapt spatial footprint based upon coverage requirements and active traffic without UE feedback. Proposal 5: The network uses higher aggregation factor for PDCCH transmission while the satellite beams are in power sharing mode. Proposal 6: PDCCH can be repeated multiple times to compensate the PDCCH coverage loss due to power sharing among the satellite beams. Proposal 7: PDSCH can be transmitted with a higher number of repetitions with fractional power transmission from satellite beams. |
| R1-2404307 | Apple | Observation 1: At 2% BLER, the required SNR for PDSCH with VoIP is -12.7 dB with 7 PRBs for 8 repetitions. Observation 2: At 10% BLER, the required SNR for PDSCH with 3 kbps is -10.4 dB with 1 PRB and 120/1024 coding rate with 8 repetitions. Observation 3: At 10% BLER, the required SNR for PDSCH with 1 Mbps is -4.9 dB with 25 PRBs and 193/1024 coding rate without repetition. Observation 4: At 10% BLER, the required SNR for Msg2 PDSCH is -6.1, -8.8 and -11.2 dB, with 3, 6 and 12 PRBs, 3 DMRS symbols and 120/1024 coding rate with scaling factor of 1, ½ and ¼, respectively. Observation 5: At 10% BLER, the required SNR for Msg4 PDSCH is -4.8 dB, -7.4 dB and -10.0 dB with 23 PRBs and 193/1024 coding rate with repetitions of 1, 2 and 4, respectively. Observation 6: At 10% BLER, the required SNR for SIB1 PDSCH is -5.8 dB and -3.7 dB with 24 PRBs for payload sizes of 800 bits and 1280 bits, respectively. Observation 7: At 10% BLER, the required SNR for PDSCH with SIB19 is -6.7 dB with 24 PRBs and 120/1024 coding rate. Observation 8: At 1% BLER, the required SNR for PDCCH is -7.9, -5.5, -2.4 and 1.7 dB at aggregation level of 8, 4, 2, 1, respectively. Observation 9: At 1% BLER, the required SNR for PBCH is -12.1 dB with a combination of 4 SSBs. Observation 10: PDSCH with 1 Mbps, PDSCH Msg2 (scaling factor of 1 or ½), PDSCH Msg4, PDSCH SIB1, PDSCH SIB19 and PDCCH have the following coverage gaps for LEO-600 set 1-3 parameters: - PDSCH with 1 Mbps: 5.0 dB - PDSCH Msg2 with scaling factor of 1: 3.8 dB - PDSCH Msg2 with scaling factor of ½: 1.1 dB - PDSCH Msg4: 5.1 dB - PDSCH SIB1: 4.1 dB - PDSCH SIB19: 3.2 dB - PDCCH: 2.0 dB Proposal 1: At least the coverage of PDCCH, PDSCH with Msg 4, PDSCH with SIB1 and PDSCH with SIB19 need to be enhanced. - Further evaluate whether the coverage of PDSCH with Msg2 needs to be enhanced. Proposal 2: To enhance the coverage of PDSCH Msg 4 and PDSCH carrying SIB1, consider introducing PDSCH repetition. Proposal 3: To enhance the coverage of PDSCH, the schemes (e.g., TB over multiple slots, DMRS bundling, etc.) used to enhance PUSCH performance should be examined for PDSCH. Proposal 4: To enhance the coverage of PDCCH, consider the following solutions: - Enhance PDCCH repetition (e.g., for PDCCH scheduling system information) - Increase CORESET symbol number and aggregation level - Reduce DCI size (e.g., 2-stage DCI) Observation 11: In LEO-600 set 1-1 and set 1-3 in FR1 with N1=N2=0 and N3=1058, the coverage ratio is 100%, with dwell time = 2 ms, revisit time = 20 ms and the periodicity of SSB and SIB1 = 20 ms. Observation 12: In LEO-600 set 1-1 and set 1-3 in FR1 with N1=0, N2=846 and N3=212, the coverage ratio is 100%, with dwell time = 6 ms (on N3 beam footprints), revisit time = 20 ms and periodicity of SSB and SIB1 = 20 ms. Observation 13: In LEO-600 set 1-2 in FR1 with N1=N2=0 and N3=1058, the coverage ratio is 100%, with dwell time = 2.4 ms, revisit time = 160 ms and periodicity of SSB and SIB1 = 160 ms. Observation 14: In LEO-600 set 1-2 in FR1 with N1=0, N2=846 and N3=212, the coverage ratio is 100%, with dwell time= 8.4 ms (on N3 beam footprints), revisit time =160 ms and periodicity of SSB and SIB1 = 160 ms. Observation 15: In LEO-600 set 1-2 in FR1 with N1=N3=0 and N2= 320, the coverage ratio is 30%, with dwell time = 1 ms (on N2 beam footprints), revisit time = 20 ms and periodicity of SSB and SIB1 = 20 ms. Observation 16: Increasing the default SSB periodicity significantly increases UE implementation complexity and leads to backward compatibility issue. Proposal 5: RAN1 considers the system level downlink coverage enhancement, using Release 18 network energy saving techniques (e.g., cell DTX/DRX, etc.) with modifications to fit NR NTN (e.g., beam-based operations). |
| R1-2404323 | LG Electronics | Observation 1: The required simultaneously active beam ratio only for SSB with 15kHz SCS is at least 1.43% when the number of SSBs within 20msec periodicity is 1. For other DL transmission including common channels, the active beam ratio of 1.5% in Set 1-2 is not feasible. Observation 2: The required simultaneously active beam ratio only for SSB with 30kHz SCS is at least 0.71% when the number of SSBs within 20msec periodicity is 1. For other DL transmission including common channels, the active beam ratio of 1.5% in Set 1-2 could be feasible. However, it would be necessary to increase BW to be more than 5MHz since the BW of the SSB with 30kHz SCS occupies 7.2MHz. Observation 3: For more accurate CNR analysis to compare with the required SNR of DL channels/signals, the payload DL TX power per satellite beam and the antenna gain with respect to the azimuth angle associated with the UE position needs to be used to compute more accurate EIRP. Observation 4: Due to the limited beam width of the antenna gain pattern, even for the beam footprint with 50km diameter at the nadir, the edge area of the beam footprint would be out of the SSB coverage. To cover these edge areas, even in FR1, each beam footprint may need to support at least two SSB with different TX beams. Observation 5: When the wide beam footprint is supported with multiple active component beam footprints with the reduced TX power, the satellite still needs to support the case where a large number of beam footprints are simultaneously active. This approach may not be suitable for Set 1-2. Observation 6: When the wide beam footprint is supported with a single active component beam footprint due to the RF limit, the active beam ratio for SSB transmission only could be 0.38%. However, the SSB detection would not be guaranteed in the large portion of the wide beam footprint due to the limited beam width of the antenna gain main lobe. Observation 7: When the wide beam footprint is supported with a single active component beam footprint due to the RF limit, if the phased array antenna is used at the serving satellite, the SSB detection could be guaranteed. For the normal beam footprint with 50km diameter, the amplitude and the phase of each antenna elements need to be adjusted to minimize inter-cell or inter-beam footprint interference. Observation 8: The SSB detection performance would be guaranteed at the elevation angle of 30-degree for Set 1-1 and Set 1-2. Observation 9: The SSB detection performance may not be guaranteed at the elevation angle of 30-degree for Set 1-3when the UE combines 4 SSB repetitions for SSB detection. Observation 10: The required simultaneously active beam ratio for SSB only is at least 1.25% for SSB with 120kHz SCS when the number of SSBs per beam footprint within 20msec periodicity is 4. For other DL transmission including common channels, the active beam ratio of 1.5% in Set 1-2 may not be sufficient. Observation 11: The required simultaneously active beam ratio for SSB only is at least 0.625% (or 0.3125%) for SSB with 120kHz SCS when the number of SSBs per beam footprint within 20msec periodicity is 2 (or 1, respectively). Even for other DL transmission including common channels, the active beam ratio of 1.5% in Set 1-2 would be sufficient. Observation 12: Due to the limited beam width of the antenna gain pattern, even for the beam footprint with 20km diameter at the nadir, the edge area of the beam footprint would be out of the SSB coverage. To cover these edge areas, each beam footprint may need to support at least two SSBs with different TX beams. Observation 13: For SSB detection performance as a reference, the value of Doppler shift needs to be determined based on the realistic scenario in terms of the relative locations among satellite, GW, and UE. For the elevation angle of 30 degree and the assumption that the UE is far away from the GW, 21 ppm can be a baseline. Observation 14: For PDSCH transmissions scheduled by non-fallback DCI, the existing PDSCH repetition schemes such as DL slot aggregation and repetition would be sufficient for NR FR1-NTN. Observation 15: For PDSCH transmissions scheduled by non-fallback DCI, to reduce the total active time for a given beam footprint, the existing scaling factor for TBS determination can be used even for SIBs. Moreover, the reserved state of the DCI indication for the scaling factor of TBS determination could be replaced with 1/8. Observation 16: For the case when a single cell consists of multiple satellite beams, the existing NR beam management may ensure that only one of the satellite beams for a single cell is activated in a time. Observation 17: Considering that the limit of user throughput, increasing the number of satellite beams to be associated with the same cell is not sufficient to have the reasonable value of the total aggregated EIRP of a satellite. Observation 18: Even if Rel-18 NES cell DTX operation is used for NR NTN, it is possible that PDSCH transmissions scheduled by DCI, CSI-RS transmissions, and/or SSB transmissions from different cells or satellite beams are overlapping in time. It could violate the active beam ratio of the serving satellite. Observation 19: If the serving satellite skips DL transmissions or reduces its TX power, it would be necessary to carefully investigate the impact on the measurement-based procedures. Observation 20: In Rel-19 NR NTN WI, it is not preferrable to have duplicated discussion for on-demand common signals/channels design which is a part of the objective of another WI. Proposal 1: Support wide beam footprint with a single active component beam footprint at least for Set 1-2. - Phased array antenna is a baseline. - FFS: Whether or how to support parabolic antenna. - A single wide beam footprint consists of at least 4 component beam footprints with 50km diameter. - A wide beam footprint is used to transmit at least SSB and common channel(s). - Component beam footprints are used to transmit the user DL traffic by using UE-specific scrambling ID. Proposal 2: For DL coverage study at system level, consider the following additional reference satellite payload parameters for LEO600km in FR2 (i.e., Ka-band): - Beam size: 20km - Satellite EIRP density /beam (dBW/MHz): 4 - Payload Total DL power level (dBW): 2.3 - Aggregated EIRP (Total) (dBW): 40.8 - Satellite Tx max Gain: 38.5 - EIRP per Satellite beam (dBW): 30 Proposal 3: For link-level enhancement in Rel-19 NR NTN, RAN1 carefully investigates followings: - PDSCH repetition for PDSCH transmissions scheduled by fallback DCI - How to indicate PDSCH repetition number - Scaling factor indicated by DCI for TBS determination for PDSCH - Whether or how to apply it to SIB transmissions - Whether or how to support the lower value (e.g., 1/8) - Search space linkage for PDCCH carrying DCI format for SIB1. Proposal 4: For system-level enhancement in Rel-19 NR NTN, RAN1 carefully investigates followings: - Prioritize dynamic and flexible power sharing mechanism in per-cell basis, but not in per-satellite-beam basis. - FFS: Whether or how to support the case when more than one satellite beams of a cell are activated in a time. - Whether or how to further restrict gNB or UE behavior outside the cell DTX Active Period. - Whether or how to reduce the DL reference power of a subset of cells served by a satellite. - Whether or how to skip DL transmissions (e.g., SIB19) other than PDCCH and SPS PDSCH during the cell DTX Non-Active Period. - Whether or how to support TDD configuration for NR NTN operation in FDD carrier. - Whether or how to support dynamic and flexible power sharing between DL transmission and UL transmission at the serving satellite side. |
| R1-2404390 | CATT | System level proposals: - For satellites with multiple active beams, a reasonable beam scheduling scheme needs to be designed to reduce the interference when frequency reusing factor is one. - The beam hopping mechanism with SSB periodicity extension can effectively improve satellite coverage and should be considered in system level enhancement. - Beam specific power control can improve power utilization efficiency from system level and can be considered in system level enhancement. Link level proposals: - Set1-3 is not feasible for coverage enhancement evaluation since SSB is not workable in this scenario. - Link level enhancement seems not be urgent from the evalution result. Meanwhile some observations are listed as follows. For system evaluation: - Under the random beam scheduling method, the interference is more severe when the number of physical beams is 106 compared to that of 16 physical beams. - In realistic deployment of LEO 600km at FR1, if one satellite can provide 16 beams activation, the coverage ratio can be optimized from 6% to 96.9%. - In realistic deployment of LEO 600km at FR1, if one satellite can provide 106 beams activation, the coverage ratio can be optimized from 40% to 80.2%. - SSB periodicity with 320ms can provide better performance in coverage ratio when only 16 beams are activated. For link level evaluation: - SSB detection without combination requires SNR about -5.5 dB, with 3.6 dB margin compared to CNR for LEO-600km set1-1 and set1-2. - Even if 4 SSBs are conbined, the link budget requirements of CNR=-9.9dB for set1-3 can not be met. - PDCCH channels can meet the link budget result of CNR=-1.9dB for Set1-1 and Set1-2, and do not require link-level enhancement. Observation 8: All cases of PDSCH can meet the link budget result of CNR=-1.9dB for Set1-1 and Set1-2, and do not require link-level enhancement. |
| R1-2404441 | Lenovo | Proposal 1: Default SSB periodicity is kept to be 20ms for NR NTN. Proposal 2: Consider beam index and power value indication for spatial/power domain beam adaptation in NR NTN. Proposal 3: Consider satellite beam level DTX/DRX. Proposal 4: In NR NTN, there are only two types of satellites, on or off, and there is no satellite beam with common message only. Proposal 5: RAN1 to define UE behaviors in off status of a satellite beam. Proposal 6: Consider the impact of large propagation delay and dynamic DL Tx power change in addition to on-off pattern indication in R19 NR NTN. Proposal 7: To consider other NES techniques only after stable discussion in AI 9.5. Proposal 8: Consider repetition of PDCCH/PDSCH, TBoMS/DMRS bundling for PDCSH, larger aggregation level/number of CORESET symbols for PDCCH for DL coverage enhancement. |
| R1-2404471 | CMCC | Observation 1. Regarding LEO-600 Satellite with Set 1-1/1-2 parameters in LOS environment, there is no coverage gap of DL physical channels considering -5.5dBi UE antenna gain. Observation 2. For LEO-600 Satellite with Set 1-1/1-2 parameters in LOS environment, it is observed that around 5dB coverage margin of PBCH with single SSB detection And for PBCH with 4 SSB combination, if Doppler frequency drift is not considered, 10.5dB coverage margin is observed, if Doppler frequency drift of 0.27 ppm/s is assumed, 8.89 coverage margin is observed. Observation 3. For LEO-600 Satellite with Set 1-1/1-2 parameters in LOS environment, when the Doppler frequency drift is considered in LLS for channels/signals before SIB19 acquisition, there is no coverage gap for PDSCH carrying SIB1 or SIB19. Observation 4. For LEO-600 satellite with Set1-3 parameters, there are coverage gaps for PDSCH both in initial access and connected state, including PDSCH carrying SIB1, PDSCH carrying SIB19, PDSCH Msg4, PDSCH for 1Mbps low-data rate services. - For initial access, 3.76 dB coverage gap of PDSCH carrying SIB1 (800bits) and 5.77dB coverage gap for PDSCH carrying SIB1 (1280 bits) is observed, and Msg4 PDSCH needs to be enhanced with 4.49 dB coverage gap. - To support 1Mbps low-data rate service, 7.73 dB coverage gap needs to be enhanced for PDSCH. Observation 5. For LEO-600 satellite with Set1-3 parameters, there is around 3dB coverage gap for PBCH when single SSB detection is applied no matter the Doppler frequency drift is considered or not. While 2.52dB coverage margin can be observed for PBCH when 4 SSB combination is considered without frequency drift, and barely no margin if considering Doppler frequency drift. Observation 6. For LEO-600 satellite with Set1-3 parameters, DL coverage needs to be enhanced with 3.52 dB for PDCCH. Observation 7. For LEO-600 in FR1, with default 20ms SSB periodicity, 4 SSBs combination can provide better coverage performance and coverage margins are observed for different satellite parameters. Observation 8. Some channels, e.g. PBCH with 4 SSB combination detection, can provide additional margins to support wider beams. Observation 9. Additional SSB detection, e.g. SSB combinations, within the illumination window would facilitate the UE implementation for the DL synchronization. Observation 10. No obvious difference between N2 and N3 is observed, - N2 beam footprints can support at least system information and UE initial access with wide/narrow beam - N3 beam footprints can support both active traffic and system information with narrow beam Observation 11. For LEO-600 Set1-1/1-2/1-3, even if all the active beams are used for common channels, it can not satisfy 100% coverage ratio with the default common channel periodicity. Observation 12. It is observed that the coverage ratio can be improved with longer duration of common channel periodicity from UE side. Observation 13. For LEO-600 Set1-1/1-2/1-3, longer revisit time can be considered to improve the coverage ratio. Proposal 1. When additional loss (e.g. steering loss) is not considered, DL coverage enhancements are not required for LEO-600 with Set 1-1/1-2 satellite parameters to serve the target service. Proposal 2. To support target services under LEO-600 scenario with set1-3 reference parameters, DL coverage enhancements need to be considered for PBCH with single SSB detection, PDSCH carrying SIB1, PDSCH carrying SIB19, Msg4 PDSCH and PDSCH for 1Mbps low data rate service. Proposal 3. For the detection of SSBs in NR NTN, SSB combinations can be considered to improve the coverage performance. Proposal 4. Additional SSB detection, e.g. SSB combinations, within the illumination window should be considered to facilitate the UE implementation for DL synchronization. Proposal 5. To improve the coverage ratio of active beams, long revisit time/period and dwelling time should be introduced. Proposal 6. Under the assumption of long revisit time, the SSB periodicity should be extended. - For Set 1-1/1-3, the default SSB detection period should be extended. - For Set 1-2, the SSB periodicity should also be extended. Proposal 7. It should be further clarified that the simultaneously active beam number as agreed for Set 1-1/2/3 are for UL or DL or for both DL and UL, which will impact the design of the illumination window and revisit periods for the system level study. Proposal 8. To extend the coverage ratio, the following system level enhancements can be considered, - Spatial domain enhancements - Broader/wide beam size to serve larger area based on the link level evaluation - Time domain enhancements - Long revisit period and illumination window for each beam/footprint should be defined to improve the coverage ratio |
| R1-2404607 | Xiaomi | Observation 1: Without considering the maximum Doppler frequency drift in the LLS evaluation, for channels before SIB19 acquisition, there is no bottleneck channel based on the evaluation results for Set1-1 FR1& Set1-2 FR1. Observation 2: For Set1-3 FR1, there is performance gap for DL channels including PDCCH, SIB1 PDSCH, SIB19 PDSCH and Msg4 PDSCH even without considering the maximum Doppler frequency drift in the LLS evaluation. Observation 3: For PDCCH coverage enhancement, the possible solutions can be repetition, payload reduction and AL increasing. Observation 4: For common PDSCH coverage enhancement, a straightforward solution can be repetition. Proposal 1:The following solutions can be studied to solve the SSB sweeping problem due to the limited simultaneously active beam ratio. - Solution 1: Increase the SSB number and change the SSB pattern for S band - Solution 2: Associated the SSB index with the specific area/cell by NW implementation - Solution 3: Change the UE’s assumption on default SSB periodicity for initial cell selection Proposal 2: Time domain NES solutions such as on-demand SIB1, cell DRX/DRX can be studied for DL coverage enhancement in NTN. Proposal 3: Beam based dynamic bandwidth adjustment for dynamic power sharing between beams can be studied for DL coverage enhancement in NTN. Proposal 4: Beam based DL reference signal power indication can be studied for DL coverage enhancement in NTN. |
| R1-2404670 | NEC | Observation 1: Adjusting the number of active transmit antennae could provide flexible physical beam patterns/size (i.e. wide or narrow) across the satellite footprint. Proposal 1: Support using different beam sizes to provide DL service of different PHY channels to meet the link-level DL coverage requirements, and to improve the rate of (N2+N3)/(N1+N2+N3) by flexible power-sharing at the system level. Proposal 2: Support beam-level time domain configurations to flexible sharing of the DL transmission power. |
| R1-2404692 | Google | Proposal 1: Support to indicate the EPRE ratio between the PDCCH and TRS to facilitate the AGC for PDCCH reception. Proposal 2: Consider the PDCCH repetition operations for some PDCCH candidates in a search space for PDCCH coverage enhancement. Proposal 3: Support to indicate the power offset between PDSCH and TRS by DCI with regard to dynamic power sharing. Proposal 4: Compared to the coverage enhancement for PDSCH scheduled by fallback DCI, the coverage enhancement for PDSCH scheduled by DCI format 1_1/1_2/1_3 should be deprioritized. Proposal 5: Support to transmit the SSB by multiple repetitions - At least the number of repetitions for SSS and PBCH within an SSB should be the same |
| R1-2404694 | Sharp | Proposal 1: Link level results can be used to determine the naximum number of simultanouse transmitted beams for each channel type. Proposal 2: Beam groups should be defined based on the determined maximim number of simultaneouse beams for SSBs and PBCHs, etc. Proposal 3: An SSB index may be configured/assocaitated with a beam group/pattern, which may be transparent to NTN UEs. Proposal 4: For RACH procedure, methods of beam determination/reduction should be considered within a beam group to reduce the number of simultaneous beams for PDSCH Msg2, and Msg 4. Proposal 5: Beam group/patterns can be defined for efficient gNB scheduling. A distributed beam pattern can reduce the interference among simultaneous transmitted beams. Proposal 6: Active simultaneous transmitting beams can be limited within a beam group at any given time. DL monitoring pattern can be introduced to minimize the UE monitoring periods. |
| R1-2404784 | ETRI | Observation 1: The CNRs for the satellite payload parameters Set 1-1, Set 1-2, and Set 1-3 are equal to -1.9 dB, -1.9 dB and -9.9 dB respectively when steering loss is not considered. Observation 2: LEO600km Set 1-3 in FR1 provides 8dB lower CNR value compared to Set 1-1 & Set 1-2 regardless of the values of steering losses. Observation 3: The additional reference satellite parameters scenarios for LEO600km Set1-1 and Set1-3 in FR1 can satisfy the SSB periodicity of 20 ms. Observation 4: The additional reference satellite parameters scenarios for LEO600km Set1-2 in FR1 cannot satisfy the SSB periodicity of 20 ms. Observation 5: Increasing the time period of common signals can accommodate the 1058 cells with the additional reference satellite parameters for LEO600km Set1 in FR1, as well as reduce overhead. Proposal 1: RAN1 to consider modifying the parameters of LEO600km Set 1-2 in FR1. Proposal 2: RAN1 to study whether the longer time period for common signals is necessary. |
| R1-2404789 | Baicells | Observation 1: Lower CNR means lower MCS or lager repetition factor/aggregation level, and does mean capacity gain in the NTN DL power sharing scenario. Observation 2: For NTN DL coverage, the working point of CNR can be optimized according to power sharing schemes and should not necessarily be a very low value. Link level coverage enhancement to achieve a very low working point of SNR is not mandatory especially for UE-specific channels due to no capacity gain. Proposal 1: For NTN DL coverage evaluation, coverage ratio (i.e., the percentage of served beam footprints in the total number of beam footprints) should be one of the key metrics. Proposal 2: For NTN DL coverage evaluation, beam hopping mechanism should be considered because it directly affects the coverage ratio. Observation 3: For NTN DL coverage, coverage ratio for common messages should be no less than that for user traffic, and therefore is more challenging and needs more attention. Observation 4: Limited to the legacy specification, the maximum coverage ratio for SSB in a cell is 40%, 6%, and 40% for FR1 Set1-1, Set1-2, and Set1-3 respectively. Observation 5: From a more practical point of view, say there is at most 32 simultaneously active beams provided by the satellite payload, then the coverage ratio for the common messages is no more than 12%. This would be a major concern. Proposal 3: For NTN DL coverage enhancement, coverage ratio for common messages should be enhanced. |
| R1-2404861 | OPPO | Proposal 1: For system level study based on analytical evaluation: - The dwell time and revisit time of the beam footprints in state “off” are 0. - The dwell time and revisit time of the beam footprints in state “common message only” and “active traffic” are reported by companies. Proposal 2: For SIB coverage evaluation, the combination of 8 SIBs is considered in the link level simulation. Proposal 3: RAN1 to consider link-level enhancement of PDCCH and Msg4 PDSCH. Observation 1: For DL coverage evaluation at system level, the majority of beam footprints are in state “off”. Observation 2: For LEO600 set 1-1/set 1-3, the coverage ratio of the beam footprints in state “common message only” and state “active traffic” can achieve 100% when 80%-90% beam footprints in state “off” is assumed. - The coverage ratio larger than 100% implies that satellite resource exceeds demand. Observation 3: For LEO600 set 1-2, most of scenarios can achieve 100% DL coverage except the high cell load scenario. - Extending the revisit time or increasing satellite beam number per cell can be considered to improve the DL coverage at system level. - Extending the revisit time faces backward compatibility issue. - When the revisit time of the N3 beam footprints in state “active traffic” is 160ms, the coverage ratio can achieve 100% for all scenarios. Observation 4: For LEO600 set1-1 and set1-2, there is no coverage issue for DL physical channels and at least 2.6 dB link margin is obtained. Observation 5: For LEO600 set1-3, PDCCH and Msg4 PDSCH present 4.5 dB and 5.4 dB coverage performance gap respectively to be compensated, so link level enhancements are needed. |
| R1-2404916 | IIT Kharagpur, CEWIT | Observation 1: It is important to sweep given coverage area with all the beams at least for common channels even though simultaneous active beams are less. This is necessary to avoid coverage blackout for the UEs in some cells. Wider beams can solve this issue. Proposal 2: For NTN system level enhancement for the DL coverage enhancement, at least for common channel wider beams can be used. - FFS Effect on the link margin and possibility to overcome using link level enhancements. Proposal 2: For Earth fixed NR-NTN cells create a table of weights where the weights are precalculated for entire satellite Path for the Targeted earth fixed NR-NTN Cell |
| R1-2405057 | NTT DOCOMO, INC. | Observation 1: - What should be simulated/enhanced for dynamic and flexible power sharing is unclear. - Satellite/NW implementation can achieve “power sharing among satellite beams or different satellite beam patterns/size across the satellite footprint” in a practical EIRP, with R18 NES techniques if necessary. Proposal 1 (for conclusion): - With respect to R18 NES techniques, no system-level enhancement beyond them is necessary from RAN1 perspective. Proposal 2: - For discussion on NTN DL coverage enhancement, prioritize Set 1-1 FR1. Proposal 3: - If longer SSB periodicity is within the scope, discuss what is the specification/UE impact beyond the existing specification. Observation 2: - Even though there is no performance gap compared to CNR in Set 1-1, there may not be sufficient margin to reduce EIRP per beam for more active beams and/or to apply wider beam. Proposal 4: - Required SNR evaluated by LLS is compared to the required SNR for SSB as well as CNR. - The required SNR for SSB is -10.1 dB if combining 4 shots is feasible; otherwise, -4.1 dB can be used instead. Proposal 5: - R19 NTN DL coverage enhancement introduces link level enhancements for: - Msg2 PDSCH; Msg4 PDSCH; SIB1 PDSCH; SIB19 PDSCH; Paging PDSCH; PDCCH Proposal 6: - R19 NTN DL coverage enhancement introduced for SIB1 / SIB19 is not restricted to SIB1 / SIB19 only. |
| R1-2405090 | MediaTek Inc. | SSB periodicity: Observation 1: For narrow beams with NSSB=4 Beam hopping: - Set 1-2: SSB periodicity of 320 ms can provide coverage of 96.8% and efficiency per beam of 96.9%. - Set 1-1/1-3: SSB periodicity of 80 ms can provide coverage of 96.6% and efficiency per beam of 87.5%. Observation 2: For wide beam covering 4 narrow beams with NSSB=4 Beam hopping: - Set 1-2, SSB periodicity of 80 ms can provide coverage of 96.8% and efficiency per beam of 96.9%. - Set 1-1/1-3, SSB periodicity of 80 ms can provide coverage of 100% and efficiency per beam of 87.5%. Observation 3: Using wide beam covering N narrow beams allows to reduce SSB periodicity by a factor N. Observation 4: The DL synchronization latency should be at least smaller than the typical on-coverage time of a narrow beam to avoid systemic failure. Link-level evaluation of SSB: Observation 5: For SSB evaluation, the following was observed with impairments: - NR PSS: 1 1% detection failure is met at SNR=-4.5 dB and -10.3 dB for 1 UE Rx antenna element, "1-shot" or "4 combining of SSB instances within 20 msec" respectively - NR PBCH: 1% BLER for 1 UE Rx antenna element is met - at SNR=-7.6 dB, 1-shot. - at SNR=-12.2 dB, with SSB periodicity 20 ms and 4 SSB combining over 80 ms (=4*20 ms). - at SNR=-11.8 dB, with SSB periodicity 80 ms and 4 SSB combining over 320 ms (=4*80 ms). - at SNR=-11.7 dB, with SSB periodicity 320 ms and 4 SSB combining over 1280 ms ms (=4*320 ms). Observation 6: In case the UE does 4 SSB combining within 20 ms, around 0.1 dB SNR gain can be achieved compared to periodicity 20 ms and 4 SSB combining (20 ms=4*20). This may require specification change to clarify the UE behaviour, where the UE assumes NSSB=4 Beam hopping of SSBs per SSB default periodicity 20 ms with only one beam (i.e. co-located 4 beams with same beam diameter, same PBCCH payload, and different beam index used in DM RS). Proposal 1: Support change in UE behaviour, where the UE assumes NSSB Beam hopping of SSBs per SSB default periodicity 20 ms with only one beam (i.e. co-located NSSB beams with same beam diameter, same PBCCH payload, and different beam index used in DM RS). Physical layer procedures for ON/OFF beams: Proposal 2: RAN1 to study potential impact of beams ON/OFF on procedures for DL synchronization, acquisition of system information for configuration of beams, and initial cell access via RACH procedure. Proposal 3: RAN1 to study potential impact of beams ON/OFF on paging procedures. Proposal 4: Wide beam should be ON to at least transmit CD-SSB and SIB1, and CORESET 0. Otherwise, there is no coverage on these wider beams, and no coverage on narrow beams within the wide beam. Proposal 5: After initial DL synchronization and system information acquisition, initial cell access can be done on the wide beam re-using legacy specifications, or on narrow beams with NCD-SSBs. |
| R1-2405117 | Panasonic | Observation 1: To achieve a uniform beam layout on Earth's surface a fully digital beamforming satellite payload is needed. Observation 2: According to our link level simulation, the following observations are made: - For set 1-1 and set 1-2 (Target CNR is -1.9dB) - All evaluated channels satisfy the target without repetitions. - For set 1-3 (Target CNR is -9.9dB) - 8 repetitions are necessary for PDSCH VoIP and PDSCH Msg4. - PDCCH with 24PRB and AL8 (because of 5MHz bandwidth) can not meet the target. - SIB1 (800 bits), SIB1 (1280 bits) and SIB19 (616 bits) can not meet the target. Multiple transmissions or repetitions (8 transmissions for SIB1 and 4 transmissions for SIB19) are necessary. Proposal 1: RAN1 to clarify that UV-based mapping of hexagonal cells are not a suitable simulation approach for the envisaged beam-hopping scenario. Proposal 2: RAN1 should discuss whether a fully digital RF-frontend for beamforming is typical given the constraints of a satellite, in particular of a smaller LEO-satellite. Proposal 3: Study the feasibility of active satellite beam switching. This includes semi-static switching and dynamic switching, e.g., every slot. Proposal 4: RAN1 should explore the connection between beam-hopping patterns and Cell DTX/DRX patterns. Proposal 5: RAN1 to explore the impact of DTX/DRX per SSB beam and related UE IDLE/INACTIVE mode UE behavior. Proposal 6: RAN1 may explore the need of on-demand beam-hopping interaction with R19 NES work item. Proposal 7: Although three satellite parameter sets are defined for the evaluation, parameter sets to be supported in Rel.19 should be carefully discussed. At least parameter set 1-1 should be prioritized. |
| R1-2405172 | Qualcomm Incorporated | Observation 1: In NR NTN, UE reception of SIBs before the acquisition of satellite ephemeris (SIB19) may suffer larger timing and frequency offsets than in TN. The issue becomes more severe if SSB transmission periodicity is increased. Observation 2: To support Set1-3, coverage enhancement is needed at least for the following channels: - PDCCH - PDSCH carrying SIBs Observation 3: At 1% BLER, removing the reserved bits for the PDCCH with CRC scrambled by SI-RNTI can improved the performance by about 1.3 dB for NR NTN TDL-C. Observation 4: Rel-18 cell DTX does not allow flexible adaptation of cell DTX patterns. Proposal 1: Consider coverage enhancements for PDSCH schedule by a PDCCH in a CSS. - At least for PDSCH scheduled by PDCCH format 1_0 with CRC scrambled by SI-RNTI. Proposal 2: Consider coverage enhancement for PDCCH. Proposal 3: Consider coverage enhancement for the PDCCH that schedules a SIB1 PDSCH without NW signaling. Proposal 4: Support the configuration of more than one cell DTX patterns to a UE. Proposal 5: Support dynamic group-common signaling for the change of cell DTX patterns. |
| R1-2405226 | NICT | Observation 1: P Performance improvement can be expected by DL reception by multiple UEs with no major specification impact. Observation 2: Allowing flexible setting of beam dwell time and revisit time, more efficient beam management becomes possible according to traffic demand of each beam. Proposal 1: DL reception by multiple UEs is to be considered as one of the DL coverage enhancement solutions. Proposal 2: Rel 19 system level enhancements for FR1-NTN and/or FR2-NTN should support flexible setting of beam dwell time and revisit time. |
| R1-2405251 | CEWiT | Observation 1: The simulation results yield a required SNR value of -5.38dB for 1% target BLER of PDCCH with 8 CCE aggregations. Observation 2: For PDCCH with AL 8 has a coverage gap of -4.52dB for the set 1-3 parameters with LEO 600 satellites. Proposal 1: PDCCH with 8 CCE aggregations can provide sufficient link margin for set 1-1 and set 1-2 parameters. Hence, it is not necessary to do coverage enhancement for LEO 600 with set 1-1 and set 1-2 parameters. Proposal 2: Coverage improvement is needed for set 1-3 for LEO 600 due to a coverage gap of -4.52 dB observed in PDCCH with 8 CCE aggregations. Proposal 3: In the NTN DL coverage enhancement, link level improvement should be considered, considering reduced EIRP per beam for practical deployment case. Link level enhancement should target 5 dB improvement at least for the PDCCH channel. Proposal 4: The maximum number of repetitions for PDCCH can be extended beyond two for NR NTN DL coverage enhancements. Proposal 5: The higher aggregation level can be supported, such as 32 and 64, to improve the link margin of PDCCH for DL coverage enhancement of NR NTN. Observation 4: Supporting dynamic aggregation levels for each PDCCH repetition can yield diversity gain which leads to DL coverage enhancements in NR NTN. Proposal 6: For PDCCH, dynamic aggregation level per repetition can be configured to improve the DL coverage of NR NTN. Proposal 7: Each DCI format can be segmented depends on the size of the DCI to provide better coverage for NR NTN Proposal 8: Study the effect of choosing the number of repetitions, aggregation level and number of segmentations based on the elevation angle, for PDCCH, to improve both coverage and spectral efficiency of NR NTN. Proposal 9: Study the adoption of number of DMRS based on the elevation angle to improve the spectral efficiency in NR NTN. Proposal 10: The maximum number of repetitions can be increased to 16 or 32 to improve the coverage of NR NTN. Proposal 11: For Rel-19 NR NTN DL coverage enhancements, develop a beam hopping methods which minimizes the adjacent beam interference. Proposal 12: System level enhancements for NTN DL coverage enhancement, at least for N2 beams consider following options: Option 1: Increase the SSB periodicity more than 20 ms. Possible values are 80ms, 160ms. - Backward compatibility issues should be considered. Option 2: Increasing the number of beams in an SSB burst. Option 3: Increasing the beam width to cover the desired coverage area with possible SSB beams - Necessary link gain loss should be compensated by link level enahcements. Option 4: Support Cell DRX/DTX from Rel-18 NES to turn ON/OFF the beams as a system level enhancement to improve the DL coverage for Rel-19 NR NTN. - FFS: any NTN specific modification necessary. Proposal 13: Support on-demand beam activation when the UE has traffic to transmit to improve the DL coverage for Re-19 NR NTN. |
| R1-2405257 | CSCN | Proposal 1: A comprehensive beam hopping and beam management plan should be considered with respect to both the network accessing for all users in a long term and the traffic demand for certain users in a short term. Proposal 2: The extended periodicity of common signals/channels such as SSB/SIBs could be considered for downlink coverage enhancement at a system level. Proposal 3: To apply Rel-18 network energy saving techniques to system level downlink coverage enhancement, at least the adaptation of signals/channels and cell DTX/DRX mechanism should be considered. Proposal 4: For PDSCH Msg.2 and Msg.4, the coverage enhancement by PDSCH repetitions could be considered. |
| R1-2405263 | Nokia, Nokia Shanghai Bell | Observation 1: Areas covered by N1 cells without being covered by other cells will be considered without coverage. Observation 2: The existing framework of NES cannot be used to which cells completely off and new signalling/procedures are required. Observation 3: Utilising NTN downlink coverage enhancement suitable NES including turning SSB transmission off or increasing periodicity, will trigger a cell (re)selection procedure in legacy UEs and thus compromising energy consumption. Observation 4: New signalling/procedures to handle UEs in RRC_IDLE/ RRC_INACTIVE will only work for Rel19 UEs and will not be backwards compatible, leading to potential increased energy consumption. Observation 5: SSB is out of scope for the DL coverage enhancements but should establish the baseline for evaluation of channels in potential need for DL coverage enhancements. Observation 6: The channels that may need DL coverage enhancements are PDCCH AL8, PDSCH carrying SIB1 and PDSCH carrying Msg4. Observation 7: For PDSCH Msg4, there is little room to expand the frequency-domain resources due to large TBS and low code rate. Observation 8: For configuration or indication of repetition factor of Type-0 PDCCH, transmission prior to SIB1 transmission must be considered such as MIB. Proposal 1: RAN1 to prioritize FR1 in the downlink coverage enhancements studies for Rel-19. Proposal 2: RAN1 to discuss what is the maximum amount of area which can be out of coverage. Proposal 3: RAN1 to adopt the above state diagram for studying the state changes and consider different solutions for evaluating coverage. Proposal 4: It is RAN1s understanding that legacy UEs will not be compatible with any new DL coverage enhancement signalling procedures. Proposal 5: RAN1 to focus on PDSCH carrying SIB1 for DL coverage enhancements. Proposal 6: RAN1 to focus on PDSCH carrying Msg4 for DL coverage enhancements. Proposal 7: RAN1 to evaluate different approaches for improving DL coverage for PDSCH for initial access. Proposal 8: RAN 1 to focus on PDCCH for common channels (PDCCH Type 0/1) for DL coverage enhancements. Proposal 9: The methods of signaling for PDSCH conveying SIB1 repetition and time resource determination need further studies. Proposal 10: RAN1 to discuss solutions for UE to report assistant information for aiding gNB scheduling of Msg4 repetition factor. Proposal 11: For coverage improvement of common PDCCH (both Type-0 and Type-1), and to take backward compatibility into account, RAN1 considers time-domain repetitions as a potential solution. Proposal 12: Type-0 PDCCH repetitions may be performed inter-slot or intra-slot with repetitions factor configure/indicated via MIB. Proposal 13: For Type-1 PDCCH with CRC scrambled with RA-RNTI, repetition factor is configured or indicated via SIB1. |

# Appendix II

RAN1#116bis made the following agreements:

Agreement

For coverage evaluation of PDCCH in NR NTN, the following table is assumed:

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Aggregation level | 8 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 24 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER optional for 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PDSCH in NR NTN, the following table is assumed:

| Parameter | Value |
| --- | --- |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 for 2GHz |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/TBS/MCS for data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. 24 PRBs for SIB1 and SIB19 |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK |
| PDSCH duration | 12 OS |
| Payload size for PDSCH of Msg.2 | 72 bits |
| Payload size for PDSCH of SIB1 | FFS |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| Payload size for PDSCH of SIB19 | FFS |
| Other parameters | Reported by companies. |

Agreement

Antenna gain reduction due to steering loss is not considered in the link level evaluation.

Note: This is aligned with the assumptions made in Rel-18 UL coverage enhancement

Observation

The CNRs for the satellite payload parameters Set 1-1, Set 1-2 and Set 1-3 are equal to -1.9 dB, -1.9 dB and -9.9 dB respectively.

Agreement

Confirm the Satellite phased-array antenna parameters for LEO 600km in FR1 defined in RAN1#116.

| Satellite phased array antenna Characteristics |  |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | Table7.3-1 in TR 38.901 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 65 for H 65 for V |
| Antenna element spacing | 0.667 lambda |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | 400 elements (20 x 20) |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi |
| Steering loss at 30° elevation angle | 4 dB |

Al least the above model is considered for SLS to ease the alignment between evaluation results. The model below can be optionally considered:

| Satellite phased array antenna Characteristics |  |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | TR38.820 section 7.2.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90 for H 90 for V |
| Antenna element spacing | 0.5 lambda |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | 676 elements (26 x 26) |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi (Note 1) |
| Steering loss at 30° elevation angle | 2.5 dB |

Note 1: The maximum antenna gain is determined by considering an overall array efficiency [of 50%.]

Agreement

For coverage evaluation of PDSCH in NR NTN, the following payload sizes for PDSCH are assumed:

| Payload | value |
| --- | --- |
| Payload size for PDSCH of SIB1 | Option 1: 800 bits Option 2: 1280 bits |
| Payload size for PDSCH of SIB19 | 616 bits |

Note: At least the above values are simulated and reported. Other values can be considered.

Note: the values above are not the TBS.

Agreement

For DL coverage study at system level, it is up to companies to report the following parameters for LEO600km Set1-1 FR2:

Beam size

Satellite EIRP density /beam (dBW/MHz)

Payload Total DL power level (dBW)

Aggregated EIRP (Total) (dBW)

Satellite Tx max Gain

EIRP per Satellite beam (dBW)

Agreement

For coverage performance evaluation of DL channels/signals before the SIB19 acquisition, the maximum Doppler frequency drift is assumed to be equal to 0.27 ppm/s based on TR 38.821.

RAN1#116 made the following agreements:

Agreement

For DL coverage study, consider the following additional reference satellite parameters scenarios for LEO600km Set1 in FR1 (i.e., S-band), referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1:

| LEO600km Set1-1 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size(Note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 31.24 |
| Aggregated EIRP (Total) (dBW) | 61.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints*** | 1058 |
| Total number of simultaneously active beams ** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set1-2 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 23 |
| Aggregated EIRP (Total) (dBW) | 53* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 16 |
| % simultaneously active beams** | 1.5 % |
| *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set 1-3 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 26 |
| Payload Total DL power level (dBW) | 23.24 |
| Aggregated EIRP (Total) (dBW) | 53.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 33 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

Note: RAN1 will aim to identify necessary enhancements for these scenarios in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage study at system level, consider the following additional reference satellite payload parameters for LEO600km in FR2 (i.e., Ka-band):

| LEO600km Set1-1 FR2 (i.e., Ka-band) |
| --- |
| Maximum Bandwidth per beam | 400 MHz |
| SCS | 120 kHz |
| Beam size | TBD in next meeting |
| Satellite EIRP density /beam (dBW/MHz) |  |
| Payload Total DL power level (dBW) |  |
| Aggregated EIRP (Total) (dBW) |  |
| Satellite Tx max Gain |  |
| EIRP per Satellite beam (dBW) |  |
| Total number of beam footprints | 800 (note 1) |
| Total number of simultaneously active beams | 12 |
| % simultaneously active beams | 1.5 % |
| Note 1: A typical deployment scenario in FR2 should consider 800 satellites beams per a single satellite coverage area with an absolute number of simultaneously active beams equal to 16 (due to limitation of RF) |  |

Agreement

Adopt the following phased array antenna parameters for LEO 600km in FR1:

| Satellite phased array antenna Characteristics | LEO-600 |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | Table7.3-1 in TR 38.901 |
| Horizontal/vertical 3 dB beam width of single element (degree) | [65] for H [65] for V |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | [400 elements (20 x 20)] |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi |
| Steering loss at 30° elevation angle | [4dB] |

Agreement

RAN1 to consider the following performance metrics for DL Coverage enhancement evaluation at system level:

At least:

- CDF of the received SINR

- The dwell time and revisit time interval for each beam illumination across the coverage

- Periodicity of common control channels (e.g. SSB, CORESET0/SIB1, SIB19) and corresponding coverage ratio

Other metrics may be reported such as

- CDF of the cell throughput

- CDF of user perceived throughput (UPT)

- CDF of Latency

- Ratio of mean served cell throughput and offered cell throughput, denoted by 𝜌 (refer to TR36.889)

For system level study based on analytical evaluation:

- N1 beam footprints are in state “off”

  - These beam footprints are not served by any signal (no satellite service in this area)

- N2 beam footprints are in state “common messages only”

  - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access.

  - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation

- N3 beam footprints are in state “active traffic”

  - These beam footprints have X active (e.g. VoNR) users each.

  - These beam footprints are also served the necessary information for cell discovery and initial access

- N1 + N2 + N3 = “Total number of beam footprints “

- N1, N2, N3, X are to be reported by companies.

- Resource utilization obtained under the assumptions above is to be reported by companies.

- Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

Agreement

For NR NTN Rel-19 DL coverage evaluation, UE characteristics for handheld terminals in Table 6.1.1.1-3 in TR 38.821 can be reused, with the following:

- -5.5 dBi antenna gain is assumed

- at least 2Rx are considered at the UE

  - 4Rx can be optionally considered and reported

Note: Redcap device is not considered in the scope of DL coverage study

Agreement

The following traffic models are considered for system level evaluation of DL coverage:

- FTP3: as in Table 6.1.1.1-7 of TR 38.821: 0.5MB as packet size, 200ms as mean inter-arrival time

- FTP3 IM: 0.1MB as packet size, 2s as mean inter-arrival time

- VoIP can be considered in the evaluation.

It is up to company report which traffic model is used among the discussed traffic models in their evaluations.

- Other models may be used as well, and parameter (e.g. packet size and arrival rate) adjustment can be optionally considered and reported.

| Traffic type | FTP | IM | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in Rel-18 NTN CE. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec |  |

Agreement

For NR NTN Rel-19 DL coverage evaluation, Beam layout defined in Table 6.1.1.1-4 in TR 38.821 can be reused.

- Using other beam layouts is not precluded, and should be reported by companies

Agreement

For NR NTN Rel-19 DL coverage evaluation, a value of beam steering latency equal to 0 at least if phase array antenna is assumed.

Values different from 0 can be optionally reported

Agreement

DL coverage is evaluated at link level with the following considerations:

- NGSO at LEO-600 operating in FR1 is considered in priority

- Additional satellite payload parameters defined for system level evaluation are used

- FFS: Antenna gain reduction due to steering loss can be considered

Agreement

For the evaluation of NTN downlink coverage at link level, reuse the target data rate from Rel-18 NTN Coverage enhancements:

- For VoIP: AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval

- For data rate service: both 3 kbps and 1Mbps can be considered

  - Companies can also use the data rates corresponding to the traffic types used for system level evaluations

Agreement

For link-level study, downlink coverage performance in NR NTN is evaluated according to the following steps.

Step 1: CNR is calculated as defined in 6.1.3.1 of TR 38.821

Step 2: Required SNR of target service is evaluated by LLS

Step 3: The CNR and the required SNR are compared

Agreement

For link-level study, for NR NTN DL coverage enhancement, the following channels/signals can be considered for evaluations:

- PDSCH for VoIP

- PDSCH for low data rate service

- PDSCH Msg.2

- PDSCH Msg.4

- PDSCH carry SIB, e.g., SIB1, SIB 19

- PDSCH for paging

- PDCCH

- Broadcast PDCCH (e.g. PDCCH of Msg.2, paging)

- SSB

Note: RAN1 will aim to identify necessary link-level enhancements for these channels in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential link-level enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread:

  - Channel model as in Table 6.1.2-4 of TR38.821, NTN-TDL-C (LOS)

- Evaluation scenario:

  - Rural (LOS)

- Channel estimation: Realistic estimation:

  - Companies are encouraged to report channel estimation method.

- SCS:

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: TBD

- Frequency offset: 0.1 ppm

Agreement

For link budget calculation, parameters in the following table are assumed:

| Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz for DL (S-band) |
| Satellite altitude | 600 km |
| Target elevation angle | 30° (LEO) |
| Atmospheric loss | Equation (6.6-8) in [38.811] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [38.811] Ionospheric loss: = 2.2 dB Tropospheric loss: Table 6.6.6.2.1-1 of [38.811] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| UE antenna gain | -5.5dBi |
| Free space path loss | Equation (6.6-2) in [38.811] |
| Polarization loss | 3dB |
| Outcome | CNR |

## References

| - R1-2403938 | Discussion on downlink coverage enhancements for NR NTN | Huawei, HiSilicon |
| --- | --- | --- |
| - R1-2403989 | On NR-NTN downlink coverage enhancement | Ericsson |
| - R1-2403993 | Further considerations on FR2-NTN analysis assumptions | Eutelsat Group |
| - R1-2404003 | Discussion on downlink coverage enhancements for NR NTN | Fraunhofer IIS, Fraunhofer HHI |
| - R1-2404041 | Discussion on NR-NTN downlink coverage enhancement | Spreadtrum Communications |
| - R1-2404132 | Discussion on downlink coverage enhancement for NR-NTN | Samsung |
| - R1-2404194 | Discussion on NR-NTN downlink coverage enhancement | vivo |
| - R1-2405342 | Discussion on NR NTN Downlink coverage enhancements | THALES, Magister |
| - R1-2404214 | Discussion on DL coverage enhancement for NR NTN | ZTE |
| - R1-2404261 | NR-NTN downlink coverage enhancement | InterDigital, Inc. |
| - R1-2404307 | Discussion on NR-NTN Downlink Coverage Enhancement | Apple |
| - R1-2404323 | Discussion on NR-NTN downlink coverage enhancement | LG Electronics |
| - R1-2404390 | Performance evaluation of downlink coverage enhancement for NR NTN | CATT |
| - R1-2404441 | Discussion on downlink coverage enhancement for NR NTN | Lenovo |
| - R1-2404471 | Discussion on NR-NTN DL coverage enhancement | CMCC |
| - R1-2404607 | Discussion on NR-NTN downlink coverage enhancement | Xiaomi |
| - R1-2404670 | NR-NTN downlink coverage enhancement | NEC |
| - R1-2404692 | Downlink Coverage Enhancement for NR NTN | Google |
| - R1-2404694 | Beam group for NR-NTN downlink coverage enhancement | Sharp |
| - R1-2404784 | Discussion on NR-NTN downlink coverage enhancement | ETRI |
| - R1-2404789 | Discussion on downlink coverage enhancement for NR NTN | Baicells |
| - R1-2404861 | Discussion on NR-NTN downlink coverage enhancement | OPPO |
| - R1-2404916 | Discussion on NR NTN Downlink Coverage Enhancements | IIT Kharagpur, CEWIT |
| - R1-2405057 | Discussion on DL coverage enhancement for NR-NTN | NTT DOCOMO, INC. |
| - R1-2405090 | Discussion on NR-NTN downlink coverage enhancement | MediaTek Inc. |
| - R1-2405117 | NR-NTN Downlink Coverage Enhancement | Panasonic |
| - R1-2405172 | Downlink coverage enhancement for NR NTN | Qualcomm Incorporated |
| - R1-2405226 | Discussion on DL coverage enhancements for NR-NTN | NICT |
| - R1-2405251 | Downlink Coverage Enhancements for NR NTN | CEWiT |
| - R1-2405257 | Discussion on downlink coverage enhancement for NR-NTN | CSCN |
| - R1-2405263 | Downlink coverage enhancements for NR over NTN | Nokia, Nokia Shanghai Bell |