3GPP TSG RAN WG1 #117									R1-2404509

Fukuoka, Japan,  20 – 24 May 2024

Agenda Item 	:	9.6.1

Source 	:	Sony

Title 	:	LP-WUS and LP-SS design

Document for 	:	Discussion and decision

# Introduction

A new, release 19, work item on low-power wake-up signal and receiver (LP-WUS/WUR) was approved, based on the SI outcome. One of the objectives is,

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported. - For IDLE/INACTIVE modes - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). This objective is to be further refined in RAN#103

In this document, we discuss OOK (OOK-1 and OOK-4) based LP-WUS and LP-SS, and describe our view on overlay OFDM sequences over OOK symbols.

# LP-WUS design

To specify a LP-WUS design applicable for both IDLE/INACTIVE and CONNECTED mode, we need to understand the potential subsequent actions after LP-WUS detection.

LP-WUS structure and its content

In IDLE/INACTIVE state the LP-WUS transmitted can be used to power up the main receiver before paging/PEI or the transmitter to initiate PRACH directly. In order to avoid false wake-up of the UEs, for the former a sub-grouping of the UEs listening to the same paging DRX-ON can be done, while for the latter individual UEs need to be woken up separately. In CONNECTED mode, on the other hand, the potential sub-sequent action not only can be related to the monitoring of the next DRX-ON but also whether to advance or delay the DRX-ON monitoring due to jitter in the XR applications. Again, it means different content/sequences are needed to wake-up the UE for different purposes. In the CONNECTED mode, the UE can only be addressed individually. Therefore, the LP-WUS design should at least allow for

- different content/sequences for different subsequent actions/purposes, and

- addressing of group of UE or individual UEs, depending on the mode of operation.

These two pieces of information can be included as a data field in the LP-WUS structure. Time and frequency multiplexing can further be used to further sub-group the UEs.

Additionally, to allow for fine synchronization and to prevent the NW from always-on transmission of LP-SS with short periodicity, it is beneficial that the LP-WUS also includes a preamble prior to its data information. Another advantage of adding such a preamble is reduced power consumption at the UE while monitoring for LP-WUS. The LP-WUR first needs to only look for the preamble and only continues to detect the data if the preamble is detected. Figure 1 shows the general signal structure used in this contribution containing sequences of length M and L bits, representing preamble and data, respectively, where data part is to indicate the subsequent action by the UE after LP-WUS detection and/or identity of (group) UE.

Figure 1 - LP-WUS structure, consisting of two fields, preamble used for synchronization and possibly cell identity purposes and data part to indicate the potential subsequent action or UE (group) identity.

As described earlier, Time and frequency multiplexing can further be used to further sub-group the UEs. By allowing to transmit the LP-WUS in smaller bandwidths, we can accommodate and address higher number of UEs in the time-frequency grid. We describe this in more detail in our other contributions [2] and [3].

Proposal 1 – Support LP-WUS structure with two fields, a preamble field for synchronization and cell identification purposes and a data field for indication of subsequent actions and/or wake-up group identity, depending on state of the operation.

OOK-based LP-WUS

An OOK-based LP-WUS design allows for low-power low-complexity WUR designs with the power consumption in the range of 50 uW to 500 uW. Low-power low-complexity design of a LP-WUR, however, typically leads to a higher noise figure and degraded sensitivity, compared to the main receiver or an OFDM based LP-WUS receiver. Depending on the LP-WUR architecture, the level of power consumption vs noise figure and sensitivity can vary.

Using longer signal sequences containing more total energy and a suitable low power digital base-band processing, we can compensate for the worse performance and therefore allow for low detection error, i.e., mis-detection and false-alarm, probabilities. This lengthening of the signals can also be used for carrying additional information, beyond the pure wake-up trigger, as mentioned earlier. More specifically, we use the structure described in Figure 1 where we transmit sequences selected from sequences such as m-sequences with good correlation properties, which allows for low mis-detection and false-alarm probabilities. The address spaces of these two sequences are typically much smaller than the M and L bits in the sequences, providing a degree of spreading that reduces error probabilities.

To tailor LP-WUR/WUS scheme to the NR protocol, the LP-WUS transmission needs to be compliant with the transmission principles of NR, i.e., based on OFDM transmission. In other words, to avoid interfering with the OFDM transmission, the LP-WUS needs to be orthogonal in the same way as OFDM sub-carriers are, despite being based on different modulation principles. Two alternative OOK-based LP-WUS transmission schemes, namely OOK-1 and OOK-4, are selected after the Rel-18 study period.

The OOK-4 transmission scheme has advantages compared to the OOK-1 waveform generation candidates in terms of system overhead, latency and low-power consumption. Assume the LP-WUS is N bits long with BWx transmission bandwidth and Nx the number of transmitted bits in one OFDM symbol. The length of LP-WUS for OOK-4 becomes N/Nx times lower than the one in OOK-1. This results in lower system overhead, delay and total power consumption in OOK-4 compared to the OOK-1 candidate. Moreover, OOK-4 is well spread over available SCs which automatically makes it robust against frequency fading. OOK-4 only has a single non-zero amplitude level on the time domain signal, making its PAPR low, particularly compared to OOK-1 where all signal energy is maximally concentrated in time due to setting all available sub-carriers to one.

Observation 1 – OOK-4 has lower system overhead and can be detected with shorter delay using lower power LP-WUR, compared to OOK-1 candidates. Furthermore, OOK-1 only has a single non-zero amplitude level on the time domain signal, making its PAPR low.

Number of bits per OFDM symbol – The number of bits per OFDM symbol, based on the above agreement, is limited to 4. The number of bits per OFDM symbol, M, needs to be decided based on LP-WUS bit rate and according to channel delay spread, preventing ISI and the tolerable time/frequency errors by the LP-WUR. The number of bits that can be accommodated in one OFDM symbol can further be calculated knowing the SCS. For a LP-WUS bit rate of Rb, the number of bits per OFDM symbol M is equal to Rb/SCS.

Proposal 2 - The number of bits per OFDM symbol, M, cannot be a fixed value and needs to be decided based on LP-WUS bit rate and according to channel delay spread, preventing ISI and the tolerable time/frequency errors by the LP-WUR.

Below we give a detailed description of OOK-4 generation, where OOK-1 can be considered as a special case where number of bits per OFDM symbol is limited to one bit.

Figure 2 – Block diagram of modified OFDM transmitter (for OOK-4 transmission mechanism)[4]

Figure 2 shows a block diagram of the modified OFDM transmitter, including the two blocks used to create the embedded WUS, allowing OOK-4 transmission scheme. The first block (A) is a sectioning block creating sections of the target LP-WUS and is only used if the length of the LP-WUS becomes longer than one OFDM symbol. The WUS shaping block (B) then shapes each LP-WUS section so that the resulting signal as output from the OFDM modulator becomes as close to the target LP-WUS as possible using the designated sub-carriers. Step (C) to (D) adds CPs to maintain orthogonality in channels with time dispersion. Figure 3 illustrates the resulting signals after each block, no Manchester coding is applied in this example.

Figure 3 - Illustration of input/output signals of each block in the transmitter of Fig. 2, using a simple example.

In table 1 we list the design parameters of a studied OOK-based LP-WUS and some of the important properties of the LP-WUR. This LP-WUS design has the following characteristics:

- Kasami sequences of equal length M and L of 63 bits are selected to represent 504 cell identities and 16 group identities. This translates to spreading factors of 7 and 16 for the two parts of the 126-bit LP-WUS, which has a total time duration of 0.52 msec.

- The LP-WUS occupies 8 OFDM-symbols, 64 REs with carrier frequency spacing of 15 kHz.

- When performing this simulation, we have taken into account a drift compensation, chosen cautiously to be 10 times that of the ultra-deep sleep of the main receiver. A signal-to-noise ratio of 13dB are required for the TDL-C channel, to reach 98.6% detection and 10-5 false-alarm rate.

| LP-WUS design parameters | Values | Notes |
| --- | --- | --- |
| (Np, Nd) | (4,4) | M=63 bits, L=63 bits, corresponding to total 8 OFDM symbols |
| WUS duration(us) | 0.52 msec | LP-WUS of 8 symbols @ SCS 15e3 is 500us. |
| No. of bits per OFDM symbol | 16 bits |  |
| LP-WUS SCS | 15 kHz |  |
| Main signal SCS | 15 kHz |  |
| No. of Res used for LP-WUS transmission | 64 |  |
| LP-WUS Bandwidth | 1.08 MHz |  |
| Bit rate | 240 kbps | Manchester coded OOK |
| Guardband width | 240 kHz | 4 Res @ SCS 15e3 at each side |
| LP-WUR Parameters/characteristics | Values | Notes |
| Zero-IF architecture | yes | - |
| LP-WUR BW | 1.08 MHz | Butterworth, order of 2 |
| ADC | 1-bit |  |
| Noise figure | 12 dB |  |

Table 1 – OOK-based LP-WUS and LP-WUR design parameters

| Channel/Signal | MIL (dB) |
| --- | --- |
| PUSCH, msg3 | 146.27 |
| OOK-based LP-WUS | 144.09 @ SNR of 15dB |

Table 2 – Comparison of MIL for OOK-based LP-WUS and PUSCH, msg 3.

Observation 2 – With a LP-WUR design having a relatively poor LP-WUR BW properties and a 1-bit ADC, the OOK-based LP-WUS can reach the PUSCH, msg3 coverage with several dB difference.

Allowing a higher number of sampling rate or an ADC with a better resolution, we can improve the SNR performance reaching PUSCH msg3 coverage. Additionally, by using repetition, we can further improve the coverage for the devices and reach beyond the PUSCH coverage. With this, however, the LP-WUR needs to be on for a longer time period, resulting in total higher power consumption.

OFDM-based LP-WUS

An OFDM-based LP-WUS design can have the following advantages:

- There is more capacity to carry more signalling information, e.g., for use in identifying UEs or UE groups

- The RRM measurements regime does not have to change too drastically – measurements such as RSRP and RSRQ can be carried out on the existing PSS/SSS signalling.

- An OFDM LP-WUS can more easily deal with frequency and time offsets because it has to incorporate means of detecting and correcting these since a coherent receiver is needed.

- An OFDM LP-WUR operates in WUS windows as an OFDM LP-WUS would have similar coverage to the main signal and so can easily be detected within a short time window, this is particularly when the UE is within the coverage of PDCCH.

An OFDM LP-WUR, however, consumes more power than an OOK-based LP-WUR because of the need for coherent reception and I, Q processing. The increased power consumption of the OFDM based LP-WUR can be mitigated by:

Reducing the bandwidth of the LP-WUS compared to the bandwidth of the component carrier. The lower sampling rate engenders less power consuming DSP for the LP-WUR. It is expected that while the OFDM LP-WUR power consumption reduces drastically compared to its main receiver, its power consumption can still be more than several times higher than an OOK-based LP-WUR.

Modulating the OFDM symbols of the LP-WUS with known quadrature sequences and carrying the signalling data in some property of the modulating sequences instead of using QAM modulation of the OFDM subcarriers. This allows decoding of the signalling without the need for channel estimation, de-mapping, FEC etc.

Concentrating the whole LP-WUS in a few OFDM symbols so that the LP-WUR is only ON for a short time. This is particularly beneficial as the OFDM based LP-WUS can reach the coverage of PDCCH coverage without any need for repetition, compared to the OOK-based LP-WUS.

Figure 4: Sequence carried on each OFDM symbol for OFDM-based LP-WUS

Figure 4 illustrates how the dot product of a Cazac S(k) and m-sequence P(k) can be used to modulate LP-WUS OFDM symbols. By choosing P(k) based on the cell ID, the LP-WUS transmitted can be made cell-specific.

| Condition | Values | Notes |
| --- | --- | --- |
| (Np, Nd) | (4,8) | Number of preamble and data symbols |
| WUS duration(us) | 500 | WUS of (4,8) symbols @ SCS 30e3 is 500us. |
| LP-WUS SCS | 30kHz |  |
| Main signal SCS | 15kHz |  |
| L | 144 | Res |
| CP | 2.34us | Dependent on symbol duration @ 30kHz SCS |
| LP-WUS Bandwidth | 4.32MHz |  |
| Guardband width | 320kHz | D = 11 Res |
| LP-WUR Bandwidth | 5MHz |  |

Table 3: Configuration parameters for an OFDM-based LP-WUS

Table 3 illustrates the design parameters of an OFDM-based LP-WUS whose frequency domain symbols are in accordance with Figure 4. The LP-WUS has the following key characteristics:

- Comprised of 12 OFDM symbols – the first Np = 4 symbols carry the WUS preamble which does not carry any data – except that the PN sequence is chosen according to the CID of the cell transmitting the WUS. The preamble is used for synchronisation and identification of the cell. The next Nd = 8 symbols carry the LP-WUS signalling in a cyclic shift of the symbol in the time domain. Each symbol can carry up to 7 bits of signalling in this manner. Each of the Nd symbols can either carry a unique 7 bits signalling or some can be repetitions for coverage enhancement reasons. If all 7 Nd bits of data are not needed and/or coverage extension is not needed, the number of signalling symbols Nd can be reduced. The preamble can also fulfil the synchronisation requirements for LP-SS by transmitting the preamble at regular intervals with/without any signalling symbols. The transmission cycle of the preamble can be configured to match the shortest possible DRX ON cycle. At the end of every LP-SS cycle, the gNB assesses whether it has a LP-WUS to send to a group of UEs. If it does, then the signalling symbols of the LP-WUS are transmitted after the preamble. If the gNB does not have a LP-WUS for transmission to any group of UEs, then the gNB transmits only the preamble symbols and stops until the next cycle.

- With the above parameters, the LP-WUR operates on the same bandwidth as is used by the SSBs. The LP-WUR can therefore also be used to carry out RRM measurements on legacy SSBs. In this case, the SSBs can be used as LP-SS too.

Options for overlaying OFDM sequences with OOK

The length of the OOK-based LP-WUS can be made longer if it is required for the LP-WUS coverage to match the main signal (e.g., PDSCH) coverage, i.e, going beyond the PUSCH coverage. This contrasts significantly with the duration required to detect a LP-WUS based on OFDM as only a few OFDM symbols are required to carry all the signalling information and meet the same coverage as the main signal. So, while the envelope detection-based LP-WUR for an OOK-based LP-WUS consumes only a smaller amount of power per OFDM symbol, the total power consumed over the full duration of an OOK-based LP-WUS with similar coverage to the main signal may be close to that consumed by an OFDM-based LP-WUR which detects the LP-WUS only over a short time.

From the SI, the desire to overlay OFDM and OOK in the LP-WUS arises because some UEs with a higher battery capacity may want to receive the LP-WUS over only a few OFDM symbols whilst small form factor UEs with lower power density batteries may be content with receiving the LP-WUS over an extended period as the power consumption per OFDM symbol is minimal. So, the desire is to transmit both an OFDM-based and an OOK-based LP-WUS at the same time for UEs to choose which to detect in accordance with their capabilities.

- Separate OFDM and OOK WUS Symbols

The OFDM and OOK LP-WUS use different transmission resources. This can be done by multiplexing the two signals either in time or frequency as illustrated in Figure 5(a) and Figure 5(b), respectively. For the time multiplexing of Figure 5(a), the OOK LP-WUR based on envelop detection can use the energy of the REs of the preceding OFDM LP-WUS section within its bandwidth to initialise its gain controllers etc.

Figure 5: Time and frequency multiplexing options for OOK-OFDM LP-WUS Overlay

Separating resources of OFDM and OOK WUS symbols double the number of resources that are needed for LP-WUS transmission. However, these resources are not necessarily used at the same time as the LP-WUS are not always transmitted and the occupancy of the resources also depends on the number of UEs supporting OOK-based LP-WUR or OFDM-based LP-WUR in the cell.

- Superpose OFDM and OOK WUS Symbols

The time domain OFDM symbol is superposed with the time domain template of the ON/OFF keying symbol before transmission. Without Manchester coding for the OOK, only ON symbols are superposed with OFDM symbols – as illustrated in Figure 6.

Figure 6: Superposition of OFDM and OOK-based LP-WUS

As superposition with OFDM symbols is done only in the ON OOK symbols, the envelope detector of the OOK LP-WUR will still detect an envelope only for ON OOK symbols. For the OFDM LP-WUR, the OFDM receiver will be preceded with an envelope detector – only for OFDM symbols with a significant envelope are subjected to further OFDM processing.

When Manchester coding is applied to the OOK and the SCS of the OFDM LP-WUS symbol is larger than that used by the OOK, then the OFDM symbol duration has only a fraction of the time domain samples of the OOK symbol. Only the high amplitude part of the OOK symbol is superposed with the LP-WUS time domain OFDM symbol as illustrated in Figure 7 in which the SCS of the OFDM symbol is double the SCS of the OFDM based of the OOK symbol.

Figure 7: Superposition of OFDM and MC-OOK based LP-WUS

With Manchester coding applied to the OOK, to determine OOK ON/OFF, it is expected that the envelope detector of the OOK LP-WUR will detect whether the energy envelope is in the first or second half of the OOK symbol. For the OFDM LP-WUR, the OFDM receiver will be preceded with this ON/OFF envelope detector – then the OFDM receiver processes the relevant half of the symbol to detect the LP-WUS and extract the embedded signaling therein.

In each case, the OOK envelope detector of the LP-WUR can just ignore the superposition. The OFDM receiver of the LP-WUR needs to first determine the OOK bit (with the relevant envelope detector for example) before it can process the OFDM signal.

Discussion

During the study period, it was concluded that the target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message 3. For the coverage within PUSCH coverage, OOK-based LP-WUS and OFDM-based LP-WUS have similar signal length, i.e., a few OFDM symbols, however, the power consumption of the LP-WUR to detect the LP-SS and LP-WUS is several to ten times lower than the OFDM-based LP-WUR, which leads to a total lower power consumption at the UE. However, if the UE needs to operate in between PUSCH and PDCCH coverage, repetition is needed for OOK-based LP-WUS but this is not needed for OFDM-based LP-WUS. In this condition, the total power consumption of the two LP-WUS designs can become comparable. The disadvantage of such repetition in an OOK-based LP-WUS is that it results in higher LP-WUS transmission latency and lower resource and spectrum efficiency.  As mentioned earlier, some UEs with a higher battery capacity may want to receive the LP-WUS over only a few OFDM symbols whilst small form factor UEs with smaller battery capacity may be content with receiving the LP-WUS over an extended period as the power consumption per OFDM symbol is minimal. So, the desire is to transmit both an OFDM-based and an OOK-based LP-WUS at the same time for UEs to choose which to detect in accordance with their capabilities.

Observation 3 – There is a trade-off between using OOK-based LP-WUS and OFDM-based LP-WUS in terms of the number of resources, delay and the total power consumption. While the OOK-based LP-WUS leads to lower power consumption than the OFDM based LP-WUS, particularly where the UE operates in the coverage area below PUSCH extended coverage, the number of resources and the latency increases for the OOK-bases LP-WUS to cover areas beyond PUSCH extended coverage.

Proposal 3 – Support to transmit both an OFDM-based and an OOK-based LP-WUS at the same time for UEs to choose which to detect in accordance with their capabilities.

# OOK LP-SS design

During the study phase, several agreements were made on synchronization and RRM measurement. For a LP-WUR that cannot receive existing PSS/SSS, periodic LP-SS signal is beneficial for the following functionalities: (a) RRM measurements by LP-WUR, if supported (b) At least coarse time synchronization of LP-WUR. (c) At least coarse frequency synchronization of LP-WUR. Periodic LP-SS coverage should be equal or better than that of LP-WUS.

As discussed earlier, OOK-4 has lower system overhead and can be detected with shorter delay using lower power LP-WUR, compared to OOK-1 candidates.

Proposal 4 – Consider OOK-4 transmission scheme for the transmission of the LP-SS.

Number of bits per OFDM symbol – Similar to our earlier discussion the number of bits per OFDM symbol, M, cannot be a fixed value and needs to be decided based on LP-WUS bit rate and according to channel delay spread, preventing ISI and the tolerable time/frequency errors by the LP-WUR, calculated by dividing the LP-SS bit rate to the SCS value of the OFDM transmitter.

Based on the RAN1 study and evaluation methodology, UE power saving gain cannot be observed if the existing Rel-18 MR RRM measurement periodicity for serving and neighbour cells are applied and UE MR enters in ultra-deep sleep during LP LP-WUS monitoring, therefore

- MR serving and neighbour cell measurement with further time domain relaxation than that is allowed in Rel-18 specification for IDLE/INACTIVE and/or

- at least serving cell RRM offloaded from MR to LR are beneficial.

RAN4 confirmed that the evaluation methodology on that the RRM measurement can be offloaded to LP-WUR for IDLE/INACTIVE mode. RAN2 has studied and concluded the feasibility for RRM measurement relaxation (including no measurement) for serving cell by MR and neighboring cell by MR at least if RRM measurement on LR for serving cell is feasible/supported.

Figure 8 – LP-SS structure consisting of two cell ID fields

In order to use LP-SS for both synchronization and RRM measurement (at least serving cell) purposes, the LP-SS needs to contain information related to its cell ID. To allow for low-power reception using OOK-based LP-SS, we propose to again use sequences from m-sequence family with good auto- and cross-correlation properties. To enable as many cell ID number required by NR, we propose a LP-SS design based on OOK-4 transmission scheme where the LP-SS contains two fields. The combination of two sequences allows to fulfill the number of cell ID needed in NR. Similar to the LP-WUS, the address spaces of these two sequences are much smaller than the P and Q bits in the sequences, providing a degree of spreading that reduces error probabilities and improve on the LP-SS coverage. For example, by using the 63 Kasami sequences we can have access to 520 number of sequences. Using the combination of this, for instance 503 of them in the first field and 16 remaining in the second field we can fulfil the number cell ID required in NR.

Proposal 5 – Support LP-SS structure with two fields using OOK-4 transmission structure and sequences from m-sequence family with good auto- and cross correlation properties.

As discussed earlier, for the OFDM based LP-WUR, measurements such as RSRP and RSRQ can be carried out on the existing PSS/SSS signalling. This implies that in a LP-SS design, over-laid OFDM sequences or signals are not necessarily needed.

Proposal 6 – For the LP-SS, do not specify the overlaid OFDM sequences(s).

# Conclusion

In this contribution, we have discussed our views on LP-WUS and LP-SS design and made the following observations and proposals.

Observation 1 – OOK-4 has lower system overhead and can be detected with shorter delay using lower power LP-WUR, compared to OOK-1 candidates. Furthermore, OOK-1 only has a single non-zero amplitude level on the time domain signal, making its PAPR low.

Observation 2 – With a LP-WUR design having a relatively poor LP-WUR BW properties and a 1-bit ADC, the OOK-based LP-WUS can reach the PUSCH, msg3 coverage with several dB difference.

Observation 3 – There is a trade-off between using OOK-based LP-WUS and OFDM-based LP-WUS in terms of the number of resources, delay and the total power consumption. While the OOK-based LP-WUS leads to lower power consumption than the OFDM based LP-WUS, particularly where the UE operates in the coverage area below PUSCH extended coverage, the number of resources and the latency increases for the OOK-bases LP-WUS to cover areas beyond PUSCH extended coverage.

.

Proposal 1 – Support LP-WUS structure with two fields, a preamble field for synchronization and cell identification purposes and a data field for indication of subsequent actions and/or wake-up group identity, depending on state of the operation.

Proposal 2 - The number of bits per OFDM symbol, M, cannot be a fixed value and needs to be decided based on LP-WUS bit rate and according to channel delay spread, preventing ISI and the tolerable time/frequency errors by the LP-WUR.

Proposal 3 – Support to transmit both an OFDM-based and an OOK-based LP-WUS at the same time for UEs to choose which to detect in accordance with their capabilities.

Proposal 4 – Consider OOK-4 transmission scheme for the transmission of the LP-SS.

Proposal 5 – Support LP-SS structure with two fields using OOK-4 transmission structure and sequences from m-sequence family with good auto- and cross correlation properties.

Proposal 6 – For the LP-SS, do not specify the overlaid OFDM sequences(s).

References

- RP-234056 New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- R1-2402977 LP-WUS operation in IDLE INACTIVE modes

- R1-2402978 LP-WUS operation in CONNECTED modes

- N. Mazloum and O. Edfors, "Interference-Free OFDM Embedding of Wake-Up Signals for Low-Power Wake-Up Receivers," in IEEE Transactions on Green Communications and Networking, vol. 4, no. 3, pp. 669-677, Sept. 2020,

- RAN1#116 Chair’s notes

Appendix

During RAN1#116, the following agreement on LP-WUS and LP-SS design were made.

Agreement Support both OOK-1 and OOK-4 for LP-WUS. - FFS how OOK-1 and OOK-4 are specified - For OOK-4, M<=4, FFS supported values - The SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as one of the SCS(s) used for other NR transmissions in the same CP-OFDM symbol - FFS different SCS. Agreement Further study the following options for LP-SS: - Option 1: OOK-1 - Option 2: OOK-4 with M=1,2,4,[8] - The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation - FFS: different SCS Agreement For LP-SS design from RAN1 perspective, consider at least the following as the design target: - For RRM measurement performed by LP-WUR based on LP-SS, UE can satisfy measurement accuracy based on X LP-SS samples within a period which is comparable to Y=the length of I-DRX cycle that is larger or equal to 1.28s. - FFS: X - Note: Y is chosen for evaluating LP-SS design. - Network overhead and network power consumption are to be considered Agreement The ‘ON-OFF’ pattern for OOK symbols of LP-SS is based on binary sequence(s) - FFS binary sequence(s) details, including the sequence type, the number of sequences, and the sequence length - FFS overlaid OFDM sequences, if supported Agreement For the overlaid OFDM sequence(s) for LP-SS, consider the following options for further down-selection: - Option 1: Do not specify the overlaid OFDM sequences(s) - Option 2: Specify the overlaid OFDM sequence(s) targeting for OOK waveform generation without targeting for sync and RRM measurement for OFDM-based LP-WUR using the overlaid sequence of LP-SS. - Option 3: Specify the overlaid OFDM sequence(s) targeting for OOK waveform generation and also targeting for sync and RRM measurement for OFDM-based LP-WUR using the overlaid sequence of LP-SS. - For Option 3, it is up to RAN4 to make decision on whether/how to define the RRM measurement requirement for OFDM-based LP-WUR using the overlaid sequence of LP-SS.

During RAN1#116bis, the following agreement on LP-WUS and LP-SS design were made.

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - FFS whether value of M depends on SCS - FFS M=1 for OOK-4 Agreement For evaluation purpose on LP-WUS, companies report the overlaid OFDM sequence(s), including: - Sequence(s) generation and how sequence(s) map in time or frequency domain (including any details with multiplexing and IFFT). - Number of candidate overlaid OFDM sequences used for information conveying - Including details on whether the number of candidate overlaid sequences is per OFDM symbol or per OOK symbol - How the proposed sequence design is processed by OFDM-based LP-WUR, e.g., in time domain or in frequency domain or in both time and frequency domain. Agreement Support to specify multiple binary LP-SS sequences for the ‘ON-OFF’ pattern: - The LP-SS sequence used in a cell is - Option 1: a sequence is configured - Option 2: a sequence is determined by predefined rule - FFS: Whether both options will be supported or only one will be supported - FFS: the number of LP-SS sequences - Note: Multiple sequences are used to differentiate LP-SS from different cells R1-2403616 Summary #2 of discussions on LP-WUS and LP-SS design Moderator (vivo) Agreement From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz - X to be down-selected between 11 and 12 PRBs - FFS the number of PRBs for 15kHz - FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz FFS: Whether the above is applicable to FR2 Agreement For timing error evaluation purpose, the following two options for residual frequency error are considered: - Option 1: The maximum frequency error (Fe) of RTC/oscillator is assumed, companies report Fe value and the applied LP-WUR type. - Option 2: The residual frequency error (Fr) after frequency error correction/clock calibration by LR or after assistance from MR is assumed, companies report Fr value, how to achieve it and the applied LP-WUR type. Agreement For frequency error evaluation purpose, the following two options for residual frequency error are considered: - Option 1: The maximum frequency error (Fe) of oscillator is assumed, companies report Fe value and the applied LP-WUR type. - Option 2: The residual frequency error (Fr) after frequency error correction by LR or after assistance from MR is assumed, companies report Fr value, how to achieve it and the applied LP-WUR type. Working Assumption Support the following options for LP-SS - Option 1: OOK-1 - Option 2: OOK-4 with M=2,4, FFS:1,8,16 - FFS whether value of M depends on SCS - The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation FFS how OOK-1 and OOK-4 are specified R1-2403751 Summary #3 of discussions on LP-WUS and LP-SS design Moderator (vivo) Agreement Regarding the LP-WUS information for idle/inactive UEs, at least consider the following： - Option 1: A bitmap with each bit corresponding to [one or more] subgroups - Option 2: A codepoint value corresponding to one or more subgroup(s) - Option 3: Multiple codepoint values with each corresponding to one or more subgroup(s) - Combination of above options are not precluded - FFS how to carry LP-WUS information, e.g., by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS. - FFS how to carry LP-WUS information by overlaid OFDM sequences. - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used - Other options are not precluded Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, at least consider the following： - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 2: A codepoint value corresponding to one or part of UE identity, e.g., C-RNTI - Option 3: A codepoint value corresponding to [one or more] UEs - Option 4: Multiple codepoint values with each corresponding to [one or more] UE(s) - Option 5: Multiple bit blocks with each corresponding to [one or more] UE(s) - Combination of above options are not precluded. - FFS how to carry LP-WUS information, e.g, by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS. - FFS how to carry LP-WUS information by overlaid OFDM sequences. - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used - FFS details of LP-WUS information to trigger PDCCH monitoring (e.g. whether above is applicable to one or more serving cells) Conclusion: For calibration purposes, companies are encouraged to report the SNR to achieve the coverage of PUSCH for message3, at least with the following assumptions: - Carrier frequency: 2.6 GHz - The number of Tx chains: 1 - MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE - Transmit antenna gain correction factors for WUS: up to company report - Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately Agreement For the purpose of further study and evaluation in RAN1, the following candidate sequences for the overlaid OFDM sequence are considered: - Gold sequence - M-sequence - ZC sequence - Chirp sequence - Walsh sequence - Golay sequence - Kasami sequence - Low density sequence - DFT/FFT sequence - QAM symbol-based sequence - Combinations and optimizations of above are not precluded Companies are encouraged to provide an assessment on performance, required complexity, and power consumption to support their preferred sequence. Companies are encouraged to provide details on their preferred sequence (e.g. references). Agreement Regarding the overlaid OFDM sequence(s) of LP-WUS, consider the following options: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol or OFDM symbol duration. OFDM-based LP-WUR can obtain the whole information bits by the presence of the overlaid sequence. - Option 1-2: The overlaid OFDM sequence is pre-determined from multiple sequences. This sequence carry NO information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the OOK ON/OFF pattern. - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol or OFDM symbol duration, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - Option 3: One sequence is selected from multiple candidates overlaid OFDM sequences on one or more OOK ‘ON’ symbols, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). - Option 4: Use of modulated overlay sequence with constellation point: overlay sequence acting as a spreading sequence and constellation point carrying information for OFDM-based LP-WUR. Other options are not precluded.