3GPP TSG RAN WG1 #117			　R1-2405018

Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

Title:	Draft LS reply on data collection to enable ML model training and inference in 5GC for Direct AI/ML based positioning

Response to:	R1-2403835/S2-2405833

Release:	Rel-19

Work Item:	FS_AIML_CN, NR_AIML_air

Source:	NTT DOCOMO (to be RAN1)

To:	SA2

Cc:	RAN2

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN1 would like to thank SA2 for the questions regarding data to be collected for direct AI/ML based positioning.

R1-2403835/S2-2405833 asks the following two questions,

- What data to be collected for ML model training for Direct AI/ML based positioning corresponding to cases 2b, 3b has been identified by RAN WG? - What data to be collected for location inference using ML models for Direct AI/ML based positioning corresponding to cases 2b, 3b has been identified by RAN WG?

RAN1 Answer:

In Rel-18, the study on data to be collected for training and inference in case 2b/3b has been completed and summarized in R1-2310681 as follows.

For positioning LCM purpose; Case; Data content; Typical data size (per data sample); Typical latency requirement; Notes. Training; All Cases; Measurements (corresponding to model input): timing, power, and/or phase info See Note 2; Size depends on number of PRS/SRS resources, measurement type (timing, power, and/or phase info) and report format: ~100 bits to 1000s bits per PRS/SRS resource See Note 3; Relaxed. Direct AI/ML positioning; Label: Location coordinates as model output; 56 to 144 bits See Note 3; Relaxed. AI/ML assisted positioning; Label: Intermediate positioning measurement (timing info, LOS/NLOS indicator) as model output See Note 2; 10s bits to 100s bits per PRS/SRS resource See Note 3; Relaxed. Inference; 1; Location coordinates as model output; 56 to 144 bits See Note 3; See Note 5. 2a, 3a; Intermediate positioning measurement (timing info, LOS/NLOS indicator) as model output See Note 2; 10s bits to 100s bits per PRS/SRS resource See Note 3; See Note 5. 2b, 3b; Measurements (corresponding to model input): Timing, power, and/or phase info See Note 2; Size depends on number of PRS/SRS resources, measurement type (timing, power, and/or phase info) and report format: ~100 bits to 1000s bits per PRS/SRS resource See Note 3; See Note 5. Monitoring; All Cases; See Note 8; See Note 8; Near-real-time; See Note 6 and 7. Note 1: The necessity and feasibility of difference cases (Case1 to Case3b) needs further discussion/conclusion. Note 2: For measurements as model input, no agreement on measurement types (i.e., time, power, and/or phase) in RAN1 for all cases (i.e., Case1 to Case3b). Measurement types (including their necessity) and sizes/dimension needs to be further discussed. Candidate measurement types discussed/evaluated for model input include CIR (contains timing, power and phase information), PDP (contains timing and power information), DP (contains timing information). For labels (i.e., model output) of AI/ML assisted positioning (Case2a, Case3a), RAN1 identified an initial listing of candidates that provide performance benefits (i.e., timing info, LOS/NLOS indicator). RSRP/RSRPP is for further discussion. Note 3: The measurement size of one data sample = (measurement data size of one PRS/SRS resource)*(number of PRS/SRS resources needed for model input). The label size of one data sample = (label data size of one PRS/SRS resource)*(number of PRS/SRS resources needed for model output). The quantization and bit representation of time, power, and phase information (including their necessity) still need to be further discussed. Existing specification allows reporting of up to 64 PRS/SRS resources per frequency layer for one positioning fix. For evaluations, most companies considered up to 18 TRPs. It should be noted that AI/ML positioning is not restricted to work only with maximum of 18 TRPs. - Example of calculation on a potential lower bound on measurement size per PRS/SRS resource: - A potential lower bound on measurement size per PRS/SRS resource can be calculated as follows (assuming timing only for 9 measurements per PRS/SRS resource): 16 + 9*8 = 88 bits. The total lower bound can be 88*N bits, where N is number of PRS/SRS resources used as model input for obtaining a positioning fix. This is based on the assumption of timing info as 16 bits for first arrival and 9 bits for relative timing. - Example of calculation of a potential upper bound on measurement size per PRS/SRS resource: - A potential upper bound on measurement size per PRS/SRS resource can be calculated as follows (assuming timing, power, and phase for 256 measurements per PRS/SRS resource and assuming 8 bit representation of each real number): 2*(8*256) = 4096 bits. The total upper bound can be 4096*N bits, where N is number of PRS/SRS resources used as model input for obtaining a positioning fix. - For location coordinates (corresponding to model output) - The bit representation of location coordinates depends on the type of shape, resolution, and uncertainty used to indicate the location (e.g., ellipsoid point, ellipsoid point with uncertainty circle, high accuracy ellipsoid with uncertainty ellipsoid, etc.) as listed in TS 23.032. The range of bit representation for location coordinates can be 7 bytes to 18 bytes (i.e., 56 to 144 bits). The location information report in existing specifications may contain additional information besides location coordinates (e.g., velocity, location error, integrity info, etc.) - For intermediate positioning measurement (corresponding to model output): - The quantization and bit representation of time, [RSRP/RSRPP], and LOS/NLOS information (including their necessity) as model output still need to be discussed in an appropriate working group. As a reference to existing timing representation in Rel17 [TS 37.355], an example on the label size can be of 21 bits per PRS/SRS resource while assuming model output produces one timing of 21 bits per PRS/SRS resource. The label size can be 21*N bits, where N is number of PRS/SRS resources for which intermediate positioning measurement has been generated. If LOS/NLOS indicator (1 bit per PRS/SRS resource assuming hard value for LOS/NLOS indicator) is included, the label size becomes 22*N bits. Note 4: No agreement on reporting types (i.e., periodicity, event-triggered/on-demand, etc.). Note 5: There are no agreements on the reporting latency. Note 6: RAN1 agreed on an initial listing of entities that can derive the monitoring metric for AI/ML positioning for different cases (Case1 to Case3b): -1: At least UE derives monitoring metric -2a: At least UE derives monitoring metric - LMF (if monitoring based on ground truth) -3a: At least gNB/TRP derives monitoring metric - LMF (if monitoring based on ground truth) -2b and 3b: At least LMF derives monitoring metric Note 7: No agreement yet on a monitoring decision entity or their mapping to other entities (e.g., entity running the inference, entity deriving the monitoring metric, etc.). Note 8: RAN1 has studied several types of related statistics where potential request/report of Monitoring related statistics and its necessity are for further discussion.

In Rel-19, RAN1 discusses what measurement types should be supported for case 2b/3b. Although some measurement types were agreed to be supported as follows (captured in R1-2403741), it is still under the discussion whether to support some additional measurement types, such as phase information.

Agreement - For AI/ML based positioning case 2b, at least the following types of time domain channel measurements are supported for UE reporting to LMF: - timing information; - paired timing information and power information. Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information.

2	Actions

To SA2

ACTION: 	RAN1 respectfully requests SA2 to take the above information into account.

3	Dates of next TSG RAN WG1 meetings

RAN1#118                           Aug 19th – 23th, 2024				Maastricht, Netherlands

RAN1#118bis              	       Oct  14th – 18th, 2024				TBC, China