3GPP TSG RAN WG1 #117			R1-2405520

Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for 8-port SRS (TDM or non-TDM, without mixture) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Futurewei), Samsung, ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 38.331, the RRC parameter nrofSRS-Ports-n8 is used to identify the port number for an 8-port SRS resource, and nrofSRS-Ports for an 1,2,4-port SRS resource. However, in a paragraph of 6.1.1.1 in TS 38.214, only the RRC parameter nrofSRS-Ports is used, which does not cover 8-port SRS resources. Also it needs to clarify if a mixture of TDMed and non-TDMed 8-port SRS resource is allowed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise the wording to be applicable to SRS resources with 1, 2, 4, or 8 ports. Note that based on the change, a mixture of TDMed and non-TDMed 8-port SRS resource in a SRS resource set is not supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The case of 8-port SRS resource is not covered in 6.1.1.1 of TS38.214. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of the CR. |  |  |  |  |  |  |  |  |

#### 6.1.1.1	Codebook based UL transmission

<Unchanged parts are omitted>

Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', when multiple SRS resources are configured by SRS-ResourceSet with usage set to 'codebook', the UE shall expect that a same higher layer parameter of nrofSRS-Ports or nrofSRS-Ports-n8 is configured in SRS-Resource in SRS-ResourceSet, and the higher layer parameter nrofSRS-Ports or nrofSRS-Ports-n8 shall be configured with the same value for all these SRS resources.

<Unchanged parts are omitted>