3GPP TSG RAN WG1 #117											R1-2404656

Fukuoka City, Fukuoka, Japan, May 20th - 24th, 2024

Agenda item:	9.1.3.3

Source:	NEC

Title: 	Discussion on other aspects of AI/ML model and data

Document for:	Discussion and Decision

- Introduction

Rel-18 AI/ML for air interface Study Item (SI) provided a comprehensive study on why and how to apply AI/ML for air interface, and Technical Report (TR) 38.843 captures the outcome of the SI [1]. Based on Rel-18 SI outcome, Rel-19 Work Item (WI) is targeting on providing specification support for the general framework as well as the selected representative use cases [2]. In the last meeting, the following agreements have been achieved [3]. In this contribution, we provide our views on the general issues of AI/ML model and data.

Conclusion From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons: - Risk of proprietary design disclosure - Burden of offline cross-vendor collaboration Conclusion From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y): - No much benefit compared to Case y - Risk of proprietary design disclosure - Large burden of offline cross-vendor collaboration - Additional burden on model storage within in 3GPP network Conclusion - It is clarified that MI-Option 4 refers to the Option 1 of CSI compression - Option 1: Fully standardized reference model (structure + parameters) Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

- Discussion on LCM

  - Model Identification

Justification on the need of model-ID can be based on the fact that functionality-based LCM may not be able to support all collaboration levels between NW and UE, especially for level Z with model transfer and model update, in particular if UE could not develop the model itself and needs to download from NW, or for two-sided models are developed by both NW and UE. It could be difficult to deliver or to transfer multiple models for the same functionality without involving model ID. In addition, according to the TR [1], functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability, and additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. Therefore, functionality based LCM cannot reflect any additional conditions. To ensure consistency between training and inference regarding NW-side additional conditions, model ID may be always required. With either Type A or Type B model identification procedure, the relationship between additional conditions and the model ID can be established.

Model ID is essential for use cases with model transfer, model update, or two-sided models, and is beneficial to differentiate additional conditions to ensure the consistency between training and inference.

Support model ID and model identification in Rel-19.

To facilitate the discussion, at least 5 different options are listed in the last meeting for model identification type B. At this stage, it may be still difficult to have consensuses on dataset transfer (MI-Option 2) and reference model standardization (MI-Option 4). Note that model identification via dataset transfer (MI-Option 2) is expected to have large signaling overhead where entire dataset for training is transmitted over the air and hence is not favorable from our point of view. And MI-Option 5 (Model identification via model monitoring) is a quite complicated method by assessing the performance before model identification.

From our perspective, we can further study Model identification with data collection related configuration (MI-Option 1) and Model identification in model transfer from NW to UE (MI-Option 3). For MI-Option 3, model identification can be accomplished quite easily where model ID is associated by network/UE whenever model is being transferred which can be used for later model management signaling. But for the case where model transfer is not defined for every AI/ML use case, then we may need to rely on MI-Option 1 where model can be identified based on the configuration of data which needs to be collected for model training. For example, for a UE-based beam management model, when a new antenna configuration is observed by a UE, then UE/UE-side may train a new model for the given antenna configuration. After the model training, UE can indicate the new model ID to the network (e.g. using Type B1) along with the configuration (for example antenna configuration) which was used for model training.

Based on above discussions, MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s) and MI-Option 3: Model identification in model transfer from NW to UE seem to be the most practical methods for now and can be pursued for further study.

RAN1 should study following options for model identification Type B for further discussion.

MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s)

MI-Option 3: Model identification in model transfer from NW to UE

Further details are necessary for MI-Option 1. RAN1 #116b discussed a procedure as an example (noted as AI-Example1) of MI-Option1 for UE-sided model developed at the UE side and agreed to further study this procedure. For the different use cases such as beam management, positioning accuracy enhancement and CSI feedback enhancement, it is essential to discuss the specific configurations and uses of associated IDs separately. This necessity arises because the data collection requirements and related procedures can differ for each use case, resulting in variations in the definitions and contents of associated IDs.

It is necessary to clarify the definition of associated ID for each use case separately.

In addition, it is also needed to clarify the data collection related configurations for each use cases. Taking BM cases for example, the data collection related configuration may be the report configuration or just the RS resource configuration without any report. For a model identification procedure initiated by the UE (e.g., Type B1) and NW may respond the request by assigning related configurations, e.g., report configuration ID, or sub configuration ID if applied. For a model identification procedure initiated by the NW (e.g., Type B2), UE may respond the similar information, for example, with a request to NW (before the step A in AI-Example1) while the final configuration is determined by NW.

For MI-Option 1, further clarify the data collection related configurations for each use case. And support UE to request the needed data collection related configurations.

The association among those different data collection configurations may need to be provided to UE, for example, via the associated ID, since UE needs to be aware that these configurations are for different LCM stages of the a same AI/ML model, and to maintain some consistency in measurement and report for those data collections. As discussed in each use case, at least the consistency across training and inference is beneficial from performance perspective. In addition, the consistency across model inference and performance monitoring may also be needed. In this case, one or more associated ID(s) can be attached to one same model ID to reflect different NW side additional conditions.

The associated ID(s) is used for at least ensuring the consistency across training and inference and one or more associated ID(s) can be attached to one same model ID to reflect different NW side additional conditions.

In addition, as discussed in step D in AI-Example1 for MI-Option 1, how model ID is determined/assigned needs to be specified. Considering that different types of model IDs may be used during model identification procedure, for example, a global model ID and a local model ID, at least the combination of Alt.1: NW assigns Model ID and Alt.2: UE assigns/reports Model ID can be supported. A global model ID is usually much more complicated than a local model ID and can be used for the initial identification, and after the models being identified, a simpler ID, e.g., local ID can be used for the following managements including model activation, deactivation, switching and selection. The figure below illustrates a model identification procedure initiated by the UE (Alt.2: UE reports a global Model ID) and NW may respond the request by assigning a local ID (Alt.1: NW assigns a local Model ID) for the future use. Meanwhile, Alt.3: Associated ID(s) is assumed as model ID(s) may not work since in Observation 2 it is noticed that one or more associated ID(s) can be attached to one same model ID to reflect different NW side additional conditions

Figure 1 an example of model ID translation

In the model identification procedure, support the combination of Alt.1: NW assigns Model ID and Alt.2: UE assigns/reports Model ID, which is used to link a UE reported global model ID to a NW assigned local model ID.

  - Conditions, Additional Conditions and Ensure the Consistency

The concept of additional conditions was introduced during the later stage of the study item where the need for the additional conditions was identified due to presence of AI/ML training variables which may not be standardised. For instance, in AI/ML based beam management use case, network topology elements like cell layout or antenna configuration, or UE internal variables like UE speed can impact the selection of the AI/ML model which should be used during AI/ML operation. However, even though these variables may be used for training of AI/ML model and thereby the AI/ML model operation require these variables as input, these variables may not be standardised due to disclosure of proprietary information. Hence, classifying these variables as additional conditions can be an enabler for optimal AI/ML operation.

Among the methods which have been considered for ensuring consistency between network and UE for these additional conditions, the methodology of “Consistency assisted by monitoring” seems to be the least optimal solution. This method is expected to result in high delay in selection of optimal AI/ML model for a UE. For instance, if multiple set of beam management models are developed for different UE speeds and network wants to use the monitoring procedure to select the optimal model, then network needs to activate multiple models at UE one by one associated to different UE speeds until it determines an AI/ML model which results in the best performance. For the duration of time when network is activating non-optimal model(s) at UE, the UE performance suffers and results in delay in selection of the most optimal model at UE.

Ensuring consistency of additional conditions using monitoring procedure results in high delay in identification of the suitable AI/ML model to run at UE, during which system performance suffers.

For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options should be considered as priority:

Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side

Model training at NW and transfer to UE, where the model has been trained under the additional condition

Information and/or indication on NW-side additional conditions is provided to UE

  - Performance Monitoring

For AI/ML model performance, methods should be identified to support the monitoring of AI/ML model performance and the required feedback signalling. The TR concluded several methods for AI/ML model monitoring, based on inference accuracy, system performance, data distribution, applicable condition respectively.

It can be observed that each method has its own application scenario, since the cause of model failure may be different. Supporting more than one of monitoring methods seems inevitable. If the monitoring method is determined at NW, configuration information of model monitoring method should be provided to UE. If the monitoring method is determined at UE, together with the model monitoring results, it may report the applied monitoring method, or it may report the cause of model failure if the monitoring results implying the model has been failed. In addition, considering the different candidate monitoring methods, UE capability on the supported methods of model monitoring shall also be reported.

Information of model monitoring methods can be provided to NW or UE. If model failure occurs, the cause of model failure may also be reported.

RAN1 had also considered the mechanism of monitoring inactive model/functionality for the purpose of model management decision making (e.g. activation/deactivation), however more discussions are required for the details of the procedure. Monitoring of inactive model/functionality is useful when a new model needs to be deployed in the network whose real time performance over different cells still needs to be validated. In such a case, the model/functionality shall be run at UE only for monitoring purpose without using the model/functionality for actual radio operation. For instance, in UE based beam prediction, UE may run legacy operation of beam measurements (e.g. by measuring full set of SSBs/CSI-RS) while at the same time a model/functionality could be run at UE (performing beam prediction) whose monitoring is performed to determine the performance of the given model/functionality.

Specify monitoring of inactive model/functionality for the purpose of activation/selection/switching of UE-side models/UE-part of two-sided models /functionalities for Rel-19 AI/ML.

Subsequently, it needs to be discussed, when an inactive model/functionality is being monitored, whether UE should run legacy operation or can UE activate a different model/functionality simultaneously for radio operations. For example, model-1 may be inactive but being monitored for beam prediction and at the same time model-2 may be activated at the same UE for beam prediction through which radio operations are being enabled. Such operation can allow testing of newly deployed AI/ML models and at the same time continuing the radio operation using older well-established AI/ML models. However, the UE capability for supporting such procedure needs to be investigated.

Concurrent inference operation of two models/functionalities at a UE (where one model/functionality is inactive but being monitored and other model/functionality is activated at UE) allows testing of newly deployed AI/ML model/functionality (using inactive model operation) and at the same time continuing the radio operation using older well-established AI/ML model/functionality.

Discuss whether a UE can perform inference of two models/functionalities concurrently where one model/functionality is inactive but being monitored and other model/functionality is activated at UE.

  - Management

Although explicit LCM signalling can be used to indicate AI/ML functionality/model selection, activation/deactivation/switching, adaptive selection of applied AI/ML functionality/model can greatly reduce signalling overhead, and possibly latency. For example, if different AI/ML functionalities/models are available for LOS/NLOS, high/low SINR, high/low velocity, more/less antenna ports/beams respectively, model/functionality switching can be adaptive to the change of configurations or the detection of change of the scenarios.

Support adaptive model/functionality selection, activation, deactivation, switching, and fallback based on additional conditions.

At least for UE side AI/ML functionality/model management, event triggered method may be considered, a timer and a counter can be used to control the management of the AI/ML functionality/model as follows. UE could activate candidate model(s) for performance assessment before switching to it.

- For AI/ML performance to be monitored, UE higher layer can initialize a counter to 0

- For an assessment duration, UE could monitor the performance of the AI/ML

- Based on assessment in the assessment duration, UE PHY layer may generate an indication to UE higher layer to indicate the performance issue of the AI/ML

- UE higher layer may start a timer when receives the indication from PHY layer for the first time

- While the timer is running,

  - when UE higher layer receives the indication from PHY layer, UE higher layer may increase the counter by 1 and restart the timer

  - If the counter reaches a threshold value, UE higher layer can make decision to deactivate the current AI/ML model

  - If the counter reaches another threshold value, UE higher layer can make decision to assess candidate model(s), if the candidate model(s) is inactive, it can be activated

- When the timer expires, the counter is set to 0

Support event triggered AI/ML functionality/model activation/deactivation/switching.

- Discussion on Collaboration Levels and Model Transfer

Network-UE collaboration levels x, y and z are considered during the Rel-18 study phase, and for Level z: signalling-based collaboration with model transfer, 5 different assumptions for model transfer are discussed, as in the following table. In the last meetings, z3 and z5 are deprioritized. And Case z2 is deprioritized at least for UE-sided model.

[1] Table 4.3-1: Model delivery/transfer cases Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top.; Outside 3GPP Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format.; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format.; 3GPP Network; NW-side. z3; model transfer in open format.; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support.; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known.; 3GPP Network; NW-side. Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization..

Several benefits of model transfer were identified during the study phase for AI/ML operation. From our perspective, supporting scenario/configuration specific models has one of the highest priorities. Although scenario/configuration specific models have not been actively considered in 3GPP, as the deployment of AI/ML accelerates, there is expected to be significant need of such models to achieve the highest possible network performance given large variations of network deployment from cell to cell. For instance, beam management study has already identified the impact of cell topology/deployment on the performance of AI/ML algorithms and various mechanisms are being considered e.g. additional conditions, dataset identifiers to handle different deployment scenarios. However, as each cell deployment is a combination of various factors (cell layout, antenna configuration, types of UEs, etc.) each of which individually impacting the AI/ML performance, covering all such combinations requires deployment of large number of AI/ML models which introduces significant burden at UE in terms of model maintenance and memory consumption. Hence, model transfer is essential to cater to such deployments where scenario/configuration specific models can be deployed at UE by gNB on demand.

Supporting model transfer is essential when considering cell/scenario-specific AI/ML deployment which is expected to happen when AI/ML deployment accelerates.

While it can be argued that model transfer can be specified at a later release when this requirement materialises, but this could imply that the framework which is defined in current release does not consider the requirements of model transfer (e.g. model format) and thereby the introduction of model transfer in future could face significant technical bottlenecks. Thereby, we believe that model transfer should be supported from the start to allow more optimal future deployments of AI/ML which can optimise the network operations.

Model transfer should be supported from Rel-19 to ensure future-proofness of AI/ML operation.

In RAN1#116bis, model transfer methodologies z2 and z3 were deprioritized on top of deprioritization of z5 during RAN1#116. Hence, now we are left with z1 and z4 for further discussion. The majority of the discussion is centered around support of CSI compression use case for which model transfer is beneficial. Hence, it is crucial to understand the exact requirements of CSI compression use case to select which model transfer methodology (z1 or z4) should be selected (if required). For instance, z1 model transfer is more appropriate for transfer of model corresponding to CSI generation part (which is expected to be developed at UE side) while x4 is more appropriate for transfer of model corresponding to CSI-reconstruction as this model is likely to be developed at gNB side. Therefore, we propose to down select (if required) between z1 and z4 based on outcome of discussion of CSI compression model transfer requirements.

Selection of model transfer methodology(ies) between z1 and z4 should be based on the outcome of CSI compression model transfer discussion.

- UE Capability Reporting and UE Assistance Information Reporting

During the SI, RAN1 also discussed the need to study how to handle UE’s internal conditions such as memory, battery which may impact AI/ML operation. Given that AI/ML models are expected to be more intensive as compared to legacy operation and may require dedicated UE hardware to optimise its operations, it seems natural to support handling UE internal conditions properly. While for the case of UE-based AI/ML operation, such kind of intervention can be performed locally at UE without any awareness of 3GPP network, for the case when AI/ML model operation is controlled by the network (e.g., activation/deactivation) it is important to discuss how UE can indicate its internal conditions to the network for optimal AI/ML operation.

It is important to discuss how UE can indicate its internal restrictions to activate or run an AI/ML model/functionality to the network for optimal AI/ML operation.

There are two types of indications from UE to network which can be supported for such application. In the first approach, UE can indicate to the network its detailed status of memory size, battery level and other hardware limitations to the network like UE Assistance Information message. However, this may involve proprietary information disclosure of UE’s operation which may be undesirable. Further it is not clear how network would be able to ascertain based on the assistance information whether UE can run an AI/ML model/functionality as AI/ML operation footprint shall be UE implementation specific.

Reporting of UE’s internal conditions such as memory size, battery level and other detailed hardware limitations to gNB for AI/ML operation may lead to UE’s proprietary information disclosure and may be hard for network to determine AI/ML applicability for a UE based on the provided information.

In other approach, UE can indicate whether it can run an AI/ML functionality/model after gNB indicates an AI/ML model/functionality for configuration or activation. UE, after receiving AI/ML information, shall determine whether it can run the AI/ML model/functionality or not based on its internal conditions. If the UE determines that it shall not be able to run an AI/ML model/functionality, it can indicate that to network about the restriction and can also provide a relevant cause value (e.g., memory limitation) which can allow network to understand the reason of failure and appropriate next action. Such a procedure keeps implementation simple and efficient.

Specify UE indication to network about its inability to run a configured/activated AI/ML model/functionality due to UE’s internal condition along with a relevant cause value for the failure.

- Conclusion

In this contribution, we provided our views on Rel-19 AI/ML for air-interface, and we have the following observations and proposals:

Observation 1:	Model ID is essential for use cases with model transfer, model update, or two-sided models, and is beneficial to differentiate additional conditions to ensure the consistency between training and inference.

Observation 2:	The associated ID(s) is used for at least ensuring the consistency across training and inference and one or more associated ID(s) can be attached to one same model ID to reflect different NW side additional conditions.

Observation 3:	Ensuring consistency of additional conditions using monitoring procedure results in high delay in identification of the suitable AI/ML model to run at UE, during which system performance suffers.

Observation 4:	Concurrent inference operation of two models/functionalities at a UE (where one model/functionality is inactive but being monitored and other model/functionality is activated at UE) allows testing of newly deployed AI/ML model/functionality (using inactive model operation) and at the same time continuing the radio operation using older well-established AI/ML model/functionality.

Observation 5:	Supporting model transfer is essential when considering cell/scenario-specific AI/ML deployment which is expected to happen when AI/ML deployment accelerates.

Observation 6:	It is important to discuss how UE can indicate its internal restrictions to activate or run an AI/ML model/functionality to the network for optimal AI/ML operation.

Observation 7:	Reporting of UE’s internal conditions such as memory size, battery level and other detailed hardware limitations to gNB for AI/ML operation may lead to UE’s proprietary information disclosure and may be hard for network to determine AI/ML applicability for a UE based on the provided information.

Proposal 1:	Support model ID and model identification in Rel-19.

Proposal 2:	RAN1 should study following options for model identification Type B for further discussion.

−	MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s)

−	MI-Option 3: Model identification in model transfer from NW to UE

Proposal 3:	It is necessary to clarify the definition of associated ID for each use case separately.

Proposal 4:	For MI-Option 1, further clarify the data collection related configurations for each use case. And support UE to request the needed data collection related configurations.

Proposal 5:	In the model identification procedure, support the combination of Alt.1: NW assigns Model ID and Alt.2: UE assigns/reports Model ID, which is used to link a UE reported global model ID to a NW assigned local model ID.

Proposal 6:	For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options should be considered as priority:

−	Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side

−	Model training at NW and transfer to UE, where the model has been trained under the additional condition

−	Information and/or indication on NW-side additional conditions is provided to UE

Proposal 7:	Information of model monitoring methods can be provided to NW or UE. If model failure occurs, the cause of model failure may also be reported.

Proposal 8:	Specify monitoring of inactive model/functionality for the purpose of activation/selection/switching of UE-side models/UE-part of two-sided models /functionalities for Rel-19 AI/ML.

Proposal 9:	Discuss whether a UE can perform inference of two models/functionalities concurrently where one model/functionality is inactive but being monitored and other model/functionality is activated at UE.

Proposal 10:	Support adaptive model/functionality selection, activation, deactivation, switching, and fallback based on additional conditions.

Proposal 11:	Support event triggered AI/ML functionality/model activation/deactivation/switching.

Proposal 12:	Model transfer should be supported from Rel-19 to ensure future-proofness of AI/ML operation.

Proposal 13:	Selection of model transfer methodology(ies) between z1 and z4 should be based on the outcome of CSI compression model transfer discussion.

Proposal 14:	Specify UE indication to network about its inability to run a configured/activated AI/ML model/functionality due to UE’s internal condition along with a relevant cause value for the failure.

- References

- TR 38.843 v18.0.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- Chair’s Notes, 3GPP TSG RAN WG1 #116bis, Changsha, Hunan Province, China, April 15th - 19th, 2024