3GPP TSG-RAN WG1 Meeting #117	Tdoc R1-2404753

Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

Agenda Item:	5

Source:	Ericsson

Title:	Discussion of LS on LTM L1 intra and inter-frequency measurements

Document for:	Discussion

# 1	Introduction

In [1], RAN2 asks two questions related to LTM L1 intra and inter-frequency.

In this contribution, we discuss the relevant RAN1 background, and provide a draft LS response in the appendix.

# 2	Discussion

In [1], RAN2 asked the following questions related to UE-capabilities for LTM L1 measurements:

| 45-1 | Intra-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure |
| --- | --- |
| 45-1a | Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure |

RAN2 asks:

Question 1 : Are the above intra-frequency and inter-frequency L1 measurement and reporting features (45-1 and 45-1a) prerequisites to support intra-frequency and inter-frequency LTM, respectively?

Question 2: The above features, 45-1 and 45-1a, from RAN1 and related RAN4 features (39-1, 39-2, 39-3-1, 39-3-2, 39-3-3, 39-3-4, 39-3-5, 39-3-6) are defined per BC for both intra-frequency and inter-frequency measurements.  RAN2 would like check with RAN1/4 for which BC (e.g. BC of current serving cells, BC including current serving cells and cell to be measured or something else) these capabilities are to be considered for L1 intra-frequency and inter-frequency LTM measurements?

For question 1, RAN1 has agreed to the following UE features [2]:

| Index | Feature group | Components | Prerequisite FGs | Type | Note | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 45-1 | Intra-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement 4. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 5. Maximum number of LTM CSI report configs | 2-21 or 2-22 or 2-23 or 2-23a | Per BC | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 5 candidate values: Aperiodic: {0,1,2,3,4} Periodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4} | Optional with capability signalling |
| 45-1a | Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of inter- frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra- and inter-frequency L1-RSRP measurement 4. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement | 2-21 or 2-22 or 2-23 or 2-23a, 45-1 | Per BC | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: L: {1,2,3,4} M: {1,2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} | Optional with capability signalling |
| 45-2 | Inclusion of current SpCell in the L1 measurement report | 1. Support of always including the current SpCell in the L1 measurement report | 45-1 | Per BC |  | Optional with capability signalling |

RAN1 has also defined the following LTM FGs:

Beam indication with joint DL/UL LTM TCI states

MAC-CE activated joint LTM TCI states

Beam indication with separate DL/UL LTM TCI states

MAC-CE activated DL/UL LTM TCI states

RACH-based early TA acquisition

RACH-based early TA acquisition with simultaneous transmission

UE-based TA measurement

TA indication in cell switch command

From the agreed feature list, RAN1 considers the LTM reporting as a separate feature. From the full list of LTM features, we see that FG 45-1 or 45-1a are not prerequisite FGs of any LTM FG (except 45-2):

45-1 or 45-1a is not a prerequisite FG of any LTM FG except 45-2.

To support intra-frequency or inter-frequency LTM, the UE would have to support

Beam indication with joint DL/UL LTM TCI states

RACH-based early TA acquisition

TA indication in cell switch command

or

Beam indication with joint DL/UL LTM TCI states

UE-based TA measurement

If the UE does not support 45-1 or 45-1a, the NW may still implement LTM using L3 measurements, or even UL measurements:

The NW may implement LTM using L3 measurements, or UL measurements.

Of course, a solution based on Observation 2 may not work for all deployments.

Based on this discussion we make the following observation:

The intra-frequency and inter-frequency L1 measurement and reporting features (45-1 and 45-1a) are not prerequisites to support intra-frequency and inter-frequency LTM, repectively.

For question 2, these capabilities are reported per BC. In our understanding, the BC refers to the BC of the current serving cells:

The UE capabilities 45-1 and 45-1a are considered for the BC of the current serving cells.

Based on this explanation, we propose

Include Observation 3 and Observation 4 in the LS response to RAN2.

A draft LS response is provided in the appendix.

# Conclusion

In the previous sections we made the following observations:

Observation 1	45-1 or 45-1a is not a prerequisite FG of any LTM FG except 45-2.

Observation 2	The NW may implement LTM using L3 measurements, or UL measurements.

Observation 3	The intra-frequency and inter-frequency L1 measurement and reporting features (45-1 and 45-1a) are not prerequisites to support intra-frequency and inter-frequency LTM, repectively.

Observation 4	The UE capabilities 45-1 and 45-1a are considered for the BC of the current serving cells.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Include Observation 3 and Observation 4 in the LS response to RAN2.

# References

R1-2404199, LS on LTM L1 intra and inter-frequency measurements, RAN2, RAN1#117, Fukuoka, Japan, May 2024

R1-2403703, Updated RAN1 UE features list for Rel-18 NR after RAN1#116bis, Moderators (AT&T, NTT DOCOMO, INC.), Changsha, China, April 2024

# Appendix: proposed reply LS

3GPP TSG WG RAN1 #117	R1-240xxxx

Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

# Title:	Draft reply LS on LTM L1 intra and inter-frequency measurements

# Response to:	R1-2404199 LS on LTM L1 intra and inter-frequency measurements

# Release:	Rel-18

# Work Item:	NR_Mob_enh2-Core

Source:	[Ericsson]

To:	RAN2

Cc:	RAN4

Contact Person:

Name:	[Claes Tidestav]

E-mail Address:	[claes.tidestav@ericsson.com]

1. Overall Description:

RAN1 thanks RAN2 for the LS on LTM L1 intra and inter-frequency measurements and would like to provide the following response.

Question:

Question 1: Are the above intra-frequency and inter-frequency L1 measurement and reporting features (45-1 and 45-1a) prerequisites to support intra-frequency and inter-frequency LTM, respectively?

Answer: The intra-frequency and inter-frequency L1 measurement and reporting features (45-1 and 45-1a) are not prerequisites to support intra-frequency and inter-frequency LTM, respectively.

Question 2: The above features, 45-1 and 45-1a, from RAN1 and related RAN4 features (39-1, 39-2, 39-3-1, 39-3-2, 39-3-3, 39-3-4, 39-3-5, 39-3-6) are defined per BC for both intra-frequency and inter-frequency measurements.  RAN2 would like check with RAN1/4 for which BC (e.g. BC of current serving cells, BC including current serving cells and cell to be measured or something else) these capabilities are to be considered for L1 intra-frequency and inter-frequency LTM measurements?

Answer: The UE capabilities 45-1 and 45-1a are considered for the BC of the current serving cells.

ACTION: 	RAN1 kindly asks RAN2 to take the above information into account.

3. Date of Next TSG-RAN1 Meetings:

3GPP TSG RAN WG1#118	19 – 23 August 2024	Maastricht, The Netherlands

3GPP TSG RAN WG1#118-bis	14 – 18 October 2024 	TBD, China