3GPP TSG-RAN WG1 Meeting #118	R1-2405830

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	5

Source:	Ericsson

Title:	On LS on Clarification of requirements for Ambient IoT

Document for:	Discussion, Decision

# 1	Introduction

SA2 has sent an LS [1] to RAN1, SA1, and SA3, asking for feedback on the following aspects:

SA2 have discussed different aspects with respect to non-volatile memory: - whether or not an Ambient IoT device can incorporate a non-volatile memory in the device design i.e., include a non-volatile memory in the Bill-of-Materials (BoM). - whether an Ambient IoT device will be able to update its non-volatile memory based on its energy status and energy storage capabilities at some point after receiving a trigger from the Reader. SA2 has not reached any common understanding. While there is no consensus in SA2 whether it is in RAN1 scope to determine devices' level of capability to support non-volatile memory; SA2 would like to ask the following question to RAN1: Question 5: SA2 would like to ask RAN1 if RAN1 can provide feedback on the non-volatile memory aspects listed above.

In this contribution, we provide our analysis for the above questions and a draft LS response.

# 2	Discussion

The content in this section corresponds to Section 2.11 in our contribution [2] to agenda item 9.4.1.2.

It is likely that A-IoT devices will need to hold information for extended periods of time, such as in inventory management, supply chain tracking, or asset management. In addition, the device may need to store security keys for encryption. This requires the use of a non-volatile (NV) memory that can store information in the device even when it is in OFF state.

Literature review reveals that the current draw for NV memories is typically sensibly higher than the power budget of Device 1, see table below. In reference [12] the write current for a 1 Kbit NVM, implemented in a 0.18 μm single-poly standard CMOS process, is below 1μA but the required supply voltage is 5V to 6V, thus still not feasible for Device 1. RRAM (Resistive RAM) is a different type of memory since it works by changing the resistance across a memristor and it’s a promising NV memory type due to its very low power consumption, high speed, and high program/erase cycle. However, there are still some key challenges such as: uniformity and repeatability, reliability, scaling, and manufacturing cost. Additionally, there are ongoing efforts to optimize the materials and processes used in RRAM fabrication to meet the requirements for mass production. It is likely that RRAM could become mature for mass production in the near future, but it is not ready yet. We hence believe that, due to maximum power consumption of 1μW, it will be difficult to implement a NV memory in Device 1, unless a higher peak power consumption (e.g., 5 μW) is accepted. In Device 2a/b, the much higher power budget allows to implement small low power NV memory.

In Device 1, a small NV memory could be feasible to implement, provided that the peak power consumption can exceed the 1μW target.

In Device 2a/2b, the higher power budget allows to implement a small low-power NV memory.

| Reference | Cell Type | Size | Read Current [μW] | Read Voltage [V] | Write Current [μW] | Write Voltage [V] | Process |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [3] | EEPROM | 2 kbits | 0.68 | 1-1.8 | 30 | 1-1.8 | 180nm |
| [4] | EEPROM | 1 kbits | 1.18 | 1 | 54 | 1.65 | 180nm |
| [5] | OTP | 2 kbits | 30.6 | ? | 104.4 | 6.6 | 180nm |
| [6] | OTP | 192 bits | 2.2 | 1.2 | 4.3 | 1.2 | 180nm |
| [7] | EEPROM | 1 kbits | 18 | 1.5 | Not Available | Not Available | 1μm |
| [8] | OTP | 128 bits | 3 | 1, 1.8, 3.5 | Not Available | Not Available | 180nm |
| [9] | C-Flash bitcell | 256 bits | 3.8 | 1.8 | Not Available | 5 | 180nm |
| [10] | EEPROM | 512 bits | 14.5 | ? | 370 | ? | 180nm |
| [11] | FeRAM | 2 kB | 13 | 3 | 15.7 | 3 | 350nm |
| [12] | Single-Poly CMOS | 1 kbits | 0.19 | ? | 0.69 | 5-6 | 180nm |

To predict how often the A-IoT device can update the NVM it is necessary to know the energy consumption during read and write operations. Therefore, the time required to perform such operations and the power required must be known. The time required to perform read or write operations on an NVM will vary depending on several factors including the type of memory (EEPROM, FRAM, etc.), the specific A-IoT device hardware, and the amount of data to be read/written. For instance, FRAM (Ferroelectric Random Access Memory) can allow faster write operations compared to EEPROM.

Read Operations: Reading is generally a fast operation. In passive RFID devices, used for inventory management, reading from the tag's memory typically takes between few 100 μs to a few milliseconds.

Write Operations: Writing to the NVM takes longer than reading because more energy is required. For instance, in passive RFID tags, write operations can take anywhere from 5 to 50 milliseconds per block of data.

Assuming a write time of 5ms and a write power consumption of 54μW (33μA @ 1.65V) as in [4], an energy of about 272nJ is required to perform a write operation. A 10μF capacitor used as energy storage can store 5μJ of energy at 1V. This means that, in the ideal case of only the memory being turned on while all other blocks are powered off, 18 write operations will empty the energy storage. However, the other device blocks will also consume energy, in particular the digital processing blocks and the clock which are required by the memory itself. Therefore, it is reasonable to assume that the A-IoT device will be capable of performing 1 or very few write operations between each energy harvesting sessions. A very low power consuming hysteresis comparator can be implemented in the device to establish when the energy storage device is fully charged, and hence the device can operate with RX, TX, and memory, or when the energy storage is getting empty, and the device should switch to harvesting mode.

UHF RFID tags typically have four memory banks. An overview of how the memory of UHF RFID Class 1 Gen 2 tags is organized is reported in the table below [13].

| Bank Name | Size | Type | Purpose |
| --- | --- | --- | --- |
| Reserved | 32+32 bits | Read/Write | Stores the kill password and the access password |
| EPC | 96 bits | Read/Write | Stores the Electronic Product Code (EPC), which is intended to uniquely identify the item to which the tag is attached |
| TID | 160 bits | Read only | Stores a unique tag identifier (TID) that is factory-encoded by the tag manufacturer and is permanent |
| User | 0 to 65 bytes | Read/Write | Optional memory that can be used to store user-defined data |

Of the four memory banks, the Reserved memory, the EPC, and the User memory are of NV type. Excluding the User memory which is optional and not feasible for low complexity A-IoT devices, the total size of the NV memory is 96+32+32 bits = 160bits. Hence, we believe that a NV memory of 100 – 200 bits can be suitable for Device 2a and 2b in A-IoT.

From BoM perspective, an NV memory size of 100-200 bits can be suitable for Devices 2a and 2b.

We believe that A-IoT devices will need to have a simpler memory structure and smaller size compared to what is normally present in RFID tags. One main difference between RFID systems and to A-IoT devices based on RF energy harvesting is the typical distance from the reader/gNB and hence the energy availability. RFID tags are usually read from a relatively close range, sometimes just a few centimeters to a few meters. This proximity allows for a more controlled and powerful energy transfer from the reader to the tag, ensuring higher amount of RF energy available at the antenna. On the other hand, ambient IoT devices that harvest RF energy will often be located more farther away from the CWT/gNB, and therefore is likely receive a much lower amount of energy at their antenna.

Considering an incoming RF power of about -30dBm to -25 dBm (1uW to 3uW) at the device antenna, and assuming a power conversion efficiency (PCE) of 10% at the RF harvester, the power available at the rectifier´s output will be around 100nW to 300nW. However, a 10% value for the PCE is rather optimistic since it reduces when the input power is very low, and thus the actual power at the rectifier´s output can be even lower than 100nW. Even though the device will have an energy storage and will be capable of accumulating energy over time during the harvesting phase, we should consider some aspects:

- The energy storage (capacitor, supercapacitor, or battery) will have a leakage. Therefore, if the energy storage is charged at a rate that is comparable to its leakage, it will not be possible to charge the energy storage or it will take an extremely long amount of time. Thus, the charging current must be well above this leakage and harvesting time should not be too long.

- Considering the assumption of extremely poor clock accuracy (104 – 105 ppm) that has been considered in RAN1, the device will have to spend a considerable amount of time and energy to correct this very high frequency error, further reducing the power available for other operations.

The memory in A-IoT devices should be smaller and simpler compared to regular UHF RFID tags.

To summarize, in our view, RAN1 can provide the following feedback to SA2 on the non-volatile memory aspects:

- Regarding whether or not an A-IoT device can include an NV memory in the BoM: Yes, all three device types can implement an NV memory from BoM perspective. Specifically, for Devices 2a and 2b, an NV memory size of 100-200 bits may be suitable.

- Regarding whether an A-IoT device will be able to update its NV memory based on its energy status and energy storage capabilities: It could be feasible to implement and program a small NV memory in Device 1 if the peak power consumption is allowed to temporarily exceed the 1μW target. The higher power budget allows to implement and update a small low power NV memory in Devices 2a and 2b.

In the next section, we provide a draft LS response based on the above analysis.

# 3	Draft LS response

Based on the analysis in the previous section, we provide the following draft LS response:

# 1 Overall description RAN1 has discussed the following questions from SA2 regarding the feasibility of incorporating a non-volatile memory in the device design for Ambient IoT: SA2 have discussed different aspects with respect to non-volatile memory: - whether or not an Ambient IoT device can incorporate a non-volatile memory in the device design i.e., include a non-volatile memory in the Bill-of-Materials (BoM). - whether an Ambient IoT device will be able to update its non-volatile memory based on its energy status and energy storage capabilities at some point after receiving a trigger from the Reader. SA2 has not reached any common understanding. While there is no consensus in SA2 whether it is in RAN1 scope to determine devices' level of capability to support non-volatile memory; SA2 would like to ask the following question to RAN1: Question 5: SA2 would like to ask RAN1 if RAN1 can provide feedback on the non-volatile memory aspects listed above.. RAN1 would like to provide the following response to the questions from SA2: Regarding whether or not an A-IoT device can include an NV memory in the BoM: Yes, all three device types can implement an NV memory from BoM perspective. Specifically, for Devices 2a and 2b, an NV memory size of 100-200 bits may be suitable. Regarding whether an A-IoT device will be able to update its NV memory based on its energy status and energy storage capabilities: It could be feasible to implement and program a small NV memory in Device 1 if the peak power consumption is allowed to temporarily exceed the 1μW target. The higher power budget allows to implement and update a small low power NV memory in Devices 2a and 2b. # 2 Actions To SA2: ACTION: RAN1 respectfully asks SA2 to take the above response into account.

We have the following proposal:

Base an LS response to SA2 on the draft LS response in this contribution.

# 3	Conclusion

In the previous sections, we made the following observations:

Observation 1	In Device 1, a small NV memory could be feasible to implement, provided that the peak power consumption can exceed the 1μW target.

Observation 2	In Device 2a/2b, the higher power budget allows to implement a small low-power NV memory.

Observation 3	From BoM perspective, an NV memory size of 100-200 bits can be suitable for Devices 2a and 2b.

Observation 4	The memory in A-IoT devices should be smaller and simpler compared to regular UHF RFID tags.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Base an LS response to SA2 on the draft LS response in this contribution.

# References

R1-2405796, “LS on Clarification of requirements for Ambient IoT”, SA2, Ericsson, RAN1#118, August 2024.

R1-2405825, “Ambient IoT device architectures”, Ericsson, RAN1#118, August 2024.

A 2 Kbits Low Power EEPROM for Passive RFID Tag IC

An Ultra Low-Power Solution for EEPROM in Passive UHF RFID Tag IC With a Novel Read Circuit and a Time-Divided Charge Pump

A Battery-Assisted Passive EPC Gen-2 RFID Sensor Tag IC With Efficient Battery Power Management and RF Energy Harvesting

An ultra low power non-volatile memory in standard CMOS process for passive RFID tags

An Ultra-Low-Power Long Range Battery/Passive RFID Tag for UHF and Microwave Bands With a Current Consumption of 700 nA at 1.5 V

A System-on-Chip EPC Gen-2 Passive UHF RFID Tag With Embedded Temperature Sensor

A low-cost low-power non-volatile memory for RFID applications

A low power CMOS compatible embedded EEPROM for passive RFID tag

A Passive UHF RF Identification CMOS Tag IC Using Ferroelectric RAM in 0.35-μm Technology

An ultra-low-power area-efficient non-volatile memory in a 0.18 μm single-poly CMOS process for passive RFID tags

EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID