3GPP TSG RAN WG1 #118		R1-2406187

Maastricht, NL, August 19th – 23rd, 2024

Source:	vivo

Title:	Discussion on Frame structure, random access, scheduling and timing aspects for Ambient IoT

Agenda Item:	9.4.2.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the frame structure and timing aspects for A-IoT, including the frame structure, synchronization, random access, scheduling and timing relationships.

- A-IoT Syncronization and Frame structure

  - R2D midamble

Following agreement was made for R2D midamble in RAN1#106bis meeting [2]:

Agreement RAN1 study the R2D transmission without midamble as the baseline if Manchester encoding is used. - FFS the necessity for the R2D transmission with midamble if PIE is used.

Both PIE and Manchester encoding are line codes, which contain at least one voltage transition for bit ‘0’ and bit ‘1’. A-IoT device can use the transition(s) e.g. counting the number of clock cycles between the edges to achieve the chip-level timing tracking. Therefore, PIE can also be viewed as the codes with self-clock. In addition, the payload size for R2D transmission is typically small, the necessity of midamble is further reduced.

Proposal 2.1-1: RAN1 study the R2D transmission without midamble as the baseline.

  - D2R midamble

In RAN1#106bis meeting, following agreement was made for A-IoT D2R midamble [2]:

Agreement For D2R transmission, study the necessity of midamble at least for the purpose of performing timing/frequency tracking or channel estimation or interference estimation, considering at least the following: - Modulation and Coding schemes, e.g., data modulation, line/channel coding - Receiving methods, e.g., coherent or non-coherent - D2R transmission length/packet size - Midamble overhead - Timing/frequency accuracy - Phase accuracy

D2R midamble is proposed for different purposes: 1. performing timing/frequency tracking; 2. performing channel estimation; 3. performing interference estimation.

From timing/frequency tracking perspective, in addition to D2R preamble, whether D2R midamble is necessary for Reader side to correct/track the timing, depends on factors like whether the device side can track/correct the clock based on the received R2D transmission using line coding, the timing error/drift at device side and D2R transmission length.

In case device cannot correct the clock based on the received R2D transmission, the SFO for D2R transmission can be as high as 10%. Then, for long D2R transmission length, reader may need to try a large number of hypotheses in order to achieve accurate timing by only D2R preamble detection. It may increase complexity at Reader side. However, in case the device can correct the clock based on the received R2D transmission, for example, by counting the number of clock cycles with a known period, its SFO for D2R transmission can be smaller than 10%, then midamble may not be necessary, reader can rely on preamble only to obtain accurate timing even for long D2R transmission.

Observation 2.2-1: Whether D2R midamble is needed for timing/frequency tracking for long D2R transmission length depends on factors like whether device can correct the timing by the received R2D transmission and whether reader can try a large number of SFO hypotheses, and the timing error/drift at device side etc.

In the following, we present the evaluation of SFO estimation accuracy at the reader side by using only the D2R preamble. We assume that BLF=160kHz and a sampling frequency is 1.92 Msps, so the number of sampling points of a chip is 48. The initial clock error Fe=10^5 ppm with clock drift of Fe_drift = 400ppm/s, which is 4000x higher drifting rate compared to the clock drifting model of LP-WUR studied in TR38.869. Given the clock drift is very small, we disregard the SFO during transmission as it requires approximately 52 seconds to significantly impact the chip's sampling point drift.

Observation 2.2-2: The clock drift with 400ppm/s device side does not cause significant timing drift for long D2R transmission duration.

The simulation indicates that the midamble is unnecessary for accurate SFO estimation if the preamble is precise. Assuming a 20-bit preamble, the receiver at reader side first uses a frequency domain conversion to estimate a rough SFO, then refines this by conducting parallel sliding correlation tests using seven hypotheses of different sampling point sizes ranging from +[-3:3]. Based on the maximum correlation, the precise sample size is selected. The CDF for SFO estimation accuracy, as displayed in Figure 2-1, shows 83% accuracy at 0dB SNR, increasing to 97.6% at 14dB, and 99.3% at 16dB SNR, 99.8% at 20dB SNR.

Figure 2-1: SFO estimation accuracy CDF

Observation 2.2-3: Reader can achieve more than 99% SFO estimation accuracy by detection of D2R preamble only with a limited number of SFO hypotheses at SNR of 16dB when the SFO at device side is 10^5 ppm.

From channel estimation perspective, channel estimation is necessary for coherent receiver to remove channel from received signal to recover the D2R signal. At least for short D2R transmission, it is natural and simple to just use D2R preamble for channel estimation. For long D2R transmission length, D2R midamble may be needed in addition to preamble. However, based on our simulation comparison between coherent detection and non-coherent detection presented in figure 5 (c) and 5 (d)in [8], it is observed that for D2R payload size of 400bits, ~6kbps using Manchester coding without FEC, to achieve 10% BLER target, the required SNR (around 15dB) is comparable between the coherent detection and noncoherent detection. However, for coherent detection, a sufficient number of midambles are needed for accurate channel estimation, resulting in more overhead and power consumption, which makes coherent detection less efficient than non-coherent detection. In addition, the processing time and complexity at the reader side may be increased since reader can only decode the data after channel estimation by using midamble. Another aspect is for FDMed D2R transmission from multiple devices, the midamble overhead will be increased for channel estimation. As observed in [6], it requires the D2R resources from other FDMed device to be mute when one device transmits the midamble. Or alternatively, the midamble should be designed and transmitted from FDMed device to ensure good orthogonality.

(c) Manchester, w/o FEC w/o SFO, payload 400bits, ~6kbps      (d) Manchester, w/o FEC, w/ SFO, payload 400bits, ~6kbps

Figure 5 Performance comparison between coherent and non-coherent detection in [8]

Observation 2.2-4: Coherent detection has benefit in noise reduction as it retains the desired signal under accurate channel estimation. While enough midambles/postamble is necessary especially for D2R with long transmission.

Observation 2.2-5: For the case of FDMA of multiple D2R transmissions, midamble used for channel estimation may increase the overhead.

Proposal 2.2-1: For comparison between non-coherent detection and coherent detection using preamble/midamble/postamble for D2R transmission, capture the following observations for detection performance.

- For D2R transmission with short duration, the performance gain obtained from coherent detection is negligible.

- For D2R transmission with long duration,

  - Error floor can be observed even with the insertion of multiple midambles/postamble.

  - Performance of non-coherent detection may outperform coherent detection in high SNR region.

For interference estimation, in our understanding, the “interference” at least includes CW interference (self-interference) and/or D2R interference that is/are from FDMed devices when checking the interference impact on the D2R performance with and without D2R midamble. It may be beneficial to also consider the interference that comes from NR for co-existence scenario. However, currently, it is difficult to consider the NR interference for D2R transmission with/without D2R midamble. Since depending on the used CW spectrum (DL or UL), Topologies etc., the NR interference can be various. It is not clear yet whether/which co-existence scenario is feasible, which is led by RAN4.

Observation 2.2-6: The interference estimated by D2R midamble, includes the CW interference and the interference comes from the FDMed A-IoT device.

For CW interference, it may not be necessary to perform the estimation during the D2R transmission, it can be estimated before the D2R transmission considering the CW signal stability can be ensured by the CW provision node. Therefore, midamble may not be needed for CW interference estimation. For estimation of the interference caused by FDMed device(s), further discussion is needed on how midamble is used and the extent of performance improvement.

Observation 2.2-7: D2R midamble may not be needed for CW interference estimation since the CW interference can be estimated before the D2R transmission.

Observation 2.2-8: D2R midamble can be used for estimation of the interference from multiple A-IoT devices and the interference impact and performance improvement need to be checked.

If midamble is necessary for PDRCH transmission, the midamble design including composition, structure, and location within the PDRCH transmission should be flexible and efficient to support different purposes.

Proposal 2.2-2: If D2R midamble is needed for PDRCH transmission, the design should be flexible and efficient to support different purposes.

  - D2R postamble

The D2R postamble was proposed for two purposes.

- Purpose 1: Indication of end of PDRCH transmission

- Purpose 2: Further time acquisition at end of PDRCH transmission at the reade side

For the purpose 2 of further time acquisition at end of PDRCH transmission

For using postamble at the Reader side to recover finer timing for PDRCH transmission, in case midamble is introduced for timing maintenance and correction, then in our view, postamble is not needed for the same purpose. In addition, the postamble is transmitted at the end of the transmission, it may have impacts on the processing timeline at the gNB/UE side. Therefore, following proposal is made.

Observation 2.3-1: If D2R postamble is used for reader to recover the timing of PDRCH transmission, it may have impacts on the processing timeline at the Reader side. In addition, postamble is not needed in case midamble is introduced.

Proposal 2.3-1: For the reader to track the timing of PDRCH transmission, D2R midamble can be used instead of D2R postamble.

  - On alignment with NR symbol boundary

In RAN1#106bis, following agreement was made [2]:

Agreement For R2D transmission, if OFDM-based waveform is used, the start of R2D transmission from reader perspective is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation.

On whether A-IoT R2D transmission from Reader perspective, D2R reception from Reader perspective need to be aligned with NR slot boundary, during last meeting discussion, it was mentioned that even in existing NR, the start and/or the end of the NR DL transmission or NR UL reception from gNB perspective does not need to be aligned with the NR slot boundary. Such flexible frame structure should also be applied to A-IoT transmission/reception from Reader perspective. It is not necessary to further discuss this point.

Then, the remaining issues are following:

- Whether the end of R2D transmission from reader perspective needs to be aligned with NR OFDM symbol boundary or not, if OFDM-based waveform is used?

- Whether the start and/or the end of the D2R transmission from device perspective and D2R reception from reader perspective can be assumed to be aligned with NR OFDM symbol boundary or not?

For R2D transmission, depending on the M value for M-chip per OFDM symbol and TBS, the overall valid information bit for the R2D transmission may not be always aligned with the NR OFDM symbol boundary. For example, in case M=6 with Manchester coding and TBS=16 bits, 32 chips which is around 5.3 OFDM symbols are needed to transmit the valid information bits with certain OOK ON-OFF pattern.

Proposal 2.4-1: For R2D transmission, the end of R2D transmission from Reader perspective may or may not be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation.

For D2R transmission, let’s first review the uplink transmission in NR system. NR is a synchronized system, each UE maintains the synchronization with the gNB and knows about the frame/subframe/slot/OFDM symbol index. As shown in TS 38.211 [7] Figure 4.3.1-1, in a given frame/subframe/slot/OFDM symbol with index i, when a UE performs uplink transmission, the UE is required to adjust its uplink transmission timing by applying timing advance (TA) with reference to the downlink frame/subframe/slot/OFDM symbol with index i, so that the received uplink transmissions at gNB side is aligned with downlink frame/subframe/slot/OFDM symbol with index i. Therefore, in NR, from the transmitter i.e., UE perspective, after applying the TA, the actual uplink transmission is not aligned with the NR OFDM symbol boundary. But from the receiver, i.e., gNB perspective, the actual reception is aligned to the NR OFDM symbol boundary within the CP range.

In A-IoT system, it is a kind of asyncronized system, different from NR UE, the A-IoT device is not able to know and maintain the NR frame/subframe/slot/OFDM symbol index. In addition, considering the short communication range (~10m - ~50m), the AIoT device does not apply or maintain a timing advance. Therefore, from the transmitter i.e., device and the receiver i.e., Reader perspective, the start of D2R transmission or reception cannot be assumed to be aligned with NR OFDM symbol boundary within the CP range.

Proposal 2.4-2: A-IoT device is not required to acquire the NR frame/subframe/slot/OFDM symbol index.

Proposal 2.4-3: For D2R transmission, A-IoT device does not apply and maintains the timing advance (TA).

For both R2D and D2R transmission, an OOK symbol/chip length can be defined as the basic unit of time-domain resource allocation for A-IoT R2D and D2R transmission. It is also desirable to support integer multiple numbers of OOK symbols within a slot for efficient resource usage and enable slot-level TDMed multiplexing between NR and AIoT. The detailed design for an OOK symbol duration also depends on target data rate, waveform/modulation, and tolerance to timing variation (e.g., timing error, or channel variation).

Proposal 2.4-4: From the A-IoT device side, an OOK symbol/chip length can be considered as the basic resource unit for constructing A-IoT DL/UL channel/signals.

- Impact of device unavailability

It was concluded in RAN1#117 meeting that RAN1 discussion related to the potential impact of device unavailability due to charging by energy harvesting will occur in agenda item 9.4.2.2. In addition, RANP#103 agreed that the duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds. In this section, we studied the impacts of device unavailability due to energy harvesting for Tx/Rx procedure for different duty-cycle based schemes. We also provide the evaluation results on the inventory completion time for multiple devices with different duty-cycle schemes and different options for Reader sending R2D transmissions.

  - Duty-cycle schemes

In this section, we briefly describe each duty cycle scheme. Before describing each scheme, it is assumed that a A-IoT device can support following states as shown in Table 1 based on [4].

Table 1: Function(s) that supported and unsupported in each device state

| Device states | Supported functions | Unsupported functions |
| --- | --- | --- |
| On | - transmission, reception for communication - maintain memory - run a clock for Tx/Rx | - energy harvesting (device has a single antenna for both communication and RF energy harvesting in TDMed manner) |
| Off (Fully or Partially off) Note | - energy harvesting | - transmission, reception for communication - maintain memory - run a clock |
| Sleep | - energy harvesting - maintain a memory content from On state - run a clock for time counting | - transmission, reception for communication |
| Note: Fully off means device is off when all energy in the storage is consumed. Partially off means device is off when the energy in the storage is lower than the threshold (e.g., 90% of available energy). |  |  |

We studied following three duty cycle schemes.

- Scheme 1: Device without sleep state and with fully off state (i.e., ‘w/o sleep w fully off’ for short)

For scheme 1, a device is in On state to continuously monitor R2D transmissions as long as it has energy in the storage and the received RF power exceeds the activation threshold. Once A-IoT device runs out of its total energy, it goes to off state and harvests energy. From reader perspective, reader does not know and cannot control the states of the device.

- Scheme 2: Device without sleep state and with partially off state (i.e., ‘w/o sleep w partially off’ for short)

For scheme 2, a device is in On state to continuously monitor R2D transmissions when the energy in the storage is above a certain threshold (e.g., 90% of the total energy in the storage) and the received RF power exceeds the activation threshold. Different from scheme 1, the device will enter into off state to be charged if the remaining energy in the storage is lower than the threshold. Same as scheme 1 that reader does not know and cannot control the states of the device. Compared to scheme 1, scheme 2 has the benefits that device unavailable time for energy harvest is short. And once device is paged, it can have sufficient energy to finish the communication with the Reader while at the cost of the implementation complexity at the device side.

- Scheme 3: Device with sleep state and under reader control (i.e., ‘w/o sleep w R2D control’ for short)

For scheme 3, it is assumed that reader can control device’s state by indicating the On and/or Sleep time and the On/Sleep periodicity for the device. For example, before inventory, reader will send such indication to align the device’s On time and device who received the indication will monitor the A-IoT paging only within the On time duration periodically. During the sleep time, the device harvest the energy. During the On time, once the device receives the target paging, it continues the communication with the Reader until the inventory is finished or device runs out of energy. Note that during sleep time, if the harvest energy cannot compensate the energy that is consumed by the device to run clock and retain the memory, eventually, the device will go to off state. Compared to scheme 1 and scheme 2, scheme 3 has the benefit to control device’s On/Sleep time to save the resource overhead.

Along with above duty cycle schemes, following two options for Reader to transmit the R2D e.g., A-IoT paging are studied.

- Option 1: Reader transmits the PRDCH periodically (i.e., call ‘P-R2D Tx’ for short)

- Option 2: Reader transmits the PRDCH multiple times on demand (i.e., call ‘A-R2D Tx’ for short)

R2D transmission Option 1 can be used for all the three duty cycle schemes. For Scheme 1 and Scheme 2, although R2D transmission is periodic, it is transparent to the device; For Scheme 3, the R2D transmission time and periodicity is aligned with device’s On time and periodicity, hence is viewed as non-transparent to the device.

Additionally, when reader cannot control and does not know device’s state, R2D transmission Option 2 is used to reduce the probability that the device missed the R2D transmission due to its Off state. Therefore, Option 2 is used for duty cycle Scheme 1 and Scheme 2.

  - Random access with duty-cycle schemes

In this section, we provide our methods on applying the duty cycle schemes to the A-IoT random access followed by the evaluation results.

Device energy initialization before random access/inventory

- For scheme 1, the device energy is initialized randomly between 0% and 100% of the maximum available energy in storage, considering the device may be in either On or Off state at the beginning of inventory.

- For scheme 2, the device energy is initialized randomly between a threshold i.e., 90% and 100% of the maximum available energy in storage.

- For scheme 3, the device energy initialization depends on the relationship of energy harvesting efficiency and the total power consumption per On/Sleep period. There are two cases:

  - In case the energy harvesting efficiency is lower than the total power consumption per period which means device energy will be consumed a little per On/Sleep period before inventory round. For this case, the device energy is initialized randomly between 0% and 100% of the maximum available energy in storage.

  - In case the energy harvesting efficiency is higher than the total power consumption per On/Sleep period, the device energy is initialized as 100% at the beginning of inventory.

During inventory

RAN2#126 meeting made following agreements:

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

4  The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

5   “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Based on RAN2 agreement, the random access/inventory procedure including following steps without considering Msg4:

- Step 1 Reader sends A-IoT Paging: The A-IoT paging message triggers all devices to access the network

  - There are two options for reader to transmit the paging, option 1 is periodic transmission which is used for all duty cycle schemes; option 2 is aperiodic or on demand transmission which is more suitable for duty cycle scheme 1 and 2 that reader cannot control device’s state/duty cycle.

- Step 2 Device transmits A-IoT Msg1.

  - For duty cycle scheme 1 and duty cycle scheme 3, the device that has received the A-IoT paging and the energy in storage is not 0% will respond to the paging and randomly select one of the multiple TDMed/FDMed Msg1 resources.

  - For duty cycle scheme 2, the device that has received the A-IoT paging and the energy in storage is not less than 90% will respond to the paging and randomly select one of the multiple TDMed/FDMed Msg1 resources.

- Step 3 Reader may or may not send A-IoT Msg2.

  - If there is no collision on at least one Msg1 resource, Reader will send the Msg2 to schedule the subsequent Msg3 transmisison.

  - If there is collision on all TDMed/FDMed Msg1 resources,

    - For Option 1 that reader transmits the PRDCH periodically, the reader will send the A-IoT paging at next period.

    - For Option 2 that reader transmits the PRDCH aperiodically, the reader will send the A-IoT paging again right after identifying the collision.

- Step 4 Device transmits A-IoT Msg3.

  - If in step 3, no collision happens , which meas Reader sends Msg2 schedules the device to transmit a Msg3 including device ID. Then this device is inventoried successfully.

Repeat above steps until all devices are inventoried successfully.

Note that for all three duty cycle schemes, for the device that joins the inventory, the device can sleep to harvest RF energy during the time gap between reception i.e., paging/Msg2 and transmission i.e., Msg1/Msg3 within current inventory process. The detailed simulation assumptions can be found in Table 2 below.

Table 2: Simulation assumptions

| Parameter |  | Values |
| --- | --- | --- |
| Number of AIoT device |  | 20 |
| Capacitance/Avail Energy size |  | 1uF/ 0.25 uJ |
| Rx power |  | -28dBm/ -32dBm |
| RF energy harvesting |  | - 0.2uW/s @ -28dBm - 0.05uW/s @ -32dBm |
| EH conversion efficiency / coverage |  | - For 0.2uW/s @ -28dBm, 12.6% / 11.57m for D1T1 and 6.04m for D2T2 - For 0.05uW/s @ -32dBm, 7.9% / 17.62m for D1T1 and 10.29m for D2T2 |
| Power consumption | Rx | 1uW |
|  | Tx | 1uW |
|  | sleep | 0.1uW |
|  | Off | 0 |
| Random access/ Inventory procedure | Paging | 1ms |
|  | Time gap between Paging and Msg1 | 1ms |
|  | Msg1 | 0.5ms |
|  | Time gap between Msg1 and Msg2 | 1ms |
|  | Msg2 | 0.5ms |
|  | Time gap between Msg2 and Msg3 | 1ms |
|  | Msg3 | 3ms |
| Number of RA resources for Msg1-RN16 |  | 4/8 resources per A-IoT paging |
| Duty cycle Schemes | Scheme1: w/o sleep w fully off | Charging time: Until 100% charged from 0% energy On time: energy in storage dropped from 100% to 0% |
|  | Scheme2: w/o sleep w partially off | Charging time: Until 100% charged On time: energy in storage dropped from 100% to 90% |
|  | Scheme3: w sleep w R2D control | Duty cycle Periodicity = 20ms or 12ms On time duration = 1ms |
| Periodicity of R2D transmission (A-IoT paging) |  | 20ms or 12ms. The periodicity is - Known for device with duty cycle scheme3; - Unknown for device with duty cycle scheme 1 and 2 |

Evaluation results

Based on the assumptions in table 2, we evaluate three cases to observe the relationship of inventory time and paging period/ received power/ RA resource number for all schemes.

- Case 1: For received power of 0.2uW/s @ -28dBm and 20ms paging period

In case 1, the received power of 0.2uW/s @-28dBm is higher than power consumption of Sleep state for scheme 3, which means energy harvesting can compensate the power consumption of maintaining memory or running a clock.

Figure 3.2-1 inventory time for case 1 with 4 RA resources

Figure 3.2-2 inventory time for case 1 with 8 RA resources

Figure 3.2-1 and 3.2-2 show inventory time for case 1 with 4 and 8 RA resources separately. Apparently, RA resources number has an important effect on scheme 3, i.e., more RA resource number and less inventory time. However, scheme 1 is not sensitive to RA resource number.  For scheme1, once the energy in storage is running out, the device must charge for a long period, which is shown as a horizontal line in figure 3.2-1 and 3.2-2. And final inventory time of case 1 with 4 and 8 RA resource is counted for scheme 1/2/3 in table 3.

Table 3: Inventory time for identifying 100% devices for case 1 of -28dBm Rx power and 20ms paging period

| Scheme | Inventory time [ms] |  |
| --- | --- | --- |
|  | 4 RA resources | 8 RA resources |
| P-R2D Tx +w/o sleep w fully off | 1428 | 1408 |
| P-R2D Tx +w/o sleep w partially off | 948 | 468 |
| P-R2D Tx +w sleep w R2D control | 908 | 188 |

Observation 3.3-1: Generally, inventory time decreases with the increased Random Access (RA) resources for all duty cycle schemes.

- Inventory time of duty cycle scheme 3(i.e., Device w sleep w R2D control) can decrease rapidly by increasing RA resources.

- Inventory time of duty cycle Scheme 1(i.e., Device w/o sleep w fully off) is not sensitive to RA resources

- Case 2: For received power of 0.05uW/s @ -32dBm and 20ms paging period

In case 2, the received power of 0.05uW/s @-32dBm is lower than power consumption of Sleep state for scheme 3, which means energy harvesting cannot compensate the power consumption of maintaining memory or running a clock.

Figure 3.2-3 inventory time for case 2 with 4 RA resoures

Figure 3.2-4 inventory time for case 2 with 8 RA resoures

Figure 3.2-3 and 3.2-4 show inventory time for case 2 with 4 and 8 RA resources separately. Apparently, the total inventory time beomes much larger since energy harvesting cannot compensate the power consumption of maintaining memory or running a clock. In this situation, even device is in Sleep state, the device is discharging due to low received power. Similar to scheme 1, there is a horizontal line of scheme 3 which means the device is running out of battery in figure 3.2-3 and 3.2-4. And final inventory time of case 2 with 4 and 8 RA resource number is counted for scheme 1/2/3 in table 4.

Table 4: Inventory time for identifying 100% devices for case 2 of -32dBm Rx power and 20ms paging period

| Scheme | Inventory time [ms] |  |
| --- | --- | --- |
|  | 4 RA resources | 8 RA resources |
| P-R2D Tx +w/o sleep w fully off | 5188 | 5068 |
| P-R2D Tx +w/o sleep w partially off | 2128 | 1608 |
| P-R2D Tx +w sleep w R2D control | 5508 | 5028 |

Observation 3.3-2: Generally, inventory time increases a lot with the reduced RF activation power.

- When the harvest RF energy is lower than the power consumption for device with scheme 3 (Device w sleep w R2D control), the inventory time becomes comparable or longer compared to the device with scheme 1 (Device w/o sleep w fully off) or scheme 2 (Device w/o sleep w partially off) respectively.

- Case 3: For received power of 0.2uW/s @ -28dBm and 12/20ms paging period

Device will go to sleep after transmit Msg3 until next paging period for periodic R2D transmission in scheme 3. In fact, reader can also page device at this time. So, to make a fair comparison between periodic R2D option with shortest paging period for scheme 3 and aperiodic R2D option for scheme 1/2 (i.e., the curves in purplish-pink). Paging with a period of 12ms is evaluated as below.

Figure 3.2-5 inventory time for case 3 with 12ms paging period

Figure 3.2-6 inventory time for case 3 with 12ms paging period

Figure 3.2-5 and 3.2-6 show inventory time for case 3 with 12ms and 20ms paging period separately. Apparently, the final inventory time can be decreased when Aperiodic paging is sent immediately once reader realize there are collision for all resources. Shorter paging period results in  less inventory time with periodic paging for scheme 2 and 3. Final inventory time of case 3 with 12ms and 20ms paging period is counted for all schemes in table 5.

Table 5: Inventory time for identifying 100% devices for case 3 of -28dBm Rx power and 12/20ms paging period

| Scheme | Inventory time [ms] |  |
| --- | --- | --- |
|  | Paging period 12ms | Paging period 20ms |
| P-R2D Tx +w/o sleep w fully off | 1448 | 1428 |
| P-R2D Tx +w/o sleep w partially off | 644 | 948 |
| P-R2D Tx +w sleep w R2D control | 548 | 908 |
| A-R2D Tx +w/o sleep w fully off | 1360 | 1368 |
| A-R2D Tx +w/o sleep w partially off | 304 | 304 |

Observation 3.3-3: Generally, inventory time decreases with the increased number of Paging transmissions by reducing the paging Tx periodicity and/or transmitting paging aperiodically/on demand.

- Scheme 2 (Device w/o sleep w partially off) with periodic paging Tx reduces the inventory time a lot compared to the paging Tx with longer(20ms) periodicity.

- Scheme 2 (Device w/o sleep w partially off) with aperiodic paging Tx has shorter inventory time compared to Scheme 3 (Device w sleep w R2D control) for both 12ms-periodicity and 20ms-periodicity of paging Tx.

Based on above analysis, following proposals can be considered.

Proposal 3-1: Following FL High Priority Proposal 4.1 in R1-2405381 can be re-considered

- FL1 High Priority Proposal 4.1: To reduce the impact of energy harvesting on device availability for transmission and reception procedures, RAN1 studies following options

  - Option 1: Reader transmits the PRDCH multiple times to handle the devices that missed the PRDCH reception due to charging

  - Option 2: Reader provides R2D signals or PRDCH indicating the wake-up time occasion/duration or sleep time occasion/duration to extend A-IoT device availability

Proposal 3-2:

- For option 1 that Reader transmits the PRDCH multiple times to handle the devices that missed the PRDCH reception due to charging, device is assumed to have two states: ON, OFF

- For option 2 that Reader provides R2D signals or PRDCH indicating the wake-up time occasion/duration or sleep time occasion/duration to extend A-IoT device availability, device is assumed to have three states: ON, OFF, SLEEP

Proposal 3-3: Following proposal in RP-241688 can be re-considered

- Identify function(s) of the device that can be assumed supported and assumed not supported for each device state, subject to:

  - ON state supports at least: transmission, reception for communication

  - OFF state does not support at least: transmission, reception for communication

  - OFF state supports at least: energy harvesting

  - SLEEP state supports at least:

    - Maintaining a memory content from ON state

    - Maintaining a timer (RAN1 to discuss purpose(s) of timer)

  - SLEEP state does not support at least: transmission

- Resource allocation for A-IoT Random access

For A-IoT random access, it was guided that the procedure should be studied by RAN2, while the physical resources and physical channel(s)/signal(s) should be studied by RAN1. For this agenda 9.4.2.2, the physical resources for the transmissions during random access, i.e., A-IoT Msg1 and Msg3 should be the focus, which is discussed in the following.

Since it was agreed to study time-domain multiple access and frequency-domain multiple access at least by utilizing a small frequency-shift in baseband for D2R transmissions. It is beneficial to study following:

- TDMA and/or FDMA of Msg.1 D2R transmissions from multiple devices corresponding to the same A-IoT paging transmission for contention-based access

- TDMA and/or FDMA of Msg.3 D2R transmissions from multiple devices corresponding to the same Msg2 transmission

In the following, we provide our views on the physical layer time-domain and/or frequency-domain resource allocation considering the cases of TDMA and/or FDMA for D2R transmissions during the contention-based access. Our discussion is based on A-IoT Msg1, but the solution developed can be applied to A-IoT Msg3 as well.

Time-domain resource allocation/determination for D2R transmission (e.g., A-IoT Msg1)

For TDMA of multiple Msg1 transmissions from multiple devices scheduled by one A-IoT paging, the time-domain resource for each Msg1 can be determined by the Msg1 transmission duration/length and the Msg1 resource start time.

For Msg1 transmission duration/length, it is decided by the Msg1 TBS and Msg1 data rate. The Msg1 TBS can be a fixed value like 16 bits and the Msg1 data rate can be further derived by the D2R chip length, coding rate and repetition factor if any which can be indicated by the A-IoT paging. In case of TDMA of multiple Msg1 transmissions, the Msg1 transmission length derived/indicated in the A-IoT paging message should be the same for all TDMed Msg1 transmissions. However, considering different device implementations may have different SFO and the effect of clock drift, the later the device transmits in the time domain, the greater the uncertainty of its actual transmission duration becomes. Therefore, some gaps in time domain between adjacent Msg1 transmission should be reserved.

As discussed above, for multiple TDMed Msg1 transmissions triggered by one A-IoT paging, other than the starting time for each Msg1 transmission, other scheduling information such as TBS, chip length, coding rate and repetition factor if any should be the same. However, for multiple TDMed Msg3 transmissions triggered by one A-IoT Msg2, besides the starting time for each Msg3 transmission is different/independent, further discussion is needed on whether other scheduling information, such as chip length, coding rate, repetition number if any, can also be different, as this may have impacts on the Msg2 format design.

Observation 4-1: For the case that one R2D triggers more than one TDMed D2R transmission from different devices, the later the D2R transmission in the time domain, the greater the uncertainty of its actual transmission duration becomes due to device’s clock drift.

Proposal 4-1: For multiple TDMed Msg1 transmissions from multiple devices corresponding to the same A-IoT paging, the Msg1 transmission duration/length is the same and determined by the Msg1 TBS and Msg1 data rate.

Proposal 4-2: For multiple TDMed Msg3 transmissions from multiple devices corresponding to the same A-IoT Msg2, the Msg3 transmission duration/length is determined by the Msg3 TBS and Msg3 data rate.

- FFS transmission duration/length is the same or can be different for each Msg3 transmission.

For Msg1 resource start time, it is also related to the Msg1 response timing. There are two options agreed in RAN1#117 meeting for studying D2R response timing, which is also further discussed in section 5.1.2. In high level, one option is based on a time window by TR2Dmin and TR2Dmax, simiar to RFID; the other option is based on transmission timing indication, simiar to NR k2 indication for NR PUSCH. In case of TDMA of multiple e.g., two Msg1 transmissions, if option 1 that the time window [TR2Dmin and TR2Dmax] is predefined/fixed in the specification, it can work for the 1st Msg1 transmission, but it is not so straightforward and efficient to be used to determine the response timing for the 2nd Msg1 transmission. Since for the 2nd Msg1 transmission, TR2Dmin and TR2Dmax from processing perspective are always met by the device. Following options can be considered to determine each Msg1 resource start time for the case of more than one Msg1 is scheduled/triggered by the same Paging.

Option 1: Extension of the time window based solution to TDMed Msg1 transmission: introducing/defining a main time window [TD2Rstart, TD2Rend] that covers all the TDMed Msg1 resources/transmission

- The main time window is split equally or unequally into sub-windows, the number of sub-windows is equal to the number of the TDMed Msg1 resources/transmission and each Msg1 resource/transmission is confined within the sub-window.

Option 2: Indication based where multiple Msg1 transmission timing is indicated by Paging

| 4-1a: Same size/length for 1st and 2nd sub-windows | 4-1b: Different size/length for 1st and 2nd sub-windows |
| --- | --- |

Figure 4-1: Illustration for Option 1 that a main time window covers all the TDMed Msg1 resources/transmission

Figure 4-2: Illustration for Option 2 that multiple Msg1 response timing is indicated by Paging

As observed, see observation x that the later the D2R transmision in the time domain, the greater the uncertainty of its actual transmission duration may become due to device’s clock drift. Therefore, the option 1 that the main time window split unequally into sub-windows can be considered to improve the resource usage, as shown in Figure 4-1b.

For above both options, the number of TDMed Msg1 resources/transmission needs to be explicitly or implicitly indicated by Paging. The maximum number of Msg1 resources that can be TDMed depends on how long the device can count the time with sufficient accuracy. Then the basic time unit for the indications e.g., start/end time for the main or sub window for option 1, the response timing indication for option 2 need to be determined. In our view, the basic time unit can be integer multiple chip length or information bit length for R2D and/or D2R to balance between the overhead and finer granularity.

In addition, on the timing error (initial/residual SFO) handling for a D2R transmission

- for option 1, the main window position in time domain (start time and end time/ window length) or the sub-window position in time domain need to be indicated by the paging. and the sub-window length should be sufficient to cover Msg1 transmission duration and device’s SFO

- for option 2, Reader should detect a D2R transmission at the indicated time e.g., TR2D with a time gap tolerance e.g., Te, for example, reader detects Msg1/Msg3 within [TR2D-Te, TR2D+Te]

Proposal 4-3: Study multiple TDMed Msg1(/Msg3) transmissions from multiple devices corresponding to the same A-IoT paging(/Msg2), including following

- The number of TDMed Msg1(/Msg3) resources is indicated by Paging(/Msg2) explicitly or implicitly.

  - FFS the maximum number of TDMed Msg1 considering how long time the device can count the time with sufficient accuracy after receiving an R2D transmission.

- The start time for each Msg1(/Msg3) resource is based on following two options

  - Option 1: Split a main time window into multiple sub-windows and each Msg1(/Msg3) resource is confined within the sub-window.

  - Option 2: The start time for each Msg1(/Msg3) resource is indicated by Paging(/Msg2).

- The basic time unit for resource allocation in time domain is integer multiple chip length or information bit length for Paging(/Msg2) and/or Msg1(/Msg3) transmission.

Frequency-domain resource allocation/determination for D2R transmission (e.g., A-IoT Msg1)

About frequency domain resource allocation for FDMA of multiple A-IoT Msg1/Msg3 transmissions, it can be determined by the frequency location and transmission bandwidth for each Msg1/Msg3 transmission.

For the transmission bandwidth, it is derived by the data rate. Similar as discussed for transmission duration for Msg1 and Msg3, for Msg1, the transmission bandwidth is the same for all the FDMed Msg1 transmissions scheduled by the same A-IoT paging. Further discussion is needed on whether the transmission bandwidth is the same or can be different for all the FDMed Msg3 transmissions corresponding to the same Msg2.

Proposal 4-4: For multiple FDMed D2R transmissions from multiple devices corresponding to the same R2D transmission, the D2R transmission bandwidth is determined by the D2R data rate.

- The transmission bandwidth is the same for each FDMed Msg1 transmission.

- FFS the transmission bandwidth is the same or can be different for each FDMed Msg3 transmission.

About the frequency location, for device 2b with internally generated D2R transmission, it is straightforward to indicate the different FDMed frequency resource for different A-IoT devices for D2R transmissions by R2D control information.

For device 1/2a with D2R backscattered on a CW, different frequency resources for different A-IoT devices can be achieved by allocating different backscatter-link frequency (BLF) values in R2D control information as shown in Figure 4-3. For this case, there would be harmonic component of backscattered D2R transmisison from an A-IoT device that interferes D2R transmisison from another A-IoT device. Some frequency-domain resource shall be reserved to reduce inter-A-IoT device interference. Therefore, the maximum number of FDMed D2R transmisisons depends on the A-IoT device frequency shift capability, reserved frequency resources and D2R data rate etc.

Proposal 4-5: For multiple FDMed D2R transmissions from multiple devices corresponding to the same R2D transmission,

- For device 1/2a with D2R backscattered on a CW, different backscatter-link frequency (BLF) values indicated by R2D control information is used to allocate different frequency resources

- For device 2b with internally generated D2R transmission, it is straightforward to indicate the different FDMed frequency resource by R2D control information.

- FFS the maximum number of D2R transmisisons that can be FDMed, considering the A-IoT D2R occupied bandwidth, D2R transmission bandwidth, frequency shift capability etc.

Figure 4-3. Example of different BLFs applied for FDMed A-IoT Devices in D2R link

This section discussed about the physical resources for the D2R transmissions during random access. Additionally, how device receives Msg2 after it transmits Msg1 needs to be discussed. This aspect is also related to the timing for R2D response to D2R, which is discussed in section 5.1.3.

- Scheduling and timing relationships

This section provides our views on timing relations and scheduling/control information for R2D and D2R transmissions.

  - Timing relations

In RAN1#106 meeting, following agreements were made for A-IoT scheduling and timing relations [1]:

Agreement For further discussion, the following terminologies are used for A-IoT for studying processing time aspects: - TR2D_min: Minimum Time between a R2D transmission and the corresponding D2R transmission following it. - TD2R_min: Minimum Time between a D2R transmission and the corresponding R2D transmission following it. - TR2D_R2D_min: Minimum Time between two different consecutive R2D transmissions to the same A-IoT device. - TD2R_D2R_min: Minimum Time between two different consecutive D2R transmissions from the same A-IoT device. - The study should consider at least following aspects - Implementation restrictions for the existing BS/UE - [Processing time is common or different for different A-IoT devices] - [Processing time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command)] - FFS other timing aspects

Note that about the exact time point (e.g. starting and/or ending time) for the above time intervals, it can be discussed after more progress is made for D2R and R2D waveform, basic resource allocation unit e.g. chip or bit symbol in time-domain, whether there is postamble or not etc.

    - Timing for D2R response to R2D

For a single D2R transmission scheduled by a R2D transmission

In RAN1#107 meeting, following agreement was made for D2R response timing to a R2D [3]:

Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D

Option 1 is the similar mechanism used in RFID. Based on last meeting’s discussion, the maximum time is kind of requirement that all the devices and readers are required to be capable of sending the transmission before no later than the maximum time. For option 1, it makes more sense to define the maximum time in the specification. If the maximum time is based on dynamic indiction in the R2D control information, then option 1 has no advantage compared to option 2.

Option 2 is the similar mechanism used in NR for PUSCH scheduling. Option 2 allows the D2R response time determined by the reader, offering a higher flexibility for scheduling as well as avoiding prolonged waiting/ambiguous time. For example, take RFID as reference, the maximum time for the delayed or inprocess relay can be as long as 20ms. In addition, as discussed in section 4, it is easier to use Option 2 to support multiple TDMed Msg1 or Msg3 tansmissions to reply to the same A-IoT paging/Msg2 transmisison.

Proposal 5.1-1: if down-selection is needed during SI, support option 2 that the corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D  TR2D_min

For TDMA of multiple D2R transmission scheduled by a R2D transmission

About the D2R response timing for the case that one R2D transmission e.g., A-IoT paging schedules multiple TDMed D2R transmissions, the same solutions discussed in section for a device determining the start time for each Msg1(/Msg3) resource can be used as well.

    - Timing for R2D response to D2R

Similar to TR2D_max, there were also some discussions on the purpose of the TD2R_max. In our understanding, this time limit provides some guidance for the device. For example:

For the inventory use case using the contention access procedure, if a device sends Msg.3 but does not receive a response within TD2R_max time, it may assume that Msg.3 transmission was successful and the contention access is finished. Then, the device can update its state and proceed with subsequent transmissions if any. However, if the device receives a NACK within TD2R_max time, it considers the contention access for the current inventory failed, the device maintains its current state and can continue to participate in the inventory for the same session.

For the command use case, if no feedback is received from the reader within TD2R_max time limit, it may be beneficial for the device to consider D2R transmission as successful and stop R2D monitoring to save power. Therefore, following basic framework on the timing for R2D response to D2R is proposed.

Proposal 5.1-2: When a R2D transmission in response to a D2R transmission is expected for a A-IoT device, study a design with a maximum time TD2R_max between the D2R transmission and the expected R2D transmission following it, so that the device expects to receive the R2D transmission within [TD2R_min, TD2R_max].

- FFS device behavior if the device does not receive the expected R2D response within the window.

- This does not preclude to study using R2D control information indicating the timing of the R2D response

Additionally, how a device receives Msg2 after it transmits Msg1 for the case that multiple TDMed Msg1 transmissions are scheduled by one A-IoT paging message also need to be discussed. Based on above time window framework, the device expects to receive the Msg2 transmission within a Msg2 response time window, following two options for the Msg2 response time window can be considered.

- Option 1: Common window based that is the starting time for the Msg2 response time window is TD2R_min after the “last” Msg1 time domain resource that is scheduled by the A-IoT paging.

- Option 2: Independent Time window based that is the starting time for the Msg2 response time window is determined based on the control information in the A-IoT paging.

For both options, the length/duration for Msg2 response time window can be pre-defined and/or indicated. Figure 5-1 gives an example.

| 5-1a: Same size/length for 1st and 2nd sub-windows | 5-1b: Different size/length for 1st and 2nd sub-windows |
| --- | --- |

Figure 5-1: illustration on options for the Msg2 response time window

Between the two options, it is obvious that option 1 is simper, but option 2 has more flexibility compared to option 1. In case Reader has the flexibility to determine one Msg2 contian one or more than one response to echo the ID(s), in other words, no need to restrict that one Msg2 should contain all the responses to echo all the ID(s) the reader received in Msg1 transmissions, then Option 2 may have more benefit to help device power saving, of course at the cost of signaling overhead and/or complexity. Both options can be studied further.

Proposal 5.1-3: Study following options for a device to determine the Msg.2 reception timing in case one A-IoT paging schedules multiple TDMed/FDMed Msg1.

- Opt.1 Common time window based: the starting time for the Msg2 response time window is TD2R_min after the “last” Msg1 time domain resource that is scheduled by the A-IoT paging.

- Opt.2 Independent Time window: the starting time for the Msg2 response time window is determined based on the control information in the A-IoT paging.

- For both options, the length/duration for Msg2 response time window can be pre-defined and/or indicated.

    - Usage for TD2R_D2R_min

From our understanding, one usage of this time limit is similar to the "in-process reply, i.e., T7" in RFID. In RFID, if a tag needs more time to execute the received command, it can respond with "in-process" and subsequently send the D2R responses as long as they meet the specific time requirements.

Proposal 5.1-4: One of the use cases for TD2R_D2R_min between two different consecutive D2R transmissions from the same A-IoT device can be similar as the “in-process reply” adopted in UHF, that a A-IoT device may reply firstly to Reader that it needs more time for processing/prepration followed by the corresponding D2R response to the R2D command.

  - Scheduling aspects

In RFID C1G2 standard, it is the reader’s responsibility to determine the DL and UL transmission parameters and send the scheduling information to Tag by using the preamble and payload when initiating an inventory round. More specifically,

- for Tag DL transmission, the modulation can be DSB-ASK, SSB-ASK, or PR-ASK, and the Tag is able to demodulate all three modulation schemes; the coding format is fixed as PIE (Pulse-interval encoding); the transmission length for data ‘0’ and data ‘1’ is determined by measuring the length of Rtcal in the preamble.

- for Tag UL transmission, the modulation can be ASK or PSK which can be determined by the Tag, and Reader should demodulate both modulation schemes; the coding format can be FM0 with M=1 or Miller-modulated subcarrier with M=2,4,8; Tag’s backscatter link frequency is also determined and indicated by the Reader by Trcal in the preamble and divide ratio (DR) and modulation scheme M indicated in the payload of a Query command that initiates an inventory round.

Note that during an inventory round, the Preamble in the Query command sets the UL transmission parameters for the remainder of the inventory session, which means the transmission parameters do not change for the entire inventory round.

In NR, the scheduling can be classified as dynamic grant based which requires dynamic signaling, i.e., DCI to schedule each DL/UL transmission and configured grant based, for which the transmission is periodic and configured by semi-static signaling, implying that the transmission parameters like resource, payload size, power cannot be dynamically changed.

    - Reader’s identification of the end of PDRCH transmission

In RAN1#106bis meeting, following agreement was made for A-IoT D2R postamble [2]:

Agreement For the reader to acquire the end of PDRCH transmission, study at least following options: - Option 1: D2R postamble immediately follows the PDRCH - Option 2: Based on control information

For the purpose 1 of indicating the end of PDRCH transmission:

The observations made for R2D postamble also apply to D2R postamble to indicate the D2R transmission length. Besides, for PDRCH transmission, using D2R postamble implies that the A-IoT device should decide the PDRCH TBS. Then it is difficult for the Reader to control the resource usage and make scheduling since the Reader does not know when D2R transmission is finished until postambe is detected.

Observation 5.2-1: for indication of the end of PDRCH transmssion,

- D2R Postamble may not be needed for PDRCH with fixed TBS.

- D2R Postamble may result in PDRCH decoding failure or additional power consumption due to false or miss detection.

- D2R Postamble makes the scheduling and resource management difficult for the Reader

For using control information to indicate the end of PDRCH transmission, the control information can be R2D and/or D2R control information. Using R2D control information is straightforward and provides controllability for the Reader. For D2R control information, there could be different interpretations for the D2R control information. Firstly, physical layer D2R control information is not well justified based on the SID of no HARQ, ARQ [5]. Therefore, if D2R control information if any, it should be higher layer D2R control information with fixed TBS, similar as RFID Error codes. In case the PDRCH transmission includes only the higher layer D2R control information with fixed TBS, a header with the command code which is located in the early part of the PDRCH can be used for the Reader to know the end of PDRCH transmission. In case the PDRCH transmission includes the D2R data transmission with variable TBS, using R2D control information to indicate/derive the end of PRDCH transmission is preferred to give the reader controllability. In our view, the reader is possible to know and should control the PDRCH TBS/transmission length for all traffic types (i.e., DT, DO-DTT and DO-A) tailored for the target use cases.

Observation 5.2-2: Physical layer D2R control information is not well justified for A-IoT operation. If any D2R control information is needed, it should be higher layer D2R control information with fixed TBS.

Observation 5.2-3: In case the PDRCH transmission includes only higher layer D2R control information with fixed TBS, the TBS/transmission length can be determined by the higher layer D2R control information; In case the PDRCH transmission includes D2R data information with variable TBS, the TBS/transmission length determined by the R2D control information is preferred.

Proposal 5.2-1: Study the reader acquires the end of PDRCH transmission at least based on R2D control information.

- FFS D2R control information for PDRCH transmission with fixed TBS

    - Device’s identification of the end of PRDCH transmission

In RAN1#106bis meeting, following agreement was made for A-IoT R2D postamble [2]:

Agreement To determine or derive the end of PRDCH transmission, study at least following options: - Option 1: R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH. - Option 2: Based on R2D control information.

In the following, we provide our views for above two options.

For Option 1, the main benefit of using postamble to mark the end of R2D transmission is to achieve the most variable and flexible R2D payload size (TBS), e.g. the payload size can be as fine as 1 bit granularity. However, for the intended A-IoT use cases like indoor inventory and command, it is not clear how much flexibility is needed for the R2D payload size. For some specific R2D transmissions, like the R2D transmission during the contention based access, the TBS can be fixed without the postamble. In case the higher layer data e.g., MAC PDU is constructed with byte aligned, the benefit of using postamble to realize the variable/flexible R2D payload size at bit-level is reduced. In addition, the use of postamble to indicate the end of PRDCH transmission may result in decoding errors and additional power consumption due to the miss detection and false alarm.

The R2D control information in Option 2 can be classified into following:

- Physical layer control information that is used to schedule the PRDCH and PDRCH transmission.

- Higher layer control information such as command(s) like Inventory triggering, Read, Write etc.

For Physical layer control information scheduling the PRDCH and/or PDRCH, whether it exists depending on the necessity and benefits to

- provide different flexibility and reliability for the control information and data information in terms of used line coding scheme(s) e.g., Manchester encoding and/or pulse-interval encoding (PIE), chip length, e.g., same or different chip length etc.

- enable device to early detect/demodulate the control information to facilitate device’s subsequent behvaior, e.g., continues or skips data detection/demodulation

In case the Physical layer control information scheduling the PRDCH needs to be introduced, it can be used to indicate the TBS or transmission length for the PRDCH.

Alternatively, higher layer control information can also be used to indicate/derive the end of PRDCH transmission. For example, the device can derive the end of PRDCH or PDRCH transmission by some kind of Control Header which is located in the early part of the PRDCH. The Control Header includes the command code without or with length field indication for fixed TBS or variable TBS od PRDCH/PDRCH transmission. Different from Physical layer control information, it is assumed that the higher layer control information uses the same transmission parameters e.g., coding scheme, coding rate, chip length as those used for subsequent data transmission. In addition, depending on the format/structure of the higher layer control information/Control Header, e.g., shared or separate CRC for the higher layer control information/ Control Header and data, whether the higher layer control information/Control Header can be decoded before the data decoding, the benefit of using Physical layer control information to help device continue or skip data demodulation by early decoding the control information may also apply to the Higher layer control information.

In summary, regardless of using Physical layer or Higher layer control information, Option 2 that based on R2D control information is more robust compared to Option 1 that based on R2D postamble. Option 2 can also provide sufficient flexibility for payload size indication for the A-IoT inventory and command use cases. In addition, Option 2 can be a unified solution for the determination on the end of PRDCH and PDRCH transmission as discussed in section 2.5.

Observation 5.2-4: Using R2D Postamble to indicate the end of PRDCH transmission can enable any TBS for PRDCH transmission. However, it is not clear how variable TBS is required for PRDCH for indoor inventory and command use cases.

Observation 5.2-5: R2D Postamble may not be needed for PRDCH with fixed TBS.

Observation 5.2-6: R2D Postamble may result in PRDCH decoding failure or additional power consumption due to false or miss detection.

Observation 5.2-7: Using R2D control information to indicate or derive the end of PRDCH transmission can enable limited number of TBS for PRDCH transmission. There is tradeoff between the overhead of R2D control information and the number of TBS for PRDCH transmission.

Observation 5.2-8: Using R2D control information to indicate or derive the end of PRDCH transmission is more robust to the ambiguity of the end of transmission.

Proposal 5.2-2: The end of PRDCH transmission that is determined or derived by the R2D control information should be studied as baseline.

- FFS the R2D control information is/are the physical layer and/or higher layer control information

- Conclusion

In this contribution, we provide our views on frame structure and timing aspects for A-IoT. The observations, conclusions and proposals are summarized as follows.

Observations

Observation 2.2-1: Whether D2R midamble is needed for timing/frequency tracking for long D2R transmission length depends on factors like whether device can correct the timing by the received R2D transmission and whether reader can try a large number of SFO hypotheses, and the timing error/drift at device side etc.

Observation 2.2-2: The clock drift with 400ppm/s device side does not cause significant timing drift for long D2R transmission duration.

Observation 2.2-3: Reader can achieve more than 99% SFO estimation accuracy by detection of D2R preamble only with a limited number of SFO hypotheses at SNR of 16dB when the SFO at device side is 10^5 ppm.

Observation 2.2-4: Coherent detection has benefit in noise reduction as it retains the desired signal under accurate channel estimation. While enough midambles/postamble is necessary especially for D2R with long transmission.

Observation 2.2-5: For the case of FDMA of multiple D2R transmissions, midamble used for channel estimation may increase the overhead.

Observation 2.2-6: The interference estimated by D2R midamble, includes the CW interference and the interference comes from the FDMed A-IoT device.

Observation 2.2-7: D2R midamble may not be needed for CW interference estimation since the CW interference can be estimated before the D2R transmission.

Observation 2.2-8: D2R midamble can be used for estimation of the interference from multiple A-IoT devices and the interference impact and performance improvement need to be checked.

Observation 2.3-1: If D2R postamble is used for reader to recover the timing of PDRCH transmission, it may have impacts on the processing timeline at the Reader side. In addition, postamble is not needed in case midamble is introduced.

Observation 3.3-1: Generally, inventory time decreases with the increased Random Access (RA) resources for all duty cycle schemes.

- Inventory time of duty cycle scheme 3(i.e., Device w sleep w R2D control) can decrease rapidly by increasing RA resources.

- Inventory time of duty cycle Scheme 1(i.e., Device w/o sleep w fully off) is not sensitive to RA resources

Observation 3.3-2: Generally, inventory time increases a lot with the reduced RF activation power.

- When the harvest RF energy is lower than the power consumption for device with scheme 3 (Device w sleep w R2D control), the inventory time becomes comparable or longer compared to the device with scheme 1 (Device w/o sleep w fully off) or scheme 2 (Device w/o sleep w partially off) respectively.

Observation 3.3-3: Generally, inventory time decreases with the increased number of Paging transmissions by reducing the paging Tx periodicity and/or transmitting paging aperiodically/on demand.

- Scheme 2 (Device w/o sleep w partially off) with periodic paging Tx reduces the inventory time a lot compared to the paging Tx with longer(20ms) periodicity.

- Scheme 2 (Device w/o sleep w partially off) with aperiodic paging Tx has shorter inventory time compared to Scheme 3 (Device w sleep w R2D control) for both 12ms-periodicity and 20ms-periodicity of paging Tx.

Observation 4-1: For the case that one R2D triggers more than one TDMed D2R transmission from different devices, the later the D2R transmission in the time domain, the greater the uncertainty of its actual transmission duration becomes due to device’s clock drift.

Observation 5.2-1: for indication of the end of PDRCH transmssion,

- D2R Postamble may not be needed for PDRCH with fixed TBS.

- D2R Postamble may result in PDRCH decoding failure or additional power consumption due to false or miss detection.

- D2R Postamble makes the scheduling and resource management difficult for the Reader

Observation 5.2-2: Physical layer D2R control information is not well justified for A-IoT operation. If any D2R control information is needed, it should be higher layer D2R control information with fixed TBS.

Observation 5.2-3: In case the PDRCH transmission includes only higher layer D2R control information with fixed TBS, the TBS/transmission length can be determined by the higher layer D2R control information; In case the PDRCH transmission includes D2R data information with variable TBS, the TBS/transmission length determined by the R2D control information is preferred.

Observation 5.2-4: Using R2D Postamble to indicate the end of PRDCH transmission can enable any TBS for PRDCH transmission. However, it is not clear how variable TBS is required for PRDCH for indoor inventory and command use cases.

Observation 5.2-5: R2D Postamble may not be needed for PRDCH with fixed TBS.

Observation 5.2-6: R2D Postamble may result in PRDCH decoding failure or additional power consumption due to false or miss detection.

Observation 5.2-7: Using R2D control information to indicate or derive the end of PRDCH transmission can enable limited number of TBS for PRDCH transmission. There is tradeoff between the overhead of R2D control information and the number of TBS for PRDCH transmission.

Observation 5.2-8: Using R2D control information to indicate or derive the end of PRDCH transmission is more robust to the ambiguity of the end of transmission.

Proposals

Proposal 2.2-1: For comparison between non-coherent detection and coherent detection using preamble/midamble/postamble for D2R transmission, capture the following observations for detection performance.

- For D2R transmission with short duration, the performance gain obtained from coherent detection is negligible.

- For D2R transmission with long duration,

  - Error floor can be observed even with the insertion of multiple midambles/postamble.

  - Performance of non-coherent detection may outperform coherent detection in high SNR region.

Proposal 2.2-2: If D2R midamble is needed for PDRCH transmission, the design should be flexible and efficient to support different purposes.

Proposal 2.3-1: For the reader to track the timing of PDRCH transmission, D2R midamble can be used instead of D2R postamble.

Proposal 2.4-1: For R2D transmission, the end of R2D transmission from Reader perspective may or may not be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation.

Proposal 2.4-2: A-IoT device is not required to acquire the NR frame/subframe/slot/OFDM symbol index.

Proposal 2.4-3: For D2R transmission, A-IoT device does not apply and maintains the timing advance (TA).

Proposal 2.4-4: From the A-IoT device side, an OOK symbol/chip length can be considered as the basic resource unit for constructing A-IoT DL/UL channel/signals.

Proposal 3-1: Following FL High Priority Proposal 4.1 in R1-2405381 can be re-considered

- FL1 High Priority Proposal 4.1: To reduce the impact of energy harvesting on device availability for transmission and reception procedures, RAN1 studies following options

  - Option 1: Reader transmits the PRDCH multiple times to handle the devices that missed the PRDCH reception due to charging

  - Option 2: Reader provides R2D signals or PRDCH indicating the wake-up time occasion/duration or sleep time occasion/duration to extend A-IoT device availability

Proposal 3-2:

- For option 1 that Reader transmits the PRDCH multiple times to handle the devices that missed the PRDCH reception due to charging, device is assumed to have two states: ON, OFF

- For option 2 that Reader provides R2D signals or PRDCH indicating the wake-up time occasion/duration or sleep time occasion/duration to extend A-IoT device availability, device is assumed to have three states: ON, OFF, SLEEP

Proposal 3-3: Following proposal in RP-241688 can be re-considered

- Identify function(s) of the device that can be assumed supported and assumed not supported for each device state, subject to:

  - ON state supports at least: transmission, reception for communication

  - OFF state does not support at least: transmission, reception for communication

  - OFF state supports at least: energy harvesting

  - SLEEP state supports at least:

    - Maintaining a memory content from ON state

    - Maintaining a timer (RAN1 to discuss purpose(s) of timer)

  - SLEEP state does not support at least: transmission

Proposal 4-1: For multiple TDMed Msg1 transmissions from multiple devices corresponding to the same A-IoT paging, the Msg1 transmission duration/length is the same and determined by the Msg1 TBS and Msg1 data rate.

Proposal 4-2: For multiple TDMed Msg3 transmissions from multiple devices corresponding to the same A-IoT Msg2, the Msg3 transmission duration/length is determined by the Msg3 TBS and Msg3 data rate.

- FFS transmission duration/length is the same or can be different for each Msg3 transmission.

Proposal 4-3: Study multiple TDMed Msg1(/Msg3) transmissions from multiple devices corresponding to the same A-IoT paging(/Msg2), including following

- The number of TDMed Msg1(/Msg3) resources is indicated by Paging(/Msg2) explicitly or implicitly.

  - FFS the maximum number of TDMed Msg1 considering how long time the device can count the time with sufficient accuracy after receiving an R2D transmission.

- The start time for each Msg1(/Msg3) resource is based on following two options

  - Option 1: Split a main time window into multiple sub-windows and each Msg1(/Msg3) resource is confined within the sub-window.

  - Option 2: The start time for each Msg1(/Msg3) resource is indicated by Paging(/Msg2).

- The basic time unit for resource allocation in time domain is integer multiple chip length or information bit length for Paging(/Msg2) and/or Msg1(/Msg3) transmission.

Proposal 4-4: For multiple FDMed D2R transmissions from multiple devices corresponding to the same R2D transmission, the D2R transmission bandwidth is determined by the D2R data rate.

- The transmission bandwidth is the same for each FDMed Msg1 transmission.

- FFS the transmission bandwidth is the same or can be different for each FDMed Msg3 transmission.

Proposal 4-5: For multiple FDMed D2R transmissions from multiple devices corresponding to the same R2D transmission,

- For device 1/2a with D2R backscattered on a CW, different backscatter-link frequency (BLF) values indicated by R2D control information is used to allocate different frequency resources

- For device 2b with internally generated D2R transmission, it is straightforward to indicate the different FDMed frequency resource by R2D control information.

- FFS the maximum number of D2R transmisisons that can be FDMed, considering the A-IoT D2R occupied bandwidth, D2R transmission bandwidth, frequency shift capability etc.

Proposal 5.1-1: if down-selection is needed during SI, support option 2 that the corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D  TR2D_min

Proposal 5.1-2: When a R2D transmission in response to a D2R transmission is expected for a A-IoT device, study a design with a maximum time TD2R_max between the D2R transmission and the expected R2D transmission following it, so that the device expects to receive the R2D transmission within [TD2R_min, TD2R_max].

- FFS device behavior if the device does not receive the expected R2D response within the window.

- This does not preclude to study using R2D control information indicating the timing of the R2D response

Proposal 5.1-3: Study following options for a device to determine the Msg.2 reception timing in case one A-IoT paging schedules multiple TDMed/FDMed Msg1.

- Opt.1 Common time window based: the starting time for the Msg2 response time window is TD2R_min after the “last” Msg1 time domain resource that is scheduled by the A-IoT paging.

- Opt.2 Independent Time window: the starting time for the Msg2 response time window is determined based on the control information in the A-IoT paging.

- For both options, the length/duration for Msg2 response time window can be pre-defined and/or indicated.

Proposal 5.1-4: One of the use cases for TD2R_D2R_min between two different consecutive D2R transmissions from the same A-IoT device can be similar as the “in-process reply” adopted in UHF, that a A-IoT device may reply firstly to Reader that it needs more time for processing/prepration followed by the corresponding D2R response to the R2D command.

Proposal 5.2-1: Study the reader acquires the end of PDRCH transmission at least based on R2D control information.

- FFS D2R control information for PDRCH transmission with fixed TBS

Proposal 5.2-2: The end of PRDCH transmission that is determined or derived by the R2D control information should be studied as baseline.

- FFS the R2D control information is/are the physical layer and/or higher layer control information

# References

- 3GPP RAN1#116 meeting, Chairman’s note

- 3GPP RAN1#116bis meeting, Chairman’s note

- 3GPP RAN1#117 meeting, Chairman’s note

- RP-241688, Moderator's summary for offline discussion about Ambient IoT, Moderator (RAN1 Vice-Chair)

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA

- IEEE Commun. Lett., Blind channel estimation for ambient backscatter communication systems, vol. 22, no. 6, pp. 1296–1299, Jun. 2018.

- 3GPP TS 38.211, Physical channels and modulation

- R1-2406188, Discussion on Downlink and uplink channel/signal aspects, vivo