3GPP TSG RAN WG1 #118	                                                                            R1-2406366

Maastricht, NL, August 19th – 23rd, 2024

Source:	CATT

Title:	On asymmetric DL sTRP/UL mTRP scenarios

Agenda Item:	9.2.4

Document for:	Discussion and Decision

- Introduction

In RAN #102 meeting, specifying enhancement for asymmetric DL sTRP/UL mTRP was agreed in the WID for Rel-19 NR MIMO [1]. In RAN1 #117 meeting, the issues related to two closed-loop PC adjustment states for SRS and PL calculation for UL transmissions to UL TRP(s) were discussed and the following agreements have been achieved [1].

Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, further study and down-select one from the Alt1 and Alt3 by RAN1#118 meeting: - Alt1: RRC configures multiple PL offset values [in PRACH-Config] and PDCCH-order DCI indicates one of them through one DCI field. - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission. - FFS: the details of DCI field design. Agreement In Rel-19, the value range of starting bit of block in DCI format 2-3 is extended from 1~31 to 1~X. - FFS the value of X>31. FFS: Condition under which the above is applicable. Agreement Introduce a new RRC parameter per BWP/CC to indicate that two separate SRS CLPC adjustment states are configured for SRS in a BWP/CC. Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). Conclusion There is no consensus on the following proposal: Proposal 1.6: Support to update a UL PL for a joint/UL TCI state as follows: - When this joint/UL TCI state is activated and it is not in the current active TCI state list, a UL PL is calculated as: UL PL = PL estimated from DL PL RS – the value of PL offset. - When this joint/UL TCI state is activated and it is in the current active TCI state list, the UE updates the UL PL as: new UL PL = current UL PL + the updated delta indicated by the NW. Agreement For the asymmetric DL sTRP/UL mTRP scenarios, study and decide the value range and candidate values of PL offset value. Agreement For the asymmetric DL sTRP/UL mTRP scenarios, study whether/how to consider PL offset in PHR calculation, including Type 1 PHR based on actual PUSCH transmission, Type 1 PHR based on reference PUSCH, Type 3 PHR based on actual SRS and Type 3 PHR based on reference SRS. Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

In this contribution, issues on PL determination for UL transmissions to UL TRP(s) and two closed-loop PC adjustment states for SRS in asymmetric DL sTRP/UL mTRP deployment scenarios are discussed.

- Discussion

  - PL determination for UL transmissions

PL offset indication for PDCCH-order PRACH

In RAN1 #117 meeting, it was agreed that for indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, further study and down-select one from the Alt1 and Alt3 by RAN1 #118 meeting:

- Alt1: RRC configures multiple PL offset values [in PRACH-Config] and PDCCH-order DCI indicates one of them through one DCI field.

- Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission.

For Alt3, the PL offset for PDCCH-order PRACH transmission is associated with one of the indicated joint/UL TCI state for UL TRP. That means the PL offset for PDCCH-order PRACH transmission is always tied with one of the TRPs currently used for PUSCH transmission. Since the PDCCH-order PRACH can be a PRACH transmitted to a TRP same or different to the TRPs currently used for PUSCH transmission, such restriction is not preferred. Compared to Alt3, Alt1 is a better solution as the PL offset for the PRACH is not always tied to the TRPs currently used for PUSCH transmission, and the PL offset for the PRACH can be indicated dynamically when the PRACH is triggered.

Proposal 1: For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, Alt1 (RRC configures multiple PL offset values [in PRACH-Config] and PDCCH-order DCI indicates one of them through one DCI field) is supported.

PL offset acquisition by gNB

For the asymmetric DL sTRP/UL mTRP deployment scenarios, the gNB has to determine the PL offset between the DL TRP and UL TRP. One possible solution is receiving the same SRS by the two TRPs, and deriving the PL offset based on the RSRP difference measured by the two TRPs. This might work in FR1. However, in FR2, directional antennas are used, thus, it is possible that the same SRS cannot be received by both of the two TRPs. Then, mechanisms on determining the PL offset in FR2 have to be considered. One potential solution is defining UL PL RS for UL PL determination, e.g., similar as that for DL PL RS, the UL PL RS is transmitted with a reference signal power, and the PL of the UL PL RS is determined based on the RSRP of the signal and the reference signal power by the gNB. Based on the derived UL PL for the two TRPs, the PL offset of the two TRPs can be obtained.

Proposal 2: For the asymmetric DL sTRP/UL mTRP deployment scenarios, the following mechanism is considered at least for FR2:

- Defining UL PL RS, e.g., SRS. The UL PL RS is transmitted with a reference signal power configured by gNB, and the PL of the UL PL RS is determined based on the RSRP of the signal and the reference signal power by the gNB.

PL offset for SRS

In previous meetings, it has been agreed that a joint/UL TCI state can be associated with a PL offset, and when a joint/UL TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint/UL TCI state. In Rel-17 unified TCI framework and Rel-18 unified TCI framework, if a joint/UL TCI state is configured for an SRS resource/SRS resource set, the UE has to determine the UL Tx spatial domain filter for the SRS transmission corresponding to the SRS resource/SRS resource set based on the RS in the joint/UL TCI state, if UL Tx spatial domain filter is applied. For the flexibility of BM, UE determines the UL Tx spatial domain filter for some SRS resource sets totally up to its implementation, i.e., some SRS resource sets are not associated with any TCI state (i.e., the SRS resource set is not configured to follow unified TCI and no SRS resource in the SRS resource set is configured with a TCI state). It is our interpretation that in Rel-17/Rel-18, if the SRS resource set is not associated with any TCI state, the UE determines the PL-RS, P0 and alpha for the SRS resource set based on the power control parameters configured for the SRS resource set. To determining the PL offset for an SRS resource set not associated with any TCI state, similar scheme can be considered, i.e., a PL offset is configured for the SRS resource set. As another alternative, to have a unified solution on power control parameters determination for SRS resource sets determine UL Tx spatial domain filter  based on the RS in a associated TCI state and SRS resource sets do not determine UL Tx spatial domain filter based on a indicated RS, the following scheme can be considered:

- The SRS resource with the lowest ID in the SRS resource set is always configured with a TCI state, and an indication is introduced for SRS resource set/SRS resource to indicate whether the UL Tx spatial relation filter of the SRS resource(s) in the SRS resource set is determined based on the TCI state configured for the SRS resource(s). The UE determines the power control parameters (i.e., PL-RS, PL offset, p0, alpha, SRS CLPC adjustment state) for the SRS transmission corresponding to the SRS resource set based on the TCI state associated with the SRS resource with the lowest ID.

Proposal 3: For the asymmetric DL sTRP/UL mTRP deployment scenarios, support one of the following alternatives for association of PL offset and SRS:

- Alt1: The SRS resource set not associated with any TCI state is associated with a PL offset configured for the SRS resource set.

- Alt2: The SRS resource with the lowest ID in the SRS resource set is always configured with a TCI state, and an indication is introduced for SRS resource set/SRS resource to indicate whether the UL Tx spatial relation filter of the SRS resource(s) in the SRS resource set is determined based on the TCI state configured for the SRS resource(s). The UE determines the power control parameters (i.e., PL-RS, PL offset, p0, alpha, SRS CLPC adjustment state) for the SRS transmission corresponding to the SRS resource set based on the TCI state associated with the SRS resource with the lowest ID.

Quantization of PL offset

For the configuration and the update of PL offset, one open issue is how to quantize the PL offset. In previous releases, DL PL is calculated based on the formula as: PLb,f,c(qd) = referenceSignalPower – higher layer filtered RSRP. As referenceSignalPower is a higher layer parameter configured by gNB, the value range of DL PL is mainly determined by the value range of higher layer filtered RSRP. As L3-RSRP is a higher layer filtered RSRP, the value range can be considered as a reference for determining the value range of PL offset. Note that the value range of L3-RSRP is determined based on the distances between UE and gNBs/TRPs of serving cell as well as other cells for mobility, the maximum value of PL offset between the UL TRP and DL TRP in the asymmetric DL sTRP/UL mTRP deployment scenarios can be smaller than the difference between the maximum value and the minimum value in the range for L3-RSRP. In TS 38.331, the value range for L3-RSRP is [-156dBm, -31dBm] with 1dB granularity. Then the difference between the maximum value and the minimum value of the range of L3-RSRP is 125dB. In the asymmetric DL sTRP/UL mTRP deployment scenarios, a UL TRP can be either closer to the UE or not compared to the DL TRP where the PL RS comes from, therefore, the range of PL offset for the asymmetric DL sTRP/UL mTRP deployment scenarios should not beyond [-125dB, 125dB]. For the quantization of PL offset, similar as that for L3-RSRP quantization, the quantization granularity of 1dB can be considered. Also, as close loop power control can be applied in asymmetric DL sTRP/UL mTRP deployment scenarios, a quantization granularity of 2dB can be also acceptable.

Proposal 4: For the asymmetric DL sTRP/UL mTRP deployment scenarios, to determine the quantization range of PL offset, the UL TRP can be assumed to be closer to the UE or not compared to the DL TRP where the PL RS comes from.

Proposal 5: For PL offset indication in asymmetric DL sTRP/UL mTRP deployment scenarios, the value range of PL offset should not beyond [-125dB, 125dB].

Proposal 6: For the quantization of PL offset in asymmetric DL sTRP/UL mTRP deployment scenarios, a granularity of 1dB or 2dB is considered.

PHR calculation with PL offset

Power Headroom Report is the process by which the UE reports the difference between its current transmission power and its maximum transmit power to the NW side via MAC CE. This difference reflects the remaining power space of UE after completing the current transmission, which is an important basis for power control and resource scheduling in the network.

In RAN1 #117 meeting, for the asymmetric DL sTRP/UL mTRP scenarios, whether and how to consider PL offset in PHR calculation were identified to be further studied, including Type 1 PHR based on actual PUSCH transmission, Type 1 PHR based on reference PUSCH transmission, Type 3 PHR based on actual SRS transmission and Type 3 PHR based on reference SRS transmission.

For PH reporting in NR systems, if the UE is configured with PUSCH transmission, Type 1 PH is reported, otherwise, Type 3 PH is reported. That is, Type 3 PHR is only used in scenarios where PUSCH is not configured. For asymmetric DL sTRP/UL mTRP scenarios, PUSCH transmission would always be configured. Therefore enhancement on Type 1 PHR calculation based on PL offset is needed and enhancement on Type 3 PHR calculation based on PL offset is not needed.

Proposal 7: For PHR in asymmetric DL sTRP/UL mTRP scenarios,

- PL offset based Type 1 PHR calculation based on actual PUSCH transmission and PL offset based Type 1 PHR calculation based on reference PUSCH transmission are supported, and

- PL offset based Type 3 PHR calculation based on actual SRS transmission and PL offset based Type 3 PHR calculation based on reference SRS transmission are not supported.

Regarding how to consider PL offset in Type 1 PHR calculation for the asymmetric DL sTRP/UL mTRP scenarios, the PL offset can be reflected either in the formula for PHR calculation or in the description on how to determine PLb,f,cqd in the formula. It is our view that the latter one is simpler.

Proposal 8: To reflecting PL offset in Type 1 PHR calculation, change the descriptions on how to determine PLb,f,cqd.

  - Two SRS CLPC adjustment states

SRS CLPC adjustment state indication through DCI format 0_1/0_2

As DCI format 2_3 has been supported for indicating TPC command for two SRS CLPC adjustment states, enhancing DCI format 0_1/0_2 for indicating TPC command for two SRS CLPC adjustment states is not essential.

Proposal 9: Do not support indicating TPC command for two SRS CLPC adjustment states by DCI format 0_1/0_2.

Modification of IE SRS-CarrierSwitching for DCI format 2_3

Since Rel-15, DCI format 2_3 is applicable for an UL carrier without PUCCH/PUSCH or an UL carrier on which the SRS power control is not tied with PUSCH power control, and srs-TPC-PDCCH-Group in IE SRS-CarrierSwitching is used for configuring whether DCI format 2_3 is typeA or type B.

However, in TS 38.331, the IE SRS-CarrierSwitching is defined to be used for configuring for SRS carrier switching when PUSCH is not configured and independent SRS power control from that of PUSCH, and it is clarified that “Type A trigger configuration for SRS transmission on a PUSCH-less SCell” and “Type B trigger configuration for SRS transmission on a PUSCH-less Scell”. DCI format 2_3 for an UL carrier on which PUSCH is configured and the SRS power control is not tied with PUSCH power control cannot works. Then for the asymmetric DL sTRP/UL mTRP deployment scenarios, the CLPC adjustment states cannot be indicated by DCI format 2_3.

To ensure DCI format 2_3 be applicable for UL carrier(s) where PUSCH is configured and the SRS power control is not tied with PUSCH power control, modifications on descriptions for IE SRS-CarrierSwitching and typeA and typeB in the IE srs-TPC-PDCCH-Group in TS 38.331 are needed. We propose to send a LS to RAN2 to ask for the modification and an accompany contribution is provided [3].

Proposal 10: To allow DCI format 2_3 to be used for the transmission of TPC commands for SRS transmissions in the UL carrier(s) in a serving cell where PUSCH is configured and the SRS power control is not tied with PUSCH power control, send a LS to RAN2 to ask for changing the descriptions for IE SRS-CarrierSwitching and typeA/typeB in the IE  in TS 38.331.

SRS CLPC adjustment state indication

In Rel-19 asymmetric DL sTRP/UL mTRP deployment scenarios, it has been agreed that for a UE configured with two SRS CLPC adjustment states separate to PUSCH, closedLoopIndex-r17 for SRS in the TCI state is used to indicate one of the SRS CLPC adjustment states. Similar as association of PL offset and SRS, for SRS CLPC adjustment state indication, how to indicate SRS CLPC adjustment state for an SRS resource set not indicated with RS(s) for determining UL Tx spatial domain filter should be considered. Similar as that for association of PL offset and an SRS resource set does not determine UL Tx spatial domain filter based on a indicated RS, one of the following alternatives can be considered:

- Alt1: An SRS CLPC adjustment state is indicated for the SRS resource set.

- Alt2: The SRS resource with the lowest ID in the SRS resource set is always configured with a TCI state, and an indication is introduced for SRS resource set/SRS resource to indicate whether the UL Tx spatial relation filter of the SRS resource(s) in the SRS resource set is determined based on the TCI state configured for the SRS resource(s). The UE determines the power control parameters (i.e., PL-RS, PL offset, p0, alpha, SRS CLPC adjustment state) for the SRS transmission corresponding to the SRS resource set based on the TCI state associated with the SRS resource with the lowest ID.

Proposal 11: When UE is configured with two SRS CLPC adjustment states separate to PUSCH, support one of the following alternatives for SRS CLPC adjustment state indication:

- Alt1: An SRS CLPC adjustment state is indicated for the SRS resource set not associated with any TCI state;

- Alt2: The SRS resource with the lowest ID in the SRS resource set is always configured with a TCI state, and an indication is introduced for SRS resource set/SRS resource to indicate whether the UL Tx spatial relation filter of the SRS resource(s) in the SRS resource set is determined based on the TCI state configured for the SRS resource(s). The UE determines the power control parameters (i.e., PL-RS, PL offset, p0, alpha, SRS CLPC adjustment state) for the SRS transmission corresponding to the SRS resource set based on the TCI state associated with the SRS resource with the lowest ID.

Starting bit of a block in DCI format 2_3

DCI format 2_3 is a group-common DCI monitored by multiple UEs. Each UE is configured with a starting bit position to commence monitoring based on its allocated bit width. The starting bit position of DCI format 2_3 is defined by higher layer parameters with a value range from 1 to 31. According to TS 38.212, the number of information bits for DCI format 2_3 should be equal to or less than the payload size of DCI format 1_0 monitored in common search space in the same serving cell.

DCI format 1_0 has a maximum size of 46 bits in FR1 or FR2-2 bands for shared spectrum and 44 bits in other spectrum. To align with the size of DCI format 1_0, the maximum size of DCI format 2_3 should also be limited to a maximum of 46 bits in FR1 or FR2-2 bands and 44 bits in other spectrum, respectively.

For the asymmetric DL sTRP/UL mTRP deployment scenarios, the size of each block would be 3 bits (2 bits for the TPC command field + 1 bit for the closed-loop indicator). So in FR1 or FR2-2 bands, the maximum value of the starting bit of the block in DCI format 2_3 can be 44, and in other spectrum, the maximum value of the starting bit of the block in DCI format 2_3 can be 42. In summary, the extension of the value range can be 1~44 bits in FR1 or FR2-2 bands and 1~42 bits in other spectrum.

Proposal 12: In Rel-19, the value range of starting bit of block in DCI format 2-3 is extended from 1~31 to 1~44 for FR1 or FR2-2 bands and 1~42 for other spectrum.

Flexible aperiodic SRS triggering for DCI format 2_3

In Rel-17, when availableSlotOffsetList is not configured, or up to one value is configured in availableSlotOffsetList, DCI format 2_3 can be used for SRS triggering. If multiple values are configured in availableSlotOffsetList, DCI format 2_3 cannot be used for SRS triggering. If the SRS request field is not configured, even when multiple values are configured in availableSlotOffsetList, DCI format 2_3 can be used for indicating TPC commands for SRS.

As whether the SRS request field is included in DCI format 2_3 can be configured by gNB, no matter whether and how availableSlotOffsetList is configured, up to gNB implementation, DCI format 2_3 can be used to indicate closed-loop PC adjustment state for SRS. Therefore it is our view that no enhancement for applying available slot for DCI format 2_3 is needed for asymmetric DL sTRP/UL mTRP deployment scenarios.

Proposal 13: Enhancement for applying available slot for DCI format 2_3 is not supported in Rel-19.

- Conclusions

In this contribution, we provided our views on two closed-loop PC adjustment states for SRS and PL calculation for UL transmissions to UL TRP(s) in asymmetric DL sTRP/UL mTRP deployment scenarios, with the following proposals provided:

Proposal 1: For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, Alt1 (RRC configures multiple PL offset values [in PRACH-Config] and PDCCH-order DCI indicates one of them through one DCI field) is supported.

Proposal 2: For the asymmetric DL sTRP/UL mTRP deployment scenarios, the following mechanism is considered at least for FR2:

- Defining UL PL RS, e.g., SRS. The UL PL RS is transmitted with a reference signal power configured by gNB, and the PL of the UL PL RS is determined based on the RSRP of the signal and the reference signal power by the gNB.

Proposal 3: For the asymmetric DL sTRP/UL mTRP deployment scenarios, support one of the following alternatives for association of PL offset and SRS:

- Alt1: The SRS resource set not associated with any TCI state is associated with a PL offset configured for the SRS resource set.

- Alt2: The SRS resource with the lowest ID in the SRS resource set is always configured with a TCI state, and an indication is introduced for SRS resource set/SRS resource to indicate whether the UL Tx spatial relation filter of the SRS resource(s) in the SRS resource set is determined based on the TCI state configured for the SRS resource(s). The UE determines the power control parameters (i.e., PL-RS, PL offset, p0, alpha, SRS CLPC adjustment state) for the SRS transmission corresponding to the SRS resource set based on the TCI state associated with the SRS resource with the lowest ID.

Proposal 4: For the asymmetric DL sTRP/UL mTRP deployment scenarios, to determine the quantization range of PL offset, the UL TRP can be assumed to be closer to the UE or not compared to the DL TRP where the PL RS comes from.

Proposal 5: For PL offset indication in asymmetric DL sTRP/UL mTRP deployment scenarios, the value range of PL offset should not beyond [-125dB, 125dB].

Proposal 6: For the quantization of PL offset in asymmetric DL sTRP/UL mTRP deployment scenarios, a granularity of 1dB or 2dB is considered.

Proposal 7: For PHR in asymmetric DL sTRP/UL mTRP scenarios,

- PL offset based Type 1 PHR calculation based on actual PUSCH transmission and PL offset based Type 1 PHR calculation based on reference PUSCH transmission are supported, and

- PL offset based Type 3 PHR calculation based on actual SRS transmission and PL offset based Type 3 PHR calculation based on reference SRS transmission are not supported.

Proposal 8: To reflecting PL offset in Type 1 PHR calculation, change the descriptions on how to determine PLb,f,cqd.

Proposal 9: Do not support indicating TPC command for two SRS CLPC adjustment states by DCI format 0_1/0_2.Proposal 9: Do not support indicating TPC command for two SRS CLPC adjustment states by DCI format 0_1/0_2.

Proposal 10: To allow DCI format 2_3 to be used for the transmission of TPC commands for SRS transmissions in the UL carrier(s) in a serving cell where PUSCH is configured and the SRS power control is not tied with PUSCH power control, send a LS to RAN2 to ask for changing the descriptions for IE SRS-CarrierSwitching and typeA/typeB in the IE  in TS 38.331.

Proposal 11: When UE is configured with two SRS CLPC adjustment states separate to PUSCH, support one of the following alternatives for SRS CLPC adjustment state indication:

- Alt1: An SRS CLPC adjustment state is indicated for the SRS resource set not associated with any TCI state;

- Alt2: The SRS resource with the lowest ID in the SRS resource set is always configured with a TCI state, and an indication is introduced for SRS resource set/SRS resource to indicate whether the UL Tx spatial relation filter of the SRS resource(s) in the SRS resource set is determined based on the TCI state configured for the SRS resource(s). The UE determines the power control parameters (i.e., PL-RS, PL offset, p0, alpha, SRS CLPC adjustment state) for the SRS transmission corresponding to the SRS resource set based on the TCI state associated with the SRS resource with the lowest ID.

Proposal 12: In Rel-19, the value range of starting bit of block in DCI format 2-3 is extended from 1~31 to 1~44 for FR1 or FR2-2 bands and 1~42 for other spectrum.

Proposal 13: Enhancement for applying available slot for DCI format 2_3 is not supported in Rel-19.

- References

- RP-234007, New WID: NR MIMO Phase 5, Samsung (Moderator), RAN#102, December 11th – 15th, 2023.

- 3GPP TSG RAN WG1 Meeting RAN1 #117 Chairman's Notes, Fukuoka, Japan, May 20th – 24th, 2024, 2024.

- R1-2406331, Discussion on higher layer parameters for DCI format 2_3, CATT, RAN#118, August 19th – 23rd, 2024