3GPP TSG RAN WG1 #118		R1-2406382

Maastricht, NL, August 19th – 23th, 2024

Source:	CATT, CICTCI

Title:	Discussion on ISAC deployment scenarios

Agenda Item:	9.7.1

Document for:	Discussion and Decision

Introduction

Integrated sensing and communication (ISAC) is widely known as a key capability for 5.5G and 6G communication system to facilitate the commercial use in vertical industries. Recently, a list of promising ISAC use cases was provided by SA1 [1]. Therefore, it is important to establish a solid channel modelling framework to enable evaluation of sensing techniques. In [2], a Rel-19 study item on channel modelling for ISAC was approved, focusing on object detection and/or tracking. The following sensing objects are selected as the example target use cases:

- UAVs

- Humans indoors and outdoors

- Automotive vehicles (at least outdoors)

- Automated guided vehicles (e.g. in indoor factories)

- Objects creating hazards on roads/railways, with a minimum size dependent on frequency

A total number of six sensing modes (TRP-to-TRP bi-static, TRP monostatic, TRP-to-UE bi-static, UE-to-TRP bi-static, UE-to-UE bi-static, UE monostatic) are to be studied. The primary frequency range is 0.5 to 52.6 GHz, with the assumption that the modelling approach should scale to 100 GHz if feasible.

In this contribution, we discuss the details of ISAC deployment scenarios corresponding to the above use cases based on the progress until RAN1#117.

Discussion

General consideration

In RAN1#117, the following agreements were reached in the discussion of deployment scenario [11]:

Agreement For each of the sensing target deployment scenarios using the template agreed in RAN#116-bis, the following principles apply: 1. For defining sensing Tx and sensing Rx properties (e.g., cell layout, BS antenna height, and minimum distance), scenario parameter values for the applicable communication scenarios in 38.901 are considered as a starting point. Updates to these evaluation parameter values for ISAC scenarios will consider the following: - aerial UEs parameter values as defined in TR 36.777 - indoor room scenario parameter values defined in TR 38.808 - automotive scenario parameter values as defined in TR 37.885, 38.859 for Urban grid/Highway - Minimum distances between Tx/Rx and target are not defined in the existing communications scenarios and shall be included in the sensing target deployment scenarios. - Note: Only deviation from the existing evaluation parameters in the applicable communication scenarios need to be explicitly defined in the ISAC scenario tables. 2. For defining sensing target properties, as a baseline - Evaluation parameter values can be taken from additional TRs, e.g., TR 36.777, 37.885, 38.859, etc. - Size of sensing targets based on TR 22.837, 37.885 (e.g. for automotive), 38.901 (e.g. for AGV size), etc Agreement For ISAC deployment scenarios, carrier frequency, bandwidth, and SCS are not included in the evaluation parameters templates for sensing scenarios, but may be included in the evaluation/calibration phase.

The above agreements provide clear guidance for future discussion. In addition, several other agreements were made for UAV sensing scenario. Due to the dividing opinions from companies, the progress was small and tough. Thus the evaluation parameters for UAV sensing scenario are incomplete. Note that UAV is assumed to be the simplest scenario. Finishing the evaluation parameters for UAV sensing scenario can provide a good model for other scenarios. The successful experience from UAV can speed up the progress for other sensing scenarios.

Proposal 1: Take the table of evaluation parameters for UAV sensing scenario as the starting point for other scenarios, and modify parameters accordingly when necessary.

In principle, we should keep the evaluation parameters reasonable, achieving a good balance between complexity and accuracy. Simplification is necessary for the sake of efficient simulation, while the effectiveness can still be hold. For example, in evaluations for communication, it is widely assumed that:

- The UE speed is unchanged during simulation.

- The typical velocity of UE is not equal to 0.

- Movement in vertical plane is not considered for UAV UE.

- When random distribution is assumed for UE location, it is assumed as uniformed distribution.

The same principles shall be applied to sensing targets in ISAC.

Proposal 2: For ISAC channel modeling, only allow reasonable complexity increment compared to communication channel. The following assumption should be avoided, unless clear benefit and necessity are identified:

- Time varying target speed;

- Target horizontal speed of 0 km/h;

- Movement in vertical plane (for UAV target);

- Non-uniform distribution.

Several TRs inside 3GPP are consider as reference to define target properties, e.g., RAN1 TR such as TR 36.777, 37.885, 38.859, SA1 TR such as TR 22.837. Additionally, some other materials from organizations outside 3GPP are received, e.g. LS from 5GAA WG4 [12]. It is good to have ample input for channel modeling study, but the situation become a little complicated that the parameters provided in different materials may be different. For example, the sizes of vehicle in TR 37.885 and TR 22.837 are different, while the sizes in TR 22.837 and LS from 5GAA WG4 are the same. We think a clear principle would be helpful for future progress. For instance, when different characteristics for the same sensing target appear in different TRs or other materials, RAN1 may prioritize those already in RAN1 TR/spec.

Proposal 3: A clear principle would be helpful for future progress when different characteristics for the same sensing target appear in different TRs or other materials.

- E.g., RAN1 may always prioritize those already in RAN1 TR/spec.

In RAN1#116, quite a lot of scenarios are agreed for the concerned sensing targets. Note that the number of scenarios is not only about potential number of evaluation cases, but also implies effort on channel modeling details. For wireless channel modeling, many characteristics are scenario specific, e.g., LOS probability, pathloss formula (see Table 7.4.1-1 and Table 7.4.2-1 in TR 38.901 [3]). It is realistic to reuse scenarios defined in TR 38.901 as much as possible, as very limited TU is allocated for ISAC channel modeling in Rel-19.

Understand that it is difficult to drop any scenarios in the table from RAN1#116 agreement for now. But the table is just a starting point. It does not mean all scenarios mentioned in the table will be captured in TR 38.901 at the end. Whether the ISAC channel model of a specific scenario will be captured in TR 38.901 depends on the matureness of the study. Our views on prioritization is shared in Section 2.3.

Proposal 4: Not all the scenarios in the table agreed in RAN1#116 have to be captured in TR 38.901. Whether the ISAC channel model of a specific scenario will be captured in TR 38.901 depends on the matureness of the study at the end of Rel-19.

Details of deployment scenarios

UAV

The following agreements were reached for UAV scenario in RAN1#117 [11].

Agreement For UAV sensing target scenarios, the following table is used as a starting point for deployment scenario parameters/values. Note: Additional parameters, value/value ranges are not precluded. Table x. Evaluation parameters for UAV sensing scenarios Parameters; Value. Applicable communication scenarios; UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV. Sensing transmitters and receivers properties; Rx/Tx Locations; Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario Note1: this may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. [In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario.]. Supported sensing modes; [All 6 sensing modes]. Sensing target; Outdoor/indoor; Outdoor. 3D mobility; Horizontal velocity: Up to 160 km/h [FFS specific velocity(ies) or random distribution] [FFS vertical plane velocity]. 3D distribution; [Uniform between a minimum and maximum height] [Uniform in horizontal domain at a given height]. Orientation; Random in horizontal domain. Physical characteristics (e.g., size); UAV object type(s) [FFS]. [Sensing area]. Minimum 3D distances between pairs of Tx/Rx and sensing target/[unintended objects]; FFS. Minimum 3D distance between sensing targets; FFS. [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.]; FFS. Agreement RAN1 agrees to the following revised evaluation parameters values for the UAV sensing target scenarios: Parameters; Value. Sensing transmitters and receivers properties; Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario Note 1: Other Rx/Tx properties (e.g. mobility) can also be taken from the corresponding communication scenario. Note 2: This may include aerial UEs as Rx/Tx that can be selected among locations in the UMi-AV, UMa-AV, RMa-AV communication scenarios..

For sensing UAV, our views are captured in Table 1. For entries that were already agreed in RAN1#117 without any FFS or squared bracket, we do not consider revision.

Table 1 Evaluation parameter for sensing UAV

| Parameters |  |  |  | Value |
| --- | --- | --- | --- | --- |
| Applicable communication scenarios |  |  |  | (already agreed in RAN1#117) |
| Sensing transmitters and receivers properties |  | Rx/Tx Locations |  | (already agreed in RAN1#117) |
| Supported sensing modes |  |  |  | (already agreed in RAN1#117) |
| Sensing target | Outdoor/indoor |  |  | (already agreed in RAN1#117) |
|  | 3D mobility |  |  | Horizontal: 3km/h, 30km/h, 60km/h, 100km/h, 160 km/h Vertical: 0km/h (baseline), FFS non-zero values, e.g. 20km/h, 40km/h. Do not consider time varying 3D mobility |
|  | 3D distribution |  |  | Horizontal: Uniformly distributed Vertical: Uniformly distributed between 1.5m and 300m - Optionally, fixed height value chosen from {50, 100, 200, 300} m. - Other optional fixed height values are not precluded. (The same as UAV UE in TR 36.777) |
|  | Orientation |  |  | (already agreed in RAN1#117) |
|  | Physical characteristics (e.g., size) |  |  | Length x Width x Height = 1.6m x 1.5m x 0.7m (baseline) |
| [Sensing area] |  |  |  | Horizontal: the whole cell layout - Optional: central cell of the cell layout Vertical: from [40m] to 300m |
| Minimum 3D distances between pairs of Tx/Rx and sensing target/[unintended objects] |  |  |  | - 3D: 10m |
| Minimum 3D distance between sensing targets |  |  |  | - 3D: 10m |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] |  |  | Types | [Bird] |
|  |  |  | 3D mobility | FFS |
|  |  |  | 3D distribution | FFS |
|  |  |  | Orientation | FFS |
|  |  |  | Physical characteristics (e.g., size) | FFS |

By largely reusing characteristics of aerial UE in TR 36.777 [4] as the characteristics of UAV to be sensed, most of the contents are aligned with TR 36.777. Additionally, we have the following consideration:

- A set of three different speeds from 3 km/h to 160 km/h are considered in horizontal plain. The speed of 60 km/h can be considered as baseline. The direction of velocity is assumed as uniformly random. This can be revisited if serious problem is found. For simplicity, the speed in vertical plain is assumed to be zero, which is the same as in TR 36.777 [4]. But speed of 0 km/h in horizontal plane should be avoided, as typical velocity of aerial UE is not equal to 0 in evaluations for communication. For a zero speed target, the sensing signal is hardly to be detected in Delay-Doppler domain precisely, since most of the strong interference is ‘static’, e.g. from neighbour TRP. It is possible that a UAV is hovering, but in this case, it should be sensed when it flew into current position. In addition, it is questionable whether zero speed is suitable in the evaluation for tracking.

- A typical UAV size (Length x Width x Height = 1.6m x 1.5m x 0.7m) is considered, the same as in [1]. However, according to our preliminary research, UAV sizes may vary from 0.3m x 0.3m x 0.2m to 3m x 3m x 1m. This will impact the modeling of Radar Cross Section (RCS). Thus other values can be added if needed in the future.

- For sensing area, two options are provided. Particularly, ‘the whole cell layout’ seems straightforward, while ‘central cell of the cell layout’ may provide similar observations with much simpler simulation.

- For minimum 3D distances, we suppose 10m for simplicity. If serious problem is identified, this can be revisited.

- Regarding environment objects (EO), a widely discussed EO in UAV scenario is bird. It may be desired to evaluate whether UAV and bird can be distinguished by sensing. We are open to consider this issue. It seems depending on the progress of RCS modeling in Agenda 9.7.2 channel modeling. Meanwhile, EO is defined as ‘objects with known location’, which seems contradictory with random distributed birds. Another possible way is to consider bird as one type of sensing target, and simulates multi-type target detection (UAV + bird).

Proposal 5: Adopt Table 1 for sensing UAV.

Humans indoors

For sensing humans indoors, our views are captured in Table 2.

Table 2 Evaluation parameter for sensing humans indoors

| Parameters |  |  | Value |
| --- | --- | --- | --- |
| Applicable communication scenarios |  |  | InF, Indoor Office |
| Sensing transmitters and receivers properties |  | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario |
| Supported sensing modes |  |  | All 6 sensing modes |
| Sensing target |  | Outdoor/indoor | Indoor |
|  |  | 3D mobility | Horizontal: speed = 3 km/h, uniformly random direction Vertical: N/A |
|  |  | 3D distribution | Horizontal: uniformly distributed within sensing area Vertical: N/A (on ground) |
|  |  | Orientation | Random in horizontal domain |
|  |  | Physical characteristics (e.g., size) | At most two types of humans (Length x Width x Height): - Adult: 0.5m x 0.5m x 1.75m (baseline) - Children: 0.3m x 0.3m x 1m |
| [Sensing area] |  |  | Horizontal: the whole cell layout (factory, room area); - Optional#1: convex hull of each horizontal BS deployment - Optional#2: convex hull of central BS Vertical: on the ground |
| Minimum 3D distances between pairs of Tx/Rx and sensing target/[unintended objects] |  |  | Between BS and sensing target (i.e., human): - InF: - 2D: 0m - 3D: the same as cell height - Indoor Office: - 2D: 0m - 3D: 3m Between UE and sensing target (i.e., human): - 2D: 3m - 3D: the same as 2D |
| Minimum 3D distance between sensing targets |  |  | - 2D: 3m - 3D: the same as 2D |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] | Types |  | FFS |
|  | 3D mobility |  | FFS |
|  | 3D distribution |  | FFS |
|  | Orientation |  | FFS |
|  | Physical characteristics (e.g., size) |  | FFS |

By largely reusing the characteristics of terrestrial UE indoor in TR 38.901[3] as the characteristics of human indoor, most of the contents are aligned with TR 38.901. Additionally, we have the following consideration:

- Several scenarios marked in square bracket are not considered in the table, i.e., [Indoor Room (TR 38.808)], [UMi, UMa]. In our understanding, Indoor Room (TR 38.808) is not so popular and hard to guarantee indoor TRP(s) in real deployment. For UMi and UMa, the gNBs/TRPs are deployed outdoor, but the sensing target is human indoor. This implies 100% NLOS link and non-negligible penetration loss between sensing transmitter/receiver and sensing target, when TRP is participating sensing (e.g., UE-to-TRP bi-static sensing mode). It is obvious that UMa and UMi are not promising scenarios for sensing human indoor. We suggest focusing on InF and Indoor Office.

- A set of two human sizes is considered. The typical human size (Length x Width x Height) is 0.5m x 0.5m x 1.75m as mentioned in [1], which is obviously for adult and should be the baseline. In addition, we can consider a smaller size of 0.3m x 0.3m x 1m for children.

- According to TR 38.901, for indoor, the minimum 2D distance between BS/TRP and UE is usually 0m. In other words, they can be fully overlapped in horizontal plane. Based on this logic, the minimum 3D distance between BS/TRP and human is equal to the height of BS/TRP. For minimum 2D distance between UE and human, or different humans, we think 3m in horizontal plane is comfortable and suitable. Since they are all on the ground, the 3D distance shall be the same as 2D distance. Note that unlike indoor office, the ceiling height of InF can be varying according to TR 38.901.

- For sensing area, three options are provided. ‘The whole cell layout’ is straightforward and should be the baseline. In Optional#1 and Optional#2, the concept of convex hull is applied, which is wildly used in positioning study, typically in InF scenario. And apparently, Optional#2 is a simplified case of Optional#1.

Proposal 6: Adopt Table 2 for sensing humans indoors.

Humans outdoors

For sensing humans outdoors, our views are captured in Table 3.

Table 3 Evaluation parameter for sensing humans outdoors

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | UMi, UMa, [RMa] |
| Sensing transmitters and receivers properties | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario |
| Supported sensing modes |  | All 6 sensing modes |
| Sensing target | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal: speed = 3 km/h (baseline), 10 km/h, 20km/h, uniformly random direction Vertical: N/A |
|  | 3D distribution | Horizontal plane: uniformly distributed within sensing area Vertical plane: N/A (on ground) |
|  | Orientation | Random in horizontal domain |
|  | Physical characteristics (e.g., size) | At most two types of humans (Length x Width x Height): - Adult: 0.5m x 0.5m x 1.75m (baseline) - Children: 0.3m x 0.3m x 1m |
| [Sensing area] |  | Horizontal: the whole cell layout; - Optional: central cell of the cell layout Vertical: on the ground |
| Minimum 3D distances between pairs of Tx/Rx and sensing target/[unintended objects] |  | Between BS and sensing target (i.e., human): - UMi: - 2D: 10m - 3D: calculated based on minimum 2D distance, if needed - UMa: - 2D: 35m - 3D: calculated based on minimum 2D distance, if needed - [RMa]: - 2D: 35m - 3D: calculated based on minimum 2D distance, if needed Between UE and sensing target (i.e., human): - 2D: 3m - 3D: the same as 2D |
| Minimum 3D distance between sensing targets |  | - 2D: 3m - 3D: the same as 2D |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] | Types | FFS |
|  | 3D mobility | FFS |
|  | 3D distribution | FFS |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | FFS |

By largely reusing the characteristics of terrestrial UE outdoor in TR 38.901[3] as the characteristics of human outdoor, most of the contents are aligned with TR 38.901. Additionally, we have the following consideration:

- RMa is still marked in square bracket. Note that whether [RMa] is pursued or not is still under discussion. The main concern is the significant wireless signal fading due to large ISD in RMa, which is possible when the sensing target could be far away from gNB. We are open to consider since sensing capability may be desired and useful in rural deployment.

- A set of three different speeds from 3 to 20 km/h are considered. Compared to human indoor (3 km/h only), it is realistic to consider a faster speed for human outdoor since he/she may be running. Still, 3 km/h can be considered as baseline since walking is more common.

- A set of two human sizes is considered, as discussed in Section 2.2.2.

- According to TR 38.901, the minimum 2D distance between BS/TRP and UE is varying from 10m to 35m, depending on exact scenarios. This can be directly applied to BS/TRP to human outdoor. Correspondingly, the minimum 3D distance can be calculated based on 2D distance and the height difference between BS/TRP, but seems unnecessary. For minimum 2D distance between UE and human, or different humans, we think 3m in horizontal plane is comfortable and suitable. Since they are all on the ground, the 3D distance shall be the same as 2D distance.

Proposal 7: Adopt Table 3 for sensing humans outdoors.

Automotive vehicles

For sensing automotive vehicles, our views are captured in Table 4.

Table 4 Evaluation parameter for sensing automotive vehicles

| Parameters |  |  | Value |
| --- | --- | --- | --- |
| Applicable communication scenarios |  |  | Highway, Urban grid |
| Sensing transmitters and receivers properties | Rx/Tx Locations |  | Rx/Tx locations are selected among the TRPs, RSUs and UEs locations in the corresponding communication scenario |
| Supported sensing modes |  |  | All 6 sensing modes |
| Sensing target | Outdoor/indoor |  | Outdoor |
|  | 3D mobility |  | Horizontal: - For Highway, reuse the vehicle mobility defined in TR 37.885 for highway scenario - For Urban grid, reuse the vehicle mobility defined in TR 37.885 for Urban grid - FFS UMa, UMi, RMa, e.g., whether to adopt characteristics of vehicle from highway scenario Vertical: N/A |
|  | 3D distribution |  | Horizontal: - For Highway, reuse the vehicle UE dropping options defined in TR 37.885 for highway scenario - For Urban grid, reuse the vehicle UE dropping options defined in TR 37.885 for Urban grid - FFS UMa, UMi, RMa, e.g., whether to adopt characteristics of vehicle from highway scenario Vertical: N/A (on ground) The sensing target is random selected from the dropped vehicle UEs |
|  | Orientation |  | The same as lane direction. |
|  | Physical characteristics (e.g., size) |  | Two types of vehicles as sensing target (Length x Width x Height): - Type 1 (passenger vehicle): 5.0m x 2.0m x 1.6 m - Type 2 (truck/bus): 13m x 2.6m x 3m |
| [Sensing area] |  |  | Horizontal: within the highway/road Vertical: on the ground |
| Minimum 3D distances between pairs of Tx/Rx and sensing target/[unintended objects] |  |  | Between BS and sensing target (i.e., vehicle): - 2D: 35m, as defined in TR 37.885/TR 36.885 - 3D: calculated based on minimum 2D distance, if needed Between pedestrian UE and sensing target (i.e., vehicle): - 2D: 3.5m, assuming sidewalk = 3m and lane width = 4m - 3D: the same as 2D Between RSU and sensing target (i.e., vehicle) - 2D: [0.5m], assuming lane width = 4m, and vehicle width = 3m, and RSU is located along the both sides of the highway as in TR 38.859 - 3D: calculated based on minimum 2D distance, if needed Between vehicle UE and sensing targets (i.e., vehicles): - 2D: reuse the vehicle UE dropping rules defined in TR 37.885, i.e., the distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec} - 3D: the same as 2D |
| Minimum 3D distance between sensing targets |  |  | - 2D: reuse the vehicle UE dropping rules defined in TR 37.885, i.e., the distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec} - 3D: the same as 2D |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] |  | Types | FFS |
|  |  | 3D mobility | FFS |
|  |  | 3D distribution | FFS |
|  |  | Orientation | FFS |
|  |  | Physical characteristics (e.g., size) | FFS |

By largely reusing the characteristics of vehicle UE in TR 37.885 [5] as the characteristics of automotive vehicles to be sensed, most of the contents are aligned with TR 37.885. Additionally, we have the following consideration:

- There is no explicit definition of vehicle UE in UMa, UMi and RMa. However, according to Section A.1 of TR 36.885 [7], the highway is naturally embedded in a typical cellular network just as UMa/UMi/RMa. Figure A.1.3-2 in TR 36.885 is copied below for reference. Note that 1732m is the typical ISD of RMa. It can be scaled down for UMa/UMi if really needed. We think there is no need to consider UMa/UMi/RMa independent from highway for this case.

ISD=1732m

Simulation region=2ISD=3464m

Wrapping around region

Wrapping around region

Figure 1 Example of freeway (highway) embedded in cellular network (Figure A.1.3-2 in TR 36.885)

- If UE-type RSU is considered as sensing transmitter/receiver, it is suggested to refer to TR 38.859 [8] for RSU deployment. According to TR 38.859 [8], UE-type RSUs are uniformly located with 200m spacing on both sides of highway symmetrically, which is different from the layout of UE-type RSU in TR 37.885, for the purpose of better positioning performance. Similar consideration should be applied to ISAC.

- The vehicle sizes in TR 37.885 and TR 22.837 are slightly different. We prefer the vehicle sizes in TR 37.885 considering that we already use RAN1 TR 37.885 as reference in other parameters.

- There are three types of vehicles defined in TR 37.885 (Type1, Type2, and Type3). However, the first two types have identical size, but just different antenna heights. As a sensing target, the sizes of first two types have no difference and should be combined into one type. So we only have two types of vehicle sizes as sensing target, i.e., passenger vehicle or bus/truck.

- In most cases, minimum 2D distance is enough. 3D distance can be calculated according to 2D distance, but seems unnecessary.

Proposal 8: Adopt Table 4 for sensing automotive vehicles.

Automated guided vehicles (AGV)

For sensing AGV, our views are captured in Table 5.

Table 5 Evaluation parameter for sensing AGVs

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | InF (including 5 sub-scenarios as defined in TR 38.901, i.e., InF-SL, InF-DL, InF-SH, InF-DH, InF-HH) |
| Sensing transmitters and receivers properties | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario |
| Supported sensing modes |  | [All 6 sensing modes] |
| Sensing target | Outdoor/indoor | Indoor |
|  | 3D mobility | Horizontal: speed = 5 km/h (baseline), 30 km/h; uniformly random direction Vertical: N/A |
|  | 3D distribution | Horizontal: uniformly distributed within sensing area Vertical: N/A (on ground) |
|  | Orientation | Random in horizontal domain |
|  | Physical characteristics (e.g., size) | Length x Width x Height = 3m x 3m x 1.5m |
| [Sensing area] |  | Horizontal: the whole cell layout (factory); - Optional#1: convex hull of each horizontal BS deployment - Optional#2: convex hull of central BS Vertical: on the ground |
| Minimum 3D distances between pairs of Tx/Rx and sensing target/[unintended objects] |  | Between BS and sensing target (i.e., AGV): - 2D: 0m - 3D: the same as cell height Between UE and sensing targets (i.e., AGVs): - 2D: 3m - 3D: the same as 2D |
| Minimum 3D distance between sensing targets |  | - 2D: 3m - 3D: the same as 2D |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] | Types | FFS |
|  | 3D mobility | FFS |
|  | 3D distribution | FFS |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | FFS |

AGV is typically deployed in an intelligent factory. Based on some research, we have the following consideration:

- Sensing mode: It is unrealistic to ensure that there will always be a UE near the AGV in an intelligent factory. We can keep all 6 sensing modes for now, but TRP-only sensing modes are more promising in this case. Thus we mark [All 6 sensing modes] in square bracket, similar to UAV case.

- Mobility: In TR 38.901, AGV is considered as blockers. The recommended parameters are captured in Table 7.6.4.2-5, as copied below. The suggested speed is ‘up to 30 km/h’. We think 5 km/h can be the baseline, and a maximum speed of 30 km/h can be additionally considered. For the sake of simplicity, we consider the direction as uniformly random direction.

Table 7.6.4.2-5 of TR 38.901

|  | Typical set of blockers | Blocker dimensions | Mobility pattern |
| --- | --- | --- | --- |
| Indoor; Outdoor; InF | Human | Cartesian: w=0.3m; h=1.7m | Stationary or up to 3 km/h |
| Outdoor | Vehicle | Cartesian: w=4.8m; h=1.4m | Stationary or up to 100 km/h |
| InF | AGV | Cartesian: w=3m; h=1.5m | Up to 30 km/h |
| InF | Industrial robot | Cartesian: w=2m; h=0.2m | Up to 3 m/s |

- Distribution: For simplicity, we consider uniformly distribution on the ground.

- Physical characteristics: Table 7.6.4.2-5 of TR 38.901 recommends a size of 3m x 1.5m, as a rectangular screen. To align with other sensing targets, we extend the recommended size into 3D as 3m x 3m x 1.5m.

- Minimum 3D distances: considering the similarity of characteristics between AGV and human indoor, we decide the parameters by referring to human indoor case.

Proposal 9: Adopt Table 5 for sensing AGV.

Objects creating hazards on roads/railways

For sensing objects creating hazards, our views are captured in Table 6.

Table 6 Evaluation parameter for sensing objects creating hazards on roads/railways

| Parameters |  |  | Value |  |
| --- | --- | --- | --- | --- |
| Applicable communication scenarios |  |  | Highway, Urban grid | High speed train (HST) |
| Sensing transmitters and receivers properties | Rx/Tx Locations |  | Rx/Tx locations are selected among the TRPs, RSUs and UEs locations in the corresponding communication scenario | Rx/Tx locations are selected among the BSs locations in the corresponding communication scenario |
| Supported sensing modes |  |  | All 6 sensing modes | TRP-monostatic, TRP-to-TRP bi-static |
| Sensing target | Outdoor/indoor |  | Outdoor |  |
|  | 3D mobility |  | Horizontal: speed = 3 km/h, 10 km/h, 20km/h, uniformly random direction Vertical: N/A |  |
|  | 3D distribution |  | Horizontal plane: uniformly distributed within sensing area Vertical plane: N/A |  |
|  | Orientation |  | Random in horizontal domain |  |
|  | Physical characteristics (e.g., size) |  | Human (pedestrian): 0.5m x 0.5m x 1.75m Animal (sheep, deer): 1.5m x 0.5m x 1 m |  |
| [Sensing area] |  |  | Horizontal: within the highway/road and near the highway/road (within 3m) Vertical: on the ground | Horizontal: within the railway and near the railway (within 3m) Vertical: on the ground |
| Minimum 3D distances between pairs of Tx/Rx and sensing target/[unintended objects] |  |  | Between BS and sensing target (i.e., human, animal): - 2D: 35m, as defined in TR 37.885/TR 36.885 - 3D: calculated based on minimum 2D distance, if needed Between RSU and sensing target (i.e., vehicle) - 2D: [0.5m], assuming lane width = 4m, and vehicle width = 3m, and RSU is located along the both sides of the highway as TR38.859 - 3D: calculated based on minimum 2D distance, if needed Between pedestrian UE and sensing target (i.e., human, animal): - 2D: 3m - 3D: the same as 2D Between vehicle UE and sensing targets (i.e., human, animal): - 2D: 3.5m assuming sidewalk width = 3m, and lane width= 4m, - 3D: the same as 2D | Between BS and sensing target (i.e., human, animal): - 2D: 35m - 3D: calculated based on minimum 2D distance, if needed |
| Minimum 3D distance between sensing targets |  |  | - 2D: 3m - 3D: the same as 2D | - 2D: 3m - 3D: the same as 2D |
| [Unintended/Environment objects] |  | Types | FFS | FFS |
|  |  | 3D mobility | FFS | FFS |
|  |  | 3D distribution | FFS | FFS |
|  |  | Orientation | FFS | FFS |
|  |  | Physical characteristics (e.g., size) | FFS | FFS |

It is a little complicated for this use case, due to the difference between railway and highway. We have the following consideration:

- Sensing transmitter/receiver: For highway or urban grid scenario, BS and/or pedestrian UE and/or vehicle UE and/or RSU may participant sensing. However, for railway scenario, it is unrealistic to ensure that there will always be a UE near the railway. Therefore, we only consider BS/TRP as sensing nodes for railway scenario.

- Sensing mode: For highway or urban grid scenario, similar to automotive vehicle case, we consider all 6 sensing modes. But due to the reason mentioned above, we consider TRP-only sensing modes for railway scenario.

- Sensing target: according to Section 5.2 and Section 5.7 of TR 22.837 [1], the characteristics of traffic participants are defined and captured in Table 5.2.1-1 and Table 5.7.1-1 of TR 22.837, copied as follows.

Table 5.2.1-1 of TR 22.837

|  | Size (Length x Width x Height) | Typical velocity |
| --- | --- | --- |
| Pedestrian (Adult) | 0.5m x 0.5m x 1.75m | 5km/h |
| Animal (Sheep/deer) | 1.5m x 0.5m x 1 m | 5km/h |
| Vehicle | 4m x 1.75m x 1.5m | 60km/h - 120km/h |

Table 5.7.1-1 of TR 22.837

|  | Size (Length x Width x Height) | Velocity |
| --- | --- | --- |
| Intruder | Pedestrian(Adult): 0.5m x 0.5m x 1.75m | 5km/h |
|  | Animal(Sheep/deer): 1.5m x 0.5m x 1 m | 5km/h |
| Trains | 24m x 3.5m x 3 m | 100km/h - 350km/h |

So, for all concerned scenarios, if we only consider objects ‘creating hazard’, i.e., ignore normal objects like vehicles or trains, the sensing targets should be pedestrians (human) and animals.

- Mobility: We think these can be considered in a way similar to humans outdoors.

- Distribution: We think these can be considered in a way similar to humans outdoors.

- Physical characteristics: For each kind of objects, i.e., human and animal, one typical size is defined according to Table 5.2.1-1 and Table 5.7.1-1 of TR 22.837 [1].

Proposal 10: Adopt Table 6 for sensing objects creating hazards on roads/railways.

Consideration on evaluation prioritization

It is true that no explicit prioritization or de-prioritization on scenarios is pursued in RAN1 for ISAC channel modeling. However, when evaluation is simulated, e.g. for calibration, down-selection on scenarios may be needed due to the limited TU. This is due to the fact that many channel modeling aspects are scenario-specific, e.g., LOS/NLOS probability, pathloss formula, cluster distribution, etc.

For evaluation, our views on prioritized scenario for each use case is shown in Table 7. Note that we do not include sensing mode or frequency range in the table, which may be contribution driven.

Table 7 Prioritized scenario for each use case for evaluation.

| Sensing target |  | Prioritized scenario |
| --- | --- | --- |
| UAV |  | UMa-AV |
| Humans | Outdoors | UMa |
|  | Indoors | One of InF/Indoor-Office |
| Automotive vehicles |  | Highway |
| AGV |  | InF |
| Objects creating hazards |  | Highway |

In Table 7, the following aspects have been taken into consideration:

- Strive for using less scenarios to cover all the use cases/sensing targets.

- UMa is considered as the most interested scenario for all outdoor cases. In the future, if we need to evaluate the performance of sensing multiple kinds of targets (e.g., human + vehicle), UMa may be focused.

- InF can be applied to both humans indoors and AGV. However, Indoor-Office is also interested as a typical indoor communication use case. One of InF/Indoor-Office can be considered if necessary.

- Highway can be applied to both automotive vehicles and objects creating hazards. Therefore, all of vehicle, human, and animal may be sensed in this scenario.

Proposal 11: For the purpose of evaluation in the future, if down-selection on deployment scenario is needed, consider the prioritized scenario for each use case/sensing target in Table 7.

Conclusion

In this contribution, we discuss the detailed scenarios of each use case for ISAC. The proposals are summarized as follows:

Proposal 1: Take the table of evaluation parameters for UAV sensing scenario as the starting point for other scenarios, and modify parameters accordingly when necessary.

Proposal 2: For ISAC channel modeling, only allow reasonable complexity increment compared to communication channel. The following assumption should be avoided, unless clear benefit and necessity are identified:

- Time varying target speed;

- Target horizontal speed of 0 km/h;

- Movement in vertical plane (for UAV target);

- Non-uniform distribution.

Proposal 3: A clear principle would be helpful for future progress when different characteristics for the same sensing target appear in different TRs or other materials.

- E.g., RAN1 may always prioritize those already in RAN1 TR/spec.

Proposal 4: Not all the scenarios in the table agreed in RAN1#116 have to be captured in TR 38.901. Whether the ISAC channel model of a specific scenario will be captured in TR 38.901 depends on the matureness of the study at the end of Rel-19.

Proposal 5: Adopt Table 1 for sensing UAV.

Proposal 6: Adopt Table 2 for sensing humans indoors.

Proposal 7: Adopt Table 3 for sensing humans outdoors.

Proposal 8: Adopt Table 4 for sensing automotive vehicles.

Proposal 9: Adopt Table 5 for sensing AGV.

Proposal 10: Adopt Table 6 for sensing objects creating hazards on roads/railways.

Proposal 11: For the purpose of evaluation in the future, if down-selection on deployment scenario is needed, consider the prioritized scenario for each use case/sensing target in Table 7.

References

- TR 22.837, Feasibility Study on Integrated Sensing and Communication, v19.0.0.

- RP-234069, New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, RAN#102, Edinburgh, Scotland, December 11th – 15th, 2023.

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, v17.0.0.

- TR 36.777, Study on Enhanced LTE Support for Aerial Vehicles, v15.0.0.

- TR 37.885, Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR, v15.3.0.

- TR 38.802, Study on New Radio Access Technology Physical Layer Aspects, v14.2.0.

- TR 36.885, Study on LTE-based V2X services, v14.0.0.

- TR 38.859, Study on expanded and improved NR positioning, v18.0.0.

- Chairman’s Notes RAN1#116, Athens, Greece, February 26th – March 1st, 2024.

- Chairman’s Notes RAN1#116bis, Changsha, China, April 15th – 19th, 2024.

- Chairman’s Notes RAN1#117, Fukuoka, Japan, May 20th – 24th, 2024.

- R1-2405964(WG4-243231) , LS on Physical Properties of Sensing Targets in Automotive Scenarios for ISAC, 5GAA Working Group 4, Maastricht, NL, 19th – 23rd August, 2024.