3GPP TSG RAN WG1#118		R1-2406429

Maastricht , Netherlands, August 19th – 23rd, 2024

Source:	Panasonic

Title: 	Discussion on downlink and uplink channels and signals for A-IoT

Agenda Item:		9.4.2.3

Document for:	Discussion

# Introduction

3GPP has studied the ambient IoT (A-IoT) in Rel. 18 in RAN and decided to continue the study in RAN-WGs in Rel. 19 [1, 2]. A-IoT is a new type of device, with low complexity and low-power consumption, to be deployed in 5G NR. In this contribution, we present the physical channel designs and essential signals for enabling A-IoT in 5G networks.

# A-IoT Characteristics

The A-IoT devices have only limited energy capacity and may obtain the required energy from the environment (e.g., light, pressure, vibration) or external carrier wave signal. The current study item considers two network topologies for the deployment of A-IoT devices. In Topology 1, an A-IoT device communicates bidirectionally with a base station, while in Topology 2, the A-IoT device communicates bidirectionally with an intermediate node, such as a UE, that is connected to a base station. For both topologies, there will be no support of RRC states and mobility. The considered traffic types include device terminated (DT) and device originated - device terminated triggered (DO-DTT). Device-originated autonomous (DO-A) is under the discussion. The spectrum deployment could be in-band, in guard-band and standalone band.

# Discussions

In this section, we present our views on the physical channels and relevant signaling designs.

## R2D control information content and corresponding physical channel

The A-IoT device cannot be synchronized with the reader for a long duration. In addition, the A-IoT device does not know when the reader performs the transmissions. The reader may also use different transmission rates (e.g., using different chip durations) for the R2D transmissions. Hence, there is a need for a preamble before the start of a R2D transmission, indicating the start of the transmission and the employed chip duration. In RAN1#117, the following agreements were made:

Agreement For the start-indicator part of the R2D time acquisition signal, study the two options below: - Option 1: ON/OFF pattern i.e. high/low voltage transmission - Option 2: OFF pattern, i.e. low voltage transmission Agreement For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration - FFS: Pattern design to support determination of chip duration

To indicate the start of a transmission and the employed chip duration, a single ON/OFF pattern could be used. The ON/OFF pattern should be known to the device. The chip duration could be determined based on the edge transition from ON to OFF or OFF to ON. The device needs to differentiate the ON/OFF pattern and the control/data information. This can be achieved, for instance, by using a unique ON/OFF pattern different from the line coding (e.g., Manchester coding) employed for control/data information. Moreover, the A-IoT device may require some additional time to tune its radio for receiving the signal, especially, if the device is not capable of harvesting the energy from the signal and receiving the signal simultaneously. For this reason, an OFF duration (i.e., low voltage transmission) could be used before the start of the ON/OFF pattern, as shown in Figure 1. When the device keeps tuning its radio for receiving the signal and there was no harvesting the energy operation, the low voltage transmission would not be required.

Figure 1. Illustration of R2D transmissions.

Proposal 1: The R2D transmission should be followed by an OFF signal and a known ON/OFF pattern when the device previous operation can be harvesting the energy. The ON/OFF pattern should be differentiated from the control/data information, e.g., using a unique sequence.

The reader may need to deliver different control information to the A-IoT devices. The possible control information includes system information, configuration of contention-based channel access, the indication of R2D data transmissions, triggering signal for D2R transmissions, and the D2R channel time boundaries. The control information may target a single or group of devices. In addition, the control signal can be carried alone or along with the data information. To improve communication efficiency, different control information could be carried with a single transmission. This requires having the possibility of sending control information with variable sizes.

Proposal 2: The control information for R2D should be carried alone or along with the data. The control information includes system information, synchronization, configuration of contention-based channel access, the indication of R2D data transmissions, and D2R channel time boundaries.

Proposal 3: The variable control information size should be supported.

Regarding the use of CRC for control and data transmissions, three different approaches could be considered. The CRC could be generated separately or jointly for the control and data information. Alternatively, the control information could be sent without CRC. Each of these approaches has its own pros and cons. For instance, the joint CRC could reduce the overhead, while the message boundary should be indicated separately, e.g., using a dedicated postamble. The separate CRCs could simplify the implementation with the cost of extra overhead. The CRC might be kept if the control information coding is robust. In our view, the content of control information and its size range should be determined first in order to decide on the proper CRC design. Hence, the CRC design should be discussed after the content of control information is determined.

Proposal 4: The CRC design of R2D should be discussed after the content of control information is determined.

The use of midamble was discussed for the purpose of synchronization. However, this requires dedicating a unique codeword for the midamble to be distinguished from the message. This could complicate encoding and decoding the message. In addition, the device could keep synchronization if the line coding, such as Manchester or PIE, is utilized.

Proposal 5: The midamble should not be considered for the R2D transmissions.

The A-IoT device also needs to determine the end of a R2D transmission. This could be achieved in two different ways. One solution is that the control information (decoded independently) indicates the size of the following data part. This requires that the A-IoT device be able to decode the control message and determine the size indication, while receiving the message information. Otherwise, the A-IoT device needs to receive signals for a duration corresponding to the maximum message size. The bitfield size for the size indication depends on the maximum message size and the granularity of message size. The other solution is to use a message end indication, e.g., a postamble. The postamble should be a known sequence appearing after the data information. The benefit of using the postamble is that the A-IoT device does not require to decode the control information before receiving the message part. The demerit can be the data information cannot contain the same sequence with postamble. In order to avoid using the same sequence with postamble, some escape sequence may be required, which is a variable payload size depending on the information sequence. To decide which solution to be used, the overhead of size indication and the postamble should be further studied. We prefer no limitation on the control/data information sequence in order to avoid variable payload size using escape sequence.

Proposal 6: The message size could be determined either using a message size indication in the control information or a postamble. The overhead of message size indication and postamble should be further studied.

Proposal 7: The message size should be deterministic by the upper layer information. Variable message size depending on the information sequence using escape sequence should be avoided.

## D2R control information content and corresponding physical channel

The A-IoT device also needs to deliver different types of control information to the reader. Some of the possible control information includes the contention-based request for channel access, request for data transmission (possibly with the indication of the data size), device type report, and power status report. Such control information could be carried alone or along with the data information. In this regard, a single transmission may contain only control information, only data information, and a combination of control and data information. For a combined control and data transmissions, a joint CRC could be considered in order to reduce the overhead compared with separate CRCs. This however might require using a postamble to indicate the end of the message. In case separate CRCs are utilized of control and data information, the message size could be derived from the control information, so there is no need to utilize the postamble. Similar to R2D, the CRC design should be discussed after the content of control information is determined.

Proposal 8: The control information for D2R should be carried alone or along with the data information. The control information includes contention-based request for channel access, request for data transmission (possibly with the indication of the data size), device type report, and power status report.

Proposal 9: The CRC design of D2R should be discussed after the content of control information is determined.

## Proximity determination

Due to the low-complexity design of A-IoT devices, it was agreed that the proximity determination is performed at the reader side. The following agreement were made in RAN1#118.

Agreement Study the following schemes for proximity determination: - Option 1: If reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near - FFS: Details on reception criteria (e.g. either successful or not) at reader and device - Option 2: Device is determined to be near the reader based on measurements at the reader side - FFS: Details on measurement methods - FFS: Whether/how transmit power of R2D and/or D2R is considered for proximity determination

With Option 1, a reader can send a triggering signal and monitors the response(s) from the A-IoT device(s). The triggering signal could be targeted to a single A-IoT device or a group of A-IoT devices. This procedure is similar or equivalent to contention-free or contention-based access. For triggering a group of devices, there is a chance that the reader cannot decode all the individual responses from the devices due to the collisions. However, even with the response collisions, the reader can determine that there are A-IoT devices in the proximity distance. This determination could be used to provide harvesting signal for the device or sending additional triggering signals for individual device identifications. With Option 2, the reader needs to measure the signal strength of the response messages. The accuracy of signal strength estimation degrades in low SINR regions as the D2R signal is a backscattered transmission. In addition, the backscattered transmission could fluctuate due to multi-path fading. Hence, the accuracy of measurement-based proximity determination is limited.

Proposal 10: A simple signal detection reusing contention-based or contention-free access should be considered for the proximity determination.

# Conclusions

In this contribution, we provided our views on physical channels and relevant signals for A-IoT. We made following proposals:

Proposal 1: The R2D transmission should be followed by an OFF signal and a known ON/OFF pattern when the device previous operation can be harvesting the energy. The ON/OFF pattern should be differentiated from the control/data information, e.g., using a unique sequence.

Proposal 2: The control information for R2D should be carried alone or along with the data. The control information includes system information, synchronization, configuration of contention-based channel access, the indication of R2D data transmissions, and D2R channel time boundaries.

Proposal 3: The variable control information size should be supported.

Proposal 4: The CRC design of R2D should be discussed after the content of control information is determined.

Proposal 5: The midamble should not be considered for the R2D transmissions.

Proposal 6: The message size could be determined either using a message size indication in the control information or a postamble. The overhead of message size indication and postamble should be further studied.

Proposal 7: The message size should be deterministic by the upper layer information. Variable message size depending on the information sequence using escape sequence should be avoided.

Proposal 8: The control information for D2R should be carried alone or along with the data information. The control information includes contention-based request for channel access, request for data transmission (possibly with the indication of the data size), device type report, and power status report.

Proposal 9: The CRC design of D2R should be discussed after the content of control information is determined.

Proposal 10: A simple signal detection reusing contention-based or contention-free access should be considered for the proximity determination.

# Reference

[1] 	TR 38.848, “Study on Ambient IoT (Internet of Things) in RAN,” V18.0.0.

[2] 	RP-233094, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR,” RAN#102.

[3]	R1-2404292, “FL summary#3 on downlink and uplink channel/signal aspects”