3GPP TSG RAN WG1 #118			R1-2406575

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	9.3.2

Source: 	Hyundai Motor Company

Title:	Discussion on SBFD random access operation

Document for:	Discussion and Decision

Introduction

This contribution shows our views about PRACH configuration and power control for SBFD-aware UE.

Discussion

PRACH configuration

For RACH configuration, following is agreed in the previous meeting [1]:

Conclusion For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1: - The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - It is up to the network to configure ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - If legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs

According to previous agreement, SBFD-aware UEs can transmit preamble not only in SBFD ROs but also in legacy ROs.  In terms of fairness, given the same amount of time, SBFD-aware UEs have more chance to transmit preamble than non SBFD-aware UEs due to the presence of UL subband as sown in figure 1. The problem might become bigger when the SBFD-aware UEs retransmit preamble by ramping up the power. If there are no constraints, it can seem to be unfair for non SBFD-aware UEs.

Figure 1: The example of unfairness between SBFD-aware UEs and non-SBFD-aware UEs in legacy RO.

Observation #1:

- There might be fairness issues between SBFD-aware UEs and non SBFD-aware UEs since SBFD-aware UEs have more chance to transmit preamble than non SBFD-aware UE.

So, RAN1 should consider the fairness between SBFD-aware UEs and non SBFD-aware UEs. we think finding alternatives to resolve the problem are necessary and examples of this can be introducing separated configuration of RSRP threshold for SBFD-aware UEs and/or dividing the preambles into subgroups that can be selected by SBFD-aware UEs and non SBFD-aware UEs respectively as shown in figure 2.

Figure 2: The example of division of preambles to resolve the unfairness between SBFD-aware UEs and non SBFD-aware UEs .

Proposal #1:

- For the sake of fairness between SBFD-aware UE and legacy UE, RAN1 should consider following alternatives as PRACH configuration:

  - Introducing separated configuration of RSRP threshold for SBFD-aware UEs

  - Dividing the preambles into subgroups that can be selected by SBFD-aware UEs and non SBFD-aware UEs respectively

PRACH (re)transmission and related power control

Regarding power control, following is captured in the previous meeting [1]:

Agreement For a single TRP scenario, support separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols - Decide in RAN1#118 whether separate UL power control for FR2 is supported based on UL spatial relation Info framework and/or unified TCI state framework.

When SBFD-aware UE can transmit msg1 both ROs in non-SBFD symbols and SBFD symbols and retransmission of msg1 is occurred in non-SBFD symbols as shown in Figure 3, there can be Tx power difference between legacy UE and SBFD-aware UE if SBFD-aware UE applies power ramping for retransmission.

Figure 3: the example of Tx power difference between SBFD-aware UE and legacy UE when SBFD-aware UE retransmit msg1 in non-SBFD symbols .

Regarding the issue, following parameters can be related and RAN1 should discuss how to define/use them to adjust performance between SBFD-aware UE and legacy UE:

  - Maximum number of msg1 transmissions

  - Transmission counter for msg1

  - Power ramping step

  - Ramping counter

Figure 4 shows an example of UE’s behaviour. Depending on how to set/define the above parameters and related detail procedures, the fairness problem can be resolved.

Figure 4: the example of Tx power difference between SBFD-aware UE and legacy UE when SBFD-aware UE retransmit msg1 in non-SBFD symbols .

Proposal #2:

- RAN1 should discuss how to define/use following parameters to adjust fairness between SBFD-aware UE and legacy UE:

  - Maximum number of msg1 transmissions

  - Transmission counter for msg1

  - Power ramping step

  - Ramping counter

Preamble sequence repetition

According to previous meeting [2] as shown in below, a slot can be composed of both SBFD symbols and non-SBFD symbols.

Agreement A slot can consist of SBFD symbols and non-SBFD symbols.

And then maximum 12 sequence repetition (e.g. format B4) can be located within a PRACH slot in accordance with current specification. A figure #2 shows the case that format A3 is configured for SBFD aware UEs. In that case, if non-SBFD symbols are configured within the same slot, UE needs to decide whether to transmit msg1 repeatedly until the end of valid symbol or not. To resolve the problem, two ways can be considered. The one way is that RAN 1 makes UE does not expect to receive the configuration index causing the issue. The other way is RAN1 decides whether UE transmits repeated msg1 within valid time duration or not (Skip msg1). Although UE may not be able to send as many repeated sequence as indication, we believe it is more desirable to expect UE to transmit sequence as many as possible withing the valid time. Furthermore, the remaining time can play a role as guard period. For example, as shown in figure 5, even though UE is indicated to transmit 6 preamble sequence repetition, only 5 repeated sequences are transmitted to consider resource efficiency.

Figure 5: the example of msg1(e.g. format A3) transmission in single a slot.

Observation #2:

- UE may not be able to transmit msg1 by using the configured preamble format exactly since the slot can consist of SBFD symbols and non-SBFD symbols.

Proposal #3:

- RAN1 needs to consider allowing UE to readjust configured the number of repeated preamble sequences and it can transmit as many as possible when configured preamble format may not be used properly due to the limited time duration.

Conclusion

In this contribution, the following conclusions were made:

PRACH configuration

Observation #1:

- There might be fairness issues between SBFD-aware UEs and non SBFD-aware UEs since SBFD-aware UEs have more chance to transmit preamble than non SBFD-aware UE.

Proposal #1:

- For the sake of fairness between SBFD-aware UE and legacy UE, RAN1 should consider following alternatives as PRACH configuration:

  - Introducing separated configuration of RSRP threshold for SBFD-aware UEs

  - Dividing the preambles into subgroups that can be selected by SBFD-aware UEs and non SBFD-aware UEs respectively

PRACH (re)transmission and related power control

Proposal #2:

- RAN1 should discuss how to define/use following parameters to adjust fairness between SBFD-aware UE and legacy UE:

  - Maximum number of msg1 transmissions

  - Transmission counter for msg1

  - Power ramping step

  - Ramping counter

Preamble sequence repetition

Observation #2:

- UE may not be able to transmit msg1 by using the configured preamble format exactly since the slot can consist of SBFD symbols and non-SBFD symbols.

Proposal #3:

- RAN1 needs to consider allowing UE to readjust configured the number of repeated preamble sequences and it can transmit as many as possible when configured preamble format may not be used properly due to the limited time duration.

Reference

[1] Chairman’s Note for RAN1#117

[2] Chairman’s Note for RAN1#116