3GPP TSG RAN WG1 #118	        		             R1-2406642

Maastricht, The Netherlands, August 19th – 23rd, 2024

Agenda item:	9.2.1

Source: 	Samsung

Title: 	UE-initiated (UEI) beam management enhancements

Document for:	Discussion and Decision

# Background

In legacy (up to Rel-18 NR specifications) beam management procedures, the network/gNB would instruct a UE, e.g., by providing a set of RSs and their respective uplink resources, to conduct channel measurement and reporting. The most resource-efficient reporting mechanism for a content or report quantity (e.g., a beam metric) is aperiodic – in conjunction with aperiodic CSI-RSs. The aperiodic measurement/reporting, however, would result in large latency (as shown in a conceptual example provided in Figure 1, an aperiodic triggering offset that typically comprises at least a few slots – in part of the overall measurement/reporting latency – is expected, in addition to the time it takes the network to determine that a new beam measurement report is needed).

Figure 1. An example of AP CSI measurement/reporting procedure

On the other hand, the network can configure periodic CSI measurement and reporting, e.g., with a well-chosen periodicity, to reduce the overall measurement/reporting latency, and therefore, to timely acquire the best beams for data/control communications. The periodic measurement/reporting (followed by semi-persistent measurement/reporting), however, would result in increased RS, control signaling and UL resource overheads. It is evident that the system performance of the aforementioned periodic, semi-persistent or aperiodic measurement and reporting would depend on how much the network/gNB knows the channel(s). In practice, there are several relevant scenarios where the network/gNB lacks full knowledge of the channel conditions – or in other words, for certain settings, the UE would know the channel conditions (e.g., change(s) in beam quality due to UE’s switching of antenna panels) better than the network/gNB. For this case, purely relying on the network to configure, activate or trigger the measurement/reporting process would become less effective and efficient, leading to performance degradation especially when the channel varies at a relatively fast pace (e.g., in a high-speed train (HST) deployment scenario). Hence, there are clear benefits to let the UE initiate its own aperiodic reporting for a given content or report quantity (RQ). For example, when the channel condition is worsened to the point of beam failure, the loss of radio link due to beam failure can be avoided if the UE can transmit an aperiodic beam report without waiting for a beam report request/trigger from the network/gNB. For another example, when the channel quality drops due to UE’s movement, the link quality degradation can also be alleviated or avoided if the UE can initiate an aperiodic CSI report.

Although UE-initiated reporting can be beneficial, efficient designs are needed to ensure that the latency is reduced and, at the same time, error events can be minimized, i.e., to offer a good trade-off between latency and reliability. This is clearly captured and described in the Rel-19 MIMO WID [1], wherein the objectives for the UEI reporting state that UL signaling content(s) and UL signaling medium/container – respectively for 1a and 1b in [1] – should be the design focus.

“

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

”

In this contribution, we provide our views on various design aspects related to the UE-initiated beam management enhancements according to the WID and the agreements made in prior RAN1 meetings including RAN1#117 [2]. We also provide our thoughts on how we could proceed with the corresponding discussions in RAN1.

# Discussions on UE-initiated (UEI) beam reporting

In this section, we focus on various design aspects for the UEI beam reporting including trigger event(s) – along with the measurement RS(s) configuration/determination, UL signaling content(s)/report format(s) and the UCI based UL signaling procedure(s) design, and present our proposals on the related subject matters.

# Event(s) and triggering condition(s)

Three additional events to the already agreed Event-2 were provided in RAN1#117 for potential down-selection or combination in RAN1#118.

Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following Event-1 and 7a/7b, are provided for down-selection or combination in RAN1#118 (possible outcome is that no new event is supported) - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality. - Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

We do not see valid use cases for Event-7a or Event-7b. For Event-7a, even if a new beam that is a threshold value better than the worst activated beam can be identified, there are still a handful number of activated beams (having better beam qualities than the worst activated beam or all of the activated beams except the worst one) that can be used for beam indication; hence, there is no need for the network to update the activated TCI states based on the new beam(s) identified during the evaluations of Event-7a. For Event-7b, the activated beam or TCI state with the best quality in general would correspond to the indicated TCI state, making Event-7b equivalent or close to Event-2. In fact, the gNB can do whatever they see fit based on the reported beam(s) – e.g., triggered by Event-2 – including updating the activated TCI state(s). This is completely subject to gNB’s implementation(s) hence we do not see a need to specify additional event(s) – hence additional signalling procedure(s) – for this purpose.

Proposal 1: On UE-initiated/event-driven beam reporting, do not support Event-7a or Event-7b due to lack of applicable use cases.

Application scenario(s) for Event-1 is clearer – it targets at preventing BFR from happening, which can be beneficial in terms of latency and overhead reductions. If Event-1 is supported, to minimize potential duplicated efforts in specifying the corresponding UE’s behaviors (as for now, almost all the discussions related to the measurement RS(s), reporting content(s)/format(s) etc. are focused on Event-2), it is preferable to reuse what RAN1 has agreed so far for this agenda item, e.g., the same reporting format, the same steps in both Mode-A and Mode-B for the UCI based UL signaling procedures and etc., for Event-1, though the event triggering condition(s), configuration/determination of measurement RS(s) may need to be customized for Event-1.

Proposal 2: On UE-initiated/event-driven beam reporting, support Event-1 and reuse at least the same reporting format and the same steps in both Mode-A and Mode-B for the UCI based UL signaling procedures.

Another open issue for Event-2 evaluation is that whether to capture the corresponding evaluation procedure(s) in PHY layer or MAC layer as stated in the FFS of the below agreement.

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

We prefer to capture the above agreed Event-2 evaluation procedure(s) in RAN1 specification considering the followings:

- Event-2 is a beam-level L1 event; it is natural to specify all the corresponding evaluation procedure(s) in L1 – note that all the essential components of the Event-2 evaluation procedure(s) including the configurable number M, the time window (which is configurable) and etc. can all be captured in RAN1 specifications (there is enough precedence for these), and the agreement does not read that the Event-2 instance has to be indicated from lower layers to higher layers of the UE

- Cross-layer signaling would increase UE’s implementation complexity and latency. This is especially true for evaluating Event-2 because the evaluation of Event-2 involves a beam-specific counting process such that the UE would need to monitor each of the configured candidate beams and identify the same new beam(s) that satisfies the event condition(s) a configurable number of times within a configurable time window. If the number of configured candidate beams is relatively large, e.g., 128, identifying Event-2 instance(s) based on measuring candidate beams at PHY layer while evaluating triggering condition(s) of Event-2 at MAC layer would involve frequent cross-layer signalings, which would be a significant source of latency and overhead.

Proposal 3: Support to capture Event-2 evaluation condition(s) and procedure(s) in RAN1 specification.

# Measurement RS(s) configuration/determination

Two schemes are supported for determining current beam RS(s) for Event-2. In Scheme-1, the current beam RS corresponds to the RS provided in the indicated TCI state, while in Scheme-2, the current beam RS corresponds to the SSB QCL’ed with the RS provided in the indicated TCI state. It was also agreed that enabling either Scheme-1 or Scheme-2 is selected by the network. First of all, we support to confirm the working assumption in the below agreement such that the same RS type between the current and new beams should be ensured.

Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows. - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state. - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. - Enabling one of either Scheme-1 or Scheme-2 is selected by NW. - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state.

Furthermore, we support to introduce an explicit RRC parameter to enable the selection, which is quite straightforward and simple enough for implementation.

Proposal 4: Regarding RS measurement for the current beam for Event-2:

- Support to confirm the working assumption, i.e., enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam(s)

- Support to introduce an explicit RRC parameter to enable the selection of either Scheme-1 or Scheme-2.

Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state. - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). - Option-4: No further enhancement (i.e., in such case, Scheme-2 is used) - Others are not precluded.

There is still an open issue on how to determine the current beam measurement RS when the indicated TCI state provides only one TRS, as in current deployments, TRS cannot be used as a measurement RS for measuring L1-RSRP. Four options are on the table for potential down-selections (the corresponding agreement made in RAN1#117 is provided above), and our preference is to use the SSB that is QCL’ed with the TRS in the indicated TCI state as the corresponding current beam measurement RS, i.e., Option-4 (no further enhancement is needed). Though Option-4 may be subject to certain level of performance loss in terms of beam tracking, it is implementation friendly for both gNB and UE, has less specification impact(s), and is more aligned with the unified TCI framework.

Proposal 5: Regarding RS measurement for the current beam for Event-2, when only one TRS is configured in the indicated TCI state, support Option-4, i.e., in such a case, Scheme-2 is used.

Agreement Regarding explicit RS configuration for new beam measurement for Event 2, down-select the following options in the RAN1#118: - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration; - FFS: The RS in the RS resource set can be updated by MAC-CE. - Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE. - FFS: If a list size is small, MAC-CE activation is not needed - Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state. - Others are not precluded. - FFS: Each RS for new beam measurement should be associated with a configured joint/DL TCI state which can be used as the indicated TCI state

In RAN1#117, explicit configuration of candidate beam measurement RS(s) for Event-2 was agreed. Design details of the explicit configuration still need to be addressed, which are provided in three candidate options for potential down-selection in RAN1#118.

In Option-1, the UE would measure all the configured candidate beam RS(s) in the RS set to evaluate Event-2, which could become quite complicated especially when the number of configured candidate beam RS(s) in the RS set is relatively large (e.g., 128). Different from legacy beam measurement, evaluations and/or triggering of Event-2 additionally involves:

- For each measurement occasion, comparing between each of the candidate beam RSs and the current beam RS with a threshold

- Identifying that at least a same new beam occurs across a number of measurement occasions (within a time window)

To resolve the above issue, the network can configure a relatively small number of candidate beam RSs, but in this case, the UE would not be able to timely track the (candidate) beam changes, which would result in performance degradation. If a small number of candidate beam RSs is configured in the set, RRC reconfiguration or MAC CE update would become unavoidable, which is another source of latency and overhead – note that for small to medium configurations (which is the case here), MAC CE update or MAC CE signaling in general would not have significant advantages (if any) over RRC signaling in terms of latency and/or overhead reduction(s). We have similar concerns for Option-2 with MAC CE based activation of a subset of the configured RSs. Furthermore, if the current beam RS and candidate beam RS(s) are updated separately (e.g., via separate signaling mediums), the UE may need to maintain separate application timelines:

- The current beam RS is updated via TCI state update (by a TCI codepoint of the TCI field in DCI format 1_1/1_2)

- The candidate beam RS(s) is updated or activated by MAC CE

which should be prohibited as this may result in complicated UE’s behaviors in evaluating Event-2, e.g., if the UE would like to reset the event instance(s) counting based on update(s) of the current beam and/or candidate beam RS(s).

For Option-3, each configured TCI-State can be associated with candidate beam RS resources at RRC level (an example of the corresponding IE is provided below) – hence no issues of signaling overhead.

Figure 2. TCI-State to provide candidate beam RS resource index(es)

With Option-3 and along with a TCI state update (hence an update of the current beam RS), the UE can just focus on a smaller set of candidate beam RSs, which would significantly ease the UE’s efforts of measuring and monitoring beam quality(s) of the candidate beam RS(s) for Event-2 evaluation(s). Furthermore, the aforementioned complications of maintaining separate application timelines does not exist for Option-3 as both the current beam RS and candidate beam RS(s) can be provided simultaneously for measurement and Event-2 evaluation as they are in the same higher layer parameter TCI-State. Hence, we propose

Proposal 6: Regarding explicit RS configuration for candidate beam measurement in evaluating Event 2, support Option-3, i.e., to provide candidate beam RS resource index(es) in TCI-State.

# Report content(s)/format(s)

Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - FFS: candidate value of ‘N’. - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams - FFS: Option-1/1a/1b/2. - Above applies at least for the single CC case Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported. - RRC can enable or disable whether current beam is always reported - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported - Note: The reported current beam is NOT counted in the N reported beams. - When disabled by RRC, N beams are reported.

In RAN1#117, a fixed payload size beam report format was agreed for Event-2. In particular, a number of N beam(s) can be reported in a reporting instance, where N is configured by the network, and at least one of the reported N beams should satisfy the Event-2 condition(s). As stated in the above agreement(s), candidate value(s) of N still needs to be decided.

Since this is UE-initiated/event-driven beam reporting, when a UE would trigger a beam reporting, the UE can often times identify L>1 new beams that satisfy the event condition(s). In this case, having the network (without knowing L) to configure N=1, i.e., to have the UE to always report only one out of L identified new beams per RRC configuration, would be quite limiting. Furthermore, when the UE declares Event-2 (meaning the UE has identified at least one new beam), the UE would anyways report the at least one new beam (Option-3), making the network to configure a N=1 somewhat redundant. When N>1 is configured, and the UE identifies L=1 new beam that satisfy the event condition, (N – 1) beams that do not satisfy the event condition can be reported – we do not see any issues for this case but rather network’s scheduling flexibility can be ensured. We therefore propose the following:

Proposal 7: On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 of Event-2, support candidate value(s) of ‘N’ as {2, 3, 4}

- If ‘N’ is not configured, one CRI/SSBRI and the corresponding L1-RSRP are reported

# UCI based UL signaling procedure(s) design

An agreement made in RAN1#116bis related to UCI based UL signaling procedure(s) – both Mode A (dynamically scheduling UCI by gNB) and Mode B (UCI in pre-configured resource(s) for second UL channel) are supported:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details.

A working assumption and an agreement made in RAN1#117 related to signaling design details of Mode-A and Mode-B:

Working Assumption On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH. - The UL-grant DCI format at least comprises DCI format 0_1/0_2. - FFS: DCI format 0_3 - FFS: How to trigger the UEI beam report by the UL-grant DCI format - FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH. - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report. - The DL-grant DCI format at least comprises DCI format 1_1/1_2. - FFS: DCI format 1_3

In RAN1#116bis, an agreement (see above) on UCI based UE-initiated/event-driven beam reporting procedure(s) was made. In RAN1#117, a working assumption and an agreement (also provided above) were achieved to address some signalling design details for both Mode A and Mode B. There are still quite a few open issues for both Mode A and Mode B, and in this contribution, we would like to discuss these open issues:

- Issue #1: Resource allocation for the first PUCCH.

- Issue #2: UCI type for the first PUCCH.

- Issue #3: Acknowledge information for the first PUCCH.

- Issue #4: Resource allocation for the second UL channel for Mode A.

- Issue #5: Resource allocation for the second UL channel for Mode B.

- Issue #6: Association between the first PUCCH and the second UL channel for Mode B.

- Issue #7: Acknowledge information for the second UL channel.

Issue #1: Resource allocation for the first PUCCH.

Considering there is no difference between Mode-A and Mode-B for the resource configuration of the first PUCCH, a unified design is preferred to minimize the specification impact as well as to ease the implementation efforts for both network and UE. To facilitate the discussion, a periodic PUCCH resource for the first PUCCH can be configured by dedicated RRC signalling.

Proposal 8: Confirm the following Working Assumption made in RAN1#117 meeting.

Working Assumption On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case.

Issue #2: UCI type for the first PUCCH.

There are two options for the UCI type for the first PUCCH. Reusing SR or defining a new UCI type.

### 5.4.4 Scheduling Request The Scheduling Request (SR) is used for requesting UL-SCH resources for new transmission. The MAC entity may be configured with zero, one, or more SR configurations. An SR configuration consists of a set of PUCCH resources for SR across different BWPs and cells. For a logical channel or for SCell beam failure recovery (see clause 5.17) and for consistent LBT failure recovery (see clause 5.21), at most one PUCCH resource for SR is configured per BWP. For a logical channel serving a radio bearer configured with SDT, PUCCH resource for SR is not configured for SDT. For beam failure recovery of BFD-RS set(s) of Serving Cell, up to two PUCCH resources for SR is configured per BWP. For positioning measurement gap activation/deactivation request, a dedicated SR configuration is configured. Each SR configuration corresponds to one or more logical channels and/or to SCell beam failure recovery and/or to consistent LBT failure recovery and/or to beam failure recovery of a BFD-RS set and/or to positioning measurement gap activation/deactivation request. Each logical channel, SCell beam failure recovery, beam failure recovery of a BFD-RS set and consistent LBT failure recovery, may be mapped to zero or one SR configuration, which is configured by RRC. The SR configuration of the logical channel that triggered a BSR (clause 5.4.5) or a DSR (clause 5.4.9) or the SCell beam failure recovery or the beam failure recovery of a BFD-RS set or the consistent LBT failure recovery (clause 5.21) (if such a configuration exists) or positioning measurement gap activation/deactivation request (clause 5.25) is considered as corresponding SR configuration for the triggered SR. Any SR configuration may be used for an SR triggered by Pre-emptive BSR (clause 5.4.7) or Timing Advance reporting (clause 5.4.8).

According to the above definition of SR captured in TS 38.321, it is very clear that SR is used for requesting UL-SCH resources for new transmission which is for UL data only. However, for Mode A, the first PUCCH is to request a resource for a second UL channel to carry beam report UCI instead of UL data. For Mode B, the first PUCCH is for notifying a second UL channel to carry beam report UCI. It is clear that the usage of the UCI in the first PUCCH channel for both Mode A and Mode B is not aligned with the existing SR.

Observation 1: The usage of the UCI in the first PUCCH for both Mode A and Mode B is not aligned with the existing SR.

It is not preferred to break the definition/usage of the existing SR, and a new UCI type should be defined for the first PUCCH to sustain a clean design of SR.

Proposal 9: On beam report transmission procedure for UE-initiated/event-driven beam reporting, support a new UCI type for the first PUCCH for both Mode A and Mode B.

Issue #3: Acknowledge information for the first PUCCH.

For Mode A, Step 2 can be considered as an ACK for the first PUCCH, thus an additional explicit ACK is not needed.

For Mode B, whether acknowledge information for the first PUCCH is needed can be separately discussed for light traffic load, medium traffic load and heavy traffic load scenarios.

For light traffic load scenario, a dedicated resource can be configured for each UE for the second UL channel and there is no need for the network to reschedule the resource of the second UL channel and the time gap between the first PUCCH and the second UL channel can be 0 to minimize the latency. In this case, the network feedback for the first PUCCH cannot apply.

For medium traffic load scenario, a dedicated resource can also be configured for each UE for the second UL channel but the network can reschedule the resource of the second UL channel with a proper time gap between the first PUCCH and the second UL channel if the network does not detect the transmission for the first PUCCH. Considering that the resources for the second UL channels are dedicated, there would be no collision for the second UL channel transmissions if the network correctly decodes the first PUCCH which is the typical case assume that the BLER of PUCCH is 1%. If the network misses the first PUCCH transmission and reschedules the resource for the corresponding second UL channel, the collision may happen but is acceptable since the probability of such collision is also about 1%. Basically, there is no difference between (i) the collision occurring to a PUSCH transmission and (ii) not correctly decoding the PUSCH, and the network scheduling a retransmission can resolve the collision issue.

For heavy traffic load scenario, there may not be enough dedicated resources for the second UL channels. If the network configures a shared resource for multiple UEs for the second UL channel and the event condition is satisfied for more than one UE, network response for the first PUCCH can help ensure up to one UE transmits the second UL channel at the cost of additional latency for the other UEs. As the objective of the WID indicates that the latency should be reduced for UEI beam reporting. Even if the collision happens and the network does not correctly decode the second UL channel, it is the same as not correctly decoding the second UL channel without collision. As long as the probability of collision is small, it is also acceptable. Another drawback of the acknowledge information for the first PUCCH is the increased overhead for PDCCH as well as increased power consumption for both network and UE.

Observation 2: Acknowledge information for the first PUCCH needs further justification for both Mode A and Mode B.

Issue #4: Resource allocation for the second UL channel for Mode A.

As agreed in RAN1#117, PUSCH is supported for the resource allocation for the second UL channel for Mode A, while whether to support PUCCH for resource allocation for the second UL channel for Mode A is still for further study. For PUCCH, it can also be easily supported with minimum specification impact. For example, 1 bit in a DL DCI format can be used to indicate the transmission of the UEI beam report. The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report. This mechanism is similar as in legacy operation when a PUCCH transmission with HARQ-ACK overlaps with a PUCCH transmission with CSI report; in this case, the UE multiplexes HARQ-ACK and CSI report in the PUCCH for HARQ-ACK.

Comparing with only having PUSCH as the second UL channel, additional support of PUCCH for the UEI beam report is beneficial for overhead reduction for PDCCH transmission; note that reduction of signalling overhead is also required by the objective of the WID. If only PUSCH is supported for UEI beam report for Mode A, the network would always need to schedule a PUSCH transmission with or without UL data arriving for a UE when detecting a first PUCCH from the UE. Consider a typical case of DL heavy traffic scenario and there is only DL data arrived for a UE: if only PUSCH is supported for the second UL channel, when the network detects the first PUCCH from the UE, the network needs to transmit PDCCH with a DL DCI format scheduling a PDSCH reception and a UL DCI format to schedule a PUSCH for the UEI beam report. For the same setting, if PUCCH can be supported for the second UL channel, the DL DCI format for scheduling a PDSCH reception can also be used to indicate the UEI beam report transmission in a PUCCH; in this case, there is no need to transmit the UL DCI format to schedule a PUSCH thus the signaling overhead can be reduced. In addition, there will be additional network energy saving gain due to the omission of PDCCH transmission for the UL DCI format. If the traffic load is heavy in the network, there may not be enough resource for transmitting the DCI formats (e.g., both DL and UL DCI formats) for all the UEs at a same time, and the latency would be increased as a result due to the delayed transmission of some DCI formats.

Observation 3: Supporting both PUCCH and PUSCH for the second UL channel transmission for Mode A is beneficial for the signaling overhead reduction as well as latency reduction as required by the objective of the WID and it is beneficial for network energy saving.

Proposal 10: In addition to PUSCH, support PUCCH for the second UL channel transmission for Mode A.

- Add 1 bit in the DL DCI format to indicate the transmission of UEI report in a PUCCH resource for HARQ-ACK.

  - HARQ-ACK and UEI beam report are multiplexed in the PUCCH resource similar as the legacy operation of multiplexing HARQ-ACK and CSI report in a PUCCH resource.

Issue #5: Resource allocation for the second UL channel for Mode B.

UEI beam report is one kind of CSI report which can be transmitted on PUCCH and PUSCH in legacy operation. It is straightforward to support both PUCCH and PUSCH for UEI beam report transmission for Mode B. For both PUCCH and PUSCH, a periodic resource can be configured by dedicated RRC signaling. For example, the PUCCH resource configuration is similar as the one for P-CSI report and the PUSCH resource can reuse CG PUSCH.

Proposal 11: Support both PUCCH and PUSCH for the second UL channel transmission for Mode B.

- For UEI beam report in a PUCCH, a periodic PUCCH resource for PUCCH format 2/3/4 is configured by dedicated RRC signaling.

- For UEI beam report in a PUSCH, a CG PUSCH can be configured by dedicated RRC signaling.

Issue #6: Association between the first PUCCH and the second UL channel for Mode B.

One essential design aspect for Mode B is how to efficiently allocate and associate the UL resource(s) for the first and second UL channels. As we have analyzed, the UL resource(s) can be pre-configured for a UE by dedicated RRC signaling. To improve the resource utilization rate, the network can re-schedule or re-allocate the pre-configured UL resource(s) to other UEs or for other purposes if the first PUCCH is not received or not detected. Below, we provide a detailed design example to illustrate various resource allocation aspects regarding Mode B.

Figure 3. An example of resource configuration/allocation for Mode B

Figure 4. Another example of resource configuration/allocation for Mode B

For simplicity, the periodicity of the first resource for the first PUCCH is assumed to be the same as that of the second resource for the second UL channel. Furthermore, a time gap between the first PUCCH and the second UL channel can be configured by the network so that the network can re-allocate or re-schedule the resources for the second UL channel to avoid wasting of the pre-configured resources. For example, for each first resource for the first PUCCH transmission, there can be an associated second resource for the second UL channel transmission as shown in a pair of Figure 3. If the network does not detect the first PUCCH, the network can schedule the corresponding second resource for other purposes/UEs. The offset between the first resource and the second resource for each pair can ensure that the gNB can have enough processing time to re-schedule the second resource before the starting the second resource. For light traffic scenario, the time offset/gap can be configured as 0, allowing the lowest possible reporting latency.

To provide scheduling flexibility, the periodicity of the first resource and the second resource is can also be different, the time offset may be different for different pairs, and the network can configure a minimum time offset (as shown in Figure 4) as a unified solution. With the minimum time offset configured, the actual time offset between different resource pairs can be different. Furthermore, in TDD, some of the configured second resource(s) for beam reporting can be collided with DL symbol(s); in this case, the second UL resource(s) that collides with the DL symbol(s) cannot be used for beam reporting. Overall, for the example shown in Figure 4, after the UE has transmitted the first channel via a first UL resource, the UE would identify an earliest available second UL resource to transmit the beam report by accounting for (1) the configured minimum time offset between the two associated UL channels, and (2) collision(s) of the second resource(s) with DL symbols (in TDD) and/or other channels/signals.

Proposal 12:  On beam report transmission procedure for UE-initiated/event-driven beam reporting Mode B, a minimum time offset between the end of a transmission occasion of the first PUCCH and the start of the associated transmission occasion of the second UL channel is configured by dedicated RRC signaling.

Proposal 13:  On beam report transmission procedure for UE-initiated/event-driven beam reporting Mode B, in addition the minimum time offset, collision(s) between transmission occasion(s) of the second UL channel and other channel(s)/signal(s) – e.g., those overlapping with DL symbols in TDD – need to be accounted for determining an earliest available second UL resource(s) to transmit the beam report.

Issue #7: Acknowledge information for the second UL channel.

The acknowledge information for the second UL channel can be considered as a further enhancement for the UEI report, whether the feature is needed may depends on the detailed design of the second UL channel. The issue can be deprioritized until the detailed design of the second UL channel becomes clearer.

Observation 4: The issue of acknowledge information for the second UL channel can be deprioritized in RAN1#118.

# Conclusions

In this contribution, we provide our views on various design aspects related to Rel-19 UE-initiated beam reporting. In particular, we present the following proposals and observations focusing on event(s) and the corresponding triggering condition(s), measurement RS(s) configuration/determination for the Event-2, UL signaling content(s)/report format(s) and UCI based UL signaling procedure(s) designs:

Proposal 1: On UE-initiated/event-driven beam reporting, do not support Event-7a or Event-7b due to lack of applicable use cases.

Proposal 2: On UE-initiated/event-driven beam reporting, support Event-1 and reuse at least the same reporting format and the same steps in both Mode-A and Mode-B for the UCI based UL signaling procedures.

Proposal 3: Support to capture Event-2 evaluation condition(s) and procedure(s) in RAN1 specification.

Proposal 4: Regarding RS measurement for the current beam for Event-2:

- Support to confirm the working assumption, i.e., enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam(s)

- Support to introduce an explicit RRC parameter to enable the selection of either Scheme-1 or Scheme-2.

Proposal 5: Regarding RS measurement for the current beam for Event-2, when only one TRS is configured in the indicated TCI state, support Option-4, i.e., in such a case, Scheme-2 is used.

Proposal 6: Regarding explicit RS configuration for candidate beam measurement in evaluating Event 2, support Option-3, i.e., to provide candidate beam RS resource index(es) in TCI-State.

Proposal 7: On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 of Event-2, support candidate value(s) of ‘N’ as {2, 3, 4}

- If ‘N’ is not configured, one CRI/SSBRI and the corresponding L1-RSRP are reported

Proposal 8: Confirm the following Working Assumption made in RAN1#117 meeting.

Working Assumption On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case.

Observation 1: The usage of the UCI in the first PUCCH for both Mode A and Mode B is not aligned with the existing SR.

Proposal 9: On beam report transmission procedure for UE-initiated/event-driven beam reporting, support a new UCI type for the first PUCCH for both Mode A and Mode B.

Observation 2: Acknowledge information for the first PUCCH needs further justification for both Mode A and Mode B.

Observation 3: Supporting both PUCCH and PUSCH for the second UL channel transmission for Mode A is beneficial for the signaling overhead reduction as well as latency reduction as required by the objective of the WID and it is beneficial for network energy saving.

Proposal 10: In addition to PUSCH, support PUCCH for the second UL channel transmission for Mode A.

- Add 1 bit in the DL DCI format to indicate the transmission of UEI report in a PUCCH resource for HARQ-ACK.

  - HARQ-ACK and UEI beam report are multiplexed in the PUCCH resource similar as the legacy operation of multiplexing HARQ-ACK and CSI report in a PUCCH resource.

Proposal 11: Support both PUCCH and PUSCH for the second UL channel transmission for Mode B.

- For UEI beam report in a PUCCH, a periodic PUCCH resource for PUCCH format 2/3/4 is configured by dedicated RRC signaling.

- For UEI beam report in a PUSCH, a CG PUSCH can be configured by dedicated RRC signaling.

Proposal 12:  On beam report transmission procedure for UE-initiated/event-driven beam reporting Mode B, a minimum time offset between the end of a transmission occasion of the first PUCCH and the start of the associated transmission occasion of the second UL channel is configured by dedicated RRC signaling.

Proposal 13:  On beam report transmission procedure for UE-initiated/event-driven beam reporting Mode B, in addition the minimum time offset, collision(s) between transmission occasion(s) of the second UL channel and other channel(s)/signal(s) – e.g., those overlapping with DL symbols in TDD – need to be accounted for determining an earliest available second UL resource(s) to transmit the beam report.

Observation 4: The issue of acknowledge information for the second UL channel can be deprioritized in RAN1#118.

# References

[1] 3GPP RAN#102, RP-233962, “New WID: NR MIMO Phase 5,” December 2023.

[2] Chairman Notes, 3GPP RAN1#117, May 20th – 24th, 2024.