3GPP TSG-RAN WG1 Meeting #118	R1-2406808

Maastricht, Netherlands, May 19th – 23rd, 2024

Agenda Item:	9.11.2

Source:	Ericsson

Title:	On HD-FDD Redcap UEs for NTN

Document for:	Discussion

# 1	Introduction

The Rel-19 Work Item Description (WID) entitled “Non-Terrestrial Networks (NTN) for NR Phase 3” includes one objective on HD-FDD (e)RedCap UEs touching upon RAN1, which was recently updated during RAN Plenary# 104 as follows [1]:

Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In this contribution we provide a follow-up on this Rel-19 objective according with the guidance from RAN Plenary.

# 2	Collision cases 3 & 4 for HD-FDD (e)RedCap in NTN

During RAN1# 117, the following conclusions were reached [2]:

Conclusion For Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4. - Note: further discussion on other cases is not precluded Conclusion For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN.

For Rel-19 HD-FDD (e)RedCap in NTN, from among all the “collision cases” that exist in terrestrial networks, only two of them (i.e., case 3 & 4) are associated with “error cases,” whereas all other cases have “prioritization rules” well defined.

RAN1 concluded that for Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4 (Existing priority rules for other collision cases can be reused).

There is a Note under the conclusion reached for collision cases 3 & 4, which means that if RAN1 ends up with an enhancement for collision cases 3 & 4 that somehow is also beneficial for other cases, then we can further discuss to apply such enhancement to other cases.

In line with the previous RAN1 conclusions, RAN Plenary #104 updated the WID as follows in relation with the work to be done in RAN1 [1]:

- For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded.

In line with RAN1 conclusions, RAN Plenary #104 updated the WID as follows:

- Enhancements are targeted for the following HD collisions cases (cases 3 and 4):

  - Semi-statically configured DL reception collides with semi-statically configured UL transmission

  - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission

- Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded.

In the sections below we provide our views on the potential enhancements for collision cases 3 and 4.

## 2.1	Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission.

| Case 3 as described in TS 38.213 clause 17.2 [3]: |  |
| --- | --- |
| A HD-UE does not expect to receive both dedicated higher layer parameters configuring transmission in a set of symbols and dedicated higher layer parameters configuring reception in the set of symbols. A HD-UE does not expect to receive both a Type-0/0A/0B/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols, except Type-2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and configured-grant based PUSCH transmission as described in clause 19.1 in the set of symbols for which case the UE follows the procedure as in clause 5.1B.2.6 in [10, TS 38.133]. The UE expects to be configured with a Type-2-PDCCH CSS set configuration for PDCCH reception such that there is at least one paging occasion that does not overlap with configured-grant based PUSCH transmission as described in clause 19.1 per SI modification period. |  |

Brief explanation of the legacy rule: Error case, i.e., HD-FDD UE does not expect to be configured with overlapping occasions of dedicated configured UL transmission and dedicated configured DL reception dedicated configured UL transmission and PDCCH in Type-0/0A/1/2 CSS set

For Case 3, we have the following observations and proposal:

Case 3 “Semi-statically configured DL reception collides with semi-statically configured UL transmission”, for a HD-FDD (e)RedCap UE in Terrestrial Networks is treated as an error case.

In relation with the previous observation, the technical specification states that “A HD-UE does not expect to receive both a Type-0/0A/0B/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols, except Type-2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and configured-grant based PUSCH transmission”.

In addition to what is mentioned in the previous observation, the technical specification states “The UE expects to be configured with a Type-2-PDCCH CSS set configuration for PDCCH reception such that there is at least one paging occasion that does not overlap with configured-grant based PUSCH transmission”.

Case 3 has been identified to be less suitable for NTN. Hence, RAN1 may define a prioritization (e.g., DL is prioritized over UL) instead of having an error case for HD-FDD (e)RedCap in NTN.

For Case 3 and a HD-FDD (e)RedCap UE in NTN, in line with the spirit of guaranteeing “at least one paging occasion,” RAN 1 could prioritize semi-statically configured DL over semi-statically configured UL. It can be discussed if in general semi-statically configured DL reception is to be prioritized over a semi-statically configured UL transmission, or if a specific semi-statically configured DL reception (e.g., Type-2-PDCCH CSS) is to be prioritized.

For Case 3, RAN1 may define a prioritization instead of inheriting an error case for HD-FDD (e)RedCap in NTN.

- In line with the spirit of guaranteeing “at least one paging occasion,” RAN 1 could consider prioritizing semi-statically configured DL over semi-statically configured UL.

- It can be discussed if in general semi-statically configured DL reception is to be prioritized over a semi-statically configured UL transmission, or if a specific semi-statically configured DL reception (e.g., Type-2-PDCCH CSS) is to be prioritized.

## 2.2	Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission.

| Case 4 as described in TS 38.213 clause 17.2 [3]: |  |
| --- | --- |
| A HD-UE does not expect to detect a DCI format scheduling a reception in a set of symbols and detect a DCI format scheduling a transmission in any symbol from the set of symbols. |  |

Brief explanation of the legacy rule: Error case, i.e., HD-FDD UE does not expect to be dynamically scheduled with overlapping DL and UL

For Case 4, we have the following observations and proposal:

Case 4 “Dynamically scheduled DL reception collides with dynamic scheduled UL transmission”, for a HD-FDD (e)RedCap UE in Terrestrial Networks is treated as an error case.

In relation with the previous observation, legacy states that “A HD-UE does not expect to detect a DCI format scheduling a reception in a set of symbols and detect a DCI format scheduling a transmission in any symbol from the set of symbols”.

Case 4 has been identified to be less suitable for NTN. Hence, RAN1 may define a prioritization (e.g., DL is prioritized over UL) instead of having an error case for HD-FDD (e)RedCap in NTN.

For Case 4 and a HD-FDD (e)RedCap UE in NTN, in view of the relevance of receiving SIB19, RAN 1 could consider prioritizing “dynamically scheduled DL reception” over “dynamic scheduled UL transmission”. It can be discussed if in general “dynamically scheduled DL reception” is to be prioritized over a “dynamic scheduled UL transmission”, or if a specific dynamically scheduled DL reception (e.g., SIB19) is to be prioritized.

For Case 4, RAN1 may define a prioritization instead of inheriting an error case for HD-FDD (e)RedCap in NTN.

- In view of the relevance of receiving SIB19, RAN 1 could consider prioritizing “dynamically scheduled DL reception” over “dynamic scheduled UL transmission”.

- It can be discussed if in general “dynamically scheduled DL reception” is to be prioritized over a “dynamic scheduled UL transmission”, or if a specific dynamically scheduled DL reception (e.g., SIB19) is to be prioritized.

## 2.3	Implementation in the technical specification of the prioritization rules for collision cases 3 and 4.

In RAN1, there have been preliminary proposals on whether the prioritization rules for collision cases 3 and 4 should be added to the specification using any of the following alternatives:

⦁	Adding a fixed rule in the specification (i.e., no signaling).

⦁	Leaving up to the UE implementation to decide the priority (i.e., Adding a statement in the specification like the one that exists for collision case 6 “the HD-UE can select based on its implementation whether to either transmit … or receive …”).

⦁	Introducing signalling for the network to decide.

On the possible ways of implementing the prioritization rules for collision cases 3 and 4, we have the following observations and proposal:

There have been preliminary proposals for adding prioritization rules for collision cases 3 and 4 using any of the following alternatives:

- Adding a fixed rule in the specification (i.e., no signalling).

- Leaving up to the UE implementation to decide the priority (i.e., Adding a statement in the specification like the one that exists for collision case 6 “the HD-UE can select based on its implementation whether to either transmit … or receive …”).

- Introducing signalling for the network to decide the priority.

In our understanding, the intention behind defining priorities for collision cases 3 and 4 is eliminating the current uncertainty of those cases being treated as error cases. Thus, adding a fixed rule in the specification seems to be a suitable approach (just as it has been done for most of the existing collision cases).

In relation with the previous observation. Our initial view is that the other alternatives (i.e., up to UE implementation, and signalling) for introducing the priorities of collision cases 3 and 4 are less preferred because of the following reasons:

- Leaving up to the UE implementation to decide the priority: In terms of eliminating the uncertainty, this alternative keeps the uncertainty and varies from time-to-time (i.e., as per UE’s discretion) what is to be prioritized.

- Introducing signalling for the network to decide the priority: The information to be potentially prioritized (e.g., SIB19) takes place during both initial access and connected mode. Thus, using signalling to define priority rules for collision cases 3 and 4 likely will end-up impacting “common control information” for initial access and “UE specific control information” for when the UE has established RRC connection with the network.

RAN1 to investigate Pros and Cos (in terms of e.g., eliminating uncertainty, spec impacts, applicability during initial access and connected mode) of the following preliminary proposals for introducing prioritization rules for collision cases 3 and 4:

- Adding a fixed rule in the specification (i.e., no signalling).

- Leaving up to the UE implementation to decide the priority (i.e., Adding a statement in the specification like the one that exists for collision case 6 “the HD-UE can select based on its implementation whether to either transmit … or receive …”).

- Introducing signalling for the network to decide the priority.

## 2.4	RAN1#117 Conclusion related to cases 1, 2, 5, and 6.

In RAN1#117, the following conclusion related to cases 1, 2, 5, and 6 was captured in [2]:

Conclusion For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN.

An overview of the legacy collision cases defined for HD-FDD UEs is provided through Table 1.

| Collisions Cases as described in RAN1#116 | Brief explanation of the legacy rule |
| --- | --- |
| Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission. | Dynamic DL is prioritized. |
| Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission. | Dynamic UL is prioritized |
| Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission. | Error Case |
| Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission. | Error Case |
| Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission. | SSB is prioritized, overlapping uplink is dropped. |
| Case 6: Dynamic or semi-static DL collides with valid RO. | Up to UE implementation whether to receive DL or transmit PRACH/MsgA |
| Case 7: Collision due to direction switching. | Not specified as a separate rule. TS 36.211 clause 4.3.2 (see [5]) describes the definitions for “DL-to-UL switching time” and “UL-to-DL switching time” which are used in other collision rules. |

As can be seen from Table 1, “collision cases 1, 2, 5 and 6” they all count with a prioritization defined, which makes those cases not as uncertain as case 3 and case 4 which are considered error cases. Having said that, “collision cases 1, 2, 5 and 6” offer functional support for HD-FDD UEs in NTN and it is not essential to optimize them.

It is important to highlight that “collision cases 1, 2, 5 and 6,” they all count with a prioritization defined, which makes those cases not as uncertain as case 3 and case 4 which in legacy are considered error cases.

In line with RAN1 conclusions, “For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN.” Collision cases 1, 2, 5, and 6 offer functional support for HD-FDD UEs in NTN and it is not essential to optimize them.

# 3	Conclusion

Based on the discussion in the previous section we made the following observations:

Observation 1	For Rel-19 HD-FDD (e)RedCap in NTN, from among all the “collision cases” that exist in terrestrial networks, only two of them (i.e., case 3 & 4) are associated with “error cases,” whereas all other cases have “prioritization rules” well defined.

Observation 2	RAN1 concluded that for Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4 (Existing priority rules for other collision cases can be reused).

Observation 3	There is a Note under the conclusion reached for collision cases 3 & 4, which means that if RAN1 ends up with an enhancement for collision cases 3 & 4 that somehow is also beneficial for other cases, then we can further discuss to apply such enhancement to other cases.

Observation 4	In line with RAN1 conclusions, RAN Plenary #104 updated the WID as follows:

o	Enhancements are targeted for the following HD collisions cases (cases 3 and 4):

	Semi-statically configured DL reception collides with semi-statically configured UL transmission

	Dynamically scheduled DL reception collides with dynamic scheduled UL transmission

o	Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded.

Observation 5	Case 3 “Semi-statically configured DL reception collides with semi-statically configured UL transmission”, for a HD-FDD (e)RedCap UE in Terrestrial Networks is treated as an error case.

Observation 6	In relation with the previous observation, the technical specification states that “A HD-UE does not expect to receive both a Type-0/0A/0B/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols, except Type-2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and configured-grant based PUSCH transmission”.

Observation 7	In addition to what is mentioned in the previous observation, the technical specification states “The UE expects to be configured with a Type-2-PDCCH CSS set configuration for PDCCH reception such that there is at least one paging occasion that does not overlap with configured-grant based PUSCH transmission”.

Observation 8	Case 3 has been identified to be less suitable for NTN. Hence, RAN1 may define a prioritization (e.g., DL is prioritized over UL) instead of having an error case for HD-FDD (e)RedCap in NTN.

Observation 9	For Case 3 and a HD-FDD (e)RedCap UE in NTN, in line with the spirit of guaranteeing “at least one paging occasion,” RAN 1 could prioritize semi-statically configured DL over semi-statically configured UL. It can be discussed if in general semi-statically configured DL reception is to be prioritized over a semi-statically configured UL transmission, or if a specific semi-statically configured DL reception (e.g., Type-2-PDCCH CSS) is to be prioritized.

Observation 10	Case 4 “Dynamically scheduled DL reception collides with dynamic scheduled UL transmission”, for a HD-FDD (e)RedCap UE in Terrestrial Networks is treated as an error case.

Observation 11	In relation with the previous observation, legacy states that “A HD-UE does not expect to detect a DCI format scheduling a reception in a set of symbols and detect a DCI format scheduling a transmission in any symbol from the set of symbols”.

Observation 12	Case 4 has been identified to be less suitable for NTN. Hence, RAN1 may define a prioritization (e.g., DL is prioritized over UL) instead of having an error case for HD-FDD (e)RedCap in NTN.

Observation 13	For Case 4 and a HD-FDD (e)RedCap UE in NTN, in view of the relevance of receiving SIB19, RAN 1 could consider prioritizing “dynamically scheduled DL reception” over “dynamic scheduled UL transmission”. It can be discussed if in general “dynamically scheduled DL reception” is to be prioritized over a “dynamic scheduled UL transmission”, or if a specific dynamically scheduled DL reception (e.g., SIB19) is to be prioritized.

Observation 14	There have been preliminary proposals for adding prioritization rules for collision cases 3 and 4 using any of the following alternatives:

	Adding a fixed rule in the specification (i.e., no signalling).

	Leaving up to the UE implementation to decide the priority (i.e., Adding a statement in the specification like the one that exists for collision case 6 “the HD-UE can select based on its implementation whether to either transmit … or receive …”).

	Introducing signalling for the network to decide the priority.

Observation 15	In our understanding, the intention behind defining priorities for collision cases 3 and 4 is eliminating the current uncertainty of those cases being treated as error cases. Thus, adding a fixed rule in the specification seems to be a suitable approach (just as it has been done for most of the existing collision cases).

Observation 16	In relation with the previous observation. Our initial view is that the other alternatives (i.e., up to UE implementation, and signalling) for introducing the priorities of collision cases 3 and 4 are less preferred because of the following reasons:

	Leaving up to the UE implementation to decide the priority: In terms of eliminating the uncertainty, this alternative keeps the uncertainty and varies from time-to-time (i.e., as per UE’s discretion) what is to be prioritized.

	Introducing signalling for the network to decide the priority: The information to be potentially prioritized (e.g., SIB19) takes place during both initial access and connected mode. Thus, using signalling to define priority rules for collision cases 3 and 4 likely will end-up impacting “common control information” for initial access and “UE specific control information” for when the UE has established RRC connection with the network.

Observation 17	It is important to highlight that “collision cases 1, 2, 5 and 6,” they all count with a prioritization defined, which makes those cases not as uncertain as case 3 and case 4 which in legacy are considered error cases.

Observation 18	In line with RAN1 conclusions, “For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN.” Collision cases 1, 2, 5, and 6 offer functional support for HD-FDD UEs in NTN and it is not essential to optimize them.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For Case 3, RAN1 may define a prioritization instead of inheriting an error case for HD-FDD (e)RedCap in NTN.

	In line with the spirit of guaranteeing “at least one paging occasion,” RAN 1 could consider prioritizing semi-statically configured DL over semi-statically configured UL.

	It can be discussed if in general semi-statically configured DL reception is to be prioritized over a semi-statically configured UL transmission, or if a specific semi-statically configured DL reception (e.g., Type-2-PDCCH CSS) is to be prioritized.

Proposal 2	For Case 4, RAN1 may define a prioritization instead of inheriting an error case for HD-FDD (e)RedCap in NTN.

	In view of the relevance of receiving SIB19, RAN 1 could consider prioritizing “dynamically scheduled DL reception” over “dynamic scheduled UL transmission”.

	It can be discussed if in general “dynamically scheduled DL reception” is to be prioritized over a “dynamic scheduled UL transmission”, or if a specific dynamically scheduled DL reception (e.g., SIB19) is to be prioritized.

Proposal 3	RAN1 to investigate Pros and Cos (in terms of e.g., eliminating uncertainty, spec impacts, applicability during initial access and connected mode) of the following preliminary proposals for introducing prioritization rules for collision cases 3 and 4:

	Adding a fixed rule in the specification (i.e., no signalling).

	Leaving up to the UE implementation to decide the priority (i.e., Adding a statement in the specification like the one that exists for collision case 6 “the HD-UE can select based on its implementation whether to either transmit … or receive …”).

	Introducing signalling for the network to decide the priority.

# 4	References

RP-241667, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, 3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024.

Session notes for 9.11 Session notes for 9.11 Non-Terrestrial Networks (NTN) for NR Phase 3 and Internet of Things (IoT) Phase 3, 3GPP TSG RAN WG1 #117, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.

3GPP TS 38.213, “NR; Physical layer procedures for control,” v18.1.0.