3GPP TSG RAN WG1 #118                                                                                    R1-2406933

Maastricht, NL, August 19th – 23rd, 2024

Source:	NTT DOCOMO, INC.

Title:	Study on device architectures for Ambient IoT

Agenda Item:	9.4.1.2

Document for: 	Discussion and Decision

- Introduction

In RAN#102 meeting, a new SID “Study on solutions for Ambient IoT (Internet of Things) in NR” was approved and revised SID in RAN#103 meeting is [1]. The objectives are as follows.

The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1.

In this contribution, we discuss device architectures for A-IoT device. Note that at the RAN1#116 meeting, following definition of device type was agreed and followings are discussed based on the definition.

Agreement For the purpose of the study, RAN1 uses the following terminologies: - Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device.

- Discussion

2.1.	Device unavailability and sustainable operation time

At the RAN#103 meeting, it was agreed to assume the charging time of energy harvesting as up to 10s.

Proposal 2 - Confirm that study of design of energy harvesting signal/waveform is out of SI scope in Rel-19 - The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study [RAN2, RAN1] - Duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds - Note: this value can be revisited in future RAN plenary meetings, if necessary - TR 38.848 clause 5.6 statement on latency remains the case with respect to a single device, i.e.: “NOTE: The time for charging the Ambient IoT device storage (if present) is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implementation issue only.” - No SID revision is necessary

In addition, device would not perform RF energy harvesting and R2D/D2R Rx/Tx simultaneously based on the following agreement at the RAN1#117 meeting.

Agreement For study purpose, assume that A-IoT device has a single antenna for both communication (tx/rx) and RF energy harvesting purposes.

Based on the agreements at the previous meetings, device availability/unavailability by energy harvesting would be further studied. To discuss the device availability/unavailability, definition of device sates was discussed in the previous meetings while no consensus has been achieved. In our assumption, at least RF energy harvesting, Rx/Tx of R2D/D2R, timing count/maintaining synchronization and memory retention should be considered as possible major operation of A-IoT device. The supported operations would be different for each device state and we suggest to consider at least three device states, i.e., ON/OFF/SLEEP state, for further study.

Proposal 1: Define ON/OFF/SLEEP state for A-IoT device.

- ON state supports at least following operation

  - Reception/monitoring of any R2D

  - Transmission of any D2R

  - Maintaining timing synchronization according to post-synchronization

  - Maintaining memory retention

- SLEEP state supports at least following operation

  - Maintaining timing synchronization according to post-synchronization

  - Maintaining memory

- OFF state supports at least following operation

  - RF energy harvesting

The definition of ON/OFF/SLEEP state may be discussed in AI 9.4.2.2.

Fig.1: Example on device ON/SLEEP/OFF state transition

On top of the device states and device unavailability which can be up to tens of seconds by RF energy harvesting, it is still unclear how long a device can keep available without running out of energy. More specifically, device should keep ON state without RF energy harvesting at least during one PRDCH Rx or PDRCH Tx. For example, assuming that 1000bits TB is transmitted in one PRDCH with low data rate such as M=2 for OOK, it takes 72ms with Manchester encoding. Even if M=8 for OOK, it takes 18ms. It is unclear whether device can sustain e.g., 72 slots as ON state, and hence device sustainable operation time without RF energy harvesting should be considered to discuss the possible TB size and corresponding data rate. In addition, device may need to keep ON/SLEEP state between e.g., R2D Rx and corresponding D2R Tx to perform the D2R transmission timing count. Thus, the device sustainable operation time should be considered for scheduling/timing aspects as well. Furthermore, device may need to keep ON/SLEEP state during whole inventory process to retain the received R2D information, e.g., ID for inventory process, in its memory. This means that the design of contention-based access and/or inventory completion time should be assessed considering device sustainable operation time.

Based on the above, we suggest studying the device sustainable operation time.

Proposal 2: Study device sustainable operation time.

- Definition of device sustainable operation time: The continuous time duration that deice does not fall into RF energy harvesting mode/OFF state.

It was agreed that the charging time would be up to several 10s. However, e.g., depending on the assumption on capacitor size, charged energy, distance from RF energy source, the charging time/device sustainable operation time would be different. Thus, at least the applicable capacitor size should be studied to assess the device’s sustainable operation time.

For the study of device sustainable operation time, at least the assumption on energy storage such as capacitor size, power consumption of ON/SLEEP state and possible time duration of ON/SLEEP state such as inventory completion time should be considered.

Proposal 3: For the study of device sustainable operation time, study the assumption at least on the followings;

- Energy storage, e.g., capacitor size

- Power consumption for each of ON and SLEEP state, e.g., considering the power consumption on the corresponding clock

- Possible duration of ON/SLEEP state

  - Clock assumptions and timing error compensation

At the RAN1#117 meeting, it was agreed to study the clock/LO assumption based on the template.

Agreement The following template is used for capturing descriptions on clock/LO. Purpose; Applicable device types; Clock speed; Power consumption; [Initial clock Accuracy]; [Accuracy after clock sync]; [Clock drift]. Purpose #1 of the clock. Purpose #N of the clock. Etc.

For the study of timing error of A-IoT device, the assumption on sampling frequency offset (SFO) and time drift model has been discussed in AI 9.4.1.1 and it was agreed as WA for evaluation purpose.

Working assumption on [0q] (sampling frequency) in link level simulation table Companies to report the Sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): - (M) Randomly select a value from the range of [0.1 ~ 1] *10^4 ppm for device 2, - (M) Randomly select a value from the range of [0.1 ~ 1] * 10^5 ppm for device 1, - (O) Randomly select a value from the range of [0.1 ~ 1] *10^5 ppm for device 2, - FFS: Optionally evaluate a fixed value SFO for device 1 and 2 - Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution - Note: Above values are only for sampling purpose. - FFS other values - Note: Above assumptions are only for LLS evaluation purpose only for R2D and [D2R]. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. - Note: Accuracy can be improved after clock calibration for at least device 2. FFS applicable for device 1 - Note: SFO after clock calibration can be applied to Fe. - FFS other models CFO for device 2b. - [100ppm/200ppm/1000ppm, 0.1ppm/s]” Note: Above assumptions are for LLS evaluation purpose only

According to the agreement, it is captured as note that accuracy can be improved after clock calibration at least for device 2. Meanwhile the value range on initial SFO is agreed as WA for each device type, it is unclear how much the accuracy can be improved after clock calibration. The decoding performance would be affected by the ability of clock calibration and it is important to align the assumption on whether/how the timing error can be compensated for each device type. The expected timing error would be different depending on the assumption on the performance of clock/LO, and hence initial clock accuracy and clock accuracy after calibration should be studied corresponding to the assumption of purpose, clock speed and power consumption of the clock/LO.

Proposal 4: Study the clock accuracy as a component of clock/LO performance.

Depending on the purpose of the clock/LO, the required clock speed and corresponding power consumption would be different, and it should be discussed first. Per our understanding, at least it should be discussed whether the same or different clock/LO performance should be assumed for 1) R2D reception/sampling and D2R transmission, 2) for backscatter transmission and active transmission and 3) timing count between reception/transmission. For the 1st and 2nd discussion point, it should be considered especially for device 2b which supports internal carrier wave generation. The clock speed for device 2b should be the same as carrier frequency and it may increase power consumption for the clock, and hence it may not be preferable to use this clock for other purposes as well. For the 3rd discussion point, i.e., whether the same clock which is used for R2D Rx/D2R Tx can be used during the timing count b/w Rx/Tx or low power consumption clock should be used during the timing count b/w Rx/Tx. In our view, it depends on e.g., the device sustainable operation time and inventory completion time. If device cannot sustain without RF energy harvesting for inventory completion time when the device operates with the clock which is used for R2D Rx/D2R Tx, low power consumption clock needs be considered for the purpose of timing count b/w Rx while it may degrade the accuracy of timing count.

Based on the discussion, we assume the following four types of clock/LO.

Proposal 5: Study the purposes of clock/LO considering the followings.

- whether the same or different clock/LO performance should be assumed for R2D reception/sampling and D2R transmission

- whether the same or different clock/LO performance should be assumed for backscatter transmission and active transmission

- whether the same or different clock/LO performance should be assumed for timing count between reception/transmission

Proposal 6: Consider the following Table 1 as a starting point for clock/LO assumption.

Table 1: Clock/LO assumption for each purpose

|  | Purpose | Applicable device types | Clock speed | Power consumption | Initial clock Accuracy | Accuracy after clock sync | [Clock drift] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Purpose #1 of the clock | Timing count b/w Rx/Tx | device 1/2a/2b | [10s] kHz to a few MHz | Less than 1uW | 10^4 ~ 10^5 ppm | < 10^4ppm | FFS |
| Purpose #2 of the clock | R2D Rx and Timing acquisition/ sync signal monitoring (sampling) [and Timing count b/w Rx/Tx] | device 1/2a/2b | < a few MHz, or same as carrier frequency | 1uW to 10s uW | 10^4 ~ 10^5 ppm | < 10^4ppm | FFS |
| Purpose #3-1 of the clock | D2R Tx on backscattering w/ frequency shit | device 1/2a | < a few MHz | 1uW to 10s uW | 10^4 ~ 10^5 ppm | < 10^4 ppm | FFS |
| Purpose #3-2 of the clock | D2R Tx on carrier frequency generation | device 2b | Same as carrier frequency | 10s ~ 100 uW | 10^3~ 10^4 ppm | < 10^3 ppm | FFS |
| Note1: clock #1 and clock #2 may be the same clock depending on the device implementation Note2: clock #2 and clock #3-1 may be the same clock depending on the device implementation |  |  |  |  |  |  |  |

In our view, the assumption on the clock and timing error compensation would affect the discussion on the frame structure and synchronization/timing acquisition.

  - Frequency shifter for each device type

Large frequency shift

At the RAN1#117 meeting, applicability of large frequency shift for A-IoT device architecture was discussed and following observation was made.

Observation For large frequency shift: - Large frequency shift can be used in shifting reflected signal in tens of MHz, e.g., from FDD DL to FDD UL frequency or vice versa. - Large frequency shift consumes 10s of uW to 100s of uW. - Large frequency shift is not feasible for device 1. - Large frequency shift requires a clock for IF generation which is accurate enough to avoid large guard band and interference to adjacent channels/bands. - Large frequency shift requires image suppression and may require harmonics suppression - Note: details of image suppression and harmonics suppression are not discussed in RAN1 - FFS: whether large frequency shift is necessary and feasible for device 2a

According to the observation, it is captured as FFS whether large frequency shift is necessary and feasible for device 2a in its architecture. Regarding the necessity of large frequency shift, it will provide the frequency shift of up to tens of MHz and A-IoT device can switch the reception in DL spectrum to transmission in UL spectrum. With this assumption, the large frequency shift can potentially mitigate the self-link interference and cross-link interference which is desired especially for the co-existing/co-site operation scenario with NR for in band operation.

Observation 1: The large frequency shift (e.g., up to tens of MHz) is beneficial to mitigate the self/cross-link interference to/from A-IoT/NR.

Regarding the feasibility of large frequency shift, according to the agreed observation, the large frequency shifter itself is feasible for device 2a from power consumption perspective. However, with a large frequency shift by A-IoT device, it is concerned that it would cause interference due to image/harmonics or out band emission. Per our understanding, minor image/harmonics suppression can be supported by a few tens of uW, however, it would be difficult to sufficiently supress the interferences with such low power consumption. Meanwhile the details of image/harmonics suppression are not discussed in RAN1, the impact by the image/harmonics with or without suppression should be assessed in RAN4.

Observation 2: From power consumption perspective, the large frequency shifter itself is feasible for device 2a while additional power consumption for image/harmonics suppression and/or for clock with enough accuracy for IF generation may need to be considered.

Proposal 7: Impacts on image/harmonics which are caused by frequency shift should be studied in RAN4.

Small frequency shift

Meanwhile it is not clearly captured in the agreed device architecture of device 1, it was pointed that existing RFID can support FM0 encoding which is equivalent to frequency shift in baseband operation, and even device 1 can support small (e.g., up to a few MHz) frequency shift. In addition, it was argued that the small frequency shift can be realized by square wave generation as well without applying line coding. With either way of small frequency shift, it can be realized by the BB processing and would not increase the device complexity. Therefore, we suggest to consider small frequency shift for both device 1 and 2a. The details on how to realize small frequency shift and impact on image/harmonics suppression can be further studied.

Observation 3: The small frequency shift (e.g., up to a few MHz) can be realized by BB processing such as line coding or square wave modulation for both device 1 and 2a.

Fig.2: Small frequency shift for CW interference mitigation and FDMA.

DSB and SSB

To support large/small frequency shift, it should be discussed whether single side band modulation for D2R can be supported by A-IoT device. We summarize the pros and cons of SSB and DSB transmission as follows.

- SSB

  - Pros: Higher spectrum efficiency, less impact on FDMA/large-FS

  - Cons: Higher complexity for image/harmonics suppression

- DSB

  - Pros: Less complexity on A-IoT device, advantage on SNR

  - Cons: Less spectrum efficiency, large impact on FDMA/large-FS

While the impacts on image/harmonics and feasibility on large frequency shifter should be studied further as discussed above, both SSB and DSB have benefit, and hence it is premature to focus on DSB for D2R transmission. Therefore, we prefer to study the characteristics of SSB as well as DSB.

Proposal 8: Study both DSB and SSB modulation for D2R considering the feasibility on SSB modulation for D2R, spectrum efficiency, impact on SNR.

  - Multi-spectrum/band support

At the RAN1#116 meeting, it was agreed to study the following cases for the assumption of spectrum for carrier wave transmission and backscattered D2R reception.

Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. - Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum - Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum - Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. - Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum - Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum - Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

In addition, in our companion contribution[2], we discuss the possible deployment scenarios for evaluation of coverage and coexistence, including the possible spectrum for R2D reception.

According to the agreement and our proposal in [2], various assumption on the combinations of spectrum for CW, R2D and D2R transmission are on the table for each topology, and the feasibility and pros/cons of each scenario would be further studied.

Regarding the assumption on the spectrum, it is unclear whether A-IoT device is capable of multiple assumption on the reception/transmission spectrum. According to the SID, it is captured that “For Topology 2, no difference in physical layer design from Topology 1.” Based on the description, physical layer design should be the same for Topology 1 and Topology 2 and whether the reader is BS or intermediate UE is transparent from A-IoT device perspective. However, we believe this does not intend that the same spectrum should be used for Topology 1 and Topology 2. For example, DL spectrum is used for CW transmission and D2R transmission for Topology 1 while UL spectrum can be used for Topology 2, while there seems no common understanding. Therefore, to discuss the feasibility of each deployment scenario for evaluation, it should be clarified whether A-IoT device is capable of using different spectrum for R2D, carrier wave and D2R depending on the deployment scenario/topology.

To discuss this, it should be studied how A-IoT device can identify the frequency location of initial R2D reception, e.g., sync-raster concept in legacy NR, depending on the deployment scenario/topology. At least there are two possibilities; A-IoT device can support blind detection of initial frequency location or the initial frequency location is implemented to an A-IoT device in advance based on the deployment scenario for the A-IoT device and the initial frequency location is fixed permanently. In our view, it is not feasible to support the blind detection of initial frequency location similar to legacy NR UE from power consumption and cost/complexity of A-IoT device perspective. On the other hand, implementing initial frequency location in advance for each deployment scenario/topology would increase the operation burden to deploy A-IoT in NW and may not be preferable.

Proposal 9: Discuss whether A-IoT device is capable of different spectrum for R2D, carrier wave and D2R depending on the deployment scenario/topology.

- Discuss how A-IoT device can identify the frequency location of initial R2D reception, e.g., sync-raster concept in NR.

- Conclusion

In this contribution, we discussed device architectures for A-IoT device. Based on the discussion, we made following observations and proposals.

Observation 1: The large frequency shift (e.g., up to tens of MHz) is beneficial to mitigate the self/cross-link interference to/from A-IoT/NR.

Observation 2: From power consumption perspective, the large frequency shifter itself is feasible for device 2a while additional power consumption for image/harmonics suppression and/or for clock with enough accuracy for IF generation may need to be considered.

Observation 3: The small frequency shift (e.g., up to a few MHz) can be realized by BB processing such as line coding or square wave modulation for both device 1 and 2a.

Proposal 1: Define ON/OFF/SLEEP state for A-IoT device.

- ON state supports at least following operation

  - Reception/monitoring of any R2D

  - Transmission of any D2R

  - Maintaining timing synchronization according to post-sync

  - Maintaining memory retention

- SLEEP state supports at least following operation

  - Maintaining timing synchronization according to post-sync

  - Maintaining memory

- OFF state supports at least following operation

  - RF energy harvesting

Proposal 2: Study device sustainable operation time.

- Definition of device sustainable operation time: The continuous time duration that deice does not fall into RF energy harvesting mode/OFF state.

Proposal 3: For the study of device sustainable operation time, study the assumption at least on the followings;

- Energy storage, e.g., capacitor size

- Power consumption for each of ON and SLEEP state, e.g., considering the power consumption on the corresponding clock

- Possible duration of ON/SLEEP

Proposal 4: Study the clock accuracy as a component of clock/LO performance.

Proposal 5: Study the purposes of clock/LO considering the followings.

- whether the same or different clock/LO performance should be assumed for R2D reception/sampling and D2R transmission

- whether the same or different clock/LO performance should be assumed for backscatter transmission and active transmission

- whether the same or different clock/LO performance should be assumed for timing count between reception/transmission

Proposal 6: Consider the following Table 1 as a starting point for clock/LO assumption.

Table 1: Clock/LO assumption for each purpose

|  | Purpose | Applicable device types | Clock speed | Power consumption | Initial clock Accuracy | Accuracy after clock sync | [Clock drift] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Purpose #1 of the clock | Timing count b/w Rx/Tx | device 1/2a/2b | [10s] kHz to a few MHz | Less than 1uW | 10^4 ~ 10^5 ppm | < 10^4ppm | FFS |
| Purpose #2 of the clock | R2D Rx and Timing acquisition/ sync signal monitoring (sampling) [and Timing count b/w Rx/Tx] | device 1/2a/2b | < a few MHz, or same as carrier frequency | 1uW to 10s uW | 10^4 ~ 10^5 ppm | < 10^4ppm | FFS |
| Purpose #3-1 of the clock | D2R Tx on backscattering w/ frequency shit | device 1/2a | < a few MHz | 1uW to 10s uW | 10^4 ~ 10^5 ppm | < 10^4 ppm | FFS |
| Purpose #3-2 of the clock | D2R Tx on carrier frequency generation | device 2b | Same as carrier frequency | 10s ~ 100 uW | 10^3~ 10^4 ppm | < 10^3 ppm | FFS |
| Note1: clock #1 and clock #2 may be the same clock depending on the device implementation Note2: clock #2 and clock #3-1 may be the same clock depending on the device implementation |  |  |  |  |  |  |  |

Proposal 7: Impacts on image/harmonics which are caused by frequency shift should be studied in RAN4.

Proposal 8: Study both DSB and SSB modulation for D2R considering the feasibility on SSB modulation for D2R, spectrum efficiency, impact on SNR.

Proposal 9: Discuss whether A-IoT device is capable of different spectrum for R2D, carrier wave and D2R depending on the deployment scenario/topology.

- Discuss how A-IoT device can identify the frequency location of initial R2D reception, e.g., sync-raster concept in NR.

Reference

[1] RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#103, Mar. 2024.

[2] R1-2406932, Study on evaluation assumptions for Ambient IoT, NTT DOCOMO, INC.