3GPP TSG RAN WG1 #118bis   		                                                                 R1-2408856

Hefei, China, October 14th – 18th, 2024

Agenda item:	9.5.2

Source: 	Qualcomm Incorporated

Title: 	On-demand SIB1 procedure

Document for:	Discussion/Decision

# Introduction

This paper provides our views on the main aspects that should be discussed as part of the following objective in [1] to support on-demand SIB1 (OD-SIB1) Case 2 for idle/inactive UEs:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

# Identification of NES cell with OD-SIB1

In previous RAN1 meetings, the following agreement was made regarding the signaling method for UE to identify the NES cell with on-demand SIB1.

RAN1-116b Agreement RAN1 to further study UE identification of NES cell with on-demand SIB1 based on one, both, or combination of the following options: - Option 1: By WUS configuration - Option 2: By PBCH payload of NES cell RAN1-117 Agreement - For further study of on-demand SIB1 in idle/inactive mode, it is assumed that always-on SSB is transmitted on the NES cell with on-demand SIB1.

As discussed in our last contribution [2] for RAN1-118, a cell may have 3 modes regarding transmission of SIB1 for a Rel19 UE supporting OD-SIB1: periodic SIB1, SIB1-less, or OD-SIB1, and a NES cell with OD-SIB1 operation shall be identified as a SIB1-less cell by a legacy UE to minimize the impact to the legacy UE. Thus, a NES cell with OD-SIB1 operation shall not have any cell-defined SSB and all its SSBs shall be Non-Cell-Defined SSBs (i.e. with KSSB>=24 in FR1 and KSSB>=12 in FR2). Since a cell with only NCD-SSB(s) can be either a cell with OD-SIB1 or a cell with SIB1-less for a Rel19 UE, additional information is required to further differentiate between these two modes.

One option can be to use the reserved value of KSSB (i.e. KSSB =30 in FR1 and KSSB=14 in FR2) to indicate a cell with OD-SIB1 for a Rel19 UE. If this option is used, the MIB parameter “pdcch-ConfigSIB1” are also reserved and does not provide assisting information for legacy UEs to search for a CD-SSB, which may not be desirable in some cases. Therefore, for a NES cell with OD-SIB1, we suggest allowing the KSSB to be set within the range for the NCD-SSB instead of being limited only to the reserved value.

Observation 1: There are following pros and cons for using the reserved value of KSSB (i.e. KSSB =30 in FR1 and KSSB=14 in FR2) to indicate a NES cell with OD-SIB1:

- Pros: a Rel19 UE can identify a NES cell with OD-SIB1 solely based on SSB(s) of the cell.

- Cons: pdcch-ConfigSIB1 is reserved and does not provide assistant information for a UE to search for a CD-SSB.

Since a Rel19 UE must have acquired UL-WUS configuration for a NES cell from a cellA before it can access the NES cell, the Rel19 UE can further identify a cell with only NCD-SSB(s) to be with OD-SIB1 or with SIB1-less based on the acquired UL-WUS configuration, i.e. the cell is with OD-SIB1 if its (carrier frequency, PCI) is indicated in the UL-WUS configuration, otherwise it is with SIB1-less.

Observation 2: The SIB1 transmission mode of a cell can be identified by a Rel19 UE as follows:

- Periodic SIB1, if the cell has a CD-SSB (i.e. with KSSB<24 in FR1 and KSSB<12 in FR2);

- OD-SIB1, if all SSB(s) of the cell are NCD-SSB(s) and the cell is associated with the UL-WUS configuration received by the UE via its carrier frequency and PCI;

- SIB1-less, otherwise.

Proposal 1: UE identifies a cell with OD-SIB1 based on both PBCH payload KSSB and the UL-WUS configuration received by the UE.

- That is, a cell is with OD-SIB1 if all SSB(s) of the cell are NCD-SSB(s) (i.e. with KSSB>=24 in FR1 and KSSB>=12 in FR2), and the cell is associated with the UL-WUS configuration.

We can see that to identify whether a cell is with OD-SIB1, UE needs to check both PBCH payload KSSB of SSB(s) and the received UL-WUS config. In some cases, a cell supporting OD-SIB1 may want to switch between periodic SIB1 mode and OD-SIB1 mode, e.g. based on traffic load in a semi-static manner. To achieve the switch, the cell can change the KSSB setting of SSB(s) based on the target SIB1 transmission mode without the need to coordinate with the cellA to change the UL-WUS configuration.

Observation 3: If a cell wants to switch its transmission mode between periodic SIB1 and OD-SIB1 semi-statically, e.g. based on traffic load, it can update the value of KSSB accordingly without the change of the UL-WUS configuration.

# UL-WUS Design and Configuration

## UL-WUS Design

Regarding design of UL-WUS, the following agreements were made in previous meetings:

RAN1-118 Agreement - For further work on on-demand SIB1 in idle/inactive mode related to dedicated PRACH resource usage, RAN1 to study the following options: - Option 1 (shared RO): The dedicated WUS resource shares the same PRACH resource pool with PRACH resource for other usages. - Option 2 (separated RO): The dedicated WUS resource uses an independent RACH resource pool with PRACH resource for other usages.

If Option 1 with shared RO is used, the NES cell only needs to monitor a single set of PRACH occasions, shared across different use cases, including requesting OD-SIB1 and initial access. These use cases can be distinguished based on the PRACH preamble index used for PRACH transmission. Conversely, if Option 2 with separated RO is implemented, the NES cell must monitor an additional set of PRACH occasions specifically for requesting OD-SIB1, on top of the set for other use cases like initial access. As a result, Option 2 would lead to higher power consumption for the NES cell compared to Option 1. We propose supporting Option 1 with shared RO but remain open to discussing any potential benefits of Option 2 if they exist.

Observation 4: The NES cell consumes more power with Option 2 with separated RO than Option 1 with shared RO.

Proposal 2: Support Option 1 with shared RO and dedicated PRACH preamble for UL-WUS.

- FFS: benefits of Option2 with separated RO.

## UL-WUS Configuration

Regarding UL-WUS configuration, the following agreement was made in previous RAN1 meetings.

RAN1-117 Agreement - For further study of on-demand SIB1 in idle/inactive mode, use the following Table I (from R1-2405106, Ericsson) as a starting point to discuss the required parameters/contents inside the UL-WUS configuration.

Regarding the required parameters/contents inside the UL-WUS configuration, we think that the proposed parameter table from the last RAN1-117 meeting shall also include the parameter “SSB-positionsInBurst”, which indicates the bitmap of the actual transmitted SSBs. This parameter is needed for UE to determine the association between transmitted SSBs and PRACH occasions, and thus is required for UL-WUS transmission. In addition, the reference SCS for “msg1-FrequencyStart” and “offsetToCarrier” is missing from the parameter table for UL-WUS config. Though there is a parameter “Msg1-SubcarrierSpacing” listed in the parameter table, it indicates the SCS for UL-WUS signal, which can be a value among (1.25, 7.5, 15, 30, 60, 120) KHz, while the “msg1-FrequencyStart” and “offsetToCarrier” may use a different SCS value among (15, 30, 60, 120) KHz. Furthermore, the reference frequency point for the parameter “msg1-FrequencyStart” needs to be clarified. As shown in TS38.211, parameter “msg1-FrequencyStart” refers to the offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0 of the UL BWP. Since UL BWP is not defined in UL-WUS config, parameter “msg1-FrequencyStart” in UL-WUS config may need to be clarified as being with respective to the first RB of the UL carrier, which is the CRB located at offsetToCarrier from pointA.

Proposal 3: Add parameter “SSB-positionInBurst” and “ULSubCarrierSpacing” to the parameter table for the UL-WUS configuration.

- Parameter “ULSubCarrierSpacing” indicates reference SCS for parameters “Msg1-FrequencyStart” and “offsetToCarrier”.

- Clarify the reference frequency point for “Msg1-FrequencyStart” as the first RB of the UL carrier (i.e. the CRB with offsetToCarrier from pointA).

# Transmission of On-demand SIB1

In last RAN1-118 meeting, the following agreement was made on the reference time point for PDCCH monitoring window.

RAN1-118 Agreement - At least for Case-2, on reference time point to determine the window starting time, RAN1 to down select from the following three options: - Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission - FFS: Definition of RAR reception time - Option 2: The reference time point is defined based on the UL-WUS transmission time - Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission

Selecting a reference time point for the PDCCH monitoring window should not be based on merely the UL-WUS transmission time. This is because the UE monitors the PDCCH window for OD-SIB1 only if an RAR is received within the RAR window following UL-WUS transmission; otherwise, the UE will retransmit UL-WUS. Hence, Option 2, which relies solely on the UL-WUS transmission time, should be disregarded.

Observation 5: UE monitors the PDCCH window only if an RAR is successfully received within the RAR window following UL-WUS transmission, and thus the reference time point for the PDCCH monitoring window should not be solely based on the UL-WUS transmission time as Option 2.

In Fig.1, we compared Option1 with reference time point based on received RAR and Option 3 based on the RAR window. As shown in Fig.1, upon reception of UL-WUS, the NES cell may transmit multiple RARs within the RAR window following the received UL-WUS to increase the reliability of RAR reception at UE side, but the NES cell is not aware which one of the multiple RARs is successfully received by the requesting UE. Using Option 1 with same offset across multiple RARs means the PDCCH monitoring window's start time at the UE-side varies based on the successfully received RAR, which is unknown to the NES cell and requires the NES cell to transmit OD-SIB1 in a larger window. In contrast, Option 3 allows both gNB and UE to have a synchronized view of the PDCCH window since both know the exact RAR window. Although Option 1 could potentially reduce SIB1 acquisition latency by starting the PDCCH window immediately after receiving the RAR, this latency reduction is minor, only up to the RAR window size with a max value of 10ms. Meanwhile, Option 1 may lead to higher power consumption for the NES cell as it needs to transmit OD-SIB1 over a larger window to account for the varying starting reception time at UE-side.

Observation 6:

- With Option 3 for reference time point based on the RAR window, both the NES cell and UE have a synchronized view of the PDCCH window.

- With Option1 for reference time point based on received RAR, the NES cell may consume higher power than Option 3 to transmit OD-SIB1 over a larger window to account for the varying starting reception time based on the received RAR at UE-side.

  - Although Option 1 may reduce the SIB1 acquisition latency, the latency reduction is minor, only up to the RAR window size with a max value of 10ms.

Proposal 4: Reference time point for PDCCH monitoring window is the end of the RAR window wherein UE successfully received a RAR.

Fig.1 Different options of reference time point for PDCCH window.

The following agreements were made in last RAN1-118 meeting on configuration of PDCCH monitoring window.

RAN1-118 Agreement - For Case-2, on how the search space zero configuration is provided, RAN1 to down select from the following options: - Option 1: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell - Option 2: searchSpaceZero for on-demand SIB1 is provided from UL WUS configuration - Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS. - Combination of multiple options is not precluded. RAN1-118 Agreement - At least for Case-2, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration

As shown in section 2, for a NES cell with OD-SIB1, its SSB(s) shall be set as NCD-SSB, wherein the KSSB and “pdcch-ConfigSIB1” do not indicate the PDCCH configuration for SIB1 including the searchSpaceZero. Therefore, Option 1 with searchSpaceZero for OD-SIB1 provided from MIB on the NES cell can be eliminated.

To support dynamic indication of searchSpaceZero and/or (starting time, duration) of PDCCH window via RAR, more specification effort is required to introduce a new RAR format, and at the same time, in our opinion, semi-static configuration of these parameters via UL-WUS configuration should be sufficient for supporting OD-SIB1.  In Fig.2, an illustration of PDCCH window with semi-static or fixed parameters is illustrated, wherein the same set of parameter values for (searchSpaceZero, starting offset of PDCCH window, duration of PDCCH window) is used for different PDCCH windows triggered by UL-WUS transmitted at different PRACH occasions (ROs). In case of multi-UEs requesting OD-SIB1, the PDCCH monitoring windows for different UEs may overlap depending on the locations of requesting ROs, and within the overlapping time interval, the NES cell can transmit SIB1s associated with different SSBs for different UEs without awareness of the requesting UEs.

- Usually, the PDCCH window duration should be set long enough to allow UEs with low SNR (for example, those at the edge of a cell) to receive and combine multiple SIB1 repetitions for successful decoding. The maximum number of required SIB1 repetitions may depend on static factors like cell size; hence, configuring PDCCH window duration semi-statically via UL-WUS configuration is advisable.

  - According to TS 38.331, SIB1 is transmitted every 160ms with a default repetition interval within 160ms based on the searchSpaceZero setting (e.g., 20ms for RMSI MUX pattern 1). The gNB schedules actual SIB1 repetitions within the 160ms using Type-0 PDCCH.

- For the starting offset of the PDCCH window, using a fixed value, e.g. based on the maximum PDSCH processing latency for RAR, should be adequate.

- Regarding parameter searchSpaceZero, as shown in Fig.2, with semi-statically configured searchSpaceZero via UL-WUS configuration, the time interval from the reference time point to the 1st corresponding PDCCH occasion (i.e. the one mapped to the same associated SSB as the transmitted UL-WUS) may vary with the location of triggering RO. Though the dynamic indication of searchSpaceZero via RAR may reduce SIB1 acquisition latency, e.g. by selecting searchSpaceZero dynamically based on triggering RO location to minimize the time interval from the reference time point to the 1st corresponding PDCCH occasion, the latency reduction is minor, up to one SIB1 transmission repetition period (i.e. 20ms for RMSI MUX pattern 1). In addition, the SIB1 acquisition latency itself is not a critical metric for UE’s performance during idle state. Thus, we do not see a strong need to support dynamic indication of searchSpaceZero via RAR.

Fig.2 PDCCH window with semi-static (or fixed) parameters

Observation 7:

- For supporting OD-SIB1, a fixed starting offset and a semi-statically configured duration for a PDCCH window should be sufficient.

  - e.g. a configurable duration to cover a target number of SIB1 repetitions based on cell size.

  - e.g. a fixed starting offset to account for PDSCH processing latency for RAR.

- Dynamic indication of (starting time, duration) of PDCCH window and/or searchSpaceZero via RAR does not clearly provide benefit over semi-static configuration, despite requiring more specification effort.

Proposal 5: SearchSpaceZero and duration of PDCCH monitoring window are indicated by UL-WUS configuration.

- Starting offset of PDCCH monitoring window is specified in spec (not configured), e.g. based on max PDSCH processing latency for RAR.

The TS 38.331 has the following specification to define SIB1 periodicity and SIB1 transmission repetition periodicity for the legacy periodic SIB1 transmission:

- “… the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;”

While the SIB1 periodicity is fixed to be 160ms, the SIB1 transmission repetition periodicity can be varied. By default, the UE assumes the repetition periodicity of 20ms for SSB/RMSI mux pattern 1. For on-demand SIB1, one question is whether SIB1 transmission repetition is supported or not? From our perspective, repetition should be supported for on-demand SIB1 transmission to ensure proper coverage, and the existing legacy SIB1 transmission repetition can be reused.

Proposal 6: SIB1 transmission repetition for on-demand SIB can be supported similar to the legacy SIB1 transmission repetition.

# Conclusion

The contribution has discussed our views on on-demand SIB1 operation for idle/inactive UEs. In particular, we make the following observations and proposals:

Observation 1: There are following pros and cons for using the reserved value of KSSB (i.e. KSSB =30 in FR1 and KSSB=14 in FR2) to indicate a NES cell with OD-SIB1:

- Pros: a Rel19 UE can identify a NES cell with OD-SIB1 solely based on SSB(s) of the cell.

- Cons: pdcch-ConfigSIB1 is reserved and does not provide assistant information for a UE to search for a CD-SSB.

Observation 2: The SIB1 transmission mode of a cell can be identified by a Rel19 UE as follows:

- Periodic SIB1, if the cell has a CD-SSB (i.e. with KSSB<24 in FR1 and KSSB<12 in FR2);

- OD-SIB1, if all SSB(s) of the cell are NCD-SSB(s) and the cell is associated with the UL-WUS configuration received by the UE via its carrier frequency and PCI;

- SIB1-less, otherwise.

Observation 3: If a cell wants to switch its transmission mode between periodic SIB1 and OD-SIB1 semi-statically, e.g. based on traffic load, it can update the value of KSSB accordingly without the change of the UL-WUS configuration.

Observation 4: The NES cell consumes more power with Option 2 with separated RO than Option 1 with shared RO.

Observation 5: UE monitors the PDCCH window only if an RAR is successfully received within the RAR window following UL-WUS transmission, and thus the reference time point for the PDCCH monitoring window should not be solely based on the UL-WUS transmission time as Option 2.

Observation 6:

- With Option 3 for reference time point based on the RAR window, both the NES cell and UE have a synchronized view of the PDCCH window.

- With Option1 for reference time point based on received RAR, the NES cell may consume higher power than Option 3 to transmit OD-SIB1 over a larger window to account for the varying starting reception time based on the received RAR at UE-side.

  - Although Option 1 may reduce the SIB1 acquisition latency, the latency reduction is minor, only up to the RAR window size with a max value of 10ms.

Observation 7:

- For supporting OD-SIB1, a fixed starting offset and a semi-statically configured duration for a PDCCH window should be sufficient.

  - e.g. a configurable duration to cover a target number of SIB1 repetitions based on cell size.

  - E.g. a fixed starting offset to account for PDSCH processing latency for RAR.

- Dynamic indication of (starting time, duration) of PDCCH window and/or searchSpaceZero via RAR does not clearly provide benefit over semi-static configuration, despite requiring more specification effort.

Observation 8: Unlike periodic SIB1 with SIB1s transmitted for all SSBs, the OD-SIB1, which transmits SIB1s only for the SSB linked to the triggering UL-WUS, could enable PDSCH slot aggregation to achieve the same SIB1 reception reliability with reduced PDCCH window duration.

Proposal 1: UE identifies a cell with OD-SIB1 based on both PBCH payload KSSB and the UL-WUS configuration received by the UE.

- That is, a cell is with OD-SIB1 if all SSB(s) of the cell are NCD-SSB(s) (i.e. with KSSB>=24 in FR1 and KSSB>=12 in FR2), and the cell is associated with the UL-WUS configuration.

Proposal 2: Support Option 1 with shared RO and dedicated PRACH preamble for UL-WUS.

FFS: benefits of Option2 with separated RO.

Proposal 3: Add parameter “SSB-positionInBurst” and “ULSubCarrierSpacing”  to the parameter table for the UL-WUS configuration.

- Parameter “ULSubCarrierSpacing” indicates reference SCS for parameters “Msg1-FrequencyStart” and “offsetToCarrier”.

- Clarify the reference frequency point for “Msg1-FrequencyStart” as the first RB of the UL carrier (i.e. the CRB with offsetToCarrier from pointA).

Proposal 4: Reference time point for PDCCH monitoring window is the end of the RAR window wherein UE successfully received a RAR.

Proposal 5: SearchSpaceZero and duration of PDCCH monitoring window are indicated by UL-WUS configuration.

- Starting offset of PDCCH monitoring window is specified in spec (not configured), e.g. based on max PDSCH processing latency for RAR.

Proposal 6: SIB1 transmission repetition for on-demand SIB can be supported similar to the legacy SIB1 transmission repetition.

# Reference

[1] RP-242354 Revised WID, Enhancements of network energy savings for NR

[2] R1-2407038 On-demand SIB1 procedure