3GPP TSG RAN WG1 Meeting #120	R1-2500142

Athens, Greece, February 17th – 21st, 2025

Title:	Draft reply LS on CSI-RS measurement with SBFD operation

Response to:	R1-2500019(R4-2420165)

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	ZTE, [RAN1]

To:	RAN4

Cc:	RAN2

Contact person:

Name:			Shuaihua Kou

E-mail address:	kou.shuaihua@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN1 thanks RAN4 for the LS R1-2500019(R4-2420165) on CSI-RS measurement with SBFD operation. RAN1 would like to provide the following answer.

For issue #1, the DL reception in the referred RAN1 agreement does not include CSI-RS resources used for measurement. From RAN1 perspective, if dynamically scheduled UL transmission overlaps with CSI-RS resources for measurement, the following collision handling rules are applied:

- Case a) if the CSI-RS resources are legacy UE-specific CSI-RS resources for measurement including L1-RSRP/SINR measurement and RLM/BFD/CBD, dynamically scheduled UL transmission is prioritized as legacy.

- Case b) if the CSI-RS resources are cell-specific periodic NZP CSI-RS resources for gNB-to-gNB CLI measurement or CSI-RS resources for RRM measurement, the CSI-RS resources for measurement are prioritized, similar as collision handling between SSB and dynamically scheduled UL transmission.

In addition, collision case 3 below does not include CSI-RS resources used for measurement. Similarly, the legacy collision handling rule is applied to legacy UE-specific CSI-RS resources for measurement, while cell-specific periodic NZP CSI-RS resources for gNB-to-gNB CLI measurement or CSI-RS resources for RRM measurement are prioritized.

Agreement If link direction indication is not supported nor provided for a SBFD symbol, collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs is an error case. - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and cell specific higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/1/2 CSS set - FFS: Additional cases

For issue #2, the listed agreements apply to the CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement.

2	Actions

To RAN4:

ACTION: 	RAN1 respectfully requests RAN4 to take the above information into account.

3	Dates of next TSG RAN1 meetings

RAN1#120bis	7th – 11th Apr 2025	Wuhan, China

RAN1#121	19th – 23rd Apr 2025	Malta