3GPP TSG RAN WG1 #120                                                                 R1-2500184

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.11.3

Source:	Spreadtrum, UNISOC, SGITG

Title:	Discussion on NR-NTN uplink capacity and throughput enhancement

Document for:	Discussion and decision

Introduction

The WID on Non-Terrestrial Networks (NTN) for NR Phase 3 is approved in [1].

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In this paper, we share our views on NR-NTN uplink capacity and throughput enhancement.

Potential specification aspects on OCC technique

In RAN1#119, the following agreement on OCC schemes for PUSCH had been achieved [2].

Agreement RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact. - RAN1 assumes no specification impact for CFO grouping - RAN1 does not pursue closed-loop frequency adjustment commands. - RAN1 assumes that RAN4 does not define new UE requirements for CFO.

In RAN1#117, the following agreement on the potential specification aspects on OCC techniques had been achieved [3].

Agreement RAN1 to at least further study the potential specification aspects on OCC techniques: - TBS calculation / Rate matching - UCI multiplexing - RV cycling across repetitions - Frequency hopping, e.g. intra /inter slot - OCC indication/configuration - Power control - FFS others aspects

In this section, we share our views on the potential specification aspects on inter-slot time-domain OCC with PUSCH repetition Type A.

UCI multiplexing

For UCI multiplexing, some important principles include:

- No looking forward operation. For example, UL channel in slot n cannot be dropped or fallback to non-OCC due to UCI multiplexing in slots after slot n.  PUCCH in slot n cannot be multiplexed into PUSCH repetitions in slots before slot n.

- Not support UCI postpone. E.g., PUCCH overlapping with PUSCH in OCC group 1, it can only multiplex on PUSCH repetitions in OCC group 1. The UCI cannot be multiplexed in PUSCH repetitions in OCC group 2 or after.

PUCCH without repetition

In RAN1#119, the following agreement on UCI multiplexing had been achieved [2].

Agreement If PUCCH without repetition overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group, the following options to be considered: - Option 1: UCI is dropped - FFS: whether all UCI is dropped - Option 2: UCI is transmitted on PUCCH, and all PUSCH repetitions within the OCC group are dropped. - Option 3: UCI is multiplexed on PUSCH with inter-slot OCC - Option 3-a: UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC - FFS: which OCC group - Option 3-b: UCI is multiplexed on PUSCH and OCC is not applied within the OCC group - Option 3-c: UCI is multiplexed on PUSCH and OCC is not applied within the PUSCH repetitions Note: combination of the above can be considered FFS Details timeline of PUCCH and PUSCH FFS: handling of PUCCH with repetition FFS: handling of different UCI types

For inter-slot time-domain OCC with PUSCH repetition type A, UCI multiplexing needs some enhancements. The issues of UCI multiplexing include:

- Handling of different UCI types

- How to do UCI multiplexing if a PUCCH overlaps with PUSCH repetition type A in a slot

- Which slot(s) that PUCCH located can be multiplexed to PUSCH and timeline requirement

The UCI on PUCCH can be HARQ-ACK, CSI and SR. Same handling applies to HARQ-ACK and/or CSI w/o SR on PUCCH. However, if there is only SR on PUCCH, the legacy rule applies, that is Option 1. The following section provides other UCI types on PUCCH.

Proposal 1: When there is only SR on PUCCH without repetition, SR is dropping (Option 1), as same as legacy.

Firstly, all types of UCI multiplexing on PUSCH are defined per slot in legacy, e.g. when PUCCH overlapping with PUSCH, the UCI is multiplexed on PUSCH and PUCCH is not transmitted. However, if UCI multiplexing is per-slot basis, it breaks the orthogonality of OCC among the repetitions in PUSCH OCC group. One way is to multiplex UCI on all repetitions of PUSCH OCC group if there is UCI in one slot. Such as an example in Figure 2, PUCCH1 with HARQ-ACK overlaps with PUSCH Rep#1, the HARQ-ACK can be piggyback on PUSCH Rep#1 only in legacy. When time-domain OCC with PUSCH applies, the HARQ-ACK can be multiplexed on all repetitions of one PUSCH OCC group in which PUCCH overlaps with.

Figure 1: Example of a PUCCH overlaps with PUSCH repetition type A in a slot

Another issue for UCI multiplexing is which slot(s) that PUCCH located can be multiplexed to PUSCH and timeline requirement. If PUCCH overlaps with PUSCH in a slot expect the first repetition in a PUSCH OCC group, whether or not the UCI of PUCCH can be multiplexed on all slots. If allowed, timeline needs to discuss, such as if at least one DCI exists among the overlapping PUCCH and PUSCH, or CSI reference resource. As shown in Figure 3, CSI on PUCCH2 overlaps with PUSCH Rep#2. If it can be multiplexed on PUSCH OCC group 0, which slot that CSI reference resource is determined, according to CSI reporting slot n+1 or slot n. In legacy, slot n+1 is the CSI reporting slot, but in this moment, slot n is the actual first CSI reporting slot. On one hand, if legacy slot n+1 reuses, the time for CSI processing is huge problem. On the other hand, if new slot n applies to determine CSI reference resource, dynamic determination is a big challenge for UE implementation and timeline processing. Frankly speaking, it is unbearable. Our proposal is only the PUCCH overlaps with PUSCH in the first repetition of a PUSCH OCC group, the UCI of PUCCH can be multiplexed on all repetitions of the PUSCH OCC group. Otherwise, the UCI on PUCCH is not transmitted/dropped.

Figure 2: Example of which slot(s) that PUCCH located can be multiplexed to PUSCH and timeline requirement.

Option 1 only and Option 2 only lead to a lot of necessary UCI dropping or PUSCH dropping. Such as UCI overlapping on the first PUSCH of an OCC group, it can be multiplexed on PUSCH with little effort, and reuse the legacy rule as much as possible. For Option 3-b and 3-c, UCI multiplexing is per-slot basis, it breaks the orthogonality of OCC among the repetitions in an OCC group, which lead to performance loss for the other UEs in one UE group. The PUSCH from other UEs cannot be decodes correctly, so it is even worse than other options.

Proposal 2: Only when HARQ-ACK and or CSI on PUCCH without repetition overlaps with PUSCH in the first repetition of a OCC group, the UCI of PUCCH can be multiplexed on the PUSCH OCC group(Option 3-a), otherwise the UCI on PUCCH is dropped (Option 1).

PUCCH with repetition

In the current specification, when a PUSCH w/o repetition overlapping with a PUCCH with repetitions in at least one slot, PUSCH is dropped in the slot.

When PUSCH repetitions with inter-slot OCC, it can be discussed separately for cases in the following table, which provides some cases of the first overlapping slot of PUSCH and PUCCH. They can lead to different UCI multiplexing operations.

| Overlapping cases | Description for the first overlapping slot of PUSCH and PUCCH |
| --- | --- |
| Case 1 | it is the first slot of an OCC group |
| Case 2 | it is not first slot of an OCC group, and PUCCH overlaps only one OCC group |
| Case 3 | it is not first slot of an OCC group, and PUCCH overlaps with more than one OCC group |

For case 1, PUCCH starts earlier or same as PUSCH, or PUCCH overlaps with PUSCH in the later OCC group. The whole PUSCH repetitions in the overlapping OCC group can be dropped, without any timeline issue, because the first overlapping slot is the first one in an OCC group.

Figure 3: Case 1

For case 2 and case 3, since there is no looking forward capability, UE do not know there will be an overlapping in later slot, it is impossible to drop PUSCH before the collision. So the issue is whether or not partial dropping of OCC group is allowed. If it is allowed, PUSCH can be dropped from the slot overlapping with PUCCH repetition. And there would be some leftover PUSCH repetitions in the OCC group.

Figure4: Case 2

Figure 5: Case 3

Proposal 3: For PUCCH with repetitions, the following cases of first overlapping slot and number of overlapping OCC groups of PUSCH and PUCCH can be considered:

- Case 1: it is the first slot of an OCC group

- Case 2: it is not first slot of an OCC group, and PUCCH overlaps only one OCC group

- Case 3: it is not first slot of an OCC group, and PUCCH overlaps with more than one OCC group

A-CSI report on PUSCH

Another issue is A-CSI triggered by DCI on PUSCH. In the current spec, the A-CSI is only multiplexed on the first repetition. As same as the first issue, the A-CSI can be multiplexed on all repetitions of the first PUSCH OCC group, instead of the first one.

Proposal 4: When a DCI triggers A-CSI report on PUSCH, the A-CSI multiplex on all repetitions of the first PUSCH OCC group.

RV cycling across repetitions

In RAN1#119, the following agreement on RV cycling for OCC with PUSCH had been achieved.

A Agreement For RV cycling for OCC with DG-PUSCH, the following are considered: - Option 1: RV cycling is used across OCC groups - Note 1: RV cycling is applied when the number of repetitions is greater than the OCC length - Option 2: Fixed RV is used across OCC groups - Option 3: For OCC length 2, fixed RV is used across two OCC groups, RV cycling is used across groups of two OCC groups

In legacy, RV of nth PUSCH repetition is determined by RV ID in DCI for dynamic grant, or RV sequence configured for a CG-PUSCH. For inter-slot time-domain OCC with PUSCH repetition Type A, to ensure orthogonality, repetitions in a PUSCH OCC group use same RV and RV cycling can be across PUSCH OCC groups. RV cycling across OCC groups can also provide the additional gain for HARQ combination. It is proposed to also enable RV cycling across OCC groups. Of course, DG-PUSCH and CG-PUSCH RV cycling are different according to current spec, which can be applied separately only across OCC groups, instead of PUSCH occasions. So option 1 is applied.

For option 3, it can be dropped. Although its intention is to have OCC2 and OCC4 in a UE grouping. However, only RV cycling keeps same within two OCC groups is not enough. Basically, all the designs of UCI multiplexing and frequency hopping should also be combined together with RV. RV cycling only is not complete solution for OCC2 and OCC4 multiplexing. Furthermore, it is too complex to have two level of OCC length. One is for UCI multiplexing and frequency hopping, and other for RV cycling. Thus, Option 3 is not a complete solution, which can be removed.

Actually, this RV determination of option 1 is similar as RV of TBoMS with N*K, where N is TB over N slots, K is repetition number. The RV of TBoMS remains same in each N slots, and do RV cycling across other N slots. Then, same mechanism applies. The following table defined in 38.214 gives RV ID for dynamic PUSCH. For time-domain OCC with PUSCH repetition type A, ((n-(n mod M))/M) mod 4 can be used to determine the RV ID, where M is OCC length.

Table 6.1.2.1-2: Redundancy version for PUSCH transmission rvid indicated by the DCI scheduling the PUSCH; rvid to be applied to nth transmission occasion (repetition Type A) or TB processing over multiple slots) or nth actual repetition (repetition Type B). ((n-(n mod N))/N) mod 4 = 0; ((n-(n mod N))/N) mod 4 = 1; ((n-(n mod N))/N) mod 4 = 2; ((n-(n mod N))/N) mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3.

Proposal 5: For RV cycling for OCC with DG-PUSCH, support option 1.

- RV cycling is used across OCC groups.

Frequency hopping

For inter-slot time-domain OCC with PUSCH repetition Type A, since the phase continuity may not be guaranteed after frequency hopping, it cannot support intra-slot frequency hopping or inter-slot frequency hopping, only inter-slot frequency hopping with enhancement is possible. Such the inter-slot FH with DMRS bundling. The frequency hopping only hops across different intervals. There is no hop among slots in one interval, the following starting RB of two hops are defined in 38.214. It can also used as a baseline.

For inter-slot time-domain OCC with PUSCH repetition Type A, frequency only hops across different PUSCH OCC group, and there is no hop among repetitions in one PUSCH OCC group. Furthermore, instead of additional frequency hopping interval, OCC length can be directly applied.

In case of inter-slot frequency hopping and when pusch-DMRS-Bundling is enabled, and when a PUSCH is not scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot is given by: RBstartnsμ=RBstartRBstart+RBoffsetmodNBWPsizensμNFH mod 2=0nsμNFH mod 2=1 where nsμ is the current slot number within a system radio frame, NFH is the value of the higher layer parameter pusch-FrequencyHopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops.

Proposal 6: For inter-slot time-domain OCC with PUSCH repetition Type A, frequency only hops across different PUSCH OCC group, and there is no hop among repetitions in one PUSCH OCC group.

- The RB start index in inter-slot frequency hopping with DMRS bundling can be used as baseline.

OCC indication/configuration

For PUSCH transmission with OCC, how UE determines OCC for PUSCH transmission need to be discussed. In the current specification, there are multiple resource scheduling methods for PUSCH transmission, as shown in Figure 4. Due to the current WID indicating that there is no need for any enhancement in the initial access process, we only need to consider the PUSCH resource scheduling methods of DCI dynamic scheduling and configured grant. Msg3 PUSCH and MsgA PUSCH are out of scope.

Figure 6: PUSCH scheduling in NR

The indication or configuration of OCC for PUSCH include OCC sequence index and OCC length.

- DCI dynamic scheduling

According to OCC length, OCC2 or OCC4 is configured or can be dynamic indicated need more discussion, and if it is dynamic indicated, how to support it in the DCI. Our view is dynamic indication of OCC length is more flexible, because one UE can be multiplexed with different UE with different repetition number. Considering the additional scheduling possibility, OCC length is indicated by DCI.

For PUSCH transmission with DCI dynamic scheduling, the following two OCC sequence index determination methods can be considered.

- Option 1: The OCC used for PUSCH transmission is implicitly determined by legacy parameter.

- Option 2: The OCC used for PUSCH transmission is explicitly indicated by the scheduling DCI.

Proposal 7: For PUSCH transmission with DCI dynamic scheduling, the following two OCC sequence index determination methods can be considered.

- Option 1: The OCC used for PUSCH transmission is implicitly determined by legacy parameter.

- Option 2: The OCC used for PUSCH transmission is explicitly indicated by the scheduling DCI.

Proposal 8: For PUSCH transmission with DCI dynamic scheduling, OCC length can be indicated by DCI.

- Configured grant

According to OCC length, OCC2 or OCC4 is configured for CG-PUSCH for both of Type-1 CG-PUSCH and Type-2 CG-PUSCH.

For configured grant type 1, the OCC used for PUSCH transmission can be configured through RRC. For configured grant type 2, the following two OCC determination methods can be considered.

- Option 1: The OCC used for PUSCH transmission is implicitly determined by legacy parameter.

- Option 2: The OCC used for PUSCH transmission is explicitly indicated by the scheduling DCI.

Proposal 9: For configured grant type 1, the OCC used for PUSCH transmission can be configured through RRC.

Proposal 10: For configured grant type 2, the following two OCC determination methods can be considered.

- Option 1: The OCC used for PUSCH transmission is implicitly determined by legacy parameter.

- Option 2: The OCC used for PUSCH transmission is explicitly indicated by the scheduling DCI.

Proposal 11: For CG-PUSCH transmission with inter-slot OCC, OCC length is configured by RRC signaling.

Dynamic UL grant for UE group

In current specification, one DCI can only grant one UE for UL/DL data transmission. When the PUSCH UL capacity is increased by means of OCC, the DL resource for PDCCH signaling to send UL grant to the UEs also increases. It will have a serious negative impact on DL capacity and throughput. To alleviate this issue, a direct way is to schedule the data transmission of a group of UEs through a DCI. In addition, the time-frequency resources for each UE within the UE group based on OCC transmission is same, which is beneficial for one DCI to schedule a group of UEs for data transmission.

Proposal 12: Dynamic UL grant for UE group can be considered for uplink capacity and throughput enhancement.

Conclusion

In this contribution, we provided views on Redcap positioning. In summary, we have following proposals:

Proposal 1: When there is only SR on PUCCH without repetition, SR is dropping (Option 1), as same as legacy.

Proposal 2: Only when HARQ-ACK and or CSI on PUCCH without repetition overlaps with PUSCH in the first repetition of a OCC group, the UCI of PUCCH can be multiplexed on the PUSCH OCC group(Option 3-a), otherwise the UCI on PUCCH is dropped (Option 1).

Proposal 3: For PUCCH with repetitions, the following cases of first overlapping slot and number of overlapping OCC groups of PUSCH and PUCCH can be considered:

- Case 1: it is the first slot of an OCC group

- Case 2: it is not first slot of an OCC group, and PUCCH overlaps only one OCC group

- Case 3: it is not first slot of an OCC group, and PUCCH overlaps with more than one OCC group

Proposal 4: When a DCI triggers A-CSI report on PUSCH, the A-CSI multiplex on all repetitions of the first PUSCH OCC group.

Proposal 5: For RV cycling for OCC with DG-PUSCH, support option 1.

- RV cycling is used across OCC groups.

Proposal 6: For inter-slot time-domain OCC with PUSCH repetition Type A, frequency only hops across different PUSCH OCC group, and there is no hop among repetitions in one PUSCH OCC group.

- The RB start index in inter-slot frequency hopping with DMRS bundling can be used as baseline.

Proposal 7: For PUSCH transmission with DCI dynamic scheduling, the following two OCC sequence index determination methods can be considered.

- Option 1: The OCC used for PUSCH transmission is implicitly determined by legacy parameter.

- Option 2: The OCC used for PUSCH transmission is explicitly indicated by the scheduling DCI.

Proposal 8: For PUSCH transmission with DCI dynamic scheduling, OCC length can be indicated by DCI.

Proposal 9: For configured grant type 1, the OCC used for PUSCH transmission can be configured through RRC.

Proposal 10: For configured grant type 2, the following two OCC determination methods can be considered.

- Option 1: The OCC used for PUSCH transmission is implicitly determined by legacy parameter.

- Option 2: The OCC used for PUSCH transmission is explicitly indicated by the scheduling DCI.

Proposal 11: For CG-PUSCH transmission with inter-slot OCC, OCC length is configured by RRC signaling.

Proposal 12: Dynamic UL grant for UE group can be considered for uplink capacity and throughput enhancement.

Reference

- RP-243300  Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

- 3GPP RAN1#119 Chairman’s Notes, Hefei, China, 14th – 18th October, 2024.

- 3GPP RAN1#117 Chairman’s Notes, Orlando, US, November 18th – 22nd, 2024.