3GPP TSG RAN WG1 #120		                                            R1-2500232

Athens, Greece, February 17th – 21st , 2025

Source:	CATT

Title:	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes

Agenda Item:	9.6.2

Document for:	Discussion and Decision

Introduction

In RAN1#119, the following agreements were achieved for studying low-power wake-up signal and receiver (LP-WUS/WUR) for NR [1]:

Agreement Confirm the following working assumption with the addition of the note at the bottom: Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion. Note: this does not intend to impact any agreements and working assumptions made in RAN1/RAN2 after the working assumption. Agreement At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams. Conclusion Do not support one-to-multiple mapping between the LP-WUS/LP-SS beams and the SSB beams. Conclusion No additional RAN1 specification impact specific to support multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams. Agreement For the offset value(s) between an LO and a reference PO/PF, down-select between - Option 1-1 - Configurability between Option 1-1 and Option 1-2 - Option 2B-1 Agreement At least for the 1:1 LO to PO mapping, - The maximum value of M for Option A or Option B with G=1 (if either of them is supported) is 4. Conclusion There is no RAN1 consensus to support additional codepoints indicating wake-up at least for 1:1 LO to PO mapping. For future meetings: Consider the following alternatives for UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}. - Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time. - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]} - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively. - Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later. Agreement Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB. Working Assumption If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration. Conclusion From RAN1 perspective, there is no consensus on the following proposal: UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if the UE has not received LP-WUS for a long time. For future meetings: Companies are encouraged to consider section 3.5 of R1-2410862 for further discussions in future meetings.

This paper discusses the aspects of LP-WUS monitoring procedures, UE behavior of LP-WUS detection error and the eDRX configuration for RRC_ IDLE/INACTIVE modes.

Discussion

As discussed in Rel-18 LP-WUS/WUR SI [2], significant UE power saving gain (up to more than 90%) can be achieved by using LP-WUS/WUR to trigger MR monitoring paging PDCCH compared with existing I-DRX operation (with and without PEI), if sufficient relaxation to MR RRM measurement is applied RRC_IDLE/INACTIVE modes UEs. Moreover, significant UE power saving gain and moderate paging latency increasing can be achieved if LP-WUS monitoring is restricted within existing PTW of eDRX. In this section, the procedure of LP-WUS monitoring and eDRX configuration are discussed.

The procedure of LP-WUS monitoring

Generally, the related procedures of LP-WUS monitoring can be described as following:

- Step1: UE capability of the supporting of LP-WUS is indicated during UE registration procedure through NAS procedure.

  - Step1-1: UE transfers its UE capability information upon receiving a UECapabilityEnquiry from the network.

- Step2: gNB broadcasts SIB1 and SIB-X with the LP-WUS configuration.

- Step3: UE determines the PF and PO from SIB1.

- Step4: UE determines its LO based on the parameters in the LP-WUS configuration of SIB-X.

- Step5: UE determines MOs in its LO based on parameters in the LP-WUS configuration of SIB-X.

- Step6: UE derives the codepoint of the LP-WUS sequences based on subgroup/UE-ID and the LP-WUS configuration.

- Step7: UE monitor the PO depends on the LP-WUS indication.

Figure 1: The procedures that UE monitors LP-WUS

Step1 and Step2 are higher layer NAS procedures. Step3 to Step7 are AS higher layer  procedures, in which UE monitors LP-WUS based on LP-WUS configuration and PO based on the LP-WUS indication. The details of each step will be discussed in the following sections.

UE capability of supporting LP-WUS monitoring

In Step1, the network should receive the report from UE whether UE supports LP-WUS and associated paging subgrouping during the NAS registration. If the UE indicates the support of LP-WUS and associated NR paging subgrouping, the UE may include the LP_WUS paging subgrouping  information in the Requested LP-WUSPS of ) the UE assistance information IE in the REGISTRATION REQUEST message. If the UE indicates support of NR paging subgrouping and the network supports the use of the LP-WUSPS for triggering the UE wakeup for paging indication, the network provides the UE with the Negotiated PEIPS assistance information, including the Paging subgroup ID, in the REGISTRATION ACCEPT message or the CONFIGURATION UPDATE COMMAND message. The Paging subgroup ID is used to determine the NR paging subgroup for paging the UE. The UE NAS layer shall indicate the Paging subgroup ID to the access stratum layer. The network shall store the Paging subgroup ID in the 5GMM context of the specific UE [3].

Proposal 1: UE capability of the supporting of LP-WUS can be indicated during UE registration procedure through NAS procedure.

In RAN1#118bis, 3 candidate values for the wake-up delay capability a UE reports was supported. How UE reports the UE capability of its wake-up delay should be discussed and the exact values of the 3 wake-up delays are FFS.

In Step1-1, the UE transfers its UE capability information upon receiving a UECapabilityEnquiry from the network in RRC signaling when UE is in RRC_CONNECTED mode, as shown in Figure 2.

Figure 2: Procedure of UE capability transfer [5]

The network initiates the procedure to a UE in RRC_CONNECTED mode when it needs UE radio access capability information. If UE would report the UE capability of the wakeup delay to the gNB for the configuration, the UE needs to transition from the RRC_ IDLE/INACTIVE modes to the RRC_CONNECTED mode. The UE capability of the wake-up delay can be included in UECapabilityInformation. After receiving UECapabilityInformation, the network shall store the wake-up delay capability in the 5GMM context of the UE. When the AMF tends to page a UE, the AMF initiates the Paging procedure by sending the PAGING message to the gNB as shown in Figure 3 [7].

Figure 3: Paging procedure from AMF to gNB [7]

Proposal 2: The UE capability of the wake-up delay can be reported through RRC signaling procedure while UE is required to transition from RRC_ IDLE/INACTIVE modes to RRC_CONNECTED mode.

Agreement—118bis Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state. - FFS the values - Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective In-RAN1#119 Consider the following alternatives for UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}. - Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time. - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]} - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively. - Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

In RAN1#118, the wake-up delay is defined as a minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. The minimum time gap is introduced considering at least the LP-WUS processing time, MR transition time for ramping up and the timing/frequency synchronization of MR.  Thus, the reported wake-up delay includes the required time for MR synchronization. The value range of the wakeup delay mainly depends on the transition time of different sleep states and the number of SSB needed for MR synchronization. If the DRX cycle is larger than 160ms, the MR receiver is not in-sync with the network.  At least 3 SSBs (i.e. 60ms) are needed for time/frequency synchronization of MR; otherwise, at least 1 SSB (i.e. 20ms) is needed. Thus, the number of SSB can be 3 for the UE in the deep sleep state and can be supported with 80 ms interval.

Proposal 3: For UE in the deep sleep state, 80ms can be supported for the wake-up delay capability report.

For ultra-deep sleep state, 400ms and 800ms are ramp-up times for UE in the ultra-deep sleep state and the number of SSBs required for MR synchronization is up to 5 as discussed in R18 LP-WUS SI [2]. Thus, the number of SSB needed for synchronization can be 3~5 for UE in the ultra-deep sleep state. Furthermore, two issues 1) whether the number of SSB for ultra-deep sleep state is configurable; 2) whether same or different number of SSB for 400ms and 800ms can be defined should be discussed. For simplicity, the number of SSB needed for synchronization can be 5 for both 400ms and 800ms. Thus, 500ms and 900ms can be considered for ultra-deep sleep state.

Proposal 4: For ultra-deep sleep state, {500ms and 900ms} can be considered for the wake-up delay capability report.

LP-WUS configuration information through system information and/or NAS signaling

For RRC_IDLE/INACTIVE modes UEs, the radio resource for the LP-WUS can be configured by Cell-specific signaling to all UEs in a cell by system information. In Rel-17, the resource allocation of PEI monitoring occasion in time domain for UE's PO is determined by a reference point and an symbol-level offset from the reference point to the start of the first PDCCH monitoring occasion. The reference point is the start of a reference frame determined by a frame-level offset from the start of the first PF of the PF(s) associated with the PEI monitoring occasions [4]. UE determines the first PDCCH MO for PEI monitoring occasions based on the frame-level offset and the symbol-level offset, which is broadcasted in SIB-1. If PEI and subgrouping information are configured to the UE, the paging group in the paging occasion can be divided into one or more subgroups. In the DCI format 2_7 field, each bit indicates the early paging indication of one paging subgroup of a PO. The paging subgroup can be either assigned by CN through NAS signaling or derived from UE_ID. A UE supporting CN assigned subgrouping can be assigned a subgroup ID (between 0 to 7) by AMF through NAS signaling. When the paging subgroup is derived from the UE_ID, the subgroup-ID is derived by UE-ID and the number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in SIB-1 or SIB-X.

In Step2, similar to the higher layer procedure of Rel-17 PEI configuration, the LP-WUS configuration information can be broadcasted in the system information.  Based on LP-WUS configuration, UE can derive the codepoint of the associated LP-WUS sequences and monitor the PO based on the LP-WUS wakeup indication. The exact RRC parameters used in each step will be discussed in the following.

Proposal 5: The parameters of the LP-WUS configuration are broadcasted and included in the SIB-1/SIB-X.

The determination of LP-WUS occasion

After receiving the system information of LP-WUS configuration, UE can determine its PO and PF based on legacy procedures as in 38.304 (i.e. Step3). The Step4  (i.e. UE determines its LO based on the parameters in the LP-WUS configuration) is discussed in this section.

The association of LO and PO

In  RAN1#118bis, the following options were down selected and Option 1 UEs  monitoring the same PO monitor the same LO was agreed:

Agreement—RAN1#117 It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options: - Option 1: UEs monitoring the same PO monitor the same LO - Option 2: UEs corresponding to different POs monitor the same LO - Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. - Combinations of the above options can be considered. Agreement—RAN1#118bis For the mapping between LO and PO, supports at least Option 1 (UEs monitoring the same PO monitor the same LO).

Option 3 was proposed  to reduce the number of subgroups indicated by LP-WUS within one MO. The determination of LO in Option 1 and Option 3 are analyzed in the  following.

In Option1, the LO location is derived by the offset value with one-to-one mapping between LO and PO. UE determines its PO and PF based on the UE-ID and the PO configuration parameters as described in Step 2. LP-WUS in the LO will indicate one codepoint corresponding to each of the subgroups or all the subgroups. The  location of the LO in time domain can be determined by a reference point and an offset from the start of the PF associated with the LO, as shown in Figure 4:

- The reference point is the start of a reference frame determined by the frame-level offset from the start of the PF associated with the LO, provided by an RRC parameter (e.g. lp-FrameOffset-r19) in SIB-X

- The offset is a symbol-level offset from the reference point to the start of the first LP-WUS MO of LO, provided by an RRC parameter (e.g.  lp_symbol-offset-r19) in SIB-X.

Figure 4: The determination of time location for the LO in Option1

Proposal 6: For Option1, the location of LO in time domain can be determined by a reference point and a configured offset.

In Option 3, UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. In other words, one LO indicates subset of subgroups. In this option, UE monitors the (i_LO+1)th LO, i_LO is the index of LO, where i_LO=0,1,…,X-1, X is the number of LO associated with a PO.

From UE’s perspective, the index of the LO can be determined by:

i_LO=(i_SG-r19 mod X),                                                           Equation 1

where,

- i_SG is the subgroup-ID, which can be either assigned by CN or formed based on UE_ID

- X can be provided by an RRC parameter (e.g. LO-NumPerPO) in SIBx.

Similar to Option 1, the location of the LO in time domain for a UE is determined by a reference point and an offset from the reference point to the start of the first LP-WUS MO of this LO:

- The reference point is the start of a reference frame determined by a frame-offset from the start of the PF associated with the first LO of associated LOs with the PO, provided by an RRC parameter (e.g. lp-FrameOffset-r19) in SIBx.

- The offset is a symbol-level offset from the reference point to the start of the first LP-WUS MO of a LO, provided by (i_LO+1)th value in an RRC parameter (e.g.  lp_symbol-offset-r19) in SIBx.

In where lp_symbol-offset-r19 is a set of LO locations with size X. For example, X=4 LOs are associated with one PO, i_LO=0, 1, 2, 3, as shown in Figure 5. The lp_symbol-offset-r19={symboloffset1, symboloffset2, symboloffset3, symboloffset4} is configured. Based on Equation 1, for UE with subgroup-ID = 5, the index of monitored LO is i_LO = (5 mod 4) =1. UE monitors the (i_LO+1)th  (i.e. 2nd )  LO and the corresponding symbol-level offset is the 2nd offset value in lp_symbol-offset-r19 i.e. symboloffset2.

Figure 5: The determination of time location for the LO in Option3

From above discussion, we can see that if the LP-WUS can support 32 subgroups, Option 3 has additional specification complexity without benefits comparing to that of Option 1.

Observation 1: If the LP-WUS can support 32 subgroups, Option 3 has additional specification complexity without benefits.

The offset value(s) between a LO and a reference PO/PF

In RAN1#119, it was agreed that down-select between Option 1-1, configurability between Option 1-1 and Option 1-2 and Option2B-1 for the offset value(s) between an LO and a reference PO/PF .

Agreement—RAN1#118bis For the offset value(s) between an LO and a reference PO/PF, consider the following options: - Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. - Option 1: gNB configures a single offset value. - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS. - Otherwise, - Option 1-1: the UE follows the legacy paging monitoring procedure. - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - Option 2: gNB configures one or multiple offset values. - For the same PO, each offset corresponds to a LO. - This does not preclude the possibility that the same LO may correspond to different POs with different offset values. - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. - FFS exactly how to choose the offset - Option 2B: - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay. - FFS exactly how to choose the offset - Otherwise, - Option 2B-1: the UE follows the legacy paging monitoring procedure. - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - FFS exactly how to choose the offset - FFS the UE shall monitor the LO associated with additional offset(s) Note: The PO mentioned above refers to legacy PO configured for the UE. Agreement—RAN1#119 For the offset value(s) between an LO and a reference PO/PF, down-select between - Option 1-1 - Configurability between Option 1-1 and Option 1-2 - Option 2B-1

Based on Step1, UE will feed back the one value of the wakeup delay in the UE capability. Since UE might report different values of offset in the UE capability, the network would determine the reported offset values of LP-WUS to configure a set of LOs for all UEs with the POs, paging subgroups, and the UE-specific offset associated with the LP-WUS. The network consideration of the LP-WUS offset associated with the PO would be the overhead of the LP-WUS and the signaling to indicate the LP-WUS configuration associated with a given UE in the subgroup. Once the offset value for a given UE subgroup is configured by the network, UE has to decide the type of sleeping (e.g., ultra deep sleep, deep sleep, light sleep, or micro sleep) based on the UE implementation in transition to turn on and off part or full of MR.

If multiple offsets values need to be configured, the network needs to reserve multiple LP-WUS resources for one UE. The higher layer signaling in indicating multiple offset values associated with one UE or UE subgroup would be complicated. UE would need to prepare to monitor the LP-WUS based on the largest offset value among multiple offset values. The additional offset values are the additional UE power consumption without any benefit. When UE uses the largest offset value of multiple offset values to determine the sleeping type and the time in the preparation of the LP-WUS monitoring, it is equivalent to have one offset value. Thus, Option 1 with single offset value should be the default setting of LP-WUS configuration.

Furthermore, when the gap between an LO and the PO associated with the offset is less than the wake-up delay of intended sleeping state (e.g., ultra deep sleep) that a UE supports, it is an UE implementation in determining the UE behavior in any sleep state satisfied the wakeup delay (deep sleep, light sleep or micro sleep) to wake up in time for the paging PDCCH monitoring at the configured PO after receiving the LP-WUS wakeup indication. In other words, UE does not need to monitor LP-WUS and UE follows the legacy paging monitoring procedure when offset is less than the wake-up delay a UE supports. Thus, Option1-1 should be supported.

Proposal 7: For the offset value(s) between an LO and a reference PO/PF, Option1 (i.e. gNB configures a single offset value). It would be an UE implementation issue on the sleeping state to satisfy the offset between LO and PO without any specified behavior.

Proposal 8: It is UE implementation in determining the sleep type of MR and the required time if the gap between an LO and the PO associated with the offset is configured by the network.

Based on current specification, the paging cycle can be 320ms, 640ms, 1280ms, 2560ms. Considering the ramp-up time i.e. at least 400ms for MR wake up from the ultra-deep sleep state to the active state of PDCCH monitoring of paging DCI in PO, it is possible that the configured offset value of the LP-WUS MO relative to the PO is longer than 400ms. Thus, the corresponding paging cycle should be configured at least larger than 320ms (i.e. 640ms, 1280ms, 2560ms). In other words, the configured offset should be within a paging cycle depends on network configuration.

Proposal 9: It is network implementation in configuring the gap between an LO and the PO associated with the offset.

LP-WUS MO configuration

In Step5, UE determines MOs in its LO based on parameters in the LP-WUS configuration of SIB-X. It was agreed in RAN1#118bis, when K larger than 1, the configuration of multiple LP-WUS MO can be down-selected between Option A and B, as shown in following agreement. In each option, the value of R, the detailed UE monitoring behavior and how the same LP-WUS information is transmitted in the R LP-WUS MOs are FFS.

Agreement—RAN1#118bis When K (K>1) LP-WUS MOs are configured for each beam in an LO, down select between - Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. A UE monitors all or some of the MO(s) within the K LP-WUS MOs. For each group of R LP-WUS MOs, the same LP-WUS information is transmitted. - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs. - M = 1 and M > 1 is supported. - FFS: detailed UE monitoring behavior - FFS R=1 or R>= 1 - Option B: K LP-WUS MOs for a beam are divided into G (G >= 1) groups of R*M (M >= 1) LP-WUS MOs. A UE monitors all or some of the MO(s) within one group of R*M LP-WUS MOs based on its subgroup ID. - Each group of R*M LP-WUS MOs is further divided into M groups of R LP-WUS MOs. - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted. - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs. - FFS: detailed UE monitoring behavior - M = 1 and M > 1 is supported. - FFS R=1 or R>=1 - Note: this achieves the same purpose as “Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. Working Assumption—RAN1#119 - If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

For Option A and Option B, the monitoring behavior of UE determines its MOs can be shown as following:

- Option A: UE monitors MO(s) for each beam in an LO. When UE detects a LP-WUS for the wakeup indication of its subgroup within the LO, the UE is not required to monitor the subsequent monitoring occasion(s) associated with the LO.

- Option B: UE monitor MO(s) in the (i_group+1)th group,  when the UE detects a LP-WUS for the wakeup indication of its subgroup within the LO, the UE is not required to monitor the subsequent monitoring occasion(s) associated with the LO. i group is the index of group, where i group =0,1,…,G-1, G is the number of groups of R*M (M >= 1)in the LO. The index of the group is determined by:

- i_ group =(i_SG-r19 mod G),

    - where, i_SG can be either assigned by CN as specified in clause 7.3.1 or formed based on UE_ID as specified in clause 7.3.2; G is provided by an RRC parameter e.g.  Group-NumPerLO in SIBX.

From above discussion, Option B has similar procedure to that of Option 3 for LO to PO mapping with additional specification complexity without benefits comparing with Option A.

Observation 2: If the LP-WUS can support 32 subgroups, Option B has additional specification complexity without benefits.

Thus, the working assumption achieved in RAN1#119 can be confirmed.

Proposal 10: Confirm the following working assumption:

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

The aspect of defining R is that whether the LP-WUS are transmitted R times or one out of R LP-WUS monitoring occasions. When R>1, for each group of R LP-WUS MOs, the same LP-WUS information would be transmitted by gNB.  UE is expected to monitor R LP-WUS MOs. If gNB would transmit LP-WUS on every LP-WUS MO, the LP-WUS repetition can be assumed by the UE to improve the detection probability by accumulating the LP-WUS received energies for LP-WUS coverage enhancement. Once the UE detects a LP-WUS to wake up itself, UE is not required to continue monitoring the remaining MO(s) associated with the same LO. If gNB does not transmit the LP-WUS on all R LP-WUS MOs, UE could not assume the LP-WUS transmission on any given LO and accumulates the received energy in the LP-WUS detection. Thus, there is no benefit in configured more than 1 MO (R>1) for UE power saving.

Proposal 11: If R>=1 is supported, gNB needs to transmit LP-WUS on every LP-WUS MO to allow UE accumulating the received energy for LP-WUS detection for coverage enhancement. Once the UE detects a LP-WUS to wake up at one of multiple MOs, the UE is not required to monitor the remaining MO(s) associated with the same LO.

The definition of LP-WUS MO should be further studied regardless of whether same or different information transmitted in LP-WUS MOs per beam. For example, as shown in Figure 6, the LP-WUS MO can be determined by configuring a set of locations of LP-WUS MOs, similar to the determination of LP-WUS LO for Option3. Once the UE determines time location of the first MO of LO, the time location of the subsequence LP-WUS MOs can be derived from the set of the locations.

Figure 6: The configuration of LP-WUS MOs

Proposal 12: The LP-WUS MO configuration should be studied.

The derivation of LP-WUS

A UE monitors LP-WUS on an LP-WUS MO. In Step6, UE derives a codepoint of the LP-WUS based on subgroup/UE-ID and the LP-WUS configuration. In RAN1#117, a LP-WUS indicating a codepoint value corresponding to one or all subgroup(s) was supported. To carry more information, the LP-WUS can be configured with multi-stage sequence design for the indication. The mapping between sequence and subgroup information can be configured by gNB. For example, a LP-WUS consists of two-stage Walsh sequence, in which the length of  first stage (Walsh1) and the second stage (Walsh2)  are 4-bit and 8-bit, respectively, are shown in Table 1.

Table 1：The 4-bit and 8-bit Walsh sequence

| Walsh1 |  |
| --- | --- |
| Walsh2 |  |

The mapping between subgrouping and sequence is shown in following Table 2.  In this way, a LP-WUS can support 32 subgroups. More details can be found in our AI-9.6.1 contribution [6].

Table 2: The mapping between subgourpID indication and Walsh index

| subgroupID indication | Two block Walsh sequence |
| --- | --- |
| 0 | [Walsh1-1 Walsh2-1] |
| 1 | [Walsh1-1 Walsh2-2] |
| … |  |

Proposal 13: LP-WUS can be can be supported by concatenated multi stage sequences.

UE behavior of LP-WUS detection errors

If the UE detects LP-WUS and indicates the wake up of UE MR in monitoring PO the UE MR would wake up and monitor the PO. When UE determines the detection failure of the LP-WUS, e.g. the decision threshold is smaller than the a predefined threshold, the UE would fall back to legacy mechanisms i.e. UE would wake up the MR to monitor PEI when UE supports PEI or UE monitors PDCCH at the PO if UE does not support PEI.

Proposal 14: The UE falls back to legacy mechanism when the LP-WUS is in detection errors.

eDRX configuration

In Rel-18 LP-WUS/WUR SI [2], significant UE power saving gain and moderate paging latency increase is observed if LP-WUS monitoring is restricted within existing PTW of eDRX with reusing of existing paging occasion determination. If the UE is configured with an extended DRX cycle no longer than 1024 radio frames, the DRX cycle equals to the configured eDRX cycle. UE monitors PO per configured eDRX cycle. Otherwise, a UE monitors POs per DRX cycle during a periodic Paging Time Window (PTW). For Rel-17, the PEI-O is associated with the DRX cycle i.e. the UE monitors one PEI occasion per DRX cycle. Similar, LO can also be associated with DRX cycle without additional specification impact.

When the LP-WUS monitoring is performed outside the PTW of eDRX, the paging latency reduction and moderate UE power saving gain are observed from Rel-18 LP-WUS/WUR study. The UE complexity would increase if the LP-WUS monitoring is outside the PTW window.  There is no significant benefit in the support of monitoring the LP-WUS and corresponding paging outside the existing PTW since the eDRX is introduced for the use case which is not sensitive to latency. Thus, latency reduction should not be the first priority for LP-WUS power saving technique.

Proposal 15: When eDRX is configured, LP-WUS is monitored within PTW.

QCL type of LP-WUS

Agreement Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB.

In RAN1#119, it was agreed that the each LP-WUS or LP-SS is QCLed with an SSB with ‘typeD’ with the same SSB, down-select between ‘typeA’ and ‘typeC’. Even the channel estimation and channel equalization operations are not expected in LP-WUS, Type A should be considered to make sure that the LP-WUS and the SSB are transmitted from the same gNB or TRP due to the same Doppler spread of LP-WUS and reference SSB is assumed. For QCL Type C, it does not guarantee the LP-WUS and SSB transmitted from the same gNB or TRP.

Proposal 16: For LP-WUS monitoring in RRC IDLE/INACTIVE mode, Type A is supported.

Conclusion

In this contribution, we discuss aspects of UE capability, configuration of LP-WUS, subgrouping, the LP-WUS monition procedures and LP-WUS detection for RRC_IDLE/INACTIVE modes and give the following observations and proposals:

Observation 1: If the LP-WUS can support 32 subgroups, Option 3 has additional specification complexity without benefits.

Observation 2: If the LP-WUS can support 32 subgroups, Option B has additional specification complexity without benefits.

Proposal 1: UE capability of the supporting of LP-WUS can be indicated during UE registration procedure through NAS procedure.

Proposal 2: The UE capability of the wake-up delay can be reported through RRC signaling procedure while UE is required to transition from RRC_ IDLE/INACTIVE modes to RRC_CONNECTED mode.

Proposal 3: For UE in the deep sleep state, 80ms can be supported for the wake-up delay capability report.

Proposal 4: For ultra-deep sleep state, {500ms and 900ms} can be considered for the wake-up delay capability report.

Proposal 5: The parameters of the LP-WUS configuration are broadcasted and included in the SIB-1/SIB-X.

Proposal 6: For Option1, the location of LO in time domain can be determined by a reference point and a configured offset.

Proposal 7: For the offset value(s) between an LO and a reference PO/PF, Option1 (i.e. gNB configures a single offset value).  It would be an UE implementation issue on the sleeping state to satisfy the offset between LO and PO without any specified behavior.

Proposal 8: It is UE implementation in determining the sleep type of MR and the required time if the gap between an LO and the PO associated with the offset is configured by the network.

Proposal 9: It is network implementation in configuring the gap between an LO and the PO associated with the offset.

Proposal 10: Confirm the following working assumption:

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Proposal 11: If R>=1 is supported, gNB needs to transmit LP-WUS on every LP-WUS MO to allow UE accumulating the received energy for LP-WUS detection for coverage enhancement. Once the UE detects a LP-WUS to wake up at one of multiple MOs, the UE is not required to monitor the remaining MO(s) associated with the same LO.

Proposal 12: The LP-WUS MO configuration should be studied.

Proposal 13: LP-WUS can be can be supported by concatenated multi- stage sequences.

Proposal 14: The UE falls back to legacy mechanism when the LP-WUS is in detection errors.

Proposal 15: When eDRX is configured, LP-WUS is monitored within PTW.

Proposal 16: For LP-WUS monitoring in RRC IDLE/INACTIVE mode, Type A is supported.

References

- Chairman notes, RAN1#119s meeting, Orlando, US, November 18th – 22nd, 2024.

- TR38.869, Study on low-power wake up signal and receiver for NR.

- TR24.501, Non-Access-Stratum (NAS) protocol for 5G System (5GS), Stage 3.

- TR38.304, NR User Equipment (UE) procedures in Idle mode and RRC Inactive state.

- TR 38.331, NR Radio Resource Control (RRC) protocol specification.

- R1- 2500231, Design of LP-WUS and LP-SS, CATT, Athens, Greece, February 17th – 21st, 2025.

- TR 38.413, NG-RAN NG Application Protocol (NGAP).