3GPP TSG RAN WG1 #120			R1-2500280

Athens, Greece, February 17th – 21st, 2025

Source: 	CMCC

Title:	Discussion on CSI enhancements

Agenda item:	9.2.2

Document for:	Discussion & Decision

# Introduction

Rel-19 NR MIMO evolution WI was revised in [1], we will discuss the following objectives in this contribution.

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1

  - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

  - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook

    - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

    - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

  - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

# CSI report for up to 128 CSI-RS ports

In Rel-19 MIMO WID, there are three aspects of enhancement on the CSI report for up to 128 CSI-RS ports:

- Type-I/II codebook refinement for up to 128 CSI-RS ports

- CRI based CSI report for hybrid beamforming

- SRS port grouping

## Type-I/II codebook refinement

### Extension of Type-I SP to <=32 ports

In RAN1#119 meeting [8], we have the following FL proposal on the extension of Type-I SP to <=32 ports:

Proposal 1.B: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to 16, 24, and 32 CSI-RS ports, for all applicable RI values with K=1 only, and without any further modification/enhancement of the sub-features pertinent to the Rel-19 Type-I SP design (including, e.g. the Rel-19 Type-I SP CBSR, soft scaling). - For the Rel-19 Type-I SP codebook, the support for 16, 24, and 32ports are 3 separate UE capabilities from the support for the previously agreed number of ports (48, 64, 128 ports) - The Rel-18 SD NES schemes applicable to Rel-15 Type-I SP codebooks are also applicable to the extension of the Rel-19 Type-I SP codebook to 16, 24, and 32 ports - FFS: whether to adopt the extended orthogonal set for the 2nd SD basis for Scheme-A, RI=2-4 and 16, 24, and 32 CSI-RS ports

We think extension to R19 Type-I codebook with <=32 ports will not bring extra spec impacts and no other UE implementation efforts with separate UE capability. It can be supported to further improve the performance of Type-I SP codebook with <=32 ports.

Also, originating from some UE vendors’ concern, 16, 24, and 32 ports are proposed as 3 separate UE capabilities for this feature, we think it can resolve some UE implementation burden.

As for the FFS point in the FL proposal, in current enhancement for the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, we have already agreed Scheme-B which supported SD basis vector independently selecting for different layers from a group of N1N2 orthogonal basis vectors. It can harvest the performance benefit of Rel-19 Type-I SP codebook refinement. So, we think it is not needed to adopt the extended orthogonal set for the 2nd SD basis for Scheme-A, RI=2-4 and 16, 24, and 32 CSI-RS ports.

Proposal 1: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to 16, 24, and 32 CSI-RS ports, for all applicable RI values with K=1 only, and without any further modification/enhancement of the sub-features pertinent to the Rel-19 Type-I SP design (including, e.g. the Rel-19 Type-I SP CBSR, soft scaling).

- For the Rel-19 Type-I SP codebook, the support for 16, 24, and 32 ports are 3 separate UE capabilities from the support for the previously agreed number of ports (48, 64, 128 ports)

## CRI based CSI report for hybrid beamforming

### Slot offset configuration for aperiodic CSI-RS resource

In RAN1#119 meeting [7], we have the following agreement on the slot offset configuration for aperiodic CSI-RS resource for the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports:

Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding aperiodic CSI-RS resource configuration, an RRC-configured resource-level slot offset (relative to the resource-set-level slot offset, using the same design as Rel-19 Type-I/II codebook refinement for 48, 64, and 128 ports) is supported for aperiodic CSI-RS resource set - FFS: The number of bits for indicating the resource-level slot offset (relative to the resource-set-level slot offset) for KS resources, including the value(s) of the slot offset - FFS: Whether, in addition, configuring an available slot offset for each CSI-RS resource within the aperiodic CSI-RS resource set - Note: “Available slot offset” is analogous to the Rel-17 SRS triggering offset enhancement

Basically, it has been approved that the same design as Rel-19 Type-I/II codebook refinement for 48, 64, and 128 ports will be reused here.

However, for the value(s) of the slot offset, maybe it can be further expanded. In the design for Rel-19 Type-I/II codebook refinement for 48, 64, and 128 ports, KS resources will be aggregated to comprise up to 128 CSI-RS ports and the value of KS is up to 4, and the per-resource higher-layer (RRC) can be configured to indicate (via 1-bit per resource) whether 1-slot offset relative to the legacy resource-set-level slot offset configuration should be assumed or not, which means that the up to 4 resources can only be located in one or two slots [5].

Agreement @RAN1#118 For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding NZP CSI-RS resource aggregation to attain 32 < P (or PCSI-RS) ≤ 128, for AP-CSI-RS where the K NZP CSI-RS resources are located in two consecutive slots, - Except for codebook refinement based on Rel-18 Type-II Doppler, introduce per-resource higher-layer (RRC) configuration to indicate (via 1-bit per resource) whether 1-slot offset relative to the legacy resource-set-level slot offset configuration should be assumed or not - For codebook refinement based on Rel-18 Type-II Doppler, introduce per-resource higher-layer (RRC) configuration to indicate (via 1-bit per resource) whether 1-slot offset relative to the resource group slot offset should be assumed or not

While for the feature of Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, the value of KS can be up to 8 [2]. In this sense, if the value of slot offset will be expanded, may be the up to 8 resources Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports can only be located in up to four slots. So, the value(s) of the slot offset can be {0, 1, 2, 3}, and the number of bits for indicating the resource-level slot offset (relative to the resource-set-level slot offset) for KS resources can be 2 bits.

Agreement @RAN1#116 For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, the supported combinations of KS value and the maximum number of ports per NZP CSI-RS resource are as follows: - FFS: UE capability on KS and the number of ports per resource KS; Maximum # ports per resource. 2, 3, 4; 32. 5, 6, 7, 8; 16.

Proposal 2: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding aperiodic CSI-RS resource configuration, the number of bits for indicating the resource-level slot offset (relative to the resource-set-level slot offset) for KS resources can be 2 bits, and the value(s) of the slot offset can be {0, 1, 2, 3}.

As for the concept of “Available slot offset”, i.e. configuring an available slot offset for each CSI-RS resource within the aperiodic CSI-RS resource set, considering in current network configuration, the number of downlink slots are much more than uplink slots, the motivation in introducing “Available slot offset” in Rel-17 SRS triggering offset enhancement is not valid anymore. Therefore, we prefer not to introduce configuring an available slot offset for each CSI-RS resource within the aperiodic CSI-RS resource set, unless clear benefit is identified.

Proposal 3: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding aperiodic CSI-RS resource configuration, not to introduce configuring an “available slot offset” for each CSI-RS resource within the aperiodic CSI-RS resource set, unless clear benefit is identified.

# UE reporting enhancement for CJT calibration

Rel-18 MIMO WI specified enhanced Type II codebook to support coherent joint transmission (CJT) assuming ideal time/frequency synchronization and backhaul among TRPs, i.e., it is assumed that the DL timings and carrier frequencies of multiple TRPs are perfectly aligned. However, in practical deployments of CJT, e.g., inter-site CJT, there will be time and frequency misalignments among TRPs due to non-ideal synchronization and backhaul.

Rel-19 MIMO WI will specify the CSI reporting enhancements for CJT deployments under non-ideal synchronization and backhaul, including inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH [1].

The previous RAN1 meetings have almost resolved all major issues on UE reporting enhancement for CJT calibration. In this section, we will discuss some remaining issues regarding the delay/frequency/phase offset reporting for CJT calibration.

## Delay offset reporting

A major issue discussed in the previous two meetings is the linkage of Rel-18 eType-II CJT CSI report and Rel-19 CJTC delay offset (DO) report. The motivation of such linkage is to achieve the PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset. The RAN1#118bis meeting agreed two cases where the two reports are separately or jointly triggered [6].

Agreement@118bis For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, confirm the following working assumptions as agreement with the following refinement: - When separately triggered, the delay offset value to be compensated is the latest reported delay offset (DO) whose reporting instance’s last symbol is before the first symbol of DCI triggering of the CJT CSI reporting - When jointly triggered, the delay offset value to be compensated is the reported delay offset (DO) in the same reporting instance FFS: Whether an additional UE procedure is needed when the reported DO value is ‘out of range’ FFS: Whether the Dd report codepoints need to be reinterpreted from intervals/ranges to values when the linkage mechanism is configured, or in general, a rule is needed to determine the value of DO for the compensation for each CMR for CJT CSI report (including if multiple Dos are reported)

Regarding the second FFS, we fail to see the necessity since it can be addressed by network implementation.

When linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, regarding the CSI processing timeline, the last meeting discussed whether to reuse or further relax the timeline for the Rel-18 eType-II CJT CSI report and made the following agreement [7].

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, the timeline (Z/Z’) is determined as Z/Z’ associated with the Rel-18 eType-II CJT, plus Drelax - The value of Drelax is a UE capability, taken from {0, drelax} - FFS: The value of drelax (>0), including whether it depends on SCS - For linking CJTC Dd and Rel-18 eType-II CJT CSI, joint triggering is a separate UE feature group from separate triggering

Regarding the CSI processing timeline, a remaining issue is the value of drelax and whether it depends on SCS. We don’t have strong views on the value of drelax, but at least it should depend on SCS.

Proposal 4: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, regarding the timeline, the value of drelax (>0) should depend on SCS.

Besides, for the linkage of CJTC Dd and Rel-18 eType-II CJT CSI reports configured with two separate triggers, RAN1#118bis meeting agreed to introduce a 1-bit indicator per CSI trigger state to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI. The last meeting further discussed two remaining issues and made the following agreements [7].

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, the 1-bit indicator per CSI trigger state applies to all the NTRP configured CSI-RS resources. Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, when a single CSI trigger state associated with >1 Rel-18 Type-II CJT CSI reports, the 1-bit indicator per CSI trigger state applies to all the Rel-18 Type-II CJT CSI reports linked with the CJTC Dd report(s).

RAN1#118bis meeting also discussed the linkage of CMRs associated with CJTC Dd and Rel-18 eType-II CJT CSI reports and achieved the following agreement. The last meeting further discussed and made a decision on the remaining issue, i.e., how to realize the linkage of CMRs when the number of CSI-RS resources configured for CJT CSI is less than the number of TRS resource sets configured for CJTC Dd, in case of separate triggers. The corresponding agreement is as follows.

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, regarding linking the CMRs in the two CSI Report Settings, for separate triggering, the UE expects that the number of configured CSI-RS resources associated with the Rel-18 eType-II CJT CSI is always the same as the number of TRS resource sets associated with the Rel-19 CJTC Dd report.

In addition, regarding the linkage of CJTC Dd and Rel-18 eType-II CJT CSI reports configured with a joint trigger, the last RAN1 meeting also discussed the UCI mapping order of the two reports in the same PUSCH. Considering that CJTC Dd report is a one-part UCI and Rel-18 eType-II CJT CSI report is a two-part UCI, the following agreement was achieved to provide clarification [7].

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint triggering carried on a same PUSCH (hence on a same slot), the UCI associated with the CJTC Dd report is multiplexed in CSI Part 1 - The previously agreed UCI design and mapping order for CJTC Dd report are reused - The legacy UCI design, UCI mapping order, and UCI omission for the Rel-18 eType-II CJT CSI are reused Note: The above proposal reuses the legacy UCI design principles, where the UCI associated with the CJTC Dd is placed in the part of UCI as TS 38212 Table 6.3.1.1.2-13; the CSI part 1 of Rel-18 eType-II CJT CSI is placed in the part of UCI as TS 38.212 Table 6.3.1.1.2-13 and the CSI part 2 of Rel-18 eType-II CJT CSI is placed in the part of UCI as TS 38.212 Table 6.3.1.1.2-14

## Frequency offset reporting

For frequency offset reporting, a remaining issue is whether to support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported frequency offset (when ReportQuantity is ‘cjtc-F’), using the same mechanism as that for UE-reported delay offset (when ReportQuantity is ‘cjtc-Dd’).

Unlike the delay offset, the phase rotation due to frequency offset will accumulate with time after UE reports the frequency offset, thus gNB needs to re-compensate the frequency offset when scheduling the PDSCH. From this perspective, it is unnecessary for UE to perform PMI calculation for the CJT CSI report assuming pre-compensation using the UE-reported frequency offset.

Proposal 5: Do not support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported frequency offset (when ReportQuantity is ‘cjtc-F’).

## DL/UL phase offset reporting

In realistic TDD systems, there would be phase offsets between UL and DL channels due to the non-ideal reciprocity caused by different Tx/Rx RF characteristics. In TDD CJT deployment, there are distinct UL-DL phase offsets over the UL-DL channels for each pair of UE and TRP. With such inter-TRP reciprocity error, the performance of TDD CJT would significantly degrade. Hence, the DL/UL phase offset reporting is supported to assist the NW in compensating for the inter-TRP reciprocity error.

The previous RAN1 meetings discussed the details for DL/UL phase offset reporting. To ensure the measurement accuracy of DL/UL phase offsets, it was agreed that for a given phase offset reporting configuration, UE can be configured with Q=1 associated SRS resource for antenna switching. Regarding the associated SRS resource, the following agreement was achieved in RAN1#118bis meeting.

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated SRS resource, - Confirm the following working assumption as agreement: “the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic” - When periodic or semi-persistent associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report - FFS: Whether ‘the earliest SRS transmission occasion after the NTRP CSI-RS occasions’ is also supported as an option - FFS: Whether determination of SRS transmission occasion is needed for aperiodic associated SRS resource, and if so, how

Regarding the configured associated SRS resource, it was agreed that when periodic or semi-persistent associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report. In previous meetings, it was proposed that ‘the earliest SRS transmission occasion after the NTRP CSI-RS occasions’ also can be supported. However, in the case of precoded CSI-RS transmission scheme, gNB needs to calculate the precoder of CSI-RS based on the estimation of SRS, so the SRS transmission occasion for determining the reference UE antenna port should be prior to the NTRP CSI-RS occasions.

Another remaining issue that should be addressed in this meeting is whether determination of SRS transmission occasion is needed for aperiodic associated SRS resource. Considering that the aperiodic associated SRS resource can be triggered multiple times, determination of SRS transmission occasion is also needed. To be specific, the SRS transmission occasion for determining the reference UE antenna port corresponds to the most recent transmission of the aperiodic associated SRS resource before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report.

Proposal 6: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated SRS resource, support the following:

- When periodic or semi-persistent associated SRS resource is configured, do not consider the earliest SRS transmission occasion after the NTRP CSI-RS occasions.

- When aperiodic associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the most recent transmission of the aperiodic associated SRS resource before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report.

# Conclusions

In this contribution, we discuss CSI enhancements in MIMO, including support for up to 128 CSI-RS ports on FR1 and UE reporting enhancement for CJT calibration, and the following proposals are made.

Proposal 1: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to 16, 24, and 32 CSI-RS ports, for all applicable RI values with K=1 only, and without any further modification/enhancement of the sub-features pertinent to the Rel-19 Type-I SP design (including, e.g. the Rel-19 Type-I SP CBSR, soft scaling).

- For the Rel-19 Type-I SP codebook, the support for 16, 24, and 32 ports are 3 separate UE capabilities from the support for the previously agreed number of ports (48, 64, 128 ports)

Proposal 2: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding aperiodic CSI-RS resource configuration, the number of bits for indicating the resource-level slot offset (relative to the resource-set-level slot offset) for KS resources can be 2 bits, and the value(s) of the slot offset can be {0, 1, 2, 3}.

Proposal 3: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding aperiodic CSI-RS resource configuration, not to introduce configuring an “available slot offset” for each CSI-RS resource within the aperiodic CSI-RS resource set, unless clear benefit is identified.

Proposal 4: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, regarding the timeline, the value of drelax (>0) should depend on SCS.

Proposal 5: Do not support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported frequency offset (when ReportQuantity is ‘cjtc-F’).

Proposal 6: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated SRS resource, support the following:

- When periodic or semi-persistent associated SRS resource is configured, do not consider the earliest SRS transmission occasion after the NTRP CSI-RS occasions.

- When aperiodic associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the most recent transmission of the aperiodic associated SRS resource before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report.

# References

- RP-242394, WID revision: NR MIMO Phase 5, RAN#105, Melbourne, Australia, September 9-12, 2024.

- Chairman’s Notes RAN1#116, Athens, Greece, February 26th - March 1st, 2024.

- Chairman’s Notes RAN1#116-bis, Changsha, Hunan Province, China, April 15th – 19th, 2024.

- Chairman’s Notes RAN1#117, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.

- Chairman’s Notes RAN1#118, Maastricht, NL, August 19th – 23rd, 2024.

- Chairman’s Notes RAN1#118bis, Hefei, China, October 14th – 18th, 2024.

- Chairman’s Notes RAN1#119, Orlando, US, November 18th – 22nd, 2024.

- R1-2410751, Moderator Summary on Rel-19 CSI enhancements , RAN1#119, Orlando, US, November 18th – 22nd, 2024.