3GPP TSG RAN WG1 #120	R1-2500303

Athens, Greece, February 17th – 21st, 2025

Source: 	CMCC

Title:	Discussion on the NR-NTN uplink capacity/throughput enhancements

Agenda item:	9.11.3

Document for:	Discussion & Decision

Introduction

Rel-19 WI on Non-Terrestrial Networks (NTN) for NR Phase 3 was updated in RP-243300[1] with the following objectives.

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In this contribution, we discuss potential enhancements on OCC based uplink capacity/throughput enhancements and provide preliminary LLS simulation results.

Signaling aspects of PUSCH with OCC

UCI multiplexing for PUSCH with inter-slot OCC

PUCCH without repetition overlaps PUSCH with inter-slot OCC

When considering PUSCH with OCC, the multiplexing of UCI presents specific challenges. According to the legacy UCI multiplexing rules outlined in TS 38.213 (for detailed information, see Annex II), if a PUSCH repetition occasion overlaps with a PUCCH carrying UCI, a UE may multiplex the UCI into the overlapped PUSCH repetition, provided certain timeline restrictions between the PUCCH and PUSCH are met.

However, this legacy rule encounters issues when PUSCH OCC is supported. As illustrated in Figure 1, assume that PUSCH repetitions for UEs 1 to 4 are multiplexed using inter-slot OCC with an OCC length of 4. If UE2 multiplexes its UCI onto the PUSCH in slot n+2, the orthogonality of the entire OCC group (i.e., from slot n to slot n+4) may be damaged, leading to the failure of all four UEs within the overlapped OCC group to demodulate their PUSCH transmissions correctly.

Therefore, potential enhancements to the UCI multiplexing rule for PUSCH with inter-slot OCC are necessary.

Figure 1. Issue of UCI multiplexing for PUSCH with inter-slot OCC.

During the RAN1#119 meeting, an agreement was reached on enhancement UCI multiplexing rule for PUSCH with inter-slot OCC [2].

Agreement If PUCCH without repetition overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group, the following options to be considered: - Option 1: UCI is dropped - FFS: whether all UCI is dropped - Option 2: UCI is transmitted on PUCCH, and all PUSCH repetitions within the OCC group are dropped. - Option 3: UCI is multiplexed on PUSCH with inter-slot OCC - Option 3-a: UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC - FFS: which OCC group - Option 3-b: UCI is multiplexed on PUSCH and OCC is not applied within the OCC group - Option 3-c: UCI is multiplexed on PUSCH and OCC is not applied within the PUSCH repetitions Note: combination of the above can be considered FFS Details timeline of PUCCH and PUSCH FFS: handling of PUCCH with repetition FFS: handling of different UCI types

Analysis of Options

- Option 1: UCI is dropped.

  - In our view, directly dropping UCI should be avoided since, generally, UCI is more critical than PUSCH repetitions.

- Option 2: UCI is transmitted on PUCCH, and all PUSCH repetitions within the OCC group are dropped.

  - As shown in Figure 2(a), if a PUCCH overlaps with a middle PUSCH occasion within an OCC group, there is insufficient time to multiplex the UCI onto all PUSCH occasions within the overlapped OCC group.

  - In this case, as depicted in Figure 2(b), it is more flexible to temporarily disable UCI multiplexing (i.e., drop the entire OCC group and carry UCI in PUCCH) to maintain the OCC orthogonality of other multiplexed UEs.

  - Although Option 2 resolves the issue of OCC orthogonality being damaged due to UCI multiplexing, the cost of PUSCH coverage performance degradation is significant since the PUSCH transmission for a UE within an entire OCC group is dropped.

  - It is preferable to combine Option 2 with Option 3-a to minimize the aforementioned cost. For example, if UCI can be multiplexed onto all PUSCH repetitions within an OCC group as certain timeline restrictions are met, adopt Option 3-a for better efficiency; otherwise, fallback to Option 2 for reliable UCI transmission.

(a)

(b)

Figure 2. Example of Option 2.

- Option 3: UCI is multiplexed on PUSCH with inter-slot OCC.

  - Option 3-a: UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC. FFS: which OCC group

    - Regarding the different interpretations of the FFS (Which OCC group), Option 3-a can be divided into two solutions.

      - Option 3-a-1 (Overlapped OCC Group): The referred OCC group is the overlapped OCC group.

        - As show in Figure 3 (a), if the timeline restrictions of PUCCH and PUSCH can be met, UCI is multiplexed onto all PUSCH repetitions within the overlapped OCC group.

        - If the aforementioned timeline restrictions cannot be met, the UE may fallback to Option 2 (i.e., temporarily disable UCI multiplexing) for reliable UCI transmission.

      - Option 3-a-2 (Next Available OCC Group): The referred OCC group is the first available OCC group after the overlapped OCC group that meets certain timeline restrictions.

        - As show in Figure 3 (b), UCI can be multiplexed onto all PUSCH repetitions within the first available OCC group after the overlapped OCC group that meets certain timeline restrictions.

        - It is evident that Option 3-a-2 can operate independently.

      - The effect of Option 3-a is illustrated in Figure 3 (c).

(a) Option 3-a-1 (overlapped OCC group)

(b) Option 3-a-2 (next available OCC group)

(c)

Figure 3. Example of Option 3-a: UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC. FFS: which OCC group.

  - Option 3-b: UCI is multiplexed on PUSCH and OCC is not applied within the OCC group

    - As shown in Figure 1, if a UE with UCI multiplexing (e.g., UE2) is already multiplexed with other UEs (e.g., UE1, UE3, UE4) using inter-slot OCC, UCI multiplexing in one slot of such UE may damage the orthogonality of the entire overlapped OCC group, resulting in the failure of all UEs within the overlapped OCC group to demodulate their PUSCH transmissions correctly. This means that Option 3-b is not viable when there is UE multiplexing.

    - On the other hand, if UE multiplexing can be avoided by gNB scheduling, there is no need for PUSCH OCC. Thus, Option 3-b is meaningless.

    - Based on the above discussion, we do not support Option 3-b.

  - Option 3-c: UCI is multiplexed on PUSCH and OCC is not applied within the PUSCH repetitions

    - A similar analysis for Option 3-b also applies to Option 3-c, i.e., if UE multiplexing is considered, Option 3-c cannot work, while if UE multiplexing is not considered, there is no need for PUSCH OCC. Thus, we do not support Option 3-c.

Regarding the Note (Note: combination of the above can be considered), the combination of Option 2 and Option 3-a-1 can be considered, i.e.,

- Combination of Option 3-a-1 and Option 2:

  - (Option 3-a-1) If the timeline restrictions of PUCCH and PUSCH can be met, UCI is multiplexed onto all PUSCH repetitions within the overlapped OCC group.

  - (Option 2) Otherwise, UCI is transmitted on PUCCH, and all PUSCH repetitions within the overlapped OCC group are dropped.

Proposal 1:

If PUCCH without repetition overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group, the following options are supported.

- Option 3-a-2: UCI can be multiplexed on all PUSCH repetitions within the first available OCC group after the overlapped OCC group that meets certain timeline restrictions.

- Combination of Option 3-a-1 and Option 2:

  - (Option 3-a-1) If the certain timeline restriction of PUCCH and PUSCH can be met, UCI is multiplex onto all PUSCH repetitions within the overlapped OCC group.

  - (Option 2) Otherwise, UCI is transmitted on PUCCH, and all PUSCH repetitions within the overlapped OCC group are dropped.

Proposal 2:

If a PUCCH without repetition overlaps with any PUSCH repetitions within an OCC group using inter-slot OCC, the following options are not supported.

- Option 1: UCI is dropped

- Option 3: UCI is multiplexed on PUSCH with inter-slot OCC

  - Option 3-b: UCI is multiplexed on PUSCH and OCC is not applied within the OCC group

  - Option 3-c: UCI is multiplexed on PUSCH and OCC is not applied within the PUSCH repetitions

PUCCH with repetition overlaps PUSCH with inter-slot OCC

Based on the legacy UCI multiplexing rule outlined in TS 38.213 (for detailed information, see Annex II), if a PUSCH repetition occasion overlaps with a PUCCH with repetition, the UE may transmit the PUCCH and not transmit the overlapped PUSCH repetition occasion.

Following the legacy design principle, if a PUCCH with repetition overlaps with any PUSCH repetitions within an OCC group using inter-slot OCC, UCI can be transmitted on PUCCH, and all PUSCH repetitions within the overlapped OCC group are dropped.

Proposal 3:

If a PUCCH with repetition overlaps with any PUSCH repetitions within an OCC group using inter-slot OCC, UCI is transmitted on PUCCH, and all PUSCH repetitions within the overlapped OCC group are dropped.

RV cycling

RV cycling can helps improve the reliability of physical layer data transmission. However, the current RV cycling pattern cannot be well compatible with the inter-slot OCC scheme.

For CG PUSCH, the higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. According to the TS38.331, three RV pattern is supported for CG PUSCH.

Only the fixed pattern with all 0 RV could be used when inter-slot OCC is applied. Considering the minimized spec impact and backward compatibility, no new RV cycling pattern is needed for CG PUSCH.

Proposal 4:

When inter-slot OCC is applied to CG PUSCH, only s3-0000 can be used for repK-RV.

Proposal 5:

No new RV cycling pattern is introduced for CG PUSCH.

For DG PUSCH, according to the TS38.214 table 6.1.2.1-2, only RV cycling pattern {0 2 3 1} is supported. To enable the inter-slot OCC scheme with DG PUSCH, enhancement/ modification for RV cycling is needed, and the following options is agreed during the last meeting.

Agreement For RV cycling for OCC with DG-PUSCH, the following are considered: - Option 1: RV cycling is used across OCC groups - Note 1: RV cycling is applied when the number of repetitions is greater than the OCC length - Option 2: Fixed RV is used across OCC groups - Option 3: For OCC length 2, fixed RV is used across two OCC groups, RV cycling is used across groups of two OCC groups

The additional combination gain can be achieved through RV cycling as in Option 1. But if the RV cycling is used across the OCC groups, since different contents or RVs are transmitted in different OCC groups, different OCC groups cannot be demodulated or de-spreading together. But on the contrary, the option 2 with fixed RV across different OCC group can combine the content from different OCC groups to improve the demodulation. The option 3 is a special case of option 1 with further grouping of OCC groups. The benefits and performance needs further clarification.

Observation 1:

The benefits and performance of Option 3 need further clarification.

Proposal 6:

Both option 1 with RV cycling and option 2 with fixed RV across OCC groups can be prioritised.

OCC indication/ configuration

OCC length can be configured via RRC signalling. And the OCC patterns can be specified. The specific OCC sequence used for PUSCH transmission can be indicated through the DCI signalling either explicitly or implicitly. The indication through dynamic signalling can bring flexibility for UE paring.

Proposal 7:

The indication of OCC sequence can be either explicit or implicit.

Frequency hopping

The intention of frequency hopping is to use the frequency diversity to counter the deep fading in the frequency domain. But considering the NTN working scenario is almost the LOS scenario. And based on the simulation assumption of very low delay spreads, the diversity gain in the frequency domain would be very low. On the other side, the performance of OCC multiplexing is very sensitive to the change of channel response and the channel estimation to keep the orthogonality between multiplexed UEs. But the frequency hopping would change the location of PRBs, which induce different channel response in the frequency domain. And the change of frequency location would also break the phase continuity if DMRS bundling or the joint channel estimation is used, which also reduce the performance of channel estimations. Then it should be further studied whether to support the frequency hopping in the NTN scenario and with the OCC multiplexing in the time domain.

Proposal 8:

It should be further studied whether to support the frequency hopping in the NTN scenario and with the OCC multiplexing in the time domain.

Conclusions

In this contribution, we discuss potential enhancements on OCC based uplink capacity/throughput enhancement and provide preliminary LLS simulation results. The observations and proposals are listed below.

Observation 1:

The benefits and performance of Option 3 need further clarification.

Proposal 1:

If PUCCH without repetition overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group, the following options are supported.

- Option 3-a-2: UCI can be multiplexed on all PUSCH repetitions within the first available OCC group after the overlapped OCC group that meets certain timeline restrictions.

- Combination of Option 3-a-1 and Option 2:

  - (Option 3-a-1) If the certain timeline restriction of PUCCH and PUSCH can be met, UCI is multiplex onto all PUSCH repetitions within the overlapped OCC group.

  - (Option 2) Otherwise, UCI is transmitted on PUCCH, and all PUSCH repetitions within the overlapped OCC group are dropped.

Proposal 2:

If a PUCCH without repetition overlaps with any PUSCH repetitions within an OCC group using inter-slot OCC, the following options are not supported.

- Option 1: UCI is dropped

- Option 3: UCI is multiplexed on PUSCH with inter-slot OCC

  - Option 3-b: UCI is multiplexed on PUSCH and OCC is not applied within the OCC group

  - Option 3-c: UCI is multiplexed on PUSCH and OCC is not applied within the PUSCH repetitions

Proposal 3:

If a PUCCH with repetition overlaps with any PUSCH repetitions within an OCC group using inter-slot OCC, UCI is transmitted on PUCCH, and all PUSCH repetitions within the overlapped OCC group are dropped.

Proposal 4:

When inter-slot OCC is applied to CG PUSCH, only s3-0000 can be used for repK-RV.

Proposal 5:

No new RV cycling pattern is introduced for CG PUSCH.

Proposal 6:

Both option 1 with RV cycling and option 2 with fixed RV across OCC groups can be prioritised.

Proposal 7:

The indication of OCC sequence can be either explicit or implicit.

Proposal 8:

It should be further studied whether to support the frequency hopping in the NTN scenario and with the OCC multiplexing in the time domain.

References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#106, Madrid, Spain, December 9-12, 2024

- Chairman’s Notes RAN1#119, Orlando, US, November 18th – 22nd, 2024.

- Chairman's Notes RAN1#118bis, Hefei, CN, October 14th – October 18th, 2024

Annex II: Quotations of section 9 from TS 38.213

[TS 38.213] - 9 UE procedure for reporting control information If a UE transmits a PUSCH over multiple slots or multiple PUSCHs over multiple slots that are scheduled by a DCI format 0_1, and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots of the multiple slots, and the PUSCH transmission in the one or more slots fulfills the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, the UE multiplexes the HARQ-ACK and/or CSI information in the PUSCH transmission in the one or more slots. The UE does not multiplex HARQ-ACK and/or CSI information in the PUSCH transmission in a slot from the multiple slots if the UE would not transmit a single-slot PUCCH with HARQ-ACK and/or CSI information in the slot in case the PUSCH transmission was absent. If a UE transmits a PUSCH with repetition Type B and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots, the UE expects all actual repetitions of the PUSCH transmission [6, TS 38.214] that would overlap with the PUCCH transmission to fulfill the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, and the UE multiplexes the HARQ-ACK and/or CSI information in the earliest actual PUSCH repetition of the PUSCH transmission that would overlap with the PUCCH transmission and includes more than one symbol. The UE does not expect that all actual repetitions that would overlap with the PUCCH transmission do not include more than one symbol. If the PUSCH transmission over the multiple slots is scheduled by a DCI format that includes a DAI field, the value of the DAI field is applicable for multiplexing HARQ-ACK information in the PUSCH transmission in any slot from the multiple slots where the UE multiplexes HARQ-ACK information. When a UE would multiplex HARQ-ACK information in a PUSCH transmission that is configured by a ConfiguredGrantConfig, and includes CG-UCI [5, TS 38.212], the UE multiplexes the HARQ-ACK information in the PUSCH transmission if the UE is provided cg-UCI-Multiplexing; otherwise, if the HARQ-ACK information and the PUSCH have same priority index, the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information in a PUCCH transmission or in another PUSCH transmission; if the HARQ-ACK information and the PUSCH have different priority indexes, the UE does not transmit the channel with the smaller priority index. - 9.2.6 PUCCH repetition procedure A UE can be indicated to transmit a PUCCH over NPUCCHrepeat slots using a PUCCH resource, where - if the PUCCH resource is indicated by a DCI format and includes pucch-RepetitionNrofSlots, NPUCCHrepeat is provided by pucch-RepetitionNrofSlots - otherwise, NPUCCHrepeat is provided by nrofSlots If a UE would transmit a PUCCH over a first number NPUCCHrepeat>1 of slots and the UE would transmit a PUSCH with repetition Type A or with TB processing over multiple slots over a second number of slots, and the PUCCH transmission would overlap with the PUSCH transmission in one or more slots, and the conditions in clause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied in the overlapping slots, the UE transmits the PUCCH and does not transmit the PUSCH in the overlapping slots. If a UE would transmit a PUCCH over a first number NPUCCHrepeat>1 of slots and the UE would transmit a PUSCH with repetition Type B over a second number of slots, and the PUCCH transmission would overlap with actual PUSCH repetitions in one or more slots, and the conditions in clause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied for the overlapping actual PUSCH repetitions, the UE transmits the PUCCH and does not transmit the overlapping actual PUSCH repetitions.